| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0182-v1 |
| --- | --- |

| Title: | AHG9: JPEG segments SEI message |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | P. de Lagrange D. Doyen E. François F. Urban C. Salmon-Legagneur | Email: | philippe.delagrange@interdigital.com |
| Source: | InterDigital |  |  |  |

_____________________________

# Abstract

During the 32nd JVET meeting, the contribution JVET-AF0141 proposed new SEI messages to embed externally defined camera-related metadata structures (JFIF, EXIF or XMP). This is said to provide somewhat redundant functionality, with EXIF additionally raising concern because of the need to refer to specifications from non-SDO bodies like CIPA and JEITA. This contribution proposes to replace or merge the JFIF SEI message with a new SEI message embedding generic JPEG segments, that is said to go around these issues while providing more functionality. Indeed, some JPEG segments named “APP segments” are free to use for applications with any kind of payload and are commonly used to embed such camera-related metadata. This solution is also reported to offer additional possibilities such as embedding full JPEG pictures in a video bitstream, for instance for indexing purpose or hi-resolution snapshots.

# Introduction

This contribution was triggered by JVET-AF0141 that proposed new SEI messages to embed externally defined camera-related metadata structures, for example JFIF and EXIF. One concern raised for this proposal was to refer to specifications from non-SDO bodies like CIPA and JEITA.

On the other hand, JPEG, a sibling working group of JVET within ISO, has defined a file format where that problem does not exist: JPEG files can readily embed JFIF of course (as part of JPEG specifications), but also various kinds of external metadata, including EXIF, XMP, IPTC, ICC color profiles, etc. This is not defined by JPEG specifications, but rather JPEG files are made of basic chunks called “segments” (see next section), some of them being “APPn segments” and being free to use for applications with any kind of payload. They usually start with a string identifying the type of data. JPEG defined APP0 with “JFIF” string to carry JFIF metadata or with “JFXX” string to carry JFIF extensions, and others followed the example: CIPA/JEITA defined that to carry EXIF into a JPEG file, the APP1 segment would be used and start with the “EXIF” string, followed by EXIF data. That way, the JPEG specifications do not depend on EXIF specifications: it is the other way round. A similar method is used for other types of metadata.

So we could just embed JPEG segments in an SEI message to indirectly support any kind of metadata already supported in JPEG files.

Actually, it even makes sense to embed full JPEG pictures in a video bitstream. Attaching still pictures as additional information to a video bitstream could be useful for several cases, for example:

- Thumbnails (for indexing purpose)

- Hi-resolution still pictures at specific times, for example in a drone footage, or a surveillance camera to capture details when triggered by specific events

- Pictures of products/objects related to video content

- Utility pictures, like maps, 360° views, non-visible-light pictures, scanned documents

# JPEG file structure (segments)

As defined in ITU-T T.81 | ISO/IEC 10918-1 annex B, section B.1, a JPEG file is made of a succession of segments (see Figure 1 to Figure 3), each starting with a “marker” (two bytes, the first being 0xFF and the second neither 0xFF nor 0x00), followed by a structure format that depends on each marker. Some markers are isolated (like SOI (Start Of Image) or EOI (End Of Image) with no payload, but most segments contain a payload length parameter (two bytes) just after the marker, followed by data bytes. A marker can be preceded by any number of 0xFF stuffing bytes.

A list of marker code assignments is found is Table B.1 of ITU-T T.81. A range of markers, 0xFFE0 to 0xFFEF (or “APPn” with n from 0 to 15) indicate “Application segments”, which payload data typically begins with a null-terminated character string. For example, APP0 followed by “JFIF” or “JFXX” is defined in ITU-T T.871. APP1 followed by “EXIF” string is commonly used to carry EXIF metadata, though this is not specified in ITU-T T.81 nor in ITU-T T.871, but more of an ad-hoc use by the industry (usually defined by the bodies that specify those metadata). Other examples are XMP, using an APP1 segment starting with the “http://ns.adobe.com/xap/1.0/” string, or ICC profiles using APP2 segments starting with the “ICC_PROFILE” string, etc.

A JPEG file can also contain general comments (free text, marker 0xFFFE known as “COM”).

Figure 1 – Example structure of a JPEG file (ITU-T T.81 Annex B)

Figure 2 – Structure of a JPEG segment with payload

Figure 3 – Example of APP0 JFIF segment (ITU-T T.871)

# Proposed SEI message

Proposed syntax and semantics are the following:

Table 1 – Proposed syntax

| jpeg_segments( payloadSize ) { | Descriptor |
| --- | --- |
| jpeg_segments_cancel_flag | u(1) |
| if( !jpeg_segments_cancel_flag ) { |  |
| jpeg_segments_persistence_flag | u(1) |
| for( i = 0; i < payloadSize-1; i++ ) { |  |
| jpeg_segments_payload_byte[ i ] | b(8) |
| } |  |
| } |  |
| } |  |

jpeg_segments_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous JPEG segments SEI message in output order. jpeg_segments_cancel_flag equal to 0 indicates that JPEG segments information follows.

jpeg_segments_persistence_flag specifies the persistence of the JPEG segments SEI message for the current layer.

jpeg_segments_persistence_flag equal to 0 specifies that the JPEG segments SEI message applies to the current decoded picture only.

jpeg_segments_persistence_flag equal to 1 specifies that the JPEG segments SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

– A new CLVS of the current layer begins.

– The bitstream ends.

– A picture in the current layer in an AU associated with a JPEG segments SEI message is output that follows the current picture in output order.

jpeg_segments_payload_byte[ i ] contains the i-th byte of a byte sequence conforming to ITU-T T.81 | ISO/IEC 10918-1 annex B, section B.1.

Notes:

– the byte sequence can contain an arbitrary collection of segments and fill bytes (as defined in ITU-T T.81 | ISO/IEC 10918-1 annex B, section B.1) and is not required to start with an SOI marker and end with an EOI marker. If we want to restrict to full JPEG files, we can just say “conforming to T.81 annex B” (leaving out section B.1, since the other B.x sections define what a full JPEG file is).

– it is valid to have a “full” JPEG file starting with SOI, ending with EOI, but containing only metadata (no picture).

– for AVC, jpeg_segments_persistence_flag can be replaced by jpeg_segments_repetition_period to be consistent with other SEI messages (it is then coded as ue(v) and values greater than 1 indicate the number of POCs the message is valid).

# Conclusion

The new JFIF, EXIF, and XMP SEI messages proposed by JVET-AF0141 seem to provide somewhat redundant functionality, namely camera-related metadata. It may be preferable to select one, and in that case our choice would be XMP (ISO 16684-1) as the most recent and flexible one.

The JPEG segments SEI message proposed in this contribution could replace or be merged with the JFIF metadata SEI message currently in JVET-AF2032, providing a broader usage.

# References

- A. Hinds, S. Wenger, “SEI messages for common image metadata formats”, JVET-AF0141, Hannover, DE, Oct. 2023

- M. Hannuksela, J. Chen, S. Deshpande, Hendry, S. McCarthy, “Technologies under consideration for future extensions of VSEI (version 2)”, JVET-AF2032, Hannover, DE, Oct. 2023

- Recommendation ITU-T T.81 (1992), Information technology - Digital compression and coding of continuous-tone still images - Requirements and guidelines

- Recommendation ITU-T T.871 (1992), Information technology - Digital compression and coding of continuous-tone still images: JPEG File Interchange Format (JFIF)

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).