| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0196-v1 |
| --- | --- |

| Title: | EE2.5-1/2: CABAC context initialization retraining and slice type based window offsets |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Galpin, F. Lo Bianco, C. Salmon-Legagneur, K. Naser V. Seregin, M. Karczewicz | Tel: Email: | firname.lastname@interdigital.com vseregin@qti.qualcomm.com |
| Source: | InterDigital and Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution summarizes EE2 Test 5.1 results for CABAC context retraining in ECM-11.0 for all slice types (I, P, and B) and EE2 Test 5.2 results when additionally, window offsets initialization is defined per slice type as for other CABAC parameters.

Test results following common test conditions using ECM-11.0 are summarized in the next tables.

Test 5.1:

|  | Y | U | V | EncT | DecT | VmPeakE | VmPeakD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| All intra | -0.04% | 0.05% | 0.01% | 98.4% | 98.2% | 99.2% | 100.0% |
| Random access | -0.14% | 0.28% | 0.15% | 98.5% | 100.1% | 99.9% | 100.1% |
| Low delay B | -0.06% | 0.77% | 0.56% | 102.5% | 103.4% | 99.8% | 103.6% |
| Low delay P | -0.02% | 0.43% | 0.65% | 101.4% | 105.0% | 99.5% | 100.3% |

Test 5.2:

|  | Y | U | V | EncT | DecT | VmPeakE | VmPeakD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| All intra | -0.04% | 0.15% | -0.01% | 99.8% | 100.2% | 99.0% | 99.5% |
| Random access | -0.13% | 0.27% | 0.16% | 98.7% | 99.8% | 100.0% | 99.9% |
| Low delay B | -0.10% | 0.60% | 0.54% | 103.7% | 104.8% | 98.8% | 103.6% |
| Low delay P | -0.15% | 0.32% | 0.42% | 103.6% | 107.4% | 99.2% | 99.7% |

# Proposed method

In ECM, CABAC has the following parameters: probability states (6 bits), short and long window sizes (4 bits), weights (6 bits), and offsets for short (8 bits) and long (8 bits) windows. In total, (6 bits + 4 bits + 6 bits) * 3 slices + 8 bits * 2 windows = 64 bits is required per context.

All parameters except offsets are defined per slice type.

Having window offsets defined for all slices is suboptimal since there may be no single optimal value for all slice types.

This contribution proposes CABAC initialization retraining for ECM-11.0 as Test 5.1, and to make window offsets initialization defined per slice type as for other CABAC parameters as Test 5.2.

In Test 5.2, window offsets for 2 slice types are added which require 8 bits * 2 windows * 2 slices = 32 bits additional memory per context.

Context training software is also provided located at source/App/CabacTraining including readme file describing how to use it.

# Simulation results

The proposed method was implemented on top of ECM-11.0 and tested following common test condition described in JVET-AF2017.

Table 1. Results of test EE2-5.1

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.04% | 0.04% | 0.07% | 99.1% | 97.2% | 99.9% | 99.9% |
| Class A2 | 0.01% | 0.03% | -0.04% | 98.9% | 98.7% | 99.9% | 100.2% |
| Class B | -0.02% | 0.10% | -0.01% | 98.5% | 98.2% | 98.2% | 99.9% |
| Class C | -0.06% | 0.01% | 0.12% | 98.1% | 98.2% | 99.5% | 100.0% |
| Class E | -0.08% | 0.05% | -0.12% | 97.8% | 98.6% | 98.6% | 100.0% |
| Overall | -0.04% | 0.05% | 0.01% | 98.4% | 98.2% | 99.2% | 100.0% |
| Class D | -0.11% | 0.05% | 0.16% | 98.7% | 97.6% | 99.6% | 100.0% |
| Class F | -0.07% | -0.21% | -0.14% | 101.3% | 102.0% | 99.8% | 100.2% |
| Class TGM | -0.16% | -0.13% | -0.23% | 99.1% | 105.8% | 99.9% | 100.1% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.13% | 0.45% | 0.26% | 99.5% | 100.5% | 101.0% | 100.2% |
| Class A2 | -0.06% | 0.18% | 0.05% | 99.2% | 101.2% | 99.9% | 100.5% |
| Class B | -0.13% | 0.36% | 0.14% | 99.1% | 102.0% | 99.4% | 100.0% |
| Class C | -0.21% | 0.10% | 0.17% | 96.7% | 96.9% | 99.8% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.14% | 0.28% | 0.15% | 98.5% | 100.1% | 99.9% | 100.1% |
| Class D | -0.34% | 0.69% | 0.40% | 97.8% | 98.2% | 99.3% | 100.1% |
| Class F | -0.26% | -0.15% | -0.07% | 96.4% | 96.5% | 99.5% | 101.5% |
| Class TGM | -0.31% | -0.37% | -0.28% | 108.9% | 111.6% | 99.9% | 104.2% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.11% | 1.22% | 1.13% | 103.4% | 102.6% | 100.0% | 104.9% |
| Class C | 0.02% | 0.57% | 0.17% | 101.8% | 103.7% | 99.9% | 102.6% |
| Class E | -0.09% | 0.27% | 0.14% | 102.1% | 104.4% | 99.3% | 102.9% |
| Overall | -0.06% | 0.77% | 0.56% | 102.5% | 103.4% | 99.8% | 103.6% |
| Class D | -0.10% | 0.49% | -0.94% | 101.0% | 102.4% | 97.7% | 100.8% |
| Class F | -0.21% | 0.38% | 0.17% | 103.9% | 105.5% | 98.2% | 103.4% |
| Class TGM | -0.53% | -0.59% | -0.51% | 104.7% | 102.1% | 99.4% | 107.7% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.13% | 0.50% | 0.74% | 102.4% | 107.8% | 100.4% | 100.3% |
| Class C | -0.03% | 0.45% | 0.53% | 100.9% | 104.4% | 100.5% | 100.1% |
| Class E | 0.20% | 0.30% | 0.68% | 100.2% | 101.3% | 96.7% | 100.4% |
| Overall | -0.02% | 0.43% | 0.65% | 101.4% | 105.0% | 99.5% | 100.3% |
| Class D | -0.09% | 0.12% | -0.35% | 100.5% | 101.2% | 97.6% | 100.0% |
| Class F | 0.42% | 1.13% | 0.30% | 98.9% | 98.6% | 99.8% | 100.3% |
| Class TGM | -0.21% | -0.31% | -0.29% | 102.4% | 111.1% | 97.8% | 100.3% |

Table 2. Results of test EE2-5.2

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | 0.15% | 0.01% | 99.8% | 100.5% | 99.8% | 99.5% |
| Class A2 | -0.02% | 0.20% | -0.03% | 99.8% | 101.4% | 99.9% | 99.7% |
| Class B | -0.03% | 0.15% | -0.04% | 99.4% | 99.0% | 98.0% | 99.6% |
| Class C | -0.06% | 0.11% | 0.01% | 100.9% | 100.9% | 99.2% | 99.3% |
| Class E | -0.08% | 0.16% | 0.03% | 98.7% | 100.1% | 98.4% | 99.3% |
| Overall | -0.04% | 0.15% | -0.01% | 99.8% | 100.2% | 99.0% | 99.5% |
| Class D | -0.10% | 0.08% | 0.15% | 99.6% | 99.2% | 99.1% | 99.3% |
| Class F | -0.06% | -0.17% | 0.10% | 101.8% | 103.8% | 99.6% | 99.5% |
| Class TGM | -0.13% | -0.09% | -0.28% | 99.7% | 103.1% | 99.7% | 99.5% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.13% | 0.41% | 0.27% | 99.6% | 100.7% | 100.9% | 100.5% |
| Class A2 | -0.06% | 0.21% | 0.21% | 100.4% | 102.5% | 100.4% | 100.3% |
| Class B | -0.13% | 0.24% | 0.17% | 98.3% | 100.2% | 99.3% | 99.7% |
| Class C | -0.18% | 0.23% | 0.04% | 97.3% | 96.6% | 99.8% | 99.5% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.13% | 0.27% | 0.16% | 98.7% | 99.8% | 100.0% | 99.9% |
| Class D | -0.29% | 0.60% | 0.12% | 99.0% | 99.6% | 99.2% | 99.5% |
| Class F | -0.27% | -0.25% | -0.42% | 97.9% | 97.3% | 99.9% | 100.8% |
| Class TGM | -0.39% | -0.39% | -0.36% | 110.5% | 112.4% | 100.3% | 103.9% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.10% | 1.11% | 0.83% | 105.0% | 103.7% | 99.9% | 104.7% |
| Class C | 0.03% | 0.47% | 0.29% | 102.5% | 104.4% | 99.8% | 102.4% |
| Class E | -0.25% | -0.09% | 0.38% | 103.2% | 107.0% | 95.8% | 103.2% |
| Overall | -0.10% | 0.60% | 0.54% | 103.7% | 104.8% | 98.8% | 103.6% |
| Class D | -0.11% | 0.48% | -0.03% | 102.5% | 102.6% | 97.7% | 100.0% |
| Class F | -0.19% | 0.81% | 0.25% | 103.6% | 105.4% | 99.2% | 103.9% |
| Class TGM | -0.55% | -0.49% | -0.54% | 105.5% | 101.4% | 100.3% | 106.4% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.14% | 0.43% | 0.27% | 107.3% | 112.3% | 100.3% | 99.9% |
| Class C | -0.11% | 0.07% | 0.39% | 101.1% | 104.1% | 99.2% | 99.5% |
| Class E | -0.20% | 0.47% | 0.72% | 100.9% | 103.9% | 97.6% | 99.7% |
| Overall | -0.15% | 0.32% | 0.42% | 103.6% | 107.4% | 99.2% | 99.7% |
| Class D | -0.18% | 0.08% | 0.79% | 102.1% | 101.8% | 97.1% | 99.3% |
| Class F | 0.20% | 1.29% | 1.70% | 101.8% | 100.3% | 99.7% | 99.7% |
| Class TGM | -0.21% | -0.34% | -0.54% | 102.8% | 111.1% | 98.8% | 100.0% |

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).