| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0199 |
| --- | --- |

| Title: | EE2-1.13: Combination tests of IntraTMP search range extension with adaptive sampling and IBC with upward-extended area |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Yoshitaka Kidani, Haruhisa Kato, Kei Kawamura (KDDI) 2Damian Ruiz Coll (Ofinno) 3Karam Naser, Philippe Bordes, Fabrice Le Léannec, Franck Galpin (InterDigital) | Tel: Email: | 1yo-kidani@kddi.com 2druizcoll@ofinno.com 3karam.naser@interdigital.com |
| Source: | KDDI Corporation (KDDI Research, Inc.), Ofinno, Interigital |  |  |

_____________________________

- Abstract

This contribution reports the experimental results of Test 1.13, which is the combination of Test 1.11c (IntraTMP search range extension with adaptive sampling) and Test 1.12 (IBC with upward-extended reference area) in the context of Exploration Experiments 2 (JVET-AF2024).

In Test 1.11c, the IntraTMP search area is extended to 2 CTU rows aligned with the IBC search region, and an adaptive sampling rate is determined as a combination of the searching distance from the current CU and the dimensions of each search region.

In Test 1.12, the reference area of IBC is further extended in an upward direction. The following two extension methods are tested for camera-captured content and screen contents: Test 1.12a is IBC with the extended reference area to the picture's upper side boundary; Test 1.12b with four CTU rows instead of the existing two CTU rows above the current CTU in HD resolution or less.

The experimental results reported show the best performance in Test 1.13a (the combination of Test 1.11c and Test 1.12a) and Test 1.13b (the combination of Test 1.11c and Test 1.12b), over ECM-11.0 as follows:

Test 1.13a

AI:

Overall: {0.02%, 0.04%, 0.04%, 100.0%, 100.7%};

Class F: {-0.39%, -0.46%, -0.36%, 100.9%, 101.3%};

Class TGM: {-1.59%, -1.57%, -1.59%, 100.5%, 100.7%}.

RA:

Overall: {%	, %, %, %, %};

Class F: {%	, %, %, %, %};

Class TGM: {%	, %, %, %, %}.

Test 1.13b

AI:

Overall: {%	, %, %, %, %};

Class F: {%	, %, %, %, %};

Class TGM: {%	, %, %, %, %}.

RA:

Overall: {%	, %, %, %, %};

Class F: {%	, %, %, %, %};

Class TGM: {%	, %, %, %, %}.

Introduction

In the 32nd JVET meeting, JVET-AF0161 [1] and JVET-AF0115 [2] investigated an IntraTMP search range extension with adaptive sampling and an IBC with upward-extended reference area only for screen content, to achieve further coding performance of ECM. The combination of these technologies is studied as Test 1.13 in this JVET meeting. Note that the proponents of JVET-AF0161 and JVET-AF0115 were requested to report the experimental results in which these technologies are applied for camera-captured content in the 32nd JVET meeting, so Test 1.13 reports the results of both camera-captured content and screen content.

Test description

In Test 1.13, the two following tests are conducted:

Test 1.13a: Test 1.11c + Test 1.12a,

Test 1.13b: Test 1.11c + Test 1.12b.

Experimental results

The proposed methods were implemented on top of ECM-11.0 [3] and tested according to the common test condition of AI and RA configuration as specified in the JVET-AF2017 [4] and JVET-AF2024 [5].

Experimental results of Test 1.13a and Test 1.13b are summarized below. More detailed results can be found in the attached spreadsheets.

Table 1 Test 1.13a results over ECM-11.0 for screen content.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class F | -0.39% | -0.46% | -0.36% | 100.9% | 101.3% | 102.7% | 100.9% |
| Class TGM | -1.59% | -1.57% | -1.59% | 100.5% | 100.7% | 99.1% | 101.7% |
|  | Random Access Intra Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

Table 2 Test 1.13a results over ECM-11.0 for camera-captured content.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | 0.07% | 0.03% | 101.6% | 103.3% | 99.0% | 105.2% |
| Class A2 | 0.02% | 0.05% | 0.06% | 100.9% | 101.8% | 100.4% | 105.1% |
| Class B | 0.01% | -0.03% | -0.05% | 98.8% | 100.3% | 98.3% | 101.9% |
| Class C | 0.00% | -0.03% | 0.07% | 99.8% | 100.1% | 101.8% | 100.2% |
| Class E | 0.07% | 0.21% | 0.15% | 99.8% | 98.7% | 100.0% | 100.7% |
| Overall | 0.02% | 0.04% | 0.04% | 100.0% | 100.7% | 99.8% | 102.4% |
| Class D | 0.03% | -0.14% | -0.06% | 99.5% | 99.8% | 100.1% | 99.9% |
|  | Random Access Intra Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.04% | 0.16% | 0.01% | 100.2% | 100.2% | 100.2% | 101.4% |
| Class C | 0.02% | -0.14% | 0.12% | 99.9% | 99.7% | 100.0% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.04% | 0.09% | -0.14% | 99.6% | 100.5% | 99.4% | 99.9% |

Table 3 Test 1.13b results over ECM-11.0 for screen content.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class F | -0.26% | -0.15% | -0.17% | 100.4% | 100.3% | 100.3% | 101.3% |
| Class TGM | -1.53% | -1.55% | -1.58% | 100.1% | 101.0% | 98.7% | 101.5% |
|  | Random Access Intra Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class F | -0.14% | -0.41% | -0.24% | 100.1% | 99.6% | 100.3% | 101.6% |
| Class TGM |  |  |  |  |  |  |  |

Table 4 Test 1.13b results over ECM-11.0 for camera-captured content.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | -0.05% | -0.06% | 101.1% | 102.1% | 101.1% | 102.4% |
| Class A2 | 0.01% | 0.04% | 0.05% | 101.0% | 102.2% | 100.0% | 101.9% |
| Class B | 0.02% | 0.08% | -0.10% | 98.9% | 100.6% | 98.9% | 101.8% |
| Class C | 0.00% | -0.06% | -0.08% | 100.1% | 99.6% | 100.4% | 101.2% |
| Class E | 0.05% | 0.22% | 0.02% | 100.2% | 98.4% | 104.4% | 101.5% |
| Overall | 0.01% | 0.04% | -0.04% | 100.1% | 100.5% | 100.7% | 101.7% |
| Class D | 0.03% | -0.17% | 0.02% | 100.0% | 99.4% | 100.2% | 100.7% |
|  | Random Access Intra Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |

Conclusion

This contribution reports combination test results of Test 1.11c (i.e., IntraTMP search range extension with adaptive sampling) and Test 1.12 (i.e., IBC with upward-extended area). For screen content, some additive coding gains with negligible complexity increases are observed, and the Test 1.13’s trade-off between the coding gain and complexity is better than that of Test 1.13b. It is recommended to adopt the Test 1.13a into the next ECM only for screen content.

Reference

- D. Ruiz Coll, B. Chen, K. Naser, P. Bordes, F. Le Léannec, and F. Galpin, “AHG12: Harmonized IBC/ITMP search area with adaptive sampling,” JVET-AF0161, Hannover, October 2023.

- Y. Kidani, H. Kato, and K. Kawamura, “EE2-3.2: IBC Reference Area Extension,” JVET-AF0115, Hannover, October 2023.

- ECM-11.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-11.0

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, October. 2023.

- V. Seregin, et al., “Exploration experiment on enhanced compression beyond VVC capability (EE2)”, JVET-AF2024, October 2023.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ofinno, LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).