| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0215-v1 |
| --- | --- |

| Title: | AHG9/AHG13: Region-dependent film grain characteristics |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | P. de Lagrange E. François M. Le Pendu C. Salmon-Legagneur | Email: | philippe.delagrange@interdigital.com |
| Source: | InterDigital |  |  |  |

_____________________________

# Abstract

This contribution, while stating that the current FGC SEI message fits its purpose to describe the grain present on scanned film or added for artistic intent on motion pictures, and enables to recover a similar grain by driving synthesis at decoder side at reasonable cost when the grain has been lost in the encoding process for broadcast or streaming purpose, acknowledges that for other uses cases, it may be desirable to describe different film grain in different regions of the picture, for example when the picture is a composite of different sources.

A region-dependent film grain characteristics SEI message is proposed, that includes the existing FGC SEI message syntax, notably independent intensity intervals with model parameters for each interval, and adds elements to describe a list of picture regions, and associate each region with a set of intervals. The commonalities with the FGC SEI message are believed to provide a simple migration path, and simple conversion back to FGC SEI messages for parts of pictures by a network processor, for example in the context of subpictures.

# Introduction

The current FGC SEI message can convey film grain characteristics through parameters of a film grain statistical model. That model is either based on spatial frequency cutoffs or 2D auto-regressive filter coefficients.

Each color component is decomposed into several value intervals, or “intensity” intervals, and model parameters are specified for each interval.

This allows to model grain that varies in both strength and shape from dark to light areas, as illustrated in Figure 1 and Figure 2. Film stock commonly has grain that is finer and stronger in lighter areas. FGC SEI can also be used to model digital camera noise, which tend to have stronger and coarser noise in darker areas but finer and much lower noise in lighter areas, or synthetic grain that can have various properties tuned by an artist.

Figure 1 – Example crops of a 1963 35mm negative film scan (around 7500 ppi)

Figure 2 – Example intervals and parameters

There is nothing wrong about this FGC SEI. It serves well its nominal purpose of describing the grain of streaming movies from scanned film or with artistic grain, to recover it when lost by video compression: low cost implementation of film grain synthesis in TV or STB chips is possible, even in the video display pipeline, and a C software model is available in [1] to guide and support implementation of hardware and firmware blocks. It is further possible to use it to control existing film grain synthesis implementations that may not support variable grain shape or a limited number of different shapes (the green curve of Figure 2), indicating an order of preference.

However, for other use cases, likely software-based like could gaming, there is some interest in describing film grain characteristics that are specific to some image regions. Figure 3 illustrates a conceptual example where the image is composed from different sources with different grain characteristics.

Figure 3 – Video with regions A, B and C (e.g. different sources with different grain)

# Proposal

It is noted that in the FGC SEI, intervals are independent: each interval specifies min/max intensity and model parameters (e.g. gain and bandwidth). They can be redundant, overlapping, out of order, and there can be holes in the intensity range (meaning no grain).

It is then proposed to create a new region-dependent film grain characteristics SEI message, that reuse the same syntax (and semantics) as the FGC SEI message to list all intervals needed for the whole picture, then list picture regions (e.g. rectangular areas) and associate sets of intervals with each region, as illustrated in Figure 4. Several regions can use the same set of intervals (e.g. B and C), and some intervals can be reused in different sets (in Figure 4, a few intervals are used by A, B, and C).

Figure 4 – Associating sets of intervals with regions

If some image areas are not listed in the region list, this means there is no grain there: for example in Figure 3, if there is grain only in A, then only A will be listed in the region list (and use all intervals), meaning that B and C have no grain.

Two methods are proposed to associate regions with sets of intervals:

- The preferred one is using min/max interval indices. This allows to have the new SEI message to begin with exactly the same syntax as the FGC SEI, keeping interval definition unchanged, and defer the set definition at the end, for each region.
This way, truncating the new SEI message would yield a legacy FGC SEI. This would likely favor an easy migration path, and easy fallback to legacy FGC SEI.
This would also enable easy network processing if regions match subpictures: the network processor would just have to extract relevant intervals (or drop irrelevant ones) to make a legacy FGC SEI that fits the extracted subpicture.

- The alternate method uses set ids (or “tags”): each interval tells its set id, and regions list the set ids they use. In Figure 4, we would have 3 ids: for example “0” for the first intervals (A only), “1” for the middle part (A, B, and C), and “2” for the end (B and C). Region A uses ids “0” and “1”, and regions B and C use ids “1” and “2”.
This allows more flexible ordering of intervals, and grouping intervals from several color components under the same id, but breaks the original FGC SEI syntax.

# Proposed SEI message (method 1)

Table 1 – Proposed syntax

| film_grain_regions_characteristics ( payloadSize ) { | Descriptor |
| --- | --- |
| fgr_characteristics_cancel_flag | u(1) |
| if( !fgr_characteristics_cancel_flag ) { |  |
| fg_model_id | u(2) |
| fg_separate_colour_description_present_flag | u(1) |
| if( fg_separate_colour_description_present_flag ) { |  |
| […] |  |
| } |  |
| fg_blending_mode_id | u(2) |
| fg_log2_scale_factor | u(4) |
| for( c = 0; c < 3; c++ ) |  |
| fg_comp_model_present_flag[ c ] | u(1) |
| for( c = 0; c < 3; c++ ) |  |
| if ( fg_comp_model_present_flag[ c ] ) { |  |
| fg_num_intensity_intervals_minus1[ c ] | u(8) |
| fg_num_model_values_minus1[ c ] | u(3) |
| for( i = 0; i <= fg_num_intensity_intervals_minus1[ c ]; i++ ) { |  |
| fg_intensity_interval_lower_bound[ c ][ i ] | u(8) |
| fg_intensity_interval_upper_bound[ c ][ i ] | u(8) |
| for( j = 0; j <= fg_num_model_values_minus1[ c ]; j++ ) |  |
| fg_comp_model_value[ c ][ i ][ j ] | se(v) |
| } |  |
| } |  |
| fgr_characteristics_persistence_flag | u(1) |
| fgr_num_regions_minus1 | u(8) |
| for( i = 0; i <= fgr_num_regions_minus1; i++ ) { |  |
| fgr_region_top[ i ] | u(16) |
| fgr_region_left[ i ] | u(16) |
| fgr_region_width[ i ] | u(16) |
| fgr_region_height[ i ] | u(16) |
| for( c = 0; c < 3; c++ ) |  |
| if ( fg_comp_model_present_flag[ c ] ) { |  |
| fgr_region_interval_min[ i ][ c ] | u(8) |
| fgr_region_interval_max[ i ][ c ] | u(8) |
| } |  |
| } |  |
| } |  |
| } |  |

Semantics for the new (highlighted) parts are the following:

fgr_num_regions_minus1 plus 1 specifies the number of picture regions for which sets of intensity intervals are signalled.

fgr_region_top[ i ], fgr_region_left[ i ], fgr_region_width[ i ], and fgr_region_height[ i ] specify the luma coordinates of the top-left corner and the width and height, respectively, of the i-th picture region.

The value of fgr_region_left[ i ] shall be in the range of 0 to PicWidthInLumaSamples − 1, inclusive.

The value of fgr_region_top[ i ] shall be in the range of 0 to PicHeightInLumaSamples − 1, inclusive.

The value of fgr_region_width[ i ] shall be in the range of 0 to PicWidthInLumaSamples − fgr_region_left[ i ], inclusive.

The value of fgr_region_height[ i ] shall be in the range of 0 to PicHeightInLumaSamples − fgr_region_top[ i ], inclusive.

fgr_region_interval_min[ i ][ c ] and fgr_region_interval_max[ i ][ c ] specify the range of interval indexes for color component c that are applicable to the i-th region. fgr_region_interval_min[ i ][ c ] shall be in the range of 0 to fg_num_intensity_intervals_minus1[ c ], inclusive, and fgr_region_interval_max[ i ][ c ] shall be in the range of fgr_region_interval_min[ i ][ c ] to fg_num_intensity_intervals_minus1[ c ], inclusive.

Other semantics are unchanged from FGC SEI message, except that the set of intervals applicable for picture samples in region i is restricted to those of index fgr_region_interval_min[ i ][ c ] to fgr_region_interval_max[ i ][ c ] inclusive:

- equations (24) and (25) of VSEI v3 are modified to include a additional condition on the interval index, depending on the regions in which x and y are included.

# Proposed SEI message (method 2)

Table 1 – Proposed syntax

| film_grain_regions_characteristics ( payloadSize ) { | Descriptor |
| --- | --- |
| fgr_characteristics_cancel_flag | u(1) |
| if( !fgr_characteristics_cancel_flag ) { |  |
| fg_model_id | u(2) |
| fg_separate_colour_description_present_flag | u(1) |
| if( fg_separate_colour_description_present_flag ) { |  |
| […] |  |
| } |  |
| fg_blending_mode_id | u(2) |
| fg_log2_scale_factor | u(4) |
| for( c = 0; c < 3; c++ ) |  |
| fg_comp_model_present_flag[ c ] | u(1) |
| for( c = 0; c < 3; c++ ) |  |
| if ( fg_comp_model_present_flag[ c ] ) { |  |
| fg_num_intensity_intervals_minus1[ c ] | u(8) |
| fg_num_model_values_minus1[ c ] | u(3) |
| for( i = 0; i <= fg_num_intensity_intervals_minus1[ c ]; i++ ) { |  |
| fgr_intensity_interval_tag_id[ c ][ i ] | u(8) |
| fg_intensity_interval_lower_bound[ c ][ i ] | u(8) |
| fg_intensity_interval_upper_bound[ c ][ i ] | u(8) |
| for( j = 0; j <= fg_num_model_values_minus1[ c ]; j++ ) |  |
| fg_comp_model_value[ c ][ i ][ j ] | se(v) |
| } |  |
| } |  |
| fgr_characteristics_persistence_flag | u(1) |
| fgr_num_regions_minus1 | u(8) |
| for( i = 0; i <= fgr_num_regions_minus1; i++ ) { |  |
| fgr_region_top[ i ] | u(16) |
| fgr_region_left[ i ] | u(16) |
| fgr_region_width[ i ] | u(16) |
| fgr_region_height[ i ] | u(16) |
| fgr_region_num_tag_ids_minus1[ i ] | u(8) |
| for( j = 0; j <= fgr_region_num_tag_ids_minus1[ i ]; j++ ) |  |
| fgr_region_tag_id[ i ][ j ] | u(8) |
| } |  |
| } |  |
| } |  |

Semantics for the new (highlighted) parts are the following:

fgr_intensity_interval_tag_id[ c ][ i ] specifices an identifier value (tag_id) associated with the intensity interval.

Note: several intervals can share the same tag_id, the purpose being to be able to referece sets of intervals by tag_id.

fgr_num_regions_minus1 plus 1 specifies the number of picture regions for which sets of intensity intervals are signalled.

fgr_region_top[ i ], fgr_region_left[ i ], fgr_region_width[ i ], and fgr_region_height[ i ] specify the luma coordinates of the top-left corner and the width and height, respectively, of the i-th picture region.

The value of fgr_region_left[ i ] shall be in the range of 0 to PicWidthInLumaSamples − 1, inclusive.

The value of fgr_region_top[ i ] shall be in the range of 0 to PicHeightInLumaSamples − 1, inclusive.

The value of fgr_region_width[ i ] shall be in the range of 0 to PicWidthInLumaSamples − fgr_region_left[ i ], inclusive.

The value of fgr_region_height[ i ] shall be in the range of 0 to PicHeightInLumaSamples − fgr_region_top[ i ], inclusive.

fgr_region_tag_id[ i ][ j ] lists possible values of fgr_intensity_interval_tag_id associated with the i-th picture region. The film grain characteristics for the i-th region are restricted to intensity intervals that specify a value of fgr_intensity_interval_tag_id[ c ][ i ] (for any c or i) that is equal to fgr_region_tag_id[ i ][ j ], for at least one value of j in the range of 0 to fgr_region_num_tag_ids_minus1[ i ]

Other semantics are unchanged from FGC SEI message, except that the set of intervals applicable for picture samples in region i is restricted to those that specify a tag_id present in the list of tag_id for the region:

- equations (24) and (25) of VSEI v3 are modified to include a additional condition on fgr_intensity_interval_tag_id[ c ][ i ], depending on the regions in which x and y are included.

# Conclusion

It is proposed to add a Region-dependent film grain characteristics SEI message based on this contribution in the next version of the “Technologies under consideration for future extensions of VSEI” document.

# References

- VFGS software repository: https://vcgit.hhi.fraunhofer.de/jvet-ahg-fgt/vfgs

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).