| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0231-v1 |
| --- | --- |

| Title: | EE2-1.7b related: unrestricted 1.7b performances |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Le Léannec, K. Naser, T. Dumas, Y. Chen, M. Radosavljević, T. Poirier | Tel: Email: | fabrice.leleannec@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution brings additional information related to test EE2-1.7b, reported in JVET-AG0137 [1]. In EE2-1.7b, some restrictions are applied, in particular to IntraTMP-LIC mode, to limit runtime.

This contribution reports the results of EE2 tests 1.7b, when these restrictions are deactivated. The unrestricted EE2-1.7b test reportedly has the following impact on ECM-11.0, for {Y, U, V, EncT, DecT, VmPeak, DecVmPeak}.

- AI CTC :          { -0,32% , -0,21% , -0,21% ,      107,5% , 106,7% , 101,2% , 100,3% }

- AI class F: 	     { -0,94% , -0,86% , -0,85% ,      105,9% , 111,0% , 99,8% , 101,5% }

- AI class TGM: { -1,49% , -1,34% , -1,36% ,      103,6% , 106,6% , 99,8% , 105,4% }

# Introduction

Test EE2-1.7b consists in the combination of the following individual test:

- Test 1.2 proposes to extend IntraTMP block vector candidates with IBC merge block vector candidates.

- Test1.3 proposes to extend geometric prediction by combining block-vector-based prediction and regular intra prediction from IBC-GPM to SGPM.

- Test 1.4 proposes to adaptively replace planar mode with block-vector based prediction in DIMD.

- Test 1.5 proposes to allow the application of LIC to blocks coded in IntraTMP mode.

In the delivered test EE2-1.7b, block vectors are searched in a different way for regular IntraTMP and for IntraTMP-LIC. That is, a SAD-based search is performed for non-LIC blocks and a MRSAD-based search is performed for intra TMP CUs coded in LIC mode.

To keep the overall of amount block vector search reasonable, some restrictions are used. They consist in a coarser sampling or the IntraTMP sparse search area, both for regular IntraTMP and Intra-LIC modes. The sampling factor is increased from 3 to 4.

Moreover, BV refinement range is reduced in LIC case and less BV candidates issued from sparse search are  kept in LIC case than in regular IntraTMP case.

This reduced BV search leads to reduced runtime, at the cost of reduced coding gain of test 1.7b over ECM-11.0.

This contribution reports the gain of unrestricted EE2-1.7b, i.e. when none of the above restrictions is applied.

# Experimental results

Unrestricted test EE2-1.7b was run in all-intra Ahg12 coding test conditions [2].

The following table Table 1 show the performance obtained over ECM-11.0. Luma coding gain -0.32% is observed. Moreover, -0.94% and -1.49% luma coding gain is obtained in classes F and TGM.

Table 1: performances of unrestricted test EE2-1.7b over ECM-11.0.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0,11% | -0,04% | -0,12% | 107,9% | 102,3% | 101,2% | 97,0% |
| Class A2 | -0,40% | -0,34% | -0,39% | 107,4% | 107,0% | 101,5% | 98,8% |
| Class B | -0,34% | -0,21% | -0,24% | 107,6% | 108,1% | 100,2% | 98,3% |
| Class C | -0,22% | -0,19% | -0,08% | 107,0% | 106,7% | 100,7% | 104,6% |
| Class E | -0,54% | -0,26% | -0,22% | 107,8% | 108,3% | 103,2% | 102,6% |
| Overall | -0,32% | -0,21% | -0,21% | 107,5% | 106,7% | 101,2% | 100,3% |
| Class D | -0,17% | -0,03% | -0,09% | 105,8% | 105,5% | 99,7% | 107,1% |
| Class F | -0,94% | -0,86% | -0,85% | 105,9% | 111,0% | 99,8% | 101,5% |
| Class TGM | -1,49% | -1,34% | -1,36% | 103,6% | 106,6% | 99,8% | 105,4% |

# Cited References

- JVET-AG0137, “EE2-1.6/1.7a/1.7b: combinations of tests 1.2, 1.3, 1.4 and 1.5”, F. Le Léannec, K. Naser, T. Dumas, Y. Chen, M. Radosavljević, T. Poirier (InterDigital)

- JVET-AF2017 , “Common test conditions and evaluation procedures for enhanced compression tool testing”, M. Karczewicz, Y. Ye, Oct. 2023

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).