| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0233-v1 |
| --- | --- |

| Title: | EE2-related: Adaptive precision for CCALF coefficients |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Marta Karczewicz, Vadim Seregin, Hongtao Wang 5775 Morehouse Dr, San Diego, CA 92121, USA | Tel: Email: | nanh@qti.qualcomm.com martak@qti.qualcomm.com vseregin@qti.qualcomm.com hongtaow@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In adaptive loop filter of ECM-11.0, when filtering a luma sample with an on-line derived filter, 7 bits are used to represent the fractional part of a coefficient. Similarly, when filtering a chroma sample with a cross component adaptive loop filter (CCALF), 7 bits are used to represent the fractional part of a coefficient. Each CCALF coefficient can only be a value in the form of power of 2. In test EE2-4.3, the number of bits used to represent the fractional part of a luma coefficient is adaptive from 5 to 8, inclusively. A two-bit syntax element is signalled for each set of luma filter set to indicate the number of bits. In this contribution, the idea of EE test is applied to CCALF, the number of bits used to represent the fractional part of a CCALF coefficient is adaptive from 7 to 10, inclusively. In addition, the power of 2 constraint is removed. The proposed method was implemented on top of ECM-11.0, with common test condition, simulation results are reported as follows.

|  | BD-rate Y | BD-rate U | BD-rate V | EncT | DecT | VmPeakE | VmPeakD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| All intra |  |  |  |  |  |  |  |
| Random access |  |  |  |  |  |  |  |
| Low delay B |  |  |  |  |  |  |  |

# Introduction

In ECM-11.0, a luma coefficient of adaptive loop filter (ALF) is represented by a fixed-point 8-bit decimal number. The fractional part is represented by a 7-bit number and 1 bit is used for the sign. The real value range of a coefficient is within [-1, 1). Similarly, a CCALF coefficient is represented by a fixed-point 8-bit decimal number. The fractional part is represented by a 7-bit number and 1 bit is used for the sign. A CCALF coefficient must be a value in the form of power of 2. In test EE2-4.3, the number of bits used to represent the fractional part of a luma coefficient can vary from 5 to 8 adaptively. For each luma filter set, which contains up to 25 filters, a 2-bit syntax element is signalled to indicate the number of bits used for the coefficients in this set. The real value range of a coefficient is not changed.

# Proposed method

In this contribution, the idea of EE2-4.3 is extended to CCALF.The number of bits used to represent the fractional part of a CCALF coefficient can vary from 7 to 10 adaptively. In an ALF adaptation parameter set (APS), a 2-bit syntax element is signalled for each chroma component to indicate the number of bits used for the CCALF coefficients for this component. In addition, the power of 2 constraint is removed.

# Simulation results

The proposed method was implemented on top of ECM-11.0 and tested following common test condition described in JVET-AF2017.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.01% | -0.22% | -0.37% |  |  | 100.0% | 99.8% |
| Class C | 0.00% | -0.34% | -0.34% |  |  | 100.0% | 99.9% |
| Class E | -0.01% | -0.33% | -0.61% |  |  | 100.0% | 100.0% |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.03% | -1.04% | -0.94% |  |  | 99.8% | 99.9% |
| Class F | 0.01% | -0.52% | -0.54% |  |  | 99.7% | 99.9% |
| Class TGM |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | 0.02% | -0.47% | -0.46% |  |  | 98.7% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.03% | -0.70% | -1.16% |  |  | 100.1% | 100.2% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).