| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0234-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AG0098 (EE2-2.7: AMVP with SbTMVP mode) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lei Zhao Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | zhaolei.0324@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution reports crosscheck results of EE2-2.7 in JVET-AG0098.

- Introduction

In EE2-2.7, given a CU coded in AMVP with SbTMVP mode, the CU is predicted in a similar way as that of SbTMVP in merge mode except that the motion shift is signaled in the bitstream instead of being derived from neighboring blocks. The motion shift is obtained using MVP with a signaled MVD, and the number of MVDs is determined according to the percentage of the area of the blocks coded in the the AMVP with SbTMVP mode in the previous coded picture..

- Cross-check results

The crosscheck results on ECM-11.0 are illustrated as follows, It is reported that partial crosscheck results are matched with the simulation results from the proponent.

Table 1: Coding performance of JVET-AD0180.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | -0.02% | -0.03% | 100.1% | 100.4% |
| Class C | -0.16% | -0.06% | -0.16% | 100.2% | 99.6% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.08% | -0.03% | -0.05% | 100.9% | 99.9% |
| Class F | -0.09% | -0.11% | -0.08% | 100.5% | 100.3% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.11% | 0.05% | -0.36% | 101.5% | 99.2% |
| Class C | -1.04% | -0.67% | -0.18% | 99.6% | 98.5% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.75% | -0.69% | -1.09% | 102.2% | 92.8% |
| Class F |  |  |  |  |  |

- Patent rights declaration(s)

Bytedance Inc. may not have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).