| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0235-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AG0122 (EE1-3.1: Deep Reference Frame Generation for Inter Prediction Enhancement) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Zhihuang Xie | Tel: Email: | xiezhihuang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution reports the cross-check results of JVET-AG0122, which is also known as EE1-3.1. In EE1-3.1, a low-complexity network architecture is studied for inter prediction to get a better trade-off between complexity and performance. Both training and inference are conducted in this crosscheck.

From the observation of the completed results, the performance difference between proponent’s model and crosschecker’s model is 0.05%(Y) for random access. Since this gap is minor, it is considered that EE1-3.1 passes training crosscheck.

Introduction

Fig. 1: The framework of the proposed DRF method

The framework of the proposed DRF method is depicted in Figure 1. In this approach, two reconstructed frames chosen from the DPB are inputted into the deep reference frame generation network to synthesize a new frame. This synthesized frame is then added to both reference picture list0 and list1, using the same POC as the current encoding frame. It serves as an additional reference frame for subsequent encoding processes.

Training process

Training crosscheck was conducted during this meeting cycle. All training scripts were provided by proponents of EE1-3.1. The training dataset is generated from the license-free dataset HAA500 instead of Vimeo triplets dataset during this EE. Training and valid datasets were converted to YUV files then compressed by VTM anchor.

The whole training process took almost 2 days to finish 100 epochs on dual A100. Finally, the best model on validation was selected to conduct inference test with NNVC-7.1.

Inference process

The proposed method was implemented on top of NNVC-7.1, and well-trained model was converted to floating operation sadl.

Simulation results

The simulation was conducted under common test condition, and both NNLF LOP.2 and NnIntrePred were enabled during testing.

Table1. results of crosschecker’s model vs. test’s model

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over JVET-AG0122 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.09% | -0.85% | -0.23% | #DIV/0! | #DIV/0! |
| Class A2 | 0.04% | -0.16% | -0.11% | #DIV/0! | #DIV/0! |
| Class B | 0.05% | -0.12% | -0.09% | #DIV/0! | #DIV/0! |
| Class C | 0.04% | 0.00% | -0.10% | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |
| Overall | 0.05% | -0.24% | -0.12% | #DIV/0! | #DIV/0! |
| Class D | 0.05% | 0.15% | -0.05% | #DIV/0! | #DIV/0! |

*Please note that running time is not reliable.

Additional crosscheck is provided for your information. In this training crosscheck, Vimeo triplets were used as training and valid dataset, and network was kept unchanged.

Table2. results of crosschecker’s model vs. test’s model (Vimeo dataset)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over JVET-AG0122-Vimeo |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.03% | 0.20% | #DIV/0! | #DIV/0! |
| Class C | -0.04% | 0.12% | 0.12% | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.04% | 0.16% | 0.14% | #DIV/0! | #DIV/0! |

Conclusion

From the completed results, the performance gap of models between proponent’s and crosschecker’s is minor, so it is considered that EE1-3.1 passes training crosscheck.

During this EE, the proponents were looking for new training dataset and simiplied the model. From the observation, it should be noted that the simplification of model and new training dataset lead to the drop of performance.