| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0241-v2 |
| --- | --- |

| Title: | Crosscheck of JVET-AG0162 (EE1-1.5: Report on implementation of HOP In-loop filter with Transformer blocks) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Du Liu | Tel: Email: | du.liu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution reports the inference crosscheck results for JVET-AG0162, which uses Transformer blocks in the HOP network. It is reported that the crosscheck results can match exactly the results in JVET-AG0162.

# Introduction and Results

In EE1-1.5 JVET-AG0162, the transformer implementation using SADL is studied. The proposed modification is tested on top of NNVC-7.1 HOP. This report crosschecked the inference of the int16 network. NNIntra is enabled.

Results are on top of NNVC-VTM.

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-VTM |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -15.13% | -13.65% | -22.03% | 417% | 164918% |
| Class A2 | -14.51% | -18.92% | -24.19% | 398% | 158630% |
| Class B | -12.41% | -24.02% | -16.02% | 442% | 167031% |
| Class C | -13.35% | -17.15% | -17.54% | 332% | 153908% |
| Class E |  |  |  |  |  |
| Overall | -13.62% | -19.09% | -19.26% | 396% | 161336% |
| Class D | -14.20% | -17.00% | -18.08% | 328% | 168078% |
| Class F | -8.45% | -12.22% | -11.79% | 615% | 79287% |

|  | Low-delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-VTM |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -9.83% | -11.74% | -5.37% | 385% | 156958% |
| Class C | -11.48% | -9.82% | -6.92% | 319% | 143971% |
| Class E | -11.19% | -9.42% | -1.12% | 874% | 134830% |
| Overall | -10.72% | -10.52% | -4.83% | 444% | 146819% |
| Class D | -12.64% | -10.21% | -3.84% | 292% | 137452% |
| Class F | -8.54% | -10.81% | -9.21% | 620% | 79693% |
|  | All intra |  |  |  |  |
|  | BD-rate Over NNVC-VTM |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -13.44% | -11.48% | -18.44% | 511% | 134993% |
| Class A2 | -12.94% | -16.83% | -19.89% | 363% | 110194% |
| Class B | -11.68% | -17.23% | -13.71% | 331% | 106730% |
| Class C | -12.60% | -14.40% | -16.89% | 246% | 67160% |
| Class E | -16.14% | -20.18% | -18.78% | 346% | 110606% |
| Overall | -13.13% | -16.07% | -17.08% | 341% | 101270% |
| Class D | -11.88% | -13.19% | -16.66% | 231% | 75264% |
| Class F | -9.38% | -13.28% | -12.18% | 217% | 83122% |

Encoding time and decoding time are not reliable since these were not run on the same machine.

# References

[1] EE1-1.5: Report on implementation of HOP In-loop filter with Transformer blocks, JVET-AG0162, Y. Li, D. Rusanovskyy, M. Karczewicz, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 33rd Meeting, by teleconference, 17–26 January 2024.

# Patent rights declaration(s)

Ericsson does not have any current or pending patent rights relating to the technology described in this contribution.