| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0243-r1 |
| --- | --- |

| Title: | AHG12: Additional Metric for IntraTMP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Naser F. Le Léannec H. Guermoud T. Poirier T. Dumas | Email: | karam.naser@InterDigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

In IntraTMP mode, the best N block vectors are found using SAD metric. Specifically, the candidate block vectors are evaluated in template matching process using SAD cost. This contribution proposes further including additional metric, to find a new set of block vectors. A flag is signaled to the decoder to indicate the metric choice. On top of ECM-11.0, the following results are obtained:

SATD as additional metric
AI: -0.10%	-0.04%	-0.06%	102.7%	102.7%

MR-SAD as additional metric
AI: -0.09%	-0.04%	-0.11%	101.7%	100.8%

# Introduction

In ECM, the prediction signal in IntraTMP mode is generated by matching the L-shaped, Top-only or Left-Only causal neighbor of the current block with another block in a predefined search area in Figure 1. The template matching cost is considered as the Sum of absolute differences (SAD).

Figure 1 Intra template matching search area used.

The encoder/decoder constructs a candidate list of up to “19” template matching block vectors that are ranked in ascending order according to the template cost (SAD). Multiple InraTMP modes are supported:

- Single predictor: a single predictor is selected from the candidate list.

- Fusion of multiple predictors: multiple predictors are blended to derive the final prediction block. The blending weights are either computed from the template matching cost of each predictor, or with Wiener-filter based weight derivation method.

- Sub-pel precision: when single predictor is used, sub-pel precision can be used with 1/2-pel precision, 1/4-pel precision and 3/4-pel precision, each with 8 possible directions.

- Linear filter model: a linear filter can be learned between the reference template and current template and be applied to reference block. This mode can be used for single predictor when sub-pel precision is not used.

# Proposal

It is noted that the whole process of template matching depends on the template cost metric. The current design of IntraTMP is optimized for finding block vectors that are closest to the current template in terms of pixel differences. However, minimizing this difference does not necessarily reduce the amount of bits required to code the current block. Therefore, this proposal introduces additional metric for template cost that yields different set of block vectors, where a flag is signaled to indicate which metric is used. Specifically,

- Test 1: SATD is considered.

- Test 2: MRSAD is considered.

# Results

The method is implemented on top of ECM-11.0. The following results are obtained for AI and RA configurations using the common testing conditions:

- Test1: using SATD as additional metric

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.06% | -0.03% | -0.08% | 103.9% | 101.5% | #DIV/0! |
| Class A2 | -0.11% | -0.09% | -0.10% | 100.8% | 96.9% | #DIV/0! |
| Class B | -0.10% | -0.08% | -0.11% | 101.9% | 100.5% | #DIV/0! |
| Class C | -0.07% | -0.04% | 0.03% | 104.0% | 107.8% | #DIV/0! |
| Class E | -0.18% | 0.09% | -0.02% | 103.2% | 107.0% | #DIV/0! |
| Overall | -0.10% | -0.04% | -0.06% | 102.7% | 102.7% | #DIV/0! |
| Class D | -0.07% | -0.12% | 0.08% | 102.6% | 106.7% | #DIV/0! |
| Class F | -0.10% | -0.25% | 0.02% | 102.2% | 103.6% | #DIV/0! |
| Class TGM | -0.23% | -0.16% | -0.27% | 101.2% | 97.3% | #DIV/0! |

- Test2: using MR-SAD as additional metric

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.06% | -0.01% | 0.02% | 102.9% | 99.6% | #DIV/0! |
| Class A2 | -0.09% | -0.07% | -0.11% | 99.8% | 95.0% | #DIV/0! |
| Class B | -0.09% | -0.05% | -0.17% | 100.9% | 98.6% | #DIV/0! |
| Class C | -0.06% | -0.11% | -0.12% | 103.0% | 105.8% | #DIV/0! |
| Class E | -0.14% | 0.09% | -0.12% | 102.3% | 105.0% | #DIV/0! |
| Overall | -0.09% | -0.04% | -0.11% | 101.7% | 100.8% | #DIV/0! |
| Class D | -0.06% | -0.07% | 0.11% | 101.7% | 104.7% | #DIV/0! |
| Class F | -0.15% | -0.13% | -0.01% | 101.3% | 101.7% | #DIV/0! |
| Class TGM | -0.23% | -0.22% | -0.27% | 100.3% | 95.5% | #DIV/0! |

# Conclusion

This contribution proposes adding SATD metric, along with the current SAD metric, for template matching cost in IntraTMP. It is proposed to study this method in the next round of EE.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).