| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0289-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AG0065 (Non-EE2: Adaptive coefficient precision for CCALF) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Nan Hu 5775 Morehouse Dr, San Diego, CA 92121, USA | Tel: Email: | nanh@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution is a crosscheck report of JVET-AG0065 (Non-EE2: Adaptive coefficient precision for CCALF). The simulations were carried out based on the common test condition. The partial results are matched in BD-rate. Timing is not reliable in the crosscheck report.

# Introduction

In ECM-11.0, a coefficient of cross component adaptive loop filter (CCALF) is represented by a fixed-point 8-bit decimal number. The fractional part is represented by a 7-bit number and 1 bit is used for the sign. In JVET-AG0065, adaptive precision is proposed to apply to CCALF coefficients. In an ALF APS, a syntax element is signalled for each color component to indicate which precision number is used. The coefficient precision candidates are 7, 8, 9 and 10.

# Simulation results

The proposed method was implemented on top of ECM-11.0 and tested following common test condition described in JVET-AF2017.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | 0.01% | -0.17% | -0.25% | 96.2% | 96.9% | 99.4% | 98.9% |
| Class E | 0.01% | -0.30% | -0.32% | 94.9% | 94.7% | 99.5% | 99.0% |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.02% | -0.26% | -0.47% | 94.8% | 94.0% | 99.2% | 98.8% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | 0.01% | -0.25% | -0.35% | 97.7% | 96.8% | 99.6% | 99.0% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.02% | -0.43% | -0.81% | 97.7% | 94.5% | 99.3% | 98.9% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.01% | -0.34% | -0.96% | 95.1% | 95.9% | 99.6% | 98.9% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |