| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0302-v1 |
| --- | --- |

| Title: | AHG15: Description of gaming content sequences proposed by Huawei |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zehui Lin Weilan Chen Yuxin Zhao Kangying Cai Johannes Sauer Elena Alshina | Tel: Email: | caikangying@huawei.com |
| Source: | Huawei Technologies |  |  |

_____________________________

# Abstract

This contribution proposes 5 new sequences to be used for the study of gaming content compressiong by AHG15. A short description for each sequence is provided in this document. A copyright notice is part of this contribution file and also attached to the file source.

# Capture of the sequences

3 sequences were created using the Unreal game engine to load and play three games. We captured the frame buffers into R10G10B10A2 Unsigned Int images by custom post-processing codes and shaders in the default render pipeline. The frame buffers are converted to YUV sequences.

## Auxiliary information

From the game engine a variety of additional information is available in addition to the scene color image. Among other this may include:

- Depth

- Motion Vector

- Camera Information

- Albedo

- Specular

- Normal

- Roughness

Examples are shown below. Auxiliary data is not provided at this point. It should be clarified first, which auxiliary information is of interest to the group. Further, formats to be used for the auxiliary information should be defined.

We propose to obtain and process the auxiliary information as described below.

- Depth

Encoded into 32-bits float images.

Instead of using the linear depth map under the world coordination directly, we encode it to the normalized value before use it.

Let WorldZ be the depth value of a pixel in the world space, and DepthAdd and DepthMultip be the normalization factors to calculate the value. Then the normalized depth value NormalizedZ can be described as:

NormalizedZ= 1.0WorldZ+DepthAdd*DepthMultip

The purpose of DepthAdd and DepthMultip here is to control the precision of normalized depth value: we would like to have objects closer to the viewer (with smaller WorldZ) with more precise normalized depth value. The value of both DepthAdd and DepthMultip should be given as part of auxiliary data.

- Motion Vector

Encoded into a two-channel 32-bits float images. Each pair of values of the motion vector refers to the corresponding position, in the previous frame, of a pixel in the current frame.

- Camera Information

This includes all the transform matrices involved in converting positions from the world space to the camera space: World to View space transform matrix and View to Camera space transform matrix. Both matrices should consist of 4x4 32-bits floating point values.

Examples:

| “ARPG” Rendered view |  |
| --- | --- |
| “ARPG” Depth |  |
| “SunTemple2” Rendered view |  |
| “SunTemple2” Albedo |  |

# Proposed sequences

## Sequence metadata

| Sequence name | Resolution | Input bit-depth | Frame rate | frame count | Md5 sum |
| --- | --- | --- | --- | --- | --- |
| DesertTown1 | 1920x1080 | 8 | 60 | 600 | 9369d1aeae138aecfb15e7ab15defe32 |
| DesertTown2 | 1920x1080 | 8 | 60 | 600 | 6fbb5f9ef998cb0618f2c9d5ded83a48 |
| SunTemple1 | 1920x1080 | 8 | 60 | 600 | be30f44c1a2a8d03dab1c423af2ae327 |
| SunTemple2 | 1920x1080 | 8 | 60 | 600 | a417325a4e768f2eceeac54091d78f7b |
| ARPG | 1920x1080 | 8 | 60 | 600 | d8fe44ed8e2c1417261b0b2e075a66ec |

## Thumbnails

| Sequence name |  |
| --- | --- |
| DesertTown1 |  |
| DesertTown2 |  |
| SunTemple1 |  |
| SunTemple2 |  |
| ARPG |  |

# Copyright

This video sequences and auxiliary data (collectively "Data") may only be used for the purpose of developing, testing and promulgating technology standards developed by ISO/IEC JTC 1/SC 29 WG 5 or ITU-T SG 16 VCEG standardization groups (Purpose), under Huawei Technologies owned or controlled copyrights, by individual(s) or organization (s) that participates, contributes or is part of such standardization group (User(s)).

•	The DesertTown1 and DesertTown2 sequences and auxiliary data are rendered using the "Western Desert Town" Unreal Engine assets created by KK Design available at https://www.unrealengine.com/marketplace/en-US/product/western-desert-town .

•	The SunTemple1 and SunTemple2 sequences and auxiliary data are rendered using the “Sun Temple” Unreal Engine assets created by Epic Games available at https://www.unrealengine.com/marketplace/en-US/product/sun-temple .

•	The ARPG sequence and auxiliary data are rendered using the “Action RPG” Unreal Engine assets created by Epic Games available at https://www.unrealengine.com/marketplace/en-US/product/action-rpg-01 .

All these assets are licensed by their respective owners and it is the User’s responsibility to comply with the license terms that are provided by such owner. Huawei Technologies claims no ownership nor liability on these assets.

RESTRICTIONS: No license whatsoever is implied except from this expressed limited personal, non-sublicensable, non-transferrable authorization. In particular no right under any patent of Huawei Technologies or its affiliates is granted.

Any other use is strictly prohibited. This Data cannot be distributed or otherwise disposed of or made available (via internet or otherwise), broadcasted, disclosed to third parties (except to other User(s) who agreed with the present terms and conditions), used for providing any services to third parties or for any commercial use, nor copied (except as technically necessary for the Purpose), modified, adapted, translated, exchanged, publicly performed (even for demonstration or educational purposes), integrated, without limitation. This Data and all intellectual property rights, titles and interests therein, are and remain the exclusive property of Huawei Technologies or its affiliates.

NO WARRANTY: This Data is supplied "as is" and without any warranty of any kind, expressed or implied, including but not limited to any warranty for accuracy or performance, fitness for a general or particular purposes, or any warranty arising by statute or by law, or non-infringement of any third parties rights. In no event will Huawei Technologies be liable for damages of any kind, including without limitation, any special, indirect, incidental, or consequential damages even if Huawei Technologies has been advised of the possibility of such damages.

Huawei Technologies may terminate this license at any time for any reason upon notice to User(s). In such situation, the relevant User(s) shall inform any Secondary User of such termination and make sure that such Secondary User shall cease to use the video.

This authorization is governed by the laws of China without regard to conflict of laws. It shall be exclusively submitted to the competent people's court in Longgang District, Shenzhen City in the event of a dispute not settled amicably.

BY USING THIS DATA USER(S) AGREES AND ACCEPTS THE ABOVE MENTIONED TERMS AND CONDITIONS. IF USER(S) DOES NOT AGREE TO THESE TERMS AND CONDITIONS, IT SHALL NOT ACCESS OR OTHERWISE USE THIS DATA. DO NOT REMOVE OR MODIFY HUAWEI TECHNOLOGIES OR ITS AFFILIATES NAMES, SIGNS, LOGOS, NOTICES OF/IN THE DATA.

# Patent rights declaration(s)

Huawei does not have any current or pending patent rights relating to the technology described in this contribution.