| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0322-v1 |
| --- | --- |

| Title: | AHG9: On Picture Modality Information for SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jingying Gao Han Boon Teo Chong Soon Lim Kiyofumi Abe Virginie Drugeon | Tel: Email: | +65-6550-5406 jingying.gao@sg.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

This contribution is a follow-up to address the revisit after the discussion on JVET-AG0077 and JVET-AG0079. It proposes the following items on the picture modality type which is currently in the technologies under consideration for future extensions of VSEI (version 2). It’s proposed to add picture modality information to VSEI, HEVC, and AVC.

- Move picture modality information from VUI parameters to a new SEI message.

- Signal minimum and maximum wavelength which indicates the spectral band of optical radiation represented by the pictures in the CLVS, by adding the following syntax elements:

- Add mi_min_wavelength_mantissa and mi_max_wavelength_mantissa to specify the mantissa part of the min and max wavelength indicating the spectral band of optical radiation represented by the pictures in the CLVS.

- Add mi_min_wavelength_exponent_plus15 and mi_max_wavelength_exponent_plus15 to specify the exponent part of the max and min wavelength indicating the spectral band of optical radiation represented by the pictures in the CLVS.

- The unit of wavelength is specified in meters.

- Add mi_modality_type_extension_bits and mi_reserved_modality_type_extension for future extension of picture modality information.

- Change the type of picture modalities from thermal infrared picture to ultraviolet picture, when mi_modality_type is equal to 3.

- Add mi_spectrum_range_present_flag to enable signaling of the min and max wavelength directly. When mantissa is equal to 0 or is not present, it denotes that the wavelength is unspecified.

- Add a constraint on VUI colour description syntax elements:

-When mi_modality_type is equal to 2 or 3, decoders shall ignore vui_colour_primaries, vui_transfer_characteristics, vui_matrix_coeffs, and vui_full_range_flag indicated in the VUI parameters

# 1	Introduction

Today's advanced image sensors, which operate across a variety of spectral ranges, are being integrated into cameras. These advanced image sensors are widely used in applications including autonomous vehicles, UAVs, industrial automation, and more. As illustrated in Figure 1, the picture can be captured from various ranges of spectrum, such as ultraviolet light, visible light, and infrared light. The source of light defines the picture modality type, and pictures of different modality types serve different purposes.

Figure 1: An example of the electromagnetic spectrum.

In the 32nd JVET meeting, signaling the picture modality type in VUI parameters proposed in JVET-AF0147 [1] was adopted into the technologies under consideration for future extensions of VSEI (version 2), with the document number of JVET-AF2032 [2]. It was commented that infrared imaging has many applications, and signaling the source properties at a high level in VVC would be interesting. During the discussion of JVET-AG0077 [3] and JVET-AG0079 [4] in JVET, it was suggested to signal the picture modality information in a new SEI message, to keep the syntax design consistent across VSEI, HEVC, and AVC, with considering backward compatibility as well. Therefore, this contribution proposed to add picture modality information to VSEI, HEVC, and AVC.

We propose to change the type of picture modalities from thermal infrared picture to ultraviolet picture. The thermal infrared picture is captured based on the thermal radiation emitted by objects, which is indeed a type of infrared light. Ultraviolet, along with visible and infrared radiation, falls under optical radiation. This spectrum is widely utilized in imaging to record light that objects emit or reflect.

It was also suggested to signal the wavelength indicating the spectral band of optical radiation during the meeting. Table 1 illustrates the spectral bands for optics and photonics specified in ISO 20473:2007, which cover the spectral range from ultraviolet light to infrared light. In this proposal, the method of signaling wavelength indicating the spectral band of optical radiation represented is addressed, wherein the decimal floating points of wavelength are represented using mantissa and exponent.

For example, the minimum wavelength in meters is derived as follows:

MinWavelength = mi_min_wavelength_mantissa * 10 mi_min_wavelength_exponent_plus15-15

mi_min_wavelength_mantissa specifies the mantissa part of the minimum wavelength. Since the wavelength has up to 3 significant digits, 10 bits are allocated to represent the mantissa part of the minimum wavelength.

mi_min_wavelength_exponent_plus15 minus 15 specifies the exponent part of the minimum wavelength. 5 bits are allocated to represent the exponent part of the minimum wavelength. The units of wavelength are specified in meters in the following semantics. The actual exponent may be negative, hence, a bias of 15 is used for the exponent part. The actual exponent ranges from -15 to 16, which is sufficient to represent the entire spectrum range. The bias is calculated as 2(k-1)-1, wherein k represents the length in bits of the exponent.

Table 1: The spectral bands for optics and photonics specified in ISO 20473:2007.

| Designation of the radiation |  | Wavelength (nm) |
| --- | --- | --- |
| Ultraviolet radiation | Extreme UV | 1 to 100 |
|  | Vacuum UV | 100 to 190 |
|  | Deep UV | 190 to 280 |
|  | Mid UV | 280 to 315 |
|  | Near UV | 315 to 380 |
| Visible radiation, light |  | 380 to 780 |
| Infrared radiation | Near IR | 780 to 3 000 |
|  | Mid IR | 3 000 to 50 000 |
|  | Far IR | 50 000 to 106 |

We also suggest having extension bits to the picture modality information, to allow additional signaling for the picture modality information in the future, by adding the following syntax elements:

- mi_modality_type_extension_bits equal to 0 specifies that mi_reserved_modality_type_extension is not present. mi_modality_type_extension_bits greater than 0 specifies the length, in bits, of mi_reserved_modality_type_extension.

- mi_reserved_modality_type_extension shall not be present in bitstreams conforming to this version of this document. However, decoders conforming to this version of this document shall allow the presence of mi_reserved_modality_type_extension in the syntax, but ignore the value. When present, the length, in bits, of mi_reserved_modality_type_extension is equal to mi_modality_type_extension_bits.

# 2	Proposed text for VSEI

## 8.1 General

Table 4 – Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| … | … |
| Shutter interval information | The CVS containing the SEI message |
| Neural-network post-filter characteristics | The CVS containing the SEI message |
| Neural-network post-filter activation | Specified by the syntax of the SEI message |
| Phase indication | Specified by the semantics of the SEI message |
| Modality Information | Specified by the syntax of the SEI message |

### 8.x.1 Modality information SEI message syntax

| modality_info( payloadSize ) { | Descriptor |
| --- | --- |
| mi_modality_info_cancel_flag | u(1) |
| if(!mi_modality_info_cancel_flag) { |  |
| mi_modality_info_persistence_flag | u(1) |
| mi_modality_type | u(8) |
| mi_spectrum_range_present_flag | u(1) |
| if( mi_spectrum_range_present_flag ) { |  |
| mi_min_wavelength_mantissa | u(11) |
| mi_min_wavelength_exponent_plus15 | u(5) |
| mi_max_wavelength_mantissa | u(11) |
| mi_max_wavelength_exponent_plus15 | u(5) |
| } |  |
| mi_modality_type_extension_bits | ue(v) |
| if(mi_modality_type_extension_bits > 0 ) |  |
| mi_reserved_modality_type_extension | u(v) |
| } |  |
| } |  |

### 8.x.2 Modality information SEI message semantics

mi_modality_info_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous modality information SEI message in output order. mi_modality_info_cancel_flag equal to 0 indicates that modality information follows.

mi_modality_info_persistence_flag specifies the persistence of the modality information SEI message for the current layer.

mi_modality_info_persistence_flag equal to 0 specifies that the modality information SEI message applies to the current decoded picture only.

mi_modality_info_persistence_flag equal to 1 specifies that the modality information SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

– A new CLVS of the current layer begins.

– The bitstream ends.

– A picture in the current layer in an AU associated with a modality information SEI message is output that follows the current picture in output order.

mi_modality_type indicates the type of modality of the decoded picture as specified in Table 1. When not present, the value of mi_modality_type is inferred to be equal to 0, denoting that the modality type of the picture is unknown or unspecified or determined by other means not specified in this Specification. When mi_modality_type is equal to 2 or 3, decoders shall ignore vui_colour_primaries, vui_transfer_characteristics, vui_matrix_coeffs, and vui_full_range_flag indicated in the VUI parameters

Table 1– Mapping of mi_modality_type to the type of picture modalities

| mi_modality_type | type of picture modality |
| --- | --- |
| 0 | Unspecified |
| 1 | Visible Picture |
| 2 | Infrared Picture |
| 3 | Ultraviolet Picture |
| 4..255 | Reserved for future use |

mi_spectrum_range_present_flag equal to 1 specifies that the spectrum band of the optical radiation wavelength represented by the pictures in the CLVS is present in the modality information SEI message. mi_spectrum_range_present_flag equal to 0 specifies that the spectrum band of the optical radiation wavelength represented by the pictures in the CLVS is not present in the modality information SEI message.

mi_min_wavelength_mantissa specifies the mantissa part of the minimum wavelength indicating the spectral band of optical radiation represented by the pictures in the CLVS. When mi_min_wavelength_mantissa is equal to 0 or is not present, the minimum wavelength indicating the spectral band of optical radiation represented by the pictures in the CLVS is unknown or unspecified or determined by other means not specified in this Specification

mi_min_wavelength_exponent_plus15 minus 15 specifies the exponent part of the minimum wavelength indicating the spectral band of optical radiation represented by the pictures in the CLVS. When mi_min_wavelength_mantissa is not present or is equal to 0, decoders shall ignore the value of mi_min_wavelength_exponent_plus15.

The value of the minimum wavelength indicating the spectral band of optical radiation represented by the pictures in the CLVS is derived as follows:

MinWavelength = mi_min_wavelength_mantissa * 10 mi_min_wavelength_exponent_plus15-15

mi_max_wavelength_mantissa specifies the mantissa part of the maximum wavelength indicating the spectral band of optical radiation represented by the pictures in the CLVS. When mi_max_wavelength_mantissa is equal to 0 or is not present, the maximum wavelength indicating the spectral band of optical radiation represented by the pictures in the CLVS is unknown or unspecified or determined by other means not specified in this Specification.

mi_max_wavelength_exponent_plus15 minus 15 specifies the exponent part of the maximum wavelength indicating the spectral band of optical radiation represented by the pictures in the CLVS. When mi_max_wavelength_mantissa is not present or is equal to 0, decoders shall ignore the value of mi_max_wavelength_exponent_plus15.

The value of the maximum wavelength indicating the spectral band of optical radiation represented by the pictures in the CLVS is derived as follows:

MaxWavelength = mi_max_wavelength_mantissa * 10 mi_max_wavelength_exponent_plus15-15

MinWavelength and MaxWavelength are in units of meters as specified in ISO/IEC 80000.

mi_modality_type_extension_bits equal to 0 specifies that mi_reserved_modality_type_extension is not present. mi_modality_type_extension_bits greater than 0 specifies the length, in bits, of mi_reserved_modality_type_extension.

The value of mi_modality_type_extension_bits shall be in the range of 0 to 2 048, inclusive. Values in the range of 1 to 2 048, inclusive, for mi_modality_type_extension_bits are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this document. Decoders conforming to this version of this document shall allow any value of mi_modality_type_extension_bits in the range of 0 to 2 048, inclusive, to appear in the syntax.

mi_reserved_modality_type_extension shall not be present in bitstreams conforming to this version of this document. However, decoders conforming to this version of this document shall allow the presence of mi_reserved_modality_type_extension in the syntax, but ignore the value. When present, the length, in bits, of mi_reserved_modality_type_extension is equal to mi_modality_type_extension_bits.

# Proposed text for HEVC

HEVC will be in the FDAM stage in April 2024, but VSEI v4 may start from April 2024, hence, it is impossible to refer the SEI of VSEI from HEVC currently. Therefore, we proposed to include the semantics and syntax of the modality information SEI message directly in HEVC.

## D.2 SEI payload syntax

### D.2.1 General SEI message syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| … |  |
| else if( payloadType = = 210 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| nn_post_filter_characteristics( payloadSize ) |  |
| else if( payloadType = = 211 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| nn_post_filter_activation( payloadSize ) |  |
| else if( payloadType = = 212 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| phase_indication( payloadSize ) |  |
| else if( payloadType = = 213 ) |  |
| modality_info( payloadSize ) |  |
| … |  |
| } |  |

### D.2.xx Modality information SEI message syntax

| modality_info( payloadSize ) { | Descriptor |
| --- | --- |
| modality_info_cancel_flag | u(1) |
| if(!modality_info_cancel_flag) { |  |
| modality_info_persistence_flag | u(1) |
| modality_type | u(8) |
| spectrum_range_present_flag | u(1) |
| if( spectrum_range_present_flag ) { |  |
| min_wavelength_mantissa | u(11) |
| min_wavelength_exponent_plus15 | u(5) |
| max_wavelength_mantissa | u(11) |
| max_wavelength_exponent_plus15 | u(5) |
| } |  |
| modality_type_extension_bits | ue(v) |
| if(modality_type_extension_bits > 0 ) |  |
| reserved_modality_type_extension | u(v) |
| } |  |
| } |  |

## D.3 SEI payload semantics

### D3.1 General SEI payload semantics

The list SingleLayerSeiList is set to consist of the payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 134 to 152, inclusive, 154 to 159, inclusive, 200 to 202, inclusive, 205, and 210 to 213, inclusive.

The list VclAssociatedSeiList is set to consist of the payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 131, 132, 134 to 152, inclusive, 154 to 159, inclusive, 200 to 202, inclusive, 205, and 210 to 213, inclusive.

The list PicUnitRepConSeiList is set to consist of the payloadType values 0, 1, 2, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 133, 135 to 152, inclusive, 154 to 159, inclusive, 200 to 202, and 210 to 213, inclusive.

| Table D.1 – Persistence scope of SEI messages (informative) |  |
| --- | --- |
| SEI message | Persistence scope |
| User data registered by Rec. ITU-T T.35 | Unspecified |
| … | … |
| Modality information | Specified by the syntax of the SEI message |

### D.3.xx Modality information SEI message semantics

modality_info_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous modality information SEI message in output order. modality_info_cancel_flag equal to 0 indicates that modality information follows.

modality_info_persistence_flag specifies the persistence of the modality information SEI message for the current layer.

modality_info_persistence_flag equal to 0 specifies that the modality information SEI message applies to the current decoded frame only.

Let picA be the current picture. modality_info_persistence_flag equal to 1 specifies that the modality information SEI message persists for the current layer in output order until any of the following conditions are true:

– A new CLVS of the current layer begins.

– The bitstream ends.

–A picture picB in the current layer in an access unit containing a modality information SEI message that is applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

modality_type indicates the type of modality of the decoded picture as specified in Table 1. When not present, the value of modality_type is inferred to be equal to 0, denoting that the modality type of the picture is unknown or unspecified or determined by other means not specified in this Specification. When modality_type is equal to 2 or 3, decoders shall ignore vui_colour_primaries, vui_transfer_characteristics, vui_matrix_coeffs, and vui_full_range_flag indicated in the VUI parameters

Table 1– Mapping of modality_type to the type of picture modalities

| mi_modality_type | type of picture modality |
| --- | --- |
| 0 | Unspecified |
| 1 | Visible Picture |
| 2 | Infrared Picture |
| 3 | Ultraviolet Picture |
| 4..255 | Reserved for future use |

spectrum_range_present_flag equal to 1 specifies that the spectrum band of the optical radiation wavelength represented by the pictures in the coded video sequence is present in the modality information SEI messages. spectrum_range_present_flag equal to 0 specifies that the spectrum band of the optical radiation wavelength represented by the pictures in the coded video sequence is not present in the modality information SEI messages.

min_wavelength_mantissa specifies the mantissa part of the minimum wavelength indicating the spectral band of optical radiation represented by the pictures in the coded video sequence. When min_wavelength_mantissa is equal to 0 or is not present, the minimum wavelength indicating the spectral band of optical radiation represented by the pictures in the coded video sequence is unknown or unspecified or determined by other means not specified in this Specification

min_wavelength_exponent_plus15 minus 15 specifies the exponent part of the minimum wavelength indicating the spectral band of optical radiation represented by the pictures in the coded video sequence. When min_wavelength_mantissa is not present or is equal to 0, decoders shall ignore the value of min_wavelength_exponent_plus15.

The value of the minimum wavelength indicating the spectral band of optical radiation represented by the pictures in the coded video sequence is derived as follows:

MinWavelength = min_wavelength_mantissa * 10 min_wavelength_exponent_plus15-15

max_wavelength_mantissa specifies the mantissa part of the maximum wavelength indicating the spectral band of optical radiation represented by the pictures in the coded video sequence. When max_wavelength_mantissa is equal to 0 or is not present, the maximum wavelength indicating the spectral band of optical radiation represented by the pictures in the coded video sequence is unknown or unspecified or determined by other means not specified in this Specification.

max_wavelength_exponent_plus15 minus 15 specifies the exponent part of the maximum wavelength indicating the spectral band of optical radiation represented by the pictures in the coded video sequence. When max_wavelength_mantissa is not present or is equal to 0, decoders shall ignore the value of max_wavelength_exponent_plus15.

The value of the maximum wavelength indicating the spectral band of optical radiation represented by the pictures in the coded video sequence is derived as follows:

MaxWavelength = max_wavelength_mantissa * 10 max_wavelength_exponent_plus15-15

MinWavelength and MaxWavelength are in units of meters as specified in ISO/IEC 80000.

modality_type_extension_bits equal to 0 specifies that reserved_modality_type_extension is not present. modality_type_extension_bits greater than 0 specifies the length, in bits, of reserved_modality_type_extension.

The value of modality_type_extension_bits shall be in the range of 0 to 2 048, inclusive. Values in the range of 1 to 2 048, inclusive, for modality_type_extension_bits are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this document. Decoders conforming to this version of this document shall allow any value of modality_type_extension_bits in the range of 0 to 2 048, inclusive, to appear in the syntax.

reserved_modality_type_extension shall not be present in bitstreams conforming to this version of this document. However, decoders conforming to this version of this document shall allow the presence of reserved_modality_type_extension in the syntax, but ignore the value. When present, the length, in bits, of reserved_modality_type_extension is equal to modality_type_extension_bits.

# Proposed text for AVC

AVC will be in the FDIS stage in April 2024, but VSEI v4 may start from April 2024, hence, it is impossible to refer the SEI of VSEI from AVC currently. Therefore, we proposed to include the semantics and syntax of the modality information SEI message directly in AVC.

## D.1 SEI payload syntax

### D.1.1 General SEI message syntax

| sei_payload( payloadType, payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| … |  |  |
| else if( payloadType = = 210 ) |  |  |
| nn_post_filter_characteristics( payloadSize ) | 5 |  |
| else if( payloadType = = 211 ) |  |  |
| nn_post_filter_activation( payloadSize ) | 5 |  |
| else if( payloadType = = 212 ) |  |  |
| phase_indication( payloadSize ) | 5 |  |
| else if( payloadType = = 213 ) |  |  |
| modality_info( payloadSize ) | 5 |  |
| else |  |  |
| reserved_sei_message( payloadSize ) | 5 |  |
| if( !byte_aligned( ) ) { |  |  |
| bit_equal_to_one /* equal to 1 */ | 5 | f(1) |
| while( !byte_aligned( ) ) |  |  |
| bit_equal_to_zero /* equal to 0 */ | 5 | f(1) |
| } |  |  |
| } |  |  |

### D.1.xx Modality information SEI message syntax

| modality_info( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| modality_info_cancel_flag | 5 | u(1) |
| if(!modality_info_cancel_flag) { |  |  |
| modality_info_persistence_flag | 5 | u(1) |
| modality_type | 5 | u(8) |
| spectrum_range_present_flag | 5 | u(1) |
| if( spectrum_range_present_flag ) { |  |  |
| min_wavelength_mantissa | 5 | u(11) |
| min_wavelength_exponent_plus15 | 5 | u(5) |
| max_wavelength_mantissa | 5 | u(11) |
| max_wavelength_exponent_plus15 | 5 | u(5) |
| } |  |  |
| modality_type_extension_bits | 5 | ue(v) |
| if(modality_type_extension_bits > 0 ) |  |  |
| reserved_modality_type_extension | 5 | u(v) |
| } |  |  |
| } |  |  |

## D.2 SEI payload semantics

### D.2.41 Modality information SEI message semantics

modality_info_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous modality information SEI message in output order. modality_info_cancel_flag equal to 0 indicates that modality information follows.

modality_info_persistence_flag specifies the persistence of the modality information SEI message.

modality_info_persistence_flag equal to 0 specifies that the modality information SEI message applies to the current decoded picture only.

Let picA be the current picture. modality_info_persistence_flag equal to 1 specifies that the modality information SEI message persists in output order until one or more of the following conditions are true:

– A new coded video sequence begins.

– The bitstream ends.

– A picture picB in an access unit containing a modality information SEI message is output having PicOrderCnt( picB ) greater than PicOrderCnt( picA ).

modality_type indicates the type of modality of the decoded picture as specified in Table 1. When not present, the value of modality_type is inferred to be equal to 0, denoting that the modality type of the picture is unknown or unspecified or determined by other means not specified in this Specification. When modality_type is equal to 2 or 3, decoders shall ignore vui_colour_primaries, vui_transfer_characteristics, vui_matrix_coeffs, and vui_full_range_flag indicated in the VUI parameters

Table 1– Mapping of modality_type to the type of picture modalities

| mi_modality_type | type of picture modality |
| --- | --- |
| 0 | Unspecified |
| 1 | Visible Picture |
| 2 | Infrared Picture |
| 3 | Ultraviolet Picture |
| 4..255 | Reserved for future use |

spectrum_range_present_flag equal to 1 specifies that the spectrum band of the optical radiation wavelength represented by the pictures in the coded video sequence is present in the modality information SEI messages. spectrum_range_present_flag equal to 0 specifies that the spectrum band of the optical radiation wavelength represented by the pictures in the coded video sequence is not present in the modality information SEI messages.

min_wavelength_mantissa specifies the mantissa part of the minimum wavelength indicating the spectral band of optical radiation represented by the pictures in the coded video sequence. When min_wavelength_mantissa is equal to 0 or is not present, the minimum wavelength indicating the spectral band of optical radiation represented by the pictures in the coded video sequence is unknown or unspecified or determined by other means not specified in this Specification

min_wavelength_exponent_plus15 minus 15 specifies the exponent part of the minimum wavelength indicating the spectral band of optical radiation represented by the pictures in the coded video sequence. When min_wavelength_mantissa is not present or is equal to 0, decoders shall ignore the value of min_wavelength_exponent_plus15.

The value of the minimum wavelength indicating the spectral band of optical radiation represented by the pictures in the coded video sequence is derived as follows:

MinWavelength = min_wavelength_mantissa * 10 min_wavelength_exponent_plus15-15

max_wavelength_mantissa specifies the mantissa part of the maximum wavelength indicating the spectral band of optical radiation represented by the pictures in the coded video sequence. When max_wavelength_mantissa is equal to 0 or is not present, the maximum wavelength indicating the spectral band of optical radiation represented by the pictures in the coded video sequence is unknown or unspecified or determined by other means not specified in this Specification.

max_wavelength_exponent_plus15 minus 15 specifies the exponent part of the maximum wavelength indicating the spectral band of optical radiation represented by the pictures in the coded video sequence. When max_wavelength_mantissa is not present or is equal to 0, decoders shall ignore the value of max_wavelength_exponent_plus15.

The value of the maximum wavelength indicating the spectral band of optical radiation represented by the pictures in the coded video sequence is derived as follows:

MaxWavelength = max_wavelength_mantissa * 10 max_wavelength_exponent_plus15-15

MinWavelength and MaxWavelength are in units of meters as specified in ISO/IEC 80000.

modality_type_extension_bits equal to 0 specifies that reserved_modality_type_extension is not present. modality_type_extension_bits greater than 0 specifies the length, in bits, of reserved_modality_type_extension.

The value of modality_type_extension_bits shall be in the range of 0 to 2 048, inclusive. Values in the range of 1 to 2 048, inclusive, for modality_type_extension_bits are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this document. Decoders conforming to this version of this document shall allow any value of modality_type_extension_bits in the range of 0 to 2 048, inclusive, to appear in the syntax.

reserved_modality_type_extension shall not be present in bitstreams conforming to this version of this document. However, decoders conforming to this version of this document shall allow the presence of reserved_modality_type_extension in the syntax, but ignore the value. When present, the length, in bits, of reserved_modality_type_extension is equal to modality_type_extension_bits.

## G.13 Supplemental enhancement information

The specifications in Annex ‎D together with the extensions and modifications specified in this clause apply.

### G.13.2 SEI payload semantics

The semantics of the SEI messages with payloadType in the range of 0 to 23, inclusive, or equal to 45, 47, 137, 142, 144, 147, 148, 149, 150, 151, 154, 155, 156, 200, 201, 202, 205, 210, 211, 212 or 213, which are specified in clause ‎D.2, are extended as follows:

–	If payloadType is equal to 3, 8, 19, 20, or 22, the following applies:

–	If the SEI message is not included in a scalable nesting SEI message, it applies to the layer representations of the current access unit that have dependency_id equal to 0 and quality_id equal to 0.

The semantics as specified in clause ‎D.2 apply to the bitstream that would be obtained by invoking the bitstream extraction process as specified in clause ‎G.8.8.1 with dIdTarget equal to 0 and qIdTarget equal to 0. All syntax elements and derived variables that are referred to in the semantics in clause ‎D.2 are syntax elements and variables for layer representations with dependency_id equal to 0 and quality_id equal to 0. All SEI messages that are referred to in clause ‎D.2 are SEI messages that apply to layer representations with dependency_id equal to 0 and quality_id equal to 0.

–	Otherwise (the SEI message is included in a scalable nesting SEI message), the SEI message applies to all layer representations of the current access unit for which DQId is equal to any value of ( ( sei_dependency_id[ i ] << 4 ) + sei_quality_id[ i ] ) with i in the range of 0 to num_layer_representations_minus1, inclusive.

For each value of i in the range of 0 to num_layer_representations_minus1, inclusive, the semantics as specified in clause ‎D.2 apply to the bitstream that would be obtained by invoking the bitstream extraction process as specified in clause ‎G.8.8.1 with dIdTarget equal to sei_dependency_id[ i ] and qIdTarget equal to sei_quality_id[ i ]. All syntax elements and derived variables that are referred to in the semantics in clause ‎D.2 are syntax elements and variables for layer representations with dependency_id equal to sei_dependency_id[ i ] and quality_id equal to sei_quality_id[ i ]. All SEI messages that are referred to in clause ‎D.2 are SEI messages that apply to layer representations with dependency_id equal to sei_dependency_id[ i ] and quality_id equal to sei_quality_id[ i ].

–	Otherwise, if payloadType is equal to 2, 6, 7, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 21, 23, 45, 47, 137, 142, 144, 147, 148, 149, 150, 151, 154, 155, 156, 200, 201, 202, 205, 210, 211, 212 or 213, the following applies:

–	If the SEI message is not included in a scalable nesting SEI message, it applies to the dependency representations of the current access unit that have dependency_id equal to 0.

The semantics as specified in clause ‎D.2 apply to the bitstream that would be obtained by invoking the bitstream extraction process as specified in clause ‎G.8.8.1 with dIdTarget equal to 0. All syntax elements and derived variables that are referred to in the semantics in clause ‎D.2 are syntax elements and variables for dependency representations with dependency_id equal to 0. All SEI messages that are referred to in clause ‎D.2 are SEI messages that apply to dependency representations with dependency_id equal to 0.

–	Otherwise (the SEI message is included in a scalable nesting SEI message), the scalable nesting SEI message containing the SEI message shall have all_layer_representations_in_au_flag equal to 1 or, when all_layer_representations_in_au_flag is equal to 0, all values of sei_quality_id[ i ] present in the scalable nesting SEI message shall be equal to 0. The SEI message that is included in the scalable nesting SEI message applies to all dependency representations of the current access unit for which dependency_id is equal to any value of sei_dependency_id[ i ] with i in the range of 0 to num_layer_representations_minus1, inclusive.

For each value of i in the range of 0 to num_layer_representations_minus1, inclusive, the semantics as specified in clause ‎D.2 apply to the bitstream that would be obtained by invoking the bitstream extraction process as specified in clause ‎G.8.8.1 with dIdTarget equal to sei_dependency_id[ i ]. All syntax elements and derived variables that are referred to in the semantics in clause ‎D.2 are syntax elements and variables for dependency representations with dependency_id equal to sei_dependency_id[ i ]. All SEI messages that are referred to in clause ‎D.2 are SEI messages that apply to dependency representations with dependency_id equal to sei_dependency_id[ i ].

When payloadType is equal to 10 for the SEI message that is included in a scalable nesting SEI message, the semantics for sub_seq_layer_num of the sub-sequence information SEI message is modified as follows:

sub_seq_layer_num specifies the sub-sequence layer number of the current picture. When the current picture resides in a sub-sequence for which the first picture in decoding order is an IDR picture, the value of sub_seq_layer_num shall be equal to 0. For a non-paired reference field, the value of sub_seq_layer_num shall be equal to 0. sub_seq_layer_num shall be in the range of 0 to 255, inclusive.

–	Otherwise, if payloadType is equal to 0 or 1, the following applies:

–	If the SEI message is not included in a scalable nesting SEI message, the following applies. When the SEI message and all other SEI messages with payloadType equal to 0 or 1 not included in a scalable nesting SEI message are used as the buffering period and picture timing SEI messages for checking the bitstream conformance according to Annex ‎C and the decoding process specified in clauses ‎2 to ‎9 is used, the bitstream shall be conforming to this Recommendation | International Standard.

The value of seq_parameter_set_id in a buffering period SEI message not included in a scalable nesting SEI message shall be equal to the value of seq_parameter_set_id in the picture parameter set that is referenced by the layer representation with DQId equal to 0 of the primary coded picture in the same access unit.

–	Otherwise (the SEI message is included in a scalable nesting SEI message), the following applies. When the SEI message and all other SEI messages with payloadType equal to 0 or 1 included in a scalable nesting SEI message with identical values of sei_temporal_id, sei_dependency_id[ i ], and sei_quality_id[ i ] are used as the buffering period and picture timing SEI messages for checking the bitstream conformance according to Annex ‎C, the bitstream that would be obtained by invoking the bitstream extraction process as specified in clause ‎G.8.8.1 with tIdTarget equal to sei_temporal_id, dIdTarget equal to sei_dependency_id[ i ], and qIdTarget equal to sei_quality_id[ i ] shall be conforming to this Recommendation | International Standard.

In the semantics of clauses ‎D.2.1 and ‎D.2.3, the syntax elements num_units_in_tick, time_scale, fixed_frame_rate_flag, nal_hrd_parameters_present_flag, vcl_hrd_parameters_present_flag, low_delay_hrd_flag, and pic_struct_present_flag and the derived variables NalHrdBpPresentFlag, VclHrdBpPresentFlag, and CpbDpbDelaysPresentFlag are substituted with the syntax elements vui_ext_num_units_in_tick[ i ], vui_ext_time_scale[ i ], vui_ext_fixed_frame_rate_flag[ i ], vui_ext_nal_hrd_parameters_present_flag[ i ], vui_ext_vcl_hrd_parameters_present_flag[ i ], vui_ext_low_delay_hrd_flag[ i ], and vui_ext_pic_struct_present_flag[ i ] and the derived variables VuiExtNalHrdBpPresentFlag[ i ], VuiExtVclHrdBpPresentFlag[ i ], and VuiExtCpbDpbDelaysPresentFlag[ i ].

The value of seq_parameter_set_id in a buffering period SEI message included in a scalable nesting SEI message with the values of sei_dependency_id[ i ] and sei_quality_id[ i ] shall be equal to the value of seq_parameter_set_id in the picture parameter set that is referenced by the layer representation with DQId equal to (( sei_dependency_id[ i ] << 4 ) + sei_quality_id[ i ]) of the primary coded picture in the same access unit.

–	Otherwise (payloadType is equal to 4 or 5), the corresponding SEI message semantics are not extended.

When an SEI message having a particular value of payloadType equal to 137 or 144, contained in a scalable nesting SEI message, and applying to a particular combination of dependency_id, quality_id, and temporal_id is present in an access unit, the SEI message with the particular value of payloadType applying to the particular combination of dependency_id, quality_id, and temporal_id shall be present a scalable nesting SEI message in the IDR access unit that is the first access unit of the coded video sequence.

All SEI messages having a particular value of payloadType equal to 137 or 144, contained in scalable nesting SEI messages, and applying to a particular combination of dependency_id, quality_id, and temporal_id present in a coded video sequence shall have the same content.

For the semantics of SEI messages with payloadType in the range of 0 to 23, inclusive, or equal to 45, 47, 137, 142, 144, 147, 148, 149, 150, 151, 154, 155, 156, 200, 201, 202, 205, 210, 211, 212 or 213, which are specified in clause ‎D.2, SVC sequence parameter set is substituted for sequence parameter set; the parameters of the picture parameter set RBSP and SVC sequence parameter set RBSP that are in effect are specified in clause ‎G.7.4.1.2.1.

Coded video sequences conforming to one or more of the profiles specified in Annex ‎G shall not include SEI NAL units that contain SEI messages with payloadType in the range of 36 to 44, inclusive, or equal to 46, which are specified in clause ‎H.13, or with payloadType in the range of 48 to 53, inclusive, which are specified in clause ‎I.13.

When an SEI NAL unit contains an SEI message with payloadType in the range of 24 to 35, inclusive, which are specified in clause ‎G.13, it shall not contain any SEI message that has payloadType less than 24 or equal to 45, 47, 137, 142, 144, 147, 148, 149, 150, 151, 154, 155, 156, 200, 201, 202, 205, 210, 211, 212 or 213 that is not included in a scalable nesting SEI message, and the first SEI message in the SEI NAL unit shall have payloadType in the range of 24 to 35, inclusive.

When an SEI NAL unit contains an SEI message with payloadType equal to 24, 28, or 29, it shall not contain any SEI message with payloadType not equal to 24, 28, or 29.

When a scalable nesting SEI message (payloadType is equal to 30) is present in an SEI NAL unit, it shall be the only SEI message in the SEI NAL unit.

The semantics for SEI messages with payloadType in the range of 24 to 35, inclusive, are specified in the following.

# 5	Reference

[1] J. Gao, H. B. Teo, C. S. Lim, and K. Abe, “AHG8/AHG9: On Picture Modality Type”, JVET-AF00147, 32nd Meeting, Hannover, DE, 13–20 October 2023.

[2] M. M. Hannuksela, J. Chen, S. Deshpande, Hendry, and S. McCarthy, “Technologies under consideration for future extensions of VSEI (version 2)”, JVET-AF2032, 32nd Meeting, Hannover, DE, 13–20 October 2023.

[3] J. Gao, H. B. Teo, C. S. Lim, K. Abe, V. Drugeon, “AHG9: On Picture Modality Information”, JVET-AG0077, 33rd Meeting, Online, 17–26 January 2024.

[4] J. Gao, H. B. Teo, C. S. Lim, K. Abe, V. Drugeon, “AHG2/AHG9: VUI extension mechanism and picture modality information for AVC and HEVC”, JVET-AG0079, 33rd Meeting, Online, 17–26 January 2024.

# 6	Patent rights declaration(s)

Panasonic may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).