| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0329-v1 |
| --- | --- |

| Title: | On update of Multilayer conformance test for multiple profiles signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shunsuke Iwamura Shimpei Nemoto Atsuro Ichigaya Fabrice Urban Philippe de Lagrange | Tel: Email: | iwamura.s-gc@nhk.or.jp Fabrice.Urban@InterDigital.com |
| Source: | NHK, InterDigital |  |  |

_____________________________

- Abstract

Additional conformance bitstream for multilayer coding configurations is defined in JVET-AE2028. During the crosscheck of the conformance bitstreams, it was found that the bitstreams are not well formed, especially the VPS does not contain intended data. This contribution proposes to update those conformance test with revised configuration and also proposes VTM encoder only changes to solve the issues identified during the regeneration of the conformance bitstream.

Introduction

In JVET-AE2028, OLSMultiProfile_A_NHK_1, OLSMultiProfile_B_NHK_1, and OLSMultiProfile_C_NHK_1 are defined as follows:

        - Test bitstream OLSMultiProfile_A_NHK

Specification: The bitstream is coded with two layers, where OLS0 including the lower layer specifies Main 10 profile whereas OLS1 including both layers specifies Multilayer Main 10

Functional stage: High-level syntax.

Purpose: Check that the decoder supporting Main 10 can properly decode OLS0 and the decode supporting Multilayer Main 10 can properly decode both OLS0 and OLS1.

        - Test bitstream OLSMultiProfile_B_NHK

Specification: The bitstream is coded with two layers, where each layer is coded as independent layer and both OLSs specifies Main 10 profile.

Functional stage: High-level syntax.

Purpose: Check that the decoder supporting Main 10 can properly decode both OLSs.

        - Test bitstream OLSMultiProfile_C_NHK

Specification: The bitstream is coded with three layers, where lowest independent layer is referenced from the two higher layers, where OLSs for higher layer specifies Multilayer Main 10 profile whereas OLS0 specifies Main 10 profile.

Functional stage: High-level syntax.

Purpose: Check that the decoder supporting Main 10 can properly decode OLS0 and the decoder supporting Multilayer Main 10 can properly decode all the OLSs.

Bitstream packages have been downloaded from

www.itu.int - /wftp3/av-arch/jvet-site/bitstream_exchange/VVCMultilayer/under_test/VTM-23.0/

Identified issues

Test bitstream OLSMultiProfile_A_NHK_1

The bitstream contains 2 layers, 2 OLSs, 2 PTLs (1 for each OLS).

- PTL index 0 is main 10 profile (general_profile_idc=1), main tier, level 2.1 (general_level_idc = 35) attached to OLS 0 outputting the base layer (416x240, 4:2:0 chroma format 10 bits)., consistent with content of SPS 0.

- PTL index 1 is multilayer main 10 profile (general_profile_idc=17), main tier, level 6.2 (general_level_idc = 102) attached to OLS 1 outputting both the base layer and dependent layer. SPS 1 signals level 2.1

In the text description file of the conformance test, it is described that the level of both PTL is a 2.1, but 6.2 is signalled which is a bit high. Given the size of the picture to be decoded, 2 pictures 416x240 at 60Hz (11 980 800 luma samples per second), level 3.0 is sufficient.

Figure 1. HLS structure of OLSMultiProfile_A_NHK_1

Test bitstream OLSMultiProfile_B_NHK_1

The bitstream contains 2 independent layers, 2 OLSs (1 for each layer), 2 PTLs (1 for each OLS).

- PTL index 0 is main 10 profile (general_profile_idc=1), main tier, level 2.1 (general_level_idc = 35) attached to OLS 0 outputting layer 0 (416x240, 4:2:0 chroma format 10 bits).

- PTL index 1 has no profile defined, level none (general_level_idc = 0) attached to OLS 1 outputting independent layer.1

- SPS index 0 has PTL main 10 profile (general_profile_idc=1), main tier, level 2.1 (general_level_idc = 35)

- SPS index 1 has PTL main 10 profile (general_profile_idc=1), main tier, level 2.0 (general_level_idc = 32)

PTL info in VPS and in SPS should be the same, from VVC specification:

“When NumLayersInOls[ i ] is equal to 1, the profile_tier_level( ) syntax structure that applies to the i-th OLS is also present in the SPS referred to by the layer in the i-th OLS. It is a requirement of bitstream conformance that, when NumLayersInOls[ i ] is equal to 1, the profile_tier_level( ) syntax structures signalled in the VPS and in the SPS for the i-th OLS shall be identical.”

Figure 2. HLS structure of OLSMultiProfile_B_NHK_1

Test bitstream OLSMultiProfile_C_NHK_1

The bitstream contains 3 layers (layers 1 and 2 depend on layer 0), 3 OLSs (1 with layer 0, 1 with layers 0&1, and 1 with layers 0&2), 3 PTLs (1 for each OLS).

- PTL index 0 is main 10 profile (general_profile_idc=1), main tier, level 2.1 (general_level_idc = 35) attached to OLS 0 outputting layer 0 (416x240, 4:2:0 chroma format 10 bits), consistent with SPS 0 definitions

- PTL index 1 has no profile defined, level 6.2 (general_level_idc = 102) attached to OLS 1 outputting layers 0 and.1

- PTL index 2 has no profile defined, level 6.2 (general_level_idc = 102) attached to OLS 2 outputting layers 0 and.2

Multilayer main 10 profile is signaled only in the SPS 1 and 2, with level 2.1, which is (i) not sufficient, and (ii) inconsistent with PTL definition.

In the text description file of the conformance test, it is described that the level of PTL 1 and 2 is also 2.1, but 6.2 is signaled, which is a bit high. Given the size of the picture to be decoded, 2 pictures 416x240 at 60Hz (11 980 800 luma samples per second), level 3.0 is sufficient.

Figure 3. HLS structure of OLSMultiProfile_C_NHK_1

Proposed update of the conformance test

Generating the new conformance test was made by NHK and crosschecked by InterDigital.

OLSMultiProfile_A_NHK_2

The conformance specification does not need to be changed. The test description of the conformance test attached in the bitstream needs to be changed to modify the levels of the PTL1 and SPS1 (changed to 3.0) to be consistent.

OLSMultiProfile_B_NHK_2

The conformance specification does not need to be changed. The test description of the conformance test attached in the bitstream needs to be changed to modify OlsPtlIdx and the level of the SPS1 to be consistent with PTL0.

OLSMultiProfile_C_NHK_2

The conformance specification does not need to be changed. The test description of the conformance test attached in the bitstream needs to be changed to modify the number of PTL signalled in VPS and the level of the PTL1, SPS1, and SPS2.

Proposed software change (encoder only)

During the regeneration of the conformance bitstreams, VTM implementation issues were identified.

- The number of OLSs are not set properly when All layers are independent layer and each layer is single output layer. The proposed change is as follows:

EncApp.cpp EncApp::xInitLibCfg()

if (vps.getAllIndependentLayersFlag())

{

vps.setEachLayerIsAnOlsFlag(m_eachLayerIsAnOlsFlag);

if (vps.getEachLayerIsAnOlsFlag() == 0)

{

vps.setOlsModeIdc(2);

}

else

{

vps.setNumOutputLayerSets(m_numOutputLayerSets);

}

}

- In the VVC specifications, when the number of PTL is equal to the number of OLS, index OlsPtlIdx for each OLS is not signalled but inferred. However, in the VTM implementation, configuration parameter “OlsPTLIdxN” (N is the index of the layer) are assigned to OlsPtlIdx of the corresponding layer index even when numPTL == numOLS. Even worse, when “OlsPTLIdxN” are not present, the initialized value 0 is assigned to OlsPtlIdx of all the layer. The proposed change is as follows:

EncApp.cpp EncApp::xInitLibCfg()

if (vps.getNumPtls() == vps.getNumOutputLayerSets())

{

for (int i = 0; i < vps.getNumOutputLayerSets(); i++)

{

vps.setOlsPtlIdx (i, i);

}

}

else

{

for (int i = 0; i < vps.getNumOutputLayerSets(); i++)

{

vps.setOlsPtlIdx (i, m_olsPtlIdx[i]);

}

}

The proposed changes are attached as a patch file in the contribution and the merge request (MR!2714) is submitted.

Conclusion

It is recommended to update the multilayer conformance bitstreams and VTM implementation.

NHK may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).