| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG1004-v1 |
| --- | --- |

| Title: | Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Errata report |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Benjamin Bross Iole Moccagatta Chris Rosewarne Gary J. Sullivan | Tel: Email: | yekui.wang@bytedance.com benjamin.bross@hhi.fraunhofer.de iole.moccagatta@intel.com chris.rosewarne@canon.com.au gary.sullivan@dolby.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document contains a list of reported errata items for VVC, VSEI, HEVC, AVC, and Video CICP, for tracking purposes. Some of the items have been confirmed by the JVET and have been agreed to require fixing. This document also provides publication status backgrounds of these standards.

Incorporated items at the JVET-AG meeting:

- For VVC:

  - In subclause 7.4.2.4.5, the first bullet item, replace "subpicture level values" with "subpicture index values". (reported by Rickard Sjöberg – thanks!)

- For VSEI:

  - Fix a typo when vui_chroma_sample_loc_type_frame is mentioned in the VSEI text. (JVET-AG0192)

  - Fix two bugs for the constraints on nnpfc_matrix_coeffs and add two NOTEs in the semantics of nnpfc_matrix_coeffs to clarify complicated cases. (JVET-AG0192)

  - Fix an asserted bug on the inference of nnpfc_full_range_flag. (JVET-AG0192)

Incorporated items at the JVET-AF meeting:

- For VVC:

  - Add a missing space in "if(IntraSubPartitionsSplitType  !=  ISP_NO_SPLIT )" (JVET-AF0064).

  - On coding of syntax elements of fix-valued, non-reserved bits using f(N) instead of u(N). (JVET-AF0064).

  - Replace the two instances of "in the first access unit of the CVS" with "in the first access unit of the CVS in decoding order". (JVET-AF0189)

- For VSEI:

  - Replace each of the 10 instances of "nuh_layer_id" in subclauses 8.22.2 and 8.23.2 with "layer identification". (from Gary J. Sullivan)

  - On cancellation of the persistence of a CTI SEI message by another CTI SEI message. (JVET-AF0064)

  - On coding of syntax elements of fix-valued, non-reserved bits using f(N) instead of u(N). (JVET-AF0064).

  - Consider adding a NOTE in VSEI telling something like: 1) AVC can reference VSEI for enabling the inclusion of only those SEI messages for which the SEI payloads are specified in VSEI but the syntax has not been extended; 2) HEVC and VVC can reference VSEI for enabling the inclusion of SEI messages for which the SEI payloads are specified in VSEI regardless of whether the syntax has been extended or not. (from Ye-Kui Wang)

- For HEVC:

  - Updated the publication status background.

  - (Perhaps for other specs such as AVC, VVC, and VSEI as well) On that the term of "true subset" should be replaced with "proper subset" or "strict subset". (from Gary J. Sullivan)

  - (Perhaps for other specs such as AVC, VVC, and VSEI as well) On that the mixed uses of Boolean operators on numerical values and vice versa should cleaned up. (from Gary J. Sullivan)

  - Removed obsoleted HEVC errata items that were included in JVET-AE1004.

- For AVC:

  - Removed obsoleted AVC errata items that were included in JVET-AE1004.

Incorporated items at the JVET-AE meeting:

- Updated the general descriptions for VVC, VSEI, HEVC, and AVC.

- Removed obsolete/resolved errata items.

- Added 16 items for the ISO text only for HEVC, identified when working on the ITU-T consent text for H.265 version 9 (edition 5).

- Added 9 items on the semantics of post-filter hint SEI message from JVET-AE0155 for both HEVC and AVC.

- Added 4 items for AVC, on semantics of the omnidirectional video SEI messages. These items were among the items for HEVC and have been resolved for HEVC.

- Added an item for AVC on use of the "should" wording for the use of the film grain characteristics SEI messages when present.

- Removed the section for codepoint usage TR as the TR part has been dropped from the title of the document a while ago.

- Added an item for HEVC regarding replacing of "Max slice segments per picture MaxSliceSegmentsPerPicture" in Table A.8 with "Max # of slice segments per picture MaxSliceSegmentsPerPicture" (reported by Cliff Reader – thanks!).

Incorporated items at the JVET-AD meeting:

- For AVC, incorporated the following items:

  - JVET-AD0078 item 4 and 5 on fixing the equation to derive the filter hint cross correlation matrix.

Incorporated items at the JVET-AC meeting:

- For VVC, all errata items have been incorporated in JVET-AC2005: New level and systems-related supplemental enhancement information for VVC (Draft 4).

- For HEVC, incorporated the following items:

  - Confirmed redudnant call to clause 8.6.1 (reported by Cliff Reader – thanks!).

  - Confirmed JVET-AB0120: Addition of interpolation for "initial CPB removal delay offset".

  - JVET-AC0311: On number of conformance tests (option 1.)

  - Updated issues 1 to 4 in Subsection 4.4 per inputs from Miska M. Hannuksela (thanks!).

  - Change the font size from 8 pt to 10 pt for "Table 9-4" in a paragraph in clause 9.3.2.2 (reported by Cliff Reader – thanks!).

- For AVC, incorporated the following items:

  - Confirmed JVET-AB0120: Addition of interpolation for "initial CPB removal delay offset".

  - An issue of talking about VUI in subclause 0.6.1 in ISO/IEC 14496-10:2022 (Edition 10) while the sentence should only talk about SEI messages.

Incorporated items at the JVET-AB meeting:

- For VVC, incorporated the following items:

  - JVET-AB0223: Text improvement for Timing / DU information SEI message in HEVC and VVC

  - Added a NOTE in the HRD text to mention that "leading pictures" associated with a DRAP picture may not be correctly decodable, per the discussion of JVET-AB0055 (On leading pictures design in DRAP SEI Message).

  - Added a NOTE to mention that the definition of IRAP in VSEI covers the case of GDR with ph_recovery_poc_cnt equal to 0, per the discussion of JVET-AB0057 (On the associated IRAP for DRAP and EDRAP pictures).

  - Tickets #1564, #1568, #1569, and #1572

  - JVET-AB0120: Addition of interpolation for "initial CPB removal delay offset"

- For HEVC, incorporated the following items:

  - JVET-AB0223: Text improvement for Timing / DU information SEI message in HEVC and VVC

  - On conformance indication of the Multiview Main, Scalable Main, Scalable Main 10, and 3D Main profiles (resulted from a discussion between Alexis Tourapis and Gary Sullivan – thanks!)

  - On inference of high_precision_offsets_enabled_flag (resulted from a discussion between Andy Fu and Gary Sullivan – thanks!)

  - On a redundant sentence in the definitionn of next_bits( n ) from Gary Sullivan

  - On addition the parenthetic abbreviation TB to the text in clause 9.3.3.1 (suggested by Cliff Reader– thanks!)

  - On indention of item 3 in the paragraph just below Equation 8-14 in clause 8.4.1 (reported and suggested by Cliff Reader – thanks!)

  - On redudnant call to clause 8.6.1 (reported by Cliff Reader – thanks!)

  - JVET-AB0120: Addition of interpolation for "initial CPB removal delay offset"

- For AVC, incorporated the following items:

  - JVET-AB0120: Addition of interpolation for "initial CPB removal delay offset"

Incorporated items at the JVET-AA meeting:

- For VVC: Added an errata item for VVC conformance.

- For VSEI: Added a pointer of some HEVC errata items that also apply for VSEI in Subsection 4.2.

- For HEVC: Removed various items that have been addressed during the editing of the DIS text of HEVC 5th edition in WG05 N00128, and added some editor's notes for the remaining items.

- For AVC:

  - Added changes on motion vector value ranges, based on JVET meeting minutes noted under JVET-AA0222.

  - Added a pointer of some HEVC errata items that also apply for AVC in Subsection 4.2.

Incorporated items at the JVET-Z meeting:

- For AVC, corrections to texts related to alpha blending corresponding to the changes adopted to VSEI from JVET-Z0119.

- Updated the summary and publication status for VVC.

- Updated the summary and publication status for VSEI.

- Updated the summary for HEVC.

Incorporated items at the JVET-Y meeting:

- Removed all errata items for VVC and VSEI, due to that for both VVC and VSEI the second edition texts are being prepared by both ITU-T and ISO/IEC, and all the errata items have been incorporated therein, including fixes for bug tickets #1533 and #1534 for VVC (fixes for these two tickets have not been included in previous versions of the errata report).

- Removed errata items for AVC that have been resolved in both the latest ITU-T text (08/2021) and the new submission (MDS21132_WG05_N00098) of the ISO/IEC 10th edition text.

Incorporated items at the JVET-X meeting:

- Updated the publication status of the standards

- For VVC (the changes are included in an attachment to this document):

  - Some changes resulted from reports/suggestions by Yue Yu and Frank Bossen (thanks!)

  - Various changes resulted from reports/suggestions by Peter de Rivaz (huge thanks!)

- For HEVC (the changes are included both below and in an attachment to this document):

  - Some changes resulted from reports/suggestions by Yue Yu, Frank Bossen, and Cliff Reader (thanks!)

  - Some changes to the semantics of the entry point offset syntax element, resulted from the discussion of JVET-X0050

  - Changes to the semantics of the alpha channel information SEI message resulted from the discussion of JVET-X0059

- For AVC (the changes are included in an attachment to this document):

  - Some changes to the semantics of the alpha blendnig related syntax elements in clause 7.4.2.1.2, message resulted from the discussion of JVET-X0059

Incorporated items at the JVET-W meeting:

- For VVC (the changes are included in an attachment to this document):

  - Fix for tickets #1488, #1511, #1491, #1493, #1494

- For HEVC:

  - Added the attachment for HEVC from JVET-W0187, which includes the following fixes:

    - Correcting an “off by one” error in the loop counter for ar_num_object_updates, which indicates the number of updated annotated objects for the annotated regions SEI message.

    - Adding a description of the purpose of some variables that are derived in the semantics of the annotated regions SEI message

    - Adding a missing descriptor to specify the type of coding for the depth_nonlinear_representation_model[ i ] syntax element (the same as used in another specification, so clearly what was intended) for the depth representation information SEI message

    - Adding missing range specifications for a few syntax elements of the depth representation information SEI message

    - Miscellaneous very small cleanups of consistency, grammar and clarity, and correction of typographical errors.

- For AVC:

  - Added the attachment for HEVC from JVET-W0187, which includes the same list of fixes as above for HEVC.

Incorporated items at the JVET-V meeting:

- For VVC (the changes are included in an attachment to this document):

  - Fix for ticket #1486

- For HEVC

  - Removed the item on the absence of “persistence flag” for the annotated regions SEI message (this was agreed to be removed at the JVET-T meeting in October 2020, but the decision was overlooked)

Incorporated items at the JVET-U meeting:

- For VVC (the changes are included in an attachment to this document):

  - Some minor editorial corrections and improvements (JVET-U0073, JVET-U0085, and one item from Hendry)

  - Fixes for tickets #1416, #1428, #1432, #1454, and #1469

- For AVC: Added a reference to JVET-T1006-v1 for the errata changes, and removed the actual errata text changes from this document. (JVET-U0049)

# General aspects and issues affecting multiple standards

## General and minor matters

Please note that the alignment between the text published by ITU-T and that published by ISO/IEC should also be checked. Minor editorial issues and aspects that are highlighted for potential further checking include the following:

- (for Video CICP only, in the ISO/IEC version only) Logical indentation nesting in the ISO/IEC version of the Video CICP colour interpretation was inadvertently removed in the publication process.

- (for Video CICP only) The range of values for the extended aspect ratio indicator in Video CICP is not clearly specified and may implicitly be interpreted as inadequate to cover the range of values expressed in the video coding standards.

# Reported errata items for VVC

## General

Both the ITU-T second edition text and the ISO/IEC second edition text have been published. The third edition for both ITU-T and ISO/IEC are in the approval and/or publication process. Besides those errata items directly included below in this Section, JVET-AE2005 may include some VVC errata changes (in addition to some amendments).

## Publication status background

Rec. ITU-T H.266 (https://www.itu.int/rec/T-REC-H.266/en)

- (04/2022, version 2) Approved 2022-04-29, published 2022-07-12

ISO/IEC 23090-3 (https://www.iso.org/standard/83531.html)

- ISO/IEC 23090-3:2022 (Edition 2); published 2022-09

Rec. ITU-T H.266 (https://www.itu.int/rec/T-REC-H.266/en)

- (08/2020, version 1) Approved 2020-08-29, published 2020-11-10

ISO/IEC 23090-3 (https://www.iso.org/standard/73022.html)

- ISO/IEC 23090-3:2021 (Edition 1); published 2021-02

## Errata items for VVC

- In subclause 7.3.11.10 (Transform unit syntax), there is an instance of “if(IntraSubPartitionsSplitType  !=  ISP_NO_SPLIT )”, where a space is missing after ‘(’.

- In subclause 7.4.2.4.5, the first bullet item, replace "subpicture level values" with "subpicture index values".

- In D.6.1, change the coding of sn_zero_bit from u(1) to f(1). It would be better to consistently use f(1) for all fix-valued, non-reserved bits. The same approach could apply for all fix-valued, non-reserved bytes, two-bytes, etc.

- Replace the two instances of "in the first access unit of the CVS" and the three instances of "in the first AU of the CVS" with "in the first AU of the CVS in decoding order". (JVET-AF0189)

## Errata items for VVC conformance

None.

# Reported errata items for VSEI

## General

Both the ITU-T second edition text and the ISO/IEC second edition text have been published. The third edition for both ITU-T and ISO/IEC are in the approval and/or publication process. JVET-AE2006 may include some VSEI errata changes (in addition to some amendments).

## Publication status background

Rec. ITU-T H.274 (https://www.itu.int/rec/T-REC-H.274/en)

- (05/2022, version 2) Approved 2022-05-22, published 2022-07-25

ISO/IEC 23002-7 (https://www.iso.org/standard/83530.html)

- ISO/IEC 23002-7:2022 (Edition 2); published 2022-10

Rec. ITU-T H.274 (https://www.itu.int/rec/T-REC-H.274/en)

- (08/2020, version 1) Approved 2020-08-29, published 2020-11-10

ISO/IEC 23002-7 (https://www.iso.org/standard/79112.html)

- ISO/IEC 23002-7:2021 (Edition 1); published 2021-01

## Errata items for VSEI

- In subclause 7.3, semantics of vui_chroma_sample_loc_type_frame, vui_chroma_sample_loc_type_top_field, and vui_chroma_sample_loc_type_bottom_field, replace "the syntax elements chroma_sample_loc_type," with "the syntax elements vui_chroma_sample_loc_type_frame,".

- In subclause 8.22.2 (Depth representation information SEI message semantics) and 8.23.2 (Alpha channel information SEI message semantics), replace each of the 10 instances of "nuh_layer_id" with "layer identification".

- In subclause 8.26.2, replace the following:

–	A picture in the current layer in an AU associated with a CTI SEI message is output that follows the current picture in output order.

with the following:

–	A picture in the current layer in an AU associated with a CTI SEI message with the same value of colour_transform_id and applicable to the current layer is output that follows the current picture in output order.

- In subclause 8.28.2.1, change the coding of nnpfc_alignment_zero_bit_a and nnpfc_alignment_zero_bit_b from u(1) to f(1). It would be better to consistently use f(1) for all fix-valued, non-reserved bits. The same approach could apply for all fix-valued, non-reserved bytes, two-bytes, etc.

- In subclause 8.28.2.2, semantics of nnpfc_matrix_coeffs, make the following changes:

When nnpfc_matrix_coeffs is not present in the NNPFC SEI message, the value of nnpfc_matrix_coeffs is inferred to be equal to vui_matrix_coeffs.

nnpfc_matrix_coeffs shall not be equal to 0 unless both of the following conditions are true:

–	Both nnpfc_out_tensor_chroma_bitdepth_minus8 and nnpfc_out_tensor_luma_bitdepth_minus8 are present, and nnpfc_out_tensor_chroma_bitdepth_minus8 is equal to nnpfc_out_tensor_luma_bitdepth_minus8.

–	Both nnpfc_out_tensor_chroma_bitdepth_minus8 and nnpfc_out_tensor_luma_bitdepth_minus8 are present, and nnpfc_out_order_idc is equal to 2, outSubHeightC is equal to 1, and outSubWidthC is equal to 1.

NOTE: When both of the above two conditions are true and vui_matrix_coeffs is equal to 0, nnpfc_matrix_coeffs is expected to be present.

nnpfc_matrix_coeffs shall not be equal to 8 unless one of the following conditions is true:

–	Both nnpfc_out_tensor_chroma_bitdepth_minus8 and nnpfc_out_tensor_luma_bitdepth_minus8 are present; and nnpfc_out_tensor_chroma_bitdepth_minus8 is equal to nnpfc_out_tensor_luma_bitdepth_minus8.

–	Both nnpfc_out_tensor_chroma_bitdepth_minus8 and nnpfc_out_tensor_luma_bitdepth_minus8 are present; and nnpfc_out_tensor_chroma_bitdepth_minus8 is equal to nnpfc_out_tensor_luma_bitdepth_minus8 + 1, nnpfc_out_order_idc is equal to 2, outSubHeightC is equal to 1, and outSubWidthC is equal to 1.

NOTE: When one of the above two conditions is true and vui_matrix_coeffs is equal to 8, nnpfc_matrix_coeffs is expected to be present.

- In subclause 8.28.2.2, semantics of nnpfc_full_range_flag, make the following changes:

nnpfc_full_range_flag indicates the scaling and offset values applied in association with the matrix coefficients as specified by nnpfc_matrix_coeffs. Its semantics are as specified for the VideoFullRangeFlag parameter in Rec. ITU-T H.273 | ISO/IEC 23091-2. When nnpfc_full_range_flag is not present and nnpfc_out_format_idc is equal to 1, the value of nnpfc_full_range_flag is inferred to be equal to vui_full_range_flag 0.

- Consider adding a NOTE in VSEI telling something like: 1) AVC can reference VSEI for enabling the inclusion of only those SEI messages for which the SEI payloads are specified in VSEI but the syntax has not been extended; 2) HEVC and VVC can reference VSEI for enabling the inclusion of SEI messages for which the SEI payloads are specified in VSEI regardless of whether the syntax has been extended or not.

# Reported errata items for HEVC

## General

Besides those errata items directly included below in this Section, JVET-AF1006 also includes some HEVC errata changes (in addition to some amendments).

## Publication status background

Rec. ITU-T H.265

- (09/2023, version 9) Approved 2023-09-13, published 2023-11-24

- (08/2021, version 8) Approved 2021-08-22, published 2021-10-13

- (10/2019, version 7) Approved 2019-11-29, published 2020-01-10

- (06/2019, version 6) Approved 2019-06-29, published 2019-09-23

- (02/2018, version 5) Approved 2018-02-13, published 2018-05-11

ISO/IEC 23008-2

- ISO/IEC 23008-2:2023 (Edition 5); published 2023-10

- ISO/IEC 23008-2:2020 (Edition 4); published 2020-08

- ISO/IEC 23008-2:2017 (Edition 3); published 2017-10

- ISO/IEC 23008-2:2017/Amd 1:2018 (Additional colour representation code point); published 2018-03

- ISO/IEC 23008-2:2017/Amd 2:2018 (Main 10 still picture profile); published 2018-03

- ISO/IEC 23008-2:2017/Amd 3:2018 (Additional supplemental enhancement information); published 2018-07

- Work items merged for FDIS ballot as publication office action; Stage 50.00 Final text received or FDIS registered for formal approval as of 2020-03-19

  - ISO/IEC DIS 23008-2:201x (Edition 4); DIS ballot closed 2018-10-06; stage 40.99, Full report circulated: DIS approved for registration as FDIS 2019-02-18; Pending FDIS ballot based on WG 11 N 18277

  - ISO/IEC 23008-2:201x (Edition 4)/DAmd 1:201x (Additional supplemental enhancement information); DAM ballot started 2019-07-10, closed 2019-10-02

## Errata items for HEVC

### Items for both the ITU-T text and the ISO text

- (Perhaps for other specs such as AVC, VVC, and VSEI as well) The text uses the phrase “true subset” (in two places) without explaining what that means. That doesn’t seem to be a well-known term. The more well-known term is “proper subset” or “strict subset”.

- (Perhaps for other specs such as AVC, VVC, and VSEI as well) The text uses “Y = !X”, with the understanding that when X is 0, Y will be 1. But it is not directly stated anywhere that !0 will be 1. The text only seems to clearly say that !0 will be TRUE, and TRUE is anything that is not equal to 0.

The fundamental problem here is that the result of !X is a Boolean value, not a number. We should not be trying to interpret it as a number. If we want our input to be a number and our output to be a number, we should simply replace the uses of Y = !X with an operation that produces a numerical result instead of a Boolean result. Instead of Y = !X, we should use Y = X − 1. We should also never interpret a number as a Boolean value. If X is a number, we should never use it as an input to “!” or “&&” or “| |”. A numerical value should only be used as an input to mathematical operators or value comparison operators (i.e., “>”, “<”, “==”, “>=” or “<=”). If n is a numerical variable, we should never use “!n” or “n && m”. Instead we should use expressions like “n != 0” to convert the number to a Boolean first. In principle we should be able to change “if x is TRUE or not equal to 0” simply with “if x is TRUE” and it would not affect anything.

- Replace the three instances of "in the first access unit of the CVS" with "in the first access unit of the CVS in decoding order". (JVET-AF0189)

### Items for the ISO text only

- In Table A.8, replace "Max slice segments per picture MaxSliceSegmentsPerPicture" with "Max # of slice segments per picture MaxSliceSegmentsPerPicture".

- In Formula C-24, change “InitCpbRemovalDelayOffset [ SchedSelIdx ]” to “InitCpbRemovalDelay‌Offset[ SchedSelIdx ]” (remove a space just before ‘[’), and change “BitRate [ SchedSelIdx − 1 ]” to “BitRate[ SchedSelIdx − 1 ]” (again (remove a space just before ‘[’)].

- There are two instances of “should” in NOTE 4 in subclause D.3.9.

- In D.3.14, change “the hint values specify a cross-correlation matrix” to “filter hint values are provided, and the provided filter hint values specify a cross-correlation matrix”.

### Items on the semantics of post-filter hint SEI message from JVET-AE0155

- It does not seem clear whether the “16-bit precision” description applies only to the cross-correlation matrix case or also to the filter coefficient cases.

- A specified 32-bit range may be excessive for a syntax element that has 16-bit precision.

- Much of the text is written from the encoder perspective. We generally do not specify things from the encoder perspective. The decoder-side processing is not clearly described or is discussed only vaguely. For example, no equations are given for the decoder’s filtering process.

- It does not seem clear whether the decoder should divide the filter tap values (or equivalently, the result of the filtering process) by the sum of the filter tap values or if the encoder is responsible for the filter normalization.

- There are issues relating to how to interpret the filter depending on whether the filter length is even (in which case the output samples may presumably be located spatially between input samples) or odd (in which case the location of an output sample may presumably have a location corresponding to the centre tap).

- The handling of the boundaries of the picture and cropping window are not described and seems unclear.

- It seems unclear why should it matter whether a horizontal filter or a vertical filter is applied first if both are to be applied. Perhaps integer rounding is expected to be applied between the two filtering operations, but this is not stated.

- The discussion of Wiener post-filtering is a bit vague (e.g., what exactly is a Wiener filter, and how would a decoder derive one from the provided data?).

- A decoder could use a subset of a provided cross-correlation matrix to derive a Wiener post-filter with a smaller region of support than the provided statistics would enable. It does not seem clear whether this is intended to be an acceptable or possible practice.

# Reported errata items for AVC

## General

See Subsection 5.3 below. JVET-AF1016 also includes some AVC errata changes (in addition to some amendments).

## Publication status background

Rec. ITU-T H.264

- (08/2021, version 14) Approved 2021-08-22, published 2021-10-13

- (06/2019, version 13) Approved 2019-06-13, published 2019-09-06

ISO/IEC 14496-10

- ISO/IEC 14496-10:2022 (Edition 10), published 2022-11 (https://www.iso.org/standard/83529.html)

- ISO/IEC 14496-10:2020 (Edition 9), published 2020-12

- ISO/IEC 14496-10:2014 (Edition 8), published 2014-09

- ISO/IEC 14496-10:2014/Amd 1:2015 (Multi-resolution frame compatible stereoscopic video with depth maps, additional supplemental enhancement information and video usability information), published 2015-11

- ISO/IEC 14496-10:2014/FDAMD 2 (Additional Levels and Supplemental Enhancement Information); stage 50.98, deleted in preparation for Edition 9

- ISO/IEC 14496-10:2014/Amd 3:2016 (Additional supplemental enhancement information); published 2016-12, published 2016-12

- ISO/IEC DIS 14496-10:201x (Edition 9); stage 40.99 Full report circulated: DIS approved for registration as FDIS as of 2018-01-31

## Errata items

- Replace the 15 instances of "in the first access unit of the coded video sequence" with "in the first access unit of the coded video sequence in decoding order". (JVET-AF0189)

- All items in Subsection 4.3.3 (Items on the semantics of post-filter hint SEI message from JVET-AE0155).

# Reported errata items for Video CICP

## General

See also Section 1.

## Publication status background

Rec. ITU-T H.273

- (07/21, version 2) Approved 2021-07-14, published 2021-09-24

- (12/16, version 1) Approved 2016-12-22, published 2017-04-27

ISO/IEC 23091-2 (previously part of ISO/IEC 23001-8)

- ISO/IEC 23091-2:2019, published 2019-07

## Errata items

- There is an error in the range of values for the sample aspect ratio width and height.

- The CICP revision should account for the various errata issues previously noted in AL1004. It was confirmed that the BT.2100 reference needs updating (to -2).

- The equations for ICtCp with HLG have not been updated and need be corrected.