| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0006-v1 |
| --- | --- |

| Title: | JVET AHG report: ECM software development (AHG6) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jie Chen Roman Chernyak Fabrice Le Léannec Kai Zhang | Tel: Email: | vseregin@qti.qualcomm.com jiechen.cj@alibaba-inc.com chernyak@global.tencent.com Fabrice.Leleannec@InterDigital.com zhangkai.video@bytedance.com |
| Source: | AHG6 on ECM software development |  |  |

_____________________________

# Abstract

This report summarizes the activities of the AHG6 on ECM software development that has taken place between the 34th and 35th JVET meetings.

# Introduction

The mandates for the AHG are as follows:

- Coordinate development of the ECM software and associated configuration files.

- Produce documentation of software usage for distribution with the software.

- Prepare and deliver ECM-13.0 software version (and potential updates) and the reference configuration encodings according to the ECM common test conditions.

- Investigate encoder speedup and other software optimization such as reduction of memory consumption.

- Coordinate with ECM algorithm description editors to identify any mismatches between software and text, make further updates and cleanups to the software as appropriate.

# Software development

ECM software repository is located at https://vcgit.hhi.fraunhofer.de/ecm/ECM.

ECM software is based on VTM-10.0 with enabled MCTF including the update from JVET-V0056, and GOP32, which is very close to VTM-11.0.

VTM-11.0ecm anchor https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/VTM11_ANC is used for ECM performance evaluation.

The following changes were integrated into ECM-13.0:

JVET-AH0223: Retrained contexts (MR 646)

JVET-AH0176: Context initialization for low delay B-slice (Test 6.1a) (MR 652)

JVET-AH0069 (Test3.1a): Chained MVP (MR 647)

JVET-AH0065 (test 2.7): Relaxing line buffer restriction (MR 649)

JVET-AH0068 (test 3.2a): Removing the one-CTU-row temporal buffer constraint for all relevant tools (MR 650)

JVET-AH0086: EIP with bias and clipping (MR 656)

JVET-AH0076 (Test 2.2): Occurrence-based intra coding (MR 648)

JVET-AH0314 EE2-3.5d: Combination of EE2-3.4 (JVET-AH0073) and EE2-3.5c (JVET-AH0186) (MR 653)

JVET-AH0066 & JVET-AH0202: Inter CCP merge with zero luma CBF (MR 651)

JVET-AH0101 (EE2-4.2: Fix on LFNST/NSPT index signalling for inter coding) (MR 661)

JVET-AH0057(Test 5.1a): Adaptive precision for CCALF coefficients (MR 659)

JVET-AH0068 (Test 3.2a): Removing the one-CTU-row temporal buffer constraint (MR 650)

JVET-AH0136: Intra chroma mode reordering (MR 658)

JVET-AH0200: Intra TMP BV reordering, JVET-AH0055: AR-BVP for intra TMP merge candidates (MR 657)

JVET-AH0103: Low-delay configurations for LFNST/NSPT (MR 663)

JVET-AH0119: Subblock merge mode improvements (Test 3.7) (MR 664)

JVET-AH0135: Temporal partitioning prediction (Test 1.1) (MR 660)

JVET-AH0209: Matrix based position dependent intra prediction replacing conventional intra modes (MR 670)

Bug fixes:

Fix missing xCalcDebCost (MR 634)

Optimization to reduce amount of array allocations (MR 635)

Fix SPS::m_itmpLicMode not being initialized at decoder if IBC is OFF (MR 636)

JVET-AG0276 fixes (MR 637)

Fix JVET_AG0098 uninitialized value (MR 638)

Compiler warning fixes for MacOS 14.4.1 (MR 643)

Add auto-build setting for macOS on ARM build server (MR 644)

Fix: Remove extra lines for LicDelta context (MR 639)

Fix the condition to create original residual buffer (MR 641)

Fix JVET_AG0061_INTER_LFNST_NSPT by checking number of non-zero coefficients when NumSignPred=0 (MR 640)

Fix Valgrind issues (MR 642)

Fix precompile ifs for disabled MC reference pic padding (MR 655)

The following changes were integrated into VTM-11ecm anchor:

JVET-AH0171: GOP-based RPR encoder control (MR 665)

JVET-AH0180: CTC low delay config change (MR 668)

Compiler warning fixes for MacOS 14.4.1 (MR 645)

ECM-13.0, and VTM-11ecm13.0 were tagged on June 3, 2024.

## CTC Performance

In this section, ECM test results following ECM CTC configuration descried in JVET-AF2017 are summarized.

ECM-13.0 performance over ECM-12.0 anchor is summarized in the tables below.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -1.90% | 13.97% | 9.47% | 104.1% | 105.7% | 100.8% | 102.2% |
| Class A2 | -2.84% | 13.16% | 12.12% | 101.9% | 106.4% | 100.8% | 102.4% |
| Class B | -1.53% | 12.77% | 12.75% | 104.4% | 102.4% | 103.4% | 103.3% |
| Class C | -1.60% | 9.98% | 9.94% | 105.4% | 106.7% | 102.2% | 103.6% |
| Class E | -1.55% | 9.83% | 10.04% | 104.5% | 103.2% | 101.8% | 103.4% |
| Overall | -1.83% | 11.92% | 11.02% | 104.2% | 104.7% | 102.0% | 103.1% |
| Class D | -1.47% | 9.71% | 9.83% | 104.9% | 111.3% | 102.8% | 103.8% |
| Class F | -1.85% | 7.40% | 7.20% | 96.3% | 90.0% | 101.7% | 103.4% |
| Class TGM | -2.12% | 2.24% | 1.96% | 91.3% | 79.5% | 101.2% | 103.1% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -2.87% | 11.44% | 7.31% | 92.3% | 105.9% | 100.6% | 102.0% |
| Class A2 | -2.55% | 9.65% | 8.60% | 94.0% | 105.4% | 100.5% | 101.7% |
| Class B | -1.43% | 12.69% | 12.62% | 90.4% | 100.9% | 101.1% | 102.9% |
| Class C | -1.57% | 11.21% | 10.77% | 95.6% | 106.8% | 101.4% | 103.5% |
| Class E |  |  |  |  |  |  |  |
| Overall | -1.98% | 11.44% | 10.26% | 92.9% | 104.4% | 101.0% | 102.6% |
| Class D | -1.38% | 11.48% | 10.82% | 93.2% | 108.0% | 102.2% | 103.7% |
| Class F | -1.95% | 8.10% | 8.30% | 96.9% | 86.2% | 101.4% | 102.9% |
| Class TGM | -4.01% | -1.89% | -1.86% | 96.8% | 80.3% | 100.1% | 101.8% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -2.12% | 16.88% | 16.41% | 94.3% | 107.0% | 102.4% | 96.0% |
| Class C | -2.98% | 13.63% | 13.57% | 98.2% | 104.6% | 102.1% | 101.2% |
| Class E | -2.71% | 16.26% | 13.84% | 99.6% | 108.8% | 107.0% | 100.9% |
| Overall | -2.56% | 15.64% | 14.82% | 96.9% | 106.7% | 103.5% | 98.9% |
| Class D | -2.10% | 15.70% | 14.90% | 98.9% | 110.0% | 103.9% | 103.3% |
| Class F | -3.04% | 6.64% | 5.95% | 96.2% | 83.9% | 104.3% | 99.1% |
| Class TGM | -3.05% | -3.88% | -4.21% | 95.2% | 81.8% | 102.0% | 100.1% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -1.44% | -1.95% | -1.74% | 94.1% | 101.7% | 101.3% | 102.8% |
| Class C | -2.17% | -2.07% | -1.60% | 101.7% | 108.8% | 101.9% | 103.6% |
| Class E | -2.21% | -1.63% | -2.05% | 100.7% | 107.5% | 100.7% | 103.0% |
| Overall | -1.88% | -1.91% | -1.77% | 98.2% | 105.4% | 101.3% | 103.1% |
| Class D | -1.79% | -3.59% | -1.39% | 100.3% | 107.6% | 102.9% | 103.8% |
| Class F | -2.09% | -2.10% | -1.90% | 98.2% | 81.2% | 102.7% | 103.0% |
| Class TGM | -5.11% | -11.21% | -11.13% | 95.9% | 83.3% | 99.6% | 102.6% |

The below tables show ECM-13.0 performance comparing to VTM-11.0ecm13.0 anchor.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0ecm13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -13.18% | -13.89% | -24.88% | 980.4% | 467.2% | #DIV/0! | #DIV/0! |
| Class A2 | -20.15% | -22.13% | -26.46% | 984.0% | 487.8% | #DIV/0! | #DIV/0! |
| Class B | -13.54% | -20.05% | -18.04% | 920.7% | 475.4% | #DIV/0! | #DIV/0! |
| Class C | -13.58% | -10.35% | -11.48% | 937.0% | 437.6% | #DIV/0! | #DIV/0! |
| Class E | -17.30% | -20.19% | -17.25% | 898.4% | 497.5% | #DIV/0! | #DIV/0! |
| Overall | -15.22% | -17.24% | -18.99% | 940.6% | 470.9% | #DIV/0! | #DIV/0! |
| Class D | -11.47% | -7.48% | -7.91% | 920.9% | 477.3% | #DIV/0! | #DIV/0! |
| Class F | -29.12% | -32.42% | -32.73% | 571.1% | 498.7% | #DIV/0! | #DIV/0! |
| Class TGM | -42.06% | -47.48% | -46.92% | 459.9% | 468.9% | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over VTM-11.0ecm13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -26.04% | -21.59% | -33.88% | 946.3% | 892.5% | #DIV/0! | #DIV/0! |
| Class A2 | -29.16% | -31.81% | -36.71% | 887.1% | 1036.9% | #DIV/0! | #DIV/0! |
| Class B | -23.43% | -28.98% | -26.76% | 747.6% | 846.1% | #DIV/0! | #DIV/0! |
| Class C | -24.99% | -20.48% | -21.31% | 821.5% | 922.3% | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |  |  |
| Overall | -25.51% | -25.80% | -28.72% | 831.6% | 911.4% | #DIV/0! | #DIV/0! |
| Class D | -25.69% | -21.07% | -22.42% | 785.7% | 1008.7% | #DIV/0! | #DIV/0! |
| Class F | -31.65% | -34.02% | -34.69% | 660.1% | 557.6% | #DIV/0! | #DIV/0! |
| Class TGM | -40.63% | -45.97% | -46.06% | 634.7% | 487.9% | #DIV/0! | #DIV/0! |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over VTM-11.0ecm13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -20.42% | -32.78% | -30.25% | 739.1% | 718.3% | #DIV/0! | #DIV/0! |
| Class C | -22.71% | -23.31% | -24.26% | 755.1% | 802.1% | #DIV/0! | #DIV/0! |
| Class E | -20.11% | -22.85% | -22.09% | 694.2% | 500.6% | #DIV/0! | #DIV/0! |
| Overall | -21.11% | -27.14% | -26.21% | 732.8% | 680.9% | #DIV/0! | #DIV/0! |
| Class D | -23.81% | -25.09% | -24.57% | 745.0% | 892.1% | #DIV/0! | #DIV/0! |
| Class F | -28.69% | -36.15% | -36.26% | 627.5% | 550.3% | #DIV/0! | #DIV/0! |
| Class TGM | -38.57% | -47.78% | -48.13% | 584.2% | 493.6% | #DIV/0! | #DIV/0! |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over VTM-11.0ecm13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -18.48% | -42.55% | -40.42% | 708.1% | 736.2% | #DIV/0! | #DIV/0! |
| Class C | -21.06% | -33.00% | -33.69% | 667.2% | 804.3% | #DIV/0! | #DIV/0! |
| Class E | -18.63% | -33.10% | -32.59% | 665.9% | 536.3% | #DIV/0! | #DIV/0! |
| Overall | -19.38% | -37.00% | -36.22% | 683.6% | 700.5% | #DIV/0! | #DIV/0! |
| Class D | -23.34% | -35.68% | -35.92% | 653.1% | 852.0% | #DIV/0! | #DIV/0! |
| Class F | -26.75% | -41.58% | -42.03% | 627.7% | 538.5% | #DIV/0! | #DIV/0! |
| Class TGM | -37.18% | -50.29% | -50.22% | 638.3% | 499.6% | #DIV/0! | #DIV/0! |

# ECM memory consumption

ECM encoder memory consumption (VmPeak, GiB) is provided in ECM encoder log files and is summarized in the table below as maximum class-wise consumption rounded up to GiB.

|  | AI | RA | LB |
| --- | --- | --- | --- |
| class A (A1& A2) | 7 | 15 |  |
| class B | 4 | 5 | 4 |
| class C | 2 | 2 | 2 |
| class D | 2 | 2 | 2 |
| class E | 2 |  | 3 |
| class F | 4 | 5 | 4 |
| class TGM | 4 | 8 | 6 |

It is encouraged to care about memory allocation when developing and integrating tools into ECM. In particular, it is strongly recommended to re-use already existed memory wherever possible, rather than systematically allocating new memory, and allocate only the required amount of memory if new memory is needed.

# Recommendations

The AHG recommends to:

- Continue to develop ECM software.

- Improve the software documentation.

- Encourage people to report all (potential) bugs that they are finding using GitLab Issues functionality https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/issues.

- Encourage people to submit merge requests fixing identified bugs.

- Encourage people to continue working on ECM memory consumption reduction.

- Encourage people to continue working on speeding up ECM encoder to reduce the simulation time.