| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0051-v4 |
| --- | --- |

| Title: | Template Matching Picture Boundary Padding for ECM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nicolas Neumann, Mathias Wien | Tel: Email: | Nicolas.Neumann@lfb.rwth-aachen.de Mathias.Wien@lfb.rwth-aachen.de |
| Source: | RWTH Aachen University |  |  |

_____________________________

# Abstract

This contribution proposes template matching padding for reference picture boundary handling. Template matching structure prediction is used to generate the outside samples of the reference picture from content inside the reference picture. For this purpose, the outside area is partitioned into so-called target blocks. Then a template matching search is performed for each target block and the search results are used to fill the target block. The method can be used as a replacement for motion compensated padding (JVET-AA0096) or can be combined with this method. The combination first applies motion compensated padding and then fills the remaining area with template matching boundary padding instead of repetitive padding. The method is described in JVET-AH0168 in detail. The document provides results for ECM 12.1 as the anchor.

The proposed method MC-TM-Padding shows improvements over ECM-12.1 for the Random Access, Low Delay B and Low Delay P configuration. Further the TM-Padding method shows improvements over ECM-12.1 in the Low Delay B and Low Delay P configuration with and average BDR-Y improvement of -0.02 and -0.04 respectively. For the Low Delay B configuration, we can observe a decoder runtime decrease of -0.02% on average.

# Introduction

In VTM repetitive padding is used for reference picture padding. For ECM motion compensated padding [1] was proposed which yields improved compression efficiency. Motion compensated padding (MC-Padding, JVET-AA0096) uses content from other reference pictures to pad the current reference picture for applicable areas. The remaining outside area is still padded with repetitive padding.

The proposed method describes a reference picture padding algorithm, which is applicable for the whole outside area of the reference picture and can be combined with motion compensated padding to substitute the repetitive padding.

The Method is described in JVET-AH0168 in detail. In the following results will be provided for:

- Repetitive padding (Rep-Padding: ECM with MC-Padding disabled)

- Template Matching Padding (TM-Padding: Template Matching instead of TM-Padding)

- Combination Padding (MC-TM-Padding: Combination of MC and TM-Padding)

ECM 12.1 is used as the anchor for all the simulations.

# Results

In the following, the Results for Random Access, Low Delay B and Low Delay P will be presented. Rep-Padding serves as a reference for how much MC-Padding gains. Results are obtained by deactivating MC-Padding (JVET_AA0096_MC_BOUNDARY_PADDING = 0). TM-Padding and MC-TM-Padding are the two proposed methods. We only report decoder runtimes due to a heterogenous cluster, that we are running the encodings on. The decoder runtimes are measures on a single machine. At this point in time, we are only able to report partial results. We will update the document as soon as the simulations are finished. When we mention MC-Padding in the following, we are referring to the anchor (ECM 12.1) and the method used in the anchor (JVET-AA0096). JVET-AF2017 Common test conditions and evaluation procedures for enhanced compression tool testing have been followed, to generate the results.

## Random Access Configuration

|  | Rep-Padding |  |  | TM-Padding |  |  | MC-TM-Padding |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class A1 | BDR-Y | BDR-U | BDR-V | BDR-Y | BDR-U | BDR-V | BDR-Y | BDR-U | BDR-V |
| Tango2 | 0.05 | 0.09 | 0.18 | -0.04 | -0.35 | 0.35 |  |  |  |
| FoodMarket4 | 0.25 | 0.01 | 0.22 | 0.12 | -0.15 | 0.06 |  |  |  |
| Campfire |  |  |  | 0.01 | 0.01 | -0.09 |  |  |  |
| average |  |  |  | 0.03 | -0.16 | 0.11 |  |  |  |
| dec. runtime increase |  |  |  | 100.7% |  |  |  |  |  |
|  | Rep-Padding |  |  | TM-Padding |  |  | MC-TM-Padding |  |  |
| Class A2 | BDR-Y | BDR-U | BDR-V | BDR-Y | BDR-U | BDR-V | BDR-Y | BDR-U | BDR-V |
| CatRobot | -0.01 | -0.02 | 0.00 | 0.02 | -0.06 | 0.06 |  |  |  |
| DaylightRoad2 | 0.21 | 0.39 | 0.41 | 0.13 | 0.49 | 0.16 |  |  |  |
| ParkRunning3 |  |  |  |  |  |  |  |  |  |
| average |  |  |  |  |  |  |  |  |  |
| dec. runtime increase |  |  |  |  |  |  |  |  |  |
|  | Rep-Padding |  |  | TM-Padding |  |  | MC-TM-Padding |  |  |
| Class B | BDR-Y | BDR-U | BDR-V | BDR-Y | BDR-U | BDR-V | BDR-Y | BDR-U | BDR-V |
| MarketPlace | 0.14 | 0.07 | 0.09 | 0.08 | 0.00 | -0.09 | 0.00 | -0.09 | 0.07 |
| RitualDance | 0.09 | 0.07 | -0.09 | 0.07 | 0.25 | 0.12 | -0.01 | 0.28 | -0.01 |
| Cactus | 0.13 | 0.01 | 0.02 | 0.02 | 0.08 | -0.13 | 0.00 | 0.14 | -0.09 |
| BasketballDrive | 0.07 | 0.29 | 0.19 | 0.06 | -0.04 | 0.24 | 0.04 | -0.16 | 0.02 |
| BQTerrace | 0.20 | 0.56 | 0.23 | 0.21 | 0.12 | 0.23 | 0.03 | 0.06 | -0.04 |
| average | 0.13 | 0.20 | 0.09 | 0.09 | 0.08 | 0.07 | 0.01 | 0.05 | -0.01 |
| dec. runtime increase |  | 100.9% |  |  | 100.0% |  | 99.5% (100.0%) |  |  |
|  | Rep-Padding |  |  | TM-Padding |  |  | MC-TM-Padding |  |  |
| Class C | BDR-Y | BDR-U | BDR-V | BDR-Y | BDR-U | BDR-V | BDR-Y | BDR-U | BDR-V |
| BasketballDrill | 0.01 | -0.12 | -0.06 | -0.08 | -0.23 | -0.13 | -0.09 | -0.03 | -0.15 |
| BQMall | 0.18 | 0.27 | 0.26 | 0.18 | 0.17 | 0.19 | 0.04 | -0.25 | -0.12 |
| PartyScene | 0.29 | 0.13 | 0.21 | 0.22 | -0.02 | 0.00 | -0.02 | -0.13 | -0.13 |
| RaceHorses | 0.15 | 0.34 | 0.22 | 0.10 | 0.25 | 0.09 | -0.07 | 0.21 | 0.28 |
| average | 0.16 | 0.15 | 0.16 | 0.10 | 0.04 | 0.04 | -0.03 | -0.05 | -0.03 |
| dec. runtime increase | 99.7% |  |  | 99.1% |  |  | 100.9% (99.9%) |  |  |
|  | Rep-Padding |  |  | TM-Padding |  |  | MC-TM-Padding |  |  |
| Class D | BDR-Y | BDR-U | BDR-V | BDR-Y | BDR-U | BDR-V | BDR-Y | BDR-U | BDR-V |
| BasketballPass | 0.11 | 0.42 | -0.04 | -0.04 | -0.21 | 0.00 | -0.08 | -0.13 | 0.15 |
| BQSquare | 0.81 | 0.79 | 0.54 | 0.71 | 0.52 | 0.55 | -0.03 | -0.13 | -0.22 |
| BlowingBubbles | 0.29 | 0.07 | 0.17 | 0.34 | -0.06 | 0.18 | 0.06 | -0.20 | 0.13 |
| RaceHorses | 0.20 | 0.11 | 0.49 | 0.12 | -0.11 | -0.02 | -0.08 | 0.38 | 0.70 |
| average | 0.35 | 0.35 | 0.29 | 0.28 | 0.03 | 0.18 | -0.03 | -0.02 | 0.19 |
| dec. runtime increase | 98.8% |  |  | 99.4% |  |  | 101.5% (100.0%) |  |  |
|  | Rep-Padding |  |  | TM-Padding |  |  | MC-TM-Padding |  |  |
| Class F | BDR-Y | BDR-U | BDR-V | BDR-Y | BDR-U | BDR-V | BDR-Y | BDR-U | BDR-V |
| BasketballDrillText | 0.01 | 0.36 | -0.07 | -0.06 | 0.20 | -0.23 | -0.04 | 0.28 | -0.15 |
| ArenaOfValor | 0.08 | 0.04 | -0.06 | 0.04 | -0.06 | -0.07 | 0.02 | -0.16 | -0.05 |
| SlideEditing | 0.00 | -0.01 | 0.04 | 0.02 | -0.01 | -0.02 | 0.03 | -0.02 | 0.00 |
| SlideShow | 0.00 | 0.12 | 0.49 | 0.22 | 0.37 | 0.57 | -0.02 | -0.12 | -0.01 |
| average | 0.02 | 0.13 | 0.10 | 0.06 | 0.12 | 0.06 | -0.01 | 0.00 | -0.05 |
| dec. runtime increase | 97.6% |  |  | 97.6% |  |  | 100.6% (98.9%) |  |  |

For Random Access, MC-TM-Padding performs better than MC-Padding. The MC-TM-Padding method, improves the compression performance (-0.03% BDR-Y for class C and D), with a decoder runtime increase of 100.9% for class C and 100.4% for class D. If the (unnecessary) padding of the refence pictures with the highest temporal ID is omitted, the decoder runtime is reduced to 99.9% and 100.0%, respectively (numbers in brackets).

## Low Delay B

|  | Rep-Padding |  |  | TM-Padding |  |  | MC-TM-Padding |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class B | BDR-Y | BDR-U | BDR-V | BDR-Y | BDR-U | BDR-V | BDR-Y | BDR-U | BDR-V |
| MarketPlace | -0.03 | 0.61 | -0.32 | -0.10 | 0.47 | 0.11 |  |  |  |
| RitualDance | 0.09 | 0.14 | -0.06 | 0.05 | 0.21 | -0.05 |  |  |  |
| Cactus | 0.04 | 0.08 | 0.57 | -0.06 | -0.26 | 0.60 |  |  |  |
| BasketballDrive | 0.01 | -0.35 | 0.79 | -0.01 | -0.37 | 0.26 |  |  |  |
| BQTerrace | 0.17 | -0.30 | -0.65 | -0.01 | 1.26 | 0.67 |  |  |  |
| average | 0.06 | 0.04 | 0.07 | -0.03 | 0.26 | 0.32 |  |  |  |
| dec. runtime increase | 99.9% |  |  | 100.2% |  |  |  |  |  |
|  | Rep-Padding |  |  | TM-Padding |  |  | MC-TM-Padding |  |  |
| Class C | BDR-Y | BDR-U | BDR-V | BDR-Y | BDR-U | BDR-V | BDR-Y | BDR-U | BDR-V |
| BasketballDrill | -0.10 | 0.56 | 0.30 | -0.07 | -0.16 | -0.01 | -0.06 | 0.14 | -0.05 |
| BQMall | 0.08 | 0.02 | -0.20 | -0.07 | -1.02 | 0.10 | -0.03 | 0.63 | -0.15 |
| PartyScene | 0.19 | -0.65 | 0.24 | 0.16 | -0.54 | 0.12 | 0.09 | -0.92 | -0.46 |
| RaceHorses | 0.11 | -0.04 | 0.20 | -0.05 | 0.31 | 0.00 | -0.07 | 0.31 | -0.01 |
| average | 0.07 | -0.03 | 0.13 | -0.01 | -0.35 | 0.05 | -0.02 | 0.04 | -0.17 |
| dec. runtime increase | 99.5% |  |  | 99.5% |  |  | 100.9% |  |  |
|  | Rep-Padding |  |  | TM-Padding |  |  | MC-TM-Padding |  |  |
| Class D | BDR-Y | BDR-U | BDR-V | BDR-Y | BDR-U | BDR-V | BDR-Y | BDR-U | BDR-V |
| BasketballPass | -0.02 | 0.61 | 0.11 | 0.05 | -0.28 | 0.20 | 0.01 | 1.71 | -0.18 |
| BQSquare | -0.28 | -0.18 | -0.16 | -0.55 | -1.52 | -1.21 | -0.70 | -3.31 | -1.37 |
| BlowingBubbles | 0.09 | 0.23 | -0.25 | -0.53 | -0.08 | -1.42 | -0.69 | -0.47 | 0.03 |
| RaceHorses | -0.12 | -0.23 | 0.67 | 0.17 | -0.50 | 0.64 | -0.03 | 0.14 | -0.27 |
| average | -0.08 | 0.11 | 0.10 | -0.21 | -0.60 | -0.45 | -0.35 | -0.48 | -0.45 |
| dec. runtime increase | 100.1% |  |  | 99.6% |  |  | 102.7% |  |  |
|  | Rep-Padding |  |  | TM-Padding |  |  | MC-TM-Padding |  |  |
| Class E | BDR-Y | BDR-U | BDR-V | BDR-Y | BDR-U | BDR-V | BDR-Y | BDR-U | BDR-V |
| FourPeople | 0.09 | 1.02 | -0.50 | 0.08 | -0.48 | -0.59 | 0.23 | 0.45 | -0.73 |
| Johnny | 0.08 | -0.21 | 0.31 | 0.08 | 0.75 | -0.41 | -0.01 | -0.41 | -2.26 |
| KristenAndSara | 0.10 | -1.13 | -1.42 | -0.27 | -0.02 | 0.03 | -0.05 | -1.47 | -0.91 |
| average | 0.09 | -0.11 | -0.54 | -0.04 | 0.08 | -0.32 | 0.06 | -0.48 | -1.30 |
| dec. runtime increase | 98.5% |  |  | 99.6% |  |  | 104.1% |  |  |
|  | Rep-Padding |  |  | TM-Padding |  |  | MC-TM-Padding |  |  |
| Class F | BDR-Y | BDR-U | BDR-V | BDR-Y | BDR-U | BDR-V | BDR-Y | BDR-U | BDR-V |
| BasketballDrillText | -0.10 | 0.25 | 0.27 | -0.20 | 0.51 | -0.20 | -0.25 | 0.76 | -0.06 |
| ArenaOfValor | -0.12 | -0.12 | 0.08 | -0.15 | -0.50 | 0.31 |  |  |  |
| SlideEditing | 0.09 | 0.03 | 0.17 | 0.18 | -0.22 | -0.05 | 0.90 | 0.66 | 1.01 |
| SlideShow | -0.80 | -2.08 | -3.32 | -0.61 | -0.61 | -2.82 | -0.65 | -0.35 | -2.41 |
| average | -0.23 | -0.48 | -0.70 | -0.19 | -0.20 | -0.69 |  |  |  |
| dec. runtime increase | 99.9% |  |  | 102.0% |  |  |  |  |  |

For Low Delay B, we observe, that TM-Padding outperforms MC-Padding on average for every Class in terms of compression efficiency. Simultaneously, the decoder runtime is reduced by -0.02% on average. MC-TM-Padding improves the results further, except for class E. The strongest improvement can be observed for class D, going from -0.21% BDR-Y to -0.35% BDR-Y. MC-TM-Padding introduces a decoder runtime increase compared to MC-Padding.

## Low Delay P

|  | Rep-Padding |  |  | TM-Padding |  |  | MC-TM-Padding |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class B | BDR-Y | BDR-U | BDR-V | BDR-Y | BDR-U | BDR-V | BDR-Y | BDR-U | BDR-V |
| MarketPlace | 0.13 | 0.05 | 0.48 | 0.09 | -0.69 | 1.02 |  |  |  |
| RitualDance | -0.03 | 0.34 | 0.14 | -0.05 | 0.42 | 0.31 |  |  |  |
| Cactus | 0.03 | -0.54 | 0.53 | -0.06 | -0.51 | 0.13 |  |  |  |
| BasketballDrive | 0.04 | -0.09 | 0.07 | -0.07 | 0.40 | 0.44 |  |  |  |
| BQTerrace | -0.09 | -0.29 | 0.62 | -0.21 | 1.21 | 2.88 |  |  |  |
| average | 0.02 | -0.11 | 0.37 | -0.06 | 0.17 | 0.96 |  |  |  |
| dec. runtime increase | 99.8% |  |  | 100.2% |  |  |  |  |  |
|  | Rep-Padding |  |  | TM-Padding |  |  | MC-TM-Padding |  |  |
| Class C | BDR-Y | BDR-U | BDR-V | BDR-Y | BDR-U | BDR-V | BDR-Y | BDR-U | BDR-V |
| BasketballDrill | -0.06 | -0.66 | -0.16 | -0.08 | -0.18 | -0.38 | -0.22 | -0.40 | -0.47 |
| BQMall | 0.03 | 0.73 | 0.20 | -0.09 | 0.16 | -0.83 | -0.15 | 0.62 | -0.20 |
| PartyScene | 0.12 | 0.33 | 0.65 | 0.14 | 0.18 | 0.46 | 0.10 | 0.52 | -0.07 |
| RaceHorses | 0.14 | 0.43 | 0.49 | 0.03 | -0.86 | 0.89 | 0.04 | -0.74 | 0.26 |
| average | 0.06 | 0.21 | 0.29 | 0.00 | -0.18 | 0.04 | -0.06 | 0.00 | -0.12 |
| dec. runtime increase | 100.6% |  |  | 100.9% |  |  | 101.8% |  |  |
|  | Rep-Padding |  |  | TM-Padding |  |  | MC-TM-Padding |  |  |
| Class D | BDR-Y | BDR-U | BDR-V | BDR-Y | BDR-U | BDR-V | BDR-Y | BDR-U | BDR-V |
| BasketballPass | -0.03 | -0.37 | 0.27 | 0.01 | 0.10 | -0.77 | -0.04 | 0.00 | -0.10 |
| BQSquare | -0.09 | -3.98 | 0.96 | -0.29 | -4.50 | -1.97 | -0.41 | -4.37 | -1.05 |
| BlowingBubbles | -0.07 | 0.41 | 0.27 | -0.75 | -0.97 | 0.01 | -0.78 | -1.46 | 0.11 |
| RaceHorses | -0.09 | 0.01 | -0.06 | -0.16 | -0.51 | 1.12 | -0.20 | -0.52 | -0.23 |
| average | -0.07 | -0.98 | 0.36 | -0.30 | -1.47 | -0.40 | -0.36 | -1.59 | -0.32 |
| dec. runtime increase | 99.9% |  |  | 102.3% |  |  | 102.9% |  |  |
|  | Rep-Padding |  |  | TM-Padding |  |  | MC-TM-Padding |  |  |
| Class E | BDR-Y | BDR-U | BDR-V | BDR-Y | BDR-U | BDR-V | BDR-Y | BDR-U | BDR-V |
| FourPeople | -0.21 | -0.30 | 0.07 | -0.09 | -0.41 | -0.19 | 0.19 | 0.12 | 0.04 |
| Johnny | 0.06 | -1.67 | 0.63 | 0.05 | -2.25 | 0.21 | 0.40 | -1.71 | -0.60 |
| KristenAndSara | -0.16 | -0.10 | 1.54 | -0.15 | 1.01 | 0.77 | -0.11 | -0.51 | -1.21 |
| average | -0.11 | -0.69 | 0.75 | -0.06 | -0.55 | 0.26 | 0.16 | -0.70 | -0.59 |
| dec. runtime increase | 99.6% |  |  | 100.6% |  |  | 103.5% |  |  |
|  | Rep-Padding |  |  | TM-Padding |  |  | MC-TM-Padding |  |  |
| Class F | BDR-Y | BDR-U | BDR-V | BDR-Y | BDR-U | BDR-V | BDR-Y | BDR-U | BDR-V |
| BasketballDrillText | -0.11 | 0.27 | 0.00 | -0.14 | -0.07 | -0.71 | -0.17 | 0.22 | -0.64 |
| ArenaOfValor | 0.04 | 0.25 | 0.14 | 0.04 | -0.02 | 0.30 | 0.00 | 0.37 | -0.13 |
| SlideEditing | -0.19 | -0.40 | -0.45 | -0.14 | -0.46 | 0.12 | 0.03 | -0.28 | -0.34 |
| SlideShow | 0.04 | -0.09 | 0.02 | 0.27 | -0.65 | -0.52 | 0.14 | -0.48 | 2.06 |
| average | -0.05 | 0.01 | -0.07 | 0.01 | -0.30 | -0.20 | 0.00 | -0.04 | 0.24 |
| dec. runtime increase | 100.5% |  |  | 101.4% |  |  | 103.3% |  |  |

For Low Delay P, we can observe a similar behaviour as for Low Delay B. TM-Padding outperforms MC-Padding in all classes. It must be noted that for class E, we see a 0.01% BDR-Y increase, but simultaneously a -0.3% BDR-U, -0.2% BDR-V decrease. We see a slight decoder runtime increase compared to MC-Padding for all classes except class B with a decrease of 0.3%. MC-TM Padding improves BD-rate for the classes C and D compared to TM-Padding. For those two classes the decoder runtime increased is also diminished compared to TM-Padding. This can be explained by enabling MC-Padding, which has only a very slight impact on runtime increase for the Low Delay P configuration. The methods pad parts of the padded area that do not need to be padded by TM-Padding consequently.

## Summary Tables

In the following, the summary tables for the different methods are shown.

Rep-Padding

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.1 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |
| Class C | 0.16% | 0.15% | 0.16% |  | 99.7% |  |
| Class E |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |
| Class D | 0.35% | 0.35% | 0.29% | #WERT! | 98.8% |  |
| Class F | 0.02% | 0.13% | 0.10% | #WERT! | 97.6% |  |
| Class TGM |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |
|  | Over ECM-12.1 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 0.06% | 0.04% | 0.07% |  | 99.9% |  |
| Class C | 0.07% | -0.03% | 0.13% |  | 99.5% |  |
| Class E | 0.09% | -0.11% | -0.54% |  | 98.5% |  |
| Overall | 0.07% | -0.02% | -0.06% |  | 99.4% |  |
| Class D | -0.08% | 0.11% | 0.10% |  | 100.1% |  |
| Class F | -0.23% | -0.48% | -0.70% |  | 99.9% |  |
| Class TGM |  |  |  |  |  |  |
|  | Low delay P Main 10 |  |  |  |  |  |
|  | Over ECM-12.1 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 0.02% | -0.11% | 0.37% |  | 99.8% |  |
| Class C | 0.06% | 0.21% | 0.29% |  | 100.6% |  |
| Class E | -0.11% | -0.69% | 0.75% |  | 99.6% |  |
| Overall | 0.00% | -0.15% | 0.44% |  | 100.0% |  |
| Class D | -0.07% | -0.98% | 0.36% |  | 99.9% |  |
| Class F | -0.05% | 0.01% | -0.07% |  | 100.5% |  |
| Class TGM |  |  |  |  |  |  |

TM-Padding

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.1 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | 0.03% | -0.16% | 0.11% |  | 100.7% |  |
| Class A2 |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |
| Class C | 0.10% | 0.04% | 0.04% |  | 99.1% |  |
| Class E |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |
| Class D | 0.28% | 0.03% | 0.18% |  | 99.4% |  |
| Class F | 0.06% | 0.12% | 0.06% |  | 97.6% |  |
| Class TGM |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |
|  | Over ECM-12.1 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.03% | 0.26% | 0.32% |  | 100.2% |  |
| Class C | -0.01% | -0.35% | 0.05% |  | 99.5% |  |
| Class E | -0.04% | 0.08% | -0.32% |  | 99.6% |  |
| Overall | -0.02% | 0.01% | 0.07% |  | 99.8% |  |
| Class D | -0.21% | -0.60% | -0.45% |  | 99.6% |  |
| Class F | -0.19% | -0.20% | -0.69% |  | 102.0% |  |
| Class TGM |  |  |  |  |  |  |
|  | Low delay P Main 10 |  |  |  |  |  |
|  | Over ECM-12.1 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.06% | 0.17% | 0.96% |  | 100.2% |  |
| Class C | 0.00% | -0.18% | 0.04% |  | 100.9% |  |
| Class E | -0.06% | -0.55% | 0.26% |  | 100.6% |  |
| Overall | -0.04% | -0.13% | 0.48% |  | 100.5% |  |
| Class D | -0.30% | -1.47% | -0.40% |  | 102.3% |  |
| Class F | 0.01% | -0.30% | -0.20% |  | 101.4% |  |
| Class TGM |  |  |  |  |  |  |

MC-TM-Padding

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.1 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |
| Class C | -0.03% | -0.05% | -0.03% | #WERT! | 100.9% (99.9%) |  |
| Class E |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |
| Class D | -0.03% | -0.02% | 0.19% | #WERT! | 101.5% (100.0%) |  |
| Class F |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |
|  | Over ECM-12.1 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |
| Class C | -0.02% | 0.04% | -0.17% |  | 101.0% |  |
| Class E | 0.06% | -0.48% | -1.30% |  | 104.1% |  |
| Overall |  |  |  |  |  |  |
| Class D | -0.35% | -0.48% | -0.45% |  | 102.7% |  |
| Class F |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |
|  | Low delay P Main 10 |  |  |  |  |  |
|  | Over ECM-12.1 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |
| Class C | -0.06% | 0.00% | -0.12% |  | 101.8% |  |
| Class E | 0.16% | -0.70% | -0.59% |  | 103.5% |  |
| Overall |  |  |  |  |  |  |
| Class D | -0.36% | -1.59% | -0.32% |  | 102.9% |  |
| Class F | 0.00% | -0.04% | 0.24% |  | 103.3% |  |
| Class TGM |  |  |  |  |  |  |

# Conclusion

The proposed method MC-TM-Padding shows improvements over ECM-12.1 for the Random Access, Low Delay B and Low Delay P configuration. For Random Access, no decoder runtime increase is observed, if the padding is disabled for reference picture with the highest temporal ID. Further the TM-Padding method shows improvements over ECM-12.1 in the Low Delay B and Low Delay P configuration with and average BDR-Y improvement of -0.02 and -0.04 respectively. For the Low Delay B configuration, we can observe a decoder runtime decrease of -0.02% on average.

Therefore, we propose to further study the methods in an exploration experiment, especially the selection of parameters and an optional adaption of parameters during decoding to increase coding gains and reduce complexity.

# References

[1] JVET-AH0168 - N. Neumann and M. Wien „ Template Matching Picture Boundary Padding“

[2] JVET-AA0096 - Fabrice Le Léannec, Pierre Andrivon, Miloš Radosavljevć „EE2-2.2: Motion compensated picture boundary padding“

[3] JVET-AF2017 – M. Karczewicz and Y. Ye „Common test conditions and evaluation procedures for enhanced compression tool testing “

# Patent rights declaration(s)

RWTH Aachen University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).