| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0053-v1 |
| --- | --- |

| Title: | AHG14: The extension of SADL library |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nianxiang Fu, Weijie Bao, Junxi Zhang, Zhenzhong Chen | Email: | {nianxiangfu, baoweijie, sissie_zhang, zzchen}@whu.edu.cn |
| Source: | Wuhan University |  |  |

_____________________________

- Abstract

This contribution presents the extensions in the Small AdHoc Deep Learning (SADL) library from Wuhan University. These extensions include layers such as BatchNorm and ReduceMean.

Overview of extensions

Table 1 below summarizes the extensions in the SADL library [1].

Table 1: The overview of extensions in the SADL library.

| Language | Pure C++, header only |
| --- | --- |
| Compatibility | Support PyTorch and TensorFlow, while also updating the Onnx to the SADL converter |
| Extended layers | BatchNorm and ReduceMean |
| Type support | float, int32, int16, int8 |
| License | BSD 3-Clause |

Extended features

BatchNorm

Extension description: This contribution introduces a new layer called BatchNorm, which normalizes the input tensor X by using the affine parameters and the running mean and variance accumulated during training. This operation involves standardizing the input based on the running mean and variance, followed by scaling and shifting using learnable affine parameters gamma (scale) and beta (shift), as follows:

BN(x)=γ(x - μσ2 + ϵ )+β,

where x represents the elements of the input tensor; μ and σ2 denote the running mean and variance, respectively; ϵ denotes a small constant for numerical stability (to prevent division by zero); γ and β denote scale and shift parameters, respectively. BatchNorm is compatible with PyTorch and TensorFlow2 frameworks and supports various data types, including integer and float.

ReduceMean

Extension description: This contribution introduces a new layer called ReduceMean, which computes the mean of elements across specified dimensions of the input tensor X. ReduceMean reduces the tensor by averaging all values along the specified axes.

For example, let tensor X with shape (B, H, W, C). The ReduceMean operation can be performed over different dimensions, resulting in different output shapes:

- Reducing over the C-dimension: This produces a tensor Y with shape (B, H, W, 1). The elements of Y are defined as:

Y[b,h,w,1]=1Cc=1CX[b,h,w,c]

- Reducing over the H and W-dimensions: This produces a tensor Y with shape (B, 1, 1, C). The elements of Y are defined as:

Y[b,1,1,c]=1H×Wh=1H w=1W X[b,h,w,c]

Other cases can be similarly derived by appropriately summing over the required dimensions and dividing by the number of elements in those dimensions.

ReduceMean is compatible with PyTorch and TensorFlow2 and supports integer and float data types.

Changes details

Changes are as follows:

- Add a new layer called BatchNorm, which normalizes the input tensor by using the affine parameters and the running mean and variance.

- Add a new layer called ReduceMean, which computes the mean of elements across specified dimensions of the input tensor.

Conclusion

This contribution presents the extensions in the Small AdHoc Deep Learning (SADL) library from Wuhan University.

Reference

- F. Galpin, T. Dumas, P. Bordes, E. François. AhG14: SADL update. JVET-AG0109, 33rd Meeting, by teleconference, 17-26 January 2024.

Acknowledgement

We are grateful to InterDigital Inc. for their valuable guidance and assistance. Their contributions have made a significant impact on this work.

Patent rights declaration(s)

Wuhan University does not have any current or pending patent rights relating to the technology described in this contribution.