| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0069-v1 |
| --- | --- |

| Title: | Additional Objective Results of VVC Multilayer Reference Implementation (VTM-21.0) on SDR and HDR UHD (4K) Content |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Olena Chubach, Yu-Ling Hsiao, Ching-Yeh Chen, Tzu-Der Chuang, Yi-Wen Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Email: | olena.chubach, yuling.hsiao, chingyeh.chen, peter.chuang, yiwen-yw.chen, cw.hsu, yuwen.huang, shawmin.lei} mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

This contribution is a follow up information about preliminary tests on spatial scalability using VVC Reference software (VTM-21.0) and initial objective evaluation results. Tests were performed for LD configuration on SDR 4K content and RA configuration on HDR 4K content. SDR test sequences are from JVET SDR Common Testing Conditions (Classes A1 and A2) and HDR content are from VVC Verification Tests and JVET HDR/WCG CTC.

SDR video in LD are reported as follows (in terms of PSNR-BD rate):

- Multi-layer v.s. single-layer: +32.4%, +43.0%, +50.6% for Y, Cb, and Cr BD-rates

- Multi-layer v.s. simulcasting: -27.9%, -23.3%, -19.8% for Y, Cb, and Cr BD-rates

HDR video in RA are reported as follows (in terms of PSNR-BD rate):

- PQ content

  - Multi-layer v.s. single-layer: +7.6%, +62.9%, +62.3% for Y, Cb, and Cr BD-rates

  - Multi-layer v.s. simulcasting: -43.4%, -14.5%, -15.2% for Y, Cb, and Cr BD-rates

- HLG content

  - Multi-layer v.s. single-layer: +22.1%, +39.2%, +41.7% for Y, Cb, and Cr BD-rates

  - Multi-layer v.s. simulcasting: -33.9%, -24.8%, -23.3% for Y, Cb, and Cr BD-rates

Introduction

VVC (Versatile Video Coding), also known as H.266, was designed to improve compression efficiency and support a wide range of applications and devices. It represents a significant advancement in video coding technology, offering a versatile solution for the evolving demands of modern multimedia applications.

Among other features, VVC allows for scalable coding (multilayer, ML), aiming to provide even more efficient scalability options than its predecessors, with support for Spatial Scalability, Temporal Scalability, Quality Scalability, etc. The scalability features of VVC make it possible to efficiently stream and store video content that can be dynamically adapted to a wide range of devices, network conditions, and user preferences.

A formal Verification Test (VT) of VVC on SDR, HDR, and 360-degree video content was finalized in July 2021. Verification Tests for VVC multilayer are still under discussion in JVET.

Some preliminary results for SDR 4K content coded using VVC Reference software (VTM-21.0) [1] with RA CTC configurations [2] are reported in [3]. The results show +16.7%, +22.8%, and +29.7% for Y, Cb, and Cr PSNR BD-rates respectively, compared to the Single Layer (SL) results. Compared to the Simulcast, average improvements of -36.6%, -33.1%, and -29.7% for Y, Cb, and Cr PSNR BD-rates respectively can be obtained. Results and details of the simulations (e.g., test conditions and settings used in tests) for RA configuration of VVC ML can also be found in [3].

Testing Conditions and Settings Used for VVC ML Evaluation

SDR 4K content

During the discussion of [3], a question was raised about the performance of VVC ML in LD configurations.

Following tests are performed:

- Test1. H.266/VVC SL: VTM-21.0 (single layer encoding of 4K content with QP=22, 27, 32, 37)  and LD configuration

- Test2. H.266/VVC ML: VTM-21.0 (2x spatial scalability) with LD configuration and following relation: QP_BL = QP_EL – 6

- Test3. H.266/VVC ML: VTM-21.0 (2x spatial scalability) with LD configuration and following relation: QP_BL = QP_EL – 7

Test2 and Test 3 are run for enhancement layer QPs (QP_EL) between 23 and 47. After that, the closest rates of Test2 and Test3 (VVC ML) are chosen for the corresponding rates of Test1.

Performance vs. simulcasting as an anchor is additionally provided. For this, corresponding base layer bitrate is added to the bitrate of SL encoding. PSNR obtained for SL is also used for simulcasting.

Following modifications were applied to the VTM-21.0 before running the tests with LD configuration:

- Modifications in the MR !2686 were applied to upscale the pictures while decoding (using the normative RPR filters)

It should be noted that the patch provided in JVET-AF0186, putting inter-layer reference picture to the first position in the reference picture list, could not be applied in LD configuration. Applying it resulted in encoder crash after encoding one frame. This issue was reported to the proponents of the JVET-AF0186 however it did not affect the configuration of the LD. The reference picture with the same POC from LayerId 0 is still put to the first position of both reference lists (L0 and L1).

The remaining settings described in [3] were also used in this test, except for the encoder_lowdelay_vtm.cfg used instead of encoder_randomaccess_vtm.cfg

Number of frames encoded – 300 frames for all tested sequences, except Tango, for which only 294 frames are available.

Objective results

Table 1 - 3 contain results for Test2 QP_BL=QP_EL – 6.

Table 1. BD-rate results of Test2 (VVC ML, QP_BL=QP_EL – 6) vs Test1 (VVC SL) as anchor for SDR content coded with VTM-21p0 @LD cfg.

| Sequence | Low Delay B, Over VTM-21.0 |  |  |  |
| --- | --- | --- | --- | --- |
|  | PSNR_Y | PSNR_Cb | PSNR_Cr | PSNR_YCbCr |
| Tango2 | -0.68% | 5.46% | 10.16% | 1.96% |
| FoodMarket4 | 5.40% | 9.58% | 9.19% | 6.28% |
| Campfire | 40.24% | 31.23% | 51.45% | 39.86% |
| CatRobot | 51.31% | 80.73% | 91.94% | 58.22% |
| DaylightRoad2 | 85.65% | 63.67% | 81.71% | 82.47% |
| ParkRunning3 | 30.16% | 71.89% | 62.89% | 35.26% |
| Overall | 35.35% | 43.76% | 51.22% | 37.34% |

Table 2. BD-rate results of Test2 (VVC ML, QP_BL=QP_EL – 6) vs Simulcast as anchor for SDR content coded with VTM-21p0 @LD cfg.

| Sequence | Low Delay B, Over VTM-21.0 |  |  |  |
| --- | --- | --- | --- | --- |
|  | PSNR_Y | PSNR_Cb | PSNR_Cr | PSNR_YCbCr |
| Tango2 | -48.68% | -45.76% | -43.42% | -47.40% |
| FoodMarket4 | -44.48% | -42.00% | -42.17% | -43.96% |
| Campfire | -14.66% | -19.44% | -8.15% | -14.80% |
| CatRobot | -14.92% | 1.76% | 7.95% | -11.03% |
| DaylightRoad2 | 6.34% | -5.26% | 4.31% | 4.67% |
| ParkRunning3 | -22.47% | 2.26% | -3.10% | -19.46% |
| Overall | -23.15% | -18.07% | -14.10% | -22.00% |

Table 3. Bitrates used for BL, total bitrate and EL bitrate percentage used for sequences from Test2 (VVC ML, QP_BL=QP_EL – 6).

| Sequence | QP_BL | QP_EL | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BL bitrate, kbps | Total bitrate, kbps | EL bitrate, % |
| Tango2 | 16 | 22 | 22980.81 | 25715.52 | 10.63 |
|  | 22 | 28 | 7213.625 | 7537.393 | 4.30 |
|  | 27 | 33 | 3361.677 | 3514.922 | 4.36 |
|  | 31 | 37 | 1983.154 | 2083.269 | 4.81 |
| FoodMarket4 | 18 | 24 | 11918.24 | 14085.38 | 15.39 |
|  | 23 | 29 | 6053.469 | 6553.118 | 7.62 |
|  | 27 | 33 | 3289.925 | 3497.757 | 5.94 |
|  | 31 | 37 | 1866.002 | 1972.859 | 5.42 |
| Campfire | 17 | 23 | 19695.96 | 53860.42 | 63.43 |
|  | 23 | 29 | 8269.852 | 12479.81 | 33.73 |
|  | 28 | 34 | 4033.675 | 5846.75 | 31.01 |
|  | 31 | 37 | 2719.502 | 3652.25 | 25.54 |
| CatRobot | 19 | 25 | 11651.18 | 19195.55 | 39.30 |
|  | 24 | 30 | 5191.573 | 6915.038 | 24.92 |
|  | 28 | 34 | 2857.102 | 3447.8 | 17.13 |
|  | 32 | 38 | 1599.515 | 1894.856 | 15.59 |
| DaylightRoad2 | 18 | 24 | 14871.41 | 31986.67 | 53.51 |
|  | 24 | 30 | 5500.782 | 8667.75 | 36.54 |
|  | 28 | 34 | 2960.542 | 3911.03 | 24.30 |
|  | 32 | 38 | 1620.157 | 1971.87 | 17.84 |
| ParkRunning3 | 18 | 24 | 56966.33 | 109626.29 | 48.04 |
|  | 23 | 29 | 27984.37 | 41580.76 | 32.70 |
|  | 28 | 34 | 11574.4 | 16297.18 | 28.98 |
|  | 32 | 38 | 5694.653 | 7291.03 | 21.90 |

Table 4 - 6 contain results for Test3 QP_BL=QP_EL – 7.

Table 4. BD-rate results of Test3 (VVC ML, QP_BL=QP_EL – 7) vs Test1 (VVC SL) as anchor for SDR content coded with VTM-21p0 @LD cfg.

| Sequence | Low Delay B, Over VTM-21.0 |  |  |  |
| --- | --- | --- | --- | --- |
|  | PSNR_Y | PSNR_Cb | PSNR_Cr | PSNR_YCbCr |
| Tango2 | -1.69% | 5.41% | 10.97% | 1.40% |
| FoodMarket4 | 3.60% | 8.29% | 7.77% | 4.62% |
| Campfire | 36.88% | 30.52% | 44.19% | 36.72% |
| CatRobot | 43.86% | 70.02% | 82.59% | 50.47% |
| DaylightRoad2 | 75.73% | 45.88% | 64.42% | 70.52% |
| ParkRunning3 | 24.25% | 70.85% | 61.65% | 30.07% |
| Overall | 32.36% | 43.00% | 50.60% | 32.30% |

Table 5. BD-rate results of Test3 (VVC ML, QP_BL=QP_EL – 7) vs Simulcast as anchor for SDR content coded with VTM-21p0 @LD cfg.

| Sequence | Low Delay B, Over VTM-21.0 |  |  |  |
| --- | --- | --- | --- | --- |
|  | PSNR_Y | PSNR_Cb | PSNR_Cr | PSNR_YCbCr |
| Tango2 | -49.29% | -45.92% | -43.15% | -47.79% |
| FoodMarket4 | -46.94% | -44.69% | -44.96% | -46.45% |
| Campfire | -17.95% | -21.07% | -13.79% | -17.95% |
| CatRobot | -24.17% | -10.13% | -3.65% | -20.66% |
| DaylightRoad2 | -0.45% | -16.25% | -6.42% | -3.19% |
| ParkRunning3 | -28.37% | -1.49% | -7.04% | -25.03% |
| Overall | -27.86% | -23.26% | -19.84% | -26.85% |

Table 6. Bitrates used for BL, total bitrate and EL bitrate percentage used for sequences from Test3 (VVC ML, QP_BL=QP_EL – 7).

| Sequence | QP_BL | QP_EL | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BL bitrate, kbps | Total bitrate, kbps | EL bitrate, % |
| Tango2 | 16 | 23 | 23885.63 | 25348.1633 | 5.77 |
|  | 22 | 29 | 7213.625 | 7444.4539 | 3.10 |
|  | 27 | 34 | 3361.677 | 3475.2538 | 3.27 |
|  | 31 | 38 | 1983.154 | 2052.64 | 3.39 |
| FoodMarket4 | 17 | 24 | 13692.8 | 15538.07 | 11.88 |
|  | 22 | 29 | 7057.582 | 7458.04 | 5.37 |
|  | 27 | 34 | 3291.235 | 3425.52 | 3.92 |
|  | 31 | 38 | 1867.677 | 1922.41 | 2.85 |
| Campfire | 16 | 23 | 23832.66 | 56275.72 | 57.65 |
|  | 23 | 30 | 8269.852 | 11820.61 | 30.04 |
|  | 27 | 34 | 4469.319 | 6170.83 | 27.57 |
|  | 31 | 38 | 2719.502 | 3275.17 | 16.97 |
| CatRobot | 18 | 25 | 13687.12 | 20456.56 | 33.09 |
|  | 23 | 30 | 6234.629 | 7684.79 | 18.87 |
|  | 27 | 34 | 3262.818 | 3750.66 | 13.01 |
|  | 31 | 38 | 1831.926 | 2043.32 | 10.35 |
| DaylightRoad2 | 18 | 25 | 13687.12 | 20456.56 | 33.09 |
|  | 23 | 30 | 6234.629 | 7684.79 | 18.87 |
|  | 27 | 34 | 3262.818 | 3750.66 | 13.01 |
|  | 31 | 38 | 1831.926 | 2043.32 | 10.35 |
| ParkRunning3 | 17 | 24 | 64717.1 | 114426.22 | 43.44 |
|  | 23 | 30 | 28025.39 | 39446.86 | 28.95 |
|  | 27 | 34 | 13451.27 | 17579.08 | 23.48 |
|  | 31 | 38 | 6735.901 | 7998.11 | 15.78 |

HDR 4K content

There are two classes of test sequences used in [4] as CTC for HDR/WCG content. Class H1 contains PQ HDR sequences in HD (1920x1080) resolution, while class H2 contains HLG HDR content in UHD (3840x2160) resolution.

Considering that resolution of interest for the current tests is 4K, it was decided to use only HLG HDR sequences from classH2 of CTC for HDR/WCG content. For PQ HDR, PQ sequences from [5] were chosen (assigned to classH3 for convenience in this document).

Following settings were used in this test:

- per-class configurations for classes H2 [4] and H3 (same as classH1.cfg in [4])

- per-sequence configurations for all sequences in H2 and H3 and corresponding per-sequence configuration files for the down-sampled version of each sequence. It should be notes that down-sampled version of the sequence was obtained using the TAppDownConvertStatic tool from the SHM repository. The following example command lines are used for 10-bit BT.2020 content

  - TAppDownConvertStatic <win> <hin> <in.yuv> <wout> <hout> <out.yuv> 10 10 0 0 0 <frames> -phase 0 0 0 0

This matches the “VerCollocatedChroma” configuration parameter of VTM, which uses it in the resolution change process for correct chroma alignment.

Following tests are performed for classes H2 and H3:

- Test4. H.266/VVC SL: VTM-21.0 (single layer encoding of 4K HDR content with QP=22, 27, 32, 37)  and RA configuration

- Test5. H.266/VVC ML: VTM-21.0 (2x spatial scalability) with RA configuration and following relation: QP_BL = QP_EL – 6

- Test6. H.266/VVC ML: VTM-21.0 (2x spatial scalability) with RA configuration and following relation: QP_BL = QP_EL – 7

Test5 and Test6 are run for enhancement layer QPs (QP_EL) between 23 and 47. After that, the closest rates of Test5 and Test6 (VVC ML) are chosen for the corresponding rates of Test4.

Following length of tests sequences coded

- classH2: 5 seconds, depending on the framerate

- classH3: 10 seconds, depending on the framerate

Objective results

Table 7 - 9 contain results for Test5 QP_BL=QP_EL – 6.

Table 7. BD-rate results of Test5 (VVC ML, QP_BL=QP_EL – 6) vs Test4 (VVC SL) as anchor for HDR content coded with VTM-21p0 @RA cfg.

| Sequence | Random Access, Over VTM-21.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNRL100 | wPsnrY |  | wPsnrU | wPsnrV | psnrY |  | psnrU | psnrV |
| H3_SparksWelding | 25.37% | 1.94% | 1.63% |  | 56.10% | 57.26% | 0.99% |  | 56.05% | 54.19% |
| H3_SparksDirtLot | 27.90% | 19.99% | 21.31% |  | 43.05% | 41.90% | 20.90% |  | 46.81% | 42.35% |
| H3_MeridianHDR5 | 16.03% | 5.98% | 7.65% |  | 45.84% | 60.47% | 8.08% |  | 45.29% | 61.74% |
| H3_MeridianHDR2 | 18.71% | -1.24% | -1.14% |  | 49.84% | 61.16% | -1.22% |  | 49.37% | 63.25% |
| H3_ChimeraHDR5 | 41.23% | 16.98% | 16.34% |  | 99.11% | 60.43% | 15.19% |  | 91.27% | 50.62% |
| Overall | 25.85% | 8.73% | 9.16% |  | 58.79% | 56.24% | 8.79% |  | 57.76% | 54.43% |
| Sequence | Random Access, Over VTM-21.0 |  |  |  |  |  |  |  |  |  |
|  | psnrY |  |  | psnrU |  |  |  | psnrV |  |  |
| H2_DayStreet2 | 34.73% |  |  | 45.75% |  |  |  | 49.23% |  |  |
| H2_FlyingBirds3 | 7.56% |  |  | 23.15% |  |  |  | 24.90% |  |  |
| H2_PeopleInShoppingCenter2 | 18.70% |  |  | 36.73% |  |  |  | 39.87% |  |  |
| H2_SunsetBeach3 | 23.84% |  |  | 48.56% |  |  |  | 59.76% |  |  |
| Overall | 21.21% |  |  | 38.55% |  |  |  | 43.44% |  |  |

Table 8. BD-rate results of Test5 (VVC ML, QP_BL=QP_EL – 6) vs Simulcast as anchor for HDR content coded with VTM-21p0 @RA cfg.

| Sequence | Random Access, Over VTM-21.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNRL100 | wPsnrY |  | wPsnrU | wPsnrV | psnrY |  | psnrU | psnrV |
| H3_SparksWelding | -32.85% | -45.39% | -45.57% |  | -16.45% | -15.61% | -45.90% |  | -16.47% | -17.27% |
| H3_SparksDirtLot | -28.16% | -32.66% | -31.92% |  | -19.61% | -20.36% | -32.15% |  | -17.51% | -20.10% |
| H3_MeridianHDR5 | -37.59% | -42.92% | -42.05% |  | -21.45% | -13.68% | -41.83% |  | -21.75% | -13.00% |
| H3_MeridianHDR2 | -37.07% | -47.57% | -47.51% |  | -20.57% | -14.57% | -47.56% |  | -20.81% | -13.47% |
| H3_ChimeraHDR5 | -25.83% | -38.50% | -38.83% |  | 4.58% | -16.19% | -39.43% |  | 0.47% | -21.29% |
| Overall | -32.30% | -41.41% | -41.18% |  | -14.70% | -16.08% | -41.37% |  | -15.21% | -17.03% |
| Sequence | Random Access, Over VTM-21.0 |  |  |  |  |  |  |  |  |  |
|  | psnrY |  |  | psnrU |  |  |  | psnrV |  |  |
| H2_DayStreet2 | -25.02% |  |  | -19.56% |  |  |  | -17.06% |  |  |
| H2_FlyingBirds3 | -44.19% |  |  | -36.14% |  |  |  | -35.25% |  |  |
| H2_PeopleInShoppingCenter2 | -36.47% |  |  | -26.80% |  |  |  | -25.16% |  |  |
| H2_SunsetBeach3 | -25.53% |  |  | -10.93% |  |  |  | -4.37% |  |  |
| Overall | -32.80% |  |  | -23.36% |  |  |  | -20.46% |  |  |

Table 9. Bitrates used for BL, total bitrate and EL bitrate percentage used for sequences from Test5 (VVC ML, QP_BL=QP_EL – 6).

| Sequence | QP_BL | QP_EL | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BL bitrate, kbps | Total bitrate, kbps | EL bitrate, % |
| H3_SparksWelding | 17 | 23 | 18421.09 | 21609.84 | 14.75601 |
|  | 22 | 28 | 9017.834 | 9954.854 | 9.412695 |
|  | 27 | 33 | 3967.251 | 4394.57 | 9.723783 |
|  | 31 | 37 | 1978.518 | 2176.115 | 9.080291 |
| H3_SparksDirtLot | 17 | 23 | 7668.302 | 11118.86 | 31.03335 |
|  | 22 | 28 | 2809.514 | 3773.028 | 25.5369 |
|  | 27 | 33 | 1134.91 | 1471.833 | 22.8914 |
|  | 32 | 38 | 519.1856 | 633.7944 | 18.08296 |
| H3_MeridianHDR5 | 17 | 23 | 4634.339 | 5917.535 | 21.68464 |
|  | 22 | 28 | 1851.643 | 2157.714 | 14.18497 |
|  | 27 | 33 | 756.1336 | 845.516 | 10.57134 |
|  | 31 | 37 | 379.4776 | 422.204 | 10.11985 |
| H3_MeridianHDR2 | 17 | 23 | 3519.657 | 3881.33 | 9.318271 |
|  | 22 | 28 | 1500.657 | 1608.177 | 6.685832 |
|  | 27 | 33 | 654.8432 | 709.6184 | 7.718966 |
|  | 31 | 37 | 340.2888 | 380.6312 | 10.59882 |
| H3_ChimeraHDR5 | 17 | 23 | 20458.96 | 25977.96 | 21.24493 |
|  | 22 | 28 | 8861.619 | 10450.61 | 15.20476 |
|  | 27 | 33 | 3939.442 | 4518.959 | 12.82412 |
|  | 31 | 37 | 2086.096 | 2321.327 | 10.13348 |
| H2_DayStreet2 | 18 | 24 | 11965.22 | 18168.34 | 34.14249 |
|  | 23 | 29 | 5343.021 | 7241.776 | 26.21947 |
|  | 27 | 33 | 2915.984 | 3643.246 | 19.96193 |
|  | 32 | 38 | 1431.344 | 1641.69 | 12.81275 |
| H2_FlyingBirds3 | 17 | 23 | 11074.97 | 13218.77 | 16.21786 |
|  | 22 | 28 | 5253.915 | 5670.296 | 7.343193 |
|  | 27 | 33 | 2620.139 | 2704.947 | 3.135292 |
|  | 31 | 37 | 1545.958 | 1578.966 | 2.090481 |
| H2_PeopleInShoppingCenter2 | 18 | 24 | 7952.646 | 10269.89 | 22.56349 |
|  | 23 | 29 | 4228.147 | 4925.173 | 14.15231 |
|  | 27 | 33 | 2544.221 | 2824.984 | 9.938577 |
|  | 31 | 37 | 1530.141 | 1650.245 | 7.277951 |
| H2_SunsetBeach3 | 18 | 24 | 30744.32 | 56336.91 | 45.42775 |
|  | 22 | 28 | 16639.69 | 27179.63 | 38.77884 |
|  | 27 | 33 | 6922.642 | 10214.05 | 32.22435 |
|  | 31 | 37 | 3168.453 | 4253.394 | 25.50765 |

Table 10 – 12 contain results for Test6 QP_BL=QP_EL – 7.

Table 10. BD-rate results of Test6 (VVC ML, QP_BL=QP_EL – 7) vs Test4 (VVC SL) as anchor for HDR content coded with VTM-21p0 @RA cfg.

| Sequence | Random Access, Over VTM-21.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNRL100 | wPsnrY |  | wPsnrU | wPsnrV | psnrY |  | psnrU | psnrV |
| H3_SparksWelding | 30.20% | 0.44% | -0.99% |  | 63.98% | 72.73% | -1.69% |  | 63.93% | 69.79% |
| H3_SparksDirtLot | 31.83% | 22.33% | 24.14% |  | 48.63% | 50.59% | 22.93% |  | 50.11% | 50.77% |
| H3_MeridianHDR5 | 16.45% | 4.45% | 6.46% |  | 47.88% | 63.22% | 6.91% |  | 47.33% | 64.41% |
| H3_MeridianHDR2 | 21.55% | -2.29% | -2.20% |  | 55.79% | 68.55% | -2.43% |  | 54.36% | 70.03% |
| H3_ChimeraHDR5 | 41.17% | 14.10% | 13.46% |  | 107.12% | 67.48% | 12.17% |  | 98.79% | 56.63% |
| Overall | 28.24% | 7.81% | 8.17% |  | 64.68% | 64.51% | 7.58% |  | 62.90% | 62.33% |
| Sequence | Random Access, Over VTM-21.0 |  |  |  |  |  |  |  |  |  |
|  | psnrY |  |  | psnrU |  |  |  | psnrV |  |  |
| H2_DayStreet2 | 37.08% |  |  | 46.92% |  |  |  | 46.19% |  |  |
| H2_FlyingBirds3 | 6.14% |  |  | 20.88% |  |  |  | 23.16% |  |  |
| H2_PeopleInShoppingCenter2 | 19.17% |  |  | 37.45% |  |  |  | 39.50% |  |  |
| H2_SunsetBeach3 | 25.95% |  |  | 51.51% |  |  |  | 57.96% |  |  |
| Overall | 22.09% |  |  | 39.19% |  |  |  | 41.70% |  |  |

Table 11. BD-rate results of Test6 (VVC ML, QP_BL=QP_EL – 7) vs Simulcast as anchor for HDR content coded with VTM-21p0 @RA cfg.

| Sequence | Random Access, Over VTM-21.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU |  | wPsnrV | psnrY |  | psnrU | psnrV |
| H3_SparksWelding | -33.19% | -48.31% | -49.02% | -16.01% |  | -11.92% | -49.38% |  | -16.06% | -13.38% |
| H3_SparksDirtLot | -26.86% | -32.14% | -31.14% | -17.89% |  | -17.14% | -31.83% |  | -16.84% | -17.02% |
| H3_MeridianHDR5 | -39.65% | -45.91% | -44.91% | -22.99% |  | -15.27% | -44.72% |  | -23.27% | -14.64% |
| H3_MeridianHDR2 | -37.81% | -50.03% | -50.03% | -20.13% |  | -13.75% | -50.13% |  | -20.78% | -12.99% |
| H3_ChimeraHDR5 | -25.87% | -40.03% | -40.37% | 8.78% |  | -12.53% | -41.04% |  | 4.41% | -18.18% |
| Overall | -32.68% | -43.28% | -43.09% | -13.65% |  | -14.12% | -43.42% |  | -14.51% | -15.24% |
| Sequence | Random Access, Over VTM-21.0 |  |  |  |  |  |  |  |  |  |
|  | psnrY |  |  |  | psnrU |  |  | psnrV |  |  |
| H2_DayStreet2 | -24.68% |  |  |  | -20.19% |  |  | -19.74% |  |  |
| H2_FlyingBirds3 | -46.70% |  |  |  | -39.10% |  |  | -38.11% |  |  |
| H2_PeopleInShoppingCenter2 | -37.40% |  |  |  | -27.82% |  |  | -26.76% |  |  |
| H2_SunsetBeach3 | -26.67% |  |  |  | -12.19% |  |  | -8.70% |  |  |
| Overall | -33.86% |  |  |  | -24.83% |  |  | -23.33% |  |  |

Table 12. Bitrates used for BL, total bitrate and EL bitrate percentage used for sequences from Test6 (VVC ML, QP_BL=QP_EL – 7).

| Sequence | QP_BL | QP_EL | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BL bitrate, kbps | Total bitrate, kbps | EL bitrate, % |
| H3_SparksWelding | 17 | 24 | 18421.09 | 20299.85 | 9.255047 |
|  | 22 | 29 | 9017.834 | 9627.946 | 6.336887 |
|  | 26 | 33 | 4790.644 | 5135.328 | 6.712015 |
|  | 31 | 38 | 1978.518 | 2107.478 | 6.119199 |
| H3_SparksDirtLot | 17 | 24 | 7668.302 | 9972.841 | 23.10814 |
|  | 22 | 29 | 2809.514 | 3500.824 | 19.74708 |
|  | 27 | 34 | 1134.91 | 1364.881 | 16.84918 |
|  | 31 | 38 | 599.2816 | 691.8992 | 13.386 |
| H3_MeridianHDR5 | 17 | 24 | 4634.339 | 5426.111 | 14.59189 |
|  | 22 | 29 | 1851.643 | 2046.918 | 9.539925 |
|  | 26 | 33 | 889.596 | 964.3584 | 7.752553 |
|  | 31 | 38 | 379.4776 | 409.9608 | 7.435638 |
| H3_MeridianHDR2 | 17 | 24 | 3519.657 | 3719.082 | 5.362206 |
|  | 22 | 29 | 1500.657 | 1577.261 | 4.856774 |
|  | 26 | 33 | 764.4312 | 815.6264 | 6.276795 |
|  | 31 | 38 | 340.2888 | 372.3528 | 8.611188 |
| H3_ChimeraHDR5 | 17 | 24 | 20458.96 | 23643.56 | 13.46918 |
|  | 22 | 29 | 8861.619 | 9867.028 | 10.18958 |
|  | 27 | 34 | 3939.442 | 4326.135 | 8.938528 |
|  | 31 | 38 | 2086.096 | 2234.537 | 6.643023 |
| H2_DayStreet2 | 18 | 25 | 11965.22 | 16775.49 | 28.6744 |
|  | 23 | 30 | 5343.021 | 6740.517 | 20.73277 |
|  | 27 | 34 | 2915.984 | 3421.797 | 14.78208 |
|  | 31 | 38 | 1629.146 | 1811.099 | 10.04658 |
| H2_FlyingBirds3 | 17 | 24 | 11074.97 | 12207.41 | 9.276624 |
|  | 22 | 29 | 5253.915 | 5480.754 | 4.138818 |
|  | 26 | 33 | 3008.139 | 3075.274 | 2.183038 |
|  | 31 | 38 | 1545.958 | 1571.101 | 1.600305 |
| H2_PeopleInShoppingCenter2 | 18 | 25 | 7952.646 | 9654.739 | 17.62961 |
|  | 22 | 29 | 4768.387 | 5387.806 | 11.49669 |
|  | 27 | 34 | 2544.221 | 2735.76 | 7.001316 |
|  | 31 | 38 | 1530.141 | 1607.614 | 4.819166 |
| H2_SunsetBeach3 | 17 | 24 | 34857.72 | 59586.86 | 41.501 |
|  | 22 | 29 | 16639.69 | 24681.2 | 32.58153 |
|  | 26 | 33 | 8334.95 | 11287.17 | 26.15553 |
|  | 31 | 38 | 3168.453 | 3845.589 | 17.60812 |

Conclusions

LD results overall are worse than RA. This might be due to multiple reasons:

- Relation between QP_BL and QP_EL used in RA configuration (e.g., equal to 6 or 7) may not be optimum for LB configuration.

- In the LD configuration, the first picture in GOP with LId>0 is still Intra coded. In RA, it is Inter coded.

- Reference picture from Layer Id 0 is always put to the first position in both RPLs (same as for RA configuration), which may not be optimum for LB configuration.

HDR content needs more investigation. Also, it is recommended to arrange an informal viewing of HDR content, if possible. The question is whether there are still visible artifacts caused by the ML coding. Sequences with largest loss in chroma component are of particular interest.

References

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

F. Bossen, X. Li, V. Seregin, K. Sharman, K. Sühring “VTM and HM common test conditions and software reference configurations for SDR 4:2:0 10 bit video”, JVET-AB2010, Mainz, DE,  Dec. 2022.

O. Chubach C.-Y. Chen T.-D. Chuang Y.-W. Chen C.-W. Hsu Y.-W. Huang S.-M. Lei “Objective Results of VVC Multilayer Reference Implementation (VTM-21.0) on SDR UHD (4K) Content from JVET CTC”, JVET-AH0074, Rennes, FR, April 2024.

A. Segall E. François W. Husak S. Iwamura D. Rusanovskyy , “VTM and HM common test conditions and evaluation procedures for HDR/WCG video”, JVET-AC2011, Teleconference, Jan. 2023.

V. Baroncini M. Wien, “VVC verification test report for high dynamic range video content”, JVET-W2020, Teleconference, Jul 2021.

Patent rights declaration(s)

MediaTek Inc. does not have any current or pending patent rights relating to the technology described in this contribution.