| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0078-v1 |
| --- | --- |

| Title: | AHG9: Design for preservation information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chul Keun Kim Hendry Tan Jangwon Lee Junghak Nam Jaehyun Lim Seung hwan Kim | Email: | chulkeun.kim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

- Abstract

This contribution proposes an SEI message that contains information about NAL units in the coded bitstream that are recommended to be preserved / not to be removed. NAL unit types that may be desirable to be preserved may include, for example, access unit delimiter (AUD) NAL unit, filler data NAL unit, and certain types of SEI message. The proposed SEI message contains the following information:

- A list of NAL unit types that are recommended to be not removed.

- If the list of the NAL unit types includes SEI NAL unit type, additional list of SEI payload types that are recommended not to be removed may be present.

Introduction

A coded bitstream may go through one or more network nodes in which removal of some data from the bitstream might be performed. Some of such removal may be performed based on the type of data in the bitstream. Such data may include, for example, access unit delimiter (AUD) NAL unit, filler data NAL unit, certain types of SEI message, and so forth.

In the case of AUD NAL unit, while many systems now do not really need AUD to be present in the coded bitstream, some older systems may still requires their presence. It would be an issue if the bitstream goes through network node that removes AUD NAL units but the bitstream later needs to be handled / processed by a system that requires AUD to be present.

Similarly, for filler data NAL unit, while it may not be always needed, it may be present in the bitstream for system that operates with constant bitrate. For such situation, preserving filler data NAL unit is important.

In the case of SEI messages, they may include not only information necessary for applications but also information related to regulations. It would be important / desirable if such SEI messages are not removed during the distribution and reproducing of content. Removal of such SEI messages from a bitstream could not only cause issues in the operation of applications but also lead to unintentional regulatory violations. The proposed SEI messages suggest a mechanism to ensure the preservation of crucial NAL unit and SEI payload types during content distribution, thereby maintaining regulatory compliance and application functionality.

It is asserted that it would be beneficial if encoder can include information that recommends NAL units that need to be preserved (i.e, not to be removed).

Use case

The following use-cases show scenarios where the proposed method for preserving SEI messages may be needed.

AI-Generated Content

Content generated by artificial intelligence must include a disclosure of AI generation as required by the EU AI Act. Content generated by AI can be disclosed using the AI marking information in the TDI SEI message. This SEI message must be preserved during transcoding to ensure regulatory compliance and prevent unnecessary regulatory violations.

Copyright Protection

When copyrighted video content is distributed on online platforms, copyright information must be preserved. Copyright can be defined in the TDI SEI message. This SEI message must be preserved during transcoding to ensure that copyright protection is maintained and to prevent unauthorized copying and use.

Mobile Content Exchange

When content is exchanged via mobile devices, transcoding is often applied to reduce quality and resolution due to bandwidth limitations and communication costs. If the content creator defines information that must not be removed, the transcoder can either automatically retain this information or prompt the user to retain it. This ensures regulatory compliance and the original intent of the content creator.

Proposal

Syntax:

| preservation_information( payloadSize ) { | Descriptor |
| --- | --- |
| pi_num_nal_types | ue(v) |
| for( i = 0; i < pi_num nal_types; i++ ) { |  |
| pi_nal_type[ i ] | ue(v) |
| if(pi_nal_type[ i ] = = PREFIX_SEI_NUT ¦¦ pi_nal_type[ i ] = = SUFFIX_SEI_NUT ) { |  |
| pi_sei_payload_info_present_flag | u(1) |
| if( pi_sei_ payload_info_present_flag ){ |  |
| pi_num_sei_payload_minus1 | ue(v) |
| for( j = 0; j <= pi_num_sei_payload_minus1; j++ ) |  |
| pi_sei_payload_type[ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |

Semantics:

Preservation information SEI message provides information NAL units in the CLVS that are not to be removed. The identification of NAL unit that are not to be removed is first given by NAL unit type and further for SEI NAL unit type, it may further include SEI payload type.

When preservation information SEI message is present in the CLVS, it shall be present in the first access unit in the CLVS.

pi_num_nal_types specifies the number of NAL unit types that are not to be removed. The value of pi_num_nal_types ranges from 0 to 255, inclusive.

pi_nal_type[ i ] speicifies the i-th NAL unit type that is not to be removed. When the i-th NAL unit type is one of SEI NAL unit types, pi_sei_payload_info_present_flag is present.

pi_sei_payload_info_present_flag, when present, equal 1 indicates that pi_num_sei_payload_minus1 is present. pi_sei_payload_info_present_flag equal 0 indicates pi_num_sei_payload_minus1 is not present. When pi_sei_payload_info_present_flag is present and its value is equal to 0, all SEI messages in the CLVS are not to be removed.

pi_num_sei_payload_minus1 plus 1 specifies the number of SEI payload types that are not to be removed. The value of pi_num_sei_payload_minus1 shall be in the range from 0 to 255, inclusive.

pi_sei_payload_type[ j ] specifies the j-th SEI payload type that is not to be removed.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).