| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JT, 12–19 July 2024 | Document: JVET-AI0093-v2 |
| --- | --- |

| Title: | EE1-4.2: On simplified super resolution model |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chaoyi Lin1, Yue Li2, Junru Li2, Kai Zhang2, Li Zhang2 1Bafangcheng 9th building F14 Yuhang District, 311100, Hangzhou City, Zhejiang Province, China 28910 University Center Lane San Diego, CA 92122, USA | Email: | linchaoyi.cy@bytedance.com yue.li@bytedance.com lijunru@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports the EE1-4.2 test results where a simplified super resolution (SR) model with 5.3 kMAC/pixel is proposed. The SR model (SR + LOP3 LF + NN intra) reportedly shows on average {-0.78%, 1.80%, 2.04%} and {-0.64%, 1.70%, 2.22%} BD-rate changes for {Y, U, V} under RA and AI configurations, respectively.

# Introduction

In NNVC 9.0, the complexity of NNVC SR model is 20 kMAC/pixel. It is desired to further decrease the complexity of SR model without losing much compression performance. A simplified super resolution (SR) model with 5.3 kMAC/pixel is proposed and tested in the EE1-4.2.

# Proposed Method

The network architecture of proposed SR model and hyper-parameters are illustrated in Figure 1. The chroma components are upsampled to the same size of luma components before feeding into the network. Then a 1×1 convolution is used to fuse the concatenated features. Then, the luma and chroma are splitted into different branches.

Figure 1 The SR architecture.

The network information in the inference stage is provided in Table 1.

Table 1 Network information for the proposed LOP SR model in inference stage.

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: | Intel(R) Xeon(R) Platinum 8260 CPU @ 2.40GHz |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 22.5 K |
|  | Total Parameter Number | 22.5 K |
|  | Parameter Precision (Bits) | Int 16 |
|  | Memory Parameter (MB) | 0.045 MB |
|  | Multiply Accumulate (MAC) | 5.3 kMAC/pixel |

The network information in the training stage is provided in Table 2.

Table 2 Network information for the proposed LOP SR model in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: Tesla-V100-SXM2-32GB |
|  | Framework: | PyTorch v1.11 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 60 |
|  | Batch size: | 128 |
|  | Training time: | 120h/model |
|  | Training data information: | DIV2K, BVI_DVC, TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | Turn on LOP2 and NN-intra |
|  | Loss function: | L1, L2 |

# Experimental results

The proposed SR model is tested on top of NNVC 9.0, according to the common test conditions [1]. The tested QP values are {22, 27, 32, 37, 42}.

The SR model (SR + LOP3 LF + NN intra) reportedly shows on average {-0.78%, 1.80%, 2.04%} and {-0.64%, 1.70%, 2.22%} BD-rate changes for {Y, U, V} under RA and AI configurations, respectively.

Table 3 test results for integer model under RA configuration

|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM |
| --- | --- | --- | --- | --- | --- | --- |
| Class A1 | -3.06% | 0.64% | 2.13% | -5.90% | -0.27% | 0.45% |
| Class A2 | -0.82% | 8.35% | 8.07% | -5.16% | 9.76% | 8.12% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E |  |  |  |  |  |  |
| Overall | -0.78% | 1.80% | 2.04% | -2.21% | 1.90% | 1.71% |

Table 4 test results for integer model under AI configuration

|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM |
| --- | --- | --- | --- | --- | --- | --- |
| Class A1 | -1.36% | 3.31% | 4.22% | -3.32% | 4.24% | 5.43% |
| Class A2 | -2.47% | 6.88% | 9.10% | -8.28% | 7.29% | 9.22% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Overall | -0.64% | 1.70% | 2.22% | -1.93% | 1.92% | 2.44% |

# Conclusions

This contribution reports the performance result of EE1-4.2. The results show that the most coding gain of the SR model can be kept with complexity reduced from 20 to 5.3 kMAC/pixel. It is suggested to adopt it in the NNVC software.

# References

[1] E. Alshina, R.-L. Liao, S. Liu, A. Segall, “Common test conditions and evaluation procedures for neural network-based video coding technology”, JVET-AH2016, Apr. 2024.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).