| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0103 |
| --- | --- |

| Title: | EE2-3.5: Additional chained motion vector prediction candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Na Zhang1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | zhangna.cynthia@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In EE2-3.5, additional CMVP candidates are introduced, and two tests are conducted in this EE.

In EE2-3.5a, additional CMVP candidates are introduced in the pairwise merge candidate list. In EE2-3.5b, additional CMVP candidates are introduced in the initial merge candidate list.

On top of ECM-13.0, simulation results of EE2-3.5 are reported as below:

EE2-3.5a

RA: {}.

LB: { -0.10%, 0.07%, -0.02%, 99.9%, 101.0%}.

EE2-3.5b

RA: {}.

LB: { }.

# Introduction

In ECM, up to 4 pairwise candidates are added in a pairwise merge list after the initial merge candidate list is reordered by the first ARMC process. The pairwise candidates will be reordered together with the candidates in the initial merge candidate list under a second ARMC process. Then diversity reordering is performed. At last, the first N (e.g., N is 10 for regular merge mode, and 4 for TM merge mode) merge candidates of the initial merge candidate list and pairwise merge list are selected to generate the final merge candidate list.

In ECM, CMVP candidates are inserted after HMVP candidates. All the candidates before the CMVP candidates can be used as the guided vectors. CMVP may not have enough positions in the inter merge candidate list.

# Proposed methods

In EE2-3.5, additional CMVP candidates are introduced, and two tests are conducted in this EE.

In EE2-3.5a, additional CMVP candidates are introduced in the pairwise merge candidate list.

It is proposed to append some CMVP candidates in the pairwise merge candidate list. The maximum size of the pairwise merge candidate list is increased by 6 and 2 for low delay and non-low delay pictures, respectively. The maximum number of pairwise candidates is kept unchanged.

When deriving the additional CMVP candidates, only the center position of the current block is checked to find traced MVs or BVs.

In EE2-3.5b, additional CMVP candidates are introduced in the initial merge candidate list.

The first kind of additional CMVP candidates are appended after original CMVP candidates until no new CMVP candidates can be derived by using the candidates before the original CMVP candidates as guided vectors or the initial merge list is full. When deriving this kind of additional CMVP candidates, all five positions including the center, top-left, top-right, bottom-left, and bottom-right of the current block are checked to find traced MVs or BVs.

The second kind of additional CMVP candidates are inserted before the zero padding candidates. When deriving this kind of additional CMVP candidates, only the center position of the current block is checked to find traced MVs or BVs.

The maximum size of the initial merge candidate list is increased by 6 and 2 for low delay and non-low delay pictures, respectively. The maximum number of the candidates before the original CMVP candidates is kept unchanged.

# Experimental Results

The proposed methods have been implemented on top of the ECM-13.0 [1]. Testing results of EE2-3.5a and EE2-3.5b under RA/LB configurations are summarized in Table 1 and Table 2. The simulations were performed following the common test conditions [2]. More detailed results can be found in the attached spreadsheets.

Table 1: Coding performance of EE2-3.5a.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.00% | 0.03% | -0.12% |  |  |  |  |
| Class C | -0.01% | -0.07% | 0.11% |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.01% | 0.08% | 0.05% |  |  |  |  |
| Class F | -0.01% | 0.03% | 0.02% |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.09% | -0.24% | -0.21% | 100.2% | 100.7% | 100.0% | 100.4% |
| Class C | -0.14% | 0.15% | -0.16% | 99.7% | 101.6% | 100.0% | 100.0% |
| Class E | -0.04% | 0.50% | 0.50% | 99.8% | 100.6% | 100.1% | 100.0% |
| Overall | -0.10% | 0.07% | -0.02% | 99.9% | 101.0% | 100.0% | 100.1% |
| Class D | -0.28% | -0.01% | -0.49% | 99.8% | 101.2% | 100.3% | 100.0% |
| Class F | -0.16% | -0.27% | -0.22% | 100.1% | 101.2% | 100.2% | 100.0% |

Table 2: Coding performance of EE2-3.5b.

- Conclusions

In EE2-3.5, additional CMVP candidates are introduced. The proposed method can bring some coding gains with reasonable coding time change. It is recommended to adopt EE2-3.5a into the next version of ECM.

# References

[1] https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-13.0

[2] M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” JVET-AF2017, Oct. 2023.

- Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).