| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0104 |
| --- | --- |

| Title: | EE2-2.2: TIMD with enhanced block vectors deployment |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser Ya Chen Fabrice Le Léannec Miloš Radosavljević Kevin Reuzé (InterDigital) Jiaye Fu Jiaqi Zhang Siwei Ma (PKU) Ying Gao Cheng Huang(ZTE) | Tel: Email: | karam.naser@InterDigital.com jiaye.fu@ucdconnect.ie |
| Source: | InterDigital, PKU, ZTE |  |  |

_____________________________

# Abstract

This contribution reports the test results of EE2-2.2 test on TIMD with enhanced block vectors deployment. This test studies the use of block vectors (BVs) in TIMD mode with the extension of BV generation through the auto-relocated block vector prediction (ARBVP) process. The following test results are reported on top of ECM-13 anchor:

AI: -0.04%	0.00%	-0.06%	100.4%	101.1% 100.3% 100.2%

RA: -0.02%	0.00%	-0.13%	99.6%	98.6% 100.0% 100.1%

# Introduction

In addition to IBC and IntraTMP modes, the block-vector-based prediction in ECM is further used in combination with directional prediction. Specifically, in JVET-AG0152 it is used with SGPM and in JVET-AG0146 it is used with DIMD. In both modes, the merge candidates (adjacent and non-adjacent) are evaluated with the template cost to select the best block vector predictor to be combined with the directional modes.

In JVET-AH0274, it is asserted that TIMD can further benefit from the block vectors prediction in the same manner as SGPM and DIMD modes. Specifically, any of the three fusion modes of TIMD can be replaced by a block-vector-based prediction from the merge candidates based on the template cost. Additionally, the merge candidates list of IntraTMP can be further expanded with the new process of auto-relocated block vector prediction. However, these new additional candidates are not yet used for DIMD. Eventually, they can also be used for TIMD mode with block vectors.

# Proposed Method

This EE test is based on the method proposed in JVET-AH0274. Specifically, the following is proposed:

- Allow TIMD with block vectors: When deriving the intra modes of TIMD (2 angular and 1 non-angular), the block vectors of the merge candidates of IntraTMP mode are also checked. If the template cost is smaller than any of the other modes, the mode is replaced by the block vector.

- Use ARBVP merge list in DIMD and TIMD: The additional block vector candidates generated by ARBVP process are included in the merge list of DIMD and TIMD, in the same manner as in IntraTMP.

# Experimental Results

The method is implemented on top of ECM-13.0. The following results are obtained:

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.02% | 0.00% | -0.02% | 100.4% | 100.8% | 98.7% | 100.5% |
| Class A2 | -0.07% | 0.03% | -0.05% | 99.6% | 97.3% | 98.6% | 100.3% |
| Class B | -0.04% | -0.08% | -0.12% | 99.9% | 101.1% | 103.3% | 100.2% |
| Class C | -0.02% | 0.04% | 0.00% | 100.9% | 104.7% | 98.7% | 100.0% |
| Class E | -0.06% | 0.04% | -0.11% | 101.1% | 100.8% | 100.9% | 100.1% |
| Overall | -0.04% | 0.00% | -0.06% | 100.4% | 101.1% | 100.3% | 100.2% |
| Class D | -0.01% | -0.03% | -0.04% | 100.8% | 103.9% | 99.8% | 100.0% |
| Class F | 0.00% | 0.19% | 0.10% | 99.1% | 97.9% | 99.9% | 99.9% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.02% | -0.14% | -0.18% | 101.0% | 98.9% | 100.1% | 100.0% |
| Class A2 | -0.04% | -0.05% | -0.01% | 98.3% | 98.7% | 100.5% | 100.1% |
| Class B | -0.03% | 0.04% | -0.30% | 100.1% | 98.5% | 100.0% | 100.1% |
| Class C | 0.00% | 0.10% | 0.03% | 98.8% | 98.3% | 99.6% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.02% | 0.00% | -0.13% | 99.6% | 98.6% | 100.0% | 100.1% |
| Class D | 0.07% | 0.02% | 0.43% | 100.9% | 100.3% | 100.1% | 100.0% |
| Class F | 0.11% | 0.27% | -0.15% | 99.6% | 99.3% | 100.0% | 100.0% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).