| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JT, 12–19 July 2024 | Document: JVET- AI0105-v2 |
| --- | --- |

| Title: | Wavelet transform for super-resolution loss function |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jiedong Ye, Kejun Wu, Qiong Liu | Email: | ye_jd@huts.edu.cn q.liu@hust.edu.cn |
| Source: | Huazhong University of Science and Technology |  |  |

_____________________________

- Abstract

In this contribution, we propose a method to further improve the coding performance of the super-resolution (SR) model without any change of the model architecture. Adding the loss of the high-frequency content of the image by using the discrete wavelet transform (DWT) in the loss function in training can improve the model's performance. And we reduced the model complexity proposed in JVET-AH0133 [1] to 5.2kMAC/pixel.

All tests are based on the nnvc_anchor, activate SR model with NN tools (NNintra + LOP3) and calculate BD-rates savings compared to NNVC-9.0-nnvc_anchor.

For the model trained by DWT loss are shown { -0.79%, 2.15%, 2.20% } and { -0.70%, 1.92%, 2.14% } BD-rate savings under RA and AI, respectively.

Introduction

Most of the recent super resolution contributions use L1 or L2 loss between ground truth and SR image as a loss function. In this contribution, we perform a DWT transform of the ground truth image and the SR image output from the network before calculating the loss. This does not require any changes to the network structure or training data.

Proposed method

Loss with DWT

By using the DWT transform, the loss is computed directly in the low and high frequency domains to improve the focus of the neural network on the high frequency part. We use L1 loss.

The network architecture is based on JVET-AH0133. Model complexity has reduced to 5.2kMAC/pixel.

Figure 1 loss with DWT

Figure 2 network architecture

Training stage information

Table 1 Network Information for our LOP SR model

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: GeForce RTX 3090 |
|  | Framework: | PyTorch v1.9.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | ~1000 |
|  | Batch size: | 32 |
|  | Loss function: | L1 |
|  | Training time (for 1 model): | ~3 days |
|  | Training data information: | TVD_4K, BVI-DVC_4K, DIV2K |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | NNVC-8.0 with NN tools on, QP{17, 22, 27, 32, 37, 42} |
| Optional | Number of iterations |  |
|  | Patch size | 72*72 |
|  | Learning rate: | 2e-4 |
|  | Learning rate update strategy | Reduced 3 times in the later part of training |
|  | Optimizer: | ADAM |
|  | Preprocessing: | flip, rotation |
|  | Mini-batch selection process: | random cropping |
|  | Training data update strategy: |  |
|  | Other information: |  |

Inference stage information

Table 3 Inference information for our LOP SR model

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | Not used |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 0.02M |
|  | Total Number of Parameters (All Models) | 0.02M |
|  | Parameter Precision (Bits) | float |
|  | Memory Parameter (MB) | 0.1 |
|  | Multiply Accumulate (MAC/pixel) | 5.2 K |
|  | Calculation Method | thop |
| Optional | Total Conv. Layers |  |
|  | Total FC Layers |  |
|  | Total Memory (MB) |  |
|  | Batch size: |  |
|  | Patch size | 256*256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

Experiment results

This section will show the coding performance in the AI and RA. The model trained with DWT loss is tested on top of NNVC 8.0, according to the common test conditions [2]. The tested QP values are {22, 27, 32, 37, 42}.

For comparison, we also show the performance of the original model trained with only L1 loss.

Model trained with DWT loss function

Our LOP SR model trained with DWT loss function shows on average { -0.79%, 2.15%, 2.20% } and { -0.70%, 1.92%, 2.14% } BD-rate changes for {Y, U, V} under RA and AI configurations, respectively.

Table 4 Results of SR model trained with DWT loss function in RA configurations

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.0 nnvc |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM |
| Class A1 | -3.10% | 2.30% | 3.09% | -6.01% | -0.60% | 0.38% |
| Class A2 | -0.85% | 8.46% | 7.89% | -5.29% | 9.74% | 7.78% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E |  |  |  |  |  |  |
| Overall | -0.79% | 2.15% | 2.20% | -2.26% | 1.83% | 1.63% |

Table 5 Results of SR model trained with DWT loss function in AI configurations

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.0 nnvc |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM |
| Class A1 | -1.35% | 4.71% | 4.96% | -3.37% | 3.60% | 5.41% |
| Class A2 | -2.87% | 6.83% | 7.88% | -8.67% | 7.67% | 8.35% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Overall | -0.70% | 1.92% | 2.14% | -2.01% | 1.88% | 2.29% |

Model trained with only L1 loss

Model trained with only L1 loss reportedly shows on average { -0.80%, 4.04%, 3.80% } and { -0.69%, 3.28%, 3.50% } BD-rate changes for {Y, U, V} under RA and AI configurations, respectively.

With DWT loss the performance has been improved as follows:

AI: {0.01%, -1.89%, -1.6%}

RA: {-0.01%, -1.36%, -1.36%}

Table 6 Results of SR model trained with only L1 loss in RA configurations

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.0 nnvc |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM |
| Class A1 | -3.15% | 10.39% | 10.26% | -5.97% | 0.21% | 1.13% |
| Class A2 | -0.84% | 9.82% | 8.74% | -5.25% | 10.11% | 8.04% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E |  |  |  |  |  |  |
| Overall | -0.80% | 4.04% | 3.80% | -2.24% | 2.06% | 1.83% |

Table 7 Results of SR model trained with only L1 loss in AI configurations

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.0 nnvc |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM |
| Class A1 | -1.39% | 10.10% | 10.77% | -3.37% | 4.06% | 5.73% |
| Class A2 | -2.78% | 9.60% | 10.23% | -8.53% | 8.19% | 8.50% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Overall | -0.69% | 3.28% | 3.50% | -1.98% | 2.04% | 2.37% |

Conclusion

In this contribution, we propose to use DWT in loss function. The performance is improved without changing any network structure or training data. And we reduce the model complexity from 20.2kMAC/pixel to 5.2kMAC/pixel.

Our SR model shows on average { -0.79%, 2.15%, 2.20% } and { -0.70%, 1.92%, 2.14% } BD-rate changes for {Y, U, V} under RA and AI configurations, respectively.

By introducing this loss function into our model, the performance increased {0.01%, -1.89%, -1.6%}, {-0.01%, -1.36%, -1.36%} in AI and RA configuration.

Reference

[1] Jiedong Ye, Xu Li, Yiqing Zhu, Qiong Liu, Chuan Zhou, M. Rafie, Zhuoyi Lv “EE1-4.3: RPR new input for super-resolution”, JVET-AH0133

[2] E. Alshina, R.-L. Liao, S. Liu, A. Segall, “Common test conditions and evaluation procedures for neural network-based video coding technology”, JVET-AF2016.

Patent rights declaration(s)

vivo Mobile Communication Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).