| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0113-v2 |
| --- | --- |

| Title: | EE2-2.4a/b: Additional TIMD mode with different cost metric |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Döne Bugdayci Sansli Saverio Blasi Jani Lainema | Email: | done.bugdayci_sansli@nokia.com saverio.blasi@nokia.com jani.lainema@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution presents simulation results for the EE2 tests 2.4a and 2.4b on additional TIMD mode with different cost metric, with and without increased template size. The proposed method is an additional TIMD based intra prediction mode, signaled as a sub-mode of TIMD. The mode derivation part of the proposed mode differs from TIMD, namely using a different cost metric, a different template size, and a different set of candidate modes. It is reported that the performance of the proposed method against ECM-13.0 anchor is measured as:

2.4a -	AI { -0.06, % 0.03, % 0.01, 100.6%,	100.9%,100.1%,100.0%

RA {%, %, %, %, %, %, %}

2.4b - 	AI -0.03 %	0.02 %	0.03 % %, %, %, %}

# Introduction

ECM-13.0 has template based intra mode derivation (TIMD) prediction mode, where up to 3 intra mode predictors are blended with associated weights [1]. For a set of candidate modes (computed using the MPM list), the SATD between the prediction and reconstruction samples of the template is calculated. The template is the L-shaped area around the current block. It is formed of 2 or 4 lines/columns around the current block, determined based on the width or height of the current block.

When more than one TIMD modes are selected, prediction fusion is used to blend the corresponding predictors, based on the calculated SATD cost of the candidate modes. The two intra prediction modes with the minimum SATD, and up to one non-angular intra prediction mode (i.e. DC or Planar) with the lowest SATD cost are selected as TIMD modes. These up to three TIMD modes are fused, where blending weights are determined based on SATD costs of the selected modes.

# Proposed Method

In this test, an additional TIMD based intra mode is proposed as in JVET-AH0083 [2]. The mode derivation process of the proposed mode differs from the regular TIMD mode in that the cost metric is changed from SATD to MR SAD, the number of template lines/columns around the current block are doubled to 4/8, and the list of TIMD candidate modes is modified to avoid redundancy with respect to the conventional TIMD mode.

The proposed method is tested in two subtests where Test 2.4a includes the modifications to the template size, whereas Test 2.4b uses the same template size as in conventional TIMD.

The new TIMD based intra mode is signaled as a sub mode of TIMD with a context coded flag, where certain block size restrictions are applied. In particular, the method is disabled for a block whose minimum size (either width or height) is equal to 4. Also, the candidate list is determined based on the sorted modes determined by the conventional TIMD process. An additional refinement to the candidate list is used based on the DIMD modes.

# Experimental Results

Simulation results for the tests a and b are reported against ECM-13.0 anchor.

Table 1 Simulation results for Test 2.4a, All Intra configuration

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.09 % | 0.07 % | -0.04 % | 101.0% | 101.3% | 100.0% | 100.0% |
| Class A2 | -0.04 % | 0.02 % | 0.04 % | 100.8% | 101.0% | 100.3% | 99.9% |
| Class B | -0.06 % | -0.03 % | -0.04 % | 100.9% | 101.0% | 100.0% | 100.0% |
| Class C | -0.05 % | 0.06 % | 0.07 % | 100.3% | 100.3% | 100.0% | 100.0% |
| Class E | -0.06 % | 0.05 % | 0.03 % | 100.1% | 101.3% | 100.0% | 100.0% |
| Overall | -0.06 % | 0.03 % | 0.01 % | 100.6% | 100.9% | 100.1% | 100.0% |
| Class D | -0.01 % | -0.01 % | -0.18 % | 100.4% | 102.0% | 99.9% | 100.0% |
| Class F | -0.04 % | 0.15 % | 0.19 % | 101.2% | 100.9% | 99.2% | 100.0% |

Table 2 Simulation results for Test 2.4a, Random Access configuration

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.04 % | -0.10 % | 0.05 % | 100.7% | 100.5% | 100.1% | 99.9% |
| Class C | -0.01 % | 0.09 % | 0.28 % | 100.4% | 99.6% | 100.4% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.04 % | -0.26 % | 0.03 % | 100.4% | 100.1% | 99.9% | 100.0% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

Table 3 Simulation results for Test 2.4b, All Intra configuration

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.05 % | -0.09 % | 0.11 % |  |  |  |  |
| Class A2 | -0.02 % | -0.02 % | 0.06 % |  |  |  |  |
| Class B | -0.03 % | -0.03 % | 0.00 % |  |  |  |  |
| Class C | -0.02 % | 0.06 % | 0.06 % |  |  |  |  |
| Class E | -0.03 % | 0.16 % | -0.07 % |  |  |  |  |
| Overall | -0.03 % | 0.02 % | 0.03 % |  |  |  |  |
| Class D | -0.02 % | 0.17 % | -0.21 % |  |  |  |  |
| Class F | -0.02 % | 0.04 % | 0.08 % |  |  |  |  |

# Conclusion

This document reports simulation results for EE tests 2.4a/b and it is proposed that test 2.4a is adopted into next version of ECM.

# References

- M. Coban, R.-L. Liao, K. Naser, J. Ström, L. Zhang, Algorithm description of Enhanced Compression Model 13 (ECM 13), JVET-AH2025, 2024/04, Rennes

- D. Bugdayci Sansli, S. Blasi, J. Lainema (Nokia), AHG12: Additional TIMD mode with different cost metric, JVET-AH0083, 2024/04, Rennes

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).