| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0119-v2 |
| --- | --- |

| Title: | AHG12: Context modeling for tu_cb_coded_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minzhe Chen, Lu Wang, Wei Zhang, Fuzheng Yang, Junyan Huo Zhitao Yu, Wei Li, Bin Li, Fang Xing | Email: | mzchen2@stu.xidian.edu.cn wanglu2@stu.xidian.edu.cn wzhang@xidian.edu.cn fzhyang@mail.xidian.edu.cn jyhuo@mail.xidian.edu.cn yuzhitao@hisense.com liweigf1@hisense.com libin19@hisense.com xingfang@hisense.com |
| Source: | Xidian University, Hisense Visual Tech. Co.Ltd. |  |  |

_____________________________

- Abstract

This contribution proposes to modify the context modeling of tu_cb_coded_flag. tu_cb_coded_flag equals 1 specifies that the Cb transform block contains one or more transform coefficient levels not equal to 0. This contribution derives the tu_cb_coded_flag ctxIdx based on the tu_cb_coded_flag of the left and above adjacent TU. It is reported that on the above of ECM-13.0, the overall coding performance impact for {Y, U, V} is {0.00 %,-0.22% -0.05%} in AI configuration and {0.xx%, 0.xx%, 0.xx%} in RA configuration.

Introduction

The syntax tu_cb_coded_flag represents whether the Cb transform block contains one or more transform coefficient levels not equal to 0 (tu_cb_coded_flag = 1) or not (tu_cb_coded_flag = 0).

The syntax intra_bdpcm_chroma_flag represents whether block-based delta pulse code modulation (BDPCM) is applied to the current chroma coding blocks (intra_bdpcm_chroma_flag = 1) or not (intra_bdpcm_chroma_flag = 0).

In ECM and VVC,  the ctxIdx of tu_cb_coded_flag depends on the value of intra_bdpcm_chroma_flag for the current TU.  For example, if initType = 0, the ctxIdx of tu_cb_coded_flag is derived in the following way:

ctxIdx = intra_bdpcm_chroma_flag ? 1 : 0

For each initType, the ctxIdx of tu_cb_coded_flag can only take two values. As is shown in Table 1. The specification of initValue and shiftIdx for ctxIdx of tu_cb_coded_flag in ECM13.0 is shown in Table 2.

Table 1 - Association of ctxIdx and syntax elements for each initializationType

| Syntax structure | Syntax element | ctxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| transform_unit( ) | tu_cb_coded_flag[ ][ ] | Table2 | 0..1 | 2..3 | 4..5 |

Table 2 - Specification of initValue and shiftIdx for ctxIdx of tu_cb_coded_flag in ECM13.0

| Initialization variable | ctxIdx of tu_cb_coded_flag |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 4 | 30 | 17 | 58 | 25 | 54 |
| shiftIdx | 5 | 4 | 5 | 0 | 5 | 0 |

Proposed method

The method in ECM and VTM only use the intra_bdpcm_chroma_flag to derive the tu_cb_coded_flag ctxIdx. It is noted that if the tu_cb_coded_flag of the left and above adjacent TU are all 0, the tu_cb_coded_flag of the current TU is equal to 0 with a high probability. If tu_cb_coded_flag of the left and above adjacent TU are all 1, the probability of tu_cb_coded_flag with 0 is lower than in the above case. Based on the observation, the spatial correlation of tu_cb_coded_flag is introduced to make the context modeling of tu_cb_coded_flag  more efficient.

In addition to the intra_bdpcm_chroma_flag of the current TU, the tu_cb_coded_flag of the left and above adjacent TU are used to derive the tu_cb_coded_flag ctxIdx.

The chroma location of current TU is (x0, y0).  It specifies the location (x0, y0) of the top-left chroma sample of the considered coding block relative to the top-left chroma sample of the picture. The location of left adjacent TU (xNbL, yNbL) is set equal to (x0-1, y0). The location of above adjacent TU (xNbA, yNbA) is set equal to (x0, y0-1). For example, if initType = 0, the tu_cb_coded_flag ctxIdx of current TU is derived as:

ctxIdx = intra_bdpcm_chroma_flag[x0][y0] ?

3 + tu_cb_coded_flag[xNbA][yNbA] + tu_cb_coded_flag[xNbL][yNbL]

: tu_cb_coded_flag[xNbA][yNbA] + tu_cb_coded_flag[xNbL][yNbL]

The derivation of tu_cb_coded_flag ctxIdx in I slice is shown in Figure 1.

Figure 1. The derivation of tu_cb_coded_flag ctxIdx in I slice

If the adjacent TU is not available, its tu_cb_coded_flag is set to 0.

For each initType, the ctxIdx of tu_cb_coded_flag has six values. As is shown in Table 3.

Table 3 - Association of ctxIdx and syntax elements for each initializationType in proposal

| Syntax structure | Syntax element | initType |  |  |
| --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 |
| transform_unit( ) | tu_cb_coded_flag[ ][ ] | 0..5 | 6..11 | 12..17 |

In this contribution, the initValue is set to CNU(35), and the shiftIdx is set to DWS(8).

Experimental  results

The proposed method is implemented on top of ECM-13.0 and tested following CTC. The test results of All Intra and Random Access configuration are summarized below.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.01% | -0.19% | -0.08% | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class A2 | 0.00% | -0.22% | 0.02% | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class B | 0.00% | -0.28% | -0.09% | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class C | 0.00% | -0.19% | -0.02% | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class E | 0.02% | -0.20% | -0.08% | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Overall | 0.00% | -0.22% | -0.05% | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class F | 0.03% | -0.22% | 0.00% | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class TGM | -0.02% | -0.16% | -0.07% | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class C | 0.05% | -0.43% | -0.10% | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class D | 0.14% | -1.29% | -0.27% | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

From the data above, the coding performance of Cb component is uniformly improved, verifying the efficient coding for tu_cb_coded_flag.

No coding time increase is expected.

- Conclusions

In this contribution, a modification about how the context modelling is done for the syntax element tu_cb_coded_flag is proposed.  Compared to ECM13.0, the contribution achieves BD-rate saving of {0.00 %(Y), -0.22%(U), -0.05%(V)} for the AI configuration and {0.xx% (Y), 0.xx% (U), 0.xx% (V)} for the RA configuration. No coding time increase is expected. It is suggested to investigate in EE.

- Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for the implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Hisense Group Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).