| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0129-v1 |
| --- | --- |

| Title: | EE2-2.6: IntraTMP candidates with overlapping refinement window enhanced |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Damian Ruiz Coll, Jung-Kyung Lee, Karam Naser, Thierry Dumas, Fabrice Le Léannec, Tangi Poirier, Hassane Guermoud. | Tel: Email: | duizcoll@ofinno.com, jlee@ofinno.com karam.naser@InterDigital.com |
| Source: | Ofinno, InterDigital |  |  |

_____________________________

# Abstract

This contribution reports test results of the EE2-2.6a test, which investigates the clustering of the IntraTMP Merge, ARBVP, and sparse candidates based on the refinement windows overlapping, and the EE2-2.6b test, which investigates, during the IntraTMP search, the shift of the grid of positions of sparse candidates using LIC with respect to the grid of positions of sparse candidates not using LIC, proposed in JVET-AH0151. Test 2.6c reports the results of the combination of both tests. Tests performance results of proposed methods, implemented on top of ECM-13.0, are as follows {for Y, U, V, EncT, DecT}:

Test 2.6a

AI: Overall {-0.04%, -0.04%, -0.05%, 100.6%, 100.4%},

RA: Overall {-0.xx%, -0.xx%, -0.xx%, 10x.x%, 10x.x%}.

Test 2.6b

AI: Overall {-0.xx%, -0.xx%, -0.xx%, 10x.x%, 10x.x%},

RA: Overall {-0.xx%, -0.xx%, -0.xx%, 10x.x%, 10x.x%}.

Test 2.6c

AI: Overall {-0.10%, -0.08%, -0.05%, 101.1%, 101.8%},

RA: Overall {-0.xx%, -0.xx%, -0.xx%, 10x.x%, 10x.x%}.

# Introduction

In ECM-13, the IntraTMP mode[1] extends the sparse candidate list with new merge candidates acquired from the spatial and non-spatial neighboring PUs [2] and Auto-relocated block vector prediction (AR-BVP) candidates [3] based on merge candidates. The 30 BVP candidates from the regular sparse, merge, and ARBVP candidates achieving a lower TMP cost are held on the enhanced sparse list for further refinement.

The refinement window around each BVP candidate varies based on the candidate type: merge candidates use an 11x11 window size, and ARBVP and sparse candidates use a 3x3 window size. The distance between the merge, ARBVP, and sparse candidates is not constrained, so the refinement windows may overlap, making the evaluation of candidates redundant and limiting the candidate list diversity.

# Proposal

## EE2-2.6a (IntraTMP merge candidates clustering based on refinement window)

In [4], a new method for clustering the IntraTMP candidates whose refinement windows overlap with other BVP candidates is proposed. This method involves testing the merge, ARBVP, and sparse candidate refinement windows overlapping. If two refinement windows overlap, the BV candidate with the lower template cost is selected and ranked in the sparse list for further refinement. Subsequently, a new refinement window that comprises both individual refinement windows is determined for the winning candidate.

Figure 1: merge and sparse candidate refinement window overlapping example.

To enhance the gain and complexity trade-off, the following processes are implemented:

- The ARBVP list is enlarged by 5 additional ARBVP candidates, using the sparse candidates instead of the merge candidates as the guiding vector.

- For block sizes 8x8 or smaller, a search region fully overlapped by a BVP candidate refinement window is skipped during sparse searching.

- In each sparse region, the searching pattern is shifted by one sample per row in order to increase the candidate diversity.

## EE2-2.6b (IntraTMP search region shifting for LIC candidates)

It is observed that an overlap may occur between the IntraTMP block vectors in non-LIC mode and those in LIC mode. Although the two use different cost metric: SAD for non-LIC mode and MR-SAD for LIC mode, the overlap may occur in the sparse search as the same starting point is used. It is therefore proposed to shift the LIC search region by half of the subsampling factor, as shown in Figure 2.

Figure 2: modified search range for metric. Positions labeled with “x” are the non-LIC positions used for the default metric (SAD). The others labeled with “o” are the ones used for the additional metric (MR-SAD). The new positions are shifted by half of the subsampling range.

For a valid position “x” within the IntraTMP search region, if its associated position “o” located on its bottom-right side corresponds to the pixel at the top-left of a candidate reconstructed block going out of the bounds of the IntraTMP search region, “o” is shifted back towards “x”.

## EE2-2.6c (Test 2.6a + Test 2.6b)

Test 2.6c combines both Test 2.6a and 2.6b at the same time.

# Experimental Results

The proposed methods are tested on top of ECM-13.0 software [5] under JVET ECM common test conditions [6] in AI and RA configurations. The tables below summarize the coding performance for the 2.6 tests. Full results are available as separate Excel spreadsheets.

## EE2-2.6a (IntraTMP merge candidates clustering based on refinement window)

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.01% | -0.09% | -0.01% | 100.6% | 100.0% | 99.5% | 99.9% |
| Class A2 | -0.03% | -0.11% | -0.02% | 100.9% | 100.3% | 99.8% | 99.8% |
| Class B | -0.04% | -0.09% | -0.14% | 100.6% | 102.5% | 102.9% | 100.0% |
| Class C | -0.03% | 0.04% | 0.12% | 100.3% | 99.4% | 101.5% | 100.0% |
| Class E | -0.09% | 0.06% | -0.19% | 100.4% | 98.9% | 101.8% | 100.0% |
| Overall | -0.04% | -0.04% | -0.05% | 100.6% | 100.4% | 101.3% | 99.9% |
| Class D | 0.00% | 0.14% | 0.10% | 102.1% | 100.5% | 100.0% | 100.0% |
| Class F | -0.06% | 0.10% | -0.07% | 103.9% | 106.6% | 101.2% | 100.0% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class B | 0.02% | -0.02% | -0.14% | 102.3% | 100.5% | 100.0% | 100.2% |
| Class C | 0.00% | 0.11% | 0.25% | 103.1% | 101.5% | 100.0% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class D | 0.02% | -0.32% | 0.24% | 103.4% | 103.9% | 99.9% | 100.0% |
| Class F | 0.00% | 0.19% | -0.23% | 103.3% | 104.0% | 99.8% | 100.1% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |

## EE2-2.6b (IntraTMP search IntraTMP search area shifting and plus-shape refinement for LIC candidates)

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.08% | -0.10% | -0.17% | 100.2% | 100.7% | 99.6% | 100.0% |
| Class C | -0.04% | -0.13% | 0.00% | 100.5% | 100.4% | 101.0% | 100.0% |
| Class E | -0.13% | 0.00% | -0.13% | 100.5% | 100.3% | 101.2% | 100.0% |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.00% | 0.07% | -0.05% | 101.0% | 100.5% | 100.0% | 100.0% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

## EE2-2.6c (Test 2.6a + Test 2.6b)

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | 0.05% | 0.10% | 101.3% | 99.4% | 99.8% | 99.9% |
| Class A2 | -0.08% | -0.07% | 0.00% | 102.0% | 102.4% | 100.5% | 99.9% |
| Class B | -0.11% | -0.09% | -0.13% | 100.6% | 103.6% | 101.5% | 100.2% |
| Class C | -0.07% | -0.08% | 0.03% | 100.5% | 101.6% | 99.5% | 100.0% |
| Class E | -0.22% | -0.18% | -0.21% | 101.6% | 100.9% | 99.2% | 100.1% |
| Overall | -0.10% | -0.08% | -0.05% | 101.1% | 101.8% | 100.2% | 100.0% |
| Class D | -0.04% | 0.02% | 0.00% | 100.0% | 99.6% | 100.0% | 100.0% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class B | -0.05% | -0.10% | -0.18% | 99.7% | 96.8% | 100.0% | 100.2% |
| Class C | 0.00% | 0.17% | 0.11% | 103.4% | 98.7% | 100.0% | 99.9% |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | -0.01% | 0.18% | 0.29% | 104.7% | 103.3% | 99.9% | 100.0% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |

# Conclusion

The experimental results showed that the combination of Test 2.6a and Test 2.6b provides a 0.10% average luma BD-Rate gain in AI configuration, with a slight runtime increase. It is suggested that the Test 2.6c method be adopted in ECM-14.0.

# References

- K. Naser, T. Poirier, F. Le Léannec, G. Martin-Cocher, “EE2: Intra Template Matching”, JVET-V0130, April 2021.

- K. Naser, M. Radosavljević, F. Le Léannec, Y. Chen, T. Dumas, T. Poirier,  " EE2-1.2: IntraTMP with Merge Candidates ", JVET-AG0151, January 2024.

- N. Qiu, J. Huo,Y. Ma, F. Yang, L. Zhang, Y. Yu, F. Wang, H. Yu, D. Wang, “EE2-2.11: Auto-relocated BVP for IntraTMP merge candidates”, JVET-AH0055, April 2024.

- D. Ruiz Coll, J.-K. Lee, K. Naser, F. Le Léannec, T. Poirier, H. Guermoud, T. Dumas, “EE2-related: IntraTMP merge candidates clustering based on refinement window”, JVET-AH0151, April 2024.

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-13.0

- M. Karczewicz and Y. Ye, "Common Test Conditions and evaluation procedures for enhanced compression tool testing," Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-AF2017, 32nd Meeting, Hannover, Oct. 2023.

# Patent rights declaration

Ofinno, LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).