| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0130-v1 |
| --- | --- |

| Title: | EE1-5.1: combination of the neural network-based intra prediction mode and ISP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | T. Dumas, F. Galpin, P. Bordes | Tel: Email: | thierry.dumas@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution reports the results of EE1-5.1. EE1-5.1 follows the contribution JVET-AH0165 presented during the 34th JVET meeting. EE1-5.1 proposes a combination of the Neural Network (NN)-based intra prediction mode and ISP inside NNVC. In addition, the objective function for training a NN for intra prediction now linearly combines the SATD between a given original block and its NN prediction and the SAD between this original block and its NN prediction as this objective function has yielded mean BD-rate gains when combining the NN-based intra prediction mode and ISP.

Compared to NNVC-9.0, the following gains are reported

- RA:  -0.06%, -0.54% and -0.43% BD-rate PSNR Y, U and V gains, respectively for a complexity of 102% and 103% at encoder and decoder respectively

- AI:   -0.07%, -0.16% and -0.13% BD-rate PSNR Y, U and V gains, respectively, for a complexity of 104% and 102% at encoder and decoder respectively.

At the same time, the worst-case complexity of the NN-based intra prediction mode is decreased from 7.8 kMAC/pixel to 4.8 kMAC/pixel.

# Introduction

NNVC [1] includes a default NN-based intra prediction mode. Moreover, JVET-AF0139 introduced a low-complexity version of the NN-based intra prediction mode, being not used by default. This contribution describes a combination of the NN-based intra prediction mode and ISP inside NNVC.

# Description

## Combination of the NN-based intra prediction mode and ISP

### Restricted combination

In the proposed combination between the NN-based intra prediction mode and ISP, ISP is allowed in luma exclusively and the same split types as in the original ISP are employed, i.e. horizontal and vertical splits. However, for a given luma CB predicted by the NN-based intra prediction mode, ISP is ruled by four additional constraints compared to the constraints specific to the original ISP.

- The luma CB can only be split into 4 equal-sized sub-partitions for both prediction and {transform coding, entropy coding of quantized transform coefficients}. For instance, a 8x8 luma CB predicted via the NN-based intra prediction mode cannot be split vertically into 2 4x8 sub-partitions for prediction and 4 2x8 sub-partitions for {transform coding, entropy coding of quantized transform coefficients}.

- A sub-partition of the luma CB cannot have a dimension strictly smaller than 4. For instance, a 4x16 luma CB predicted via the NN-based intra prediction mode cannot be split vertically via ISP into 4 1x16 sub-partitions.

- At least one of the two dimensions of the luma CB must be equal to 4.

- The luma CB must belong to a CU in separate tree.

To summarize 1), 2), 3), and 4), for a given luma CB predicted by the NN-based intra prediction mode, ISP is allowed if ispAllowed is true.

ispAllowed=!isPrelim  &&  (h≪2≥4 ∥ w≪2≥4)

isPrelim=!isSepTree∥h<4 ∥w<4 ∥h !=4  &&  w !=4

h : height of the luma CB.

w : width of the luma CB.

isSepTree : true if the luma CB is part of a CU in separate tree.

### Additional combination with LFNST

For a given luma CB predicted by a non-NN intra prediction mode, when ISP and LFNST are used, the LFNST implicit signaling is determined by the index of the intra prediction mode, and all the luma TBs arising from the ISP split share the same LFNST implicit signaling. In contrast, for a given luma CB predicted via the NN-based intra prediction mode, when ISP and LFNST are used, for each luma TB independently, the NN infers the LFNST implicit signaling, c.f. [1], for this luma TB while predicting it.

### Encoding heuristic

At the encoder side, for a given luma CB, if the combination of a non-NN intra prediction mode and ISP is tested in rate-distortion, the index of this intra prediction mode is placed at the same position within the ISP list for horizontal split and the ISP list for vertical split. This position depends notably on the index value and the ranking of this intra prediction mode during the previous rate-distortion tests not involving ISP. However, if the combination of the NN-based intra prediction mode and ISP is allowed for a given ISP split, the index of the NN-based intra prediction mode is placed first inside the ISP list for this given split. Therefore, for a given luma CB, the index of the NN-based intra prediction mode can appear in only one of the two ISP lists.

## Objective function

The objective function for training a NN for intra prediction linearly combines the SATD between a given original block and its NN prediction and the SAD between this original block and its NN prediction. Thus, for a single training example, the prediction component L of the objective function can be written as

Lx, y; θ=SATDy, fh, wx; θh,w+SADy, fh, wx; θh, w

y : original w×h block.

x : context of reference samples surrounding y.

fh, wx; θh,w : neural network predicting blocks of size w×h, parametrized by θh,w.

# AHG11 template

| Network information in training stage |  |  |
| --- | --- | --- |
| Mandatory | GPU type | GPU: Tesla-P100-16GB |
|  | Framework | Tensorflow 2.6 |
|  | Number of GPUs per task | 1 |
|  | Epoch | 15 |
|  | Batch size | 2048 |
|  | Training time | 8 hours to 3 days, depending on the training cycle. |
|  | Training data information | During the generation of the training sets for training a NN for intra prediction, pairs of a block and its context are extracted from the partitioning of YUV frames encoded/decoded via NNVC-9.0. |
|  | Training configurations for generating compressed training data | QP belonging to {18, 19, 20, 21, 22, 27, 32, 37}. |
|  | Patch size | The NN predicting w×h blocks is trained on pairs of a w×h block and its context of reference samples. |
|  | Learning rate | Decreasing from 1e-4 to 5e-8. |
|  | Optimizer | ADAM. |
|  | Loss function | See Section 2.2. |
|  | Preprocessing | See [1]. |

| Network inference (low-complexity NN-based intra prediction mode) |  |  |
| --- | --- | --- |
| Parameter number | block size | Parameter number |
|  | 4×4 8×4 16×4 32×4 8×8 16×8 16×16 | 77477 120816 182356 229048 194023 238768 291311 |
| Parameter precision | 16-bit signed integer. |  |
| MAC/pixel | block size | MACs/pixel |
|  | 4×4 8×4 16×4 32×4 8×8 16×8 16×16 | 4837 3773 2848 1789 3030 1865 1138 |
| Total conv. layers |  |  |
| Total FC layers | 4 cascaded fully-connected layers for the NN predicting 16×16 blocks. 3 cascaded fully-connected layers for the other six NNs. |  |
| Mem (MB) | 4.9 MB for the whole NN-based intra prediction mode. |  |
| GPU Type | No GPU |  |
| CPU Type | Xeon 2.4GHz |  |
| Framework | SADL (C++ with a single thread) |  |

The worst-case number in MACs/pixel of the default NN-based intra prediction mode is 7.8 kMACs/pixel. The worst-case number in MACs/pixel of the low-complexity NN-based intra prediction mode is 4.8 kMACs/pixel.

# Experimental results

NNVC-9.0 involving LOP-3, the low-complexity NN-based intra prediction mode, and the proposed combination between the NN-based intra prediction mode and ISP is run following the AHG11 CTC [2], see Table 1. The anchor corresponds to NNVC-9.0 CTC.

Table 1: NNVC-9.0 with LOP-3, the low-complexity NN-based intra prediction mode, and the proposed combination between the NN-based intra prediction mode and ISP over NNVC-9.0 CTC.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.0 CTC |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 | 0.00% | -0.05% | 0.06% | 0.07% | -0.20% | 0.01% | 104% | 103% | 100% |
| Class A2 | -0.33% | -0.44% | -0.46% | -0.24% | -0.46% | -0.45% | 106% | 102% | 100% |
| Class B | -0.09% | -0.16% | -0.12% | -0.10% | -0.30% | -0.40% | 106% | 105% | 100% |
| Class C | -0.01% | -0.10% | -0.01% | -0.05% | -0.09% | -0.05% | 101% | 99% | 100% |
| Class E | 0.09% | -0.08% | -0.15% | 0.11% | -0.01% | -0.40% | 104% | 102% | 100% |
| Overall | -0.07% | -0.16% | -0.13% | -0.05% | -0.21% | -0.26% | 104% | 102% | 100% |
| Class D | -0.02% | -0.08% | 0.16% | -0.12% | -0.29% | 0.28% | 101% | 100% | 100% |
| Class F | 0.02% | -0.02% | -0.07% | -0.05% | -0.32% | -0.16% | 103% | 104% | 100% |

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.0 CTC |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 | 0.06% | -0.19% | -0.25% | 0.03% | -0.50% | -0.48% | 101% | 101% | 100% |
| Class A2 | -0.11% | -0.75% | -0.51% | -0.12% | -0.91% | -0.76% | 103% | 103% | 100% |
| Class B | -0.08% | -0.80% | -0.48% | -0.13% | -1.14% | -0.94% | 103% | 105% | 100% |
| Class C | -0.07% | -0.32% | -0.44% | -0.10% | -0.73% | -0.89% | 102% | 102% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.06% | -0.54% | -0.43% | -0.09% | -0.86% | -0.80% | 102% | 103% | 100% |
| Class D | -0.08% | -0.94% | -0.15% | 0.03% | -1.26% | -0.65% | 102% | 103% | 100% |
| Class F | -0.08% | -0.43% | -0.36% | -0.12% | -0.59% | -0.37% | 103% | 104% | 100% |

# Additional experimental results

Besides, Table 2 shows the results of the simulations in Table 1 but replacing the anchor by NNVC-9.0 with LOP-3 and the low-complexity NN-based intra prediction mode. This means that, in the anchor, the default NN-based intra prediction mode is replaced by the low-complexity one (introduced by JVET-AF0139).

Table 2: NNVC-9.0 with LOP-3, the low-complexity NN-based intra prediction mode, and the proposed combination between the NN-based intra prediction mode and ISP over NNVC-9.0 with LOP-3 and the low-complexity NN-based intra prediction mode.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.0 with LOP-3 and the low-complexity NN-based intra prediction mode |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 | -0.31% | -0.25% | -0.04% | -0.25% | -0.32% | -0.10% | 103% | 102% | 100% |
| Class A2 | -0.31% | -0.36% | -0.25% | -0.19% | -0.21% | -0.15% | 104% | 100% | 100% |
| Class B | -0.34% | -0.24% | -0.25% | -0.35% | -0.27% | -0.36% | 105% | 101% | 100% |
| Class C | -0.70% | -0.75% | -0.84% | -0.67% | -0.89% | -0.80% | 105% | 102% | 100% |
| Class E | -0.50% | -0.70% | -0.59% | -0.58% | -0.78% | -0.90% | 106% | 100% | 100% |
| Overall | -0.44% | -0.45% | -0.41% | -0.42% | -0.49% | -0.47% | 105% | 101% | 100% |
| Class D | -0.86% | -0.55% | -0.71% | -0.90% | -0.59% | -1.03% | 105% | 104% | 100% |
| Class F | -0.39% | -0.35% | -0.55% | -0.37% | -0.59% | -0.48% | 104% | 102% | 100% |

|  | Random access Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.0 with LOP-3 and the low-complexity NN-based intra prediction mode |  |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |  |
| Class A1 | -0.09% | -0.33% | -0.25% | -0.07% | -0.51% | -0.46% | 99% | 101% | 100% |  |
| Class A2 | -0.07% | -0.37% | -0.31% | -0.04% | -0.42% | -0.60% | 100% | 100% | 100% |  |
| Class B | -0.12% | -0.58% | -0.47% | -0.17% | -0.88% | -0.92% | 106% | 109% | 100% |  |
| Class C | -0.41% | -0.64% | -0.59% | -0.46% | -0.52% | -0.64% | 102% | 105% | 100% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.18% | -0.51% | -0.43% | -0.20% | -0.62% | -0.69% | 102% | 104% | 100% |  |
| Class D | -0.40% | -1.01% | -0.67% | -0.56% | -1.70% | -0.83% | 103% | 108% | 100% |  |
| Class F | -0.25% | -0.58% | -0.33% | -0.39% | -0.75% | -0.02% | 104% | 112% | 100% |  |

# Conclusion

This contribution presents a combination of the NN-based intra prediction mode and ISP inside NNVC.

Compared to NNVC-9.0 CTC, the tested proposition reports the following mean BD-rate gains and savings of 3 kMACs/pixel for the worst-case for the NN-based intra prediction mode.

- RA:  -0.06%, -0.54% and -0.43% BD-rate PSNR Y, U and V gains, respectively for a complexity of 102% and 103% at encoder and decoder respectively

- AI:   -0.07%, -0.16% and -0.13% BD-rate PSNR Y, U and V gains, respectively, for a complexity of 104% and 102% at encoder and decoder respectively.

It is suggested to adopt EE1-5.1 into the next version of NNVC, replacing the default NN-based intra prediction mode by this improved low-complexity one in the NNVC CTC.

# Cited references

- JVET-AH2019, “Algorithm description for neural network-based video coding (NNVC-9.1)”, F. Galpin, Y. Li, D. Rusanovskyy, J. Ström, L. Wang. 34th Meeting, Rennes, FR, 17–24 April 2024.

- JVET-AF2016, “JVET common test conditions and evaluation procedures for neural network-based video coding technology”, E. Alshina, R. L. Liao, S. Liu, A. Segall, 32nd Meeting, Hannover, DE, 13–20 October 2023.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).