| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0138-v2 |
| --- | --- |

| Title: | AHG4: preparation of test material for film grain visual testing |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Information |  |  |  |
| Author(s) or Contact(s): | P. de Lagrange | Email: | philippe.delagrange@interdigital.com |
| Source: | InterDigital |  |  |  |

_____________________________

# Abstract

This contribution lists the test sequences considered for film grain visual testing in JVET-AH2021, how they are obtained, and encodings available. 9 UHD and 4 HD sequences are prepared, and encoded with HM and VTM, using a wide range of QPs to address the first two test categories of JVET-AH2021.

# General description

This document describes how the test sequences considered in JVET-AH2021 are obtained, and lists encodings available. The goal is to enable experimental viewings and expert tests at the current meeting, using both HEVC and VVC encodings. HEVC was added to answer a request from 3GPP in a liaison letter. A wide range of QPs have been prepared to span both the high quality range (for test category 2) and a broader range of qualities (for test category 1). Film grain parameters have been manually determined, using the frequency filtering model for the sake of simplicity considering the tools available.

# Test sequences and encodings available

## CrowdRun and OldTownCross (2004, SVT FairyTale)

Figure 1: Thumbnail of CrowdRun and OldTownCross

Those sequences are unchanged from JVET-AD0276. Encodings available:

- VTM 17.0 and HM 16.24

- All QPs from 12 to 51

Film grain parameters are depicted in the following figures:

Figure 2: Y/U/V film grain parameters for  CrowdRun

Figure 3: Y/U/V film grain parameters for  OldTownCross

## DinnerScene (Netflix Chimera)

Figure 4: Thumbnail of DinnerScene2

Compared to JVET-AD0276, the start frame has been changed (to 149) to leave 4 seconds after the scene cut, and the image width is cropped to 3840. It is now called DinnerScene2.

Encodings available:

- VTM 23.1 and HM 18.0

- QP 14 15 16 17 18 19 20 21 22 24 26 28 29 30 31 32 33 34 36 37 38 40 42

Film grain parameters are depicted in the following figure:

Figure 5: Y/U/V film grain parameters for DinnerScene2

## Meridian “smoker” and “intocar” scenes (Netflix Meridian)

Figure 6: Thumbnails of Meridian Smoker

Figure 7: Thumbnails of Meridian IntoCar

The intent is to select one 10s clip of each scene. This movie has been used by JVET/AG5 to generate test sequences, but only in HD resolution. Also, the “Meridian2” selection (car scene) leaves a cut of short length at the end. A new work has been done to select 10s clips suitable for film grain viewing tests, regenerating UHD sequences from the original material.

The goal of the selection is to get clips that are easy to watch and rate, and exercise film grain with different kinds of background: textured or smooth, with stable or changing light, at various light levels, and also where compression could be challenging and film grain could hide some artefacts (e.g. inaccurate motion on subtle details hidden by smoke, or block/banding/erratic motion on smooth gradations).

The command line used is the following:

MERIDIAN=MERIDIAN_SHR_C_EN-XX_US-NR_51_LTRT_UHD_20160909_OV_01.mxf

ffmpeg -y -ss $TIMECODE -i $MERIDIAN -vf zscale=f=lanczos \
    -pix_fmt yuv420p10le -vframes 600 $DEST

Selection for the “smoker” scene (two policemen discussing a case, with the older one sitting on his desk and smoking a cigar):

- Preferred choice:

  - Timecode 00:00:51.969 (image 3115): 3s static shot on portrait with subtle smoke shades, then young policeman (4s), then older (smoking)

- Alternate choices:

  - Timecode 00:01:45.072 (image 6298): full room view, then young policeman, then older

  - Timecode 00:02:21.208 (image 8464): full room view, then closer to older policeman

  - Timecode 00:02:55.609 (image 10526): full room view

Selection for the “into car” scene (car approaching on a road between mountains, then closeup on the driver, with weather suddenly changing to storm and strange apparition in the rear mirror).

- Preferred choice:

  - Timecode 00:03:18.065 (image 11872): car approaching, then 3.5s of closeup on driver

- Alternate choice:

  - Timecode 00:03:46.894 (image 13600): closeup on driver, weather suddenly changing to storm.

The original material has 60 frames per second (59,94 to be precise). A derivative at 24fps may be considered to evaluate the visual impact of grain at different frame rates, as 60fps is unusual for movies which are the target use case of this activity. The frame rate is reduced to 24fps by taking one frame out of 2 or 3 alternatively (3:2 pulldown).

Encodings available:

- VTM 23.1 and HM 18.0

- QP 14 15 16 17 18 19 20 21 22 24 26 28 29 30 31 32 33 34 36 37 38 40 42

- 59.94fps for MeridianSmoker1/3/4 and MeridianIntoCar1/2

- 24fps for MeridianSmoker1/2/4 and MeridianIntoCar1/2

Film grain parameters are depicted in the following figures:

Figure 8: Y/U/V film grain parameters for Meridian Smoker scene 1

Figure 9: Y/U/V film grain parameters for Meridian IntoCar scene (landscape)

Figure 10: Y/U/V film grain parameters for Meridian IntoCar scene (close up)

Figure 11: Y film grain parameters for Meridian IntoCar (alternate choice, parts a and b)

## Meridian “oldfilm” scene (Netflix Meridian)

Figure 12: Thumbnails of Meridian OldFilm

The Meridian test movie starts with 30s of archive films of low quality, to settle the action in 1947. This part appears to be upscaled from an HD source, which is obvious on the text overlays; a low pass with downscale/upscale does not show any difference. The image definition is rather coarse; grain is very strong and may almost reach HD resolution, though generally pretty large and rough.

Also, half of the cuts are 3:2 pulled down to generate 60fps from the original 24fps film; the other cuts are motion interpolated with strange effects on the grain.

The source UHD material has then been downscaled to HD, and inverse 3:2 pulled down to recover the original 24 frames per seconds. On motion interpolated cuts, this generates jerky motion but is still better to evaluate the grain.

From the 30s source, 10s have been extracted, starting from the first scene cut (image 311).

The first step (downscale) used the following command line:

ffmpeg -y -i $MERIDIAN -vf zscale=f=lanczos -s 1920x1080 -pix_fmt yuv420p10le \
    -vframes 1782 meridian-oldfilm_1920x1080_60Hz_420_10b.yuv

The next steps to reduce frame rate involved some manual checks to identify the 3:2 pattern for each scene cut.

Encodings available:

- VTM 23.1 and HM 18.0

- QP 13 to 38, then 40 42 44

Film grain parameters are depicted in the following figures:

Figure 13: Y/U/V film grain parameters for Meridian OldFilm scene 1

Figure 14: Y film grain parameters for Meridian OldFilm scenes 2 and 3

## MarketStreet (1906, Miles Brothers, Prelinger archives)

Figure 15: Thumbnail of MarketStreet

This is a very challenging movie, with very fine grain at high levels and jerky camera motion, because of the zoom camera mounted on a cable car and significantly shaking on road/rail bumps, maybe unstable film reel motion (or shutter interval), and low frame rate (16fps).

The goal of the selection was to find 10s where camera motion is moderate, with a balanced number of moving objects, decent sharpness, and a broad range of light levels, so that both encoding and grain is well exercised, and visual assessment is not too difficult.

The source material can be found on https://archive.org/details/MarketStreet19064KScan20181016.

The command line used to generate the UHD test clip was the following:

SOURCE=Market_Street_1906_4K_scan_20181016.mov

DEST=MarketStreet_3840x2160_16fps_420_10b.yuv

ffmpeg -y -ss 00:02:48 -i $SOURCE \
  -vf "crop=3840:2160:128+104:456,zscale=f=lanczos" -pix_fmt yuv420p10le \
  -f rawvideo -vframes 240 - | \
  $PULLUP -w 3840 -h 2160 -b 10 -f 420 -p 21 -i 0 - $DEST

PULLUP is a purpose-made utility to perform inverse pulldown with arbitrary repeat pattern and starting point. This is needed because the source stores the original 16fps material into 24fps because of limitations of the container format (or software package used).

Encodings available:

- VTM 23.1 and HM 18.0

- QP from 14 to 38, then 40 42 44 46

Film grain parameters are depicted in the following figure:

Figure 16: Film grain parameters for MarketStreet (Y only)

## AmericanChoice (1963, Jam Handy, Prelinger Archives)

Figure 17: Thumbnails of AmericanChoice

The source material is a high quality scan of a 35mm negative film with clean sharp picture and grain.

Here the goal of the selection was to find 10s with a balance of sharp detail, texture and flat areas to test both encoder and grain behaviour, on a scene that is easy to watch and rate.

It was advised to avoid too much foliage since viewers are often lost among the profusion of leaves that are hard to code but with a quality that is hard to evaluate. Indeed, the material available is a few reels of outtakes that mostly consist of foliage or flowers.

The preferred scene is a slow travelling in front of aligned houses found at the end of reel 3, with both texture and sharp details, with a fair number of flat zones (windows, doors, stairs, pavement) with various light levels.

A second choice is a park scene (middle of reel 3), walking on a road bordered by tree blossoms.

The source scan can be found on https://archive.org/details/200643_American_Choice_Outtakes_R3. It is very high resolution and has been downscaled and cropped to UHD resolution:

SOURCE=200643_American_Choice_Outtakes_R3_master.intros.mov

ffmpeg -y -ss 00:05:16 -i $SOURCE -vf "crop=6144:3456,zscale=f=lanczos" \
  -s 3840x2160 -pix_fmt yuv420p10le -f rawvideo -vframes 240 $DEST

Park scene timecode is 00:02:04.

Encodings available:

- VTM 23.1 and HM 18.0

- Pair QPs from 14 to 46, street scene only.

Film grain parameters are depicted in the following figure:

Figure 18: Y/U/V film grain parameters for AmericanChoice (street scene)

## AmericanMaker (1960, Jam Handy, Prelinger Archives)

Figure 19: Thumbnail of AmericanMaker

This sequence is from a finished movie, with much softer grain and picture than AmericanChoice, probably because of the overlay of many negatives and positives to produce the final print. Consequently, the source has been downscaled to HD resolution.

The source scan can be found on https://archive.org/details/200723_American_Maker.

The selection goal was still the same: a balance of motion, sharp elements, flat areas, light levels, easy to watch and rate, without cuts.

SOURCE=200723_American_Maker_master.intros.mov

DEST=./source/AmericanMaker_1920x1080_24fps_420_10b.yuv

ffmpeg -y -ss 00:08:30 -i $SOURCE -vf "crop=5120:2880,zscale=f=lanczos" \
  -s 1920x1080 -pix_fmt yuv420p10le -f rawvideo -vframes 240 $DEST

Encodings available:

- VTM 23.1 and HM 18.0

- QP from 13 to 26, then 28 31 34 36 38 40 42 44 (plus 30 and 32 for VTM)

Film grain parameters are depicted in the following figure:

Figure 20: Y/U/V film grain parameters for AmericanMaker

## TearOfSteel (denoised and regrained)

Figure 21: Thumbnails of TearsOfSteel scenes 004 and 044

These test sequences were proposed in JVET-AF0262. The selection accounts for the feedback reported in JVET-AH0299. The goal was to select two sequences, with a balance of motion, detail, light levels, textured and flat regions, and easy to watch and rate. Sequences with potentially distracting defects were avoided. Grain variants selected are visible but not too prominent, with a grain size that match the details in the scene, and do not introduce too much DC bias, so that we can still compare clean and grainy version encodings.

Selection if scene 004 (from image 16) with grain variant 07, and scene 044 (from image 74) with grain variant 02.

Encodings available:

- VTM 23.1 and HM 18.0

- Pair QPs from 14 to 46

- Both clean and grainy versions

Film grain parameters are depicted in the following figure:

Figure 22: Film grain parameters for TearsOfSteel scenes 004 (fg07) and 044 (fg02)

## StEM 2014 (from ASC / DCI)

Figure 23: Thumbnails of MagicHour and WarmNight

Two HD SDR scenes from the ASC / DCI “Standard Test evaluation Material” are used.

Encodings available:

- VTM 23.1 and HM 18.0

- MagicHour (from picture 121): QP 13 to 26, then 29 32 35 38, plus QP 12 for HM

- WarmNight (from picture 58): QP 13 to 26, then 28 31 34 37 40, plus QP 12 for HM

Film grain parameters are depicted in the following figure:

Figure 24: Film grain parameters for TearsOfSteel scenes 004 (fg07) and 044 (fg02)