| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0148-v2 |
| --- | --- |

| Title: | Non-EE2: Non-adjacent DIMD mode derivation for TMRL intra mode candidate list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vasily Rufitskiy Alexey Filippov Jacek Konieczny Hongdong Qin 7/F, Block G1, TCL International E-City 1001 Zhongshanyuan Road Nanshan, Shenzhen, Guangdong 518052 People’s Republic of China | Email | vasily.rufitskiy@tcl.com alexey.filippov@tcl.com jacek.k@tcl.com hongdong.qin@tcl.com |
| Source: | TCL |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to add such new candidates to the TMRL list, where intra prediction modes are determined using the DIMD process applied to the reference area around a corresponding reference line. In addition, the list of TMRL candidates is reordered so that the candidates, which result in similar predictors, are placed at the bottom of this list. On top of ECM-13.0, the following BD-rate gain results are reportedly obtained {Y/U/V}: {-0.03%, 0.03%, -0.01%} for AI configuration.

# Introduction

## Histogram of gradients calculation in a decoder-side intra mode derivation (DIMD) process

DIMD performs a gradient analysis on samples (e.g., luma samples) of a template that comprises neighbouring samples of current block. The template is L-shaped and has a width of 3 samples. When DIMD is in effect for a current block, the encoder and decoder calculate horizontal and vertical gradients inside the template to determine prevailing angular directions. In ECM, the encoder and decoder apply a 3x3 Sobel edge operator to positions of a 3x3 window within a template. The 3x3 Sobel gradient filters are defined as follows:

Fx=-1-2-1000121

Fy=10-120-210-1

These gradients are further used to update histogram of gradients (HoG). The maximum values of HoG indicate the prevailing angular directions inside a template. An encoder may indicate a flag, that for the current block one or several prevailing directions could be used to determine one or several intra prediction modes that are used to intra-predict the current block.

## Template-based multiple reference line intra prediction (TMRL)

Description of template-based multiple reference line intra prediction (TMRL) mode could be found in [1]. TMRL combines reference line and prediction mode together and uses a template matching mechanism to construct a list of candidate combinations. An index of a candidate in the combination list is coded to indicate which reference line and prediction mode is used in coding the current block.

A concept of multiple reference lines is shown in Figure 1.

Figure 1. Concept of multiple reference lines.

The TMRL candidate is constructed as follows. There are 5x10=50 combinations of the extended reference line and the allowed intra-prediction modes for a block. Since the extended reference line starts from reference line 1, the area covered by reference line 0 is used for template matching. The SAD costs over the template area are calculated between the predictions (generated by 50 combinations) and the reconstructions. The 20 combinations with the least SAD cost are selected in an ascending order to form the TMRL candidate list.

TMRL candidate list is shown in Figure 2.

Figure 2. TMRL candidate list.

Indication of the selected index is performed by a Rice code as shown in Figure 3. The codeword for the index coding is composed of

- a prefix code, which is a truncated unary code, and

- a fixed-length suffix code.

Figure 3. Signaling of a TMRL candidate index.

# Proposed method

In this contribution, we propose to reuse the DIMD processing in TMRL candidate list construction. For this purpose, DIMD is applied to non-adjacent reference lines, i.e. HoG is calculated in the area determined by a reference line as shown in Figure 4. The non-adjacent DIMD is aimed at deriving angular modes by estimating gradients and, eventually, directionality within the reference area pointed out by a reference line. The derivation of the non-adjacent DIMD mode list for a reference line is performed as follows:

-	Obtain a Histogram of Gradients (HoG) using the DIMD operator applied to the template area specified for a reference line

-	Sort the HoG by magnitude value

-	Select the first N modes from the sorted HoG (N=5)

These steps are performed independently per each reference line so that a specific list of non-adjacent DIMD modes is obtained per each reference line.

Figure 4. Example of non-adjacent DIMD.

The resulting candidate list is further reordered in such a way that similar modes are placed at the bottom of the candidate list.

The candidate list sorted in ascending order by the value of template matching cost (as specified by the TMRL process) is iterated starting from the top of the list (i.e. TMRL candidates with the minimum template cost). On each iteration, a similarity check is performed for a pair of candidate predictors to find which of them have a difference lower than a predetermined threshold.

The similarity check for a pair of candidate predictors is performed by calculating a metric value for the difference of the predictors obtained using intra prediction parameters defined for these candidates.

Thus, this contribution neither modifies the TMRL signaling mechanism nor introduces any new flags nor changes the parsing process in any way.

# Experimental results

The method was implemented in the ECM-13.0 [2] codebase and the test results were obtained under the common test conditions [3]. The experimental results are presented in Table 1.

Table 1. Experimental results of the proposed method

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.01% | 0.05% | #DIV/0! | #DIV/0! | #NUM! | #NUM! |
| Class A2 | -0.03% | -0.02% | 0.04% | #DIV/0! | #DIV/0! | #NUM! | #NUM! |
| Class B | -0.02% | 0.02% | -0.09% | #DIV/0! | #DIV/0! | #NUM! | #NUM! |
| Class C | -0.01% | 0.04% | 0.01% | #DIV/0! | #DIV/0! | 99.9% | 100.9% |
| Class E | -0.06% | 0.09% | 0.00% | #DIV/0! | #DIV/0! | 102.4% | 100.8% |
| Overall | -0.03% | 0.03% | -0.01% | #DIV/0! | #DIV/0! | #NUM! | #NUM! |
| Class D | -0.01% | 0.11% | -0.04% | #DIV/0! | #DIV/0! | 100.6% | 100.9% |
| Class F | #VALUE! | #VALUE! | #VALUE! |  |  | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! |  |  | #DIV/0! | #DIV/0! |

# Conclusion

In this contribution, a TMRL candidate list is extended by new candidates obtained by applying the DIMD process to non-adjacent reference line areas. The resulting candidate list is further reordered in such a way that similar TMRL candidates are placed at the end of the list.

We recommended the proposed method is included in the next round of EE2.

# References

- M. Coban, R.-L. Liao, K. Naser, J. Ström, L. Zhang, “Algorithm description of Enhanced Compression Model 12 (ECM 12)”, document JVET-AG2025, Jan. 2024

- ECM-13.0: https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-13.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, document JVET-AF2017, Oct. 2023.

# Patent rights declaration(s)

TCL may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).