| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0152-v1 |
| --- | --- |

| Title: | Non-EE2: Coding Information-based Classification for ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenbin Yin1, Kai Zhang2, Yang Wang1, Zhipin Deng1, Lei Zhao1, Na Zhang1, Li Zhang2 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | yinwenbin.hit@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

Abstract

In this contribution, a method of coding information-based classification is proposed for Adaptive Loop Filter (ALF). In the proposed method, the partitioning information is introduced to associate the classification of online filter. Moreover, the partitioning information and residual information are further introduced into the classification of offline filter to adjust the offset accordingly.

On top of ECM-13.0, the simulation results of the proposed method are reported as below:

AI: -0.01%, -0.01%, -0.01%, 100.2%, 100.5%.

RA: -0.08%, -0.02%, 0.03%, 100.4%, 101.2%.

LB: -0.12%, -0.02%, -0.02%, 100.5%, 101.3%.

- Introduction

In the current design of online filter for ALF, there are totally 3 classifiers known as texture-based, band-based, and residual-based classifiers. The encoder collects statistical data for each classifier and selects the most suitable classifier for each CTU. On the other hand, in the current design of offline filter, there is only one texture-based classifier which is derived from reconstruction samples before ALF. The coding information such as partitioning information and residual information are not considered to determine the offline filter offset.

- Proposed Method

In this contribution, a method of coding information-based classification for ALF is proposed, towards both online filter and offline filter.

For the online filter, each 2×2 unit is classified into 2 noise levels according to the partitioning information. For a 2×2 unit, it is classified into noise level 1 if it is located at a CU or TU boundary, and into noise level 0 if it is not. The class number of existing classifiers is reduced from 25 to 12. These 12 classes are further combined with the proposed 2 noise levels to generate final 12×2 = 24 classes in total.

For the offline filter, each 2×2 unit is classified into 2 noise levels in the same way as the online-filter classification. Besides, each 2×2 unit is classified into 2 residual levels based on a predefined threshold also in the same way as the online-filter classifier. Furthermore, the generated fixed-filter offset is adjusted based on the boundary level and residual level accordingly, where a stronger fixed-filter offset is applied on positions at boundaries or with high residuals.

The proposed method is always applied and no signaling is required. The number of online-filter classifiers is kept as 3 and no additional encoder selection is introduced.

- Simulation Results

The proposed method was implemented on top of the ECM-13.0 [1]. Simulations were performed following the common test conditions [2]. The performance of the proposed method compared to ECM-13.0 is shown in Table 1. More detailed results could be found in the attached excel sheets.

Table 1 Performance of the Proposed Method

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.04% | -0.01% | -0.01% | 100.2% | 100.9% | 100.1% | 102.0% |
| Class A2 | -0.02% | -0.02% | -0.02% | 99.7% | 100.1% | 100.5% | 102.0% |
| Class B | -0.01% | -0.01% | -0.01% | 100.3% | 100.2% | 101.4% | 101.0% |
| Class C | 0.01% | 0.00% | 0.00% | 100.1% | 100.4% | 99.5% | 100.3% |
| Class E | 0.00% | 0.00% | 0.00% | 100.8% | 101.1% | 99.2% | 100.5% |
| Overall | -0.01% | -0.01% | -0.01% | 100.2% | 100.5% | 100.3% | 101.1% |
| Class D | 0.02% | 0.04% | 0.04% | 100.0% | 100.8% | 99.8% | 100.1% |
| Class F | 0.03% | -0.02% | -0.02% | 100.5% | 101.4% | 100.4% | 100.6% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.05% | -0.05% | -0.12% | 99.8% | 100.5% | 100.2% | 101.2% |
| Class A2 | -0.07% | -0.13% | 0.05% | 100.5% | 100.9% | 99.8% | 101.1% |
| Class B | -0.06% | 0.01% | 0.02% | 101.2% | 101.7% | 99.5% | 100.7% |
| Class C | -0.16% | 0.06% | 0.12% | 99.7% | 101.3% | 99.3% | 100.2% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.08% | -0.02% | 0.03% | 100.4% | 101.2% | 99.6% | 100.7% |
| Class D | -0.15% | 0.06% | -0.08% | 100.9% | 101.4% | 99.4% | 100.1% |
| Class F | -0.05% | 0.13% | 0.07% | 100.3% | 101.1% | 99.5% | 100.5% |
|  | Low Delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.06% | -0.30% | -0.14% | 100.5% | 101.5% | 99.3% | 100.6% |
| Class C | -0.27% | 0.02% | -0.27% | 100.9% | 101.1% | 99.7% | 100.3% |
| Class E | -0.01% | 0.38% | 0.52% | 99.9% | 101.2% | 100.0% | 100.5% |
| Overall | -0.12% | -0.02% | -0.02% | 100.5% | 101.3% | 99.6% | 100.4% |
| Class D | -0.19% | 0.03% | -0.59% | 100.5% | 101.5% | 99.8% | 100.1% |
| Class F |  |  |  |  |  |  |  |

- Conclusions

In this contribution, the coding information based classification for ALF is proposed. The proposed method can achieve promising coding performance with negligible coding complexity. It is recommended to include the proposed method into the next round of EE2.

- Reference

- ECM-13.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-13.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-AF2017, DE, October 2023.

- Patent Rights Declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).