| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0177-v1 |
| --- | --- |

| Title: | AHG9: Constituent Rectangle Nesting SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce Miska M. Hannuksela | Email: | {jill.boyce, miska.hannuksela}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

A constituent rectangles nesting (CRN) SEI message is proposed to enable nesting of SEI messages for constituent rectangles, similar to the capability provided in the VVC scalable nesting SEI message for subpictures. The CRN SEI message is proposed for inclusion in a future version of VSEI.

# Proposal

The constituent rectangle SEI message was proposed in JVET-AH0162 and is included in the TuC. It enables composition of multiple constituent rectangles within a coded picture and provides information about the rectangles, including ID, type, text description, location, and size.

The VVC scalable nesting SEI message provides the capability to specify that one or more SEI messages applies to one or more subpictures. This contribution proposes adding a similar capability to specify that one or more SEI messages applies to one or more constituent rectangles.

The proposed functionality would be useful, for example, if a coded picture contained multiple alpha channel constituent rectangles along with texture constituent rectangles, by enabling an alpha channel info SEI message to be applied to the multiple alpha channel constituent rectangles.

The constituent rectangle nesting (CRN) SEI message contains syntax to indicate a set of constituent rectangles for which a list of SEI messages applies. The CRN SEI message may optionally be a scalable- nested SEI message, e.g. included in the VVC scalable nesting (SN) SEI message. If the CRN SEI message is a scalable-nested SEI message, the CR-nested SEI messages apply to the identified CRs in the set of layers identified in the SN SEI message. If the CRN SEI is not a scalable-nested SEI message, the CR-nested SEI messages apply to the identified CRs in the current layer containing the CRN SEI message.

The CRN SEI message is proposed to be specified in a future version of VSEI, with usage from VVC and eventually from HEVC. It is noted that the existing scalable nesting SEI message is specified in VVC, rather than VSEI.  Syntax and semantics for VSEI and user interface semantics for VVC are provided in section 2. In the VVC user interface the VVC sei_messages( ) syntax structure is assigned to the CRN SEI’s crn_sei_messages( ) syntax structure, to avoid the need to have payload type syntax elements included within VSEI. Alternative syntax and semantics in section 3 use the crn_sei_msg_byte[ i ][ j ] syntax element to signal in the contents of the CR-nested SEI messages, as defined in the VVC user interface semantics. The alternative syntax in section 3 is an editorial alternative that results in the same “bits on the wire” as the preferred syntax provided in section 2.

A list of the SEI messages which may be nested using the CRN SEI message are provided in the VVC semantics in section 2.3.

# Syntax and semantics

## VSEI Syntax

| constituent_rectangle_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| crn_num_rects_minus1 | ue(v) |
| crn_rect_id_len_minus1 | ue(v) |
| for( i = 0; i <= crn_num_crs_minus1; i++ ) |  |
| crn_rect_id[ i ] | u(v) |
| crn_num_seis_minus1 | ue(v) |
| while( !byte_aligned( ) ) |  |
| crn_zero_bit /* equal to 0 */ | u(1) |
| for( i = 0; i <= crn_num_seis_minus1; i++ ) |  |
| crn_sei_message( ) |  |
| } |  |

## VSEI Semantics

The constituent rectangle nesting (CRN) SEI message provides a mechanism to associate SEI messages with specific sets of constituent rectangles. A CRN SEI message contains one or more SEI messages. The SEI messages contained in the constituent rectangle nesting SEI message are referred to as the CR-nested SEI messages.

A CRN SEI message shall not be present in a current PU unless there is an CR SEI message in the current PU or in a PU that precedes the current PU in decoding order within the current CLVS.

When a PU contains both an CRN SEI message and a CR SEI message, the CR SEI message shall precede the CRN SEI message in decoding order.

Use of this SEI message requires the definition of the following variables,

–	The layers to which the CRN SEI message applies, denoted herein as NestedLayers

Use of this SEI message requires the definition of the crn_sei_message( ) syntax structure, which specifies a CR-nested SEI message and shall include the payload type value, the number of bytes in the SEI message payload, and the SEI message payload.

crn_num_rects_minus1 plus 1 specifies the number of constituent rectangles in each picture in the NestedLayers to which the CR-nested SEI messages apply. The value of crn_num_rects_minus1 shall be less than or equal to the value of cr_num_rects_minus1 in the CR SEI message referred to by the pictures.

It is a requirement of bitstream conformance that the value of cr_num_rects_minus1 shall be the same in all CR SEI messages for all layers in NestedLayers.

crn_rect_id_len_minus1 plus 1 specifies the number of bits used to represent the syntax element crn_rect_id[ i ]. The value of crn_rect_id_len_minus1 shall be in the range of 0 to 15, inclusive.

crn_rect_id[ i ] indicates the rect ID of the i-th constituent rectangle in each picture in the NestedLayers to which the CR-nested SEI messages apply. The length of the crn_rect_id[ i ] syntax element is crn_rect_id_len_minus1 + 1 bits.

crn_num_seis_minus1 plus 1 specifies the number of CR-nested SEI messages. The value of crn_num_seis_minus1 shall be in the range of 0 to 63, inclusive.

crn_zero_bit shall be equal to 0.

## VVC Semantics

### D.X Use of the constituent rectangle nesting SEI message in VVC

The semantics of a CR-nested SEI message apply individually to each constituent rectangle identified by the values of cr_rect[ i ] instead of a decoded picture or cropped decoded picture.

It is a requirement of bitstream conformance that the CR-nested SEI messages shall have payloadType equal to any of the following:

–	4 (user data registered)

–	5 (user data unregistered)

–	165 (alpha channel information)

–	177 (depth representation information)

–	202 (annotated regions)

–	217 (object mask information)

–	137 (mastering display colour volume)

–	142 (colour transform information)

–	144 (content light level information)

–	147 (alternative transfer characteristics information)

–	148 (ambient viewing environment information)

–	149 (content colour volume)

–	179 (multiview acquisition information)

–	218 (modality information)

–	210 (neural network post filter characteristics)

–	211 (neural network post filter activation)

–	219 (text description)

–	XXX (packed regions information)

For purposes of interpretation of the constituent rectangle nesting SEI message the following variables are specified:

–	The variables NestedLayers is derived as follows:

–	If the constituent rectangle nesting SEI message is a scalable-nested SEI message the following applies:

–	If sn_ols_flag is equal to 1, NestedLayers are the layers in the OLSs to which the scalable-nested SEI messages apply.

–	Otherwise (sn_ols_flag is equal to 0), NestedLayers are the layers to which the scalable-nested SEI messages apply.

–	Otherwise (the CRN SEI is not a scalable-nested SEI message) the following applies:

–	NestedLayers is the current layer.

For purposes of interpretation of the constituent rectangle nesting SEI message, the syntax structure crn_sei_message( ) is set equal to sei_message( ).

# Alternative syntax and semantics

An alternative to a portion of the syntax and semantics proposed in section 2 is provided below, which results in the same bits in the bitstream, so it is an editorial alternative. The section 2 syntax and semantics are preferred.

In section 2, the crn_sei_message( ) syntax structure is used to signal the contents of the CR-nested SEI messages, with the syntax structure defined by VVC as being the sei_message( ) syntax structure.  Other codec specifications would similarly need to provide an interface syntax structure for use of the CRN SEI message.

In this section, the crn_sei_msg_byte[ i ][ j ] syntax element is used to signal in the contents of the CR-nested SEI messages, as defined in the VVC user interface semantics.

## VSEI syntax

| constituent_rectangle_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| … |  |
| for( i = 0; i <= crn_num_seis_minus1; i++ ) |  |
| for( j = 0; j < CrnSeiMsgSize[ i ]; j++ ) |  |
| crn_sei_msg_byte[ i ][ j ] | b(8) |
| } |  |

## VSEI semantics

Use of this SEI message requires the definition of the following variables,

–	The layers to which the CRN SEI message applies, denoted herein as NestedLayers

–	The number of bytes used for signalling the i-th CR-nested SEI message, herein denoted as CrnSeiMsgSize[ i ] for in the range of 0 to crn_num_seis_minus1

crn_sei_msg_byte[ i ][ j ] is the j-th byte of the i-th CR-nested SEI message.

## VVC semantics

For purposes of interpretation of the constituent rectangles nesting SEI message the following variables are specified:

…

–	CrnSeiMsgSize[ i ] is set equal to the sum of the count of payload_type_byte syntax elements of the i-th CR-nested SEI message, the count of payload_size_byte syntax elements of the i-th CR-nested SEI message, and payloadSize of the i-th CR-nested SEI message.

The sequence of bytes crn_sei_msg_byte[ i ][ j ] for the values of j in the range of 0 to CrnSeiMsgSize[ i ] - 1, inclusive, shall conform to the sei_message( ) syntax structure.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and conditioned on reciprocity is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).