| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, Japan, 12–19 July 2024 | Document: JVET-AI0189-v2 |
| --- | --- |

| Title: | AHG9: On signalling of and/or specifying GFV translator, generator, and enhancer NNs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Kaifa Yang2, Yue Li2, Yiling Xu2 1Bytedance Inc., US 2Shanghai Jiao Tong University, China | Email: | yekui.wang@bytedance.com sekiroyyy@sjtu.edu.cn yueli983@sjtu.edu.cn yl.xu@sjtu.edu.cn |
| Source: | Bytedance Inc., Shanghai Jiao Tong University |  |  |

_____________________________

# Abstract

This contribution proposes/includes the following changes and discussions:

- Change the constraint requiring the presence of the translator NN to be as follows: When a GFV SEI message with gfv_cnt equal to 0 is present in an IRAP picture unit, gfv_nn_present_flag shall be present and equal to 1.

- Change the constraint requiring the presence of the enhancer NN to be as follows: When when a GFVE SEI message with gfve_gfv_cnt equal to 0 is present in an IRAP picture unit, gfve_nn_present_flag shall be present and equal to 1.

- Change the derivation of the applicable translator NN to be as follows:

When gfv_nn_present_flag is equal to 0 and TranslatorNN is referenced in the semantics of the GFV SEI message, the following applies for derivation of the applicable TranslatorNN:

- If gfv_cnt is greater than 0 and there exists one or more preceding GFV SEI messages in decoding order in the current picture unit that has the same value of gfv_id as that in the current GFV SEI message and gfv_nn_present_flag equal to 1, the applicable TranslatorNN is defined by the last preceding GFV SEI message in decoding order in the current picture unit that has the same value of gfv_id as that in the current GFV SEI message and gfv_nn_present_flag equal to 1.

- Otherwise, the applicable TranslatorNN is defined by a GFV SEI message that is present in the last preceding picture unit puB in output order in the current CLVS that has the same value of gfv_id as that in the current GFV SEI message and gfv_nn_present_flag equal to 1. When there are multiple such GFV SEI messages present in the picture unit puB that have the same value of gfv_id as that in the current GFV SEI message and gfv_nn_present_flag equal to 1, the applicable TranslatorNN is defined by the last of such GFV SEI messages in decoding order.

- Add the following constraints to require the presence of the basis GFV SEI message used for the derivation of the applicable translator NN:

When gfv_nn_present_flag is equal to 0 and TranslatorNN is referenced in the semantics of the GFV SEI message, the following constraint applies:

- If gfv_cnt is equal to 0, there shall be at least one GFV SEI message present in a preceding picture unit in output order in the current CLVS and having the same value of gfv_id as that in the current GFV SEI message and gfv_nn_present_flag equal to 1.

- Otherwise (gfv_cnt is greater than 0), there shall be at least one GFV SEI message that is present in either the current picture unit or a preceding picture unit in output order in the current CLVS and has the same value of gfv_id as that in the current GFV SEI message and gfv_nn_present_flag equal to 1.

- A discussion on the fixed format of facial parameters

- A discussion on specifying the generator NN

# On requiring the presence of the translator NN

Currently the following is specified:

When a GFV SEI message with a particular value of gfv_id is present in an IRAP picture unit or in a picture unit that follows an IRAP picture unit in output order and is not preceded in output order by any picture unit that follows the IRAP picture unit in output order and has a GFV SEI message with that particular value of gfv_id, gfv_nn_present_flag shall be present and equal to 1.

Requiring gfv_nn_present_flag to be present and equal to 1 for a GFV SEI message in a picture unit that is an IRAP picture unit is easily understandable, for providing full random access capabilities at IRAP picture units. However, there is no need to require gfv_nn_present_flag to be present and equal to 1 for a GFV SEI message in a picture unit that is not an IRAP picture unit.

Furthermore, when an IRAP picture unit contains multiple GFV SEI messages with a particular value of gfv_id and different values of gfv_cnt, there is no need to require gfv_nn_present_flag to be present and equal to 1 for all those GFV SEI messages.

Therefore, it is proposed to change the above constraint to be as follows:

When a GFV SEI message with gfv_cnt equal to 0 is present in an IRAP picture unit, gfv_nn_present_flag shall be present and equal to 1.

# On requiring the presence of the enhancer NN

Currently the following is specified:

When a GFVE SEI message is the first GFVE SEI message with a particular value of gfve_id is present in an IRAP picture unit or in a picture unit that follows IRAP picture unit in output order and is not preceded in output order by any picture unit that follows the IRAP picture unit in output order and has a GFVE SEI message with that particular value of gfve_id, gfve_nn_present_flag shall be present and equal to 1.

Similarly as in the above analysis on requiring the presence of the translator NN, requiring gfve_nn_present_flag to be present and equal to 1 for a GFVE SEI message in a picture unit that is an IRAP picture unit is easily understandable, for providing full random access capabilities at IRAP picture units. However, there is no need to require gfve_nn_present_flag to be present and equal to 1 for a GFV SEI message in a picture unit that is not an IRAP picture unit.

Furthermore, when an IRAP picture unit contains multiple GFVE SEI messages with a particular value of gfve_id and different values of gfve_gfv_cnt, there is no need to require gfve_nn_present_flag to be present and equal to 1 for all those GFVE SEI messages.

Therefore, it is proposed to change the above constraint to be as follows:

When when a GFVE SEI message with gfve_gfv_cnt equal to 0 is present in an IRAP picture unit, gfve_nn_present_flag shall be present and equal to 1.

# On derivation of the applicable translator NN

Currently the following is specified:

When gfv_nn_present_flag is equal to 0 and TranslatorNN is referenced in the semantics of this SEI message, the applicable TranslatorNN is defined by the GFV SEI message that is present in the last preceding picture unit in output order that has the same value of gfv_id as that in the current GFV SEI message and gfv_nn_present_flag equal to 1.

However, since it is possible to have multiple GFV SEI messages with different values of gfv_cnt and a particular value of gfv_id within the same picture unit, the derivation of the applicable TranslatorNN does not always have to be based on a GFV SEI message in a preceding picture unit in output order. Furthermore, the picture unit containing the GFV SEI message that defines the applicable TranslatorNN needs to be in the same CLVS as the current picture unit (i.e., the current CLVS) and that requirement is missing.

Therefore, it is proposed to replace the above sentence with the following:

When gfv_nn_present_flag is equal to 0 and TranslatorNN is referenced in the semantics of the GFV SEI message, the following applies:

- If gfv_cnt is greater than 0 and there exists one or more preceding GFV SEI messages in decoding order in the current picture unit that has the same value of gfv_id as that in the current GFV SEI message and gfv_nn_present_flag equal to 1, the applicable TranslatorNN is defined by the last preceding GFV SEI message in decoding order in the current picture unit that has the same value of gfv_id as that in the current GFV SEI message and gfv_nn_present_flag equal to 1.

- Otherwise, the applicable TranslatorNN is defined by a GFV SEI message that is present in the last preceding picture unit puB in output order in the current CLVS that has the same value of gfv_id as that in the current GFV SEI message and gfv_nn_present_flag equal to 1. When there are multiple such GFV SEI messages present in the picture unit puB that have the same value of gfv_id as that in the current GFV SEI message and gfv_nn_present_flag equal to 1, the applicable TranslatorNN is defined by the last of such GFV SEI messages in decoding order.

# On requiring the presence of the basis for derivation of the applicable translator NN

When gfv_nn_present_flag is equal to 0 for a GFV SEI message, the TranslatorNN referenced in the semantics of the GFV SEI message is derived based on another GFV SEI message with gfv_nn_present_flag equal to 1. However, there lacks a constraint for requiring such a GFV SEI message to be present in the current CLVS.

Therefore, the following constraint is proposed:

When gfv_nn_present_flag is equal to 0 and TranslatorNN is referenced in the semantics of the GFV SEI message, the following applies:

- If gfv_cnt is equal to 0, there shall be at least one GFV SEI message present in a preceding picture unit in output order in the current CLVS and having the same value of gfv_id as that in the current GFV SEI message and gfv_nn_present_flag equal to 1.

- Otherwise (gfv_cnt is greater than 0), there shall be at least one GFV SEI message that is present in either the current picture unit or a preceding picture unit in output order in the current CLVS and has the same value of gfv_id as that in the current GFV SEI message and gfv_nn_present_flag equal to 1.

# A discussion on the fixed format of facial parameters

The facial parameter translator network, denoted as TranslatorNN( ), is a neural network that may be used to convert various formats of facial parameters signalled in the GFV SEI message into a fixed format of parameters. However, the fixed format of parameters is not signalled or specified in the GFV SEI message.

Should the fixed format of parameters be either signalled in the GFV SEI message or specified in the semantics? Otherwise, how would an encoder know how to derive the TranslatorNN( ) to be signalled?

Another question for discussion: When the format of the facial parameters signalled in the GFV SEI message is actually the fixed format, the translator network is then not needed, correct?

# A discussion on specifying the generator NN

The first sentence in the semantics of the GFV SEI message says that the GFV SEI message indicates, among other things, a face picture generator neural network. But how is the generator NN indicated by the GFV SEI message?

If the GenerativeNN is fixed and does not need to be signalled, then does it need to be specified in the spec, e.g., similarly as in JPEG AI by providing an electronic attachment?

# Patent rights declaration(s)

Bytedance Inc. and Shanghai Jiao Tong University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).