| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, Japan, 12–19 July 2024 | Document: JVET-AI0191-v3 |
| --- | --- |

| Title: | AHG9: On GFV facial parameters signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Kaifa Yang2, Yue Li2, Yiling Xu2 1Bytedance Inc., US 2Shanghai Jiao Tong University, China | Email: | yekui.wang@bytedance.com sekiroyyy@sjtu.edu.cn yueli983@sjtu.edu.cn yl.xu@sjtu.edu.cn |
| Source: | Bytedance Inc., Shanghai Jiao Tong University |  |  |

_____________________________

# Abstract

This contribution proposes the following changes related to GFV facial parameters signalling:

- Move gfv_kps_pred_flag to be immediately before gfv_coordinate_precision_factor_minus1, such that it can be used to condition the signalling of the four syntax elements from gfv_coordinate_precision_factor_minus1 to gfv_coordinate_z_max_value_minus1.

- Additionally condition the presence of the four syntax elements from gfv_coordinate_precision_factor_minus1 to gfv_coordinate_z_max_value_minus1 on ( gfv_base_pic_flag  | |  !gfv_kps_pred_flag ).

- Add the following constraint to ensure the presence of a prediction basis for the key points dimension parameters: When gfv_coordinate_present_flag is equal to 1, gfv_base_pic_flag is equal to 0, and gfv_kps_pred_flag is equal to 1, there shall be a previous GFV SEI message in decoding order with the same gfv_id as the current GFV SEI message and gfv_base_pic_flag equal to 1 in the current CLVS.

- When gfv_coordinate_present_flag is equal to 1, gfv_base_pic_flag is equal to 0, and gfv_kps_pred_flag is equal to 1, the values of the four syntax elements from gfv_coordinate_precision_factor_minus1 to gfv_coordinate_z_max_value_minus1 are inferred to be equal to the values of the corresponding syntax elements of the previous GFV SEI message in decoding order with the same gfv_id as the current GFV SEI message and gfv_base_pic_flag equal to 1.

- Move gfv_matrix_pred_flag to be immediately before gfv_matrix_element_precision_factor_minus1, such that it can be used to condition the signalling of the nine syntax elements from gfv_matrix_element_precision_factor_minus1 to gfv_matrix_for_3D_space_flag[ i ].

- Additionally condition the presence of the nine syntax elements from gfv_matrix_element_precision_factor_minus1 to gfv_matrix_for_3D_space_flag[ i ] on ( gfv_base_pic_flag  | |  !gfv_matrix_pred_flag ).

- Add the following constraint to ensure the presence of a prediction basis for the metrics dimension parameters: When gfv_matrix_present_flag is equal to 1, gfv_base_pic_flag is equal to 0, and gfv_matrix_pred_flag is equal to 1, there shall be a previous GFV SEI message in decoding order with the same gfv_id as the current GFV SEI message and gfv_base_pic_flag equal to 1 in the current CLVS.

- When gfv_matrix_present_flag is equal to 1, gfv_base_pic_flag is equal to 0, and gfv_matrix_pred_flag is equal to 1, the values of the nine syntax elements from gfv_matrix_element_precision_factor_minus1 to gfv_matrix_for_3D_space_flag[ i ] are inferred to be equal to the values of the corresponding syntax elements of the previous GFV SEI message in decoding order with the same gfv_id as the current GFV SEI message and gfv_base_pic_flag equal to 1.

- Changed the syntax such that the matrix element syntax elements from gfv_matrix_element_int[ i ][ j ][ k ][ m ] to gfv_matrix_delta_element_sign_flag[ i ][ j ][ k ][ m ] may also be signalled for matrix types other than 4 to 6, inclusive. Note that this fix is also included in JVET-AI0156.

- Changed the equations for derivation of the key point coordinate values and the matrix element values, such that when gfv_base_pic_flag is equal to 0 and the gfv_cnt currGfvCnt is greater than 0, the key point coordinate values and/or the matrix element values are predicated from the previous GFV SEI message with gfv_cnt equal to currGfvCnt − 1, instead of being predicted from the previous GFV SEI message with gfv_base_pic_flag equal to 1.

- Some editorial changes.

The detailed changes are provided in an attachment to this document.

# Issues observed on GFV facial parameters signalling

## On gfv_num_kps_minus1

Two issues on gfv_num_kps_minus1 have been observed.

- When gfv_coordinate_present_flag is equal to 0, the syntax element gfv_num_kps_minus1 is still used in the equation that includes the derivation of the key point coordinate array sigKeyPoint, as can be seen from the excerpt of the equation below:

if( gfv_coordinate_present_flag ) {
  for ( i = 0; i< = gfv_num_kps_minus1; i++ ) {
    sigKeyPoint[ i ][ 0 ] = coordinateX[ i ]
    sigKeyPoint[ i ][ 1 ] = coordinateY[ i ]
    if ( gfv_coordinate_z_present_flag ) 
      sigKeyPoint[ i ][ 2 ] = coordinateZ[ i ]
  }
}
else {
  for ( i = 0; i <= gfv_num_kps_minus1; i++ ) {
    sigKeyPoint [ i ][ 0 ] = 0
    sigKeyPoint [ i ][ 1 ] = 0
    if ( gfv_coordinate_z_present_flag ) 
      sigKeyPoint [ i ][ 2 ] = 0
  }
}

However, when gfv_coordinate_present_flag is equal to 0, according to the current syntax of the GFV SEI message, the syntax element gfv_num_kps_minus1 is not present, and according to the current semantics of the GFV SEI message, when the syntax element is not present, its value is not inferred. For the above equation to work, when gfv_num_kps_minus1 is not present, its value needs to be inferred.

- When gfv_kps_pred_flag is equal to 1, the coordinates of the GFV key points are to be predicted, and in this case the value of gfv_num_kps_minus1 should be the same as that in the previous GFV SEI message that contains the basis GFV key points to be predicted from. However, there lacks a constraint for requiring this. Furthermore, in this case the value of gfv_num_kps_minus1 does not need to be signalled and can be inferred.

## On gfv_coordinate_precision_factor_minus1 and gfv_coordinate_z_max_value_minus1

These syntax elements have a similar issue as the second issue on gfv_num_kps_minus1.

When gfv_kps_pred_flag is equal to 1, the coordinates of the GFV key points are to be predicted, and in this case the values of gfv_coordinate_precision_factor_minus1 and gfv_coordinate_z_max_value_minus1 should be respectively the same as that in the previous GFV SEI message that contains the basis GFV key points to be predicted from. However, there lacks a constraint for requiring this. Furthermore, in this case the values of these syntax elements do not need to be signalled and can be inferred.

## On gfv_num_matrix_types_minus1

Two similar issues as on gfv_num_kps_minus1 have been observed on gfv_num_matrix_types_minus1.

- When gfv_matrix_present_flag is equal to 0, the syntax element gfv_num_matrix_types_minus1 is still used in the equation that includes the derivation of the matrix sigMatrix, as can be seen from the excerpt below:

if( gfv_matrix_present_flag ) {
  for ( i = 0; i <= gfv_num_matrix_types_minus1; i++ ) {
    for ( j = 0; j < numMatrices[ i ]; j++ ) {
      for( k = 0; k < matrixHeight [ i ]; k++ ) {
        for ( l = 0;l < matrixWidth [ i ]; l++) {
          sigMatrix[ i ][ j ][ k ][ l ] = matrixElementVal[ i ][ j][ k][ l ] 
        }
      }
    }
  }
}
else {
  for ( i = 0; i <= gfv_num_matrix_types_minus1; i++ ) {
    for ( j = 0; j < numMatrices[ i ]; j++ ) {
      for( k = 0; k < matrixHeight [ i ]; k++ ) {
        for ( l = 0;l < matrixWidth [ i ]; l++) {
          sigMatrix [ i ][ j ][ k ][ l ] = 0
        }
      }
    }
  }
}

However, when gfv_matrix_present_flag is equal to 0, according to the current syntax of the GFV SEI message, the syntax element gfv_num_matrix_types_minus1 is not present, and according to the current semantics of the GFV SEI message, when the syntax element is not present, its value is not inferred. For the above equation to work, when gfv_num_matrix_types_minus1 is not present, its value needs to be inferred.

- When gfv_matrix_pred_flag is equal to 1, the element values of the GFV matrices are to be predicted, and in this case the value of gfv_num_matrix_types_minus1 should be the same as that in the previous GFV SEI message that contains the basis GFV matrices to be predicted from. However, there lacks a constraint for requiring this. Furthermore, in this case the value of gfv_num_matrix_types_minus1 does not need to be signalled and can be inferred.

## On gfv_matrix_element_precision_factor_minus1, gfv_matrix_type_idx[ i ], gfv_num_matrices_info[ i ], gfv_num_matrices_minus1[ i ], gfv_matrix_width_minus1[ i ], gfv_matrix_height_minus1[ i ], and gfv_matrix_for_3D_space_flag[ i ]

These syntax elements have a similar issue as the second issue on gfv_num_matrix_types_minus1.

When gfv_matrix_pred_flag is equal to 1, the coordinates of the GFV key points are to be predicted, and in this case the values of gfv_matrix_element_precision_factor_minus1, gfv_matrix_type_idx[ i ], gfv_num_matrices_info[ i ], gfv_num_matrices_minus1[ i ], gfv_matrix_width_minus1[ i ], gfv_matrix_height_minus1[ i ], and gfv_matrix_for_3D_space_flag[ i ] should be respectively the same as that in the previous GFV SEI message that contains the basis GFV key points to be predicted from. However, there lacks a constraint for requiring this. Furthermore, in this case the values of these syntax elements do not need to be signalled and can be inferred.

## On possible absence of prediction basis

- When gfv_kps_pred_flag is equal to 1, the coordinates of the GFV key points are to be predicted. However, there lacks a constraint for requiring the existence a previous GFV SEI message that contains the basis GFV key points to be predicted from.

- When gfv_matrix_pred_flag is equal to 1, the element values of the GFV matrices are to be predicted. However, there lacks a constraint for requiring the existence a previous GFV SEI message that contains the basis GFV matrices to be predicted from.

## On missing signalling of matrix element values for matrix types other than 4, 5, and 6

In the current syntax of the the GFV SEI message, the matrix element syntax elements from gfv_matrix_element_int[ i ][ j ][ k ][ m ] to gfv_matrix_delta_element_sign_flag[ i ][ j ][ k ][ m ] are only signalled when gfv_matrix_present_flag is equal to 1 and the following condition is true: "gfv_matrix_type_idx[ i ]  >=  4  &&  gfv_matrix_type_idx[ i ]  <=  6  &&  !gfv_coordinate_present_flag". For example, they are never signalled for matrix types (indicated by gfv_matrix_type_idx[ i ]) other than 4 to 6, inclusive. For example, for matrix types 0, 1, 2, 3, and 7, these syntax elements are never signalled at all. However, for matrix types 0, 1, 2, 3, and 7, these syntax elements are also be needed under some conditions. We noticed that this issue has also been addressed in JVET-AI0156.

## On prediction of key point coordinates and matrix elements

Since GFV SEI messages of the same gfv_id with gfv_cnt greater than 0 would all need to be within the same picture unit as the corresponding GFV SEI message with gfv_cnt equal to 0, all these SEI messages should be and would almost always be transmitted in the same transport unit (packet/segment, etc.). Therefore it makes sense to enable the prediction of the GFV key point coordinates of a GFV SEI message with gfv_cnt greater than 0 from a second GFV SEI message with the same gfv_id but a smaller gfv_cnt in the same picture unit, regardless of whether the second GFV SEI message has gfv_base_pic_flag equal to 1 or 0.

Similarly, it makes sense to enable the prediction of matrix elements of a GFV SEI message with gfv_cnt greater than 0 from a second GFV SEI message with the same gfv_id but a smaller gfv_cnt in the same picture unit, regardless of whether the second GFV SEI message has gfv_base_pic_flag equal to 1 or 0.

# Proposal

To resolve the above issues, this contribution proposes a set of changes as summarized in the abstract.

# Patent rights declaration(s)

Bytedance Inc. and Shanghai Jiao Tong University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).