| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0201-v2 |
| --- | --- |

| Title: | AHG12: neural network-based intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Urban, T. Dumas, F. Galpin, E. François | Tel: Email: | thierry.dumas@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes a new intra prediction mode based on neural networks in ECM-13.0. It is built from the algorithm integrated into Neural Network Video Coding (NNVC) and described in JVET-AH2019. The following results are reported.

- RA:  -…%, -…% and -…% BD-rate PSNR Y, U and V gains, respectively for a complexity of …% and …% at encoder and decoder

- AI:   -0.60%, -0.43% and -0.46% BD-rate PSNR Y, U and V gains, respectively, for a complexity of 107.0% and 119.5% at encoder and decoder.

# Introduction

A single neural network-based intra prediction mode exists in NNVC. Both its inference inside NNVC and its entire training have already been cross-checked through two different EE1 rounds. The same algorithm has been implemented and evaluated on top of ECM-13.0.

# Description

The neural network-based intra prediction algorithm and models are described in JVET-AH2019 [1] and JVET-AH0156 [2].

- SADL for inference

- 16-bit integer for model weights and intermediate results, using 32-bit integer for internal computation

- 7 models depending on the block size

- sparse weights enforced at training time for complexity reduction. SADL supports a simple sparse matrix storage (compressed row method) and sparse matrix multiplication algorithm, exploiting the matrices sparsity. Sparsity constraints have been strengthened compared to JVET-AH0156.

Table 1 shows the parameters count and storage memory required for each model. Memory usage reported is informative since it depends on the implementation for sparse matrix storage. Table 1 reports the total memory storage of the actual tested ECM implementation.

Table 1: parameters count and storage memory needed for each model.

| model | parameters count | non-zero parameters count | memory footprint |
| --- | --- | --- | --- |
| 4x4 8x4 16x4 32x4 8x8 16x8 16x16 | 968k 1008k 1888k 2122k 2122k 2317k 1699k | 77k 121k 182k 229k 194k 239k 291k | 155 kB 242 kB 365 kB 458 kB 388 kB 477 kB 583 kB |

The complexity measured in MACs/pixel for the different neural network models is reported in Table 2. The bottom part of the table shows the complexity for blocks predicted with down-sampled reference samples.

Table 2: complexity measured in MACs/pixel for the different neural network models.

| model | complexity (MACs/pixel) |
| --- | --- |
| 4x4 8x4 16x4 32x4 8x8 16x8 16x16 | 4837 3773 2848 1789 3030 1865 1138 |
| 32x8 32x16 | 932 569 |
| 16x32 | 569 |
| 32x32 | 284 |
| 64x64 | 71 |
| 128x128 | 18 |

# Experimental results

The simulations of ECM-13.0 including the proposed intra prediction mode are run following the JVET CTC [3] and reported in Table 3.

Table 3: experimental results of ECM-13.0 including the proposed intra prediction mode over ECM-13.0.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.84% | -0.77% | -0.68% | 104.8% | 113.2% | 99.1% | 103.7% |
| Class A2 | -0.43% | -0.26% | -0.26% | 107.6% | 114.0% | 104.6% | 104.3% |
| Class B | -0.49% | -0.37% | -0.48% | 107.4% | 120.9% | 104.5% | 106.0% |
| Class C | -0.58% | -0.40% | -0.34% | 107.6% | 125.9% | 105.9% | 107.4% |
| Class E | -0.77% | -0.40% | -0.56% | 106.7% | 121.3% | 106.4% | 107.2% |
| Overall | -0.60% | -0.43% | -0.46% | 107.0% | 119.5% | 104.2% | 105.8% |
| Class D | -0.64% | -0.54% | -0.54% | 105.4% | 128.9% | 103.9% | 107.6% |
| Class F | -0.35% | 0.05% | -0.02% | 107.4% | 119.1% | 101.9% | 106.6% |

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.24% | -0.18% | -0.19% | 103.6% | 107.1% | 102.2% | 106.5% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.25% | -0.62% | -0.18% | 100.6% | 101.1% | 103.7% | 107.3% |
| Class F |  |  |  |  |  |  |  |

# Conclusion

This contribution presents a new intra prediction mode based on neural networks in ECM-13.0. It is reported that, on top of ECM-13.0, this intra prediction tool yields the following results.

- RA:  -…%, -…% and -…% BD-rate PSNR Y, U and V gains, respectively for a complexity of …% and …% at encoder and decoder

- AI:   -0.60%, -0.43% and -0.46% BD-rate PSNR Y, U and V gains, respectively, for a complexity of 107.0% and 119.5% at encoder and decoder.

It is suggested to study this intra prediction tool in the next round of EE2.

# Cited references

- JVET-AH2019, “Algorithm description for neural network-based video coding (NNVC-9.1)”, F. Galpin, Y. Li, D. Rusanovskyy, J. Ström, L. Wang. 34th Meeting, Rennes, FR, 17–24 April 2024.

- JVET-AH0156, “AhG12: neural network-based intra prediction”, F. Urban, T. Dumas, F. Galpin, E. François, 34th Meeting, Rennes, FR, 17–24 April 2024.

- JVET-AF2017, “Common test conditions and evaluation procedures for enhanced compression tool testing”, M. Karczewicz, Y. Ye, 32nd Meeting, Hannover, DE, 13–20 October 2023

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).