| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12-19 July 2024 | Document: JVET-AI0205-v1 |
| --- | --- |

| Title: | AHG9: Combined SEI messages for common image metadata formats |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Arianne Hinds Stephan Wenger Gilles Teniou | Tel: Email: | ahinds@global.tencent.com swenger@global.tencent.com teniou@global.tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

At the 34th meeting, JVET-AH0343 combined, into a single SEI message, multiple SEI message proposals that share a common syntax based on a text string. The rationale for doing so was “that a common SEI message which provides text information for various purposes avoids the need to define multiple SEI messages which provide specific type of text information, while providing future extensibility.” This proposal makes the observation that the same rationale may be applied to the image metadata formats proposed in JVET-AF0141 and subsequently adopted into the TuC starting in JVET-AF2032. Hence a single SEI message to carry the binary metadata of the JVET-AF0141 image metadata format SEIs is proposed herein to achieve similar benefits of having to avoid the need to define multiple SEI messages that provide specific image metadata information.

# Problem statement

The current technologies under consideration document published as JVET-AH2032 [1] contains three separate SEI messages to carry the binary information associated with the metadata for EXIF[2], JFIF[3], and XMP[4,5,6]. These SEI messages are collectively labelled as “image format metadata SEI messages” within the TuC.  The primary syntax element across the payloads for these SEI messages is a payload byte with the descriptor of b(8), which is read from the video bitstream to collect the binary payloads for each of the SEIs.

A single SEI message to carry the binary metadata associated with the image format metadata SEI messages may benefit from the same rationale used to create a single SEI that employs a syntax based on a text string, as described in JVET-AH0343. Such an SEI message would leverage the common syntax of binary payload bytes, with an option to carry the payload via a URI.

# Proposed approach

## General

A single SEI message is proposed to carry each of the image format metadata SEI messages. Similar to the purpose syntax element in JVET-AH0343, a “type” syntax element is defined to signal which of the metadata formats is being carried in the SEI message payload. As well, the option to reference the image metadata at a location determined by a URI is preserved for such a single SEI so that the image metadata payload may be accessed from the location provided by the URI.

# Proposal

## Image metadata SEI message syntax

| image_metadata( payloadSize ) { | Descriptor |
| --- | --- |
| im_cancel_flag | u(1) |
| if (!im_cancel_flag) { |  |
| im_persistence_flag | u(1) |
| im_num_metadata_payloads | u(8) |
| while( !byte_aligned( ) ) |  |
| im_bit_equal_to_zero | f(1) |
| for (i=0; i<im_num_metadata_payloads; i++) { |  |
| im_type_id | u(8) |
| im_mode_id | u(8) |
| if(im_mode_id == 0) { |  |
| im_metadata_len_minus1 | u(e) |
| for( i = 0; i <= im_metadata_len; i++ ) |  |
| im_data_payload_byte | b(8) |
| } |  |
| else if(im_mode_id == 1) { |  |
| im_data_URI | st(v) |
| } |  |
| } |  |
| } |  |

## Image metadata SEI message semantics

The image metadata SEI message specifies an SEI message for which the payload of Exif metadata, JFIF metadata, and XMP metadata can be carried in the video bitstream.

im_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous image metadata SEI message in output order. im_cancel_flag equal to 0 indicates that image metadata information follows.

im_persistence_flag specifies the persistence of the image metadata SEI message for the current layer.

im_persistence_flag equal to 0 specifies that the image metadata SEI message applies to the current decoded picture only.

im_persistence_flag equal to 1 specifies that the imagemetadata SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an Exif metadata SEI message is output that follows the current picture in output order.

im_num_metadata_payloads indicates the number of metadata payloads that follow.

im_bit_equal_to_zero shall be equal to zero.

im_type_id indicates the type of the metadata payload as defined in Table xx.

Table xx – Definition of im_type_id

| Value | Interpretation |
| --- | --- |
| 0 | Exif metadata |
| 1 | JFIF metadata from one or more concatenated APP0 marker segments |
| 2 | one or more JFIF extension markers |
| 3 | first APP0 JFIF marker following the 0xFFD8 Start of Image marker |
| 4 | XMP metadata |
| 5-255 | Reserved |

When im_type_id is equal to 0, the following image metadata payload is Exchangeable Image File (Exif)  metadata. The Exif Format for digital still cameras includes a set of metadata that captures information regarding the digital photography process that was used to record an image. Such metadata includes: the acceleration vector of the camera at the time the image was captured, camera lens information, GPS data, the color space used, spectral sensitivity, maximum lens aperture, and more.

Exif metadata is specified by any of the Exchangeable image file (Exif) format for digital still cameras standards developed jointly by the Camera & Imaging Products Association (CIPA) and the Japan Electronics and Information Technology Industries Association (JEITA), for the associated video source pictures prior to encoding, e.g., for camera-captured content. To date, there are multple versions of Exif deployed, each with its own indicator to signal the version number in use.

[Ed. Note: Neither CIPA nor JEITA are listed in the organizations for which normative references may be made to documents published by CIPA or JEITA. The list of such organizations is available at: https://www.itu.int/net4/ITU-T/lists/sdo.aspx. To add either CIPA or JEITA to the list, the A.4 process needs to be executed for one or both organizations, and their status as A.5 organizations needs to be approved by SG16. There would probably also need to be a specific list of documents referenced rather than the concept of “any of the” standards produced by those organizations.]

When im_type_id is equal to 1, 2, or 3, the following metadata payload is JFIF metadata, as specified by ITU-T Recommendation T.871 | ISO/IEC International Standard 10918-5.

JFIF metadata and its semantics are specified by ITU-T Recommendation T.871 | ISO/IEC International Standard 10918-5 (hereafter, the JFIF standard). Of particular importance is the distinction by JFIF between an APP0 marker that contains information describing the organization of the image data, and an APP0 marker that contains “extension” information. [Ed. Note: Needs formal reference (in clause 2). Does that document define what an “APP0 marker” is?]

A single APP0 marker that carries the information to describe the organization of the image is signalled by the zero-terminated string “JFIF” (‘0x4A46494600’) within that APP0 marker. One or more subsequent APP0 markers may follow the first APP0 marker in which the subsequent APP0 marker(s) carries “extension” information. In particular, the JFIF extension mechanism can be used to carry image thumbnails, although there is no specification by the JFIF standard for the precise type of data carried in JFIF extensions.

Approaches for carriage of JFIF within an SEI message include:

–	The JFIF payload is carried entirely as a single “container” within an SEI message. In this approach, the JFIF payload includes the payloads from the first and all, if any, subsequent JFIF APP0 markers for an image.

–	The JFIF payload from only extension APP0 markers, i.e., all JFIF APP0 markers that appear subsequent to the first APP0 marker, is carried as a single container. This approach facilitates the carriage of thumbnail images without the overhead of the carriage of the first JFIF APP0 marker.

–	The JFIF payload from only the first JFIF APP0 marker is carried as a single container.

When im_type_id is equal to 1, the following image metadata payload is JFIF metadata from one or more concatenated APP0 marker segments without the APP0 marker segment identifiers themselves, i.e., an APP0 marker segment with  bytes containing 0x4A46494600, i.e, the zero-terminated string "JFIF", according to Rec. ITU-T T.50 or ISO 646 coding, and one or more APP0 marker segments containing 0x4A46585800, i.e,  the zero-terminated string "JFXX", according to Rec. ITU-T T.50 or ISO 646 coding.

When im_type_id is equal to 2, the following image metadata payload contains bytes from one or more JFIF extension markers, i.e., one or more APP0 markers that include 0x4A46585800, i.e,  the zero-terminated string "JFXX", according to Rec. ITU-T T.50 or ISO 646 coding.

When im_type_id is equal to 3, the following image metadata payload contains bytes obtained from the first APP0 marker following the 0xFFD8 Start of Image marker within the image. The first APP0 marker includes 0x4A46494600, i.e,  the zero-terminated string "JFIF", according to Rec. ITU-T T.50 or ISO 646 coding.

When im_type_id is equal to 4, the following image metadata payload contains XMP metadata as specified by [1] in XML or [3] in JSON. [Ed. Note: Fix or remove unresolved references and undefined abbreviations.]

Extensible Metadata Platform (XMP) metadata is specified in ISO 16684-1: Graphic Technology – Extensible metadata platform (XMP) specification. [Ed. Note (GJS): Formal reference needed in clause 2.] XMP is widely deployed by digital cameras and digital image editing packages to record provenance and editing history with an image. The most common set of metadata included in XMP refers to a vocabulary defined by the Dublin Core Metadata Initiative (DCMI), e.g., to store information such as digital rights ownership and names of software packages used to modify the image.

im_mode_id specifies the mode by which the image metadata is obtained.

–	If im_mode_id is equal to 0, the image metadata is obtained directly from the payload of the SEI message.

–	Otherwise if im_mode_id is equal to 1, the imagemetadata is obtained from a URI with syntax and semantics as specified in IETF Internet Standard 66.

All other values of im_mode_id are reserved.

im_metadata_len_minus1 plus 1 indicates the length of image metadata according to the im_type_id as described in Table xx.

imp_data_payload_byte shall be a byte containing data having syntax and semantics according to im_type_id as described in Table xx.

im_data_URI shall contain a URI with syntax and semantics as specified in IETF Internet Standard 66 identifying the image metadata according to the im_type_id as described in Table xx.

# References

[1]  Technologies under consideration for future extensions of VSEI (version 4), Joint Video Experts Team       (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 JVET-AH2032.

[2]   Exchangeable image file format for digital still cameras: Exif Version 3.0, Camera & Imaging Products Association, May 2023.

[3] Recommendation ITU-T T.871, Information technology - Digital compression and coding of continuous-tone still images: JPEG File Interchange Format (JFIF).

[4] ISO 16684-1: Graphic Technology – Extensible metadata platform (XMP) specification – Part 1 Data model, serialization and core properties.

[5] ISO 16684-2: Graphic Technology – Extensible metadata platform (XMP) specification – Part 2 Description of XMP schemas using RELAX NG.

[6] ISO 16684-3: Graphic Technology – Extensible metadata platform (XMP) specification – Part 3 JSON-LD serialization of XMP.

[7] https://www.cipa.jp/e/guide/aboutcipa.html

[8] https://www.jeita.or.jp/english/

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

https://www.dublincore.org