| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0208-v2 |
| --- | --- |

| Title: | EE2-2.8: Modifications to Matrix-based Intra Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongtao Wang, Patrick Garus, Bappaditya Ray, Muhammed Coban, Vadim Seregin, Marta Karczewicz, Samuel Eadie 5775 Morehouse drive San Diego, CA 92121, USA | Email: | hongtaow@qti.qualcomm.com pgarus@qti.qualcomm.com bray@qti.qualcomm.com mcoban@qti.qualcomm.com vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document reports the results of EE2 test 2.8, where the MIP matrix sizes are increased for the selected block sizes. When the proposed method is used, an L-shaped causal template is used to generate the prediction block. In test 2.8a, the prediction block generated by matrix multiplication has the same size as the current block, while in test 2.8b, for blocks sizes of 32x16, 16x32 and 32x32, a prediction block of 16x16 is generated by matrix multiplication and then up sampled to the size of the current block.

On top of ECM-13.0, the simulation results reportedly show:

Test 2.8a:

AI: -0.08% -0.03% -0.06% EncT: 100.5% DecT: 100.5%

RA: -0.04% 0.01% -0.01% EncT: 100.0% DecT: 99.8%

Test 2.8b:

AI: -0.07% -0.08% -0.04% EncT: 100.5% DecT: 100.9%

RA: -0.03% -0.09% -0.01% EncT: 100.0% DecT: 99.5%

# Proposed method

In this test, matrix sizes of the MIP modes are increased for the selected block sizes. The selected block sizes include those up to 32x32, with exceptions of 4x32, 32x4, 8x32 and 32x8. The proposed matrices use the L-shaped causal template (Fig. 1) as input to generate the WxH prediction block.

The prediction of a sample P(x,y) can be derived as:

P(x,y) = ∑k F(x,y,k)*r(k),

Where r(k) is the kth item in the L-shaped template, and F(x,y) is the matrix weights corresponding to the position (x,y).

Fig. 1: L shaped neighborhood for a given predicted block.

In test 2.8a, the size of the prediction block generated by matrix multiplication equals to the current block size. While in test 2.8b, for blocks sizes of 32x16, 16x32 and 32x32, a prediction block of 16x16 is generated by matrix multiplication and then up sampled to the size of the current block.

The memory usage of matrix coefficients is summarized in the following table.

Table 1 Memory usage of EE2-2.8a and EE2-2.8b

|  | EE2-2.8a | EE2-2.8b |
| --- | --- | --- |
| Memory usage | 3.68MB | 1.92MB |

The filters are trained with ~105K images of the Open Images Dataset.

# Experimental results

The proposed method has been implemented on top of the ECM-13.0. The simulations were performed following the common test conditions [2] and summarized in the below tables.

Table 2 Simulation results of EE2-2.8a

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.02% | 0.01% | 0.00% | 100.6% | 100.8% | 100.2% | 100.7% |
| Class A2 | -0.05% | -0.01% | 0.05% | 100.5% | 99.9% | 100.2% | 100.7% |
| Class B | -0.07% | -0.09% | -0.10% | 100.5% | 100.1% | 101.9% | 101.3% |
| Class C | -0.11% | -0.06% | -0.02% | 100.5% | 101.6% | 100.8% | 101.7% |
| Class E | -0.14% | 0.07% | -0.19% | 100.8% | 99.8% | 100.7% | 101.6% |
| Overall | -0.08% | -0.03% | -0.06% | 100.5% | 100.5% | 100.9% | 101.2% |
| Class D | -0.06% | 0.05% | -0.14% | 100.2% | 99.7% | 101.0% | 101.8% |
| Class F | -0.06% | -0.08% | 0.05% | 99.7% | 100.3% | 100.6% | 101.5% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class B | -0.06% | -0.04% | -0.18% | 99.6% | 100.0% | 100.4% | 101.0% |
| Class C | -0.04% | 0.07% | 0.19% | 100.3% | 99.9% | 100.2% | 101.5% |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class D | -0.05% | -0.49% | 0.19% | 100.0% | 100.5% | 100.9% | 101.8% |
| Class F | 0.00% | 0.06% | -0.07% | 100.1% | 99.9% | 100.2% | 101.1% |

Table 3 Simulation results of EE2-2.8b

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.01% | -0.02% | 0.02% | 100.5% | 100.9% | 100.2% | 100.7% |
| Class A2 | -0.05% | -0.09% | 0.03% | 100.4% | 101.0% | 100.2% | 100.7% |
| Class B | -0.05% | -0.07% | -0.03% | 100.7% | 101.6% | 100.5% | 101.4% |
| Class C | -0.08% | -0.04% | -0.03% | 100.7% | 100.1% | 100.8% | 101.7% |
| Class E | -0.13% | -0.18% | -0.21% | 100.1% | 100.6% | 100.7% | 101.6% |
| Overall | -0.07% | -0.08% | -0.04% | 100.5% | 100.9% | 100.5% | 101.3% |
| Class D | -0.05% | 0.03% | -0.20% | 101.1% | 101.5% | 101.0% | 101.8% |
| Class F | -0.06% | -0.02% | 0.00% | 100.0% | 99.6% | 100.6% | 101.5% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.05% | -0.16% | -0.17% | 100.3% | 99.1% | 100.1% | 100.7% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class B | -0.03% | -0.11% | -0.07% | 99.6% | 99.8% | 99.8% | 101.0% |
| Class C | -0.03% | -0.01% | 0.18% | 100.2% | 99.7% | 100.2% | 101.6% |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class D | -0.03% | -0.37% | 0.10% | 100.1% | 99.8% | 100.8% | 101.8% |
| Class F | -0.03% | 0.01% | -0.03% | 100.4% | 100.4% | 100.2% | 101.2% |

# References

- ECM-13.0: https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-13.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, Oct. 2023.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).