| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0226-v1 |
| --- | --- |

| Title: | AHG 12: Longer motion compensation interpolation filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ruoyang Yu Han Huang Vadim Seregin Marta Karczewicz | Email: | ruoyyu@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In ECM-13.0, the motion compensation (MC) interpolation filter used for generating luma prediction samples is 12-tap. In this contribution, an additional longer (16-tap) MC interpolation filter is proposed for the luma prediction sample generation. The proposed longer MC interpolation filter is activated when a current block satisfies a certain criterion. Otherwise, the current 12-tap interpolation filter is used. One example of such a criterion is whether the current block is a bi-predictive block. The proposed modification is reported to be implemented on top of ECM-13.0. The BD-rate PSNR impact is reported to be -0.XX% and -0.XX% under RA and LDB common test configurations, respectively.

# Introduction and Proposal

In ECM-13.0, the motion compensation (MC) interpolation filter used for generating luma prediction samples is 12-tap.

In this contribution, an additional longer (16-tap) MC interpolation filter is proposed for the luma prediction sample generation. The proposed longer MC interpolation filter is activated when a current block satisfies a certain criterion, otherwise, the current 12-tap interpolation filter is used. One example of such a criterion is when the current block is a bi-predictive block.

The proposed 16-tap interpolation filter coefficients are shown in the following table.

| Fractional sample position | Interpolation filter coefficients |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0/16 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 256 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1/16 | -1 | 2 | -4 | 5 | -7 | 11 | -19 | 259 | 12 | -3 | 0 | 1 | -1 | 1 | -1 | 1 |
| 2/16 | -2 | 3 | -5 | 7 | -11 | 17 | -31 | 254 | 31 | -11 | 5 | -2 | 1 | 0 | 0 | 0 |
| 3/16 | -2 | 4 | -6 | 9 | -14 | 22 | -41 | 245 | 51 | -19 | 10 | -5 | 3 | -1 | 0 | 0 |
| 4/16 | -2 | 4 | -7 | 11 | -16 | 26 | -48 | 234 | 72 | -27 | 14 | -8 | 4 | -2 | 1 | 0 |
| 5/16 | -2 | 4 | -7 | 11 | -17 | 28 | -52 | 219 | 95 | -35 | 19 | -11 | 7 | -4 | 2 | -1 |
| 6/16 | -2 | 4 | -7 | 12 | -18 | 29 | -54 | 202 | 117 | -41 | 22 | -13 | 8 | -4 | 2 | -1 |
| 7/16 | -2 | 4 | -7 | 11 | -18 | 29 | -53 | 183 | 140 | -47 | 25 | -15 | 9 | -5 | 3 | -1 |
| 8/16 | -1 | 3 | -6 | 10 | -17 | 28 | -51 | 162 | 162 | -51 | 28 | -17 | 10 | -6 | 3 | -1 |
| 9/16 | -1 | 3 | -5 | 9 | -15 | 25 | -47 | 140 | 183 | -53 | 29 | -18 | 11 | -7 | 4 | -2 |
| 10/16 | -1 | 2 | -4 | 8 | -13 | 22 | -41 | 117 | 202 | -54 | 29 | -18 | 12 | -7 | 4 | -2 |
| 11/16 | -1 | 2 | -4 | 7 | -11 | 19 | -35 | 95 | 219 | -52 | 28 | -17 | 11 | -7 | 4 | -2 |
| 12/16 | 0 | 1 | -2 | 4 | -8 | 14 | -27 | 72 | 234 | -48 | 26 | -16 | 11 | -7 | 4 | -2 |
| 13/16 | 0 | 0 | -1 | 3 | -5 | 10 | -19 | 51 | 245 | -41 | 22 | -14 | 9 | -6 | 4 | -2 |
| 14/16 | 0 | 0 | 0 | 1 | -2 | 5 | -11 | 31 | 254 | -31 | 17 | -11 | 7 | -5 | 3 | -2 |
| 15/16 | 1 | -1 | 1 | -1 | 1 | 0 | -3 | 12 | 259 | -19 | 11 | -7 | 5 | -4 | 2 | -1 |

# Results

The proposal has been implemented in ECM-13.0 and evaluated under the ECM RA and LDB common test configurations. The following table shows the objective results. It should be noted that the encoding and decoding run time are unreliable.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.72% | -0.23% | -0.37% | 103.1% | 107.7% | 100.1% | 100.0% |

|  | Low delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).