| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0229-v1 |
| --- | --- |

| Title: | EE2-Related: LIC for Screen Content |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tae Meon Bae Sachin Deshpande | Email: | taemeon@sharplabs.com sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This input document relates to LIC prediction for screen content. To account for the low noise and clear, accurate texture of screen content, non-subsampled LIC prediction and a reduced regularization weight are proposed. Additionally, L1 loss (Sum of Absolute Error) based LIC parameter refinement is performed to compensate for the effect of reduced regularization weight on outliers. On top of ECM-13.0, the performance of the proposed method is reported as follows:

LB: {-0.34%, 0.88%, 0.46%, 100.2%, 99.7%} for class F, {-0.17%, 0.14%, -0.08%, 101.0%, 99.7%} for class TGM.

RA: {-0.12%, -0.11%, -0.03%, 98.9%, 99.2%} for class F, {-0.09%, -0.12%, -0.01%, 99.7%, 99.6%} for class TGM.

# Proposed method

It is asserted that compared to camera captured content, screen content has lower noise and sharper, clear texture. To utilize these low-noise characteristics, the regularization weight used for the LIC prediction is reduced. And, to leverage the clear texture of screen contents, sub-sampling for large block is disabled. Furthermore, LIC parameter refinement is performed based on minimizing L1 loss (Sum of Absolute Error) instead of minimizing L2 loss (Sum of Square Error) which is currently used for LIC parameter prediction.

## LIC prediction configuration for Screen Contents

Two modifications are applied to LIC parameter prediction to enhance the prediction accuracy for screen content.

- Disable sub-sampling for large blocks.

- Reduce regularization weight: change the value of m_LICRegShift from 7 to 8.

Disabling sub-sampling takes advantage of the sharp texture of screen content. The characteristics of low noise and clear texture are addressed by reducing the regularization weight.

## LIC parameter refinement

Since LIC parameters are predicted from templates, some pixels in the templates may have different statistical characteristics from pixels in the CU, thus needing to be handled as outliers. Reducing regularization weight has a negative effect on LIC parameter prediction due to outliers. It is reported that L1 loss is more robust to outliers than L2 loss [1]. Therefore, to compensate for the effect of reduced regularization weight on outliers, LIC parameter refinement is performed after LIC prediction. After LIC parameter prediction, the predicted slope s and offset o are refined as follows:

- Predict offsets o1 and o2 corresponding to s+1 and s-1. If s is equal to 1<<shift, then do not perform LIC parameter refinement.

- Compute the L1 losses for (a, b) = ax+b -y, where (a, b) ∈ {(s,o), (s+1, o1), (s-1, o2)}.

- Select new slope and offset set that shows minimum sum of absolute error.

# Test Results

The proposed methods were implemented on top of the ECM-13.0 software [2] and tested based on the common test condition [3]. The test results are summarized below.

|  | Low delay B Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class F | -0.34% | 0.88% | 0.46% | 100.2% | 99.7% | #NUM! |
| Class TGM | -0.17% | 0.14% | -0.07% | 101.0% | 99.7% | #NUM! |

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class E |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class F | -0.12% | -0.11% | -0.03% | 98.9% | 99.2% | #NUM! |
| Class TGM | -0.09% | -0.12% | -0.01% | 99.7% | 99.6% | #NUM! |

# Conclusion

This input document proposes modifying LIC prediction for screen content. To consider the low noise, sharp, and clear texture characteristics of screen contents, regularization weight reduction, non-subsampled LIC prediction, and L1 loss-based LIC parameter refinement are introduced. The proposed method achieves a -0.34% coding gain for class F in LB. It is suggested to investigate the proposed method in EE2.

# References

- “Analysis of least absolute deviation”, Biometrika, Vol 95, Issue 1, Mar., 2008, pp. 107–122.

- ECM13.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-13.0

- M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, Oct., 2023.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Acknowledgement
These research results were obtained from the commissioned research (No.05101) by National Institute of Information and Communications Technology (NICT), Japan.