| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0296-v1 |
| --- | --- |

| Title: | AHG9: Questions regarding NNPFC SEI message with temporal extrapolation purpose |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard François, Didier Doyen, Philippe de Lagrange, Fabrice Urban | Tel: Email: | 1stname.lastname@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

In VSEI v4, a new purpose has been added to the NNPFC SEI message, related to temporal extrapolation. This feature allows the NNPF to generate one or more extrapolated pictures that follow all input pictures of the NNPF in output order. However, with current draft syntax, it is not possible to generate more than one temporally extrapolated picture per time instance, which may be an important feature for picture extrapolation applications where multiple hypotheses may be generated for the same temporal position.

This contribution suggests considering extending the temporal extrapolation purpose of the NNPFC SEI message to support generating multiple temporally extrapolated picture per time instance.

# Introduction

In VSEI v4, a new purpose has been added to the NNPFC SEI message, related to temporal extrapolation. This feature allows the NNPF to generate one or more extrapolated pictures that follow all input pictures of the NNPF in output order. Example use cases are very low delay computer vision applications (e.g. robotics, autonomous driving) to facilitate anticipatory decision making, or very low-latency applications (e.g. cloud gaming) to anticipate the rendering by emulating several hypotheses of actions.

When this new purpose was proposed [JVET-AG0089], it was commented that it may be relevant to consider generating multiple temporally extrapolated picture instances for a same time instance (or POC) [JVET-AG_notes_dF.docx], but that this option was not supported in current specification:

“It was pointed out that in picture extrapolation applications (navigation, cloud gaming) it is also common that multiple hypotheses are generated for the same temporal position. This would not be supported by the proposed syntax, but could be added and would make another differentiation from the current picture rate upsampling.”

In clause 8.28.1.1 of VSEI v3 or v4 or VSEI TuC text, the following sentence indicates that only one picture per time instance can be output from the NNPF:

“Within ListNnpfOutputPics there shall be no more than one picture pertaining to any particular output time instance. When for any particular picture in CroppedDecodedPictures there are multiple NNPFs activated and only one of the NNPFs is allowed to be chosen to be applied although any of the NNPFs may be chosen, the above constraint shall apply regardless of which NNPF is chosen to be applied to the particular picture.”

This constraint may need to be adapted to allow generating more than one temporally extrapolated picture per time instance.

This contribution raises this question and suggests finding a solution to address this problem.

# Open questions

Because the possibility of generating several instances of a same temporally extrapolated picture does not seem to be supported by VSEI v3, v4 or VSEI TuC documents, it is questioned whether one (or some) of the following solutions should be considered.

Option 1

One solution is to enable the generation from a same NNPF (and therefore NNPFC SEI) of different versions of temporally extrapolated picture(s) at same temporal position, by using different inputs to the NNPF. The same NNPF is run several times with different input and generates different output at same temporal position.

For example, in addition to the decoded pictures used as input, additional parameters may be used as input of the NNPF allowing to generate different output pictures from a same NNPF using same input decoded pictures.

The following text change in clause 8.28.1.1 of VSEI v3 or v4 or VSEI TuC text is suggested:

“When TemporalExtrapolationFlag is equal to 0, within ListNnpfOutputPics there shall be no more than one picture pertaining to any particular output time instance. When for any particular picture in CroppedDecodedPictures there are multiple NNPFs activated and only one of the NNPFs is allowed to be chosen to be applied although any of the NNPFs may be chosen, the above constraint shall apply regardless of which NNPF is chosen to be applied to the particular picture.

When TemporalExtrapolationFlag is equal to 1, within ListNnpfOutputPics there may be more than one picture pertaining to any particular output time instance. When for any particular picture in CroppedDecodedPictures there are multiple NNPFs activated and more than one of the NNPFs with TemporalExtrapolationFlag equal to 1 are chosen to be applied, each of the applied NNPFs may generate one picture pertaining to any particular output time instance.”

Option 2

Signaling of more than one NNPFC SEIs with the same nnpfc_id, where different NNPFC SEIs may generate different instances of a same temporally extrapolated output picture, and signaling of a new syntax element when nnpfc_purpose is temporal extrapolation, to indicate the id of the temporally extrapolated output picture instance. A syntax element may also be added to indicate the maximum number of instances. This syntax element must be the same for all NNPFC SEIs with the same nnpfc_id.

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| … |  |
| if( PictureRateUpsamplingFlag ) |  |
| for( i = 0; i < nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_interpolated_pics[ i ] | ue(v) |
| if( TemporalExtrapolationFlag ) { |  |
| nnpfc_extrapolated_pics_minus1 | ue(v) |
| nnpfc_extrapolated_pics_max_id | ue(v) |
| nnpfc_extrapolated_pics_id | ue(v) |
| } |  |
| … |  |

One NNPFC SEI may be used as base model for the other NNPFC SEIs potentially used for temporal extrapolation.

One illustration of this option is depicted in Figure 1.

Figure 1. illustration of option 1. Several NNPFC SEIs with same Id are signaled and potentially used to generate several instances of temporally extrapolated picture(s) at same time instance.

Option 3

Signaling of more than one (N) NNPFC SEIs with different nnpfc_id, and activating of the N NNPFC SEIs by N NNPFA SEIs, each one referring to one of the N NNPFC SEIs, and enabling the non-cascaded application of N temporal extrapolation NNPF processes to the same decoded pictures to generate multiple instances of each temporally extrapolated picture, using the N NNPFs.

One illustration of this option is depicted in Figure 2. The main difference with Figure 1 is that Ids of NNPFC0 and NNPFC1 are different, as well as reference Ids of NNPFA0 and NNPFA1.

Figure 2. illustration of option 2. Several NNPFC SEIs with different Ids are signaled and potentially used to generate several instances of temporally extrapolated picture(s) at same time instance.

Option 4

Signaling of different NN models in a same NNPFC SEI, each model targeting one instance of each extrapolated picture. In the example syntax table below, the maximum instance id is indicated (parameter nnpfc_extrapolated_pics_max_id gives the max id number, and therefore the number of instances is deduced as nnpfc_extrapolated_pics_max_id+1).

Corresponding proposed syntax changes are depicted in table below.

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| nnpfc_id | ue(v) |
| nnpfc_base_flag | u(1) |
| nnpfc_mode_idc | ue(v) |
| if( nnpfc_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_alignment_zero_bit_a | u(1) |
| nnpfc_tag_uri | st(v) |
| nnpfc_uri | st(v) |
| } |  |
| … |  |
| if( TemporalExtrapolationFlag ) { |  |
| nnpfc_extrapolated_pics_minus1 | ue(v) |
| nnpfc_extrapolated_pics_max_id | ue(v) |
| if( nnpfc_mode_idc = = 1 ) { |  |
| for( i = 1; i <= nnpfc_extrapolated_pics_max_id; i++ ) { |  |
| nnpfc_tag_uri_instance[ i ] | st(v) |
| nnpfc_uri_instance[ i ] | st(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| /* ISO/IEC 15938-17 bitstream */ |  |
| if( nnpfc_mode_idc = = 0 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_alignment_zero_bit_b | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnpfc_payload_byte[ i ] | b(8) |
| if( TemporalExtrapolationFlag ) { |  |
| for( j = 1; j <= nnpfc_extrapolated_pics_max_id; j++ ) { |  |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnpfc_payload_byte[ j ][ i ] | b(8) |
| } |  |
| } |  |
| } |  |
| } |  |

# Conclusion

This contribution suggests ensuring that the NNPFC / NNPFA SEI messages specification enable generating multiple instances of temporally interpolated picture for a same time instance, as this is a relevant feature for practical use cases requiring low latency.

Some possible options are proposed.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).