| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0324-v2 |
| --- | --- |

| Title: | AHG12: temporal ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luhang Xu, Yue Yu, Haoping Yu, Dong Wang | Tel: Email: | xuluhang@oppo.com |
| Source: | OPPO |  |  |

- Abstract

This contribution proposes an adaptive loop filter that uses temporal reconstructed information as the input to a filter whose output will enhance the current picture. The proposed temporal adaptive loop filter (TALF) is implemented on the ECM-13.0 reference software. The simulation results show that the proposed method can achieve the following BD-rates change against the reference software:

RA:   class-B -0.19%, class-C -0.03%, class-D -0.05%, class-F -0.23%,

LDB: class-B -0.xx%, class-C -0.09%, class-D -0.21%, class-E -0.09%.

Introduction

In ECM-13.0, ALF uses spatial reconstructed information, residual information, deblocking information, and fixed filtered information as input to improve the current reconstructed picture. The online-trained ALF coefficients and the information on filter precision and non-linear clipping are transmitted in the bitstream. The history ALF filters can also be used in the current picture’s filtering.

This proposal proposes a temporal adaptive loop filter (TALF) that uses temporal reconstructed information to improve the current picture’s quality. Similar to ALF, the filter coefficients and related information are transmitted in the bitstream and the history of TALFs can also be used in the current picture’s filtering.

Proposed method

The proposed temporal ALF can have up to 8 sets of coefficients for each slice. The proposed TALF tool is enabled by a CTU-level flag.

Figure-1. The proposed TALF in the loop filters

Filter shape

The proposed TALF supports both uni-directional and bi-directional filtering structures. There is one TALF uni-directional filter that is a 7x7 diamond-shaped symmetric filter with 13 taps. There are two TALF bi-directional filters that are both 5x5 diamond-shaped symmetric filters with 7 taps each. The proposed filters are illustrated in Figure 2.

Figure-2. The uni-directional filter (left) and the bi-directional filters (right)

Filtering mode

The proposed TALF has four filtering modes.

The first filtering mode is a uni-filtering mode, which uses the uni-Mv and reference picture information to obtain the offset positions in the reference picture and uses those positions for the temporal uni-filtering. The current position that doesn’t have uni-Mv will skip the filtering.

The second filtering mode is also a uni-filtering mode, it uses the reference picture information to obtain the collocated positions in the reference picture for the temporal uni-filtering.

The third filtering mode is a bi-filtering mode, it uses the bi-Mvs and reference picture information to obtain the offset positions in the two reference pictures and uses those positions for temporal bi-filtering. The current position that doesn’t have bi-Mvs will skip the filtering.

The fourth filtering mode is also a bi-filtering mode, it uses the reference picture information to obtain the collocated positions in the two reference pictures for the temporal bi-filtering.

Figure-3. The input of the uni-directional filter is the reconstructed samples guided by the uni-MV of the current position. The MV for guiding the position offset is rounded into integer precision.

Figure-4. The input of the bi-directional filter are the reconstructed samples guided by the bi-MVs of current position. The MVs for guiding the position offset are rounded into integer precision.

Other features

The uni-directional and the bi-directional filters can inherit the history of uni-directional filters and the bi-directional filters that are used in coding the previous blocks. Two history tables with a max size equal to 8 are utilized to maintain the history of uni-directional filters and bi-directional filters. A similar non-linear clipping method as well as the adaptive precision for the filter coefficients used in ALF is also implemented in TALF.

Simulation results

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class B | -0.19% | 0.14% | -0.02% | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class C | -0.03% | 0.07% | 0.06% | 101.4% | 106.7% | 101.1% | 100.6% |
| Class D | -0.05% | 0.12% | 0.29% | 101.8% | 108.5% | 100.0% | 100.2% |
| Class F | -0.23% | 0.15% | 0.11% | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

|  |  | Low delay B Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over ECM-13.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | U |  | V |  | EncT |  | DecT |  | EncVmPeak |  | DecVmPeak |  |
| Class B |  | -0.15% |  | 0.16% |  | 0.13% |  | #DIV/0! |  | #DIV/0! |  | #DIV/0! |  | #DIV/0! |  |
| Class C |  | -0.09% |  | 0.17% |  | -0.30% |  | #DIV/0! |  | #DIV/0! |  | #DIV/0! |  | #DIV/0! |  |
| Class E |  | -0.09% |  | 0.47% |  | 0.72% |  | #DIV/0! |  | #DIV/0! |  | #DIV/0! |  | #DIV/0! |  |
| Overall |  | -0.11% |  | 0.24% |  | 0.14% |  | #DIV/0! |  | #DIV/0! |  | #DIV/0! |  | #DIV/0! |  |
| Class D |  | -0.21% |  | -0.04% |  | 0.40% |  | #DIV/0! |  | #DIV/0! |  | #DIV/0! |  | #DIV/0! |  |

Conclusions

The proposed method can bring coding benefits for both RA and LDB configurations on top of ECM-13.0. It is proposed to further study the proposed method in the next round of EE2.

References

- ECM13.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-13.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, Oct, 2023.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).