| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, Japan, 12–19 July 2024 | Document: JVET-AI1004-v3 |
| --- | --- |

| Title: | Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Errata report |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Benjamin Bross Iole Moccagatta Chris Rosewarne Gary J. Sullivan | Tel: Email: | yekui.wang@bytedance.com benjamin.bross@hhi.fraunhofer.de iole.moccagatta@intel.com chris.rosewarne@canon.com.au gary.sullivan@dolby.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document contains a list of reported errata items for VVC, VSEI, HEVC, AVC, and Video CICP, for tracking purposes. Some of the items have been confirmed by the JVET and have been agreed to require fixing. This document also provides publication status backgrounds of these standards.

Incorporated items at the JVET-AI meeting:

- For VVC:

  - Spec tickets #1609, #1617, #1618, #1624, #1627, #1628, #1629, #1630, #1631, #1632, #1634, #1635, and #1644 and comments relating to those

  - Updated publication status

  - Essentially typographical comments (about “Y = !X” convention, consistency issues for spaces, fractions expressed with the “/” operator)

  - Incorrect variable name “max_val_initial_removal_delay_for_splicing”.

- For HEVC:

  - Minor editorial wording corrections in clause 9.3.1 relating to cabac_bypass_alignment_enabled_flag

  - Spec ticket #1644

  - Updated publication status

  - Refined comments about “Y = !X” convention

  - Added comments related to high throughput, multiview and scalable profile specifications

  - Added error in description of version for ISO/IEC

- For AVC

  - Updated publication status

  - Comment about minor matters reported for other standards (e.g., the “Y = !X” convention)

- For CICP

  - Updated publication status

Incorporated items at the JVET-AH meeting:

- For VVC: Updated the publication status

- For VSEI: Updated the publication status

- For HEVC: Removed errata items that have been integrated into JVET-AH1006 as well as into the ITU-T consent text and the ISO/IEC FDIS text.

- For AVC: Removed errata items that have been integrated into JVET-AH1016 as well as into the ITU-T consent text and the ISO/IEC FDIS text.

Incorporated items at the JVET-AG meeting:

- For VVC:

  - In subclause 7.4.2.4.5, the first bullet item, replace "subpicture level values" with "subpicture index values". (reported by Rickard Sjöberg – thanks!)

- For VSEI:

  - Fix a typo when vui_chroma_sample_loc_type_frame is mentioned in the VSEI text. (JVET-AG0192)

  - Fix two bugs for the constraints on nnpfc_matrix_coeffs and add two NOTEs in the semantics of nnpfc_matrix_coeffs to clarify complicated cases. (JVET-AG0192)

  - Fix an asserted bug on the inference of nnpfc_full_range_flag. (JVET-AG0192)

Incorporated items at the JVET-AF meeting:

- For VVC:

  - Add a missing space in "if(IntraSubPartitionsSplitType  !=  ISP_NO_SPLIT )" (JVET-AF0064).

  - On coding of syntax elements of fix-valued, non-reserved bits using f(N) instead of u(N). (JVET-AF0064).

  - Replace the two instances of "in the first access unit of the CVS" with "in the first access unit of the CVS in decoding order". (JVET-AF0189)

- For VSEI:

  - Replace each of the 10 instances of "nuh_layer_id" in subclauses 8.22.2 and 8.23.2 with "layer identification". (from Gary J. Sullivan)

  - On cancellation of the persistence of a CTI SEI message by another CTI SEI message. (JVET-AF0064)

  - On coding of syntax elements of fix-valued, non-reserved bits using f(N) instead of u(N). (JVET-AF0064).

  - Consider adding a NOTE in VSEI telling something like: 1) AVC can reference VSEI for enabling the inclusion of only those SEI messages for which the SEI payloads are specified in VSEI but the syntax has not been extended; 2) HEVC and VVC can reference VSEI for enabling the inclusion of SEI messages for which the SEI payloads are specified in VSEI regardless of whether the syntax has been extended or not. (from Ye-Kui Wang)

- For HEVC:

  - Updated the publication status background.

  - (Perhaps for other specs such as AVC, VVC, and VSEI as well) On that the term of "true subset" should be replaced with "proper subset" or "strict subset". (from Gary J. Sullivan)

  - (Perhaps for other specs such as AVC, VVC, and VSEI as well) On that the mixed uses of Boolean operators on numerical values and vice versa should cleaned up. (from Gary J. Sullivan)

  - Removed obsoleted HEVC errata items that were included in JVET-AE1004.

- For AVC:

  - Removed obsoleted AVC errata items that were included in JVET-AE1004.

Incorporated items at the JVET-AE meeting:

- Updated the general descriptions for VVC, VSEI, HEVC, and AVC.

- Removed obsolete/resolved errata items.

- Added 16 items for the ISO text only for HEVC, identified when working on the ITU-T consent text for H.265 version 9 (edition 5).

- Added 9 items on the semantics of post-filter hint SEI message from JVET-AE0155 for both HEVC and AVC.

- Added 4 items for AVC, on semantics of the omnidirectional video SEI messages. These items were among the items for HEVC and have been resolved for HEVC.

- Added an item for AVC on use of the "should" wording for the use of the film grain characteristics SEI messages when present.

- Removed the section for codepoint usage TR as the TR part has been dropped from the title of the document a while ago.

- Added an item for HEVC regarding replacing of "Max slice segments per picture MaxSliceSegmentsPerPicture" in Table A.8 with "Max # of slice segments per picture MaxSliceSegmentsPerPicture" (reported by Cliff Reader – thanks!).

Incorporated items at the JVET-AD meeting:

- For AVC, incorporated the following items:

  - JVET-AD0078 item 4 and 5 on fixing the equation to derive the filter hint cross correlation matrix.

Incorporated items at the JVET-AC meeting:

- For VVC, all errata items have been incorporated in JVET-AC2005: New level and systems-related supplemental enhancement information for VVC (Draft 4).

- For HEVC, incorporated the following items:

  - Confirmed redudnant call to clause 8.6.1 (reported by Cliff Reader – thanks!).

  - Confirmed JVET-AB0120: Addition of interpolation for "initial CPB removal delay offset".

  - JVET-AC0311: On number of conformance tests (option 1.)

  - Updated issues 1 to 4 in Subsection 4.4 per inputs from Miska M. Hannuksela (thanks!).

  - Change the font size from 8 pt to 10 pt for "Table 9-4" in a paragraph in clause 9.3.2.2 (reported by Cliff Reader – thanks!).

- For AVC, incorporated the following items:

  - Confirmed JVET-AB0120: Addition of interpolation for "initial CPB removal delay offset".

  - An issue of talking about VUI in subclause 0.6.1 in ISO/IEC 14496-10:2022 (Edition 10) while the sentence should only talk about SEI messages.

Incorporated items at the JVET-AB meeting:

- For VVC, incorporated the following items:

  - JVET-AB0223: Text improvement for Timing / DU information SEI message in HEVC and VVC

  - Added a NOTE in the HRD text to mention that "leading pictures" associated with a DRAP picture may not be correctly decodable, per the discussion of JVET-AB0055 (On leading pictures design in DRAP SEI Message).

  - Added a NOTE to mention that the definition of IRAP in VSEI covers the case of GDR with ph_recovery_poc_cnt equal to 0, per the discussion of JVET-AB0057 (On the associated IRAP for DRAP and EDRAP pictures).

  - Tickets #1564, #1568, #1569, and #1572

  - JVET-AB0120: Addition of interpolation for "initial CPB removal delay offset"

- For HEVC, incorporated the following items:

  - JVET-AB0223: Text improvement for Timing / DU information SEI message in HEVC and VVC

  - On conformance indication of the Multiview Main, Scalable Main, Scalable Main 10, and 3D Main profiles (resulted from a discussion between Alexis Tourapis and Gary Sullivan – thanks!)

  - On inference of high_precision_offsets_enabled_flag (resulted from a discussion between Andy Fu and Gary Sullivan – thanks!)

  - On a redundant sentence in the definitionn of next_bits( n ) from Gary Sullivan

  - On addition the parenthetic abbreviation TB to the text in clause 9.3.3.1 (suggested by Cliff Reader– thanks!)

  - On indention of item 3 in the paragraph just below Equation 8-14 in clause 8.4.1 (reported and suggested by Cliff Reader – thanks!)

  - On redudnant call to clause 8.6.1 (reported by Cliff Reader – thanks!)

  - JVET-AB0120: Addition of interpolation for "initial CPB removal delay offset"

- For AVC, incorporated the following items:

  - JVET-AB0120: Addition of interpolation for "initial CPB removal delay offset"

Incorporated items at the JVET-AA meeting:

- For VVC: Added an errata item for VVC conformance.

- For VSEI: Added a pointer of some HEVC errata items that also apply for VSEI in Subsection 4.2.

- For HEVC: Removed various items that have been addressed during the editing of the DIS text of HEVC 5th edition in WG05 N00128, and added some editor's notes for the remaining items.

- For AVC:

  - Added changes on motion vector value ranges, based on JVET meeting minutes noted under JVET-AA0222.

  - Added a pointer of some HEVC errata items that also apply for AVC in Subsection 4.2.

Incorporated items at the JVET-Z meeting:

- For AVC, corrections to texts related to alpha blending corresponding to the changes adopted to VSEI from JVET-Z0119.

- Updated the summary and publication status for VVC.

- Updated the summary and publication status for VSEI.

- Updated the summary for HEVC.

Incorporated items at the JVET-Y meeting:

- Removed all errata items for VVC and VSEI, due to that for both VVC and VSEI the second edition texts are being prepared by both ITU-T and ISO/IEC, and all the errata items have been incorporated therein, including fixes for bug tickets #1533 and #1534 for VVC (fixes for these two tickets have not been included in previous versions of the errata report).

- Removed errata items for AVC that have been resolved in both the latest ITU-T text (08/2021) and the new submission (MDS21132_WG05_N00098) of the ISO/IEC 10th edition text.

Incorporated items at the JVET-X meeting:

- Updated the publication status of the standards

- For VVC (the changes are included in an attachment to this document):

  - Some changes resulted from reports/suggestions by Yue Yu and Frank Bossen (thanks!)

  - Various changes resulted from reports/suggestions by Peter de Rivaz (huge thanks!)

- For HEVC (the changes are included both below and in an attachment to this document):

  - Some changes resulted from reports/suggestions by Yue Yu, Frank Bossen, and Cliff Reader (thanks!)

  - Some changes to the semantics of the entry point offset syntax element, resulted from the discussion of JVET-X0050

  - Changes to the semantics of the alpha channel information SEI message resulted from the discussion of JVET-X0059

- For AVC (the changes are included in an attachment to this document):

  - Some changes to the semantics of the alpha blendnig related syntax elements in clause 7.4.2.1.2, message resulted from the discussion of JVET-X0059

Incorporated items at the JVET-W meeting:

- For VVC (the changes are included in an attachment to this document):

  - Fix for tickets #1488, #1511, #1491, #1493, #1494

- For HEVC:

  - Added the attachment for HEVC from JVET-W0187, which includes the following fixes:

    - Correcting an “off by one” error in the loop counter for ar_num_object_updates, which indicates the number of updated annotated objects for the annotated regions SEI message.

    - Adding a description of the purpose of some variables that are derived in the semantics of the annotated regions SEI message

    - Adding a missing descriptor to specify the type of coding for the depth_nonlinear_representation_model[ i ] syntax element (the same as used in another specification, so clearly what was intended) for the depth representation information SEI message

    - Adding missing range specifications for a few syntax elements of the depth representation information SEI message

    - Miscellaneous very small cleanups of consistency, grammar and clarity, and correction of typographical errors.

- For AVC:

  - Added the attachment for HEVC from JVET-W0187, which includes the same list of fixes as above for HEVC.

Incorporated items at the JVET-V meeting:

- For VVC (the changes are included in an attachment to this document):

  - Fix for ticket #1486

- For HEVC

  - Removed the item on the absence of “persistence flag” for the annotated regions SEI message (this was agreed to be removed at the JVET-T meeting in October 2020, but the decision was overlooked)

Incorporated items at the JVET-U meeting:

- For VVC (the changes are included in an attachment to this document):

  - Some minor editorial corrections and improvements (JVET-U0073, JVET-U0085, and one item from Hendry)

  - Fixes for tickets #1416, #1428, #1432, #1454, and #1469

- For AVC: Added a reference to JVET-T1006-v1 for the errata changes, and removed the actual errata text changes from this document. (JVET-U0049)

# General aspects and issues affecting multiple standards

## General and minor matters

Please note that the alignment between the text published by ITU-T and that published by ISO/IEC should also be checked. Minor editorial issues and aspects that are highlighted for potential further checking include the following:

- None noted.

# Reported errata items for VVC

## General

The ITU-T and ISO/IEC third edition texts have been published. Besides those errata items directly included below in this section, JVET-AI2005 may include some VVC errata changes (in addition to some amendments).

## Publication status background

ISO/IEC 23090-3 (https://www.iso.org/standard/86516.html)

- ISO/IEC 23090-3:2024 (Edition 3); published 2024-07-17

Rec. ITU-T H.266 (https://www.itu.int/rec/T-REC-H.266/en)

- (09/2023, version 3) Approved 2023-09-29, published 2023-11-29

Rec. ITU-T H.266 (https://www.itu.int/rec/T-REC-H.266/en)

- (04/2022, version 2) Approved 2022-04-29, published 2022-07-12

ISO/IEC 23090-3 (https://www.iso.org/standard/83531.html)

- ISO/IEC 23090-3:2022 (Edition 2); published 2022-09

Rec. ITU-T H.266 (https://www.itu.int/rec/T-REC-H.266/en)

- (08/2020, version 1) Approved 2020-08-29, published 2020-11-10

ISO/IEC 23090-3 (https://www.iso.org/standard/73022.html)

- ISO/IEC 23090-3:2021 (Edition 1); published 2021-02

## Errata items for VVC

- Spaces missing or extra spaces present:

  - In subclause 7.3.11.10 (Transform unit syntax), there is an instance of “if(IntraSubPartitionsSplitType  !=  ISP_NO_SPLIT )”, where a space is missing after ‘(’. Several other similar instances can be found with a wildcard search in 7.3 for “if\([A-Z]” and “if\([a-z]”.

  - In subclause 7.3.2.4 (Sequence parameter set RBSP syntax), there is an instance of “if( !sps_independent_subpics_flag)”, where a space is missing before ‘)’. Similar other similar instances can be found with a wildcard search in 7.3 for "[a-u]\)” and “[A-Z]\)”.

  - In 7.3.7, there is a “!” followed by a space, which is inconsistent with all other uses of the “!” operator.

- In subclause 7.4.2.4.5, the first bullet item, replace "subpicture level values" with "subpicture index values". (This problem is not present in the ISO/IEC 3rd edition, but it is present in the ITU 3rd edition.)

- In D.6.1, change the coding of sn_zero_bit from u(1) to f(1). It would be better to consistently use f(1) for all fix-valued, non-reserved bits. The same approach could apply for all fix-valued, non-reserved bytes, two-bytes, etc.

- Replace the two instances of "in the first access unit of the CVS" and the three instances of "in the first AU of the CVS" with "in the first AU of the CVS in decoding order". (JVET-AF0189)

- Spec ticket #1609: NoBackwardPredFlag derivation ambiguity

swarrington: 8.3.6 derivation of NoBackwardPredFlag is as below:

- If DiffPicOrderCnt( aPic, currPic ) is less than or equal to 0 for each active picture aPic in RefPicList0 or RefPicList1 of the current slice, NoBackwardPredFlag is set equal to 1.

- Otherwise, NoBackwardPredFlag is set equal to 0.

or seems somewhat ambiguous here. It could be interpreted that if the DiffPicOrderCnt condition is valid for either L0 or L1 list then you should set NoBackwardPredFlag to 1. Using and seems to make the meaning more unambiguous.

bbross: Good catch, thanks! I agree that using "and" instead of "or" remove the ambiguity. Will be fixed in v4.

- Spec ticket #1617: Not initialized NumCtusInSlice[0] to 0

djpark0115: In VVC V2 6.5.1, NumCtusInSlice[0] = 0 is missing.

if( pps_single_slice_per_subpic_flag ) {
	if( !sps_subpic_info_present_flag ) /* There is no subpicture info and only one slice in a picture. */
		for( j = 0; j < NumTileRows; j++ )
			for( i = 0; i < NumTileColumns; i++ )
				AddCtbsToSlice( 0, TileColBdVal[ i ], TileColBdVal[ i + 1 ], TileRowBdVal[ j ],
						TileRowBdVal[ j + 1 ] )

should be replaced by

if( pps_single_slice_per_subpic_flag ) {
	if( !sps_subpic_info_present_flag ) /* There is no subpicture info and only one slice in a picture. */
		NumCtusInSlice[0] = 0
		for( j = 0; j < NumTileRows; j++ )
			for( i = 0; i < NumTileColumns; i++ )
				AddCtbsToSlice( 0, TileColBdVal[ i ], TileColBdVal[ i + 1 ], TileRowBdVal[ j ],
						TileRowBdVal[ j + 1 ] )

bbross: Good catch, will be fixed in VVC v4.

- Spec ticket #1618: [Multilayer Profiles] Potential Mismatch of VTM22.0 & Specification Related To Derivation Process For Merge Motion Vector Difference

ksthey: In "8.5.2.7 Derivation process for merge motion vector difference", Equation 563:
mMvdL1[ 0 ] = Sign( currPocDiffL0 ) == Sign( currPocDiffL1 ) ? mMvdL0[ 0 ] : −mMvdL0[ 0 ].

The equivalent code is in VTM22.0, InterPrediction.cpp, MergeCtx::getMmvdDeltaMv(), starting from line 2478:

if ((poc1 - currPoc)*(poc0 - currPoc) > 0)
{

deltaMv[1] = deltaMv[0];

}
else
{

deltaMv[1].set(-1 * deltaMv[0].getHor(), -1 * deltaMv[0].getVer());

}

According to specification, definition of Sign(x):
Sign(x) = 1; x > 0
Sign(x) = 0; x == 0
Sign(x) = -1; x < 0

Let's assume:

- result of condition below in specification as r_spec: r_spec = Sign( currPocDiffL0 ) == Sign( currPocDiffL1 )

- result of condition below in VTM22.0 as r_vtm: r_vtm = (currPocDiffL1 * currPocDiffL0 > 0)

Use combination of currPocDiffL0 & currPocDiffL1 as input:
1.) (currPocDiffL0 < 0) && (currPocDiffL1 < 0)

r_spec = 1; r_vtm = 1; ==> r_spec == r_vtm;

2.) (currPocDiffL0 < 0) && (currPocDiffL1 > 0)

r_spec = 0; r_vtm = 0; ==> r_spec == r_vtm;

3.) (currPocDiffL0 > 0) && (currPocDiffL1 < 0)

r_spec = 0; r_vtm = 0; ==> r_spec == r_vtm;

4.) (currPocDiffL0 < 0) && (currPocDiffL1 < 0)

r_spec = 1; r_vtm = 1; ==> r_spec == r_vtm;

5.) (currPocDiffL0 == 0) && (currPocDiffL1 != 0)

r_spec = 1; r_vtm = 1; ==> r_spec == r_vtm;

6.) (currPocDiffL0 != 0) && (currPocDiffL1 == 0)

r_spec = 1; r_vtm = 1; ==> r_spec == r_vtm;

7.) (currPocDiffL0 == 0) && (currPocDiffL1 == 0)

r_spec = 1; r_vtm = 0; ==> r_spec != r_vtm;

Case 5, 6 & 7 are only possible in multilayer profile (current picture and reference picture same POC, but different layers).
As shown above, for case 7, results of specification & VTM are different.

So, which is the correct one? specification or VTM?

bbross: Thanks for reporting and I see the mismatch for the multi-layer case. This should be fixed in v4 but before, we need to clarify which one is the intended behaviour.

- Spec ticket #1624: Incorrect indexing in computation of motion vector offset

peterderivaz: In the decoding process for inter blocks (8.5.6.1) there are three places where the expression refMvLX[ xSbIdx ][ xSbIdx ] is used. I believe these should be replaced by refMvLX[ xSbIdx ][ ySbIdx ]. (This applies to "ITU-T H.266 (V3) (09/2023)" but I couldn't find an appropriate version in the dropdown.)

bbross: Good catch, thanks! Should be fixed in v4 as suggested.

- Spec ticket #1627: "Decoding process for palette mode" does not say what to do with output samples

peterderivaz: Section 8.4.5.3 "Decoding process for palette mode" has an output of an array recSamples[ x ][ y ], but in each of the three places where this process is invoked there is no description of what to do with the output samples. I think the samples should probably be copied to a rectangle with either SLu, SCb, or SCr, and that the IsAvailable flag for the locations should be set. i.e. operations similar to those in 8.7.5.1. Also IbcVirBuf may need to be set if it is legal for intra block copy to reference palette mode decodes.

bbross: Thanks for bringing that up and I can confirm that the output needs to be copied into the reconstructed picture. Should be fixed for v4 and I will think about an appropriate solution and update the ticket with one for review.

- Spec ticket #1628: Derivation of ModeTypeCondition should say "one or more"

peterderivaz: The derivation of ModeTypeCondition reads:

The variable ModeTypeCondition is derived as follows:

If one or more of the following conditions are true, ModeTypeCondition is set equal to 0:

– sh_slice_type is equal to I and sps_qtbtt_dual_tree_intra_flag is equal to 1;

– modeTypeCurr is not equal to MODE_TYPE_ALL;

– sps_chroma_format_idc is equal to 0;

– sps_chroma_format_idc is equal to 3;

– Otherwise, if one or more of the following conditions is true, ModeTypeCondition is set equal to 1:

– cbWidth * cbHeight is equal to 64 and split_qt_flag is equal to 1;

– cbWidth * cbHeight is equal to 64 and split_qt_flag is equal to 0 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_TT_HOR or SPLIT_TT_VER;

– cbWidth * cbHeight is equal to 32 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_BT_HOR or SPLIT_BT_VER;

– Otherwise, if one of the following conditions is true, ModeTypeCondition is set equal to 1 + ( sh_slice_type != I ?

1 : 0 ):

– cbWidth * cbHeight is equal to 64 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_BT_HOR or SPLIT_BT_VER and sps_chroma_format_idc is equal to 1;

– cbWidth * cbHeight is equal to 128 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_TT_HOR or SPLIT_TT_VER and sps_chroma_format_idc is equal to 1;

– cbWidth is equal to 8 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_BT_VER;

– cbWidth is equal to 16 and split_qt_flag is equal to 0 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_TT_VER;

– Otherwise, ModeTypeCondition is set equal to 0.

The "if one of the following conditions is true" should read "if one or more of the following conditions is true".

This can cause decoding differences for legal bitstreams. For example, cbWidth = 16, cbHeight = 8, MttSplitMode[ x0 ][ y0 ][ mttDepth ] = SPLIT_TT_VER, sps_chroma_format_idc = 1, split_qt_flag = 0.

In this case, two of the conditions can be true at the same time.

bbross: Good catch, thanks! Should be be fixed in v4 as suggested.

- Spec ticket #1629: mtt_split_cu_vertical_flag context uses undefined variable chType

peterderivaz: The context derivation for mtt_split_cu_vertical_flag says it has an input of "the dual tree channel type chType" but it is not clear where this comes from.

mtt_split_cu_vertical_flag is read in the coding tree syntax which does not mention chType.

chType is mentioned in a different clause (the coding unit syntax), but it seems unlikely this is the value to be used because it has an apparent dependency on mtt_split_cu_vertical_flag via:

- mtt_split_cu_vertical_flag is used to compute MttSplitMode

- MttSplitMode is used to compute ModeTypeCondition

- ModeTypeCondition is used to compute modeType

- modeType is used to compute treeType

- treeType is used to compute chType.

What value should be used for chType when reading the mtt_split_cu_vertical_flag syntax element?

bbross: Thanks and this needs indeed clarification. Will look for a solution and update the post when I have cone up with one.

- Spec ticket #1630: Missing equations for applying AmvrShift

peterderivaz: In the semantics for amvr_precision_idx, some motion vectors are adjusted as follows:

Otherwise (inter_affine_flag[ x0 ][ y0 ] is equal to 1), the variables

MvdCpL0[ x0 ][ y0 ][ 0 ][ 0 ],

MvdCpL0[ x0 ][ y0 ][ 0 ][ 1 ],

MvdCpL0[ x0 ][ y0 ][ 1 ][ 0 ],

MvdCpL0[ x0 ][ y0 ][ 1 ][ 1 ],

MvdCpL0[ x0 ][ y0 ][ 2 ][ 0 ]

and MvdCpL0[ x0 ][ y0 ][ 2 ][ 1 ] are modified as follows:

MvdCpL0[ x0 ][ y0 ][ 0 ][ 0 ] = MvdCpL0[ x0 ][ y0 ][ 0 ][ 0 ] << AmvrShift (165)

MvdCpL1[ x0 ][ y0 ] [ 0 ][ 1 ] = MvdCpL1[ x0 ][ y0 ][ 0 ][ 1 ] << AmvrShift (166)

MvdCpL0[ x0 ][ y0 ][ 1 ][ 0 ] = MvdCpL0[ x0 ][ y0 ][ 1 ][ 0 ] << AmvrShift (167)

MvdCpL1[ x0 ][ y0 ] [ 1 ][ 1 ] = MvdCpL1[ x0 ][ y0 ][ 1 ][ 1 ] << AmvrShift (168)

MvdCpL0[ x0 ][ y0 ][ 2 ][ 0 ] = MvdCpL0[ x0 ][ y0 ][ 2 ][ 0 ] << AmvrShift (169)

MvdCpL1[ x0 ][ y0 ] [ 2 ][ 1 ] = MvdCpL1[ x0 ][ y0 ][ 2 ][ 1 ] << AmvrShift (170)

Note that the description says that 6 locations in the array MvdCpL0 are modified, while the equations modify 3 locations in the array MvdCpL0 and 3 locations in the array MvdCpL1.

I wonder if both the description and the equations are incomplete, and the correct thing to do is to modify the 6 locations in the array MvdCpL0 and also the same 6 locations in the array MvdCpL1?

bbross: Good catch, thanks! This is probably a copy/paste typo from the non-affine Mvd arrays above and should be fixed to:

Otherwise (inter_affine_flag[ x0 ][ y0 ] is equal to 1), the variables

MvdCpLX[ x0 ][ y0 ][ 0 ][ 0 ],

MvdCpLX[ x0 ][ y0 ][ 0 ][ 1 ],

MvdCpLX[ x0 ][ y0 ][ 1 ][ 0 ],

MvdCpLX[ x0 ][ y0 ][ 1 ][ 1 ],

MvdCpLX[ x0 ][ y0 ][ 2 ][ 0 ]

and MvdCpLX[ x0 ][ y0 ][ 2 ][ 1 ], with X being equal to 0 or 1, are modified as follows:

MvdCpLX[ x0 ][ y0 ][ 0 ][ 0 ] = MvdCpLX[ x0 ][ y0 ][ 0 ][ 0 ] << AmvrShift (165)

MvdCpLX[ x0 ][ y0 ] [ 0 ][ 1 ] = MvdCpLX[ x0 ][ y0 ][ 0 ][ 1 ] << AmvrShift (166)

MvdCpLX[ x0 ][ y0 ][ 1 ][ 0 ] = MvdCpLX[ x0 ][ y0 ][ 1 ][ 0 ] << AmvrShift (167)

MvdCpLX[ x0 ][ y0 ] [ 1 ][ 1 ] = MvdCpLX[ x0 ][ y0 ][ 1 ][ 1 ] << AmvrShift (168)

MvdCpLX[ x0 ][ y0 ][ 2 ][ 0 ] = MvdCpLX[ x0 ][ y0 ][ 2 ][ 0 ] << AmvrShift (169)

MvdCpLX[ x0 ][ y0 ] [ 2 ][ 1 ] = MvdCpLX[ x0 ][ y0 ][ 2 ][ 1 ] << AmvrShift (170)

The use of the word “modified” also seems questionable, since the variables are not actually modified when AmvrShift is equal to 0.

- Spec ticket #1631: Should "Motion vector storing process for geometric partitioning mode" store HpelIfIdx?

peterderivaz: In the General clause of "Decoding process for inter blocks", the array HpelIfIdx is written at the same time as other motion vector information is stored:

MvL0[ xSb + x ][ ySb + y ] = mvL0[ xSbIdx ][ ySbIdx ] (887)

MvL1[ xSb + x ][ ySb + y ] = mvL1[ xSbIdx ][ ySbIdx ] (888)

MvDmvrL0[ xSb + x ][ ySb + y ] = refMvL0[ xSbIdx ][ ySbIdx ] (889)

MvDmvrL1[ xSb + x ][ ySb + y ] = refMvL1[ xSbIdx ][ ySbIdx ] (890)

RefIdxL0[ xSb + x ][ ySb + y ] = refIdxL0 (891)

RefIdxL1[ xSb + x ][ ySb + y ] = refIdxL1 (892)

PredFlagL0[ xSb + x ][ ySb + y ] = predFlagL0[ xSbIdx ][ ySbIdx ] (893)

PredFlagL1[ xSb + x ][ ySb + y ] = predFlagL1[ xSbIdx ][ ySbIdx ] (894)

HpelIfIdx[ xSb + x ][ ySb + y ] = hpelIfIdx (895)

BcwIdx[ xSb + x ][ ySb + y ] = bcwIdx (896)

However, in the "Motion vector storing process for geometric partitioning mode", HpelIfIdx is not written:

MvL0[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y ] = mvL0 (1045)

MvL1[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y ] = mvL1 (1046)

MvDmvrL0[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y ] = mvL0 (1047)

MvDmvrL1[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y ] = mvL1 (1048)

RefIdxL0[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y ] = refIdxL0 (1049)

RefIdxL1[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y ] = refIdxL1 (1050)

PredFlagL0[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y ] = predFlagL0 (1051)

PredFlagL1[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y ] = predFlagL1 (1052)

BcwIdx[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y ] = 0 (1053)

As far as I can understand, I think it is possible for these unspecified HpelIfIdx values to affect the decoding process?

bbross: Good catch, thanks! HpekIfIdx should always be written since it might be accessed later so it is definitely missing in "Motion vector storing process for geometric partitioning mode" and shall be added for v4.

- Spec ticket #1632: Incorrect indexing used for choosing matrix intra sample prediction

peterderivaz: Clause 8.4.5.2.1 reads:

Inputs to this process are:

– a sample location ( xTbCmp, yTbCmp ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture,

...

if MipChromaDirectFlag[ xTbCmp ][ yTbCmp ] is equal to 1 and cIdx is not equal to 0,

This looks up values in MipChromaDirectFlag based on counts of chroma samples.

However, when MipChromaDirectFlag is derived, the indexing is done based on counts of luma samples:

Derivation process for chroma intra prediction mode

Input to this process are:

– a luma location ( xCb, yCb ) specifying the top-left sample of the current chroma coding block

relative to the top-left luma sample of the current picture,

...

In this process, the chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] and the MIP chroma direct mode flag

MipChromaDirectFlag[ xCb ][ yCb ] are derived.

I think this can be fixed by changing 8.4.5.2.1 to use "MipChromaDirectFlag[ xTbCmp * SubWidthC ][ yTbCmp * SubHeightC ]" instead of "MipChromaDirectFlag[ xTbCmp ][ yTbCmp ]".

bbross: Good catch, thanks! This was indeed an oversight and should take the chroma subsampling into account as suggested. Will be fixed in v4.

- Spec ticket #1634: Matrices QStateTransTable, levelScale, AlfFixFiltCoeff, AlfClassToFiltMap are incorrectly transposed

peterderivaz: The spec defines the ordering of indices in a matrix and initialization syntax in 5.10.

However, the matrices QStateTransTable (7.4.12.11), levelScale (in 8.7.3), AlfFixFiltCoeff (7.4.3.18), AlfClassToFiltMap (7.4.3.18) do not follow this ordering, but instead use C based ordering (i.e. they are transposed).

For example,

The array QStateTransTable[ ][ ] is specified as follows:

QStateTransTable[ ][ ] = { { 0, 2 }, { 2, 0 }, { 1, 3 }, { 3, 1 } }

This could be fixed either by transposing the matrices in the spec, i.e.

QStateTransTable = { {0, 2, 1, 3}, {2, 0, 3, 1} }

or by swapping the indices when it is accessed, i.e. changing things like
QState = QStateTransTable[ QState ][ AbsLevel[ xC ][ yC ] & 1 ]
to
QState = QStateTransTable[ AbsLevel[ xC ][ yC ] & 1 ][ QState ]

bbross: Good catch, thanks! Will be fixed in v4 and I prefer transposing the matrices when defined.

GJS: Since many aspects of our pseudocode and syntax specification follow C conventions, if we are going to use a different convention for multi-dimensional array initialization (described in 5.10), we should add a NOTE to point out the difference to readers of the standard.

- Spec ticket #1635: Incorrect inference for tu_y_coded_flag

peterderivaz: tu_y_coded_flag is used in clause 8.8.3.5 to choose the boundary filtering strength used in the deblocking filter.

When tu_y_coded_flag is not present, the inference rules in clause 7.4.12.10 say:

When tu_y_coded_flag[ x0 ][ y0 ] is not present and treeType is not equal to DUAL_TREE_CHROMA, its value is inferred as follows:

– If cu_sbt_flag is equal to 1 and one of the following conditions is true, tu_y_coded_flag[ x0 ][ y0 ] is inferred to be equal to 0:

– subTuIndex is equal to 0 and cu_sbt_pos_flag is equal to 1;

– subTuIndex is equal to 1 and cu_sbt_pos_flag is equal to 0.

– Otherwise, tu_y_coded_flag[ x0 ][ y0 ] is inferred to be equal to 1.

I think this is missing some cases that should infer 0.

For example, if cu_coded_flag is equal to 0, then no transform_tree structure will be present, and tu_y_coded_flag will be inferred equal to 1 according to the spec. However, I believe the deblocking in the VTM works as if tu_y_coded_flag was inferred equal to 0 in this case.

bbross: Thanks for bringing that up. This should be fixed in v4 but I would like to make sure that this is the only missing case when inferring tu_y_coded_flag to be equal to 0.

- Spec ticket #1644: Correction of wrong reference on VVC Annex C.4 regarding DpbOutputInterval constraint

jhlim: DpbOutputInterval constraint in the following text seems to be referencing the wrong part.

- Related subclause: VVC Annex C.4 Bitstream conformance

"9. The value of DpbOutputInterval[ n ] as given by Formula (1605), which is the difference between the output times of a picture and the first picture following it in output order and having PictureOutputFlag equal to 1, shall satisfy the constraint expressed in subclause A.4.1 for the profile, tier and level specified in the bitstream using the decoding process specified in Clauses 2 through 9."

The actual constraint applied on DpbOutputInterval can be found in A.4.2 not in A.4.1. as described in following:

"b) The difference between consecutive output times of pictures of different AUs from the DPB, as specified in subclause C.3.3, shall satisfy the constraint that DpbOutputInterval[ n ] is greater than or equal to Max( AuSizeMaxInSamplesY[ n ] / MaxLumaSr, FrVal ), where MaxLumaSr is the value specified in Table A.3 for AU n, provided that AU n has a picture that is output and AU n is not the last AU of the bitstream that has a picture that is output."

The same issue exists in the HEVC spec text also.

yk: Good catch, thanks! Added to the errata report in JVET-AI1004 (and to be fixed later in VVCv4 and HEVCv11).

- In a few places, the text uses “Y = !X”, with the understanding that when X is 0, Y will be 1. But it is not directly stated anywhere that !0 will be 1. The text only seems to clearly say that !0 will be TRUE, and TRUE is anything that is not equal to 0. For further detail, see the discussion of the same issue in the section about HEVC.

- There are several places in the text with “1/4”, “1/16”, or “1/32” used somewhat informally. According to the formal definition in 5.2, these are all equivalent to the value 0; however, the meaning is probably generally understood in context.

- In D.4.2, “max_val_initial_removal_delay_for_splicing” should be “bp_max_initial_removal_delay_for_concatenation” (reported by Lulin Chen, 2024-10-04).

## Errata items for VVC conformance

None.

# Reported errata items for VSEI

## General

The ITU-T third edition text has been published. As of 2024-07-08, the ISO/IEC third edition text was targeted to be published on 2025-01-30. Besides those errata items directly included below in this section, JVET-AH2006 may include some VSEI errata changes (in addition to some amendments).

## Publication status background

Rec. ITU-T H.274 (https://www.itu.int/rec/T-REC-H.274/en)

- (09/2023, version 3) Approved 2023-09-29, published 2024-03-12

Rec. ITU-T H.274 (https://www.itu.int/rec/T-REC-H.274/en)

- (05/2022, version 2) Approved 2022-05-22, published 2022-07-25

ISO/IEC 23002-7 (https://www.iso.org/standard/83530.html)

- ISO/IEC 23002-7:2022 (Edition 2); published 2022-10

Rec. ITU-T H.274 (https://www.itu.int/rec/T-REC-H.274/en)

- (08/2020, version 1) Approved 2020-08-29, published 2020-11-10

ISO/IEC 23002-7 (https://www.iso.org/standard/79112.html)

- ISO/IEC 23002-7:2021 (Edition 1); published 2021-01

## Errata items for VSEI

- In subclause 7.3, semantics of vui_chroma_sample_loc_type_frame, vui_chroma_sample_loc_type_top_field, and vui_chroma_sample_loc_type_bottom_field, replace "the syntax elements chroma_sample_loc_type," with "the syntax elements vui_chroma_sample_loc_type_frame,". (This problem is present in both the ITU version 3 text and the ISO/IEC edition 2 text.)

- In subclause 8.22.2 (Depth representation information SEI message semantics) and 8.23.2 (Alpha channel information SEI message semantics), replace each of the 10 instances of "nuh_layer_id" with "layer identification".

- In subclause 8.26.2, replace the following:

–	A picture in the current layer in an AU associated with a CTI SEI message is output that follows the current picture in output order.

with the following:

–	A picture in the current layer in an AU associated with a CTI SEI message with the same value of colour_transform_id and applicable to the current layer is output that follows the current picture in output order.

- In subclause 8.28.2.1, change the coding of nnpfc_alignment_zero_bit_a and nnpfc_alignment_zero_bit_b from u(1) to f(1). It would be better to consistently use f(1) for all fix-valued, non-reserved bits. The same approach could apply for all fix-valued, non-reserved bytes, two-bytes, etc.

- In subclause 8.28.2.2, semantics of nnpfc_matrix_coeffs, make the following changes:

When nnpfc_matrix_coeffs is not present in the NNPFC SEI message, the value of nnpfc_matrix_coeffs is inferred to be equal to vui_matrix_coeffs.

nnpfc_matrix_coeffs shall not be equal to 0 unless both of the following conditions are true:

–	Both nnpfc_out_tensor_chroma_bitdepth_minus8 and nnpfc_out_tensor_luma_bitdepth_minus8 are present, and nnpfc_out_tensor_chroma_bitdepth_minus8 is equal to nnpfc_out_tensor_luma_bitdepth_minus8.

–	Both nnpfc_out_tensor_chroma_bitdepth_minus8 and nnpfc_out_tensor_luma_bitdepth_minus8 are present, and nnpfc_out_order_idc is equal to 2, outSubHeightC is equal to 1, and outSubWidthC is equal to 1.

NOTE: When both of the above two conditions are true and vui_matrix_coeffs is equal to 0, nnpfc_matrix_coeffs is expected to be present.

nnpfc_matrix_coeffs shall not be equal to 8 unless one of the following conditions is true:

–	Both nnpfc_out_tensor_chroma_bitdepth_minus8 and nnpfc_out_tensor_luma_bitdepth_minus8 are present; and nnpfc_out_tensor_chroma_bitdepth_minus8 is equal to nnpfc_out_tensor_luma_bitdepth_minus8.

–	Both nnpfc_out_tensor_chroma_bitdepth_minus8 and nnpfc_out_tensor_luma_bitdepth_minus8 are present; and nnpfc_out_tensor_chroma_bitdepth_minus8 is equal to nnpfc_out_tensor_luma_bitdepth_minus8 + 1, nnpfc_out_order_idc is equal to 2, outSubHeightC is equal to 1, and outSubWidthC is equal to 1.

NOTE: When one of the above two conditions is true and vui_matrix_coeffs is equal to 8, nnpfc_matrix_coeffs is expected to be present.

- In subclause 8.28.2.2, semantics of nnpfc_full_range_flag, make the following changes:

nnpfc_full_range_flag indicates the scaling and offset values applied in association with the matrix coefficients as specified by nnpfc_matrix_coeffs. Its semantics are as specified for the VideoFullRangeFlag parameter in Rec. ITU-T H.273 | ISO/IEC 23091-2. When nnpfc_full_range_flag is not present and nnpfc_out_format_idc is equal to 1, the value of nnpfc_full_range_flag is inferred to be equal to vui_full_range_flag 0.

- Consider adding a NOTE in VSEI telling something like: 1) AVC can reference VSEI for enabling the inclusion of only those SEI messages for which the SEI payloads are specified in VSEI but the syntax has not been extended; 2) HEVC and VVC can reference VSEI for enabling the inclusion of SEI messages for which the SEI payloads are specified in VSEI regardless of whether the syntax has been extended or not.

# Reported errata items for HEVC

## General

Besides those errata items directly included below in this section, JVET-AH1006 also includes some HEVC errata changes (in addition to some amendments).

## Publication status background

Rec. ITU-T H.265

- (07/2024, version 10) Approved 2024-07-29, pre-published 2024-08-06

- (09/2023, version 9) Approved 2023-09-13, published 2023-11-24

- (08/2021, version 8) Approved 2021-08-22, published 2021-10-13

- (10/2019, version 7) Approved 2019-11-29, published 2020-01-10

- (06/2019, version 6) Approved 2019-06-29, published 2019-09-23

- (02/2018, version 5) Approved 2018-02-13, published 2018-05-11

ISO/IEC 23008-2

- ISO/IEC 23008-2:2023 (Edition 5); published 2023-10

- ISO/IEC 23008-2:2020 (Edition 4); published 2020-08

- ISO/IEC 23008-2:2017 (Edition 3); published 2017-10

- ISO/IEC 23008-2:2017/Amd 1:2018 (Additional colour representation code point); published 2018-03

- ISO/IEC 23008-2:2017/Amd 2:2018 (Main 10 still picture profile); published 2018-03

- ISO/IEC 23008-2:2017/Amd 3:2018 (Additional supplemental enhancement information); published 2018-07

- Work items merged for FDIS ballot as publication office action; Stage 50.00 Final text received or FDIS registered for formal approval as of 2020-03-19

  - ISO/IEC DIS 23008-2:201x (Edition 4); DIS ballot closed 2018-10-06; stage 40.99, Full report circulated: DIS approved for registration as FDIS 2019-02-18; Pending FDIS ballot based on WG 11 N 18277

  - ISO/IEC 23008-2:201x (Edition 4)/DAmd 1:201x (Additional supplemental enhancement information); DAM ballot started 2019-07-10, closed 2019-10-02

## Errata items for HEVC

### General

- The FDIS draft of the ISO/IEC sixth edition says “This is the fifth version of this document and the fourth edition published by ISO/IEC.”

- In clause 9.3.1, make the following changes:

When cabac_bypass_alignment_enabled_flag is equal to 1, the request for a value of a syntax element is for either the syntax elements coeff_abs_level_remaining[ ] or coeff_sign_flag[ ], and escapeDataPresent is equal to 1, the alignment process prior to aligned bypass decoding as specified in clause 9.3.4.3.6 is invoked.

- Spec ticket #1644: Correction of wrong reference on VVC Annex C.4 regarding DpbOutputInterval constraint

jhlim: DpbOutputInterval constraint in the following text seems to be referencing the wrong part.

- Related subclause: VVC Annex C.4 Bitstream conformance

"9. The value of DpbOutputInterval[ n ] as given by Formula (1605), which is the difference between the output times of a picture and the first picture following it in output order and having PictureOutputFlag equal to 1, shall satisfy the constraint expressed in subclause A.4.1 for the profile, tier and level specified in the bitstream using the decoding process specified in Clauses 2 through 9."

The actual constraint applied on DpbOutputInterval can be found in A.4.2 not in A.4.1. as described in following:

"b) The difference between consecutive output times of pictures of different AUs from the DPB, as specified in subclause C.3.3, shall satisfy the constraint that DpbOutputInterval[ n ] is greater than or equal to Max( AuSizeMaxInSamplesY[ n ] / MaxLumaSr, FrVal ), where MaxLumaSr is the value specified in Table A.3 for AU n, provided that AU n has a picture that is output and AU n is not the last AU of the bitstream that has a picture that is output."

The same issue exists in the HEVC spec text also.

yk: Good catch, thanks! Added to the errata report in JVET-AI1004 (and to be fixed later in VVCv4 and HEVCv11).

- (Also for VVC and perhaps for other specs such as AVC as well; this problem is not evident in VSEI) The text uses “Y = !X”, with the understanding that when X is 0, Y will be 1. But it is not directly stated anywhere that !0 will be 1. The text only seems to clearly say that !0 will be TRUE, and TRUE is anything that is not equal to 0.

The fundamental problem here is that the result of !X is a Boolean value, not a number. We should not be trying to interpret it as a number. If we want our input to be a number and our output to be a number, we should simply replace the uses of Y = !X with an operation that produces a numerical result instead of a Boolean result. Instead of Y = !X, we could use Y = 1 − X. We should also never interpret a number (or at least a number other than 0 or 1, since those are the values of flag syntax elements) as a Boolean value. If X is a number other than 0 or 1, we should never use it as an input to “!” or “&&” or “| |” (since those are logical operations). A clearly numerical value should only be used as an input to mathematical operators or value comparison operators (i.e., “>”, “<”, “= =”, “>=” or “<=”). If n is a numerical variable, we should never use “!n” or “n && m”. Instead we should use expressions like “n  !=  0” to convert the number to a Boolean first. In principle we should be able to change “if x is TRUE or not equal to 0” simply with “if x is TRUE” and it would not affect anything.

Moreover, in these uses, the X and Y are a letter that is part of a variable name, not a variable themselves, which is quite irregular.

- In A.3.6, which specifies the high throughput profiles, there is a sentence that says “However, decoders conforming to the format range extensions profiles shall allow other combinations as specified below in this clause to occur in the bitstream.” In this sentence, “format range extensions” should be “high throughput”.

- In A.3.8, which specifies the high throughput screen content coding exensions profiles, there is a sentence that says “However, decoders conforming to the screen content coding extensions profiles shall allow other combinations as specified below in this clause to occur in the bitstream.” In this sentence, “screen content coding extensions” should be “high throughput screen content coding extensions”.

- In F.11.2  (based on the pre-published H.265v10 edition), there are sentences intended to require decoders to support “subset” bitstreams (including possible future profile specifications) in a manner similar to statements in A.3.5 for the format range extension profiles, A.3.6 for the high throughput profiles, A.3.7 for the screen content coding profiles, and A.3.8 for the high throughput screen content coding profiles. An example of a future “subset” profile would be a Scalable Monochrome 10 profile (as there is currently no such profile, although this would be a subset of the current Scalable Monochrome 12 and Scalable Monochrome 16 profiles). However, the following issues are noted.

  - In two places, F.11.2 clearly incorrectly refers to constraint flag values being “less than or equal to” the specified values instead of being “greater than or equal to” the specified values.

  - In the above-identified places in Annex A, there is a clear statement that “Decoders conforming to a … profile … shall be capable of decoding all bitstreams and sub-layer representations for which … the value of each constraint flag listed in Table A.[X] is greater than or equal to the value(s) specified in the row of Table A.[X]”. There is no similar explicit statement in F.11.2. Instead, the similar decoder requirement is only explicitly expressed as a way of constructing a list of profiles and in a column heading of Table F.3 (“Profiles that the decoder shall support”). Taken too literally, future “subset” profiles are not in the current lists and could potentially be interpreted as not supported by a decoder.

  - In the above-identified places in Annex A, there is a requirement that “decoders conforming to the [format range extensions | screen content coding extensions] profiles shall allow other combinations as specified below in this clause to occur in the bitstream”. Somewhat similar statements using the phrase “other combinations as specified below in this clause” are found in G.11.1.2 for multiview format range extensions profiles and in H.11.1.2 for scalable format range extensions profiles. However:

    - These statements in G.11.1.2 and H.11.1.2 are obviously incorrect when referring to “below in this clause”, because in each case they are the last sentence in their subclause. Instead, they are intending to refer to the statements described above in the previous subitem b, which are above in F.11.2, not below in annexes G and H. (The statements that are referred to were probably originally located below in the same subclause that refers to them and were later moved into F.11.2 without remembering to update the description of where to find them.)

    - The phrasing of the statements in F.11.2 that these are intending to refer to do not seem as clear as those in Annex A in regard to requiring support for currently reserved combinations for future profile specifications.

  - Statements are found in F.11.2 to express compatibility requirements relating to “format range extensions” profiles and “scalable format range extensions” profiles, but no similar statement is provided there for (the new) “multiview format range extensions” profiles.

- In G.11.1.1 (based on the pre-published H.265v10 edition):

  - There is an improper space before “sub_layer_max_12bit_constraint_flag[ OpTid ]” at the beginning of a line of text in three places.

  - Regarding the sentences “For a layer in an output operation point associated with an OLS in a bitstream, the layer conforming to the Multiview Main profile, the following applies: Let olsIdx be the OLS index of the OLS, the sub-bitstream subBitstream and the base layer sub-bitstream baseBitstream are derived as specified in clause ‎F.11.3.”.

    - The phrasing of both sentences seems awkward.

    - The first sentence of that should include the Multiview Extended and Multiview Extended 10 profiles. If those are not included, subBitstream and baseBitstream do not appear to be defined.

  - Each set of conditions on baseBitstream and subBitstream should be prefixed with expressed conditions about conformance to specific profiles.

  - It would be helpful if there was some verbal explanation of the difference between the Multiview Main and Multiview Extended profiles.

  - We note that “multiview format range extensions” profiles and “scalable format range extensions” profiles are not considered “format range extensions” profiles. This seems potentially confusing to readers, and perhaps should be addressed in NOTEs, as was done in A.3.6 and A.3.7 for the “high throughput screen content coding extensions” profiles (which are not considered “high throughput” profiles and are not considered “screen content coding extensions” profiles). Alternatively, these profiles could be renamed as the “multiview monochrome” profiles, since they are all monochrome profiles and the other new profiles were put into a different subclause.

  - (This item is somewhat redundant with other items reported above.) It may have been thought at the time that it would be hypothetically possible to re-insert the previously drafted “Multiview Main 10” profile later, such that its bitstreams would be decodable by the Multiview Extended 10 profile decoders, since the bitstream characteristics of the previously proposed “Multiview Main 10” profile are a subset of the capabilities of the Multiview Extended 10 profile. However, because the CompatibleProfileList in Table F.3 is based on the names of profiles rather than the values of general_profile_idc and constraint flags, and because the statement about decoders being required to decode bitstreams with other values for the constraint flags, this is not the case. This was likely unintentional. When defining the Multiview Extended and Multiview Extended 10 profiles, the Multiview Extended profile was assigned the general_profile_idc value of 13 (instead of 6), and it was agreed to omit the specification of a “Multiview Main 10” profile with general_profile_idc equal to 6, general_max_10bit_constraint_flag equal to 1, and general_max_8bit_constraint_flag equal to 0. Thus, the combination that had been planned as the “Multiview Main 10” profile cannot be re-added with general_profile_idc equal to 6. The closest we could come in the current situation would be to give such a profile the general_profile_idc value of 13, which could be confusing and would not match the previous intent.

- In G.11.1.2 (based on the pre-published H.265v10 edition):

  - The value 10 for general_profile_idc that is used for the (new) multiview format range extensions profiles is the same as the value used for scalable format range extensions profiles in H.11.1.2. It does not appear that a decoder can distinguish these profiles from each other.

- (Reported by Alexis M. Tourapis) The content of subclause I.11.2 (based on the pre-published H.265v10 edition) is as follows:

The specification in sub-clause ‎G.11.2 and its sub-clauses apply with the following modifications:

"Multiview Main profile" is replaced by "3D Main profile"

However, due to changes made to subclause ‎G.11.2, the above needs to be adapted accordingly.

### Items on the semantics of post-filter hint SEI message from JVET-AE0155

- It does not seem clear whether the “16-bit precision” description applies only to the cross-correlation matrix case or also to the filter coefficient cases.

- A specified 32-bit range may be excessive for a syntax element that has 16-bit precision.

- Much of the text is written from the encoder perspective. We generally do not specify things from the encoder perspective. The decoder-side processing is not clearly described or is discussed only vaguely. For example, no equations are given for the decoder’s filtering process.

- It does not seem clear whether the decoder should divide the filter tap values (or equivalently, the result of the filtering process) by the sum of the filter tap values or if the encoder is responsible for the filter normalization.

- There are issues relating to how to interpret the filter depending on whether the filter length is even (in which case the output samples may presumably be located spatially between input samples) or odd (in which case the location of an output sample may presumably have a location corresponding to the centre tap).

- The handling of the boundaries of the picture and cropping window are not described and seems unclear.

- It seems unclear why should it matter whether a horizontal filter or a vertical filter is applied first if both are to be applied. Perhaps integer rounding is expected to be applied between the two filtering operations, but this is not stated.

- The discussion of Wiener post-filtering is a bit vague (e.g., what exactly is a Wiener filter, and how would a decoder derive one from the provided data?).

- A decoder could use a subset of a provided cross-correlation matrix to derive a Wiener post-filter with a smaller region of support than the provided statistics would enable. It does not seem clear whether this is intended to be an acceptable or possible practice.

# Reported errata items for AVC

## General

See Subsection 5.3 below. JVET-AH1016 also includes some AVC errata changes (in addition to some amendments).

## Publication status background

Rec. ITU-T H.264

- (08/2024, version 15) Approved 2024-08-13, pre-published 2024-08-29

- (08/2021, version 14) Approved 2021-08-22, published 2021-10-13

- (06/2019, version 13) Approved 2019-06-13, published 2019-09-06

ISO/IEC 14496-10

- ISO/IEC 14496-10:2022 (Edition 10), published 2022-11 (https://www.iso.org/standard/83529.html)

- ISO/IEC 14496-10:2020 (Edition 9), published 2020-12

- ISO/IEC 14496-10:2014 (Edition 8), published 2014-09

- ISO/IEC 14496-10:2014/Amd 1:2015 (Multi-resolution frame compatible stereoscopic video with depth maps, additional supplemental enhancement information and video usability information), published 2015-11

- ISO/IEC 14496-10:2014/FDAMD 2 (Additional Levels and Supplemental Enhancement Information); stage 50.98, deleted in preparation for Edition 9

- ISO/IEC 14496-10:2014/Amd 3:2016 (Additional supplemental enhancement information); published 2016-12, published 2016-12

- ISO/IEC DIS 14496-10:201x (Edition 9); stage 40.99 Full report circulated: DIS approved for registration as FDIS as of 2018-01-31

## Errata items

- All items in Subsection 4.3.2 (Items on the semantics of post-filter hint SEI message from JVET-AE0155).

- Minor matters reported for other standards (e.g., the “Y = !X” issue discussed above) should also be checked for AVC.

# Reported errata items for Video CICP

## General

See also section 1.

## Publication status background

Rec. ITU-T H.273

- (07/2024, version 4) Approved 2024-07-14, pre-published 2024-08-06, published 2024-10-07

- (09/2023, version 3) Approved 2023-09-29, pre-published 2023-10-11, never published

- (07/2021, version 2) Approved 2021-07-14, published 2021-09-24

- (12/2016, version 1) Approved 2016-12-22, published 2017-04-27

ISO/IEC 23091-2 (previously part of ISO/IEC 23001-8)

- ISO/IEC 23091-2:2021 (edition 3), FDIS issued 2024-04

- ISO/IEC 23091-2:2021 (edition 2), published 2021-10

- ISO/IEC 23091-2:2019 (edition 1), published 2019-07

## Errata items

- None noted (previously included items were addressed in the 2nd editions).