| Joint Video Experts Team (JVET) of ITU-T SG21 WP3/21 and ISO/IEC JTC 1/SC 29 38th Meeting, by teleconference, 26 March – 4 April 2025 | Document: JVET-AL_notes_d1 |
| --- | --- |

| Title: | Meeting Report of the 38th Meeting of the Joint Video Experts Team (JVET), by teleconference, 26 March – 4 April 2025 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from the chair of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen Melatener Straße 23 D-52074 Aachen | Tel: Email: | +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chair of JVET |  |  |

_____________________________

# Summary

The Joint Video Experts Team (JVET) of ITU-T WP3/21 and ISO/IEC JTC 1/‌SC 29 held its thirty-eighth meeting during 26 March – 4 April 2025 meeting as an online-only meeting.

For ISO/IEC purposes, JVET is alternatively designated ISO/IEC JTC 1/‌SC 29/‌WG 5, and this was the nineteenth meeting as WG 5. The JVET meeting was held under the chairmanship of Dr Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 2.16 of this document. It is further noted that work items which had originally been conducted by the Joint Collaborative Team on Video Coding (JCT-VC) were continued in JVET as a single joint team, and explorations towards possible future need of standardization in the area of video coding are also conducted by JVET, as negotiated by the parent bodies.

The JVET meeting began at approximately 0500 UTC on Wednesday 26 March 2025. Meeting sessions were held on all days except the weekend days of Saturday and Sunday 29 and 30 March 2025, until the meeting was closed at approximately XXX hours UTC on Friday 4 April 2025. Approximately XXX people attended the JVET meeting, and approximately XXX input documents (not counting crosschecks, reports, and summary documents), 18 AHG reports, 2 EE summary reports, XX BoG report(s), and XX incoming liaison document(s) were discussed. The meeting took place in coordination with a meeting of various SC29 Working Groups and Advisory Groups – where WG 5 is representing the Joint Video Coding Team(s) and their activities from the perspective of the SC 29 parent body, under whose auspices this JVET meeting was held. The subject matter of the JVET meeting activities consisted of work on further development and maintenance of the twin-text video coding technology standards Advanced Video Coding (AVC), High Efficiency Video Coding (HEVC), Versatile Video Coding (VVC), Coding-independent Code Points (Video) (CICP), and Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (VSEI), as well as related technical reports, reference software and conformance testing packages. Further important goals were reviewing the results of the Exploration Experiment (EE) on Neural Network-based Video Coding, of the EE on Enhanced Compression beyond VVC capability, of other technical input on novel aspects of video coding technology, and to plan next steps for investigation of candidate technology towards further standard development.

As a primary goal, the JVET meeting reviewed the work that had been performed in the interim period since the thirty-seventh JVET meeting in producing the following documents:

a) JVET documents

- JVET-AK1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP

- JVET-AK1006 HEVC additional profiles and SEI messages, also issued as WG 5 CDAM N 336

- JVET-AK1017 Support for additional VSEI messages in AVC (draft 1)

- JVET-AK2005 Additions and corrections for VVC version 4 (Draft 11), also issued as WG 5 preliminary DAM update N 337

- JVET-AK2006 Additional SEI messages for VSEI version 4 (Draft 5), also issued as WG 5 preliminary DAM update N 335

- JVET-AK2010 VTM and HM common test conditions and software reference configurations for SDR 4:2:0 10 bit video

- JVET-AK2019 Description of algorithms version 10 and software version 12 in neural network-based video coding (NNVC)

- JVET-AK2020 Film grain synthesis technology for video applications ed. 2 (Draft 2)

- JVET-AK2023 Exploration experiment on neural network-based video coding (EE1)

- JVET-AK2024 Exploration experiment on enhanced compression beyond VVC capability (EE2)

- JVET-AK2025 Algorithm description of Enhanced Compression Model 16 (ECM 16)

- JVET-AK2026 Testing of video coding technology beyond conditions of exploration experiments

- JVET-AK2030 Optimization of encoders and receiving systems for machine analysis of coded video content (draft 8)

- JVET-AK2032 Technologies under consideration for future extensions of VSEI (version 7)

- JVET-AK2038 Draft white paper on VSEI

- JVET-AK2039 Announcement of JVET AHG17 Meeting, Aachen (Germany), 10-12 March 2025

b) documents produced as WG 5 documents only:

WG 5 N 338 Request for ISO/IEC 23090-15 Conformance testing for VVC (3rd ed.)

WG 5 N 340 Request for ISO/IEC 14496-10:202x (11th ed.) Amd.1 Support for additional SEI messages

c) document forwarded by JVET and Q6/21 for ITU-T agreement:

SG21 TD44R1/Plen H-Series Supplement 21 (ex. H.Sup-FGST) Film grain synthesis technology for video applications

As main results, the JVET produced XX output documents from the current meeting (update):

- JVET-AK1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP

- JVET-AK1006 HEVC additional profiles and SEI messages, also issued as WG 5 CDAM N 336

- JVET-AK1017 Support for additional VSEI messages in AVC (draft 1)

- JVET-AK2005 Additions and corrections for VVC version 4 (Draft 11), also issued as WG 5 preliminary DAM update N 337

- JVET-AK2006 Additional SEI messages for VSEI version 4 (Draft 5), also issued as WG 5 preliminary DAM update N 335

- JVET-AK2010 VTM and HM common test conditions and software reference configurations for SDR 4:2:0 10 bit video

- JVET-AK2019 Description of algorithms version 10 and software version 12 in neural network-based video coding (NNVC)

- JVET-AK2020 Film grain synthesis technology for video applications ed. 2 (Draft 2)

- JVET-AK2023 Exploration experiment on neural network-based video coding (EE1)

- JVET-AK2024 Exploration experiment on enhanced compression beyond VVC capability (EE2)

- JVET-AK2025 Algorithm description of Enhanced Compression Model 16 (ECM 16)

- JVET-AK2026 Testing of video coding technology beyond conditions of exploration experiments

- JVET-AK2030 Optimization of encoders and receiving systems for machine analysis of coded video content (draft 8)

- JVET-AK2032 Technologies under consideration for future extensions of VSEI (version 7)

- JVET-AK2038 Draft white paper on VSEI

- JVET-AK2039 Announcement of JVET AHG17 Meeting, Aachen (Germany), 10-12 March 2025

- The following X documents were produced as WG 5 documents only, without a corresponding JVET output document or direct repetition of their content in this meeting report:

WG 5 N 338 Request for ISO/IEC 23090-15 Conformance testing for VVC (3rd ed.)

WG 5 N 340 Request for ISO/IEC 14496-10:202x (11th ed.) Amd.1 Support for additional SEI messages

For the organization and planning of its future work, the JVET established XX “ad hoc groups” (AHGs) to progress the work on particular subject areas. At this meeting, X Exploration Experiments (EE) were defined. The next eight JVET meetings were planned for 26 June – 4 July 2025 under ISO/IEC JTC 1/‌SC 29 auspices in Daejeon, KR; during 3 – 12 October 2025 under ITU-T SG21 auspices in Geneva, CH; during 14 – 23 January 2026 under ISO/IEC JTC 1/‌SC 29 auspices, to be conducted as teleconference meeting; during 24 April – 1 May 2026 under ISO/IEC JTC 1/‌SC 29 auspices in Santa Eulària, ES; during 7 – 15 July 2026 under ITU-T SG21 auspices in Geneva, CH; during 14 – 23 October 2026 under ISO/IEC JTC 1/‌SC 29 auspices in Hangzhou, CN; during January 2027 under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.; and during April 2027 under ITU-T SG21 auspices, date and location t.b.d.

The document distribution site https://jvet-experts.org/ was used for distribution of all documents. It was noted that the previous sites http://phenix.int-evry.fr/jvet/, http://phenix.int-evry.fr/jct/, and http://phenix.int-evry.fr/jct3v/ were shut down, but the most recent versions of JCT-VC and JCT-3V documents can now be accessed directly via the JVET site, whereas all uploaded versions are also available from http://wftp3.itu.int/av-arch/jctvc-site/ and http://wftp3.itu.int/av-arch/jct3v-site/, respectively.

The reflector to be used for discussions by the JVET and all its AHGs is the JVET reflector:
jvet@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see https://lists.rwth-aachen.de/postorius/lists/jvet.lists.rwth-aachen.de/.

# Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Video Experts Team (JVET) is a group of video coding experts from the ITU-T Study Group 21 Visual Coding Experts Group (VCEG) and ISO/IEC JTC 1/‌SC 29/‌WG 5. The parent bodies of the JVET are ITU-T WP3/21 and ISO/IEC JTC 1/‌SC 29.

The Joint Video Experts Team (JVET) of ITU-T WP3/21 and ISO/IEC JTC 1/‌SC 29 held its thirty-eighth meeting during 26 March – 4 April 2025 meeting as an online-only meeting.

It is further noted that the unabbreviated name of JVET was formerly known as “Joint Video Exploration Team”, but the parent bodies modified it when entering the phase of formal development of the Versatile Video Coding (VVC) and Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (VSEI) standards, as well as associated conformance test sets, reference software, verification testing, and non-normative guidance information. Furthermore, starting from the twentieth meeting, work items which had originally been conducted by the Joint Collaborative Team on Video Coding (JCT-VC) were continued to be conducted in JVET as a single joint team, as negotiated by the parent bodies. This particularly consists of work on:

- High Efficiency Video Coding (HEVC) and its extensions, the development of associated conformance test sets, reference software, verification testing, and non-normative guidance information,

- Specification of Coding-independent Code Points (Video) (CICP), and associated technical report(s),

- Maintenance and enhancement work on the Advanced Video Coding (AVC) standard, associated conformance test sets and reference software.

Furthermore, explorations towards possible future need of standardization in the area of video coding are also conducted by JVET. Currently, the following topics are under investigation:

- Exploration on Neural Network-based Video Coding

- Exploration on Enhanced Compression beyond VVC capability

As requested by the parent bodies, JVET is currently also preparing a call for evidence on video compression with capability beyond existing standards.

This report contains three important annexes, as follows:

- Annex A contains a list of the documents of the JVET meeting

- Annex B contains a list of the meeting participants, as recorded by the teleconferencing tool used for the meeting

- Annex C contains the meeting recommendations of ISO/IEC JTC 1/‌SC 29/‌WG 5 for purposes of results reporting to ISO/IEC.

## Meeting logistics

Information regarding logistics arrangements for the meeting had been provided via the email reflector jvet@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jvet-site/2025_03_AL_Virtual/.

## Primary goals

As a primary goal, the JVET meeting reviewed the work that was performed in the interim period since the thirty-seventh JVET meeting in producing the following output documents:

a) JVET documents

- JVET-AK1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP

- JVET-AK1006 HEVC additional profiles and SEI messages, also issued as WG 5 CDAM N 336

- JVET-AK1017 Support for additional VSEI messages in AVC (draft 1)

- JVET-AK2005 Additions and corrections for VVC version 4 (Draft 11), also issued as WG 5 preliminary DAM update N 337

- JVET-AK2006 Additional SEI messages for VSEI version 4 (Draft 5), also issued as WG 5 preliminary DAM update N 335

- JVET-AK2010 VTM and HM common test conditions and software reference configurations for SDR 4:2:0 10 bit video

- JVET-AK2019 Description of algorithms version 10 and software version 12 in neural network-based video coding (NNVC)

- JVET-AK2020 Film grain synthesis technology for video applications ed. 2 (Draft 2)

- JVET-AK2023 Exploration experiment on neural network-based video coding (EE1)

- JVET-AK2024 Exploration experiment on enhanced compression beyond VVC capability (EE2)

- JVET-AK2025 Algorithm description of Enhanced Compression Model 16 (ECM 16)

- JVET-AK2026 Testing of video coding technology beyond conditions of exploration experiments

- JVET-AK2030 Optimization of encoders and receiving systems for machine analysis of coded video content (draft 8)

- JVET-AK2032 Technologies under consideration for future extensions of VSEI (version 7)

- JVET-AK2038 Draft white paper on VSEI

- JVET-AK2039 Announcement of JVET AHG17 Meeting, Aachen (Germany), 10-12 March 2025

b) documents produced as WG 5 documents only:

WG 5 N 338 Request for ISO/IEC 23090-15 Conformance testing for VVC (3rd ed.)

WG 5 N 340 Request for ISO/IEC 14496-10:202x (11th ed.) Amd.1 Support for additional SEI messages

c) document forwarded by JVET and Q6/21 for ITU-T agreement:

SG21 TD44R1/Plen H-Series Supplement 21 (ex. H.Sup-FGST) Film grain synthesis technology for video applications

Further important goals were reviewing the results of the EE on Neural Network-based Video Coding, of the EE on Enhanced Compression beyond VVC capability, of other technical input on novel aspects of video coding technology, and planning next steps for investigation of candidate technology towards further standard development.

## Documents and document handling considerations

### General

The document distribution site https://jvet-experts.org/ was used for distribution of all documents.

Document registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting follow the CEST timezone (local time in Geneva), except as otherwise noted.

Highlighting of recorded decisions in this report is practised as follows:

- Decisions made by the group that might affect the normative content of a future standard are identified in this report by prefixing the description of the decision with the string “Decision:”.

- Decisions that affect one of the various software packages but have no normative effect on text are marked by the string “Decision (SW):”.

- Decisions that fix a “bug” in one of the test model descriptions such as VTM, HM, etc. (an error, oversight, or messiness) or in the associated software package are marked by the string “Decision (BF):”.

- Decisions that are merely editorial without effect on the technical content of a draft standard are marked by the string "Decision (Ed.):". Such editorial decisions are merely suggestions to the editor, who has the discretion to determine the final action taken if their judgment differs.

- Other parenthetical comments may be used for describing the impact or motivation of a decision. Some decisions are recorded with the word “agreed” rather than “Decision:”, especially for work items under study, non-normative, editorial and planning matters.

This meeting report is based primarily on notes taken by the JVET chair. The preliminary notes were also circulated publicly by ftp and http during the meeting on a daily basis. It should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much information about the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report. Expressions such as “X.XX%” indicate that the desired results were not available at the time the information was recorded.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Wednesday, 19 March 2025. Any documents uploaded after 1159 hours Paris/Geneva time on Thursday 20 March 2025 were considered “officially late”, with a grace period of 12 hours (to accommodate those living in different time zones of the world). The deadline does not apply to AHG reports and other such reports which can only be produced after the availability of other input documents.

All contribution documents with registration numbers higher than JVET-AL0239 were registered after the “officially late” deadline (and therefore were also uploaded late). However, some documents in the “late” range might include break-out activity reports that were generated during the meeting, or documents which were requested to be produced for the purpose of improving specification text, and are therefore better considered as report documents rather than as late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions has been to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This has been agreed to be a helpful approach to be followed at the meeting.

The following technical design proposal contributions were registered and/or uploaded late:

- JVET-AL0XXX (a proposal on …), uploaded 03-XX,

- … .

It may be observed that some of the above-listed contributions were submissions made in response to issues that arose in discussions during the meeting or from the study of other contributions, and thus could not have been submitted by the ordinary deadline.

The following other documents not proposing normative technical content, but with some need for consideration, were registered and/or uploaded late:

- JVET-AL0XXX (a study on …), uploaded 03-XX,

- … .

All cross-verification reports at this meeting were registered late, and/or uploaded late. In the interest of brevity, these are not specifically identified here. Initial upload times for each document are recorded in Annex A of this report.

The following contribution registrations were noted that were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JVET-AL0088, JVET-AL0089, JVET-AL0090, JVET-AL0146, JVET-AL0163, JVET-AL0170, JVET-AL0172, JVET-AL0180, JVET-AL0267, … .

The following cross-verification reports were still missing by the end of the meeting, but were uploaded later: JVET-AL0XXX, … . The following reports had not become available yet three weeks after the end of the meeting: JVET-AL0XXX, … . These were marked as withdrawn by the JVET chair, assuming the registration had become obsolete.

“Placeholder” contribution documents that were basically empty of content, or lacking any results showing benefit for the proposed technology, and obviously uploaded with an intent to provide a more complete submission as a revision, had been agreed to be considered unacceptable and to be rejected in the document management system until a more complete version was available (which would then be counted as a late contribution if the update was after the document deadline). At the current meeting, this situation did apply to documents JVET-AL0183, and JVET-AL0195 which were initially empty of results and were flagged as late in the list above, based on the time of the first reasonable document upload.

It was remarked that documents that are substantially revised after the initial upload can also be a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This can especially be a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the “comments” field on the web site can be used to indicate what is different in a revision, although participants tend to seldom notice what is recorded there.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when there was a consensus to consider them and there was sufficient time available for their review. Again, an exception is applied for AHG reports, CE and HLS topic summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there may have been some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

A few contributions may have had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). Any such issues were corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the responsible coordinators).

Some other errors may have also noticed in other initial document uploads (wrong document numbers or meeting dates or meeting locations in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Outputs of the preceding meeting

All output documents of the previous meeting, particularly the meeting report JVET-AK1000, the Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP JVET-AK1004, the HEVC additional profiles and SEI messages (draft 2) JVET-AK1006, the Support for additional SEI messages in AVC (draft 1)  JVET-AK1017, the Additions and corrections for VVC version 4 (Draft 11) JVET-AK2005, the Additional SEI messages for VSEI version 4 (Draft 5) JVET-AK2006, the VTM and HM common test conditions and software reference configurations for SDR 4:2:0 10 bit video JVET-AK2010, the Description of algorithms version 10 and software version 12 in neural network-based video coding (NNVC) JVET-AK2019, the Film grain synthesis technology for video applications ed. 2 (Draft 2) JVET-AK2020 (not delivered yet by beginning of the 38th meeting), the Description of the EE on Neural Network-based Video Coding JVET-AK2023, the Description of the EE on Enhanced Compression beyond VVC capability JVET-AK2024, the Algorithm description of Enhanced Compression Model 16 (ECM 16) JVET-AK2025, the Testing of video coding technology beyond conditions of exploration experiments JVET-AK2026, the Optimization of encoders and receiving systems for machine analysis of coded video content (draft 8, for future updates) JVET-AK2030, the Technologies under consideration for future extensions of VSEI (version 7) JVET-AK2032, the Draft white paper on VSEI JVET-AK2038, and the Announcement of JVET AHG17 Meeting in Aachen, DE, 10-12 March 2025 JVET-AK2039, had been completed and those which were available were approved. In a few cases, the corresponding WG 5 N-numbered documents had not yet been uploaded, and this was requested to be done as soon as possible. The software implementations of VTM versions 23.7 and 23.8, ECM versions 16.0 and 16.1, and NNVC version 12.0 were also approved.

Only minor editorial issues were found in the meeting report JVET-AK1000; no need to produce an update was identified (see section 2.14 for details).

The available output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

## Attendance

The list of participants in the JVET meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/21 or ISO/IEC JTC 1/‌SC 29/‌WG 5 (including experts who had been personally invited as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the responsible coordinators.

It was further announced that it is necessary to register for the meeting through the ISO Meetings website for ISO/IEC experts, and through the Q6/21 rapporteur for ITU-T experts. The password for meeting access had been sent to registered participants via these channels. Links to the Zoom sessions (without the necessary password) were available in the posted meeting logistics information and the calendar of meeting sessions in the JVET web site.

The following rules were established for those participating remotely via Zoom teleconference meeting:

- Use the “hand-raising” function to enter yourself in the queue to speak (unless otherwise instructed by the session chair). If you are dialed in by phone, request your queue position verbally. The online queue will be interleaved with the room queue, though it may not always be guaranteed that the sequence perfectly follows the sequence by which hand raising occurred.

- Stay muted unless you have something to say. People are muted by default when they join and need to unmute themselves to speak. The chair may mute anyone who is disrupting the proceedings (e.g. by forgetting they have a live microphone while chatting with their family or by causing bad noise or echo).

- Identify who you are and your affiliation when you begin speaking. The same applies for speakers in the room to let online participants know who is speaking.

- Use your full name and company/organization and country affiliation in your joining information, since the participation list of Zoom would also be used to compile the online part of attendance records.

- Turn on the chat window and watch for chair communication and side commentary there as well as by audio.

- Generally, do not use video for the teleconferencing calls in order to avoid overloading internet connections; enable only voice and screen sharing.

- Extensive use of screen sharing is encouraged, to enable participants to view the presented material and the meeting notes. At times, multiple sources of screen sharing may be enabled, so it may be necessary for participants to understand that this is happening and to understand how to select which shared screen they want to see.

## Agenda

The agenda for the meeting, for the further development and maintenance of the twin-text video coding technology standards Advanced Video Coding (AVC), High Efficiency Video Coding (HEVC), Versatile Video Coding (VVC), Coding-independent Code Points (Video) (CICP), and Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (VSEI), as well as related technical reports, software and conformance packages, was as follows:

Opening remarks and review of meeting logistics and communication practices

Roll call of participants

Approval of the agenda

Code of conduct policy reminder

IPR policy reminder and declarations

Contribution document allocation

Review of results of the previous meeting

Review of target dates

Reports of ad hoc group (AHG) activities

Report of exploration experiment on neural-network-based video coding (EE1)

Report of exploration experiment on enhanced compression beyond VVC capability (EE2)

Consideration of contributions on high-level syntax

Consideration of contributions and communications on project guidance

Consideration of video coding technology contributions

Consideration of contributions on conformance and reference software development

Consideration of contributions on software development in exploration activities

Consideration of contributions on coding-independent code points for video signal type identification

Consideration of contributions on film grain synthesis technology

Consideration of contributions on gaming content compression

Consideration of contributions on generative face video compression

Consideration of contributions on optimization of encoders and receiving systems for machine analysis of coded video content

Consideration of contributions on testing video coding technology beyond common testing conditions, and preparation of a call for evidence towards future video coding standardization

Consideration of contributions on aspects of ultra-low latency and packet loss resilience in the context of video compression

Consideration of contributions on errata relating to standards in the domain of JVET

Consideration of contributions on technical reports relating to standards and exploration study activities in the domain of JVET

Consideration of contributions providing non-normative guidance relating to standards and exploration study activities in the domain of JVET

Consideration of information contributions

Consideration of future work items

Coordination of visual quality testing

Liaisons, coordination activities with other organizations

Review of project editor and liaison assignments

Approval of output documents and associated editing periods

Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments (if any), establishment of AHGs, future meeting planning, other planning issues

Other business as appropriate for consideration

Closing

- The agenda was approved as suggested.

- The plans for the times of meeting sessions were established as follows, in UTC (which for this meeting was 1 hour behind the time in Geneva and Paris – 2 hours from Sunday 30 March; 7 hours ahead of the time in Los Angeles, 8 hours behind the time in Beijing, 9 hours behind the time in Seoul and Tokyo, etc.). No session was scheduled to last longer than 2 hrs.

  - 0500–0700 1st “morning” session [break after 2 hours]

  - 0720–0920 2nd “morning” session

  - [“midday” break – nearly 4 hours]

  - 1300–1500 1st “afternoon” session [break after 2 hours]

  - 1520–1720+1 2nd “afternoon” session

- It was also pointed out that the session times had been changed from meeting to meeting, such that different time zones of the world might be treated approximately equally fairly either in one meeting or another. For the current meeting, the same UTC session times were used as in the 27th JVET meeting (which had been the tenth meeting conducted as an online-only meeting, whereas the current one is the thirteenth).

## ISO and IEC Code of Conduct reminders

Participants were reminded of the ISO and IEC Codes of Conduct, found at

https://www.iso.org/publication/PUB100011.html.

https://www.iec.ch/basecamp/iec-code-conduct-technical-work

These include points relating to:

- Behave ethically

- Promote and enable all voices to be heard

- Engage constructively in ISO and IEC activities

- Respect others (in meetings, when writing, on social media)

- Declare actual and potential conflicts of interest and manage them appropriately

- Protect confidential information

- Protect ISO and IEC assets

- Avoid and prevent any form of bribery or corruption

- Uphold the consensus process

- When disputes arise, escalate, resolve and uphold agreed resolution

note to add ITU-T code of conduct

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JVET and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JVET necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jvet-site (JVET contribution templates)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

The responsible coordinators invited participants to make any necessary verbal reports of previously-unreported IPR in technology that might be considered as prospective candidate for inclusion in future standards, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that the VTM and ECM software implementation packages use the same software copyright license header as the HEVC reference software, where the latter had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time. This license header language is based on the BSD license with a preceding sentence declaring that other contributor or third party rights, including patent rights, are not granted by the license, as recorded in N 10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11 of June/July 2009. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software header is also used in the process of designing the VTM and ECM software, and for evaluating proposals for technology to be potentially included in these designs. This software or parts thereof might be published by ITU-T and ISO/IEC as an example implementation of a future video coding standard and for use as the basis of products to promote adoption of such technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

These considerations apply to the 360Lib video conversion software and HDRTools as well. The SADL and NNVC packages for neural network-based video coding use the same licensing terms.

Software packages that had been developed in prior work of the JVT, the JCT-VC and the JCT-3V have similar considerations and are maintained according to the past practice in that work.

## Communication practices

The documents for the meeting can be found at https://jvet-experts.org/. The site contains links for creating a user account for document uploading and for contacting an administrator – e.g., to update the email address associated with an account, and experts were reminded to keep this information up to date. It was reminded to send a notice to the chairs in cases of changes to document titles, authors, etc.

It was noted that also the most recent versions of JCT-VC and JCT-3V documents can be accessed directly via the JVET site. A mirror for JCT-VC, JCT-3V and JVET document access (including all versions with original upload dates) has also been provided via the ITU ftp site, providing a “documents” subfolder in the directory of each respective meeting.

JVET email lists are managed through the site https://lists.rwth-aachen.de/postorius/lists/jvet.lists.rwth-aachen.de/, and to send email to the reflector, the email address is jvet@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JVET participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work. The current number of subscribers on the JVET email list was 1292 (as of 25 March 2025). All discussions (including those on AVC, HEVC, VVC, CICP, etc.) shall be conducted on the JVET reflector rather than any of the old reflectors (including JVT, JCT-VC, and JCT-3V) which are retained for archiving purposes.

It is further emphasized that the document JVET-AJ1012 gives valuable hints about communication practices as well as other IT resources used in JVET, such as software, conformance, and test materials.

## Terminology

- ACT: Adaptive colour transform

- AFF: Adaptive frame-field

- AI: All-intra

- AIF: Adaptive interpolation filtering

- ALF: Adaptive loop filter

- AMP: Asymmetric motion partitioning – a motion prediction partitioning for which the sub-regions of a region are not equal in size (in HEVC, being N/2x2N and 3N/2x2N or 2NxN/2 and 2Nx3N/2 with 2N equal to 16 or 32 for the luma component)

- AMVP: Adaptive motion vector prediction

- AMT or MTS: Adaptive multi-core transform, or multiple transform selection

- AMVR: (Locally) adaptive motion vector resolution

- APS: Adaptation parameter set

- ARC: Adaptive resolution conversion (synonymous with DRC, and a form of RPR)

- ARMC: Adaptive re-ordering of merge candidates

- ARSS: Adaptive reference sample smoothing

- ATM: AVC-based multiview and 3D test model

- ATMVP or “subblock-based temporal merging candidates”: Alternative temporal motion vector prediction

- AU: Access unit

- AUD: Access unit delimiter

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10

- BA: Block adaptive

- BC: See CPR or IBC

- BCW: Biprediction with CU based weighting

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001)

- BDOF: Bi-directional optical flow (formerly known as BIO)

- BDPCM: Block-wise DPCM

- BL: Base layer

- BMS: Benchmark set (no longer used), a former preliminary compilation of coding tools on top of VTM, which provide somewhat better compression performance, but are not deemed mature for standardzation

- BoG: Break-out group

- BR: Bit rate

- BT: Binary tree

- BV: Block vector (used for intra BC prediction)

- CABAC: Context-adaptive binary arithmetic coding

- CBF: Coded block flag(s)

- CC: May refer to context-coded, common (test) conditions, or cross-component

- CCALF: Cross-component ALF

- CCLM: Cross-component linear model

- CCCM: Cross-component convolutional model

- CCP: Cross-component prediction

- CCSAO: Cross-component SAO

- CE: Core Experiment – a coordinated experiment conducted toward assessment of coding technology

- CG: Coefficient group

- CGS: Colour gamut scalability (historically, coarse-grained scalability)

- CIIP: Combined inter/intra prediction

- CL-RAS: Cross-layer random-access skip

- CPB: Coded picture buffer

- CPMV: Control-point motion vector

- CPMVP: Control-point motion vector prediction (used in affine motion model)

- CPR: Current-picture referencing, also known as IBC – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction

- CST: Chroma separate tree

- CTC: Common test conditions

- CVS: Coded video sequence

- DCI: Decoder capability information

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics)

- DCTIF: DCT-derived interpolation filter

- DF: Deblocking filter

- DIMD: Decoder intra mode derivation

- DMVR: Decoder motion vector refinement

- DoCR: Disposition of comments report

- DPB: Decoded picture buffer

- DPCM: Differential pulse-code modulation

- DPS: Decoding parameter sets

- DRC: Dynamic resolution conversion (synonymous with ARC, and a form of RPR)

- DT: Decoding time

- DQ: Dependent quantization

- ECS: Entropy coding synchronization (typically synonymous with WPP)

- EMT: Explicit multiple-core transform

- EOTF: Electro-optical transfer function – a function that converts a representation value to a quantity of output light (e.g., light emitted by a display

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element)

- ECM: Enhanced compression model – a software codebase for future video coding exploration

- ECV: Extended Colour Volume (up to WCG)

- EIP: Extrapolation based intra prediction

- EL: Enhancement layer

- EOS: End of (coded video) sequence

- ET: Encoding time

- FRUC: Frame rate up conversion (pattern matched motion vector derivation)

- GCI: General constraints information

- GDR: Gradual decoding refresh

- GFV: Generative face video

- GLM: Gradient linear model

- GOP: Group of pictures (somewhat ambiguous)

- GPM: Geometry partitioning mode

- GRA: Gradual random access

- HBD: High bit depth

- HDR: High dynamic range

- HEVC: High Efficiency Video Coding – the video coding standard developed and extended by the JCT-VC, formalized by ITU-T as Rec. ITU-T H.265 and by ISO/IEC as ISO/IEC 23008-2

- HLS: High-level syntax

- HM: HEVC Test Model – a video coding design containing selected coding tools that conforms to the HEVC standard design (possibly with under-development extensions) – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM)

- HMVP: History based motion vector prediction

- HOP: High-complexity operating point for neural network-based filter.

- HRD: Hypothetical reference decoder

- HTM: HEVC-based multiview and 3D test model (developed by JCT-3V)

- HyGT: Hyper-cube Givens transform (a type of NSST)

- IBC (also Intra BC): Intra block copy, also known as CPR – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction

- IBDI: Internal bit-depth increase – a technique by which lower bit-depth (8 bits per sample) source video is encoded using higher bit-depth signal processing, ordinarily including higher bit-depth reference picture storage (ordinarily 12 bits per sample)

- IBF: Intra boundary filtering

- ILP: Inter-layer prediction (in scalable coding)

- ILRP: Inter-layer reference picture

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC and HEVC)

- IRAP: Intra random access picture

- ISP: Intra subblock partitioning

- JCCR: Joint coding of chroma residuals

- JCT-3V: Joint collaborative team on 3D video (for AVC and HEVC)

- JCT-VC: Joint collaborative team on video coding (for HEVC)

- JEM: Joint exploration model – a software codebase previously used for video coding exploration

- JM: Joint model – the primary software codebase that has been developed for the AVC standard

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions

- JVET: Joint video experts team (initially for VVC, later expanded)

- JVT: Joint video team (for AVC)

- KLT: Karhunen-Loève transform

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP

- LFNST: Low-frequency non-separable transform

- LIC: Local illumination compensation

- LM: Linear model

- LMCS: Luma mapping with chroma scaling (formerly sometimes called “in-loop reshaping”)

- LOP: Low-complexity operating point for neural network-based filter.

- LP or LDP: Low delay P – the variant of the LD conditions that uses P frames

- LUT: Look-up table

- LTRP: Long-term reference picture

- MANE: Media-aware network element

- MC: Motion compensation

- MCP: Motion compensated prediction

- MCTF: Motion compensated temporal pre-filtering

- MDNSST: Mode dependent non-separable secondary transform

- MIP: Matrix-based intra prediction

- MMLM: Multi-model (cross component) linear mode

- MMVD: Merge with MVD

- MPEG: Moving picture experts group (an alliance of working groups and advisory groups in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JVET)

- MPM: Most probable mode (in intra prediction)

- MRL: Multiple reference line intra prediction

- MV: Motion vector

- MVD: Motion vector difference

- NAL: Network abstraction layer

- NNPF: Neural network post filter

- NNVC: Neural network-based video coding (experimental software package)

- NSQT: Non-square quadtree

- NSPT: Non-separable primary transform

- NSST: Non-separable secondary transform

- NUH: NAL unit header

- NUT: NAL unit type (as in AVC and HEVC)

- OBMC: Overlapped block motion compensation (e.g., as in H.263 Annex F)

- OETF: Opto-electronic transfer function – a function that converts to input light (e.g., light input to a camera) to a representation value

- OLS: Output layer set

- OOTF: Optical-to-optical transfer function – a function that converts input light (e.g. l,ight input to a camera) to output light (e.g., light emitted by a display)

- ONNX: Open Neural Network Exchange – a format used to convert code from common neural network software packages into SADL code

- operation point: A temporal subset of an OLS

- PDPC: Position-dependent (intra) prediction combination

- PERP: Padded equirectangular projection (a 360° projection format)

- PH: Picture header

- PHEC: Padded hybrid equiangular cubemap (a 360° projection format)

- PMMVD: Pattern-matched motion vector derivation

- POC: Picture order count

- PoR: Plan of record

- PROF: Prediction refinement with optical flow

- PPS: Picture parameter set

- PTL: Profile/tier/level combination

- QM: Quantization matrix

- QP: Quantization parameter (sometimes conflated with quantization step size)

- QT: Quadtree

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD)

- RADL: Random-access decodable leading (type of picture)

- RASL: Random-access skipped leading (type of picture)

- R-D: Rate-distortion

- RDO: Rate-distortion optimization

- RDOQ: Rate-distortion optimized quantization

- RDPCM: Residual DPCM

- ROI: Region of interest

- ROT: Rotation operation for low-frequency transform coefficients

- RPL: Reference picture list

- RPLM: Reference picture list modification

- RPR: Reference picture resampling (e.g., as in H.263 Annex P), a special case of which is also known as ARC or DRC

- RPS: Reference picture set

- RQT: Residual quadtree

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q)

- RVM: Rate variation measure

- SADL: Small adhoc deep learning librar

- SAO: Sample-adaptive offset

- SBT: Subblock transform

- SbTMVP: Subblock based temporal motion vector prediction

- SCIPU: Smallest chroma intra prediction unit

- SD: Slice data; alternatively, standard-definition

- SDH: Sign data hiding

- SDT: Signal-dependent transform

- SE: Syntax element

- SEI: Supplemental enhancement information

- SH: Slice header

- SHM: Scalable HM

- SHVC: Scalable high efficiency video coding

- SIF: Switchable (motion) interpolation filter

- SIMD: Single instruction, multiple data

- SMVD: Symmetric MVD

- SPO: SEI processing order

- SPS: Sequence parameter set

- SR: Super-resolution

- STMVP: Spatial-temporal motion vector prediction

- STRP: Short-term reference picture

- STSA: Step-wise temporal sublayer access

- TBA/TBD/TBP: To be announced/determined/presented/public

- TGM: Text and graphics with motion – a category of content that primarily contains rendered text and graphics with motion, mixed with a relatively small amount of camera-captured content

- TIMD: Template-based intra mode derivation

- TM: Template matching

- TMVP: Temporal motion vector prediction

- TS: Transform skip

- TSRC: Transform skip residual coding

- TT: Ternary tree

- UGC: User-generated content

- UWP: Unequal weight prediction

- VCEG: Visual coding experts group (ITU-T Q.6/21, the relevant rapporteur group in ITU-T WP3/21, which is one of the two parent bodies of the JVET)

- VCM: Video coding for machines

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy

- VQA: Visual quality assessment

- VT: Verification testing

- VTM: VVC Test Model

- VUI: Video usability information

- VVC: Versatile Video Coding

- WCG: Wide colour gamut

- WG: Working group, a group of technical experts (usually used to refer to the WGs of ISO/IEC JTC 1/SC 29).

- WPP: Wavefront parallel processing (usually synonymous with ECS)

- Block and unit names in HEVC:

  - CTB: Coding tree block (luma or chroma) – unless the format is monochrome, there are three CTBs per CTU.

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU), with a size of 16x16, 32x32, or 64x64 for the luma component.

  - CB: Coding block (luma or chroma), a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma), the level at which the prediction mode, such as intra versus inter, is determined in HEVC, with a size of 2Nx2N for 2N equal to 8, 16, 32, or 64 for luma.

  - PB: Prediction block (luma or chroma), a luma or chroma block of a PU, the level at which the prediction information is conveyed or the level at which the prediction process is performed in HEVC.

  - PU: Prediction unit (containing both luma and chroma), the level of the prediction control syntax within a CU, with eight shape possibilities in HEVC:

    - 2Nx2N: Having the full width and height of the CU.

    - 2NxN (or Nx2N): Having two areas that each have the full width and half the height of the CU (or having two areas that each have half the width and the full height of the CU).

    - NxN: Having four areas that each have half the width and half the height of the CU, with N equal to 4, 8, 16, or 32 for intra-predicted luma and N equal to 8, 16, or 32 for inter-predicted luma – a case only used when 2N×2N is the minimum CU size.

    - N/2x2N paired with 3N/2x2N or 2NxN/2 paired with 2Nx3N/2: Having two areas that are different in size – cases referred to as AMP, with 2N equal to 16 or 32 for the luma component.

  - TB: Transform block (luma or chroma), a luma or chroma block of a TU, with a size of 4x4, 8x8, 16x16, or 32x32.

  - TU: Transform unit (containing both luma and chroma), the level of the residual transform (or transform skip or palette coding) segmentation within a CU (which, when using inter prediction in HEVC, may sometimes span across multiple PU regions).

- Block and unit names in VVC:

  - CTB: Coding tree block (luma or chroma) – there are three CTBs per CTU in a P or B slice or in an I slice that uses a single tree, and one CTB per luma CTU and two CTBs per chroma CTU in an I slice that uses separate trees.

  - CTU: Coding tree unit (synonymous with LCU, containing both luma and chroma in a P or B slice or in an I slice that uses a single tree, containing only luma or only chroma in an I slice that uses separate trees), with a size of 16x16, 32x32, 64x64, or 128x128 for the luma component.

  - CB: Coding block, a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma in P/B slice, containing only luma or chroma in I slice), a leaf node of a QTBT. It’s the level at which the prediction process and residual transform are performed in JEM. A CU can be square or rectangle shape.

  - PB: Prediction block, a luma or chroma block of a PU.

  - PU: Prediction unit, has the same size as a CU in the VVC context.

  - TB: Transform block, a luma or chroma block of a TU.

  - TU: Transform unit, has the same size as a CU in the VVC context.

## Standards, TRs, supplements and technical papers approval and publication status

- MPEG-2 | H.262 (the video coding specification is common text)

  - ITU-T H.262 V3 was approved in 2012-02; Amd.1 was approved in 2013-03 and was not published separately; it was instead incorporated directly into the V3 text and published 2013-09

  - ISO/IEC 13818-2:2013 (Ed. 3) FDIS ballot closed 2012-05-08; FDAM 1 ballot closed 2013-04-12 and was not published separately; it was instead incorporated directly into the V3 text and published 2013-10

  - Conformance testing (not joint with ITU-T)

    - ISO/IEC 13818-4:2004 (Ed. 2) FDIS closed 2004-08-22, published 2004-12-12; it specifies conformance testing for Part 1 (Systems), Part 2 (Video), Part 3 (Audio), and Part 7 (AAC)

    - ISO/IEC 13818-4:2004/Amd 3:2009 Level for 1080@50p/60p conformance testing

    - Cor 1:2007, Cor 2:2009, Cor 3:2012, Cor 4:2011 may also have video relevance

  - Reference software (not joint with ITU-T)

    - ISO/IEC TR 13818-5:2005 (Ed. 2) FDIS closed 2005-07-24, published 2005-10; it specifies reference software for Part 1 (Systems), Part 2 (Video), Part 3 (Audio), Part 7 (AAC) and Part 11 (IPMP)

- AVC (twin text)

  - ITU-T H.264 V14 was Consented at 22nd meeting on 2021-04-30 (with annotated regions, shutter interval, and miscellaneous corrections), approved 2021-08-22, published 2021-10-13

  - ISO/IEC 14496-10:2020 (Ed. 9) FDIS ballot closed 2020-11-27, published 2020-12-15

  - ISO/IEC 14496-10:2022 (Ed. 10), had been forwarded from DIS directly for publication 2022-01-21 (with annotated regions, shutter interval, and miscellaneous corrections) with an editing period, submitted to ITTF in 2022-05 after consultation with ISO staff on format of graphics files, upgraded to “DIS approved for registration” in ISO Project system 2022-07-04, published 2022-11-07

  - Preliminary draft text for YCgCo-Re and YCgCo-Ro issued at 26th meeting, second draft including SMPTE ST 2128 IPT-PQ-C2 issued at 28th meeting 2022-10, third draft issued at 29th meeting 2023-01, fourth draft issued at 30th meeting 2023-04, formal project requested and CD of 11th edition issued at 31st meeting 2023-07, DIS issued at 32nd meeting 2023-10, DAM ballot closed 2024-04-15.

    - H.264 V15 Consented 2024-04-26, last call began 2024-07-16, approved 2024-08-13, published 2024-11-13

    - ISO/IEC 14496-10:202x (Ed. 11) FDIS issued at 34th meeting 2024-04, DIS approved for registration as FDIS 2024-09-17, FDIS ballot issued 2025-01-14, FDIS ballot closed 2025-03-23; pending publication

  - Amendment to support some SEI messages of VSEI v4 requested at 37th meeting 2025-01, registered to work programme 2025-01-25, ready for action to issue ISO/IEC CDAM

  - Conformance testing (twin text)

    - ITU-T H.264.1 V6 Approved 2016-02-13, published 2016-06-17

    - Various amendments of ISO/IEC 14496-4:2004, including:

      - ISO/IEC 14496-4:2004/AMD 6:2005 Advanced Video Coding conformance

      - ISO/IEC 14496-4:2004/AMD 9:2006 AVC fidelity range extensions conformance

      - ISO/IEC 14496-4:2004/AMD 30:2009 Conformance testing for new profiles for professional applications

      - ISO/IEC 14496-4:2004/AMD 31:2009 Conformance testing for SVC profiles

      - ISO/IEC 14496-4:2004/AMD 38:2010 Conformance testing for Multiview Video Coding

      - ISO/IEC 14496-4:2004/AMD 41:2014 Conformance testing of MVC plus depth extension of AVC

      - ISO/IEC 14496-4:2004/AMD 42:2014 Conformance testing of Multi-Resolution Frame Compatible Stereo Coding extension of AVC

      - ISO/IEC 14496-4:2004/AMD 43:2015 3D-AVC conformance testing

      - ISO/IEC 14496-4:2004/AMD 45:2016 Conformance Testing for the Multi-resolution Frame Compatible Stereo Coding with Depth Maps Extension of AVC

  - Reference software (twin text)

    - ITU-T H.264.2 V7 Approved 2016-02-13, published 2016-05-30

    - Various amendments of ISO/IEC 14496-5:2001 have been published, including:

      - ISO/IEC 14496-5:2001/AMD 6:2005 Advanced Video Coding (AVC) and High Efficiency Advanced Audio Coding (HE AAC) reference software

      - ISO/IEC 14496-5:2001/AMD 8:2006 AVC fidelity range extensions reference software

      - ISO/IEC 14496-5:2001/AMD 15:2010 Reference software for Multiview Video Coding

      - ISO/IEC 14496-5:2001/AMD 18:2008 Reference software for new profiles for professional applications

      - ISO/IEC 14496-5:2001/AMD 19:2009 Reference software for Scalable Video Coding

      - ISO/IEC 14496-5:2001/AMD 33:2015 Reference software for MVC plus depth extension of AVC

      - ISO/IEC 14496-5:2001/AMD 34:2014 Reference software of the multi-resolution frame compatible stereo coding of AVC

      - ISO/IEC 14496-5:2001/AMD 35:2015 3D-AVC Reference software

      - ISO/IEC 14496-5:2001/AMD 39:2016 Reference software for the Multi-resolution Frame Compatible Stereo Coding with Depth Maps of AVC

      - ISO/IEC 14496-5:2001/AMD 42:2017 Reference software for the alternative depth information SEI message extension of AVC

- HEVC (twin text)

  - ITU-T H.265 V7 approved 2019-11-29, published 2020-01-10

  - ISO/IEC 23008-2:2020 (Ed. 4) FDIS ballot closed 2020-07-16, published 2020-08-27

  - ITU-T H.265 V8 Consented at the 22nd meeting (shutter interval information SEI message and miscellaneous corrections), published 2020-10-13

  - ISO/IEC 23008-2:2020/AMD 1:2021 (shutter interval information SEI message) published 2021-07-12

  - ISO/IEC 23008-2:2023 (Ed. 5) began as CDAM 2 High-range levels output of 25th meeting of January 2022, CDAM ballot closed 2022-04-15, conversion to 5th edition with miscellaneous corrections planned at 26th meeting of 2022-04, text submitted for DIS ballot 2022-07-10, DIS ballot closed 2023-01-10, FDIS issued 29th meeting of 2023-01, FDIS ballot opened 2023-08-06, closed 2023-10-02, published 2023-10-30

  - ITU-T H.265 V9 Consented at 31st meeting 2023-07, approved 2023-09-13, and pre-published 2023-09, published 2023-11-24.

  - Preliminary draft HEVC text for YCgCo-Re and YCgCo-Ro issued at 26th meeting 2022-04, second draft including SMPTE ST 2128 issued at 28th meeting 2022-10, third draft at 29th meeting 2023-01, fourth draft at 30th meeting 2023-04, formal work item requested and CDAM1 issued 31st meeting 2023-07, DAM issued with new (multiview) profiles and SEI messages at 32nd meeting 2023-10, DAM ballot closed 2024-04-08

    - H.265 V10 Consented 2024-04-26, last call opened 2024-07-01, closed 2024-07-28, approved 2024-07-29, pre-published 2024-08-06, published 2024-10-07

    - ISO/IEC 23008-2:2025 (Ed. 6) FDIS issued from 35th meeting 2024-04, DIS approved for registration as FDIS 2024-10-03, FDIS registered for formal approval 2024-10-13, FDIS ballot issued 2024-12-30, FDIS ballot closed 2025-02-24, published 2025-03-20

  - Amendment to support additional (multiview) profiles and some SEI messages of VSEI v4, project requested at 36th meeting 2024-11, project registered in ISO/IEC work programme 2025-01-25, CDAM issued at 37th meeting, consulation began 2025-02-01, pending closure of comment period

  - Conformance testing (twin text)

    - ITU-T H.265.1 V3 approved 2018-10-14, published 2019-01-15

    - ISO/IEC 23008-8:2018 (Ed. 2) Conformance specification for HEVC, published 2018-08-06

    - ISO/IEC 23008-8:2018/AMD 1:2019 Conformance testing for HEVC screen content coding (SCC) extensions and non-intra high throughput profiles, published 2019-10-15

  - Reference software (twin text)

    - ITU-T H.265.2 V4 approved 2016-12-22, published 2017-04-10

    - ISO/IEC 23008-5:2017 (Ed. 2) Reference software for high efficiency video coding, FDIS issued from 2016-02 meeting, published 2017-03-01

    - ISO/IEC 23008-5:2017/AMD 1:2017 Reference software for screen content coding extensions, FDAM issued from 2017-04 meeting, FDAM ballot opened 2017-08-10, closed 2017-10-07, published 2017-11-09

- VVC (twin text)

  - ITU-T H.266 V1 approved 2020-08-29, published 2020-11-10

  - ISO/IEC 23090-3:2021 (Ed. 1) published 2021-02-16

  - ITU-T H.266 V2 with operation range extensions, Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-06-06, published 2022-07-12

  - ISO/IEC 23090-3:2022 (Ed. 2) with operation range extensions, approval at WG level to proceed to FDIS 2022-01-21, FDIS ballot opened 2022-06-29, closed 2022-08-24, published 2022-09-25

  - ISO/IEC 23090-3:2024 (Ed. 3), initated as (Ed. 2) / Amd.1 New level and systems-related supplemental enhancement information, CDAM 1 issued from 26th meeting, ballot closed 2022-07-14, DAM 1 issued from 27th meeting, ballot closed 2023-01-03, FDIS issued at WG level 2023-07, FDIS ballot opened 2024-05-11, closed 2024-06-26, published 2024-07-17

  - ITU-T H.266 V3 Consented 2023-07, approved 2023-09-29 and pre-published 2023-09, published 2023-11-29

  - ISO/IEC 23090-3:2024/CDAM 1 Request & CDAM issued 2024-04, consultation deferred due to meeting timing, updated text issued 2024-07, consultation initiated 2024-09-05, closed 2024-10-31, ready for action to issue ISO/IEC DAM (or DIS)

  - Conformance testing (twin text)

    - ITU-T H.266.1 V1 Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-05-17, published 2022-07-12

    - ISO/IEC 23090-15:2022 (Ed. 1) approval at WG level to proceed to FDIS 2022-10-15, upgraded to “DIS approved for registration” in ISO Projects system 2021-10-24, upgraded to “FDIS registered for formal approval” 2022-07-11, FDIS ballot closed 2022-11-04, published 2022-11-24

    - ISO/IEC 23090-15:2024 (Ed. 2) began as Amd.1 Operation range extensions – CDAM 1 issued from 24th meeting 2021-10, DAM 1 issued from 25th meeting 2022-01-21, upgraded to “CD approved for registration as DIS” status in ISO Projects system 2022-05-31, upgraded to “DIS registered” 2022-06-22, DAM ballot closed 2022-11-15, consolidated into FDIS 2nd edition issued as an output of the 29th meeting in January 2023, ballot opened 2024-04-08, closed 2024-06-03, published 2024-07-04.

    - ITU-T H.266.1 V2 Consented 2023-07, approved 2023-09-13 and pre-published 2023-09, published 2023-10-19

    - ISO/IEC 23090-15 (Ed. 3) project requested at 37th meeting, project registered in work programme 2025-01-25, ready for action to issue CD.

  - Reference software (twin text)

    - ITU-T H.266.2 V1 Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-05-17, published 2022-07-12

    - ISO/IEC 23090-16:2022 V1 approval at WG level to proceed to FDIS 2022-01-21, upgraded to “DIS approved for registration” status in ISO Projects system 2022-04-21, upgraded to “FDIS registered for formal approval” 2022-04-22, FDIS ballot initiated 2022-07-24, FDIS ballot closed 2022-09-19, published 2022-10-23

    - H.266.2 V2 Consented 2024-04-26, last call opened 2024-05-16, closed 2024-06-12, approved 2024-06-13, pre-published 2024-08-06, published 2024-10-09

    - ISO/IEC 23090-16:202x (Ed. 2) Request & CD issued 2024-04, consultation deferred due to meeting timing, consultation initiated 2024-09-06, closed 2024-11-01, DIS issued at 36th meeting 2024-11, approved for registration as DIS 2024-12-18, DIS registered 2025-01-7, DIS ballot opened 2025-03-09, no action at the current meeting (Roughly corresponding to H.266.2 V2 already approved and published in ITU-T)

- VSEI (twin text)

  - ITU-T H.274 V1 approved 2020-08-29, published 2020-11-10

  - ISO/IEC 23002-7:2021 (Ed. 1) published 2021-01-28

  - ITU-T H.274 V2 Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-05-22 (after 1 Last Call comment and Additional Review), pre-published 2022-06-17, published 2022-07-25

  - ISO/IEC 23002-7:2022 (Ed. 2) approval at WG level to proceed to FDIS 2022-01-21, upgraded to “DIS approved for registration” status in ISO Projects system 2022-05-05 and “FDIS registered for formal approval” 2022-05-08, FDIS ballot closed 2022-09-27, published 2022-10-30

  - ISO/IEC 23002-7:2024 (Ed. 3) began as (2nd Ed.) Amd.1 Request for new edition and CD for additional SEI messages issued at 27th meeting, ballot closed 2022-10-10, DAM registered 2022-11-13, DAM ballot closed 2022-04-06, FDIS 3rd edition issued 2023-07, FDIS ballot began 2024-07-25, closed 2024-09-20, published 2024-10-30

  - ITU-T H.274 V3 Consent 2023-07, approved 2023-09-29, pre-published 2023-10-11, published 2024-03-12.

  - ISO/IEC 23002-7:202x (Ed. 4) Request & CDAM 1 issued 2024-04, consultation deferred due to meeting timing, updated text issued 2024-07, CD consultation initiated 2024-09-05, closed 2024-10-31, ready for action to issue ISO/IEC DAM (or DIS)

- CICP (twin text)

  - ITU-T H.273 V2 (with 4:2:0 sampling alignment and corrections for range of values for sample aspect ratio, ICTCP equations for HLG, and transfer characteristics function for sYCC of IEC 61966-2-1) Consented on 2021-04-30, Last Call closed during the 23rd meeting with approval on 2021-07-14, published 2021-09-24

  - ISO/IEC 23091-2:2021 (Ed. 2) had been forwarded from DIS directly for publication in 2021-04 and published 2021-10-18

  - ITU-T H.273 V3 Consent 2023-07, approved 2023-09, not published due to waiting for publication of SMPTE ST 2128.

  - ITU-T H.273 V4 Consented 2024-04-26, last call opened 2024-06-16, closed 2024-07-13, approved 2024-07-14, pre-published 2024-08-06, published 2024-10-07.

  - ISO/IEC 23091-2:2025 (Ed. 3) Request for new edition and CD for new edition (including YCgCo-Re and YCoCg-Ro) issued at 27th meeting, ballot closed 2022-10-10, DIS registered 2022-11-13, DIS ballot closed 2023-04-06, preliminary draft text for including SMPTE ST 2128 issued at 28th meeting, incorporated into preliminary FDIS at 30th meeting 2023-04, FDIS issued 2024-04 (after waiting for publication of SMPTE ST 2128, then proceeding), FDIS registered for formal approval 2024-10-24, FDIS ballot issued 2024-12-03, FDIS ballot closed 2025-01-28, published 2025-02-25.

- Conversion and coding practices for HDR/WCG Y′CbCr 4:2:0 video with PQ transfer characteristics (twin text)

  - H.Sup15 V1, approved 2017-01-27, published 2017-04-12

  - ISO/IEC TR 23008-14:2018 (Ed. 1) published 2018-08-06

- Signalling, backward compatibility and display adaptation for HDR/WCG video coding (twin text)

  - H.Sup18 V1, approved 2017-10-27, published 2018-01-18

  - ISO/IEC TR 23008-15:2018 (Ed. 1) published 2018-08

- Usage of video signal type code points (twin text)

  - H.Sup19 V3 approved 2021-04-30, published 2021-06-04

  - ISO/IEC TR 23091-4:2021 (Ed. 3) published 2021-05-23

- Working practices using objective metrics for evaluation of video coding efficiency experiments (twin text)

  - HSTP-VID-WPOM V1 approved 2020-07-03, published 2020-11

  - ISO/IEC TR 23002-8:2021 (Ed. 1) published 2021-05-20

- Film grain synthesis technologies for video applications (twin text)

  - ISO/IEC TR 23002-9:2024 (Ed. 1) Request for subdivision and WD 1 issued at 25th meeting 2022-01-21, WD 2 issued at 27th meeting, WD 3 issued at 28th meeting, CDTR issued at 29th meeting 2023-01, consultation period ended 2023-07-09, DTR text was issued from the 31st meeting in July 2023, put on hold by ISO staff editors, DTR ballot opened 2024-03-05, closed 2024-04-30, NB comments handled by ISO staff editor in consultation with project editors, published 2024-07-24

  - ITU-T H-Series Supplement 21 (ex H.Sup-FGST), Agreement 2025-01-24 by ITU-T SG21, pre-publication website malfunctioning as of 2025-03-25

  - Edition 2 planned but not yet in formal work programme of ISO/IEC

- Optimization of encoders and receiving systems for machine analysis of coded video content (twin text)

  - ISO/IEC 23888-3 (Ed. 1) Request for subdivision issued from 33rd JVET meeting 2024-01, CDTR issued from 34th meeting 2024-04, consultation deferred due to meeting timing, updated text issued from 35th meeting 2024-07, consultation further deferred due to meeting timing, further updated text issued from 36th meeting 2024-11, consultation initiated 2025-01-14, consultation period ended 2025-03-11, ready for action to issue DTR

  - ITU-T provisional name H.Sup-MACVC

- The following freely available standards are published here in ISO/IEC:
https://standards.iso.org/ittf/PubliclyAvailableStandards/index.html as of the time of the current meeting:

  - ISO/IEC 13818-4:2004 Conformance testing for MPEG-2

  - ISO/IEC 13818-4:2004/Amd 3:2009 Level for 1080@50p/60p conformance testing

  - ISO/IEC TR 13818-5:2005 Software simulation for MPEG-2

  - Various amendments of ISO/IEC 14496-4:2004 Conformance for AVC

  - Various amendments of ISO/IEC 14496-5:2001 Reference software for AVC

  - ISO/IEC 14496-10:2022 (Ed. 10) AVC

  - ISO/IEC 23090-16:2022 (Ed. 1) Reference software for VVC

- The following standards that have been intended by JVET to be publicly available were not available at https://standards.iso.org/ittf/PubliclyAvailableStandards/index.html as of the time of the current meeting. (These should be checked for previously issued requests for free availability.)

  - ISO/IEC 23002-7:2024 (Ed. 3) – VSEI, published 2024-10-30 (public availability requested 2023-07)

  - ISO/IEC 23008-2:2025 (Ed. 6) HEVC, published 2023-10-30 (need to check for public availability request)

  - ISO/IEC 23008-5:2017 (Ed. 2) Reference software for HEVC, published 2017-03-01 (public availability requested 2016-02)

  - ISO/IEC 23008-5:2017/AMD 1:2017 Reference software for HEVC screen content coding extensions, published 2017-11-09 (not requested 2017-01, 2017-04, 2017-07, 2017-10, 2018-01)

  - ISO/IEC 23008-8:2018 (Ed. 2) Conformance specification for HEVC, published 2018-08-06 (public availability requested 2018-10)

  - ISO/IEC 23008-8:2018/AMD 1:2019 Conformance testing for HEVC screen content coding (SCC) extensions and non-intra high throughput profiles, published 2019-10-15

  - ISO/IEC 23090-3:2024 (Ed. 3) VVC, published 2024-07-17

  - ISO/IEC 23090-15:2024 (Ed. 2) Conformance specification for VVC, published 2024-07-04.

  - ISO/IEC 23091-2:2025 (Ed. 3) CICP, published 2025-02-25

- The following technical reports by JVET may not have been requested to be publicly available due to a former policy interpretation by ITTF that has since been changed. (These should be checked for previously issued requests for free availability.)

  - ISO/IEC TR 23008-14:2018 (Ed. 1) Conversion and coding practices for HDR/WCG Y′CbCr 4:2:0 video with PQ transfer characteristics, published 2018-08-06

  - ISO/IEC TR 23008-15:2018 (Ed. 1) Signalling, backward compatibility and display adaptation for HDR/WCG video coding, published 2018-08

  - ISO/IEC TR 23091-4:2021 (Ed. 3) Usage of video signal type code points, published 2021-05-23

  - ISO/IEC TR 23002-8:2021 (Ed. 1) Working practices using objective metrics for evaluation of video coding efficiency experiments, published 2021-05-20

  - ISO/IEC TR 23002-9:2024 (Ed. 1) Film grain synthesis technologies for video applications, published 2024-07-24

- It appears necessary to check if all older software and conformance packages are publicly available – it might be that this was never requested, e.g. for those that were produced by JCT-3V. This topic was left TBD until the next meeting – perhaps it would be best to compile a list of all relevant software and conformance parts of AVC, HEVC, MPEG-2 aka H.262, CICP, and request these in bulk.

## Draft standards progression status for active work items

- AVC ISO/IEC 14496-10:202x/CDAM 1 to support some SEI messages of VSEI v4 requested at 37th meeting 2025-01, registered to work programme 2025-01-25, ready for action to issue ISO/IEC CDAM

- HEVC ISO/IEC 23008-2:2025/CDAM 1 to support additional (multiview) profiles and some SEI messages of VSEI v4, project requested at 36th meeting 2024-11, project registered in ISO/IEC work programme 2025-01-25, CDAM issued at 37th meeting, consulation began 2025-02-01, pending closure of comment period

- VVC ISO/IEC 23090-3:2024/CDAM 1, Request & CDAM issued 2024-04, consultation deferred due to meeting timing, updated text issued 2024-07, consultation initiated 2024-09-05, closed 2024-10-31, ready for action to issue ISO/IEC DAM (or DIS)

- VVC conformance ISO/IEC 23090-15 (Ed. 3) project requested at 37th meeting, project registered in work programme 2025-01-25, ready for action to issue CD.

- VVC reference software ISO/IEC 23090-16:202x (Ed. 2) Request & CD issued 2024-04, consultation deferred due to meeting timing, consultation initiated 2024-09-06, closed 2024-11-01, DIS issued at 36th meeting 2024-11, approved for registration as DIS 2024-12-18, DIS registered 2025-01-7, DIS ballot opened 2025-03-09, no action at the current meeting. (Roughly corresponding to H.266.2 V2 already approved and published in ITU-T.)

- VSEI ISO/IEC 23002-7:202x (Ed. 3) / CDAM 1 Request & CDAM issued 2024-04, consultation deferred due to meeting timing, updated text issued 2024-07, CD consultation initiated 2024-09-05, closed 2024-10-31, ready for action to issue ISO/IEC DAM (or DIS)

- Film grain synthesis technology for video applications – Edition 2 planned but not yet in formal work programme of ISO/IEC

- Optimization of encoders and receiving systems for machine analysis of coded video content – ISO/IEC 23888-3 Request for subdivision issued from 33rd JVET meeting 2024-01, CDTR issued from 34th meeting 2024-04, consultation deferred due to meeting timing, updated text issued from 35th meeting 2024-07, consultation further deferred due to meeting timing, further updated text issued from 36th meeting 2024-11, consultation initiated 2025-01-14, consultation period ended 2025-03-11, ready for action to issue DTR. ITU-T provisional name H.Sup-MACVC.

- A request for free availability in ISO/IEC has to be made for each edition, amendment and corrigendum, and the request needs to be approved in the WG 5 Recommendations. A request form also needs to be filled out (but the form does not need to be issued as a WG 5 document). A freely available URL for the ITU publication should be provided for the ongoing work items when they become finalized.

## Opening remarks

Remarks during the opening session of the meeting Wednesday 26 March at 0535-0620 UTC were as follows.

- Timing and organization of the meeting and online access and calendar posting of session plans were reviewed

  - The initial number of documents was slightly higher than for last meeting (approximately 215 vs. 205 by the time of opening the meeting) – parallel sessions were announced to be necessary.

  - Start of parallel sessions (HLS) Thursday morning

  - JVET will not meet during the MPEG information exchange sessions on Monday 31 March (0500-0800 UTC) and Wednesday 02 April (0500-0600 UTC). The information exchange on Friday 04 April (2100 UTC) is scheduled after the regular JVET time slots.

- Plans for subsequent hybrid meetings (with best-effort remote access) were reviewed: June/July 2025 (Daejeon), Oct. 2025 (Geneva), April 2026 (Santa Eulària), June/July 2026 (Geneva), October 2026 (Hangzhou).

- The January 2026 meeting is currently planned to be virtual.

- Depending on the status of preparing future standardization activities, it may be necessary to extend the duration of meetings.

- Significant workload was expected at this meeting for AHG17 activities – a hybrid AHG meeting in Aachen was held, and follow-up discussion is necessary to make a step forward in drafting the Call for Evidence. It was suggested to start this discussion early at the current meeting (Wednesday afternoon) – joint meetings with parent bodies are expected starting from Monday.

- The meeting logistics, agenda, working practices, policies, and document allocation considerations were reviewed.

  - Access to the meeting was provided using Zoom. The meeting notes by the session chair were to be continually shared via zoom screen sharing. In the meeting room C, it is also possible to show them on a separate projection screen in parallel with another presentation.

  - Having text and software available is crucial (and not just arriving at the end of the meeting).

- The results of the previous meeting and the meeting report JVET-AK1000 were reviewed. Only minor issues in the meeting report were noted and were not considered sufficient to warrant issuing a revision.

- At the current meeting, on-site attendance will be recorded via the zoom records. It is therefore important to follow the conventions of naming participants. Participants who cannot be correctly identified will not appear in the attendance sheet and will also be removed from the zoom sessions.

- All cross-check documents were late, whereas the number of late non-cross-check documents seemed to have slightly decreased relative to the last meeting. Several contributions did not report any results initially, and were also flagged as late (so-called “placeholder” uploads). There were no objections voiced in the opening plenary to the consideration of late contributions.

- There were again a few documents registered where authors’ given names were not abbreviated according to the JVET custom (which helps produce shorter headings in the JVET meeting report), and/or company affiliation was missing in the authors’ list. Participants were reminded to stick to JVET’s conventions.

- Experts were asked not to pick a specific JVET number for regular documents – this function is reserved for AHG reports, summary reports, and output docs. Reserving numbers without filling a precise title shall also be avoided – the chair may flag such documents as withdrawn, as they cannot be allocated to a certain category in the meeting notes.

- Experts were asked to always register JVET documents via the “jvet-experts.org” site, not via the MPEG DMS site, as WG 5 docs (as that feature of the DMS site has not been working properly).

- Experts were asked to inform the chair when the title of a document is changed, or if authors are added. Otherwise, that might not be correct in the meeting notes. Provisional titles such as “EEx contribution” shall be avoided.

- The practice introduced in the Kemer meeting, submitting WG 5 N-numbered output documents only in cases of standards text submitted for ballot, DoCRs, standards/parts requests, and meeting reports has turned out to be useful. This has also helped in making the documents in the MPEG dms and ISO documents available in a timely fashion.

- Still, only the newest versions of the JCT-VC and JCT-3V documents are available from the links in the JVET site, but a second source for JVET, JCT-VC and JCT-3V documents was set up in the ITU ftp directories of each meeting. This includes all versions of documents with original upload times.

- The following ballot results had become available through the SC 29 secretariat:

  - CDTR on machine analysis m72107

  - CDAM HEVC  m72110

- DAM for VVC and VSEI version/edition 4, as well as HEVC are planned to be done at the current meeting. In order to have but ballot results available prior to the October meeting (when also submission for ITU consent is planned), only a relatively short editing period is possible (April 25 at latest). It had been emphasized in the previous meeting that only minor improvements might be made during the March/April meeting. Considering the large number of input documents requesting changes, this is critical – some priority needs to be assigned, and potentially leave some aspects open until the final stage. Draft DoCs exist reflecting the status of JVET-AK2005 and JVET-AK2006.

- The CDTR could also be promoted to DTR (as SEI messages in VSEI v4 DAM are referred in the draft). The version issued from the last meeting appears stable, no input requesting changes was submitted. Editors will look at the ballot comments and report back. Revisit.

- DIS of VVC software was submitted for ballot – matching the ITU edition that was issued in April. Starting work towards an amendment or a next edition could be targeted for October, considering the implementation maturity of new SEI messages.

- The primary goals of the meeting were:

  - VVC and VSEI DAM texts, for ISO/IEC ballot

  - HEVC DAM text, for ISO/IEC ballot

  - AVC CDAM text, for ISO/IEC ballot

  - VVC conformance CD text, for ISO/IEC ballot?

  - New software versions VTM/HM/JM – support for SEI messages?

  - Lot of activity in AHG17 – draft/preliminary CfE as output

  - Exploration Experiments

    - Neural network-based video coding

    - Enhanced compression beyond VVC

  - Liaison communication?

  - VSEI and VVC white papers – further work until next meeting

- Joint meetings were expected with MPEG AG 5 (on matters involving visual quality assessment), and with MPEG WG 2 Requirements and ITU-T VCEG on future video standardization (see section 7.3).

- As a follow-up to liaison communication after the April meeting, parent bodies were expected to conduct discussion about future JVET management structures. This has not happened so far.

- Principles of standards development were discussed.

- Scheduling of sessions was discussed – see under sections 2.6 and 2.12.

## Scheduling of discussions

- The plans for the times of meeting sessions were established as follows, in UTC (which for this meeting was 1 hour behind the time in Geneva and Paris – 2 hours from Sunday 30 March; 7 hours ahead of the time in Los Angeles, 8 hours behind the time in Beijing, 9 hours behind the time in Seoul and Tokyo, etc.). No session was scheduled to last longer than 2 hrs.

- 0500–0700 1st “morning” session [break after 2 hours]

- 0720–0920 2nd “morning” session

- [“midday” break – nearly 4 hours]

- 1300–1500 1st “afternoon” session [break after 2 hours]

- 1520–1720+1 2nd “afternoon” session

Particular scheduling notes are shown below, although not necessarily 100% accurate or complete. Times are recorded in the local timezone of the meeting venue, except as otherwise noted:

- Wed. 26 March, 1st day

  - Morning sessions:

    - 0500–0620 Opening remarks, review of practices, agenda, IPR policy reminder

    - 0625–0700 Reports of AHGs 1-3

    - 0720–0925 Reports of AHGs 4-11

    - …

  - Afternoon sessions:

    - 1300–1400 Reports of AHGs 12-18

    - 1405–1505 CfE planning

    - 1525–1725 CfE planning

- Thu. 27 March, 2nd day

  - Morning sessions:

    - 0500–0700 EE1/EE2 review (summary reports)

    - 0720–0920 EE1/EE2 review (summary reports)

    - 0520–0920 HLS (chaired by J. Boyce)

  - Afternoon sessions:

    - 1300–1500 EE2 review

    - 1520–1720 EE2 review

    - 1320–1720 HLS (chaired by J. Boyce)

- Fri. 28 March, 3rd day

  - Morning sessions:

    - 0500–0700 …

    - 0720–0920 …

    - …

  - Afternoon sessions:

    - 1300–1500 …

    - 1520–1720 …

    - …

- Mon. 31 March, 4th day

  - 0500–0800 MPEG information sharing session

  - Morning sessions

    - 0820–0920 …

    - …

  - Afternoon sessions:

    - 1300–1500 …

    - 1520–1720 …

    - …

- Tue 1 April, 5th day

  - Morning sessions:

    - 0500–0700 …

    - 0720–0920 …

    - …

  - Afternoon sessions:

    - 1300–1500 …

    - 1520–1720 …

    - …

- Wed. 2 April, 6th day

  - 0500–0600 MPEG information sharing session

  - Morning sessions:

    - 0600–0700 …

    - 0720–0920 …

    - …

  - Afternoon sessions:

    - 1300–1500 …

    - 1520–1720 …

    - …

- Thu. 25 January, 7th day

  - Morning sessions:

    - 0500–0700 …

    - 0720–0920 …

    - …

  - Afternoon sessions:

    - 1300–1500 …

    - 1520–1720 …

    - …

- Fri. 26 January, 8th day

  - Morning sessions:

    - 0500–0700 …

    - 0720–0920 …

    - …

  - 1300–1700 (with break) JVET wrap-up plenary:

    - Approval of output docs

    - Establishment of AHGs

    - Review of meeting recommendations

    - Future planning, a.o.b.

  - 2100–2300 MPEG information sharing session

  - XXXX–XXXX WG 5 approval of meeting recommendations, closing of meeting

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized as follows (note that the noted document counts do not include crosschecks and summary reports, and may not be completely accurate; documents which are allocated to multiple sections are only counted in one of them):

- AHG reports (18) (section 3)

- Project development (section 4)

  - AHG1: Development, deployment and advertisement of standards (6)

  - AHG2: Text development and errata reporting (2)

  - AHG3: Software development (0)

  - AHG3: Test conditions (3)

  - AHG4: Subjective quality testing and verification testing (1)

  - AHG4: Test and training material (0)

  - AHG5: Conformance test development (0)

  - AHG7: ECM tool assessment (5)

  - AHG8: Optimization of encoders and receiving systems for machine analysis of coded video content (1)

  - AHG10: Encoding algorithm optimization (3)

  - AHG13: Film grain synthesis (1)

  - Implementation studies (0)

  - Profile/tier/level specification (0)

  - AHG15: Gaming content compression (0)

  - AHG16: Generative face video (2)

  - AHG17: CfE preparation (14)

  - AHH18: Ultra-low latency and error resilience (3)

- Low-level tool technology proposals (section 4.16) with subtopics (number counts excluding BoG and summary reports)

  - AHG11/AHG14 and EE1: Neural network-based video coding (27) (section 5.1)

  - AHG6/AHG12 and EE2: Enhanced compression beyond VVC capability (76) (section 5.2)

- AHG9: High-level syntax (HLS) proposals (section 6) with subtopics

  - Additions for HEVC and AVC (6) (section 6.1)

  - SEI messages in VSEIv4 (47) (section 6.2)

  - SEI messages in TuC doc (15) (section 6.3)

  - SEI messages on other topics (10) (section 6.4)

  - Non-SEI HLS aspects (0) (section 6.5)

- Joint meetings, plenary discussions, BoG reports (0) liaison (0), summary of actions (section 7)

- Project planning (section 8)

- Establishment of AHGs (section 9)

- Output documents (section 10)

- Future meeting plans and concluding remarks (section 11)

The document counts above do not include cross-checks and summary reports.

# AHG reports (18)

These reports were discussed during 0625–0700, 0720–0925 and 1300–1400 on Wednesday 26 March 2025 (chaired by JRO).

JVET-AL0001 JVET AHG report: Project Management (AHG1) [J.-R. Ohm (chair), G. J. Sullivan (vice chair)]

The work of the JVET overall had proceeded well in the interim period with similar number of input documents (as compared to the previous meeting) submitted to the current meeting. Intense discussion had been carried out on the group email reflector, and most output documents from the preceding meeting had been produced as planned.

Output documents from the preceding meeting had been made initially available at the JVET web site (https://jvet-experts.org/) or the ITU-based JVET ftp site (http://wftp3.itu.int/av-arch/jvet-site/2025_01_AK_Geneva/). It is noted that the previous document sites http://phenix.int-evry.fr/jvet/, http://phenix.int-evry.fr/jct/, and http://phenix.int-evry.fr/jct3v/ were shut down, but JCT-VC and JCT-3V documents can be accessed directly via the JVET site. All documents of JCT-VC, JCT-3V and JVET are also available from the ITU-based ftp site, where sub-folders ‘./documents/’ were created in the folders of the respective meetings (it is noted that for practicality reasons, this is only done after all meeting documents including output documents are completely available in all versions). All those files come with the original time stamps by which the original versions of documents were uploaded.

The list of output documents produced since the last meeting included the following:

- JVET-AK1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP [Posted 2025-02-18, last update 2025-03-06]

- JVET-AK1006 HEVC additional profiles and SEI messages, also issued as WG 5 CDAM N 336 [Posted 2025-01-29]

- JVET-AK1017 Support for additional VSEI messages in AVC (draft 1) [Posted 2025-02-18]

- JVET-AK2005 Additions and corrections for VVC version 4 (Draft 11), also issued as WG 5 preliminary DAM update N 337 [Posted 2025-02-18, last update 2025-02-27]

- JVET-AK2006 Additional SEI messages for VSEI version 4 (Draft 5), also issued as WG 5 preliminary DAM update N 335 [Posted 2025-02-07, last update 2025-03-13]

- JVET-AK2010 VTM and HM common test conditions and software reference configurations for SDR 4:2:0 10 bit video [Posted 2025-03-22]

- JVET-AK2019 Description of algorithms version 10 and software version 12 in neural network-based video coding (NNVC) [Posted 2025-03-25]

- JVET-AK2020 Film grain synthesis technology for video applications ed. 2 (Draft 2) [not available yet by beginning of meeting]

- JVET-AK2023 Exploration experiment on neural network-based video coding (EE1) [Posted 2025-01-22, last update 2025-02-07]

- JVET-AK2024 Exploration experiment on enhanced compression beyond VVC capability (EE2) [Posted 2025-01-22, last update 2025-02-21]

- JVET-AK2025 Algorithm description of Enhanced Compression Model 16 (ECM 16) [Posted 2025-03-17]

- JVET-AK2026 Testing of video coding technology beyond conditions of exploration experiments [Posted 2025-03-03]

- JVET-AK2030 Optimization of encoders and receiving systems for machine analysis of coded video content (draft 8, for future updates) [Posted 2025-03-13]

- JVET-AK2032 Technologies under consideration for future extensions of VSEI (version 7) [Posted 2025-02-28]

- JVET-AK2038 Draft white paper on VSEI [Posted 2025-03-26]

- JVET-AK2039 Announcement of JVET AHG17 Meeting, Aachen (Germany), 10-12 March 2025 [Posted 2025-02-03]

The eighteen ad hoc groups had made progress, and reports from those activities had been submitted. Various teleconferences had been held by AHGs, and a hybrid meeting of AHG17 was conducted during 10-12 March 2025 in Aachen, DE, attended by almost 30 on-site participants. Furthermore, two exploration experiments (EE) on neural network-based video coding and on enhanced compression beyond VVC capability were conducted.

The arrangements for the 38th meeting had been announced in the JVET reflector, in the JVET logistics document (https://www.itu.int/wftp3/av-arch/jvet-site/2025_03_AL_Virtual/JVET-AL_Logistics.docx), and in the WG 5 calling notice (N 341) and agenda (N 342) for the 19th WG 5 meeting.

Software integration was finalized approximately according to the plan.

Various problem reports relating to asserted bugs in the software, draft specification text, and reference encoder description had been submitted to an informal "bug tracking" system. That system is not intended as a replacement of our ordinary contribution submission process. However, the bug tracking system was considered to have been helpful to the software coordinators and text editors. The bug tracker reports had been automatically forwarded to the group email reflector, where the issues were discussed – and this is reported to have been helpful.

Roughly 220 input contributions (not counting the AHG reports, summary reports and crosschecks) had been registered for consideration at the current meeting.

It is further noted that, starting from the twentieth JVET meeting, work items which had originally been conducted by the Joint Collaborative Team on Video Coding (JCT-VC) were continued to be conducted in JVET as a single joint team, as negotiated by the parent bodies. This particularly consists of work on

- High Efficiency Video Coding (HEVC) and its extensions, the development of associated conformance test sets, reference software, verification testing, maintenance, and non-normative guidance information,

- Specification of Coding-independent Code Points (Video) (CICP), and associated technical report(s),

- Maintenance and enhancement work on the Advanced Video Coding (AVC) standard, associated conformance test sets and reference software.

To retain a consistent numbering scheme, the number range of output documents starting from 1001 was reserved for the previous JCT-VC and JCT-3V topic items listed above, whereas the number range starting from 2001 was retained for VVC, VSEI and exploration activities.

A preliminary basis for the document subject allocation and meeting notes for the 38h meeting had been made publicly available on the ITU-hosted ftp site as http://wftp3.itu.int/av-arch/jvet-site/2025_03_AL_Virtual/JVET-AL_notes_d0.docx.

- Recommendations

- The AHG recommends its continuation.

- The AHG recommends continued communication between JVET’s parent bodies about appointment of an extended management support team.

JVET-AL0002 JVET AHG report: Draft text and test model algorithm description editing (AHG2) [B. Bross, C. Rosewarne (co-chairs), F. Bossen, A. Browne, S. Kim, S. Liu, J.-R. Ohm, G. J. Sullivan, A. Tourapis, Y.-K. Wang, Y. Ye (vice chairs)]

  - Output documents produced

    - JVET-AK1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP

This document contains a list of reported errata items for VVC, VSEI, HEVC, AVC, and Video CICP, for tracking purposes. Some of the items have been confirmed by the JVET and have been agreed to require fixing. This document also provides publication status backgrounds of these standards.

Incorporated items at the JVET-AK meeting:

- For VSEI:

  - Removed/updated VSEI errata items that have been fully or partially integrated into JVET-AK2006.

- For HEVC:

  - Added the following item: In H.265v10 published by ITU-T, there’s an extraneous bullet item “k)” in clause A.4.2 immediately above Table A.9, with font size of 4 (thus not easily visible). (reported by Cliff Reader – thanks!) The error is not in the HEVC 6th edition that was in the final stage of the publication process as 2025-03-03.

    - JVET-AK1006 HEVC additional profiles and SEI messages (draft 2)

This document contains the draft text for changes to the High Efficiency Video Coding (HEVC) standard (Rec. ITU-T H.265 | ISO/IEC 23008-2). The changes include the support of a new profile, namely the Multiview Main 10 profile, and the support of five new SEI messages, namely the source picture timing information, modality information, digitally signed content initialization, digitally signed content selection, and digitally signed content verification SEI messages, through referencing the Versatile Supplemental Enhancement Information (VSEI) standard (Rec. ITU-T H.274 | ISO/IEC 23002-7). The changes also include some corrections to the previous version of the HEVC standard.

Changes that have been integrated for the 37th JVET meeting in January 2025:

- JVET-AK0107 AHG9: Modality Information SEI for HEVC

- JVET-AK0168 AHG9: On SPTI SEI message: Robustness and source constant framerate

- JVET-AK0194 AHG9: Digitally Signed Content SEI messages for AVC and HEVC

    - JVET-AK1017 Support for additional SEI messages in AVC (draft 1)

This document contains the draft text for changes to the Advanced Video Coding (AVC) standard (Rec. ITU-T H.264 | ISO/IEC 14496-10). The changes include the support of five new SEI messages, namely the source picture timing information, modality information, digitally signed content initialization, digitally signed content selection, and digitally signed content verification SEI messages, through referencing the Versatile Supplemental Enhancement Information (VSEI) standard (Rec. ITU-T H.274 | ISO/IEC 23002-7).

Changes that have been integrated for the 37th JVET meeting in January 2025:

- JVET-AK0107 AHG9: Modality Information SEI for HEVC

- JVET-AK0168 AHG9: On SPTI SEI message: Robustness and source constant framerate

- JVET-AK0194 AHG9: Digitally Signed Content SEI messages for AVC and HEVC

    - JVET-AK2005 Additions and corrections for VVC version 4 (Draft 11)

This document contains the draft text for changes to the Versatile Video Coding (VVC) standard (Rec. ITU-T H.266 | ISO/IEC 23090-3). The changes include the support of 15 new SEI messages, namely 1) SEI processing order, 2) processing order nesting, 3) encoder optimization information, 4) source picture timing information, 5) object mask information, 6) modality information, 7) text description information, 8) digitally signed content initialization, 9) digitally signed content selection, 10) digitally signed content verification, 11) generative face video, 12) generative face video enhancement, 13) AI usage restrictions, 14) packed regions information, and 15) image format metadata, through referencing the Versatile Supplemental Enhancement Information (VSEI) standard (Rec. ITU-T H.274 | ISO/IEC 23002-7). The changes also include updates to the interface text for support of the two neural-network post-filter (NNPF) SEI messages as well as some corrections to the previous version of the VVC standard.

Changes to be integrated or checked:

- Elements from JVET-AG1004 (errata), and bug fixes from JVET-AH0002 for tickets #1609 (NoBackwardPredFlag derivation ambiguity), #1617 (Not initialized NumCtusInSlice[ 0 ] to 0), #1624 (Incorrect indexing in computation of motion vector offset), #1628 (Derivation of ModeTypeCondition should say “one or more”), #1630 (Missing equations for applying AmvrShift), #1631 (Should “Motion vector storing process for geometric partitioning mode” store HpelIfIdx? BBross: HpekIfIdx should always be written since it might be accessed later so it is definitely missing in "Motion vector storing process for geometric partitioning mode".), #1632 (Incorrect indexing used for choosing matrix intra sample prediction), #1634 (Matrices QStateTransTable,levelScale,AlfFixFiltCoeff,AlfClassToFiltMap are incorrectly transposed).

Changes that have been integrated for the 37th JVET meeting in January 2025:

- JVET-AK0053 AHG9: NNPF interface text in VVC

- JVET-AK0114 AHG9: Updates and suggestion on AI usage restrictions SEI message (add the support of the AI usage restrictions SEI message)

- JVET-AK0140 AHG9: AHG9/AHG8: Showcase for packed regions information SEI (add the support of the packed regions information SEI message)

- JVET-AK0168 AHG9: On SPTI SEI message: Robustness and source constant framerate

- JVET-AK0178 AHG9: Proposed plan for initiating version 5 of VSEI (add the support of the image format metadata SEI message)

- JVET-AK0194 AHG9: Digitally Signed Content SEI messages for AVC and HEVC (inclusion of the VPS NAL unit type to nonVclDigitallySignedNalUnitsList)

- JVET-AK0080 AHG9: Comments on the GFV and GFVE SEI messages (inclusion of subclauses for the use of the GFV and GFVE SEI messages)

- JVET-AK0160 AHG9: On VVC interface of object mask information and annotated regions SEI messages (removal of ConfWinLeftOffset and ConfWinTopOffset from the interface of the annotated regions SEI message and inclusion of a subclause for the use of the OMI SEI message)

- JVET-AK0281 AHG9: On AI usage restrictions SEI message (inclusion of the payload type value of the AI usage restrictions SEI message in the SeiProcessingOrderSeiList)

- JVET-AK0075 AHG9: Additional information on object-based optimization for the encoder optimization information SEI message (inclusion of a subclause "Use of the encoder optimization information SEI message")

    - JVET-AK2006 Additional SEI messages for VSEI version 4 (Draft 5)

This document contains the draft text for changes to the versatile supplemental enhancement information messages for coded video bitstreams (VSEI) standard (Rec. ITU-T H.274 | ISO/IEC 23002-7), to specify 15 additional SEI messages, namely 1) SEI processing order, 2) processing order nesting, 3) encoder optimization information, 4) source picture timing information, 5) object mask information, 6) modality information, 7) text description information, 8) digitally signed content initialization, 9) digitally signed content selection, 10) digitally signed content verification, 11) generative face video, 12) generative face video enhancement, 13) AI usage restrictions, 14) packed regions information, and 15) image format metadata. The changes also include updates to some of the existing SEI messages, including the two neural-network post-filter (NNPF) SEI messages, as well as some corrections to the previous version of the VSEI standard.

Editors’ notes and changes yet to be integrated:

- All automatically generated numbering and cross-reference fields will need to be updated after the changes are integrated into the basis text.Check the use of “pertain”; the word is not used in a similar way anywhere in the existing standard.payloadType values are to be specified the corresponding VVC, HEVC or AVC draft text. The use of prefix SEI NAL unit was enabled for all new SEI messages, whereas the use of suffix SEI NAL unit was enabled for encoder optimization information and object mask information SEI messages, since their content may be determined while the encoder encodes the associated picture.

- (JVET-AH0121 item 5) check/fix the ChromaFormatIdc derivation for the use of the colour transform information SEI message as a part of a processing chain in a similar manner as done for the film grain characteristics SEI message in JVET-AG2027 (to carefully check and communicate offline for correctness, e.g., w.r.t. JVET-AH0047).

- (JVET-AH0121 item 6) check/fix the interface variable derivation for the use of the NNPFC and NNPFA SEI messages as a part of a processing chain in a similar manner as done for the film grain characteristics SEI message in JVET-AG2027 (to carefully check and communicate offline for correctness, e.g., w.r.t. JVET-AH0047).

To be integrated:

- General editorial improvements

  - JVET-AJ0185 Editorial updates for NNPFC, SPO and TDI SEI messages

    - Item 2

Changes that have been integrated:

A description of the contributions reflected in this document relative to JVET-AJ2006:

- Source Picture Timing Information

  - JVET-AK0168	AHG9: On SPTI SEI message: Robustness and source constant framerate

- Object Mask Information

  - JVET-AK0160	AHG9: On VVC interface of object mask information and annotated regions SEI messages

  - JVET-AK0330	AHG9: Fixes to object mask information SEI message

- Generative Face Video

  - JVET-AK0080	AHG9: Comments on the GFV and GFVE SEI messages

  - JVET-AK0164	AHG9: Supplementation of value range definition and editorial bugs fixes on the GFVE pupil position SEI messages

  - JVET-AK0127	AHG9: On miscellaneous aspects of GFV and DSCI SEI messages (Item 1)

  - JVET-AK0128	AHG9: Editorial updates for GFV SEI message

  - JVET-AK0238	AHG9: Semantics fixes for generative face video

  - JVET-AK0239	AHG9: On generative face video enhancement

- SEI Processing Order

  - JVET-AK0055	AHG9: Semantics of the SPO SEI message

  - JVET-AK0205	AHG9: On SEI messages in spoPropertySeiList

  - JVET-AK0333	AHG9: On SPO sub-chain signaling

- NN Post Filter

  - JVET-AK0054	AHG9: Semantics of the NNPFC and NNPFA SEI messages

  - JVET-AK0072	AHG9: Comments on NNPF SEI messages

  - JVET-AK0326	AHG9: Proposed specification of seed value for NNPF

  - JVET-AK0152	AHG9: On Spatial Extrapolation for NNPF

- Digitally Signed Content

  - JVET-AK0109	AHG9: On signaling and constraint for RefDigest in digitally signed content SEI messages

  - JVET-AK0125	AHG9: On presence and persistency of digitally signed content SEI messages

  - JVET-AK0127	AHG9: On miscellaneous aspects of GFV and DSCI SEI messages (Item 2 & 3)

  - JVET-AK0206	AHG9: On digitally signed content SEI messages

  - JVET-AK0287	AHG9: Multilayer digitally signed content authentication SEI messages

- Encoder optimization information

  - JVET-AK0075	AHG9: Additional information on object-based optimization for the encoder optimization information SEI message

- AI usage restrictions

  - JVET-AK0114	AHG9: Updates and suggestion on AI usage restrictions SEI message

  - JVET-AK0281	On AI usage restrictions SEI message

- Image format metadata

  - JVET-AK0178	Move IFM to VSEI-v4

- Packed regions information

  - JVET-AK0140 	Move PRI to VSEI-v4

- Annotated regions

  - JVET-AK0160	AHG9: On VVC interface of object mask information and annotated regions SEI messages

- General editorial improvements

- Errata items 1 to 6 in subclause 3.3 of JVET-AJ1004-v1

Related input contributions

Input contributions were noted as relevant to the work of this ad hoc group:

- JVET-AL0223 AHG2: Corrections and clarifications for profile-related aspects of the draft text for HEVC [G. J. Sullivan (Dolby Labs)]

Remaining VVC spec tickets

Closed since JVET-AK0002 was reported:

- (none)

Carried over (strikethough means addressed in the indicated errata report and can be closed):

- #1609 NoBackwardPredFlag derivation ambiguity (Addressed in JVET-AI1004)

- #1617 Not initialized NumCtusInSlice[0] to 0. (Addressed in JVET-AI1004)

- #1624 Incorrect indexing in computation of motion vector offset. (Addressed in JVET-AI1004)

- #1627 "Decoding process for palette mode" does not say what to do with output samples. (Discussed in JVET-AI1004)

- #1628 Derivation of ModeTypeCondition should say "one or more". (Addressed in JVET-AI1004)

- #1629 mtt_split_cu_vertical_flag context uses undefined variable chType. (Discussed in JVET-AI1004)

- #1630 Missing equations for applying AmvrShift. (Discussed in JVET-AI1004)

- #1631 Should "Motion vector storing process for geometric partitioning mode" store HpelIfIdx? (addressed in JVET-AI1004)

- #1632 Incorrect indexing used for choosing matrix intra sample prediction. (Addressed in JVET-AI1004)

- #1634 Matrices QStateTransTable, levelScale, AlfFixFiltCoeff, AlfClassToFiltMap are incorrectly transposed. (Addressed in JVET-AI1004)

- #1635 Incorrect inference for tu_y_coded_flag. (Discussed in JVET-AI1004)

- #1644 Correction of wrong reference on VVC Annex C.4 regarding DpbOutputInterval constraint. Applicable to and corrected in HEVC as well. (Addressed in JVET-AI1004)

- #1650 Incorrect indexing for the h location component in ALF process. (Discussed in JVET-AJ1004)

- #1651 References to various non-existent syntax elements relating to sps_partition_constraints_override_enabled_flag.

New (since JVET-AK0002 was reported)

- #1652 Incorrect heading style in JVET-S2001-v17.

Remaining HEVC spec tickets

Closed since JVET-AK0002 was reported:

- (none)

Carried over:

- #1427 (8-155) and (8-157) do not seem to be used

- #1491 Duplicate invocation of 9.3.4.3 arithmetic decoding process

- #1498 Typos in the Table 9-43

- #1500 Typo in equation (8-69),(8-70)

- #1504 Small typos in profile_tier_level syntax in tabular form (7.3.3)

- #1505 Misleading bitstream requirement related to EOB NAL unit

- #1507 Duplicate row entries for CU QP delta syntax elements in Table 9-48

- #1520 Some smaller errors in the multiview spec

- #1522 Offset issue in clause 8.5.4.3

- #1644 Correction of wrong reference on VVC Annex C.4 regarding DpbOutputInterval constraint. Filed as VVC ticket but applicable to and corrected in HEVC as well. (Addressed in JVET-AI1004)

New (since JVET-AK0002 was reported):

- (none)

Recommendations

The AHG recommends to:

- Approve JVET-AK1004, JVET-AK1006, JVET-AK1017, JVET-AK2005, and JVET-AK2006 documents as JVET outputs,

- Compare the VVC documents with the VVC software and resolve any discrepancies that may exist, in collaboration with the software AHG,

- Encourage the use of the issue tracker to report issues with the text of both the VVC specification text and the algorithm and encoder description,

- Continue to improve the editorial consistency of VVC text specification and Test Model documents,

- Ensure that, when considering changes to VVC, properly drafted text for addition to the VVC Test Model and/or the VVC specification text is made available in a timely manner,

- Review bug tickets, and other AHG2 related inputs and act on them if found to be necessary.

It was further suggested to move the bug tracking to gitlab for better stability, and keep the old system for archiving.

JVET-AL0003 JVET AHG report: Test model software development (AHG3) [F. Bossen, X. Li, K. Sühring (co-chairs), E. François, Y. He, K. Sharman, V. Seregin, A. Tourapis (vice chairs)]

The software model versions prior to the start of the meeting were:

- VTM 23.8 (Feb. 2025)

- HM-18.0 (Apr. 2023)

- HM-16.21+SCM-8.8 (Mar. 2020)

- SHM 12.4 (Jan. 2018)

- HTM 16.3 (Jul. 2018)

- JM 19.1 (Apr. 2023)

- JSVM 9.19.15

- JMVC 8.5

- 3DV ATM 15.0 (no version history)

- HDRTools 0.24 (March 2023)

Software for MFC and MFCD is only available as published by ITU-T and ISO/IEC. It is planned to create repositories with the latest versions available in ITU-T H.264.2 (02/2016). All development history is lost.

- Software development

Development was continued on the GitLab server, which allows participants to register accounts and use a distributed development workflow based on git.

The server is located at:

https://vcgit.hhi.fraunhofer.de

The registration and development workflow are documented at:

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/wikis/VVC-Software-Development-Workflow

Although the development process is described in the context of the VTM software, it can be applied to all other software projects hosted on the GitLab server as well.

Between the 31st and 32nd meetings, the GitLab server was affected by a huge number of spam issues, which were generated using newly registered accounts. Administrator user account confirmation was enabled to protect the server from spam postings. Proponents should plan for some time for server admins to confirm new accounts.

Between the 37th and 38th JVET meeting extensive renewal work was performed on the build server infrastructure. To finish these works, changes to the build configurations are required for all software projects using build pipelines. The necessary information will be communicated to the software coordinators during or shortly after the 38th JVET meeting.

- VTM related activities

The VTM software can be found at

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/

The software development continued on the GitLab server. VTM version 23.7 was tagged on Jan. 30, 2025. VTM 23.8 was tagged on Feb. 2. VTM 23.9 is expected during the 38th JVET meeting.

VTM 23.7 was tagged on Jan. 30, 2025. Changes include:

- DSC corrections

- SEI related cleanups

- JVET-AJ0234: Allow for patch-wise operation when using spatial extrapolation

- JVET-AJ0128: Adding functionality to po_processing_degree_flag[i] of the SPO SEI message

- Fix for decoder when DSC SEI is not present

- JVET-AJ0131: Add nnpfc_inband_prompt_flag to gate NNPF text prompt

- fix ScalingList for multilayer, when used for only one layer that is not the...

- JVET-AJ0151: Digitally Signed Content SEI messages

- Port ET with millisecond precision (from ECM MR 803)

- JVET-AJ0063: Proposal 1,2,4,5

- JVET-AJ0207: GFV SEI message

- Fix GCC 14 warning

- JVET-AJ0241: Text description SEI purpose for encoder description

- JVET-AJ0129: Add more SEI types to the list of allowed SEI types in SPO SEI

- JVET-AJ0225: Changes to RDO for enhancement layer

- Take bit depth into account when applying temporal prefilter

VTM 23.8 was tagged on Feb. 2, 2025. Changes only include removal of macros from previous meeting cycles.

VTM 23.9 is expected to be tagged during the 38th JVET meeting. Changes are expected to include:

- JVET-AK2006: Updates for SPTI SEI message

- JVET-AH0174/JVET-AJ0073: Add nnpfc_scan_type_idc syntax element in the NNPFC SEI message

- JVET-AK0239: GFVE SEI message

- Merge and move configuration files for RA adaptive resolution

- JVET-AK0181: alternative configurations

- Rename "high performance" configuration folder for better reference in CTC

- JVET-AK0072 item 3: NNPF prompt null string constraint

- fix memory leak

- JVET-AJ0104_AJ0114: Implementation of input prompt and input picture shift in NNFPA SEI message

- fixes for multilayer

- JVET-AJ0048: Update SeiProcessingOrderSeiList

- JVET-AK0075: QP threshold for object-based optimization in EOI SEI

- JVET-AK0072 items 1 and 2: nnpfc_input_pic_filering flag inference and constraint

- Fix the JVET document number reference in the macro

- Update copyright header to include the year 2025

- Check minimum required version of OpenSSL

- JVET-AK0181: Fix to remove enabling of the DMVR encoder control when GOPbasedRPR is enabled

- Add cmake option ENABLE_SEARCH_OPENSSL

- JVET-AK0194 related: add VPS to list of signed non-VCL NAL units

- JVET-AJ0170: Flag to indicate the direction of the signalled source picture intervals

  - CTC Performance

VTM 23.8 shows identical performance compared to VTM 23.6 in SDR CTC.

For the HDR CTCs, coding performance of VTM 23.8 and VTM 23.6 are identical. Encoding/decoding run times are very close.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-23.5 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 103% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 103% | 100% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-23.5 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 102% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 103% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-23.5 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 102% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-23.5 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 103% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

For the high bit depth CTCs, VTM 23.8 also shows no changes in coding performance compared to VTM 23.6.

With VTM 23.8 the “Random access, adaptive resolution” configuration gives slightly better compression that the “Random access” configuration:

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 23.8 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.97% | -1.49% | -1.48% | 98% | 87% |
| Class A2 | -0.53% | 3.35% | 2.41% | 97% | 92% |
| Class B | 0.04% | 0.12% | -0.16% | 101% | 96% |
| Class C | 0.00% | 0.00% | 0.01% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.29% | 0.41% | 0.13% | 99% | 94% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.01% | 0.01% | 0.00% | 102% | 100% |

  - Issues in VTM affecting conformance

The following issues in VTM master branch may affect conformance:

- Missing HLS features (see sections below)

Several merge requests for proposals from the 36th or 37th JVET meeting, mostly related to SEI messages in VSEI are still pending review and/or waiting for resolution of comments and rebasing.

  - Status of implementation of proposals of previous JVET meetings

The following list contains all adoptions of the Q and R meetings that were not marked as merged (or submitted) or specification only change in the software coordinator tracking sheet:

- JVET-Q0112

- JVET-Q0154: Disallow mixing of GDR and IRAP (Disallow mixing of GDR with any non-GDR).

- JVET-Q0164

- JVET-Q0402

- JVET-R0178: Require that when no_aps_constraint_flag is equal to 1, sps_lmcs_enabled_flag and sps_scaling_list_enabled_flag shall be equal to 0

- JVET-R0221

- JVET-R0046: Change the description of the bitstream extraction process per the value of max_tid_il_ref_pics_plus1[ ][ ] (aspect 1.2 per JVET-R0046-v4).

- JVET-R0065: Specify that GDR AUs shall be complete – i.e., all of the layers in the CVS shall have a picture in the AU (as with IRAP AUs).

- JVET-R0191: Update the range value for num_ols_hrd_params_minus1.

- JVET-R0222 aspect 1: Infer vps_max_sublayers_minus1 to be equal to 6 when sps_video_parameter_set_id is equal to 0 (i.e. VPS is not present). The exact editorial expression is at the discretion of the editor.

- JVET-S0196 (JVET-S0144 item 17)

- JVET-S0227 (JVET-S0144 item 22)

- JVET-S0077 (JVET-S0139 item 5)

- JVET-S0174 aspect 2 (JVET-S0139 item 18.b)

- JVET-S0156 aspect 3 (JVET-S0139 item 21)

- JVET-S0139 item 26 (no source listed, text only?)

- JVET-S0188 aspect 1 (JVET-S0139 item 28)

- JVET-S0139 item 40 (item does not exist)

- JVET-S0042 (JVET-S0142 item 1.b)

- JVET-S0174 aspect 1 (JVET S0143 item 19)

- JVET-S0096 aspect 3 (JVET-S0140 item 10)

- JVET-S0096 aspect 4 (JVET-S0140 item 13)

- JVET-S0159 aspect 3 (JVET-S0140 item 16)

- JVET-S0171 (JVET-S0256)

- JVET-S0118 (JVET-S0141 item 7)

- JVET-S0102 (JVET-S0141 item 9.a)

- JVET-S0157 item 2 (JVET-S0141 item 13)

- JVET-S0157 item 4 (JVET-S0141 item 14)

- JVET-S0175 aspect 3 (JVET-S0141 item 16)

- JVET-S0175 aspect 1, 2 (JVET-S0141 item 17)

- JVET-S0175 aspects 4 and 5 (JVET-S0141 item 18)

- JVET-S0175 aspect 6 (JVET-S0141 item 19)

- JVET-S0198/ JVET-S0223 (JVET-S0141 item 24)

- JVET-S0173 aspect 2 (JVET-S0141 item 40.b)

- JVET-S0173 item 1 (JVET-S0141 item 51)

- JVET-S0173 item 3 (JVET-S0141 item 52)

- JVET-S0173 item 5 (JVET-S0141 item 53)

- JVET-S0173 item 6 (JVET-S0141 item 54)

- JVET-S0173 item 4 (JVET-S0141 item 56)

- JVET-S0176 item 4 (JVET-S0141 item 60)

- JVET-S0154 aspect 5 (JVET-S0141 item 68)

- JVET-S0154 aspect 6 (JVET-S0141 item 69)

- JVET-S0154 aspect 8 (JVET-S0141 item 71)

- JVET-S0095 aspect 5 (JVET-S0145 item 5)

- JVET-S0095 aspect 6 (JVET-S0145 item 6)

- JVET-S0100 aspect 1, depends on JVET-R0193 (JVET-S0147 item 2)

- FINB ballot comments

- Make high tier support up to 960.

  - SEI TuC software

Per the decision during the 32nd JVET meeting, an SEI TuC repository was created based on VTM-22.2. The repository is located at:

https://vcgit.hhi.fraunhofer.de/jvet-tuc/VVCSoftware_VTM

VTM-22.2-TuC-4.0 was tagged on Jan. 31, 2025. Changes include:

- JVET-AI0182: Bitdepth range information SEI (review pending)

- Fix typos in constituent rectangles implementation (review pending)

- JVET-AI0181: Support for Display Overlays Information (DOI) SEI (minor comments pending)

It should be noted that some of the technologies for which merge requests were submitted to TuC, got moved into the working draft. The merge requests were closed, when appropriate merge requests for VTM were available.

VTM-22.2-TuC-5.0 is expected to be tagged during the 38th JVET meeting. The following merge requests were submitted:

- JVET-AK0142: "AHG9: Display Rectangles SEI"

- JVET-AK0153: Photosensitive content SEI messages

- JVET-AJ0245: Multi-layer support for Constituent Rectangles SEI

- Draft: JVET_AJ0060: multilayer adaptation of Packed regions information SEI message

- JVET-AI0340: Implementation of AI-restrictions usage SEI message

- HM related activities

There was no new HM version tagged during this meeting cycle.

The following MRs were merged:

- Implement phase indication SEI message (JVET-AE0101)

- Update auto-build from VTM build definition

- Update copyright headers to include 2025

- JVET-AK0107: Modality Information SEI

The following MRs are pending [with status indicated]:

- JVET-AK0194: Digitally Signed Content SEI messages (pending review)

- Add support for NNPF SEI messages (pending review/rebasing)

- JVET-AL0062: AI usage restrictions SEI message (upcoming meeting to be decided)

- JVET-AK2006: Add support for SPTI SEI message (pending review/rebasing)

- JVET-AL0061: Import encoder optimization information SEI Message form VTM (pending review/rebasing)

- Port the Y4M support [one issue remains]

- Mark the current picture as short-term ref (for SCM) [need SCC expert reviewer]

A new version of HM is expected to be tagged during or after the 38th JVET meeting.

The HM SCC (SCM) branch (HM-16.21+SCM-8.8) has not been updated for the recent HM versions. Updating SCM to, for example, HM-18.0+SCM-8.8 should be considered. It may though be helpful to move SCC related functionality into separate source files. Volunteer work towards merging the branches would be appreciated.

As reported in the previous reports, further information on lambda optimisation in HM would be appreciated, including comparison of allocation of bits within the GOP structures between HM and VTM.

Otherwise the HEVC bug tracker lists:

- 44 tickets for “HM”, (most of which were created before 2019; 1 created in last year),

- 1 ticket for “HM RExt” (created in 2020),

- 9 tickets for “HM SCC” (most of which were created before 2018),

- 1 ticket for “RExt Text” (created in 2015),

- 1 ticket for “SCC Text” (created in 2016),

- 8 tickets for text (6 created before 2021; 2 created in the last year),

- 2 tickets for encoder description (created in 2020 and 2014)

Help to address these tickets would be appreciated.

- 360Lib related activities

The latest 360Lib software can be found at https://vcgit.hhi.fraunhofer.de/jvet/360lib/-/tags/360Lib-13.6

The following table is for the projection formats comparison using VTM-23.0 according to 360-degree video CTC (JVET-U2012) compared to that using VTM-22.0 (VTM-22.0 as anchor).

|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class S1 | 0.00% | -0.01% | 0.05% | 0.00% | 0.00% | 0.05% |
| Class S2 | 0.08% | 0.16% | 0.34% | 0.08% | 0.15% | 0.34% |
| Overall | 0.03% | 0.06% | 0.17% | 0.03% | 0.06% | 0.16% |

The following table compares generalized cubemap (GCMP) coding and padded equi-rectangular projection (PERP) coding using VTM-23.0 (PERP as anchor).

|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class S1 | -11.52% | -5.47% | -6.09% | -11.50% | -5.41% | -6.03% |
| Class S2 | -3.65% | 0.98% | 1.48% | -3.62% | 1.08% | 1.56% |
| Overall | -8.37% | -2.89% | -3.06% | -8.35% | -2.82% | -3.00% |

The following tables are for PERP and GCMP coding comparison between VTM-23.0 and HM-16.22 (HM as anchor), respectively.

|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class S1 | -30.91% | -38.82% | -41.24% | -30.91% | -38.88% | -41.24% |
| Class S2 | -36.84% | -37.23% | -39.43% | -36.82% | -37.27% | -39.49% |
| Overall | -33.28% | -38.19% | -40.51% | -33.28% | -38.23% | -40.54% |

|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class S1 | -34.64% | -40.04% | -42.14% | -34.59% | -40.02% | -42.09% |
| Class S2 | -38.34% | -38.18% | -40.06% | -38.32% | -38.19% | -40.11% |
| Overall | -36.12% | -39.30% | -41.31% | -36.08% | -39.29% | -41.30% |

- SCM related activities

There had not been any further developments to SCC’s SCM during this meeting cycle.

- SHM related activities

There had not been any further developments to SHVC’s SHM during this meeting cycle.

- HTM related activities

There had not been any releases of HTM of MV-HEVC and 3D-HEVC.

One merge request is was merged:

- JVET-AE0295: MV Main 10 profile support (waiting for proponent response)

The next release will also include the following changes:

- JVET-Z0209: Early termination during calculating RDcost of depth

- HDRTools related activities

There had not been any further developments to HDRTools during this meeting cycle.

- JM, JSVM, JMVM related activities

There had not been any further developments to JSVM or JMVM during this meeting cycle.

There was no new JM version tagged during this meeting cycle, although merge requests were submitted for SEI messages in VSEI.

The following MRs are pending [with status indicated]:

- JVET-AK0107: Modality Information SEI (pending review)

- JVET-AK2006: NNPFC and NNPFA SEI message (pending review)

- JVET-AL0062: AI usage restrictions SEI message (upcoming meeting to be decided)

- JVET-AK2006: SPTI SEI Message (pending review)

- JVET-AL0061: Encoder Optimization Information SEI Message (pending review)

- JVET-AE0101: implement phase indication SEI message (pending review)

For the previous release of JM only basic testing was performed to confirm that the code still works, but performance testing is pending. Performance changes are unlikely but may occur due to bug fixes. A possible encoder/decoder mismatch was reported privately, but needs more investigation.

- Bug tracking

The bug tracker for VTM and specification text is located at:

https://jvet.hhi.fraunhofer.de/trac/vvc

The bug tracker uses the same accounts as the HM software bug tracker. Users may need to log in again due to the different sub-domain.

https://hevc.hhi.fraunhofer.de/trac/hevc

New account creation was disabled due to a large amount of spam registration. Extensive maintenance of the Trac bug tracker would be required to keep it secure. It should be considered to move bug tracking to GitLab, although no automatic conversion of existing bugs is available.

Bug tracking for HDRTools is located at:

https://gitlab.com/standards/HDRTools/-/issues

Please file all issues related to the VVC reference software and HDRTools into the appropriate bug tracker. Try to provide all the details, which are necessary to reproduce the issue. Patches for solving issues and improving the software are always appreciated.

- CTC alignment and merging

There are currently 8 JVET CTC documents:

JVET-AK2010	VTM/HM 4:2:0 test conditions

JVET-Z2011		VTM/HM HDR test conditions

JVET-AA2018	VTM/HM high bit depth test conditions (without spreadsheet)

JVET-T2013		VTM non-4:2:0 test conditions

JVET-AA1100	HM non-4:2:0 test conditions

JVET-U2012		VTM 360 video test conditions

JVET-AC1009	SHVC test conditions

JVET-AC1015	SCM test conditions

JVET-AE1013		3DV test conditions

Further merging of HM RExt CTC into the appropriate VVC CTC was investigated (non 4:2:0 chroma formats), but proper comparable HM configuration files were not yet available by the beginning of this meeting.

The HM HDR test conditions require additional post-processing stages, due to WPSNR, deltaE100 and PSNRL100 values not being generated directly by the HM encoder. Merging of respective functions from VTM is recommended to streamline the process.

- Guidelines for reference software development

Current versions of the software guidelines are:

- JVET-AC1001 Guidelines for HM-based software development [K. Sühring, F. Bossen, X. Li (software coordinators)]

- JVET-AJ2003 Guidelines for VTM-based software development [F. Bossen, X. Li, K. Sühring]

- Recommendations

The AHG recommends to:

- Continue to develop reference software.

- Improve documentation, especially the software manual.

- Encourage people to test VTM and other reference software more extensively outside of common test conditions.

- Encourage people to report all (potential) bugs that they are finding.

- Encourage people to submit bitstreams/test cases that trigger bugs in VTM and other reference software.

- Encourage people to submit non-normative changes that either reduce encoder run time without significantly sacrificing compression performance or improve compression performance without significantly increasing encoder run time.

- Design and add configuration files to the VTM software for testing of HLS features.

- Review VTM-related contributions and determine whether features should be added (or removed) from the software.

- Continue to investigate the merging of branches.

- Continue to investigate merging of CTC documents.

- Verify correctness of CTC documents and issue updates as appropriate

- Keep common test conditions aligned for the different standards.

Significant work on implementing SEI messages in VTM. Some which were moved from TuC to v4 are still in the TuC branch of the software. Still no responsibility identified for SPO/PON. Revisit.

Also merge requests submitted implementing new SEI messages in HM and JM, no new versions released yet. For implementation on multiview profiles in HM, see further notes under AHG5.

It was further suggested to move the bug tracking to gitlab for better stability, and keep the old system for archiving.

JVET-AL0004 JVET AHG report: Test material and visual assessment (AHG4) [V. Baroncini, T. Suzuki, M. Wien (co-chairs), W. Husak, S. Iwamura, P. de Lagrange, S. Liu, X. Meng, S. Puri, A. Segall, S. Wenger (vice-chairs)]

Activities

  - Testing of video coding technology beyond conditions of exploration experiments

AHG4 has supported the activities of AHG17 on test sequence collection and characterization for the purpose of assessing video coding technology beyond the CTC.

  - Test sequences

The server providing JVET test sequences has been updated. The new site is https://vqa.lfb.rwth-aachen.de. A mirror of this site is available at https://datacloud.hhi.fraunhofer.de with the same login credentials. The directory structure of the previous ftp server has been maintained. The test sequences used for CfP/CTC are available in directory “/ctc”.

Due to copyright restrictions, the JVET database of test sequences is only available to accredited members of JVET (i.e., members of ISO/IEC MPEG and ITU-T VCEG). Members of JVET may contact the JVET chair for login information. It is suggested to update the password for the site after each meeting with the Zoom password of that meeting.

Related contributions

| JVET-AL0042 | AHG4/AHG17: Additional cropped HDR sequences for non-CTC testing | A. Filippov, J. Konieczny, Z. Zhi, V. Rufitskiy, H. Qin, T. Dong, X. Tang (TCL), S. Shen, Y. Qin, Y. Wang, X. Wang, Y. Guan (NERC-DTV) |
| --- | --- | --- |

Recommendations

The AHG recommends:

- To progress the work in AHG17 towards setting up a test set suitable for performance assessment of video compression technology beyond the JVET CTC.

- To consider the input contribution related to AHG4/AHG17 in the development of the draft text for a potential CfE.

- To continue to discuss and to update the non-finalized categories of the verification test plan for multilayer VVC and the subjective quality test plan for FGC, including those which have not been addressed yet.

- To collect volunteers to conduct further verification tests and subjective quality tests.

- To collect volunteers to actively contribute to the verification test development.

- To review the set of available test sequences for the verification tests as well as subjective quality tests and potentially collect more test sequences with a variety of content.

- To continue to collect new test sequences available for JVET with licensing statement.

Multi-layer to be further discussed in context of JVET-AL0289.

For potential multi-view tests, 10 bit stereo material would be needed.

It was suggested to make updates to various CTC documents regarding the location on content only in cases when they are changed on other issues. Mentioning the location in JVET-AL1012 should be sufficient, or refer to that document in CTC documents.

JVET-AL0005 JVET AHG report: Conformance testing (AHG5) [I. Moccagatta (chair), F. Bossen, T. Ikai, S. Iwamura, H.-J. Jhu, K. Kawamura, P. de Lagrange, S. Paluri, K. Sühring, Y. Yu (vice chairs)]

Activities

The AHG communication is conducted through the main JVET reflector, jvet@lists.rwth-aachen.de, with [AHG5] in message headers. However, no correspondence marked as AHG5 was sent between the 36th and 37th meetings.

Timeline

The progress on the Conformance testing specification is proceeding per the timeline below:

- VVCv1 conformance:

  - ISO/IEC FDIS 23090-15 issued from 2021-10 meeting, FDIS registered for formal approval 2022-07-11, FDIS ballot closed 2022-11-04, standard published 2022-11-24

  - H.266.1 V1 Consent 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-05-17, standard published 2022-07-12.

- VVCv2 conformance:

  - ISO/IEC 23090-15/Amd.1 CDAM: 2021-10

  - ISO/IEC 23090-15/Amd.1 DAM: 2022-01

  - DAM ballot closed 2022-11-15

  - ISO/IEC FDIS 23090-15:202x 2nd edition text output of 2023-01, preparation delayed to 2023-09, FDIS ballot opened 2024-04-08, FDIS ballot closed 2024-06-03, published 2024-07-04

  - H.266.1 V2 Consent 2023-07, Last Call began 2023-08-16, Approved 2023-09-13, pre-published 2023-10-06, published 2023-10-19

- VVCv3 conformance:

  - CDAM: 2025-04-15

  - DAM: 2025-07-25

  - FDAM: 2026-02-20

  - IS: 2026-06-30

Status on bitstream submission

The status at the time of preparation of this report is as follows:

- conformance bitstreams for VVC:

  - 104 bitstream categories have been identified

  - At least one bitstream has been submitted in each identified category

  - 283 total bitstreams have been provided, checked, and made available

  - Three streams have been re-generated

  - No changes between 37th and 38th meeting.

- conformance bitstreams for VVC operation range extensions:

  - 57 bitstream categories have been identified

  - 1 bitstream of 1 identified category has been re-generated

  - 128 bitstreams of 57 identified categories have been cross-checked and uploaded.

  - No changes between 37th and 38th meeting.

- additional conformance bitstreams for VVC Multilayer:

  - 3 bitstream categories have been identified

  - At least one bitstream has been submitted in each identified category

  - 7 total bitstreams have been provided, cross-checked, and made available

  - No changes between 37th and 38th meeting.

- conformance bitstreams for new HEVC Multiview profiles

  - 2 HEVC Multiview Extended 10 bitstreams have been provided, cross-checked, and made available, no changes between 37th and 38th meeting

  - 2 HEVC Multiview Extended bitstreams have been provided, cross-checked, and made available, no changes between 37th and 38th meeting

  - 2 HEVC Multiview Main 10 bitstreams have been generated and are in the process of being cross-checked.

Activities and Discussion

The AHG activities are on schedule with the preliminary timeline shown in section 2.

VVC activities:

The 3 re-generated streams are available at ​https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVC2ndEd_regenerated. No change in the other bitstreams and/or packages.

VVC operation range extensions activities:

No change in bitstreams and/or packages, 2nd edition packages are available at https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVC2ndEd/

VVC Multilayer activities:

The streams are available at https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVCMultilayer/. No change in bitstreams and/or packages.

HEVC Multiview supporting extended bit depth activities:

As far as the new HEVC Multiview Main 10 profile in JVET-AJ0213 added at the 36th meeting and in output document AJ1006, two bitstreams have been generated and are in the process of being cross-checked. It has been reported by the cross-checker that the latest HTM can decode them.

No change in bitstreams and/or packages for the 2 HEVC Multiview Extended 10 and the 2 HEVC Multiview Extended bitstreams. The 4 packages are available at https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/HEVCMultiview/ .

No update on the implementation of the HEVC Multiview monochrome profiles (Multiview Monochrome, Multiview Monochrome 10, Multiview Monochrome 12, and Multiview Monochrome 16 profiles, collectively referred to as the multiview format range extensions profiles) in JVET-AH1006 and in WG5 N0281. Last input report was JVET-AJ0214 from 36th meeting. No update on the generation of corresponding conformance streams either.

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de).

The AHG5 chairs and JVET chairs can be reached at jvet-conformance@lists.rwth-aachen.de. Participants should not subscribe to this list but may send emails to it.

Contributions

No contributions.

Ftp site information

The procedure to exchange the bitstream (ftp cite, bitstream files, etc.) is specified in Sec 2 “Procedure” of JVET-R2008. The ftp and http sites for downloading bitstreams are

- VVC:

ftp://ftp3.itu.int/jvet-site/bitstream_exchange/VVC

https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVC/

- VVC operation range extensions:

ftp://ftp3.itu.int/jvet-site/bitstream_exchange/VVCv2

https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVCv2

The ftp site for uploading bitstream file is as follows.

ftp://ftp3.itu.int/jvet-site/dropbox/

(user id: avguest, passwd: Avguest201007)

If using FileZilla, the following configuration is suggested:

In the Filezilla Edit  Settings  Connection menu, it was previously necessary to set the minimum TLS level to 1.0. Since around January 2024, this configuration issue went away, and ITU disabled the use of TLS 1.0 and 1.1 on its servers.

Recommendations

The AHG recommends the following:

- Maintain and update the conformance bitstream database and contribute to report problems in JVET document 1004.

- Continue the generation, cross-checking, and documentation of the conformance streams for the HEVC Multiview profiles supporting extended bit depth and the new HEVC Multiview Main 10 profile.

- Prepare for a new edition of VVC conformance with the new multi-layer streams and the corrections of the existing streams.

Bitstreams available for MV 10, MV extended (8), and MV extended 10. No bitstreams yet on multiview monochrome profiles, no activity on implementation since the 36th meeting. Revisit – plan when to issue an HEVC conformance amendment.

It was suggested to issue CD on VVC conformance 3rd ed., and also have a recommendation to not use OSD editing.

JVET-AL0006 JVET AHG report: ECM software development (AHG6) [V. Seregin (chair), J. Chen, R. Chernyak, F. Le Léannec, K. Zhang (vice-chairs)]

Software development

ECM software repository is located at https://vcgit.hhi.fraunhofer.de/ecm/ECM.

ECM software is based on VTM-10.0 with enabled MCTF including the update from JVET-V0056, and GOP32, which is very close to VTM-11.0.

VTM-11.0ecm anchor https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/VTM11_ANC is used for ECM performance evaluation.

The following changes were integrated into ECM-16.0:

Merge AHG7 branch with tools control [MR 822]

JVET-AK0135: CABAC contexts retraining [MR 819]

JVET-AK0064: LFNST/NSPT set derivation for CCP coded block (Test 3.2) [MR 817]

JVET-AK0081 AHG6: wPSNR log output for gaming content in ECM [MR 821]

JVET-AK0076: Extended OBMC for non-inter blocks (Test 1.1a) [MR 816]

JVET-AK0061: On MPM with matrix-based position dependent intra prediction (Test 1.12c) [MR 814]

JVET-AK0091: Using Laplacian Info in ALF [MR 820]

JVET-AK0065: Temporal ALF [MR 818]

JVET-AK0185: TMVP Selection (Test 2.5) [MR 823]

JVET-AK0121: Boundary-Aware Offset Refinement for Loop-Filters(Test 4.3b) [MR 828]

JVET-AK0101: Regression-based GPM with intra and inter prediction (Test 2.3) [MR 827]

JVET-AK0123: Restrictions on ALF coefficient values (Test 4.1a) [MR 830]

JVET-AK0118: Bilateral Filtering for Intra Prediction(Test 1.10a) [MR 829]

JVET-AK0056: Weighted OBIC (Test 1.9) [MR 815]

JVET-AK0095: Enhanced derivation of affine merge candidates (Test 2.2c) [MR 831]

JVET-AK0187: Implicit MTS Extension (Test 3.8b) [MR 825]

JVET-AK0212: OBMC modifications (Test 2.1a) [MR 833]

JVET-AK0097: Last position signaling (Test 3.4b) [MR 838]

JVET-AK0329: MTSS, combination of tests 3.1b* and 3.7 [MR 836]

JVET-AK0087: 8-tap non-smoothing filter for angular intra prediction (Test EE2-1.11b) [MR 839]

JVET-AK0059: Intra mode coding based on HoG of neighboring templates (Test 1.2c) [MR 832]

JVET-AK0085: TM Boundary Padding (Test 2.6) [MR 844]

Fixes:

Fix: Remove encoder setting based on hash for AJ0260 [MR 834]

Fix clang 16 compiling issues (unused variables) [MR 848]

Convert assert to CHECK [MR 853]

Implementation of Control for JVET-AJ0161 (OBMC Extension with Intra Prediction) [MR 869]

ET with millisecond precision [MR 803]

Store cost for timdSAD [MR 805]

The following changes were integrated into ECM-16.1:

Add missing controls for group5 and group1-5 [MR 883]

Fix MD5sum mismatch with AHG7 group1 off [MR 885]

Fix groupoff4 test [MR 884]

Fix: Change initialization value to 0 for intra mode to be valid in tool off tests for AK0217/AK0187 [MR 886]

Convert some CHECK to CHEKCD [MR 888]

The following changes were integrated into VTM-11.0ecm anchor:

ET with millisecond precision [MR 804]

ECM-16.0 and VTM-11.0ecm16.0 were tagged on February 20, 2025.

ECM-16.1 was tagged on February 28, 2025.

  - CTC Performance

In this section, ECM test results following ECM CTC configuration descried in JVET-AI2017 are summarized.

ECM-16.0 performance over ECM-15.0 anchor is summarized in the tables below.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-15.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.32% | -0.63% | -0.34% | 119.1% | 101.3% | 101.7% | 97.3% |
| Class A2 | -0.36% | -0.54% | -0.61% | 111.3% | 100.9% | 104.4% | 97.3% |
| Class B | -0.39% | -0.67% | -0.80% | 111.2% | 103.3% | 98.8% | 97.3% |
| Class C | -0.65% | -0.70% | -0.74% | 108.2% | 105.6% | 102.3% | 97.0% |
| Class E | -0.66% | -0.71% | -0.65% | 114.4% | 103.3% | 106.0% | 96.9% |
| Overall | -0.47% | -0.66% | -0.65% | 112.3% | 103.1% | 102.2% | 97.2% |
| Class D | -0.62% | -0.84% | -1.03% | 108.1% | 107.8% | 101.8% | 96.9% |
| Class F | -0.45% | -0.60% | -0.66% | 103.7% | 97.1% | 102.5% | 97.2% |
| Class TGM | -0.24% | -0.14% | -0.18% | 102.5% | 87.7% | 102.5% | 97.5% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-15.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.53% | -0.39% | -0.48% | 113.6% | 107.1% | 102.7% | 100.7% |
| Class A2 | -0.57% | -0.65% | -0.51% | 112.9% | 107.9% | 102.0% | 101.2% |
| Class B | -0.82% | -0.63% | -0.58% | 115.9% | 105.3% | 105.6% | 100.5% |
| Class C | -0.58% | -0.54% | -0.64% | 111.7% | 103.5% | 103.9% | 100.3% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.65% | -0.56% | -0.56% | 113.7% | 105.7% | 103.8% | 100.7% |
| Class D | -0.51% | -0.99% | -0.69% | 111.2% | 103.6% | 102.7% | 100.1% |
| Class F | -0.65% | -0.63% | -0.14% | 126.9% | 100.5% | 103.8% | 100.5% |
| Class TGM | -0.39% | 0.05% | 0.03% | 122.4% | 93.9% | 106.9% | 100.6% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-15.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.54% | -0.39% | -0.33% | 112.9% | 104.3% | 107.3% | 99.7% |
| Class C | -0.50% | -0.44% | -0.21% | 108.0% | 101.1% | 104.7% | 98.0% |
| Class E | -0.43% | -1.31% | -1.63% | 137.2% | 103.9% | 109.1% | 99.7% |
| Overall | -0.50% | -0.64% | -0.62% | 116.8% | 103.1% | 106.8% | 99.1% |
| Class D | -0.55% | -0.30% | -0.41% | 108.9% | 101.8% | 103.7% | 97.4% |
| Class F | -0.50% | -0.50% | -0.64% | 121.6% | 106.1% | 106.6% | 98.1% |
| Class TGM | -0.57% | 0.02% | -0.21% | 120.5% | 90.3% | 107.6% | 99.2% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-15.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.62% | -0.28% | -0.34% | 119.5% | 100.7% | 107.6% | 98.0% |
| Class C | -0.48% | 0.23% | -0.83% | 114.3% | 108.7% | 104.5% | 97.3% |
| Class E | -0.71% | 1.17% | 0.89% | 152.0% | 100.0% | 107.0% | 97.3% |
| Overall | -0.60% | 0.25% | -0.20% | 125.1% | 103.1% | 106.4% | 97.6% |
| Class D | -0.68% | -1.65% | -0.55% | 112.1% | 105.4% | 103.5% | 97.0% |
| Class F | -0.22% | -0.40% | 0.27% | 125.1% | 96.2% | 107.3% | 98.4% |
| Class TGM | -0.60% | -0.17% | 0.14% | 127.2% | 98.7% | 106.0% | 100.7% |

ECM-16.1 performance over ECM-15.0 anchor is summarized in the tables below.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-15.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.32% | -0.63% | -0.34% | 105.4% | 99.9% | 101.7% | 97.3% |
| Class A2 | -0.36% | -0.54% | -0.61% | 103.9% | 101.3% | 104.4% | 97.3% |
| Class B | -0.39% | -0.67% | -0.80% | 106.0% | 103.6% | 98.8% | 97.3% |
| Class C | -0.65% | -0.70% | -0.74% | 104.2% | 106.4% | 102.3% | 97.0% |
| Class E | -0.66% | -0.71% | -0.65% | 103.7% | 104.3% | 106.0% | 96.9% |
| Overall | -0.47% | -0.66% | -0.65% | 104.8% | 103.3% | 102.2% | 97.2% |
| Class D | -0.62% | -0.84% | -1.03% | 103.7% | 106.8% | 101.8% | 96.9% |
| Class F | -0.45% | -0.60% | -0.66% | 96.6% | 97.6% | 102.5% | 97.2% |
| Class TGM | -0.24% | -0.14% | -0.18% | 94.2% | 88.5% | 102.5% | 97.5% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-15.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.53% | -0.39% | -0.48% | 104.9% | 105.2% | 102.7% | 100.7% |
| Class A2 | -0.57% | -0.65% | -0.51% | 105.4% | 107.5% | 102.0% | 101.2% |
| Class B | -0.82% | -0.63% | -0.58% | 107.1% | 105.8% | 105.6% | 100.5% |
| Class C | -0.58% | -0.54% | -0.64% | 105.0% | 103.9% | 103.9% | 100.3% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.65% | -0.56% | -0.56% | 105.7% | 105.5% | 103.8% | 100.7% |
| Class D | -0.51% | -0.99% | -0.69% | 104.9% | 103.7% | 102.7% | 100.1% |
| Class F | -0.65% | -0.63% | -0.14% | 105.5% | 102.9% | 103.8% | 100.5% |
| Class TGM | -0.39% | 0.05% | 0.03% | 100.9% | 93.6% | 106.9% | 100.6% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-15.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.54% | -0.39% | -0.33% | 102.9% | 104.9% | 107.3% | 99.7% |
| Class C | -0.50% | -0.44% | -0.21% | 102.1% | 101.6% | 104.7% | 98.0% |
| Class E | -0.43% | -1.31% | -1.63% | 110.6% | 105.1% | 109.1% | 99.7% |
| Overall | -0.50% | -0.64% | -0.62% | 104.5% | 103.8% | 106.8% | 99.1% |
| Class D | -0.55% | -0.30% | -0.41% | 101.7% | 102.6% | 103.7% | 97.4% |
| Class F | -0.50% | -0.50% | -0.64% | 102.7% | 102.6% | 106.6% | 98.1% |
| Class TGM | -0.57% | 0.02% | -0.21% | 99.6% | 88.4% | 107.6% | 99.2% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-15.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.62% | -0.28% | -0.34% | 103.7% | 104.3% | 107.6% | 98.0% |
| Class C | -0.48% | 0.23% | -0.83% | 105.5% | 109.9% | 104.5% | 97.3% |
| Class E | -0.71% | 1.17% | 0.89% | 111.6% | 100.9% | 107.0% | 97.3% |
| Overall | -0.60% | 0.25% | -0.20% | 106.3% | 105.3% | 106.4% | 97.6% |
| Class D | -0.68% | -1.65% | -0.55% | 102.1% | 104.6% | 103.5% | 97.0% |
| Class F | -0.22% | -0.40% | 0.27% | 102.7% | 99.5% | 107.3% | 98.4% |
| Class TGM | -0.60% | -0.17% | 0.14% | 98.2% | 97.9% | 106.0% | 100.7% |

The below tables show ECM-16.1 performance comparing to VTM-11.0ecm16.0 anchor.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0ecm16.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -14.59% | -15.70% | -26.79% | 1197.4% | 527.4% |  |  |
| Class A2 | -21.11% | -23.87% | -28.39% | 1175.6% | 576.4% |  |  |
| Class B | -14.71% | -22.37% | -20.35% | 1115.0% | 622.9% |  |  |
| Class C | -14.85% | -11.82% | -12.99% | 1050.3% | 586.3% |  |  |
| Class E | -19.17% | -22.39% | -20.59% | 1033.0% | 649.6% |  |  |
| Overall | -16.53% | -19.17% | -21.17% | 1109.1% | 594.2% |  |  |
| Class D | -12.71% | -8.95% | -9.88% | 1027.3% | 624.9% |  |  |
| Class F | -30.26% | -33.90% | -34.10% | 719.4% | 656.4% |  |  |
| Class TGM | -43.26% | -48.84% | -48.12% | 543.2% | 622.8% |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over VTM-11.0ecm16.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -27.30% | -23.64% | -35.77% | 1204.6% | 1070.0% |  |  |
| Class A2 | -30.50% | -33.70% | -39.05% | 1162.8% | 1307.0% |  |  |
| Class B | -25.08% | -31.91% | -29.21% | 999.8% | 1139.8% |  |  |
| Class C | -26.76% | -22.11% | -22.88% | 1053.2% | 1226.1% |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall | -27.06% | -28.00% | -30.80% | 1084.5% | 1179.5% |  |  |
| Class D | -27.52% | -22.95% | -23.94% | 995.5% | 1342.6% |  |  |
| Class F | -33.16% | -35.27% | -35.81% | 918.3% | 851.5% |  |  |
| Class TGM | -42.58% | -47.84% | -47.64% | 742.3% | 613.8% |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over VTM-11.0ecm16.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -22.10% | -35.80% | -32.15% | 992.7% | 940.5% |  |  |
| Class C | -24.50% | -24.84% | -26.38% | 936.2% | 984.3% |  |  |
| Class E | -21.74% | -26.55% | -25.95% | 952.2% | 623.4% |  |  |
| Overall | -22.81% | -29.83% | -28.68% | 963.4% | 861.6% |  |  |
| Class D | -25.88% | -25.77% | -26.68% | 949.4% | 1139.8% |  |  |
| Class F | -30.60% | -38.50% | -38.39% | 846.9% | 748.6% |  |  |
| Class TGM | -40.89% | -50.16% | -50.14% | 737.2% | 550.1% |  |  |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over VTM-11.0ecm16.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -19.96% | -44.71% | -41.93% | 867.7% | 913.2% |  |  |
| Class C | -22.51% | -33.82% | -35.20% | 816.9% | 984.3% |  |  |
| Class E | -20.46% | -34.86% | -35.18% | 869.3% | 633.6% |  |  |
| Overall | -20.93% | -38.62% | -38.00% | 850.8% | 854.5% |  |  |
| Class D | -25.29% | -37.55% | -37.81% | 800.0% | 1061.4% |  |  |
| Class F | -28.56% | -43.86% | -44.00% | 837.7% | 739.5% |  |  |
| Class TGM | -39.05% | -52.66% | -52.08% | 776.3% | 600.3% |  |  |

ECM memory consumption

ECM encoder memory consumption (VmPeak, GiB) is provided in ECM encoder log files and is summarized in the table below as maximum class-wise consumption rounded up to GiB.

|  | AI | RA | LB |
| --- | --- | --- | --- |
| class A (A1& A2) | 9 | 15 |  |
| class B | 4 | 6 | 5 |
| class C | 2 | 3 | 3 |
| class D | 2 | 2 | 2 |
| class E | 3 |  | 3 |
| class F | 4 | 6 | 5 |
| class TGM | 3 | 8 | 6 |

It is encouraged to care about memory allocation when developing and integrating tools into ECM. In particular, it is strongly recommended to re-use already existed memory wherever possible, rather than systematically allocating new memory, and allocate only the required amount of memory if new memory is needed.

Recommendations

The AHG recommends to:

- Continue to develop ECM software.

- Improve the software documentation.

- Encourage people to report all (potential) bugs that they are finding using GitLab Issues functionality https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/issues.

- Encourage people to submit merge requests fixing identified bugs.

- Encourage people to continue working on ECM memory consumption reduction.

- Encourage people to continue working on speeding up ECM encoder to reduce the simulation time.

JVET-AL0007 JVET AHG report: ECM tool assessment (AHG7) [X. Li (chair), L.-F. Chen, Z. Deng, J. Gan, E. François, R. Ishimoto, H.-J. Jhu, J. Lainema, X. Li, J. Pardo, A. Stein, H. Wang (vice chairs)]

Group off tests

  - Test settings and crosschecking

The following five groups of tools were defined.

- Group 1: Inter template matching tools

- Group 2: Coding tools that interleave the (merge/skip/AMVP/subblock/IBC/etc) list derivation with the intra prediction/reconstruction process

- Group 3: Intra and IBC template matching (with search) related tools

- Group 4: Tools that require more processing on the neighboring reconstructed samples than VVC

- Group 5: Tools with large memory access

The testers and crosscheckers are planed in the table below. However, a crosschecker has a computing resource issue. Another crosschecker ran the test instead but could not achieve full results before the meeting.

| Tests | Testers / Crosscheckers | Testers / Crosscheckers |
| --- | --- | --- |
| Group 1 off | Johan Pardo (johan.esprit.pardo1@huawei.com) | Charles Salmon-Legagneur (charles.salmon-legagneur@interdigital.com) |
| Group 2 off | Jonathan Gan (v-jonathan.gan@oppo.com) | Xinwei Li (sid.lxw@alibaba-inc.com) |
| Group 3 off | Ryo Ishimoto (ishimoto.ryo@mail.sharp) | Zhipin Deng (zhipin.deng@bytedance.com) |
| Group 4 off | Hong-Jheng Jhu (jhuhong-jheng@kwai.com) | Alan Stein (alan.stein@v-nova.com) |
| Group 5 off | Lien-Fei Chen (lienfei.chen@global.tencent.com) | Jani Lainema (jani.lainema@nokia.com) |
| Group 1-5 off | Xiang Li (xlxiangli@google.com) | Hongtao Wang (hongtaow@qti.qualcomm.com) |

ECM-16.0 and ECM-16.1 were used in the AHG7 tool off tests, as summarized in the table below. Note that encoding/decoding issues were observed for group1, group4 and group5 off. Those issues were fixed in ECM-16.1. The cfg files used are included in the ECM software package.

| Tests | Testers / Crosscheckers | Testers / Crosscheckers |
| --- | --- | --- |
| Group 1 off | ECM-16.1 | ECM-16.1 |
| Group 2 off | ECM-16.0 | ECM-16.0 |
| Group 3 off | ECM-16.0 | ECM-16.0 |
| Group 4 off | ECM-16.1 | ECM-16.1 |
| Group 5 off | ECM-16.1 | ECM-16.1 |
| Group 1-5 off | ECM-16.0 (with updated cfg) | ECM-16.0 (with updated cfg) |

  - Group 1 off

Group 1 includes inter template matching tools. The offgroup1.cfg was used in addition to ECM CTC settings.

|  |  |  |  |  |  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Over ECM-16.1 |  |  |  | Over VTM-11ecm16 |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101.4% | 100.5% | #DIV/0! | -14.59% | -15.70% | -26.79% | 1179.1% | 600.5% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100.7% | 100.4% | #DIV/0! | -21.11% | -23.87% | -28.39% | 1159.8% | 640.9% |
| Class B | 0.00% | -0.01% | -0.01% | 99.9% | 97.4% | #DIV/0! | -14.72% | -22.37% | -20.35% | 1080.0% | 659.8% |
| Class C | -0.01% | -0.01% | -0.01% | 100.0% | 99.9% | #DIV/0! | -14.85% | -11.82% | -13.00% | 1077.6% | 618.8% |
| Class E | -0.01% | -0.01% | -0.01% | 98.0% | 99.4% | #DIV/0! | -19.18% | -22.40% | -20.60% | 991.2% | 688.4% |
| Overall | -0.01% | -0.01% | -0.01% | 100.0% | 99.3% | #DIV/0! | -16.54% | -19.17% | -21.17% | 1092.7% | 641.8% |
| Class D | -0.03% | -0.02% | -0.03% | 99.3% | 98.4% | #DIV/0! | -12.73% | -8.97% | -9.90% | 1030.3% | 649.7% |
| Class F | -0.01% | -0.01% | -0.01% | 97.5% | 96.2% | #DIV/0! | -30.27% | -33.90% | -34.11% | 639.3% | 644.2% |
| Class TGM | -0.01% | -0.01% | -0.01% | 99.8% | 99.1% | #DIV/0! | -43.26% | -48.85% | -48.12% | 544.2% | 654.9% |
|  |  |  |  |  |  | Random Access Main 10 |  |  |  |  |  |
|  |  |  |  | Over ECM-16.1 |  |  |  | Over VTM-11ecm16 |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 4.87% | 4.62% | 5.07% | 85.1% | 74.0% | #DIV/0! | -23.73% | -20.29% | -32.35% | 1010.2% | 948.4% |
| Class A2 | 5.75% | 5.74% | 5.95% | 85.7% | 67.8% | #DIV/0! | -26.50% | -29.96% | -35.41% | 965.8% | 1017.3% |
| Class B | 4.62% | 4.78% | 4.71% | 81.3% | 67.4% | #DIV/0! | -21.58% | -28.61% | -25.84% | 790.3% | 918.4% |
| Class C | 4.93% | 5.01% | 5.46% | 77.0% | 63.6% | #DIV/0! | -23.11% | -18.20% | -18.65% | 788.7% | 884.8% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 4.98% | 5.00% | 5.23% | 81.7% | 67.7% | #DIV/0! | -23.40% | -24.44% | -27.14% | 863.5% | 934.1% |
| Class D | 4.03% | 4.16% | 4.30% | 75.3% | 62.8% | #DIV/0! | -24.53% | -19.69% | -20.62% | 729.1% | 926.2% |
| Class F | 3.77% | 3.85% | 4.02% | 87.9% | 73.5% | #DIV/0! | -30.54% | -32.64% | -33.13% | 736.3% | 656.3% |
| Class TGM | 4.59% | 4.43% | 4.36% | 89.8% | 80.2% | #DIV/0! | -39.94% | -45.47% | -45.30% | 648.9% | 617.5% |
|  |  |  |  |  |  | Low delay B Main10 |  |  |  |  |  |
|  |  |  |  | Over ECM-16.1 |  |  |  | Over VTM-11ecm16 |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 5.89% | 6.28% | 6.05% | 68.6% | 58.3% | #DIV/0! | -17.48% | -31.87% | -28.17% | 672.1% | 666.5% |
| Class C | 5.74% | 5.52% | 6.35% | 68.6% | 54.1% | #DIV/0! | -20.16% | -20.72% | -21.71% | 634.7% | 620.4% |
| Class E | 6.28% | 7.27% | 7.10% | 73.3% | 67.3% | #DIV/0! | -16.82% | -21.31% | -20.74% | 692.4% | 558.8% |
| Overall (Ref) | 5.94% | 6.28% | 6.41% | 69.7% | 58.9% | #DIV/0! | -18.21% | -25.51% | -24.16% | 664.3% | 622.7% |
| Class D | 5.20% | 5.27% | 5.26% | 69.0% | 54.9% | #DIV/0! | -22.00% | -21.84% | -22.86% | 638.1% | 665.5% |
| Class F | 5.63% | 5.52% | 4.08% | 82.7% | 67.4% | #DIV/0! | -26.61% | -35.08% | -35.67% | 669.5% | 535.1% |
| Class TGM | 6.62% | 6.46% | 6.41% | 84.6% | 68.5% | #DIV/0! | -36.91% | -46.76% | -46.68% | 620.7% | 480.8% |
|  |  |  |  |  |  | Low delay P Main10 |  |  |  |  |  |
|  |  |  |  | Over ECM-16.1 |  |  |  | Over VTM-11ecm16 |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 4.13% | 4.83% | 4.92% | 78.6% | 61.9% | #DIV/0! | -16.62% | -42.19% | -39.25% | 662.1% | 645.8% |
| Class C | 3.99% | 3.73% | 4.96% | 77.8% | 53.6% | #DIV/0! | -19.39% | -31.33% | -31.88% | 598.4% | 579.8% |
| Class E | 4.98% | 4.16% | 4.10% | 84.8% | 70.7% | #DIV/0! | -16.46% | -32.21% | -32.65% | 681.6% | 524.5% |
| Overall (Ref) | 4.30% | 4.29% | 4.73% | 79.8% | 61.0% | #DIV/0! | -17.50% | -36.08% | -35.14% | 644.8% | 591.4% |
| Class D | 3.93% | 4.23% | 5.40% | 75.1% | 53.5% | #DIV/0! | -22.26% | -34.80% | -34.28% | 570.8% | 607.0% |
| Class F | 4.38% | 3.17% | 3.11% | 90.0% | 68.8% | #DIV/0! | -25.36% | -41.94% | -41.99% | 679.2% | 530.8% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

  - Group 2 off

Group 2 includes coding tools that interleave the (merge/skip/AMVP/subblock/IBC/etc) list derivation with the intra prediction/reconstruction process. The offgroup2.cfg was used in addition to ECM CTC settings.

|  |  |  |  |  |  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Over ECM16 |  |  |  | Over VTM11ecm16 |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 0.15% | 0.46% | 0.45% | 94.4% | 96.5% | 100% | -14.46% | -15.33% | -26.43% | 1221.8% | 551.3% |
| Class A2 | 0.25% | 0.41% | 0.39% | 93.8% | 97.7% | 100% | -20.90% | -23.58% | -28.12% | 1137.3% | 579.1% |
| Class B | 0.20% | 0.49% | 0.51% | 92.1% | 95.8% | 96% | -14.54% | -22.02% | -19.93% | 1071.0% | 612.8% |
| Class C | 0.14% | 0.37% | 0.43% | 91.8% | 94.9% | 100% | -14.72% | -11.49% | -12.60% | 1038.8% | 571.3% |
| Class E | 0.31% | 0.47% | 0.56% | 93.4% | 96.5% | 99% | -18.91% | -22.04% | -20.14% | 1031.1% | 653.4% |
| Overall | 0.21% | 0.44% | 0.47% | 92.9% | 96.1% | 99% | -16.36% | -18.83% | -20.79% | 1091.4% | 593.5% |
| Class D | 0.08% | 0.42% | 0.51% | 90.6% | 91.6% | 100% | -12.63% | -8.58% | -9.41% | 1005.8% | 612.1% |
| Class F | 0.58% | 0.62% | 0.80% | 92.6% | 97.3% | 98% | -29.88% | -33.48% | -33.60% | 610.8% | 636.2% |
| Class TGM | 0.44% | 0.51% | 0.50% | 93.4% | 99.7% | 100% | -43.00% | -48.58% | -47.86% | 519.8% | 638.1% |
|  |  |  |  |  |  | Random Access Main 10 |  |  |  |  |  |
|  |  |  |  | Over ECM16 |  |  |  | Over VTM11ecm16 |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 2.36% | 1.28% | 2.05% | 88.8% | 98.3% | 100% | -25.59% | -22.60% | -34.48% | 1080.7% | 1168.3% |
| Class A2 | 1.39% | 1.11% | 1.64% | 88.0% | 98.2% | 100% | -29.52% | -32.96% | -38.02% | 1034.4% | 1470.1% |
| Class B | 1.45% | 0.90% | 1.20% | 88.3% | 97.5% | 99% | -23.98% | -31.28% | -28.28% | 911.9% | 1246.2% |
| Class C | 1.00% | 1.14% | 1.08% | 87.0% | 94.8% | 98% | -26.04% | -21.22% | -22.06% | 960.9% | 1311.5% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 1.50% | 1.08% | 1.42% | 88.0% | 97.1% | 99% | -25.96% | -27.20% | -29.81% | 981.1% | 1289.0% |
| Class D | 0.48% | 0.36% | 0.71% | 85.7% | 95.2% | 99% | -27.16% | -22.64% | -23.38% | 924.7% | 1530.4% |
| Class F | 1.96% | 2.25% | 1.64% | 90.3% | 95.2% | 99% | -31.85% | -33.83% | -34.69% | 818.7% | 804.9% |
| Class TGM | 3.39% | 5.38% | 5.56% | 92.2% | 99.6% | 99% | -40.76% | -45.32% | -45.05% | 732.5% | 711.4% |
|  |  |  |  |  |  | Low delay B Main10 |  |  |  |  |  |
|  |  |  |  | Over ECM16 |  |  |  | Over VTM11ecm16 |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 1.81% | 3.71% | 3.45% | 82.1% | 95.0% | 99% | -20.69% | -33.49% | -29.90% | 850.2% | 1043.4% |
| Class C | 1.58% | 2.63% | 3.64% | 82.8% | 90.7% | 99% | -23.36% | -22.92% | -23.73% | 823.0% | 1033.0% |
| Class E | 1.26% | 2.99% | 4.56% | 85.7% | 93.3% | 99% | -20.74% | -24.44% | -22.50% | 879.4% | 707.0% |
| Overall (Ref) | 1.60% | 3.17% | 3.79% | 83.2% | 93.1% | 99% | -21.59% | -27.70% | -26.00% | 848.2% | 943.5% |
| Class D | 0.85% | 2.78% | 3.68% | 78.9% | 92.1% | 98% | -25.22% | -23.65% | -23.90% | 823.1% | 1274.3% |
| Class F | 2.51% | 2.97% | 3.69% | 87.1% | 93.9% | 99% | -28.89% | -36.78% | -36.14% | 783.8% | 765.2% |
| Class TGM | 2.57% | 7.47% | 8.76% | 90.2% | 95.7% | 101% | -39.49% | -46.78% | -46.42% | 710.4% | 653.8% |

  - Group 3 off

Group 3 includes intra and IBC template matching (with search) related tools. The offgroup3.cfg was used in addition to ECM CTC settings.

|  |  |  |  |  |  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Over ECM16 |  |  |  | Over VTM11ecm16 |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 1.14% | 3.90% | 6.19% | 72.7% | 90.4% | 100% | -13.61% | -12.68% | -22.69% | 923.7% | 508.9% |
| Class A2 | 2.46% | 6.29% | 6.16% | 71.3% | 79.9% | 100% | -19.11% | -19.44% | -24.43% | 857.9% | 482.3% |
| Class B | 2.46% | 5.08% | 5.34% | 69.4% | 79.4% | 100% | -12.64% | -18.82% | -16.34% | 783.7% | 504.5% |
| Class C | 2.20% | 2.80% | 3.16% | 68.9% | 79.0% | 100% | -13.01% | -9.44% | -10.32% | 756.3% | 476.0% |
| Class E | 3.46% | 4.93% | 4.57% | 70.7% | 78.8% | 100% | -16.37% | -18.71% | -17.00% | 756.9% | 525.0% |
| Overall | 2.35% | 4.55% | 5.00% | 70.4% | 81.0% | 100% | -14.58% | -15.80% | -17.52% | 806.6% | 498.3% |
| Class D | 1.44% | 2.31% | 2.62% | 69.4% | 79.9% | 100% | -11.45% | -6.88% | -7.52% | 738.1% | 524.6% |
| Class F | 6.31% | 7.16% | 7.45% | 72.0% | 70.4% | 99% | -26.06% | -29.54% | -29.53% | 486.3% | 436.5% |
| Class TGM | 11.70% | 13.57% | 13.98% | 76.4% | 57.8% | 100% | -36.77% | -42.07% | -41.07% | 433.3% | 368.2% |
|  |  |  |  |  |  | Random Access Main 10 |  |  |  |  |  |
|  |  |  |  | Over ECM16 |  |  |  | Over VTM11ecm16 |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 1.09% | 2.84% | 6.70% | 92.2% | 98.5% | 100% | -26.52% | -21.56% | -32.29% | 1065.5% | 1117.1% |
| Class A2 | 1.52% | 4.01% | 3.84% | 93.1% | 98.0% | 100% | -29.45% | -31.16% | -36.76% | 1037.0% | 1347.2% |
| Class B | 1.27% | 4.70% | 4.73% | 90.5% | 96.6% | 100% | -24.13% | -28.75% | -25.90% | 880.9% | 1180.7% |
| Class C | 0.92% | 1.42% | 1.89% | 93.1% | 95.1% | 100% | -26.08% | -20.98% | -21.38% | 949.2% | 1320.7% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 1.19% | 3.31% | 4.19% | 92.0% | 96.9% | 100% | -26.19% | -25.72% | -28.15% | 964.4% | 1235.3% |
| Class D | 0.78% | 2.09% | 2.21% | 94.3% | 101.5% | 100% | -26.96% | -21.30% | -22.22% | 891.4% | 1468.4% |
| Class F | 4.27% | 5.42% | 4.99% | 90.2% | 89.5% | 100% | -30.52% | -32.22% | -32.95% | 791.5% | 732.1% |
| Class TGM | 7.20% | 8.92% | 9.40% | 93.8% | 94.9% | 100% | -38.46% | -43.21% | -42.75% | 748.6% | 652.4% |
|  |  |  |  |  |  | Low delay B Main10 |  |  |  |  |  |
|  |  |  |  | Over ECM16 |  |  |  | Over VTM11ecm16 |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.49% | 1.62% | 1.99% | 93.9% | 100.9% | 100% | -21.73% | -34.84% | -31.00% | 903.3% | 1014.7% |
| Class C | 0.51% | 0.84% | 1.13% | 94.6% | 96.4% | 100% | -24.12% | -24.21% | -25.56% | 829.8% | 1060.0% |
| Class E | 0.86% | 2.72% | 4.22% | 95.7% | 94.1% | 100% | -21.05% | -24.60% | -22.67% | 901.1% | 663.1% |
| Overall (Ref) | 0.59% | 1.64% | 2.26% | 94.6% | 97.7% | 100% | -22.35% | -28.74% | -27.10% | 877.6% | 925.7% |
| Class D | 0.33% | 0.10% | 1.35% | 95.9% | 104.2% | 100% | -25.63% | -25.75% | -25.64% | 879.3% | 1203.6% |
| Class F | 3.49% | 5.01% | 5.06% | 92.0% | 97.9% | 100% | -28.32% | -35.78% | -35.55% | 818.3% | 744.6% |
| Class TGM | 4.94% | 5.94% | 6.54% | 91.9% | 96.7% | 100% | -38.00% | -47.35% | -47.05% | 742.7% | 617.5% |

  - Group 4 off

Group 4 includes tools that require more processing on the neighboring reconstructed samples than VVC.

The offgroup4.cfg was used in addition to ECM CTC settings.

|  |  |  |  |  |  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Over ECM-16.1 |  |  |  | Over VTM-11-ECM16 |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 1.67% | 7.00% | 9.34% | 85.0% | 86.4% | 100% | -13.17% | -10.24% | -20.41% | 725.6% | 317.9% |
| Class A2 | 2.59% | 7.03% | 8.34% | 81.3% | 90.6% | 100% | -19.16% | -18.88% | -23.08% | 710.8% | 360.4% |
| Class B | 1.04% | 7.49% | 6.42% | 89.0% | 93.7% | 99% | -13.83% | -17.28% | -15.53% | 913.7% | 542.3% |
| Class C | 1.01% | 3.54% | 3.54% | 83.6% | 92.2% | 100% | -14.01% | -8.81% | -9.98% | 864.0% | 545.4% |
| Class E | 1.43% | 7.39% | 5.94% | 86.2% | 96.5% | 100% | -18.03% | -17.09% | -16.01% | 843.5% | 572.3% |
| Overall | 1.46% | 6.44% | 6.51% | 85.3% | 92.1% | 100% | -15.35% | -14.46% | -16.45% | 821.8% | 468.2% |
| Class D | 0.91% | 3.25% | 3.27% | 84.0% | 97.1% | 100% | -11.91% | -6.03% | -6.91% | 843.9% | 547.0% |
| Class F | 1.29% | 4.13% | 4.03% | 90.5% | 95.7% | 99% | -29.38% | -31.30% | -31.51% | 501.1% | 544.3% |
| Class TGM | 2.22% | 3.96% | 4.15% | 90.3% | 99.6% | 100% | -42.06% | -46.91% | -46.07% | 450.7% | 569.3% |
|  |  |  |  |  |  | Random Access Main 10 |  |  |  |  |  |
|  |  |  |  | Over ECM-16.1 |  |  |  | Over VTM-11-ECM16 |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 1.67% | 3.87% | 6.29% | 91.1% | 100.0% | 100% | -26.10% | -20.82% | -32.23% | 777.0% | 686.5% |
| Class A2 | 1.26% | 3.83% | 4.20% | 92.5% | 101.2% | 100% | -29.62% | -31.26% | -36.58% | 747.8% | 840.9% |
| Class B | 1.04% | 6.22% | 5.93% | 94.3% | 97.9% | 100% | -24.30% | -27.86% | -25.22% | 837.9% | 996.7% |
| Class C | 1.12% | 2.00% | 2.35% | 93.4% | 100.3% | 100% | -25.95% | -20.55% | -21.06% | 885.9% | 1179.5% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 1.23% | 4.15% | 4.70% | 93.1% | 99.6% | 100% | -26.16% | -25.18% | -27.79% | 818.9% | 935.2% |
| Class D | 0.88% | 2.60% | 2.05% | 93.0% | 95.0% | 100% | -26.86% | -20.98% | -22.42% | 855.7% | 1164.0% |
| Class F | 1.33% | 3.04% | 3.01% | 97.2% | 101.2% | 100% | -32.35% | -33.53% | -33.99% | 734.4% | 699.0% |
| Class TGM | 0.83% | 1.54% | 1.60% | 94.8% | 103.1% | 99% | -42.13% | -47.05% | -46.83% | 636.3% | 590.5% |
|  |  |  |  |  |  | Low delay B Main10 |  |  |  |  |  |
|  |  |  |  | Over ECM-16.1 |  |  |  | Over VTM-11-ECM16 |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 1.07% | 1.54% | 1.52% | 97.1% | 99.0% | 100% | -21.27% | -34.90% | -31.34% | 886.9% | 838.9% |
| Class C | 1.31% | 0.90% | 0.85% | 98.0% | 97.7% | 99% | -23.52% | -24.20% | -25.78% | 855.7% | 921.4% |
| Class E | 0.99% | 4.86% | 4.08% | 99.2% | 101.9% | 99% | -20.96% | -23.14% | -22.84% | 826.0% | 578.2% |
| Overall (Ref) | 1.13% | 2.16% | 1.93% | 97.9% | 99.3% | 100% | -21.94% | -28.40% | -27.36% | 861.0% | 788.6% |
| Class D | 0.95% | -0.18% | 0.12% | 98.6% | 98.7% | 99% | -25.12% | -25.94% | -26.62% | 842.9% | 1027.4% |
| Class F | 1.22% | 2.64% | 0.96% | 96.8% | 96.7% | 100% | -29.78% | -37.06% | -37.84% | 691.8% | 586.9% |
| Class TGM | 0.51% | 1.28% | 1.51% | 97.7% | 103.6% | 100% | -40.60% | -49.51% | -49.41% | 671.4% | 531.0% |

  - Group 5 off

Group 5 includes tools which need large memory access.

The offgroup5.cfg was used in addition to ECM CTC settings.

|  |  |  |  |  |  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Over ECM-16.1 |  |  |  | Over VTM-11.0ecm16.0 |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 4.43% | 5.07% | 4.83% | 79.0% | 82.8% | 90% | -10.76% | -11.45% | -23.26% | 806.3% | 401.5% |
| Class A2 | 3.60% | 3.45% | 4.04% | 81.0% | 87.7% | 90% | -18.24% | -21.35% | -25.68% | 809.6% | 473.0% |
| Class B | 4.19% | 4.18% | 4.69% | 80.5% | 86.6% | 79% | -11.13% | -19.28% | -16.65% | 773.0% | 494.8% |
| Class C | 6.04% | 4.42% | 5.17% | 79.9% | 88.7% | 91% | -9.86% | -8.05% | -8.68% | 766.9% | 493.2% |
| Class E | 6.01% | 5.43% | 4.20% | 81.0% | 86.9% | 87% | -14.31% | -18.31% | -17.28% | 729.5% | 528.3% |
| Overall | 4.84% | 4.47% | 4.63% | 80.3% | 86.6% | 87% | -12.50% | -15.66% | -17.59% | 775.6% | 479.2% |
| Class D | 4.90% | 4.63% | 5.41% | 78.7% | 87.8% | 97% | -8.43% | -4.72% | -4.97% | 731.5% | 514.8% |
| Class F | 4.19% | 2.63% | 3.40% | 87.6% | 87.2% | 88% | -27.16% | -31.86% | -31.64% | 566.6% | 522.3% |
| Class TGM | 0.54% | 0.63% | 0.60% | 88.2% | 88.9% | 81% | -42.95% | -48.53% | -47.83% | 473.5% | 555.8% |
|  |  |  |  |  |  | Random Access Main 10 |  |  |  |  |  |
|  |  |  |  | Over ECM-16.1 |  |  |  | Over VTM-11.0ecm16.0 |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 3.88% | 3.35% | 4.32% | 92.8% | 91.0% | 97% | -24.50% | -21.05% | -33.27% | 1039.9% | 947.8% |
| Class A2 | 3.40% | 2.59% | 3.02% | 93.9% | 92.8% | 97% | -28.15% | -32.02% | -37.22% | 1021.1% | 1224.1% |
| Class B | 4.10% | 2.86% | 3.53% | 92.3% | 92.0% | 84% | -22.01% | -30.03% | -26.72% | 860.0% | 1076.6% |
| Class C | 5.30% | 3.55% | 4.26% | 95.6% | 93.8% | 91% | -22.86% | -19.38% | -19.65% | 926.6% | 1241.5% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 4.24% | 3.09% | 3.78% | 93.6% | 92.5% | 91% | -23.97% | -25.79% | -28.25% | 943.1% | 1118.5% |
| Class D | 4.60% | 3.95% | 4.37% | 93.2% | 93.9% | 96% | -24.14% | -19.87% | -20.58% | 841.1% | 1358.4% |
| Class F | 3.26% | 2.98% | 2.91% | 87.6% | 90.5% | 89% | -30.83% | -33.24% | -33.75% | 699.1% | 694.5% |
| Class TGM | 0.83% | 0.93% | 1.02% | 85.4% | 82.0% | 87% | -42.07% | -47.33% | -47.11% | 600.0% | 513.3% |
|  |  |  |  |  |  | Low delay B Main10 |  |  |  |  |  |
|  |  |  |  | Over ECM-16.1 |  |  |  | Over VTM-11.0ecm16.0 |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 3.42% | 1.11% | 1.33% | 93.6% | 87.9% | 80% | -19.46% | -35.08% | -31.41% | 850.8% | 860.7% |
| Class C | 4.98% | 1.88% | 1.85% | 95.2% | 88.7% | 91% | -20.77% | -23.47% | -25.10% | 814.7% | 963.5% |
| Class E | 4.20% | 3.03% | 2.22% | 87.7% | 89.2% | 89% | -18.44% | -24.42% | -24.30% | 739.9% | 624.1% |
| Overall (Ref) | 4.13% | 1.85% | 1.73% | 92.6% | 88.5% | 86% | -19.64% | -28.54% | -27.53% | 809.8% | 824.7% |
| Class D | 4.33% | 2.09% | 1.40% | 94.9% | 90.4% | 96% | -22.64% | -24.13% | -25.42% | 814.6% | 1122.9% |
| Class F | 3.45% | 2.38% | 2.03% | 87.1% | 88.2% | 86% | -28.12% | -37.14% | -37.11% | 673.5% | 642.3% |
| Class TGM | 1.14% | 1.08% | 1.22% | 85.5% | 87.0% | 85% | -40.19% | -49.61% | -49.51% | 596.5% | 513.9% |

  - Group 1-5 off

In this test, all the tools in the group 1-5 are switched off. The offgroup1-5.cfg was used in addition to ECM CTC settings.

|  |  |  |  |  |  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Over ECM-16 |  |  |  | Over VTM-11-ECM16 |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 9.22% | 17.28% | 22.96% | 32.9% | 48.0% | 89% | -6.66% | -1.96% | -11.21% | 379.9% | 183.3% |
| Class A2 | 10.82% | 19.55% | 20.91% | 34.6% | 43.4% | 89% | -12.54% | -9.93% | -14.71% | 360.3% | 173.1% |
| Class B | 9.83% | 18.94% | 18.42% | 32.5% | 36.9% | 85% | -6.33% | -9.01% | -6.26% | 387.5% | 207.7% |
| Class C | 11.01% | 12.32% | 13.74% | 33.8% | 39.1% | 96% | -5.69% | -1.28% | -1.35% | 378.6% | 217.3% |
| Class E | 14.91% | 21.13% | 18.09% | 32.2% | 37.3% | 96% | -7.14% | -6.80% | -6.37% | 377.7% | 216.9% |
| Overall | 11.00% | 17.66% | 18.50% | 33.1% | 40.2% | 91% | -7.41% | -5.90% | -7.42% | 378.0% | 200.8% |
| Class D | 8.72% | 10.98% | 12.39% | 33.4% | 37.2% | 97% | -5.10% | 1.02% | 1.32% | 357.0% | 211.9% |
| Class F | 18.03% | 21.07% | 22.21% | 43.5% | 36.7% | 94% | -17.96% | -20.81% | -20.18% | 274.5% | 210.3% |
| Class TGM | 18.84% | 22.74% | 23.75% | 46.1% | 37.9% | 87% | -32.83% | -37.49% | -36.17% | 259.2% | 213.6% |
|  |  |  |  |  |  | Random Access Main 10 |  |  |  |  |  |
|  |  |  |  | Over ECM-16 |  |  |  | Over VTM-11-ECM16 |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 15.74% | 16.15% | 25.90% | 49.2% | 62.4% | 97% | -15.92% | -11.52% | -20.80% | 510.9% | 444.3% |
| Class A2 | 15.52% | 19.69% | 20.85% | 50.4% | 58.8% | 97% | -19.75% | -21.02% | -26.49% | 492.2% | 510.1% |
| Class B | 14.18% | 21.19% | 21.30% | 44.5% | 58.0% | 83% | -14.42% | -17.78% | -14.30% | 468.2% | 565.3% |
| Class C | 14.72% | 15.46% | 17.11% | 46.2% | 53.2% | 90% | -15.92% | -10.13% | -9.77% | 482.3% | 600.0% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 14.90% | 18.36% | 21.01% | 47.0% | 57.7% | 90% | -16.19% | -15.14% | -16.83% | 485.0% | 536.2% |
| Class D | 11.83% | 13.79% | 15.33% | 44.9% | 53.7% | 95% | -18.79% | -12.29% | -12.25% | 452.2% | 625.3% |
| Class F | 18.31% | 20.53% | 21.40% | 38.8% | 60.2% | 90% | -21.06% | -22.56% | -22.52% | 441.8% | 411.3% |
| Class TGM | 20.18% | 25.20% | 26.00% | 38.8% | 54.6% | 89% | -31.10% | -34.95% | -34.38% | 403.3% | 309.2% |
|  |  |  |  |  |  | Low delay B Main10 |  |  |  |  |  |
|  |  |  |  | Over ECM-16 |  |  |  | Over VTM-11-ECM16 |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 13.33% | 14.66% | 15.81% | 37.7% | 40.3% | 78% | -11.70% | -26.65% | -21.85% | 439.6% | 359.3% |
| Class C | 14.41% | 12.76% | 14.94% | 40.6% | 38.8% | 89% | -13.70% | -15.37% | -15.44% | 388.7% | 364.2% |
| Class E | 14.48% | 21.63% | 18.12% | 34.0% | 56.9% | 83% | -10.41% | -11.34% | -12.44% | 434.6% | 322.3% |
| Overall (Ref) | 13.98% | 15.77% | 16.10% | 37.6% | 43.4% | 83% | -12.04% | -19.06% | -17.36% | 420.8% | 351.3% |
| Class D | 12.22% | 13.02% | 15.45% | 37.5% | 39.8% | 94% | -16.75% | -16.17% | -15.36% | 362.4% | 424.6% |
| Class F | 17.98% | 18.45% | 19.91% | 35.9% | 48.6% | 87% | -18.17% | -27.74% | -26.53% | 390.4% | 303.2% |
| Class TGM | 18.98% | 26.46% | 28.74% | 38.5% | 50.8% | 84% | -29.74% | -37.41% | -36.71% | 444.1% | 262.1% |

  - Summary

The tool-off results on top of the recent ECM versions are summarized below for BD-PSNR-Y. Note that SCC results are not included.

Issues

  - Resolved issues

- #101, ECM 16.0 decoding, MD5sum mismatch with AHG7 group1 off

- #100, ECM 16.0 encoding crash with AHG7 group4 off

- #96, Current master tip (dad860d2) encoding crash with AHG7 group1-5 off

  - Open issues

- #91, MaxTU setting

- #87, ECM-15.0 decoder crashes when "InterLFNST=0"

- #80, Compilation Issue ECM14.0 when disabling GPM reordering

- #78, Encoder crash in RA with --LMChroma=0

- #77, Encoder crash in RA with --DepQuant=0 --NumSignPred=0

- #71, Encode/decode mismatch when using single tree

- #66, Difference in encoding results of ECM-12.0 in AhG7 group 2 tool off test

- #65, Different encoding results of ECM12 in AHG7 group1-4 off tests

- #64, Encode/decode mismatch and decoder crash when inter-CCCM is disable

- #53, Decoding mismatch was observed when AMVR is off

Input contributions

| JVET-AL0083 | m71879 | 2025-03-18 18:30:00 | 2025-03-19 17:13:13 | 2025-03-19 17:13:13 | AHG7: On Luma-Chroma Gain Rebalancing | X. Li (Google), Y. Wang, K. Zhang, L. Zhang (Bytedance) |
| --- | --- | --- | --- | --- | --- | --- |
| JVET-AL0171 | m71984 | 2025-03-19 13:31:57 | 2025-03-19 14:05:58 | 2025-03-19 14:05:58 | AHG7: Tools interaction analysis of ECM160 | R. Ishimoto, Z. Fan, T. Chujoh, T. Ikai (Sharp) |
| JVET-AL0194 | m72007 | 2025-03-19 17:14:06 | 2025-03-19 17:19:06 | 2025-03-19 17:19:06 | AHG7: Assessment Perspectives of Codec/Coding Tools | X. Li (Google) |
| JVET-AL0213 | m72028 | 2025-03-19 22:00:11 | 2025-03-19 22:18:38 | 2025-03-20 00:06:39 | AHG7: On common test conditions for fixed-QP objective rate-distortion testing of "low delay" encoding | G. J. Sullivan, P. Yin, T. N. Canh (Dolby Labs) |
| JVET-AL0242 | m72065 | 2025-03-20 17:27:19 | 2025-03-20 18:49:47 | 2025-03-20 18:49:47 | [AHG7][AHG17] ECM performance under different MTT configurations | S. Puri, K. Naser, F. Le Léannec, E. François (InterDigital) |
| JVET-AL0245 | m72084 | 2025-03-21 12:48:25 | 2025-03-21 13:34:37 | 2025-03-21 13:34:37 | [AHG7][AHG17] Further consideration of common test condition | T. Ikai, K.-W. Liang (Sharp) |
| JVET-AL0247 | m72092 | 2025-03-21 15:44:06 | 2025-03-21 15:47:36 | 2025-03-21 15:47:36 | [AHG-7] On Max Transform Size in ECM | K. Naser, F. Le Léannec, S. Puri, C. Bonnineau, E. François (InterDigital) |

Recommendations

- Continue and improve tool assessment

- Resolve identified software issues related to the tool assessment

JVET-AL0008 JVET AHG report: Optimization of encoders and receiving systems for machine analysis of coded video content (AHG8) [S. Liu, J. Ström, S. Wang, M. Zhou (AHG chairs)]

  - Software and Common Test Conditions

AHG 8 related software and documents can be accessed at https://vcgit.hhi.fraunhofer.de/jvet-ahg-ofm. This repository contains two projects: common test conditions, reporting templates with anchor results, evaluation scripts and task networks are available in https://vcgit.hhi.fraunhofer.de/jvet-ahg-ofm/ofm-ctc, and software implementation examples are hosted in https://vcgit.hhi.fraunhofer.de/jvet-ahg-ofm/vtm-ofm.

For this meeting cycle, common test conditions remain unchanged as described in output document JVET-AI2031. Some software implementation examples are available in the repository:

| Branch name | Description |
| --- | --- |
| JVET-AB0275 | Region-of-Interest-based adaptive QP |
| JVET-AC0086 | Foreground/background separation |
| JVET-AD0122 | Temporal QP offsets |
| JVET-AE0143 | Spatial resampling (Experimental) |
| JVET-AG0212 | Post-processing filter (Removed*) |
| JVET-AH0130 | Spatial resampling |
| JVET-AH0157 | Combined pre- and post-processing |
| JVET-AJ0178 | Lightweight post-processing |
| JVET-AJ0181 | Combined software JVET-AB0275, JVET-AC0086, JVET-AJ0178 |
| JVET-AJ0254 | Pre-analysis based temporal resampling |
| JVET-AK0094 | Combined software of adaptive temporal resampling, pre-processing, post-processing and ROI-based adaptive QP |

* Note that the current description of the post-processing algorithm in CDTR A.3 is based on the lightweight design proposed in JVET-AJ0178. Consequently, the reference code for the previous post-processing algorithm, JVET-AG0212, has been removed from the software repository.

It was decided in the last meeting to upload the dense QP bitstreams corresponding to dense QP experiments and results reported in JVET-AK0122 to MPEG content server. They can now be accessed at https://content.mpeg.expert/data/MPEG-05/AHG8/bitstreams_for_JVET-AK0122/.

  - Technical Report

The eight draft of the technical report (TR) JVET-AK2030 “Optimization of encoders and receiving systems for machine analysis of coded video content (draft 8)” was produced, including the following additions compared with draft 7:

- Text on Packed regions info SEI (to clause 10.5) JVET-AK0141

- Tool combination examples (to annex B) JVET-AK0122

- Tool combination examples (to annex B) JVET-AK0094

The following combined tool examples have been included in the draft TR, besides single tool implementation examples.

| Technology 1 | Technology 2 | Technology 3 | Technology 4 |
| --- | --- | --- | --- |
| Adaptive QP (Clause 8.1 and Annex A.1) | Temporal layer QP offset (Clause 8.2) |  |  |
| De-noising filter (Clause 7.5) | Temporal layer QP offset (Clause 8.2) |  |  |
| Pre-processing (Clause 7.2 and Annex A.2) | NNPF (Clause 9.3 and Annex A.3) |  |  |
| 4:4:4 coding (Clause 8) | Reduced resolution (Clauses 7.4 and 9.2) |  |  |
| Pre-processing (Clause 7.2 and Annex A.2) | NNPF (Clause 9.3 and Annex A.3) | Adaptive QP (Clause 8.1 and Annex A.1) |  |
| NNPF (Clause 9.3) | Adaptive QP (Clause 8.1 and Annex A.1) |  |  |
| NNPF (Clause 9.3) | Reduced resolution (Clauses 7.4 and 9.2) |  |  |
| Pre-processing (Clause 7.2 and Annex A.2) | NNPF (Clause 9.3 and Annex A.3) | Adaptive QP (Clause 8.1 and Annex A.1) | QP offset adjustment for higher temporal layers (Clause 8.1) |
| Pre-processing (Clause 7.2 and Annex A.2) | NNPF (Clause 9.3 and Annex A.3) | Adaptive QP (Clause 8.1 and Annex A.1) | Temporal resampling (Clause 9.1 and Annex A.4) |

Input contributions

There are 2 input contributions related to AHG 8 mandates (by the time this report is uploaded). They are listed below.

| Report |  |  |
| --- | --- | --- |
| JVET-AL0008 | JVET AHG report: Optimization of encoders and receiving systems for machine analysis of coded video content (AHG8) | S. Liu, J. Ström, S. Wang, M. Zhou (AHG chairs) |
| Proposals |  |  |
| JVET-AL0152 | AHG8: Dense QP coding results of combining adaptive temporal resampling, pre-processing, ROI-based adaptive QP, QP offset adjustment for higher temporal layers and post-processing algorithms for machine vision | S. Wang, J. Chen, Y. Ye, B. Li (Alibaba), S. Wang (CityUHK) |
| Crosschecks |  |  |

Recommendations

The AHG recommends to:

- Review all input contributions.

- Continue improving draft TR based on CDTR feedback and other inputs.

- Discuss and plan for finalization of TR (version 1).

- Continue investigating non-normative technologies and their uses for machine vision applications and machine consumptions.

It was agreed that software of the non-normative tools should be attached to the TR (as also requested in context of ballot)

It was clarified that the bitstreams described in JVET-AK0122 and JVET-AL0152 are not intended to be used by JVET (as they are going beyond the CTC document JVET-AI2031), but are delivered as a service to WG 4 such that they can exercise non-normative tools in their own CTC.

Break until 0845 UTC

JVET-AL0009 JVET AHG report: SEI message studies (AHG9) [S. McCarthy, Y.-K. Wang (co-chairs), J. Boyce, T. Chujoh, S. Deshpande, C. Fogg, M. M. Hannuksela, P. de Lagrange, G. J. Sullivan, H. Tan, A. Tourapis, S. Wenger, P. Wu (vice-chairs)]

A total of 73 contributions are identified relating to the mandates of AHG9. Some contributions relate to more than one mandate and/or SEI message. Some contributions also relate to the work of AHG13 and AHG16.

The following is a list of contributions related to the mandates of AHG9.

  - Study the SEI messages in VSEI, VVC, HEVC and AVC

    - Additions for HEVC and AVC (6)

JVET-AL0057 AHG9: Inclusion of the encoder optimization information SEI message in HEVC [M. M. Hannuksela, J. Boyce (Nokia)]

JVET-AL0059 AHG9: Inclusion of the packed regions information SEI message in HEVC [J. . Boyce, M. M. Hannuksela (Nokia)]

JVET-AL0061 AHG 9: Encoder optimization information for AVC and HEVC [C. Kim, H. Tan, J. Nam, J. Lee, J. Lim, S. Kim (LGE)]

JVET-AL0062 AHG 9: AI usage restrictions SEI message for AVC and HEVC [C. Kim, H. Tan, J. Nam, J. Lee, J. Lim, S. Kim (LGE)]

JVET-AL0148 AHG9: Generative face video and generative face video enhancement SEI messages for HEVC and AVC [J. Chen, B. Chen, Y. Ye (Alibaba), S. Gehlot, G.-M. Su, P. Yin, S. McCarthy (Dolby), A. Trioux, F. Yang (Xidian Univ.), Y.-K. Wang (Bytedance), H.-B. Teo, J.-Y. Thong, K. Abe (Panasonic), Y. Xu, 	K. Yang, Y. Li (SJTU)]

JVET-AL0210 AHG9: On SEI processing order SEI message for HEVC and AVC [Y. Sanchez, T. M. Borges, R. Skupin, C. Hellge, T. Schierl (Fraunhofer HHI)]

  - Study JVET-AK2006 Additional SEI messages for VSEI version 4

    - Film grain characteristics SEI message (3)

JVET-AL0086 AHG9/AHG13: On Spatial Resolution for the Film Grain Characteristics (FGC) SEI message [J. Samuelsson-Allendes, S. Deshpande (Sharp)]

JVET-AL0204 AHG9: On picture width and height for the film grain characteristics SEI message [M. M. Hannuksela, J. Boyce (Nokia)]

JVET-AL0211 AHG9: Resolution nesting for FGC SEI message [R. Skupin, Y. Sanchez, A. Wieckowski, T. M. Borges, C. Hellge, T. Schierl (Fraunhofer HHI)]

JVET- AL0211 also relates to clause 2.5 potential needs for additional SEI messages

    - NNPF SEI messages (4)

JVET-AL0075 AHG9: On nnpfa_num_input_pic_shift [M. M. Hannuksela (Nokia)]

JVET-AL0076 AHG9: On multi-purpose NNPFs [M. M. Hannuksela, F. Cricri (Nokia)]

JVET-AL0096 AHG9: On signalling of extension syntax elements in NNPFA SEI message[C. Kim, H. Tan, J. Nam, J. Lee, J. Lim, S. Kim (LGE)]

JVET-AL0159 AHG9: On backbone and tail of NNPFs [F. Cricri, M. M. Hannuksela (Nokia)]

    - SEI processing order and processing order nesting SEI messages (5)

JVET-AL0063 AHG9: On SPO SEI message[Y. He, M. Karczewicz (Qualcomm)]

JVET-AL0064 AHG9: On SPO sub-chain signaling [Y. He, M. Karczewicz (Qualcomm)]

JVET-AL0065 AHG9: On PON SEI message [Y. He, M. Karczewicz (Qualcomm)]

JVET-AL0178 AHG9: Use cases for energy savings information in SPO SEI messages [C-H. Demarty, E. François, F. Aumont, O. Le Meur (InterDigital)]

JVET-AL0208 AHG9: On SPO root-process signaling constraint [T. M. Borges, Y. Sanchez, R. Skupin, C. Hellge, T. Schierl (Fraunhofer HHI)]

    - Encoder optimization information SEI message (3)

JVET-AL0056 AHG9: On the encoder optimization information SEI message [M. M. Hannuksela (Nokia)]

JVET-AL0123 AHG9: On the encoder optimization information (EOI) SEI message [J. Xu, Y.-K. Wang (Bytedance)]

JVET-AL0221 AHG9: EOI SEI message with luma range adaptation for machine analysis [T. Partanen (Tuni), M. M. Hannuksela, H. Zhang, A. Aminlou (Nokia)]

    - Source picture timing information SEI message (1)

JVET-AL0130 AHG9: Source picture timing for interlaced video with SPTI SEI message [J. Lee, H. Tan, J. Nam, C. Kim, J. Lim, S. Kim (LGE)]

    - Object mask information SEI message (3)

JVET-AL0066 AHG9: Lossy compression with Object mask info SEI [J. Boyce, M. M. Hannuksela (Nokia)]

JVET-AL0067 AHG9: On the OMI SEI [J. Boyce, M. M. Hannuksela (Nokia)]

JVET-AL0071 AHG9: On OMI SEI message [Y. He, M. Karczewicz (Qualcomm)]

JVET-AL0249 AHG9: Proposed modifications to VSEI to address national body comments [J. Boyce (Nokia)]

JVET- AL0249 also relates to the text description information SEI message

    - Text description information SEI message (1)

JVET-AL0077 AHG9: On the text description information SEI message [M. M. Hannuksela, J. Boyce (Nokia)]

JVET-AL0249 AHG9: Proposed modifications to VSEI to address national body comments [J. Boyce (Nokia)]

JVET- AL0249 also relates to the object mask information SEI message

    - Generative face video SEI messages (3)

JVET-AL0102 AHG9/AHG16: SEI message and further experiments on QP-adaptive GFVC [W. Kang, L. Liu, C. Jung (Xidian Univ.)]

JVET-AL0155 AHG9: Further fixes and cleanup on GFV and GFVE SEI messages [J. Chen, B. Chen, Y. Ye (Alibaba)]

JVET-AL0156 AHG9/AHG16: Colour calibration for generative face video coding [J. Chen, B. Chen, Y. Ye (Alibaba), S. Yin, S. Wang (CityUHK)]

    - Digitally signed content messages (11)

JVET-AL0078 AHG9: On Digital Signing	[S. Deshpande (Sharp)]

JVET-AL0103 AHG9: Editorial changes for the three DSC SEI messages [Y.-K. Wang, J. Xu  (Bytedance)]

JVET-AL0115 AHG9: Substream dependency signalling in the DSCI SEI message [Y.-K. Wang (Bytedance), Y. Li, K. Yang (SJTU), J. Xu (Bytedance) Y. Xu (SJTU)]

JVET-AL0116 AHG9: On DSCI content UUID [Y. Li (SJTU), J. Xu, Y.-K. Wang (Bytedance), K. Yang, Y. Xu (SJTU)]

JVET-AL0117 AHG9: On association of NAL units to DSC verification substreams [Y.-K. Wang, J. Xu, (Bytedance), Y. Li, K. Yang, Y. Xu (SJTU)]

JVET-AL0118 AHG9: Miscellaneous changes for the three DSC SEI messages [Y.-K. Wang, J. Xu (Bytedance), Y. Li, K. Yang, Y. Xu (SJTU)]

JVET-AL0119 AHG9: Support of low-delay DSC verification process [Y.-K. Wang, J. Xu (Bytedance), Y. Li, K. Yang, Y. Xu (SJTU)]

JVET-AL0131 AHG9: Proposed changes for miscellaneous aspects of SEIs in VSEI v4 [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET- AL0131 also relates to the AI usage restrictions and packed regions SEI messages

JVET-AL0132 AHG9: Editorial updates for VSEI v4 [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET- AL0132 also relates to the packed regions and image format metadata SEI messages

JVET-AL0186 AHG9: Subpicture support for digitally signed content SEI messages [M. Pettersson, R. Sjöberg, M. Damghanian (Ericsson)]

JVET-AL0222 AHG9: On Digitally Signed Content SEI messages [K. Suehring, T. Hinz, Y. Sanchez, J. Pfaff, H. Schwarz, D. Marpe, T. Wiegand (Fraunhofer HHI)]

    - AI usage restrictions SEI message (2)

JVET-AL0058 AHG9: On the AI usage restrictions SEI message [M. M. Hannuksela, L. Kondrad, K. Kammachi-Sreedhar, E. B. Aksu, J. Boyce (Nokia)]

JVET-AL0131 AHG9: Proposed changes for miscellaneous aspects of SEIs in VSEI v4 [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET- AL0131 also relates to the digitally signed content and packed regions SEI messages

    - Packed regions information SEI message (8)

JVET-AL0070 AHG9: On packed regions information SEI message [T. Chujoh, Z. Fan, R. Ishimoto, T. Ikai (Sharp)]

JVET-AL0072 AHG9: On PRI SEI message [Y. He, M. Karczewicz (Qualcomm)]

JVET-AL0098 AHG9: On target picture in packed regions information SEI message [J. Nam, H. Tan, J. Lee, C. Kim, J. Lim, S. Kim (LGE)]

JVET-AL0120 AHG9: On the packed regions information SEI message - part 1 [Y.-K. Wang, J. Xu (Bytedance)]

JVET-AL0122 AHG9: On the packed regions information (PRI) SEI message - part 2 [J. Xu, Y.-K. Wang (Bytedance)]

JVET-AL0129 AHG9: On signaling of regions in PRI SEI message [J. Lee, H. Tan, J. Nam, C. Kim, J. Lim, S. Kim (LGE)]

JVET-AL0131 AHG9: Proposed changes for miscellaneous aspects of SEIs in VSEI v4 [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET- AL0131 also relates to the digitally signed content and AI usage restrictions SEI messages

JVET-AL0132 AHG9: Editorial updates for VSEI v4 [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET- AL0132 also relates to the digitally signed content and image format metadata SEI messages

    - Image format metadata SEI message (4)

JVET-AL0068 AHG9: On image format metadata (IFM) SEI [J. Boyce, M. M. Hannuksela (Nokia)]

JVET-AL0094 AHG9: On payload length of image format metadata (IFM) SEI [A. T. Hinds, S. Wenger (Tencent)]

JVET-AL0128 AHG9: On image format metadata SEI message [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AL0132 AHG9: Editorial updates for VSEI v4 [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET- AL0132 also relates to the digitally signed content and packed regions SEI messages

  - Study JVET-AK2032 TuC for VSEI

    - General (1)

JVET-AL0093 AHG 9: On value range for syntax elements coded as u(v) [H. Tan, C. Kim, J. Nam, J. Lee, J. Lim, S. Kim (LGE)]

    - NNPF SEI messages (3)

JVET-AL0175 AHG9: On Target Mastering Colour Volume Information in the NNPFC SEI message for Tone Mapping [C-H. Demarty, E. François, F. Aumont, O. Le Meur (InterDigital)]

JVET-AL0177 AHG9: On Signalling Tone Mapping Related Information in NNPFA [C-H. Demarty, E. François, F. Aumont, O. Le Meur (InterDigital)]

JVET-AL0209 AHG9: On providing robustness against layer dropping in multi-layer NNPF [T. M. Borges, Y. Sanchez, R. Skupin, C. Hellge, T. Schierl (Fraunhofer HHI)]

    - Multiplane image information SEI message (1)

JVET-AL0091 AHG9: On the multiplane image information (MPII) SEI message [S. McCarthy, S. Oh, W. Husak (Dolby)]

    - Constituent rectangle SEI messages (2)

JVET-AL0095 AHG 9: On constituent rectangles SEI message [C. Kim, H. Tan, J. Nam, J. Lee, J. Lim, S. Kim (LGE)]

JVET-AL0139 AHG9: On multilayer support for CR SEI [T. Biatek, J. Boyce, M. M. Hannuksela (Nokia)]

    - Quality metrics SEI message (1)

JVET-AL0140 AHG9: Region-specific quality metrics for QM SEI [T. Biatek, J. Boyce, M. M. Hannuksela (Nokia)]

    - Lens optical correction SEI messages (1)

JVET-AL0216 [AHG9] Lens Optical Correction SEI - floating point parameters representation [G. Teniou, S. Wenger, A. Hinds]

    - Display rectangles SEI message (4)

JVET-AL0097 AHG 9: Gaussian blur filling method in display rectangles SEI message [C. Kim, H. Tan, J. Nam, J. Lee, J. Lim, S. Kim (LGE)]

JVET-AL0099 AHG9: On design of display rectangles SEI message [J. Nam, H. Tan, J. Lee, C. Kim, J. Lim, S. Kim (LGE)]

JVET-AL0138 AHG9: VVC interface for Display Rectangles SEI [T. Biatek, J. Boyce, M. M. Hannuksela (Nokia)]

JVET-AL0141 AHG9: Showcase on the implementation of display rectangles SEI [T. Biatek, J. Boyce, M. M. Hannuksela (Nokia)]

JVET- AL0141 also relates to clause 2.4 software and showcase information

    - Picture segment information SEI message (1)

JVET-AL0226 AHG9: Proposed segmentation plane test sequences (AUX_SEGMENT) [E. Thomas, E. Potetsianakis, E. Alexiou, M.-L. Champel (Xiaomi)]

  - Collect software and showcase information for SEI messages (1)

JVET-AL0141 AHG9: Showcase on the implementation of display rectangles SEI [T. Biatek, J. Boyce, M. M. Hannuksela (Nokia)]

JVET-AL0141 also relates to clause 2.3 study of TuC for VSEI

  - Identify potential needs for additional SEI messages, including study of AVC and HEVC SEI messages for use in VVC (8)

JVET-AL0069 AHG9/AHG18: Error recovery SEI message [J. Boyce, M. M. Hannuksela (Nokia)]

JVET-AL0073 AHG9/AHG18: Loss recovery information SEI message [C. Kim, H. Tan, J. Nam, J. Lee, J. Lim, S. Kim (LGE)]

JVET-AL0092 AHG9: Confidence information SEI message [J. Boyce, T. Biatek, M. M. Hannuksela (Nokia)]

JVET-AL0127 AHG9: Danmu information SEI message [J. Xu, Y.-K. Wang, L. Zhang (Bytedance)]

JVET-AL0133 AHG9: Signaling of thumbnail information [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET- AL0211 also relates to clause 2.2 study of VSEI v4

JVET-AL0212 AHG9/AHG15: Depth-adaptive picture scaling information SEI [V. Zakharchenko, T. Biatek, J. Boyce (Nokia)]

JVET-AL0219 AHG9: Colour mapping information SEI [J. Boyce, T. Biatek, M. M. Hannuksela (Nokia)]

JVET-AL0224 AHG9: Auxiliary sampling alignment information SEI [V. Zakharchenko, J. Boyce, D. Rusanovskyy, M. M. Hannuksela (Nokia)]

Activities

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de) with [AHG9] in message headers. No emails with [AHG9] were exchanged.

Recommendations

The AHG recommends to:

- Review all related contributions; and

- Continue SEI messages studies.

J. Arumurgan volunteered on checking whether the SPO/PON message implementation in software is matching with the current text (software was originally developed by Ittiam) – if not matching, a solution needs to be found who takes responsibility among experts from the companies wo proposed subsequent changes. Revisit.

JVET-AL0010 JVET AHG report: Encoding algorithm optimization (AHG10) [K. Andersson, P. de Lagrange, A. Duenas (co-chairs), T. Ikai, T. Solovyev, A. Tourapis (vice chairs)]

6 contributions and one cross-check document were identified relating to AHG10 and summarized in the following sections.

  - JVET-AL0055: AhG10: On constrained encoder complexity configuration of VTM

The document describes three alternative configurations for the VTM to achieve additional complexity performance trade-offs. A preferred variant shows an impact of 37%/46%/44%/43% to the encoder runtime with a bd rate impact of 4.47%/1.49%/4.17%/4.34% relatively to the default VTM 23.5 RA, AI, LDB, and LDP configurations respectively. The details of the preferred configuration are shown below:

Less extensive partitioning search:

--MaxMTTHierarchyDepth=1

--MaxMTTHierarchyDepthISliceL=2

--MaxMTTHierarchyDepthISliceC=2

Implicit MTS instead of explicit one:

--MTS=0

--MTSImplicit=1

Lower number of merge candidates:

--MaxNumMergeCand=5

A cross-check of this proposal is available in JVET-AL0263.

  - JVET-AL0074: AHG10: Teleconference on encoder optimization

The document summarizes discussions held during a teleconference on February 24, mainly related to multi-layer spatial scalability, but also to multi-layer multi-view, multi-layer content layering, and reduced complexity performance trade-offs.

  - JVET-AL0114: AHG10: On intra mode selection

The document proposes modifications to the intra mode selection of the ECM specifically to reduce the matrix-based position-dependent intra prediction (MPDIP)  mode tests. The following results are currently included in this contribution.

All-Intra: {-0.01%, 0.04%, 0.02%, EncT: 99.3%, DecT: 99.8%}

Random-Access: {} (Note: currently missing)

  - JVET-AL0207: AHG10/AHG17: On matching target bitrate for subjective quality evaluation

The document proposes an alternative approach for matching the target bitrate for subjective evaluations. Instead of increasing the base QP by one at a given frame, which may give an abrupt change in quality after the QP change, it is proposed to start from the base QP closest to the target bitrate and to adjust the lambda to correspond to a fractional QP that is in-between the two closest QPs. In this way it is claimed that a more equal quality can be reached for all frames rather than having a change of quality for frames after a change of the base QP.

The document proposes to add this functionality in the VTM reference software and suggests using the method for matching a target bitrate in the context of subjective quality evaluation.

  - JVET-AL0213: AHG7: On common test conditions for fixed-QP objective rate-distortion testing of "low delay" encoding

The document requests JVET to consider modifying the method used for routine objective rate-distortion testing of “low-delay” configurations for high-resolution video content to combat with the encoding of a long series of frames in a sequential manner. The suggested alternative is to divide a test sequence into closed-GOP segments that are coded independently of each other and to measure only the bit rate and distortion for the non-intra frames. A hypothetical alternative approach of using temporal sublayers is also suggested to be a possible way to reduce encoding times. In such a scheme, the lowest temporal layer would be coded first, followed by parallel encoding of the higher temporal sublayers, with all frames using only forward temporal prediction.

The intent of this document is to stimulate the discussion and study of the potential approaches for reducing encoding times for such tests.

  - JVET-AL0233: Information on low complexity encoding experiments

The document claims that the ability of supporting a low complexity encoder with reasonable compression efficiency is an essential functionality for a next generation video coding specification design. Experiments related to the ECM have shown that changing the encoder’s configuration can have a large impact on the  encoder complexity especially in later versions of the ECM. Some of the operating points seem to achieve a better RD-complexity trade offs than the ECM anchor points, and some operating points seem to might be able to reach the desired complexity in encoding time. The latest version of the ECM, ECM-16.0 seems unable to reach the desired complexity in both the encoder and the decoder. A significant modification of the design and the software may be required to reduce decoder complexity such that it can reach the desired decoder complexity.

  - JVET-AL0245: [AHG7][AHG17] Further consideration of common test condition

This document further explores constrained test conditions by analyzing experiment results on ECM-16.1 under various MTT depth settings. It recommends configuring the encoder setting during the development of the next standard to approximately 2x to 4.5x of the encoding time of the VTM, and by changing the MTT depth parameters to the following (about 3x faster with around 4% BD-rate loss):

--MaxMTTHierarchyDepth=2 --MaxMTTHierarchyDepthISliceL=2

--MaxMTTHierarchyDepthISliceC=2 --MaxMTTHierarchyDepthByTid=221111

- Recommendation

The AHG recommends that the related input contributions are reviewed, and to further continue the study of encoding algorithm optimizations in JVET.

JVET-AL0011 JVET AHG report: Neural network-based video coding (AHG11) [E. Alshina, F. Galpin, S. Liu, A. Segall (co-chairs), J. Li, Y. Li, R.-L. Liao, M. Santamaria, T. Shao, M. Wien, P. Wu (vice chairs)]

    - Anchor Encoding

Anchor for the NN-based video coding activity made available though the Git repository used for the AHG activity:

https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc/-/blob/master/Anchor%20performance/NNVC-10-VTM_vs_NNVC-12.xlsm

also distributed by AhG14 in JVET-AK0014.

  - New training materials

BVI-AOM training set was used in EE1-2 (NN-Inter) and EE1-3 (retraining NN-filters) categories. Results of training are analyzed in relevant EE1 contributions. So far, some promising performance improvement is observed for HOP NN-filter. It is suggested to conduct detailed discussion, opinion exchange and brainstorming on training using additional materials.

  - Dataset access

Training data generation and md5 sums for BVI-AOM been discussed during AhG11/AhG14 teleconference in scope of EE1-3 tests.

  - Interaction with ECM

Some version of neural network based Intra included into ECM already. Interface for NNVC in-loop filter testing in ECM has been added and some test results reported in incoming AhG11 contribution.

In random access configuration NN-based in-loop filter provides roughly 1%, 2% and 5% BD-rate gain on top of ECM with only 1%, 2% and 8% CPU encoding run-time increment for VLOP, LOP and HOP versions of filter correspondently.

| JVET-AL0230 | AhG11/AhG12: Performance of the NNVC ILF in ECM | D. Rusanovskyy, K. Panusopone, S. Hong, L. Wang (Nokia) |
| --- | --- | --- |
| JVET-AL0228 | EE2-related: NNLF interface in ECM | T. Poirier, F. Galpin, G. Boisson (InterDigital) |

  - EE Coordination

The AHG finalized, conducted, and discussed the EE on NN based video coding. A summary report for the EE is available at this meeting as:

| JVET-AL0023 | EE1: Summary report of exploration experiment on neural network-based video coding | E. Alshina, R. Chang, F. Galpin, Yue Li, Yun Li, M. Santamaria, J. Ström, Z. Xie (EE coordinators) |
| --- | --- | --- |

  - Teleconferences

The AHG conducted two joint teleconferences with AHG14 and EE1 during the interim period. The teleconferences were held on February 05 and March 05, 2025. In those teleconferences, the following topics were discussed:

- New version of NN-based filter (LOP5) training report and LOP5 inference cross-check

- NNVC12.0 software integration status and anchors performance,

- EE1 description finalization

- Training on BVI-AOM

Combination of two proposals on LOP  JVET-AK0195 (luma input to chroma processing at the later stage of NN filter) on top of JVET-AK0150 (attention mechanism) was verified by training cross-check and adoption from last meeting confirmed during teleconference.

| JVET-AL0043 | [AHG11] [AHG14] Teleconference on NNVC | E. Alshina, F. Galpin |
| --- | --- | --- |

  - Performance Evaluation

The performance and complexity of NN-based tools available in NNVC SW is summarized in the table below. All test data provided by AhG14. Encoding and decoding run time is very dependent on cluster used for simulation. Run time data in this table are all from InterDigital.

In NNVC-12, LOP filter performance improved by 0.6% (in random access test). There is no performance change for other configuration, but there is some speed-up (due to SADL optimization).

| Test vs NNVC (configured as VTM) | Random Access cfg |  |  |  |  | kMAC/pxl |  |  |  | Param (Mprm) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc | Dec | Total | Filter | Intra | SR | Total | Filter | Intra | SR |
| NN-Intra & LOP filter (2 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-12.0 (LOP5) | -8.2% | -15.3% | -13.5% | 1.2 | 35 | 21.4 | 16.6 | 4.8 | 0 | 1.5 | 0.247 | 1.3 | 0 |
| NNVC-11.0 (LOP4) | -7.6% | -14.3% | -13.2% | 1.2 | 36 | 21.6 | 16.8 | 4.8 | 0 | 1.5 | 0.21 | 1.3 | 0 |
| NNVC-10.0 (LOP3) | -7.4% | -13.6% | -11.6% | 1.2 | 33 | 21.7 | 16.9 | 4.8 | 0 | 1.5 | 0.21 | 1.3 | 0 |
| NNVC-9.1(LOP3) | -7.3% | -13.1% | -11.3% | 1.2 | 81 | 24.8 | 16.9 | 7.9 | 0 | 1.7 | 0.21 | 1.5 | 0 |
| NNVC-8.0(LOP2) | -6.9% | -13.2% | -12.1% | 1.2 | 73 | 25.0 | 17.1 | 7.9 | 0 | 1.6 | 0.05 | 1.5 | 0 |
| NNVC-7.1(LOP2) | -6.9% | -13.2% | -12.1% | 1.3 | 86 | 25.0 | 17.1 | 7.9 | 0 | 1.6 | 0.05 | 1.5 | 0 |
| NNVC-9.1(LOP2CA) | -8.2% | -16.5% | -15.5% | 2.5 | 69 | 25.5 | 17.6 | 7.9 | 0 | 1.6 | 0.05 | 1.5 | 0 |
| NN-Intra & HOP filter (2 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-10.0 (HOP5) | -14.2% | -19.5% | -19.9% | 2.6 | 1135 | 471 | 466 | 4.8 | 0 | 2.7 | 1.4 | 1.3 | 0 |
| NNVC-9.1(HOP4) | -14.1% | -19.2% | -19.6% | 2.9 | 1447 | 484 | 476 | 7.9 | 0 | 3.0 | 1.44 | 1.5 | 0 |
| NNVC-8.0(HOP3) | -13.7% | -13.9% | -14.5% | 2.5 | 1092 | 474 | 466 | 7.9 | 0 | 2.9 | 1.40 | 1.5 | 0 |
| NNVC-7.0(HOP2) | -13.6% | -12.5% | -14.2% | 4.1 | 2071 | 485 | 477 | 7.9 | 0 | 3.0 | 1.50 | 1.5 | 0 |
| NN-Intra & VLOP filter (2 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-11.0 (VLOP3) | -5.8% | -6.6% | -5.7% | 1.2 | 7 | 9.9 | 5.10 | 4.8 | 0 | 1.3 | 0.07 | 1.3 | 0 |
| NNVC-10.0 (VLOP2) | -5.6% | -7.6% | -6.4% | 1.1 | 15 | 10 | 5.16 | 4.8 | 0 | 1.3 | 0.06 | 1.3 | 0 |
| NNVC-9.1(VLOP) | -5.3% | -5.4% | -5.2% | 1.2 | 40 | 13 | 5.12 | 7.9 | 0 | 1.5 | 0.02 | 1.5 | 0 |
| NN-Intra & LOP filter content adaptive (2 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-10.0 aLOP3 | -8.2% | -16.7% | -15.5% | 2.2 | 34 | 21.8 | 16.9 | 4.8 | 0 | 1.5 | 0.21 | 1.3 | 0 |
| NN-Intra & LOP filter & adaptive resolution coding (3 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-11.0 NNSR | -8.5% | -12.2% | -10.9% |  |  | 26.3 | 16.8 | 4.8 | 4.7 | 1.4 | 0.05 | 1.3 | 0.05 |
| NNVC-8.0 RPR | -7.5% | -10.9% | -9.7% |  |  | 25.0 | 17.1 | 7.9 | 0 | 1.6 | 0.05 | 1.5 | 0 |
| NNVC-8.0 NNSR | -7.8% | -11.8% | -10.5% |  |  | 45.3 | 17.1 | 7.9 | 20.3 | 1.8 | 0.21 | 1.5 | 0.1 |

More details and analysis for tools and tools combination is expected in AhG14 report.

  - Architectural changes

Following proposals from 36th and 37th meetings several companies propose framework for E2E AI coded reference picture insertion. Promising gain reported for this approach (4% and 1% in all intra and random-access configuration correspondingly).

In order to support this feature bit-exact reconstruction of E2E AI coded picture is required. One contribution discusses fundamental aspects of bit-exact reproducibility and potential implementation using different platforms (GPU, NPU, CPU, ASIC).

  - NNVC algorithms description

One contribution submitted to this meeting resolves mismatch between LOP5 implementation in NNVC SW and description.

Input contributions

There are 33 input contributions related to the AHG mandates. The list of input contributions is provided below.

| Reporting (5) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| JVET-AL0023 | EE1: Summary report of exploration experiment on neural network-based video coding |  |  |  | E. Alshina, R. Chang, F. Galpin, Yue Li, Yun Li, M. Santamaria, J. Ström, Z. Xie (EE coordinators) |  |  |
| JVET-AL0043 | [AHG11] [AHG14] Teleconference on NNVC |  |  |  | E. Alshina, F. Galpin |  |
| JVET-AL0230 | AhG11/AhG12: Performance of the NNVC ILF in ECM |  |  |  | D. Rusanovskyy, K. Panusopone, S. Hong, L. Wang (Nokia) |  |
| JVET-AL0228 | EE2-related: NNLF interface in ECM |  |  |  | T. Poirier, F. Galpin, G. Boisson (InterDigital) |  |
| JVET-AL0291 | EE1-related: Recommendations for resolving mismatches between LOP5 description and implementation |  |  |  | Nam Le, Francesco Cricri (Nokia) |  |
| Architectural change and implementation aspects (4) |  |  |  |  |  |  |
| JVET-AL0080 | AHG11: Bit-exact reconstruction for NN video tools |  |  |  | L. Kerofsky, Y. Li, M. Karczewicz (Qualcomm) |  |
| JVET-AL0196 | AHG 11: Neural Network Coded Reference Frame for Intra Coding with Residual Coding and Intra Blocks |  |  |  | F. Brand, T. Solovyev, E. Alshina (Huawei) |  |
| JVET-AL0203 | [AHG11] A Hybrid Framework Integrating End-to-End Learned Image Codec with Conventional Codec |  |  |  | N. Zou, A. Hallapuro, F. Cricri, H. Zhang, A. B. Koyuncu, J. Ahonen, M. M. Hannuksela (Nokia) |  |
| JVET-AL0243 | [AHG11] Multilayer framework for supporting a hybrid codec using End-to-End Learned Image Codec and Conventional Video Codec |  |  |  | F. Urban, Y. Chen, F. Galpin, E. François (InterDigital) |  |
| EE1 contributions (11) |  |  |  |  |  |  |
| JVET-AL0084 | EE1-1.2: LOP5 improvement with parallel 1x3/3x1 Backbone | T. Shao, P. Yin, S. McCarthy (Dolby), J. N. Shingala, A. Shyam, A. Suneja, S. P. Badya (Ittiam) |  |  |  |
| JVET-AL0085 | EE1-3.1: NNVC-LOP5 retraining with additional BVI-AOM dataset | A. Suneja, J. N. Shingala, A. Shyam, S. P. Badya (Ittiam), T. Shao, P. Yin, S. McCarthy (Dolby) |  |  |  |
| JVET-AL0104 | EE1-2.1: Lightweight Multiscale Reference Frame Generation for VVC Inter Coding | P. Li, C. Jung, Q. Qin (Xidian Univ.) |  |  |  |
| JVET-AL0105 | EE1-2.2: RA/LDB Unified Reference Frame Synthesis for VVC Inter Coding | Q. Qin, C. Jung (Xidian Univ.) |  |  |  |
| JVET-AL0144 | EE1-3.1: Retraining LOP4 and LOP5 using extended dataset from BVI-AOM | D. Liu, J. Ström, M. Damghanian, P. Wennersten (Ericsson) |  |  |  |
| JVET-AL0145 | EE1-1.4: Conditional loop-filter | M. Santamaria, F. Cricri (Nokia) |  |  |  |
| JVET-AL0164 | EE1-1.1: Multiscale blocks in LOP5 and VLOP3 filters | R. Yang, M. Santamaria, F. Cricri, H. Zhang, J.Lainema, M.M. Hannuksela (Nokia) |  |  |  |
| JVET-AL0169 | EE1-1.3 Dimension-wise decomposed representation of multiplier for content-adaptive loop filtering | Z. Xu, J. Konieczny, A. Filippov, C. Hollmann, V. Rufitskiy, T. Dong (TCL) |  |  |  |
| JVET-AL0187 | EE1-3.1: NNVC-LOP4 and LOP5 retraining with additional BVI-AOM dataset | T. Dumas, A. Monier, F. Galpin (InterDigital) |  |  |  |
| JVET-AL0189 | EE1-3.2 – NNVC-HOP5 retraining adding BVI-AOM | F. Galpin (InterDigital) |  |  |  |
| JVET-AL0190 | EE1-3.3 – NNVC-VLOP3 retraining adding BVI-AOM | F. Galpin, Z. Ameur (InterDigital), R. Chang, L. Wang, X. Xu, S. Liu (Tencent) |  |  |  |
| New NNVC tools in AhG11 or EE1 related contributions (7) |  |  |  |  |  |  |
| JVET-AL0166 | EE1 related: Improved VLOP with Attention |  | Y. Li, L. Kerofsky, M. Karczewicz (Qualcomm |  |  |  |
| JVET-AL0167 | EE1 related: Further simplification of VLOP with Attention |  | Y. Li, L. Kerofsky, M. Karczewicz (Qualcomm) |  |  |  |
| JVET-AL0184 | EE1-related: Deep Reference Frame Generation for Inter Prediction Enhancement |  | D. Ding, X. Chen, Z. Chen (Wuhan Univ.) |  |  |  |
| JVET-AL0121 | AHG11: Sample-based adaptive blending weight selection for LOP |  | H. Kwon, H. Ko (HYU) |  |  |  |
| JVET-AL0136 | AHG11: Over-Parameterized LOP In-Loop Filter |  | J. Han, C. Jung, Q. Qin (Xidian Univ.) |  |  |  |
| JVET-AL0137 | AHG11: Backbone Block Enhancement of LOP In-Loop Filter with Over-Parameterized Training and Variable Channels |  | J. Han, C. Jung, Q. Qin (Xidian Univ.) |  |  |  |
| JVET-AL0250 | AHG11: Cross-component enhanced NNSR |  | T. Yang, W.-X. He, Y.-Q. Zhu, J.-D. Ye, X.-T. Xie, J.-S. Gong, Q. Liu (HUST), Z.-Y. Lv (vivo) |  |  |  |
| Cross-checks (6 some not yet uploaded at the time report was prepared) |  |  |  |  |  |  |  |
| JVET-AL0173 | Crosscheck of JVET-AL0164 (EE1-1.1: Multiscale blocks in LOP5 and VLOP3 filters) |  |  | Y. Li (Qualcomm) |  |  |
| JVET-AL0185 | Crosscheck of JVET-AL0169 (EE1-1.3 Dimension-wise decomposed representation of multiplier for content-adaptive loop filtering) |  |  | M. Santamaria (Nokia) |  |  |
| JVET-AL0246 | Crosscheck of JVET-AL0084 (EE1-1.2: LOP5 improvement with parallel 1x3/3x1 Backbone) |  |  | Y. Li (Bytedance) |  |  |
| JVET-AL0284 | Crosscheck of JVET-AL0145 (EE1-1.4: Conditional loop-filter) |  |  | J. Strom (Ericsson) |  |  |
| JVET-AL0297 | Crosscheck of JVET-AL0104 (EE1-2.1: Lightweight Multiscale Reference Frame Generation for VVC Inter Coding) |  |  | L. Murn (Nokia) |  |  |
| JVET-AL0298 | Crosscheck of JVET-AL0105 (EE1-2.2: RA/LDB Unified Reference Frame Synthesis for VVC Inter Coding) |  |  | L. Murn (Nokia) |  |  |

Recommendations

The AHG recommends:

- Review all input contributions.

- Continue investigating neural network-based video coding tools, including coding performance and complexity.

- Continue collecting training materials for neural network-based video coding tool development and investigate training stability.

- Conduct detailed discussion on training, exchange opinions and observations among interested parties (potential BoG)

- Add study of bit-exact reproducibility to the NNVC complexity assessment

Results indicate that adding BVI-AOM to training is not sufficient for optimizing inter prediction part of EE.

JVET-AL0012 JVET AHG report: Enhanced compression beyond VVC capability (AHG12) [M. Karczewicz, Y. Ye, L. Zhang (co-chairs), B. Bross, R. Chernyak, X. Li, K. Naser, Y. Yu (vice-chairs)]

The primary activity of the AHG was the “Exploration experiment on enhanced compression beyond VVC capability” (JVET-AK2024). The combined improvements of the ECM-16.1 over VTM-11.0ecm16.0 anchor for AI, RA and LB configurations are:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -14.59% | -15.70% | -26.79% | 1197.4% | 527.4% |
| Class A2 | -21.11% | -23.87% | -28.39% | 1175.6% | 576.4% |
| Class B | -14.71% | -22.37% | -20.35% | 1115.0% | 622.9% |
| Class C | -14.85% | -11.82% | -12.99% | 1050.3% | 586.3% |
| Class E | -19.17% | -22.39% | -20.59% | 1033.0% | 649.6% |
| Overall | -16.53% | -19.17% | -21.17% | 1109.1% | 594.2% |
| Class D | -12.71% | -8.95% | -9.88% | 1027.3% | 624.9% |
| Class F | -30.26% | -33.90% | -34.10% | 719.4% | 656.4% |
| Class TGM | -43.26% | -48.84% | -48.12% | 543.2% | 622.8% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -27.30% | -23.64% | -35.77% | 1204.6% | 1070.0% |
| Class A2 | -30.50% | -33.70% | -39.05% | 1162.8% | 1307.0% |
| Class B | -25.08% | -31.91% | -29.21% | 999.8% | 1139.8% |
| Class C | -26.76% | -22.11% | -22.88% | 1053.2% | 1226.1% |
| Class E |  |  |  |  |  |
| Overall | -27.06% | -28.00% | -30.80% | 1084.5% | 1179.5% |
| Class D | -27.52% | -22.95% | -23.94% | 995.5% | 1342.6% |
| Class F | -33.16% | -35.27% | -35.81% | 918.3% | 851.5% |
| Class TGM | -42.58% | -47.84% | -47.64% | 742.3% | 613.8% |

|  | Low Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -22.10% | -35.80% | -32.15% | 992.7% | 940.5% |
| Class C | -24.50% | -24.84% | -26.38% | 936.2% | 984.3% |
| Class E | -21.74% | -26.55% | -25.95% | 952.2% | 623.4% |
| Overall | -22.81% | -29.83% | -28.68% | 963.4% | 861.6% |
| Class D | -25.88% | -25.77% | -26.68% | 949.4% | 1139.8% |
| Class F | -30.60% | -38.50% | -38.39% | 846.9% | 748.6% |
| Class TGM | -40.89% | -50.16% | -50.14% | 737.2% | 550.1% |

The rate reduction for natural sequences over VTM in RA configuration for {Y, U, V} increased from ECM-15.0’s {-26.55%, -27.57%, -30.37%} to ECM-16.1’s {-27.06%, -28.00%, -30.80%}. For SCC sequences (class TGM) the rate reduction for RA configuration increased from ECM-15.0’s { -42.33%, -47.83%, -47.63%} to ECM-16.1’s { -42.58%, -47.84%, -47.66%}.

Contributions

In addition to 31 EE2 contributions, 30 (comparing to 35 last meeting) EE2-related and AHG12-related contributions were received. The EE2-related and AHG12-related contributions can be subdivided as follows:

  - Intra (9)

JVET-AL0174, "Non-EE2: TIMD fusion with neural network based intra prediction", Y.-H Lin, K.-W Liang, C.-Y Teng, Y.-C Yang (Sharp)

JVET-AL0179, "Non-EE2: Intra Merge Mode Enhancements", M. Blestel, P. Andrivon (Ofinno), Y. Chang, V. Seregin, M. Karczewicz (Qualcomm Inc.),

JVET-AL0183, "Non-EE2: Block Vector-based Intra Mode Derivation", J.-K Lee, D. Ruiz Coll, M. Blestel (Ofinno)

JVET-AL0192, "EE2-related: Harmonization of SGPM-BV and LIC", J. Huo, Y. Fei, L. Wang, Y. Ma, F. Yang (Xidian Univ.)

JVET-AL0195, "Non-EE2: TIMD-BV extension with enhanced IntraTMP merge list", J. Fu, J. Zhang, Y. Zhao, S. Ma (PKU), Y. Gao, C. Huang (ZTE), K. Naser, M. RadosavljeviÄ‡, T. Dumas (InterDigital), D. Ruiz Coll, J.-K Lee (Ofinno)

JVET-AL0202, "EE2-related: Adaptive picture-level vertical mirroring based on decimated video", D. Mieloch, M. Lorkiewicz, A. Dziembowski (PUT)

JVET-AL0231, "Non-EE2: On interpolation filter for TIMD", Y. Wang, W. Yin, K. Zhang, Z. Deng, N. Zhang, L. Zhao, M. Salehifar, L. Zhang (Bytedance)

JVET-AL0241, "Non-EE2: Intra TMP sub-modes depending on the template type information", T. Dumas, K. Naser, M. Radosavljević, F. Le Léannec (Interdigital)

JVET-AL0290, "[AHG12] Combination of JVET-AL0174, JVET-AL0195 and JVET-AL0241 on Enhanced TIMD with NNIP and Optimized Block Vector Derivation", K. Naser, S. Puri, T. Dumas, M. Radosavljević, F. Le Léannec, E. François (InterDigital), J. Fu, Y. Zhao, J. Zhang, S. Ma (PKU), C. Huang (ZTE), D. Ruiz Coll, J.-K. Lee (Ofinno), Y-H. Lin, C.-Y. Teng, K.-W. Liang, Y.-C. Yang (Sharp Corporation)

  - Inter (6)

JVET-AL0082, "AHG12: Modifications to regression-based GPM with intra and inter prediction", R. Yu, V. Seregin, M. Karczewicz (Qualcomm)

JVET-AL0111, "Non-EE2: Matrix-based position dependent intra prediction for GPM/CIIP", Z. Sun, Y. Yu, L. Xu, H. Yu, D. Wang (OPPO)

JVET-AL0193, "Non-EE2: Extension of Template Types in Regression-Based GPM", Y. Yao, J. Huo, W. Zhang, F. Yang (Xidian Univ.), B. Li, F. Xing, P. Han, Z. Wang (Hisense)

JVET-AL0234, "EE2-related: affine bilateral matching merge mode", H. Huang, V. Seregin, M. Karczewicz (Qualcomm)

JVET-AL0248, "AHG12: On Motion Vector Prediction", D. Bugdayci Sansli, P. Astola, J. Lainema (Nokia)

JVET-AL0287, "AHG12: Generated Merge Candidates",	D. Bugdayci Sansli, P. Astola, J. Lainema (Nokia)

  - Entropy and Coefficient Coding (5)

JVET-AL0087, "AHG12: Predictive transform coefficient coding", T. N. Canh, F. Pu, P. Yin, S. McCarthy (Dolby)

JVET-AL0109, "EE2-related: On binarization of a coefficient level in TSRC", Y. Yu, L. Xu, J. Gan, H. Yu, D. Wang (OPPO)

JVET-AL0113, "AHG12: Directional sign prediction", L. Xu, Y. Yu, Z. Sun, L. Zhang, H. Yu, D. Wang (OPPO)

JVET-AL0150, "Non-EE2: CABAC context switch of sig_coeff_flag and abs_level_gtx_flag for LFNST/NSPT", T. Kusakabe, K. Abe, T. Sugio, T. Nishi (Panasonic)

JVET-AL0198, "Non-EE2: Optimization of probability estimation in CABAC", D. Karwowski, D. Mieloch, M. Lorkiewicz, J. Stankowski (PUT)

  - In Loop Filters (9)

JVET-AL0110, "EE2-related: On ALF-CCCM", N. Song, L. Xu, Y. Yu, H. Yu, D. Wang (OPPO)

JVET-AL0163, "Non-EE2: Look-up table based loop filtering for ECM", Y. Du, J. Chen, A. Li, J. Liu, C. Zhu, L. Luo, H. Guo (UESTC), Y. Huo, L. Liu (Transsion)

JVET-AL0182, "AhG-12: On TALF reference picture extensions", P. Onno, B. Galmiche, G. Laroche (Canon)

JVET-AL0220, "AHG12: Reuse of ALF control information", N. Hu, H. Wang, M. Karczewicz, V. Seregin (Qualcomm)

JVET-AL0225, "EE2-related: On In-Loop Filtering in ECM", D. Rusanovskyy, M. Santamaria, N. Le, F. Cricri, K. Panusopone, S. Hong, L. Wang, J. Lainema (Nokia)

JVET-AL0228, "EE2-related: NNLF interface in ECM", T. Poirier, F. Galpin, G. Boisson (InterDigital)

JVET-AL0230, "AhG11/AhG12: Performance of the NNVC ILF in ECM", D. Rusanovskyy, K. Panusopone, S. Hong, L. Wang (Nokia)

JVET-AL0232, "Non-EE2: on TALF input extension", C. Ma, X. Xiu, X. Wang (Kwai)

JVET-AL0275, "Non-EE2: Non-linear clipping for CCALF", N. Song, L. Xu, Y. Yu, H. Yu, D. Wang (OPPO)

Recommendations

The AHG recommends to:

- To review all the related contributions.

JVET-AL0013 JVET AHG report: Film grain technologies (AHG13) [W. Husak, P. de Lagrange (co-chairs), A. Duenas, X. Meng, A. Segall, G. Teniou, A. Tourapis (vice-chairs)]

This period was slow with no meetings or discussions held.

From the previous meetings, the following five items should be in the second edition:

- Updated TR text to fix errors and clumsy wording

- Updated tools for the TR

- Video test sequences specific to film grain to be added to the TR

- Metrics – discussion and/or survey of available techniques or metrics

- Subjective testing of Film Grain

The following are worthy of discussion for the second edition:

- Conformance – this is interesting due to being an SEI

- Analysis – preprocessing analysis (parameter estimation)

These are additional topics for consideration and discussion:

- EG/RP (?) – What would be useful information for guiding usage? (guidance document)

- Future parameters and signaling – not a v2 topic

Related contributions

Six contributions related to AHG13 were identified as of 03/25/2025.

- One contribution was the AHG report:

  - JVET-AL0013 JVET AHG report: Film grain technologies (AHG13)

- Five other contributions were uploaded at the time of the report drafting:

  - JVET-AL0086 AHG9/AHG13: On Spatial Resolution for the Film Grain Characteristics (FGC) SEI message

  - JVET-AL0204 AHG9: On picture width and height for the film grain characteristics SEI message

  - JVET-AL0211 AHG9: Resolution nesting for FGC SEI message

  - JVET-AL0282 AHG13: Film grain analysis improvement

  - JVET-AL0288 AHG2, AHG9, AHG13: on FGC SEI reference picture size

  - Contributions

There were five contributions registered other than the AHG report.  Three were uploaded as of 03/25/25.

    - JVET-AL0086 AHG9/AHG13: On Spatial Resolution for the Film Grain Characteristics (FGC) SEI message

This contribution relates to the Film Grain Characteristics (FGC) SEI message. It is proposed to add signalling of width and height as extension of the FGC SEI message. It is asserted that this signalling allows for encoders and decoders to properly handle the combination of resolution changes and Film Grain Synthesis (FGS). The proposed solution is claimed to have the following advantages:

- It is backwards compatible in the sense that no change is expected from existing encoders and decoders with support for the FGC SEI message.

- It does not reinterpret/overload existing syntax elements with properties they were not intended for.

- It applies not only to VVC and RPR, but also to general Adaptive Bit Rate (ABR) streaming with VVC, HEVC, and AVC.

The same change is proposed for VSEI, HEVC and AVC.

    - JVET-AL0204 AHG9: On picture width and height for the film grain characteristics SEI message

The following aspects are proposed for the film grain characteristics (FGC) SEI message in this contribution:

- Addition of fg_pic_width_in_luma_samples and fg_pic_height_in_luma_samples to an extension of the FGC SEI message. When present, they indicate the width and the height of the picture for which the film grain modelling information is indicated in units of luma samples. When fg_pic_width_in_luma_samples and fg_pic_height_in_luma_samples are not present, there is no change to the existing specifications on picture width and height for the FGC SEI message.

Clarifications how the input image Î to which the film grain model applies is derived.

    - JVET-AL0211 AHG9: Resolution nesting for FGC SEI message

This proposal addresses the issue of applying FGS at a resolution different than the coded resolution reported in JVET-AK0197 and also addressed in JVET-AK0339. The document asserts that the solutions proposed at the last meeting are not well suited for adaptive streaming environments, have backward-compatibility issues and are unclear regarding conformance window cropping. A solution based on nesting is proposed by adding a new resolution nesting SEI message to VSEI to carry so-called resolution-nested SEI messages to be applied to a resampled decoded picture. Interface text for VVC is provided.

    - JVET-AL0282 AHG13: Film grain analysis improvement

Not uploaded.

    - JVET-AL0288 AHG2, AHG9, AHG13: on FGC SEI reference picture size

This contribution is a continuation of JVET-AK0197; it proposes to amend the VSEI specification and VVC interface text to refer to SPS instead of PPS to define reference picture resolution for the interpretation of the film grain characteristics SEI message, the intent being to describe film grain characteristics at source picture resolution independently of encoder decisions to perform reduced-resolution coding.

An experiment is reported to showcase the effect of grain synthesis at either SPS max resolution, or coded resolution when using RPR functionality testing of the VTM, switching between full and half resolution.

It is also proposed to indicate that unspecified cropping and padding can be applied as needed.

Recommendations

The AHG recommends:

- Continue editing the second edition for the TR;

- the related input contributions are reviewed;

- any liaisons are reviewed;

- testing of FGS be discussed;

- continued conformance discussion;

- SEI message extensions;

- discussion topics for an EG/RP; and

- continue the study of film grain technologies in JVET.

It was announced that JVET-AL2020 will be made available during the current meeting.

JVET-AL0014 JVET AHG report: NNVC software development (AHG14) [F. Galpin (chair), R. Chang, Yue Li, Yun Li, M. Santamaria, J. N. Shingala, Z. Xie (vice-chairs)]

- Software development

  - Location

NNVC repository is located at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM

NNVC software is based on VTM-11.0 with enabled MCTF including the update from JVET-V0056, GOP32, enabling deblocking in the RDO, JVET-AH0054 with improved MCTF, JVET-AI0124 for reference picture alignment.

NNVC-12.0 anchor at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc is used for NNVC performance evaluation.

  - Software changes

  - NNVC-12.0

Several commits were merged in the NNVC repository. The following changes were integrated:

| Contribution | MR | description |
| --- | --- | --- |
| AK0093 | MR 290 | EE1-1.5.2 (off in ctc) : Adaptive skip of LOP filtering based on boundary strength partitions |
| AK0150 | MR 289 | AK0150 attention module (LOP only) |
| AK0311 | MR 288/291 | JVET-AK0311 - AHG11/AHG14: Content-adaptive LOP4 filter |
|  | MR 144 | Feature: backtrack layers to run from used output |
|  | MR 293 | merge new SADL in NNVC |
|  | MR 294 | update to SADL v12 |
|  | MR 295 | correction for HOP5 training |
| AK0195 | MR296 | LOP5(JVET-AK0150 & JVET-AK0195) |
|  | MR297 | Nnvc 6.1 aom dataset assets |

In SADL, the following changes were integrated:

| Contribution | MR | description |
| --- | --- | --- |
|  | MR 147 SADL | SIMD add implementation |
|  | MR 133 SADL | Simd mul |
|  | MR 134 SADL | Simd relu |
| AK0257 | MR 138 SADL | Extend the Resize operator to support 2x upsampling in Bicubic mode |
| AK0257 | MR 139 SADL | Extend the GridSample operator to support Bicubic mode |
| AK0151 | MR 141 SADL | Explicit full SIMD for 1x3, 3x1, and 3x3 DW convolution |
|  | MR 142 SADL | Fix #53: Resize input width and height in count_mac only when the input has 3 or more dimensions |

A new branch allowing dataset extension has been created to ease the process of dataset creation ( see https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/NNVC-6.1_aom_dataset?ref_type=heads )

  - Software version

NNVC-12 was tagged February 6th, 2024 (containing final LOP5 model)

NNVC-12rc1 was tagged February 5th, 2024 (containing new intermediate LOP5 model)

NNVC-11.0 was tagged December 10th, 2024 (bit accurate with 11rc)

NNVC-11.0rc was tagged December 3rd, 2024

NNVC-10.0 was tagged August 9th, 2024

NNVC-9.1 was tagged May 28th, 2024

NNVC-9.0 was tagged May 13th, 2024

NNVC-8.0 was tagged February 20th, 2024

NNVC-8.0rc3 was tagged February 20th, 2024.

NNVC-7.1 was tagged November 29th, 2023.

NNVC-7.0 was tagged November 3rd, 2023.

NNVC-6.1 was tagged September 25th, 2023 (fix).

NNVC-6.0 was tagged September 6th, 2023.

NNVC-5.1 was tagged July 19th, 2023.

NNVC-5.0 was tagged May 11th, 2023.

NNVC-3.0 (a.k.a VTM-11.0_nnvc3.0) was tagged December 1st, 2022.

NCS-1.0 (a.k.a NNVC-3.0wip2) was tagged September 4th, 2022 (first release containing the FilterSets, using NNVC 2.0 as a base).

VTM-11.0_nnvc-2.0 was tagged August 4th, 2022 (add deblocking in RDO).

VTM-11.0_nnvc-1.0 was tagged May 6th, 2021 (VTM-11.0 base with MCTF enabled).

- CTC performance

See configurations section for naming convention.

  - Comparison to VTM

    - NNVC-11.0 VTM vs NNVC-12.0 VTM

VTM configuration of NNVC is not changed between version 10.0 and 12.0.

Note: 1 redundant bit in SPS is removed between the 2 versions (see MR 265). In practice NNVC-10 VTM is used as anchor.

    - NNVC-12.0 VTM vs NNVC-12.0 anchor

The NNVC-12.0 anchor includes LOP.5 filter and Intra Prediction tools activated.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-10.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -9.44% | -13.81% | -15.06% | -9.67% | -16.12% | -17.77% | 119% | 3756% |
| Class A2 | -8.43% | -15.41% | -9.80% | -8.75% | -16.06% | -9.98% | 117% | 3619% |
| Class B | -7.83% | -16.23% | -14.70% | -8.16% | -18.57% | -17.48% | 116% | 3610% |
| Class C | -7.63% | -14.80% | -13.61% | -8.03% | -15.73% | -15.29% | 110% | 3133% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -8.22% | -15.20% | -13.50% | -8.54% | -16.82% | -15.45% | 115% | 3505% |
| Class D | -8.11% | -13.77% | -12.49% | -7.36% | -15.21% | -13.59% | 105% | 2816% |
| Class F | -4.44% | -8.80% | -7.54% | -5.54% | -12.17% | -11.56% | 117% | 1542% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-10.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -6.00% | -11.62% | -10.75% | -6.57% | -11.47% | -13.05% | 107% | 3324% |
| Class C | -6.12% | -12.78% | -10.34% | -6.96% | -12.06% | -8.63% | 102% | 3116% |
| Class E | -6.62% | -7.98% | -5.71% | -7.97% | -6.80% | -6.13% | 109% | 2797% |
| Overall | -6.19% | -11.10% | -9.35% | -7.05% | -10.50% | -9.85% | 106% | 3116% |
| Class D | -6.98% | -9.73% | -6.65% | -7.15% | -7.63% | -4.39% | 98% | 3028% |
| Class F | -3.58% | -5.89% | -5.19% | -5.78% | -6.59% | -8.53% | 108% | 1687% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay P Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-10.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -6.59% | -13.84% | -12.80% | -6.85% | -14.83% | -15.94% | 109% | 3302% |
| Class C | -6.59% | -15.10% | -12.65% | -7.04% | -14.89% | -13.26% | 104% | 3035% |
| Class E | -7.63% | -9.40% | -7.19% | -9.23% | -9.21% | -9.18% | 116% | 3306% |
| Overall | -6.85% | -13.15% | -11.35% | -7.51% | -13.44% | -13.35% | 109% | 3212% |
| Class D | -7.42% | -11.49% | -9.48% | -7.53% | -9.29% | -8.74% | 99% | 2777% |
| Class F | -3.99% | -7.42% | -6.72% | -5.92% | -9.50% | -12.42% | 108% | 1657% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | All Intra Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-10.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -10.04% | -15.76% | -15.86% | -9.90% | -18.72% | -18.73% | 161% | 2722% |
| Class A2 | -8.39% | -15.93% | -12.29% | -8.97% | -16.56% | -12.16% | 166% | 2401% |
| Class B | -8.35% | -15.94% | -16.39% | -8.69% | -18.44% | -18.75% | 172% | 2437% |
| Class C | -8.31% | -14.16% | -14.18% | -8.99% | -17.53% | -17.83% | 149% | 1758% |
| Class E | -11.85% | -17.56% | -18.40% | -12.46% | -17.80% | -19.96% | 162% | 2511% |
| Overall | -9.21% | -15.78% | -15.46% | -9.63% | -17.87% | -17.64% | 162% | 2314% |
| Class D | -8.20% | -11.93% | -12.73% | -8.35% | -16.84% | -17.48% | 139% | 1556% |
| Class F | -5.77% | -10.64% | -9.99% | -6.21% | -13.77% | -13.81% | 130% | 1898% |

Note: Results from Interdigital, crosschecked by Qualcomm (AI/RA) and Nokia.

    - NNVC-11.0 VTM mode vs NNVC-11.0 HOP.5

The NNVC-12.0 had not change the HOP5 model. Performance is the same as NNVC-11.0 with a slight decoding time increase.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -15.60% | -13.69% | -21.60% | -17.26% | -19.06% | -23.79% | 305% | 155418% |
| Class A2 | -15.13% | -20.72% | -26.68% | -14.49% | -20.24% | -23.74% | 283% | 135823% |
| Class B | -13.10% | -24.21% | -17.36% | -11.95% | -22.58% | -16.70% | 294% | 146069% |
| Class C | -13.92% | -17.30% | -16.86% | -12.52% | -14.58% | -14.20% | 229% | 111863% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -14.22% | -19.57% | -19.94% | -13.67% | -19.28% | -18.86% | 275% | 135747% |
| Class D | -14.83% | -17.41% | -17.34% | -11.03% | -12.96% | -11.40% | 226% | 112306% |
| Class F | -8.99% | -11.35% | -10.48% | -9.24% | -12.20% | -10.80% | 412% | 59637% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -13.53% | -10.72% | -17.20% | -15.00% | -16.58% | -19.47% | 360% | 105630% |
| Class A2 | -13.56% | -17.53% | -21.27% | -14.30% | -18.54% | -18.26% | 265% | 84182% |
| Class B | -12.13% | -16.96% | -14.32% | -12.03% | -17.06% | -14.79% | 259% | 80684% |
| Class C | -13.07% | -14.27% | -16.16% | -12.79% | -13.89% | -15.46% | 194% | 52806% |
| Class E | -16.66% | -19.95% | -18.84% | -16.38% | -18.82% | -19.54% | 271% | 89903% |
| Overall | -13.56% | -15.91% | -17.12% | -13.80% | -16.81% | -17.09% | 259% | 78755% |
| Class D | -12.19% | -12.68% | -15.79% | -11.24% | -12.70% | -14.68% | 178% | 49898% |
| Class F | -9.96% | -12.44% | -10.41% | -9.68% | -12.47% | -10.99% | 168% | 65233% |

Note: Results from InterDigital, crosschecked by xxx.

    - NNVC-12.0 VTM mode vs NNVC-12.0 VLOP

The NNVC-12.0 had not change the VLOP3 model. Performance is the same as NNVC-11.0 with a slight decoding time decrease.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-10.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -7.18% | -5.57% | -5.74% | -7.66% | -6.78% | -6.23% | 107% | 1606% |
| Class A2 | -5.92% | -6.40% | -4.16% | -6.51% | -7.01% | -3.96% | 109% | 1481% |
| Class B | -5.45% | -7.56% | -6.50% | -6.25% | -8.21% | -7.34% | 110% | 1516% |
| Class C | -4.95% | -6.49% | -5.83% | -6.01% | -7.79% | -6.45% | 110% | 1382% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -5.75% | -6.65% | -5.70% | -6.52% | -7.57% | -6.20% | 109% | 1489% |
| Class D | -4.87% | -5.48% | -4.91% | -5.46% | -6.03% | -5.38% | 108% | 1307% |
| Class F | -2.93% | -3.49% | -3.22% | -3.87% | -5.61% | -4.54% | 112% | 726% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-10.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -8.24% | -8.99% | -8.35% | -8.36% | -10.77% | -9.22% | 150% | 1248% |
| Class A2 | -6.80% | -8.35% | -6.85% | -7.16% | -8.55% | -6.21% | 151% | 1154% |
| Class B | -6.73% | -8.80% | -8.72% | -7.11% | -9.70% | -9.50% | 154% | 1176% |
| Class C | -6.25% | -7.86% | -7.59% | -7.16% | -9.37% | -8.65% | 143% | 966% |
| Class E | -9.34% | -10.06% | -9.98% | -10.08% | -9.77% | -9.18% | 153% | 1223% |
| Overall | -7.32% | -8.76% | -8.30% | -7.83% | -9.63% | -8.67% | 150% | 1141% |
| Class D | -6.10% | -6.89% | -6.85% | -6.61% | -8.78% | -8.50% | 136% | 911% |
| Class F | -4.31% | -5.60% | -5.58% | -4.75% | -7.00% | -6.89% | 122% | 861% |

Note: Results from Interdigital, crosschecked by Qualcomm (RA/AI) and Nokia.

    - NNVC-12.0 VTM vs NNVC-12.0 NNSR

The NNVC-12.0 had not change the NNSR model. Performance is the same as NNVC-11.0 (performance on NNVC-11 for reference).

Note: only class A sequences are changed, LDx configuration are thus omitted.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -12.07% | -10.17% | -10.56% | -19.18% | -21.21% | -20.88% |  |  |
| Class A2 | -8.86% | -6.49% | -1.95% | -17.40% | -15.13% | -14.57% |  |  |
| Class B | -7.29% | -15.44% | -14.36% | -7.96% | -17.45% | -16.84% |  |  |
| Class C | -6.91% | -13.77% | -13.51% | -7.78% | -14.78% | -15.09% |  |  |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -8.46% | -12.15% | -10.89% | -12.04% | -17.02% | -16.73% |  |  |
| Class D | -7.13% | -13.31% | -12.30% | -6.87% | -14.56% | -13.06% |  |  |
| Class F | -4.02% | -8.28% | -7.29% | -5.03% | -10.94% | -10.87% |  |  |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -10.98% | -9.96% | -11.73% | -13.82% | -13.69% | -13.78% |  |  |
| Class A2 | -11.00% | -4.54% | -1.25% | -17.09% | -6.78% | -2.52% |  |  |
| Class B | -7.86% | -15.36% | -16.39% | -8.27% | -17.73% | -18.36% |  |  |
| Class C | -7.71% | -14.17% | -14.72% | -8.57% | -17.34% | -17.80% |  |  |
| Class E | -11.22% | -17.07% | -18.08% | -11.90% | -17.26% | -19.04% |  |  |
| Overall | -9.43% | -12.68% | -13.00% | -11.34% | -15.07% | -14.95% |  |  |
| Class D | -7.61% | -12.34% | -13.39% | -7.97% | -16.35% | -17.00% |  |  |
| Class F | -5.39% | -9.85% | -9.70% | -5.81% | -12.75% | -13.41% |  |  |

Note: Results from HUTS, crosschecked by xxx.

  - Comparison to NNVC-12.0 anchor

    - NNVC-11 anchor vs NNVC-12.0 anchor

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-11 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -0.62% | -0.96% | -0.90% | -0.26% | -1.79% | -2.13% | 100% | 97% |
| Class A2 | -0.59% | -1.31% | -0.13% | -0.42% | -1.23% | -0.52% | 100% | 99% |
| Class B | -0.58% | -1.01% | -0.41% | -0.21% | -1.51% | -0.82% | 97% | 96% |
| Class C | -0.77% | -1.21% | -0.10% | -0.28% | -1.19% | -0.25% | 94% | 94% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.64% | -1.11% | -0.37% | -0.28% | -1.42% | -0.87% | 97% | 96% |
| Class D | -1.07% | -0.54% | -0.22% | -0.52% | -0.93% | -0.37% | 93% | 92% |
| Class F | -0.44% | -0.65% | -0.32% | -0.54% | -1.57% | -0.88% | 95% | 90% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-11 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.16% | -0.11% | 0.32% | 0.41% | 0.38% | 0.21% | 99% | 90% |
| Class C | -0.55% | -1.30% | 0.50% | 0.05% | -2.00% | 0.59% | 100% | 98% |
| Class E | -0.11% | -3.78% | 2.22% | 0.56% | -4.88% | 0.39% | 101% | 78% |
| Overall | -0.28% | -1.42% | 0.85% | 0.33% | -1.73% | 0.39% | 100% | 89% |
| Class D | -0.69% | -0.89% | 1.37% | -0.07% | -3.36% | 2.04% | 100% | 101% |
| Class F | -0.26% | -0.49% | 0.52% | -0.28% | -1.35% | -1.18% | 101% | 93% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay P Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-11 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.29% | 0.60% | 0.65% | 0.54% | 1.82% | 1.30% | 98% | 91% |
| Class C | -0.64% | -1.62% | 0.09% | 0.16% | -2.29% | -0.01% | 101% | 100% |
| Class E | -0.29% | -3.94% | 1.87% | 0.29% | -4.75% | -0.93% | 103% | 90% |
| Overall | -0.41% | -1.27% | 0.77% | 0.35% | -1.19% | 0.31% | 100% | 94% |
| Class D | -0.87% | -0.59% | -0.21% | -0.12% | -1.97% | -0.07% | 100% | 100% |
| Class F | -0.36% | -0.51% | 0.17% | -0.46% | -1.46% | -0.58% | 100% | 99% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | All Intra Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-11 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -0.95% | -0.18% | 1.04% | -0.57% | -0.40% | -0.78% | 94% | 96% |
| Class A2 | -0.47% | -0.66% | -0.09% | -0.47% | -0.63% | -0.32% | 97% | 97% |
| Class B | -0.53% | -0.73% | 0.01% | -0.46% | -0.95% | -0.50% | 100% | 100% |
| Class C | -0.65% | -0.02% | 0.64% | -0.45% | -0.30% | -0.09% | 97% | 94% |
| Class E | -0.71% | -0.59% | -0.41% | -0.62% | -0.70% | -1.20% | 98% | 96% |
| Overall | -0.65% | -0.45% | 0.24% | -0.50% | -0.62% | -0.54% | 97% | 97% |
| Class D | -0.64% | 0.41% | 0.73% | -0.42% | -0.69% | -0.77% | 95% | 93% |
| Class F | -0.41% | -0.95% | -0.36% | -0.42% | -1.31% | -0.53% | 96% | 97% |

Note: Results from InterDigital, crosschecked by Nokia.

- Contributions

We have 2 contributions for AhG14 and 1 telco report.

| JVET-AL0043 | m71759 | [AHG11] [AHG14] Teleconference on NNVC | E. Alshina, F. Galpin |
| --- | --- | --- | --- |
| JVET-AL0100 | m71900 | AHG14: The extension of SADL library | W. Ma, N. Fu, W. Bao, Z. Chen (Wuhan Univ.) |
| JVET-AL0265 | m72339 | AhG14: SADL update | F. Galpin (InterDigital) |

Another contribution of interest for the software development in AhG11 was also available:

| JVET-AL0080 | m71875 | AHG11: Bit-exact reconstruction for NN video tools | L. Kerofsky, Y. Li, M. Karczewicz (Qualcomm) |
| --- | --- | --- | --- |

- Configurations

The following configurations is used to generate the different NNVC results.

The column “tested” is read as follow:

- Y: the configuration has been tested using the new NNVC software

- P: the results are the ones from previous NNVC software basis

- N: not tested.

The column “xcheck” is read as follow:

- Y: the test has been cross-checked

- P: no cross-checked performed but results are consistent with previous version on NNVC

- N: no cross-check available

| Name | Tools | Configuration |
| --- | --- | --- |
| NNVC VTM mode | None | encoder_xxx_vtm.cfg |
| NNVC Anchor/EE1 | Intra Pred + LOP.5 | encoder_xxx_nnvc.cfg |
| NNVC. HOP | Intra Pred + HOP.5 | encoder_xxx_nnvc.cfg + nn-based/HOP5.cfg |
| NNVC. VLOP | Intra Pred + VLOP.3 | encoder_xxx_vtm.cfg + nn-based/vlop3.cfg |
| SR | Super-resolution | nn-based/nnsr.cfg + nn-based/nnsr_classAx.cfg |
| PF | Adaptive post-filters | nn-based/nnpf/nnpf_xxx.cfg |
| ALOP | Intra pred+adaptive LOP | encoder_xxx_vtm.cfg +nn-based/intra.cfg+xxx |

Deprecated options:

| set0 | Loop filter set #0 | encoder_xxx_vtm.cfg + nn-based/NnlfOption_1.cfg |
| --- | --- | --- |
| set1 | Loop filter set #1 | encoder_xxx_vtm.cfg + nn-based/NnlfOption_2.cfg |
| set0+rdo | Loop filter set #0 + Rdo | nn-based/NnlfOption_1.cfg + --EncNnlfOpt=1 |
| set1+rdo+intra+temporal filter | Loop filter set #1 + Temporal filter | nn-based/NnlfOption_2.cfg + --NnlfSet1Multiframe=1 |

- Recommendations

The AHG recommends to:

- Continue to develop NNVC software.

- Improve the software documentation.

- Encourage people to report all (potential) bugs that they are finding using GitLab Issues functionality https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/issues

Encourage people to submit merge requests fixing identified bugs.

JVET-AL0015 JVET AHG report: Gaming content compression (AHG15) [S. Puri, J. Sauer (co-chairs), R. Chernyak, A. Duenas, L. Wang, V. Zakharchenko (vice chairs)]

Software developed by AHG15 consists of forks of VTM/ECM that have support for reading auxiliary data that is provided for sequences of classes G1/G3. A third repository contains scripts for the verification of the auxiliary information. The repositories can be found at https://vcgit.hhi.fraunhofer.de/jvet-ahg-gcc.

- Test results

Test results were obtained according to JVET-AJ2027. The list of sequences/classes is copied for convenience below:

| Class | Sequence name | Frame count | Low delay frame count | Frame rate | Bit depth | Intra | Random access | Low-delay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| G1 (SDR) | Level1_SDR | 600 | 300 | 60 | 10 | M | M | M |
|  | Darktree_SDR | 600 | 300 | 60 | 10 | M | M | M |
|  | ARPG2_SDR | 600 | 300 | 60 | 10 | M | M | M |
|  | DesertTown3_SDR | 600 | 300 | 60 | 10 | M | M | M |
|  | SunTemple3_SDR | 600 | 300 | 60 | 10 | M | M | M |
| G2 | GTAV | 600 | 300 | 60 | 8 | M | M | M |
|  | Minecraft | 600 | 300 | 60 | 8 | M | M | M |
|  | JianlingTemple | 600 | 300 | 60 | 8 | M | M | M |
|  | BaoleiYard | 300 | - | 60 | 8 | M | M | - |
| G3 (HDR) | Level1_HDR | 600 | 300 | 60 | 10 | O | O | O |
|  | Darktree_HDR | 600 | 300 | 60 | 10 | O | O | O |
|  | ARPG2_HDR | 600 | 300 | 60 | 10 | O | O | O |
|  | DesertTown3_HDR | 600 | 300 | 60 | 10 | O | O | O |
|  | SunTemple3_HDR | 600 | 300 | 60 | 10 | O | O | O |

For class G3 the HDR versions of Level1 and Darktree are still missing.

  - VTM-11.0ecm16.0 vs ECM-16.0

    - SDR

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over Anchor |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | -11.59% | -23.99% | -18.80% | 1153.56% | 617.35% | #REF! |
| Class G2 | -18.38% | -22.87% | -27.01% | 1216.37% | 694.86% | #REF! |
| Overall | -14.61% | -23.49% | -22.45% | 1181.07% | 650.67% | #REF! |

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over Anchor |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | -27.49% | -42.71% | -34.63% | 921.82% | 1545.85% | #REF! |
| Class G2* | -24.45% | -36.52% | -37.83% | 907.19% | 1517.11% | #REF! |
| Overall | -26.35% | -40.39% | -35.83% | 916.31% | 1535.01% | #REF! |

* ECM crashed for BaoleiYard. Results are averaged without BaoleiYard

|  | Low delay B Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over Anchor |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | -20.75% | -51.68% | -41.07% | 869.00% | 1318.26% | #REF! |
| Class G2 | -18.03% | -47.02% | -46.82% | 907.92% | 1380.22% | #REF! |
| Overall | -19.73% | -49.94% | -43.22% | 879.94% | 1335.67% | #REF! |

|  | Low delay P Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over Anchor |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | -19.03% | -57.76% | -49.24% | 807.46% | 1077.85% | #REF! |
| Class G2 | -15.86% | -54.20% | -54.39% | 831.03% | 1183.64% | #REF! |
| Overall | -17.84% | -56.43% | -51.17% | 816.22% | 1116.37% | #REF! |

    - HDR

There are still no HDR versions for Level1 and Darktree.

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM11.0ecm16.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class G3 | -24.71% | -12.14% | -11.47% | -43.01% | -39.88% | -11.45% | -43.19% | -39.59% | 1475% | 592% |

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM11.0ecm16.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class G3 | -40.68% | -26.92% | -28.06% | -58.09% | -56.31% | -28.28% | -58.58% | -56.50% | 1018% | 1413% |

|  | Low Delay B |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM11.0ecm16.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class G3 | -36.74% | -20.79% | -19.98% | -61.91% | -57.96% | -20.26% | -62.59% | -57.91% | 823% | 1132% |

|  | Low Delay P |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM11.0ecm16.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class G3 | -36.46% | -20.41% | -19.68% | -61.93% | -56.74% | -19.94% | -62.57% | -56.64% | 953% | 1028% |

- Input contributions

- AHG9/AHG15: Depth-adaptive picture scaling information SEI (JVET-AL0212)

- Recommendations

The AHG recommends to:

- Review input documents on gaming content compression

- Identify if/how auxiliary information can be used for coding of gaming content

JVET-AL0016 JVET AHG report: Generative face video compression (AHG16) [Y. Ye (chair), H.-B. Teo, Z. Lyu, S. McCarthy, S. Wang (vice chairs)]

Regarding the mandate on developing and maintaining the GFVC software, the AHG16 GFVC software tool and accompanying usage instructions and exemplar configurations for experimentation are maintained in the GIT repository at https://vcgit.hhi.fraunhofer.de/jvet-ahg-gfvc. During this AHG period, the GFVE pupil position model and its interface as provided in JVET-AK0164 were integrated into the AHG16 software repository. At this meeting, 4 contributions have been received on potential updates to the AHG16 software tool, including applying the generator as post-processing (JVET-AL0101), QP-adaptive GFVC (JVET-AL0102), updating the parameter translators (JVET-AL0147), and adding a color calibration process (JVET-AL0156) as post-processing.

Regarding coordination with AHG9 to develop the GFV SEI message, 4 contributions related to the GFV and GFVE SEI messages have been received. One of these contributions (JVET-AL0148) suggests to add support for GFV and GFVE SEI messages to HEVC and AVC. Other contributions suggest further enhancements and/or fixes to the GFV & GFVE SEI messages.

Related contributions

The following input contributions to this meeting are related to the activities of AHG16:

- JVET-AL0101, AHG16: Further experiments on VVC GFVC [L. Liu, C. Jung (Xidian Univ.)]

- JVET-AL0102, AHG9/AHG16: SEI message and further experiments on QP-adaptive GFVC [W. Kang, L. Liu, C. Jung (Xidian Univ.)]

- JVET-AL0147, AHG16: Refined parameter translator of generative face video coding [S. Yin, S. Wang, Z. Zhang (CityUHK), B. Chen, Y. Ye, R.-L. Liao, J. Chen (Alibaba)]

- JVET-AL0156, AHG9/AHG16: Colour calibration for generative face video coding [J. Chen, B. Chen, Y. Ye (Alibaba), S. Yin, S. Wang (CityUHK)]

Additionally, the following input contributions are related to the high-level syntax aspects of the generative face video (GFV) and/or generative face video enhancement (GFVE) SEI messages:

- JVET-AL0148, AHG9: Generative face video and generative face video enhancement SEI messages for HEVC and AVC [J. Chen, B. Chen, Y. Ye (Alibaba), S. Gehlot, G.-M. Su, P. Yin, S. McCarthy (Dolby), A. Trioux, F. Yang (Xidian Univ.), Y.-K. Wang (Bytedance), H.-B. Teo, J.-Y. Thong, K. Abe (Panasonic), Y. Xu, K. Yang, Y. Li (SJTU)]

- JVET-AL0155, AHG9: Further fixes and cleanup on GFV and GFVE SEI messages [J. Chen, B. Chen, Y. Ye (Alibaba)]

Recommendations

The AHG recommends to:

- Review all related contributions, including those on the high-level syntax aspects of the GFV and GFVE SEI messages;

- Continue AHG16 to study GFVC-related topics.

Contributions to AHG9 can be reviewed separately in HLS track.

JVET-AL0017 JVET AHG report: Testing of video coding technology beyond CTC (AHG17) [M. Wien (chair), E. Alshina, V. Baroncini, P. de Lagrange, Y. Ye (vice chairs)]

Two online AHG calls were held on 2025-02-06 and 2025-03-05, respectively. Furthermore, a hybrid AHG meeting was held at RWTH Aachen University between 2025-03-10 and 03-12. The reports of these meetings are available in JVET-AL0041.

  - Identification of test sequences and preparation of bitstreams

The set of sequences explored at the previous JVET meeting as well as newly proposed test sequences were encoded and the corresponding RD data and bitstreams were collected in a significant preparatory effort by the volunteers. The test sequences and proposed QP values for the ECM and the VTM were assessed during the hybrid AHG meeting in a number of subjective test sessions.

The discussion and the results of this activity are reported in JVET-AL0041.

Documents JVET-AL0041, JVET-AL0042, JVET-AL0045, JVET-AL0046, JVET-AL0047, JVET-AL0048, JVET-AL0049, and JVET-AL0050 have been initially reviewed in the hybrid AHG meeting. Documents JVET-AL0053 and JVET-AL0054 reflect the results of the testing activity at this meeting.

Related contributions

| JVET-AL0017 | JVET AHG report: Testing of video coding technology beyond CTC (AHG17) | M. Wien (chair), E. Alshina, V. Baroncini, P. de Lagrange, Y. Ye (vice chairs) |
| --- | --- | --- |
| JVET-AL0041 | AHG17: AhG meeting notes | M. Wien |
| JVET-AL0042 | AHG4/AHG17: Additional cropped HDR sequences for non-CTC testing | A. Filippov, J. Konieczny, Z. Zhi, V. Rufitskiy, H. Qin, T. Dong, X. Tang (TCL), S. Shen, Y. Qin, Y. Wang, X. Wang, Y. Guan (NERC-DTV) |
| JVET-AL0045 | [AHG17] Proposal of 6 candidates SDR HD clips | E. François, P. de Lagrange, C. Salmon-Legagneur (InterDigital) |
| JVET-AL0046 | [AHG17] UGC test sequences from vivo | Z. Lyu, Y. Li, C. Zhou, G. Wang (vivo) |
| JVET-AL0047 | Proposed draft text: Call for Evidence on video compression with capability beyond existing standards | J.-R. Ohm, M. Wien |
| JVET-AL0048 | [AHG17] Proposal of new sequences in UHD-SDR-RA category | M. Abdoli, A. Tissier, R. G. Youvalari, F. Plowman, M.-L. Champel (Xiaomi) |
| JVET-AL0049 | [AHG17] Proposal of 4 sequence candidates for the SDR-LD-HD category | R.-L. Liao, Y. Ye (Alibaba) |
| JVET-AL0050 | AHG17: UGC test sequences from Huawei | J. Pardo, Y. Zhao, Y. Sun, P. Liu, Y. Lu, C. Wang (Huawei) |
| JVET-AL0051 | [AHG17] Proposed CfE language regarding ultra low delay and error resilience | S. Wenger (Tencent) |
| JVET-AL0052 | AhG17: Comments on proposed draft text: Call for Evidence on video compression with capability beyond existing standards | E. Alshina, T. Solovyev (Huawei) |
| JVET-AL0053 | AHG17: Results of expert viewing of non-CTC sequences | M. Wien, A. Wieckowski, K. Andersson |
| JVET-AL0054 | AHG17: adjusted QPs for non-CTC sequences | E. François (InterDigital), E. Alshina (Huawei), P. Nikitin (Qualcomm) |
| JVET-AL0154 | AHG17: Resampling test for large resolution sequences | G. Clare (bcom), F. Henry (Orange), M. Tarchouli (Ateme), M. Riviere (Ateme) |
| JVET-AL0207 | AHG10/AHG17: On matching target bitrate for subjective quality evaluation | K. Andersson, P. Wennersten (Ericsson) |
| JVET-AL0239 | [AHG17] Proposal of a new sequence in HD-SDR-LB category | A. Tissier, M. Abdoli, R. G. Youvalari, F. Plowman, M. -L. Champel (Xiaomi) |
| JVET-AL0242 | [AHG7][AHG17] ECM performance under different MTT configurations | S. Puri, K. Naser, F. Le Léannec, E. François (InterDigital) |
| JVET-AL0245 | [AHG7][AHG17] Further consideration of common test condition | T. Ikai, K.-W. Liang (Sharp) |

Recommendations

The AHG recommends:

- To review the remaining input contributions related to AHG17 during the meeting.

- To review the documents JVET-AL0053 and JVET-AL0054 and take their results into consideration for the development of an enhanced draft CfE document.

- To progress the draft CfE document with a proposed set of test categories with respective test sequences.

JVET-AL0018 JVET AHG report: Ultra-low latency and packet loss resilience (AHG18) [S. Ikonin, S. Wenger, V. Zakharchenko (co-chairs), S. Deshpande, S. Fößel, C. Kim, X. Ma, S. Puri, J. Ström (vice-chairs)]

The repository for AHG related simulation software was established, and can be accessed at: https://vcgit.hhi.fraunhofer.de/jvet-ahg-ull/VVCSoftware_VTM.

  - Teleconferences

Two teleconferences were held on February 19th and March 14th, with 51 and 26 participant correspondingly.

The following topics were discussed:

- VTM + network model simulation software introduction.

- Network bandwidth simulation point introduction.

- Transmission latency derivation logic based on end-to-end latency.

- ULL metrics calculation example and correspondent excel template.

- Scalable transmission scenario simulation example.

- No-feedback IPPI-based transmission scenario example.

- Latency calculation consistency fix, and list of other modifications.

The meeting minutes, presented materials and video demonstrations are available in JVET-AL0044.

- Related contributions

A total of 8 contributions are identified relating to the mandates of AHG18. They are listed below. Some contributions also relate to the work of AHG17 and AHG9.

| Report |  |  |
| --- | --- | --- |
| JVET-AL0018 | JVET AHG report: Ultra-low latency and packet loss resilience (AHG18) | S. Ikonin, S. Wenger, V. Zakharchenko (co-chairs), S. Deshpande, S. Fößel, C. Kim, X. Ma, S. Puri, J. Ström (vice-chairs) |
| JVET-AL0044 | AHG18: Teleconference on ultra-low latency and packet loss resilience | S. Ikonin, S. Wenger, V. Zakharchenko (co-chairs), S. Deshpande, J. Ström, X. Ma, C. Kim, S. Puri, S. Fößel (vice-chairs) |
| Test condition and evaluation methodology |  |  |
| JVET-AL0051 | [AHG17] Proposed CfE language regarding ultra low delay and error resilience | S. Wenger (Tencent) |
| JVET-AL0176 | AHG18: Proposed methodology and test conditions for ultra-low latency and error resilience performance evaluation | S. Ikonin, X. Ma, I. Gribushin, M. Sychev, E. Alshina (Huawei) |
| Proposals |  |  |
| JVET-AL0069 | AHG9/AHG18: Error recovery SEI message | J. Boyce, M. M. Hannuksela (Nokia) |
| JVET-AL0073 | AHG9/AHG18: Loss recovery information SEI message | C. Kim, H. Tan, J. Nam, J. Lee, J. Lim, S. Kim (LGE) |
| JVET-AL0200 | AHG18: Simulation software description | S. Ikonin, V. Khamidullin, R. Shabaev, I. Gribushin, M. Sychev, E. Alshina (Huawei) |
| JVET-AL0201 | AHG18: Performance evaluation of VTM under proposed ULL test conditions and proposed software modifications | S. Ikonin, V. Khamidullin, R. Shabaev, I. Gribushin, M. Sychev, E. Alshina (Huawei) |

- Recommendations

The AHG recommends:

- Review all input contributions.

- Discuss test conditions and evaluation methodology.

- Collect test cases and specific requirements.

- Continue development of simulation software.

- Continue the study of ultra-low latency and packet loss resilience technologies in JVET.

It was suggested to make the software available to all JVET experts.

# Project development (41)

## AHG1: Development, deployment and advertisement of standards (6)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

JVET-AL0020 Deployment status of the HEVC standard [G. J. Sullivan]

JVET-AL0021 Deployment status of the VVC standard [G. J. Sullivan]

JVET-AL0217 CICP support for monochrome content [J. Boyce, T. Biatek, M. M. Hannuksela (Nokia)]

JVET-AL0218 CICP support for colour mapping [J. Boyce, T. Biatek, M. M. Hannuksela (Nokia)]

JVET-AL0235 On the immersive requirements for the next generation video codec [B. Kroon, C. Varekamp (Philips)]

For discussion in joint meeting with parent bodies.

JVET-AL0278 Huawei comments on use case and requirements for NextGenVideoCodec [T. Solovyev, S. Ikonin, E. Alshina (Huawei)] [late]

For discussion in joint meeting with parent bodies.

## AHG2: Text development and errata reporting (2)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

JVET-AL0223 AHG2: Corrections and clarifications for profile-related aspects of the draft text for HEVC [G. J. Sullivan (Dolby Labs)]

JVET-AL0288 AHG2, AHG9, AHG13: on FGC SEI reference picture size [P. de Lagrange (InterDigital)] [late] [miss]

See section 4.2

## AHG3: Software development (0)

This section is kept as a template for future use.

## AHG3: Test conditions (3)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

JVET-AL0197 AHG3: Intra period for random access set in configuration files [A. Dziembowski, D. Mieloch (PUT)]

JVET-AL0213 AHG7: On common test conditions for fixed-QP objective rate-distortion testing of "low delay" encoding [G. J. Sullivan, P. Yin, T. N. Canh (Dolby Labs)]

JVET-AL0245 [AHG7][AHG17] Further consideration of common test condition [T. Ikai, K.-W. Liang (Sharp)] [late]

JVET-AL0304 Crosscheck of JVET-AL0245 ([AHG7][AHG17] Further consideration of common test condition) [J. Pardo (Huawei)] [late] [miss]

## AHG4: Subjective quality testing and verification testing (1)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

JVET-AL0289 AHG4: potential updates for VVC multi-layer verification test plan [P. de Lagrange (InterDigital)] [late]

## AHG4: Test and training material (0)

This section is kept as a template for future use.

Proposals on new test materials are also included in 4.16 and 5.1.4.

## AHG5: Conformance test development (0)

This section is kept as a template for future use.

## AHG7: ECM tool assessment (5+2)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

JVET-AL0083 AHG7: On Luma-Chroma Gain Rebalancing [X. Li (Google), Y. Wang, K. Zhang, L. Zhang (Bytedance)]

JVET-AL0171 AHG7: Tools interaction analysis of ECM16.0 R. Ishimoto, [Z. Fan, T. Chujoh, T. Ikai (Sharp)]

JVET-AL0194 AHG7: Assessment Perspectives of Codec/Coding Tools [X. Li (Google)]

JVET-AL0213 AHG7: On common test conditions for fixed-QP objective rate-distortion testing of "low delay" encoding [G. J. Sullivan, P. Yin, T. N. Canh (Dolby Labs)]

See also section 4.4

JVET-AL0242 [AHG7][AHG17] ECM performance under different MTT configurations [S. Puri, K. Naser, F. Le Léannec, E. François (InterDigital)] [late]

JVET-AL0245 [AHG7][AHG17] Further consideration of common test condition [T. Ikai, K.-W. Liang (Sharp)] [late]

See also section 4.4

JVET-AL0247 [AHG-7] On Max Transform Size in ECM [K. Naser, F. Le Léannec, S. Puri, C. Bonnineau, E. François (InterDigital)] [late]

JVET-AL0266 Crosscheck of JVET-AL0247 On Max Transform Size in ECM [X. Li (Google)] [late] [miss]

## AHG8: Optimization of encoders and receiving systems for machine analysis of coded video content (1)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

JVET-AL0152 AHG8: Dense QP coding results of combining adaptive temporal resampling, pre-processing, ROI-based adaptive QP, QP offset adjustment for higher temporal layers and post-processing algorithms for machine vision [S. Wang, J. Chen, Y. Ye, B. Li (Alibaba), S. Wang (CityUHK)]

## AHG10: Encoding algorithm optimization (3+2)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

JVET-AL0074 AHG10: Teleconference on encoder optimization [P. de Lagrange]

JVET-AL0055 AhG10: On constrained encoder complexity configuration of VTM [T. Solovyev, J. Sauer, J. E. F. Pardo, E. Alshina (Huawei)]

JVET-AL0263 Crosscheck of JVET-AL0055 (AhG10: On constrained encoder configuration of VTM) [H. Le Lais, P. de Lagrange (InterDigital)] [late]

JVET-AL0114 AHG 10: On intra mode selection [Y. Yu, L. Zhang, Z. Sun, J. Gan, F. Wang, H. Yu, D. Wang (OPPO)]

JVET-AL0207 AHG10/AHG17: On matching target bitrate for subjective quality evaluation [K. Andersson, P. Wennersten (Ericsson)]

JVET-AL0233 Information on low complexity encoding experiments [L. Li, M. W. Park, M. Park, Y. Kim, K. P. Choi (Samsung)]

See also section 4.16

## AHG13: Film grain synthesis (1+2)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

JVET-AL0086 AHG9/AHG13: On Spatial Resolution for the Film Grain Characteristics (FGC) SEI message [J. Samuelsson-Allendes, S. Deshpande (Sharp)]

JVET-AL0282 AHG13: Film grain analysis improvement [M. Radosavljević, F. Lefebvre, Z. Ameur, P. de Lagrange (InterDigital] [late] [miss]

JVET-AL0288 AHG2, AHG9, AHG13: on FGC SEI reference picture size [P. de Lagrange (InterDigital)] [late] [miss]

See section 4.2

Refer +204, 211, should all be discussed together

## Implementation studies (0)

This section is kept as a template for future use.

## Profile/tier/level specification (0)

This section is kept as a template for future use.

## AHG15: Gaming content compression (0)

This section is kept as a template for future use.

## AHG16: Generative face video (2+2)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

Also refer to section 6.1.4.

JVET-AL0101 AHG16: Further experiments on VVC GFVC [L. Liu, C. Jung (Xidian Univ.)]

JVET-AL0102 AHG9/AHG16: SEI message and further experiments on QP-adaptive GFVC [W. Kang, L. Liu, C. Jung (Xidian Univ.)]

See also section 6.2.6

JVET-AL0147 AHG16: Refined parameter translator of generative face video coding [S. Yin, S. Wang, Z. Zhang (CityUHK), B. Chen, Y. Ye, R.-L. Liao, J. Chen (Alibaba)]

JVET-AL0156 AHG9/AHG16: Colour calibration for generative face video coding [J. Chen, B. Chen, Y. Ye (Alibaba), S. Yin, S. Wang (CityUHK)]

See also section 6.2.6

## AHG17: CfE preparation (14+4)

Contributions in this area were discussed during 1405–1505 and 1525–1725 on Wednesday 26 March 2025 (chaired by JRO).

Some aspects discussed under section 4.4 could also be relevant here.

JVET-AL0041 AHG17: AhG meeting notes [M. Wien]

An overview on the activities was given, and the viewing methodology was explained (results in JVET-AL0053)

- ACR MOS for some sequences was available from Geneva, for additional sequences was acquired in Aachen

- Direct comparison of quality of successive rate points per sequence

- DCR MOS was acquired for some selected sequences (including UGC)

It was generally concluded that a suitable set of non-CTC sequences would be available in the 7 categories for a Call for Evidence.

JVET-AL0042 AHG4/AHG17: Additional cropped HDR sequences for non-CTC testing [A. Filippov, J. Konieczny, Z. Zhi, V. Rufitskiy, H. Qin, T. Dong, X. Tang (TCL), S. Shen, Y. Qin, Y. Wang, X. Wang, Y. Guan (NERC-DTV)]

Was presented in AHG meeting, and sequences included in viewing

JVET-AL0045 [AHG17] Proposal of 6 candidates SDR HD clips [E. François, P. de Lagrange, C. Salmon-Legagneur (InterDigital)]

Was presented in AHG meeting, and sequences included in viewing

JVET-AL0046 [AHG17] UGC test sequences from vivo [Z. Lyu, Y. Li, C. Zhou, G. Wang (vivo)]

Was presented in AHG meeting, and sequences included in viewing

JVET-AL0047 Proposed draft text: Call for Evidence on video compression with capability beyond existing standards [J.-R. Ohm, M. Wien]

V2 (modified version from AHG17 meeting) was presented 1625 UTC on Wednesday 26 March.

Concern was raised about the idea which came up in the AHG17 meeting, such that ECM would be a “submission” to the CfE. It was argued that ECM does not provide a reasonable “evidence” in terms of complexity tradeoff, and it might be strange if JVET responds to its own CfE. On the other hand, ECM could be seen as a reasonable comparison point in terms of compression benefit, where good evidence could be provided by proposals which achieve something comparable in compression at lower complexity.

It was therefore agreed that ECM should be retained as an anchor (as in v1 of JVET-AL0047), it could also be a reasonable comparison point for judging the compression/complexity tradeoff of other submissions.

It could anyway not be avoided that some submission uses ECM or part of it as a basis.

In principle, JVET would be prepared to define testing conditions for the purpose of investigating compression benefit (section 3 of JVET-AL0047), whereas more discussion is needed about how to investigate the complexity aspects (section 4 of JVET-AL0047), e.g. based on JVET-AL0052, to be further conducted during the meeting.

It is asserted be useful in any case to perform a “dry run” for the CfE by the next meeting, which should include rate-matched VTM and ECM anchors, and potentially other configuration e.g. from the investigations of AHG7/AHG10. It would be useful to have this as an output document (title tbd), also to allow companies knowing about the purpose to have enough time for preparing submissions for the actual CfE.

Continuation of review of other documents in this category on Friday; meeting with parent bodies should be conductedon Monday.

JVET-AL0048 [AHG17] Proposal of new sequences in UHD-SDR-RA category [M. Abdoli, A. Tissier, R. G. Youvalari, F. Plowman, M.-L. Champel (Xiaomi)]

Was presented in AHG meeting, and sequences included in viewing

JVET-AL0049 [AHG17] Proposal of 4 sequence candidates for the SDR-LD-HD category [R.-L. Liao, Y. Ye (Alibaba)]

Was presented in AHG meeting, and sequences included in viewing

JVET-AL0050 AHG17: UGC test sequences from Huawei [J. Pardo, Y. Zhao, Y. Sun, P. Liu, Y. Lu, C. Wang (Huawei)]

Was presented in AHG meeting, and sequences included in viewing

JVET-AL0051 [AHG17] Proposed CfE language regarding ultra low delay and error resilience [S. Wenger (Tencent)]

JVET-AL0052 AhG17: Comments on proposed draft text: Call for Evidence on video compression with capability beyond existing standards [E. Alshina, T. Solovyev (Huawei)]

JVET-AL0053 AHG17: Results of expert viewing of non-CTC sequences [M. Wien, A. Wieckowski, K. Andersson]

DCR results which had not yet been readily evaluated during the AHG meeting were presented.

For some UGC sequences, MOS for original is available, which are usually slightly higher than the VTM or ECM at highest rate point (not for all done ). This might be useful to get information  in particular if the original has already visible compression artifacts.

Generally, it can be concluded that the DCR test also could work well for UGC content (where however in most cases the “original” quality was quite high)

It was suggested to conduct a comparison of DCR-MOS vs. ACR-MOS.

It was requested to generate a document which includes snapshots of the selected sequences (as per JVET-AL0041), results in terms of MOS, and suggested rate points from JVET-AL0054.

JVET-AL0054 AHG17: adjusted QPs for non-CTC sequences [E. François (InterDigital), E. Alshina (Huawei), P. Nikitin (Qualcomm)]

This contribution comprises an updated spreadsheet from JVET-AL0053 with proposed adjusted QPs based on results of viewing performed during interim Ahg17 meeting (March 10-12, 2025). The target of QPs adjustment was to provide recommendations for target rates per sequence, covering a quality range from very bad to very good, with four well distinguishable (during viewing) intermediate steps.

In v1, the QPs adjustment was made for the sequences initially considered for CfE before the interim meeting.

During this interim meeting, new sequences have been considered and the set of CfE candidate sequences has been updated. In v2 of the current document, the QPs adjustment has been completed for all the  candidate sequences selected for CfE during the interim meeting.

Based on the adjustment of QPs, recommendations for target rates for each CfE candidate sequence are provided.

It was suggested to make the “quantization” of rates more coarse for highest bit rate points.

It was further suggested to perform the combination with the MOS results (DCR, or ACR when DCR not available) from JVET-AL0053 and check if the proposed rates are also useful in terms of quality ranges of ECM.

It was commented that correction seems necessary for Gregory Cactus (highest rate may be too high, lowest too low).

It was requested to generate plots where

- When DCR results are available, mark the position of the suggested rates compared to the available rates

- When ACR results are available (only for lowest and highest rate points, and only for ECM), to generate plots where the suggested rates are marked in comparison with the rates used in the test

The rates should be adjusted such that likely ECM quality at lowest rate is around MOS 3, but not approaching MOS 2, and at highest rate not higher than MOS 8.

It is generally agreed that for most sequences the suggested rates are already more or less appropriate for a CfE, which is also meant as a “dry run” for a later CfP (if successful).

JVET-AL0154 AHG17: Resampling test for large resolution sequences [G. Clare (bcom), F. Henry (Orange), M. Tarchouli (Ateme), M. Riviere (Ateme)]

JVET-AL0207 AHG10/AHG17: On matching target bitrate for subjective quality evaluation [K. Andersson, P. Wennersten (Ericsson)]

See also section 4.10

JVET-AL0233 Information on low complexity encoding experiments [L. Li, M. W. Park, M. Park, Y. Kim, K. P. Choi (Samsung)]

JVET-AL0239 [AHG17] Proposal of a new sequence in HD-SDR-LB category [A. Tissier, M. Abdoli, R. G. Youvalari, F. Plowman (Xiaomi)] [late]

JVET-AL0242 [AHG7][AHG17] ECM performance under different MTT configurations [S. Puri, K. Naser, F. Le Léannec, E. François (InterDigital)] [late]

See also section 4.8

JVET-AL0245 [AHG7][AHG17] Further consideration of common test condition [T. Ikai, K.-W. Liang (Sharp)] [late]

See also section 4.4

## AHG18 Ultra-low latency and packet loss resilience (3+3)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

JVET-AL0044 AHG18: Teleconference on ultra-low latency and packet loss resilience [S. Ikonin, S. Wenger, V. Zakharchenko (co-chairs), S. Deshpande, J. Ström, X. Ma, C. Kim, S. Puri, S. Fößel (vice-chairs)]

JVET-AL0069 AHG9/AHG18: Error recovery SEI message [J. Boyce, M. M. Hannuksela (Nokia)]

JVET-AL0073 AHG9/AHG18: Loss recovery information SEI message [C. Kim, H. Tan, J. Nam, J. Lee, J. Lim, S. Kim (LGE)]

JVET-AL0176 AHG18: Proposed methodology and test conditions for ultra-low latency and error resilience performance evaluation [S. Ikonin, X. Ma, I. Gribushin, M. Sychev, E. Alshina (Huawei)]

JVET-AL0200 AHG18: Simulation software description [S. Ikonin, V. Khamidullin, R. Shabaev, I. Gribushin, M. Sychev, E. Alshina (Huawei)]

JVET-AL0201 AHG18: Performance evaluation of VTM under proposed ULL test conditions and proposed software modifications [S. Ikonin, V. Khamidullin, R. Shabaev, I. Gribushin, M. Sychev, E. Alshina (Huawei)]

# Low-level tool technology proposals (104)

## AHG11/AHG14: Neural network-based video coding (27)

### Summary and BoG reports (2)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

JVET-AL0023 EE1: Summary report of exploration experiment on neural network-based video coding [E. Alshina, R. Chang, F. Galpin, Yue Li, Yun Li, M. Santamaria, J. Ström, Z. Xie (EE coordinators)]

JVET-AL0043 [AHG11] [AHG14] Teleconference on NNVC [E. Alshina, F. Galpin]

### EE1 contributions: Neural network-based video coding (11)

Contributions in this area were discussed in the context of the EE summary report JVET-AL0023.

JVET-AL0084 EE1-1.2: LOP5 improvement with parallel 1x3/3x1 Backbone [T. Shao, P. Yin, S. McCarthy (Dolby), J. N. Shingala, A. Shyam, A. Suneja, S. P. Badya (Ittiam)]

JVET-AL0246 Crosscheck of JVET-AL0084 (EE1-1.2: LOP5 improvement with parallel 1x3/3x1 Backbone) [Y. Li (Bytedance)] [late]

JVET-AL0085 EE1-3.1: NNVC-LOP5 retraining with additional BVI-AOM dataset [A. Suneja, J. N. Shingala, A. Shyam, S. P. Badya (Ittiam), T. Shao, P. Yin, S. McCarthy (Dolby)]

JVET-AL0104 EE1-2.1: Lightweight Multiscale Reference Frame Generation for VVC Inter Coding [P. Li, C. Jung, Q. Qin (Xidian Univ.)]

JVET-AL0297 Crosscheck of JVET-AL0104 (EE1-2.1: Lightweight Multiscale Reference Frame Generation for VVC Inter Coding) [L. Murn (Nokia)] [late]

JVET-AL0105 EE1-2.2: RA/LDB Unified Reference Frame Synthesis for VVC Inter Coding [Q. Qin, C. Jung (Xidian Univ.)]

JVET-AL0298 Crosscheck of JVET-AL0105 (EE1-2.2: RA/LDB Unified Reference Frame Synthesis for VVC Inter Coding) [L. Murn (Nokia)] [late]

JVET-AL0144 EE1-3.1: Retraining LOP4 and LOP5 using extended dataset from BVI-AOM [D. Liu, J. Ström, M. Damghanian, P. Wennersten (Ericsson)]

JVET-AL0145 EE1-1.4: Conditional loop-filter [M. Santamaria, F. Cricri (Nokia)]

JVET-AL0284 Crosscheck of JVET-AL0145 (EE1-1.4: Conditional loop-filter) [J. Ström (Ericsson)] [late] [miss]

JVET-AL0164 EE1-1.1: Multiscale blocks in LOP5 and VLOP3 filters [R. Yang (Nokia)]

JVET-AL0173 Crosscheck of JVET-AL0164 (EE1-1.1: Multiscale blocks in LOP5 and VLOP3 filters) [Y. Li (Qualcomm)] [late]

JVET-AL0169 EE1-1.3 Dimension-wise decomposed representation of multiplier for content-adaptive loop filtering [Z. Xu, J. Konieczny, A. Filippov, C. Hollmann, V. Rufitskiy, T. Dong (TCL)]

JVET-AL0185 Crosscheck of JVET-AL0169 (EE1-1.3 Dimension-wise decomposed representation of multiplier for content-adaptive loop filtering) [M. Santamaria (Nokia)] [late] [miss]

JVET-AL0187 EE1-3.1: NNVC-LOP4 and LOP5 retraining with additional BVI-AOM dataset [T. Dumas, A. Monier, F. Galpin (InterDigital)] [late]

JVET-AL0189 EE1-3.2 – NNVC-HOP5 retraining adding BVI-AOM [F. Galpin (InterDigital)] [late]

JVET-AL0190 EE1-3.3 – NNVC-VLOP3 retraining adding BVI-AOM [F. Galpin, Z. Ameur (InterDigital), R. Chang, L. Wang, X. Xu, S. Liu (Tencent)] [late]

### EE1 related and beyond-EE contributions: Neural network-based video coding (14)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

JVET-AL0060 Sandwiched Compression: Repurposing Standard Codecs with Neural Network Wrappers [O. Guleryuz (Google)]

JVET-AL0080 AHG11: Bit-exact reconstruction for NN video tools [L. Kerofsky, Y. Li, M. Karczewicz (Qualcomm)]

JVET-AL0121 AHG11: Sample-based adaptive blending weight selection for LOP [H. Kwon, H. Ko (HYU)]

JVET-AL0136 AHG11: Over-Parameterized LOP In-Loop Filter [J. Han, C. Jung, Q. Qin (Xidian Univ.)]

JVET-AL0137 AHG11: Backbone Block Enhancement of LOP In-Loop Filter with Over-Parameterized Training and Variable Channels [J. Han, C. Jung, Q. Qin (Xidian Univ.)]

JVET-AL0165 AHG11: Training NNSR using Reparameterization and Progressive Activation [H. Cho, S. Bahk, H. Kim (KHU), D. Kim, S. Lim (ETRI)]

JVET-AL0166 EE1 related: Improved VLOP with Attention [Y. Li, L. Kerofsky, M. Karczewicz (Qualcomm)]

JVET-AL0167 EE1 related: Further simplification of VLOP with Attention [Y. Li, L. Kerofsky, M. Karczewicz (Qualcomm)]

JVET-AL0184 EE1-related: Deep Reference Frame Generation for Inter Prediction Enhancement [D. Ding, X. Chen, Z. Chen (Wuhan Univ.)]

JVET-AL0196 AHG 11: Neural Network Coded Reference Frame for Intra Coding with Residual Coding and Intra Blocks [F. Brand, T. Solovyev, E. Alshina (Huawei)]

JVET-AL0203 [AHG11] A Hybrid Framework Integrating End-to-End Learned Image Codec with Conventional Codec [N. Zou, A. Hallapuro, F. Cricri, H. Zhang, M. M. Hannuksela (Nokia)]

JVET-AL0230 AhG11/AhG12: Performance of the NNVC ILF in ECM [D. Rusanovskyy, K. Panusopone, S. Hong, L. Wang (Nokia)]

See also sec. 5.2.4.4

JVET-AL0243 [AHG11] Multilayer framework for supporting a hybrid codec using End-to-End Learned Image Codec and Conventional Video Codec [F. Urban, Y. Chen, F. Galpin, E. François (InterDigital)] [late]

JVET-AL0250 AHG11: Cross-component enhanced NNSR [T. Yang, W.-X. He, Y.-Q. Zhu, J.-D. Ye, X.-T. Xie, J.-S. Gong, Q. Liu (HUST), Z.-Y. Lv (vivo)] [late]

JVET-AL0291 EE1-related: Recommendations for resolving mismatches between LOP5 description and implementation [N. Le, F. Cricri (Nokia)] [late]

### SADL and NNVC implementation, CTC (2)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

JVET-AL0100 AHG14: The extension of SADL library [W. Ma, N. Fu, W. Bao, Z. Chen (Wuhan Univ.)]

JVET-AL0265 AhG14: SADL update [F. Galpin (InterDigital)] [late]

## AHG6/AHG12: Enhanced compression beyond VVC capability (76)

### Summary and BoG reports (1)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

JVET-AL0024 EE2: Summary report of exploration experiment on enhanced compression beyond VVC capability [V. Seregin, J. Chen, R. Chernyak, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang (EE coordinators)]

### EE2 contributions: Enhanced compression beyond VVC capability (31)

There was no presentation or discussion about specific proposals in this category.

For actions decided to be taken, see section 5.2.1, unless otherwise noted.

JVET-AL0079 EE2-3.5: On affine motion compensation [H. Huang, R. Yu, Z. Zhang, Y. Zhang, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AL0273 Crosscheck of JVET-AL0079 (EE2-3.5: On affine motion compensation) [C. Ma (Kwai)] [late]

JVET-AL0081 EE2-3.9: Extended BDOF usage for MV refinement [R. Yu, H. Huang, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AL0252 Crosscheck of JVET-AL0081 (EE2-3.9: Extended BDOF usage for MV refinement) [Y. Wang (Bytedance)] [late] [miss]

JVET-AL0106 EE2-2.4: block vector guided EIP [Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AL0307 Crosscheck of JVET-AL0106 (EE2-2.4: block vector guided EIP) [K. Panusopone (Nokia)] [late] [miss]

JVET-AL0107 EE2-2.11: a combination of EE2-2.4 and EE2-2.10 [Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO), Z. Lv, C. Zhou, G. Wang (vivo)]

JVET-AL0296 Crosscheck of JVET-AL0107 (EE2-2.11: a combination of EE2-2.4 and EE2-2.10) [Z. Deng (Bytedance)] [late] [miss]

JVET-AL0108 EE2-2.7: Block vector guided DIMD [L. Zhang, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AL0280 Crosscheck of JVET-AL0108 (EE2-2.7: Block vector guided DIMD) [Y. Wang (Bytedance)] [late] [miss]

JVET-AL0124 EE2-2.10: Multiple filter taps for EIP [Z. Lyu, C. Zhou, G. Wang (vivo)]

JVET-AL0258 Crosscheck of JVET-AL0124 (EE2-2.10 Multiple filter taps for EIP) [Z. Deng (Bytedance)] [late] [miss]

JVET-AL0125 EE2-2.6: Improvement on MPM [G. Wang, C. Zhou, Z. Lv (vivo)]

JVET-AL0262 Crosscheck of JVET-AL0125 (EE2-2.6a/c: Improvement on MPM) [J. Fu, Z. Li (PKU)] [late] [miss]

JVET-AL0295 Crosscheck of JVET-AL0125 (EE2-2.6b/c: Improvement on MPM) [Y. Liu, Y. Huo (Transsion)] [late] [miss]

JVET-AL0126 EE2-2.3: CCP merge mode with adjustment [Y. Wang, K. Zhang, W. Yin, Z. Deng, N. Zhang, L. Zhao, M. Salehifar, L. Zhang (Bytedance)]

JVET-AL0274 Crosscheck of JVET-AL0126 (EE2-2.3: CCP merge mode with adjustment) [C. Ma (Kwai)] [late] [miss]

JVET-AL0134 EE2-3.4: Joint reordering of GPM split modes and partition indices [C. Ma, X. Xiu, X. Wang (Kwai)]

JVET-AL0135 EE2-5.3: Cross-Chroma Input for Chroma-ALF and CC-ALF [W. Yin, K. Zhang, H. Liu, Y. Wang, Z. Deng, N. Zhang, L. Zhao, M. Salehifar, L. Zhang (Bytedance), C. Ma, X. Xiu, C.-W. Kuo, X. Wang (Kwai)]

JVET-AL0277 Crosscheck of JVET-AL0135 (EE2-5.3: Cross-Chroma Input for Chroma-ALF and CC-ALF)) [N. Hu (Qualcomm)] [late] [miss]

JVET-AL0142 EE2-5.2: CCSAO with reused CTU control [C.-W. Kuo, X. Xiu, X. Wang (Kwai)]

JVET-AL0253 Crosscheck of JVET-AL0142 (EE2-5.2: CCSAO with Reused CTU Control) [W. Yin (Bytedance)] [late] [miss]

JVET-AL0143 EE2-1.1: Chroma partition prediction in separate tree condition [P.-H. Lin, J.-L. Lin, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AL0279 Crosscheck of JVET-AL0143 (EE2-1.1: Chroma partition prediction in separate tree condition) [R.-L. Liao (Alibaba)] [late] [miss]

JVET-AL0285 Crosscheck of JVET-AL0143 (EE2-1.1: Chroma partition prediction in separate tree condition) [C.-W. Kuo (Kwai)] [late] [miss]

JVET-AL0149 EE2-2.12: Non-adjacent DIMD for TMRL [V. Rufitskiy, A. Filippov, T. Dong, H. Qin, J. Konieczny, K. Ding (TCL)]

JVET-AL0151 EE2-3.2: Extension on spatial and temporal merge candidates [J.-L. Lin, P.-H. Lin, Z. Zhang, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AL0293 Crosscheck of JVET-AL0151 (EE2-3.2: Extension on spatial and temporal merge candidates) [L. Zhang (OPPO)] [late] [miss]

JVET-AL0153 EE2-5.1: ALF-CCCM [P. Astola, I. Jumakulyyev, D. Bugdayci Sansli, J. Lainema (Nokia)]

JVET-AL0238 Crosscheck of JVET-AL0153 (EE2-5.1: ALF-CCCM) [R. G. Youvalari (Xiaomi)] [late] [miss]

JVET-AL0157 EE2-3.3: Additional inter merge candidates [N. Zhang, K. Zhang, Z. Deng, L. Zhao, Y. Wang, W. Yin, M. Salehifar, L. Zhang (Bytedance), J.-L. Lin, P.-H. Lin, Z. Zhang, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AL0269 Crosscheck of JVET-AL0157 (EE2-3.3: Additional inter merge candidates) [X. Li (Alibaba)] [late] [miss]

JVET-AL0294 Crosscheck of JVET-AL0157 (EE2-3.3e: Additional inter merge candidates) [L. Zhang (OPPO)] [late] [miss]

JVET-AL0158 EE2-4.3: On coefficient level binarization in Transform Skip [M. Abdoli, R. G. Youvalari, F. Plowman, A. Tissier (Xiaomi)]

JVET-AL0240 Crosscheck of JVET-AL0158 (EE2-4.3: On coefficient level binarization in Transform Skip) [P. Astola (Nokia)] [late] [miss]

JVET-AL0160 EE2-3.1: Subblock-based spatial MVP [Z. Deng, K. Zhang, N. Zhang, L. Zhao, Y. Wang, W. Yin, M. Salehifar, L. Zhang (Bytedance)]

JVET-AL0251 Crosscheck of JVET-AL0160 (EE2-3.1: Subblock-based spatial MVP) [Z. Lyu (vivo)] [late] [miss]

JVET-AL0161 EE2-3.7: On interpolation filter for template matching [Z. Dai, R.-L. Liao, J. Chen, X. Li, Y. Ye (Alibaba)]

JVET-AL0255 Crosscheck of JVET-AL0161 (EE2-3.7: On interpolation filter for template matching)) [L. Xu (OPPO)] [late] [miss]

JVET-AL0162 EE2-3.8: CMVP extension for constructed affine merge candidates [C. Li, R.-L. Liao, J. Chen, Y. Ye (Alibaba)]

JVET-AL0261 Crosscheck of JVET-AL0162 (EE2-3.8: CMVP extension for constructed affine merge candidates) [L. Zhao (Bytedance)] [late] [miss]

JVET-AL0168 EE2-5.4: On the performance improvement of NN-based ILF in ALF [Y. Li, M. Karczewicz, J. Wang, L.Kerofsky, H. Wang, N. Hu, R. Yu, M. Coban, V. Seregin (Qualcomm)]

JVET-AL0181 EE2: Tests 4.1 on Advanced SBT with direction and position inference [G. Laroche, P. Onno (Canon)]

JVET-AL0188 EE2-2.5: Flip-aware BV prediction in SGPM [J. Huo, Y. Fei, L. Wang, Y. Ma, F. Yang (Xidian Univ.)]

JVET-AL0237 Crosscheck of JVET-AL0188 (EE2-2.5: Flip-aware BV prediction in SGPM) [X. Li (Alibaba)] [late] [miss]

JVET-AL0191 EE2-2.2: On cross-component intra prediction [Y.-J. Chang, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AL0268 Crosscheck of Test 2.2c and d in JVET-AL0191 (EE2-2.2: On cross-component intra prediction) [H. Huang (OPPO)] [late] [miss]

JVET-AL0283 Crosscheck of JVET-AL0191 (EE2-2.2: On cross-component intra prediction) for tests EE2-2.2a and EE2-2.2b [J. Lainema (Nokia)] [late]

JVET-AL0199 EE2-6.1: Adaptive picture-level vertical mirroring [D. Mieloch, J. Stankowski, M. Lorkiewicz, A. Dziembowski, D. Karwowski (PUT)]

JVET-AL0236 Crosscheck of JVET-AL0199 (EE2-6.1: Adaptive picture-level vertical mirroring) [M. Abdoli (Xiaomi)] [late] [miss]

JVET-AL0205 EE2-2.1: EIP filters with diagonal shapes [K. Panusopone, M. He, S. Hong, L. Wang, J. Lainema, D. Rusanovskyy (Nokia)]

JVET-AL0206 EE2-2.9: Test 2.1 + Test 2.4 [K. Panusopone, M. He, S. Hong, L. Wang, J. Lainema, D. Rusanovskyy (Nokia), Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AL0264 Crosscheck of JVET-AL0206 (EE2-2.9: Test 2.1 + Test 2.4) [P. Andrivon (Ofinno)] [late] [miss]

JVET-AL0214 EE2-3.6: MV refinement for TMVP [Z. Zhang, J.-L. Lin, Y. Zhang, H. Huang, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AL0257 Crosscheck of JVET-AL0214 (EE2-3.6 MV refinement for TMVP) [Z. Deng (Bytedance)] [late] [miss]

JVET-AL0215 EE2-4.4: IntraNN NSPT set [G. Verba, M. Coban, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AL0244 Crosscheck of JVET-AL0215 (EE2-4.4cd: IntraNN NSPT set) [M. Abdoli (Xiaomi)] [late] [miss]

JVET-AL0281 Crosscheck of JVET-AL0215 (EE2-4.4ab: IntraNN NSPT set) [T. Dong, V. Rufitskiy (TCL)] [late] [miss]

JVET-AL0227 EE2-4.2a: Advanced SBT with simplified search method [Y. Zhang, E. Ye, H. Wang, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AL0259 Crosscheck of JVET-AL0227 (EE2-4.2a: Advanced SBT with simplified search method) [G. Laroche (Canon)] [late]

JVET-AL0229 EE2-4.2b: Combination test of EE2-4.1 and EE2-4.2a [G. Laroche, P. Onno (Canon), Y. Zhang, E. Ye, H. Wang, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AL0260 Crosscheck of JVET-AL0229 (EE2-4.2b: Combination test of EE2-4.1 and EE2-4.2a) [L. Zhao (Bytedance)] [late] [miss]

### EE2 related contributions (7)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

JVET-AL0109 EE2-related: On binarization of a coefficient level in TSRC [Y. Yu, L. Xu, J. Gan, H. Yu, D. Wang (OPPO)]

JVET-AL0299 Crosscheck of JVET-AL0109 (EE2-related: On binarization of a coefficient level in TSRC) [K. Panusopone (Nokia)] [late] [miss]

JVET-AL0110 EE2-related: On ALF-CCCM [N. Song, L. Xu, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AL0276 Crosscheck of JVET-AL0110 (EE2-related: On ALF-CCCM) N. Hu (Qualcomm)

JVET-AL0192 EE2-related: Harmonization of SGPM-BV and LIC [J. Huo, Y. Fei, L. Wang, Y. Ma, F. Yang (Xidian Univ.)]

JVET-AL0270 Crosscheck of JVET-AL0192 (EE2-related: Harmonization of SGPM-BV and LIC) [X. Li (Alibaba)] [late] [miss]

JVET-AL0202 EE2-related: Adaptive picture-level vertical mirroring based on decimated video [D. Mieloch, M. Lorkiewicz, A. Dziembowski (PUT)]

JVET-AL0225 EE2-related: On In-Loop Filtering in ECM [D. Rusanovskyy, M. Santamaria, N. Le, F. Cricri, K. Panusopone, S. Hong, L. Wang, J. Lainema (Nokia)]

JVET-AL0228 EE2-related: NNLF interface in ECM [T. Poirier, F. Galpin, G. Boisson (InterDigital)]

JVET-AL0234 EE2-related: affine bilateral matching merge mode [H. Huang, V. Seregin, M. Karczewicz (Qualcomm)]

### ECM modifications and software improvements beyond EE2 (38)

#### Intra and CIIP (8)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

JVET-AL0111 Non-EE2: Matrix-based position dependent intra prediction for GPM/CIIP [Z. Sun, Y. Yu, L. Xu, H. Yu, D. Wang (OPPO)]

JVET-AL0272 Crosscheck of JVET-AL0111 (Non-EE2: Matrix-based position dependent intra prediction for GPM/CIIP) [X. Li (Alibaba)] [late] [miss]

JVET-AL0174 Non-EE2: TIMD fusion with neural network based intra prediction [Y.-H Lin, K.-W Liang, C.-Y Teng, Y.-C Yang (Sharp)]

JVET-AL0179 Non-EE2: Intra Merge Mode Enhancements [M. Blestel, P. Andrivon (Ofinno)]

JVET-AL0183 Non-EE2: Block Vector-based Intra Mode Derivation [J.-K Lee, D. Ruiz Coll, M. Blestel (Ofinno)] [late]

Initial version rejected as “placeholder”

JVET-AL0195 Non-EE2: TIMD-BV extension with enhanced IntraTMP merge list [J. Fu, J. Zhang, Y. Zhao, S. Ma (PKU), C. Huang (ZTE), K. Naser, M. Radosavljević, T. Dumas (InterDigital), D. Ruiz Coll, J.-K Lee (Ofinno)] [late]

Initial version rejected as “placeholder”

JVET-AL0271 Crosscheck of JVET-AL0195 (Non-EE2: TIMD-BV extension with enhanced IntraTMP merge list) [X. Li (Alibaba)] [late] [miss]

JVET-AL0231 Non-EE2: On interpolation filter for TIMD [Y. Wang, W. Yin, K. Zhang, Z. Deng, N. Zhang, L. Zhao, M. Salehifar, L. Zhang (Bytedance)]

JVET-AL0241 Non-EE2: Intra TMP sub-modes depending on the template type information [T. Dumas, K. Naser, M. Radosavljević, F. Le Léannec (Interdigital)] [late]

JVET-AL0290 [AHG12] Combination of JVET-AL0174, JVET-AL0195 and JVET-AL0241 on Enhanced TIMD with NNIP and Optimized Block Vector Derivation [K. Naser, S. Puri, T. Dumas, M. Radosavljević, F. Le Léannec, E. François (InterDigital), J. Fu, Y. Zhao, J. Zhang, S. Ma (PKU), C. Huang (ZTE), D. Ruiz Coll, J.-K. Lee (Ofinno), Y.-H. Lin, C.-Y. Teng, K.-W. Liang, Y.-C. Yang (Sharp)] [late]

#### Inter (2)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

JVET-AL0248 AHG12: On Motion Vector Prediction [D. Bugdayci Sansli, P. Astola, J. Lainema (Nokia)] [late]

JVET-AL0287 AHG12: Generated Merge Candidates [D. Bugdayci Sansli, P. Astola, J. Lainema (Nokia)] [late]

#### GPM (2)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

JVET-AL0193 Non-EE2: Extension of Template Types in Regression-Based GPM [Y. Yao, J. Huo, W. Zhang, F. Yang (Xidian Univ.), B. Li, F. Xing, P. Han, Z. Wang (Hisense)]

JVET-AL0082 AHG12: Modifications to regression-based GPM with intra and inter prediction [R. Yu, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AL0256 Crosscheck of JVET-AL0082 (AHG12: Modifications to regression-based GPM with intra and inter prediction) [K. Jia (Alibaba)] [late] [miss]

#### In-Loop Filters (6)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

JVET-AL0110 EE2-related: On ALF-CCCM [N. Song, L. Xu, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AL0300 Crosscheck of JVET-AL0110 (EE2-related: On ALF-CCCM) [P. Astola (Nokia)] [late] [miss]

JVET-AL0182 AhG-12: On TALF reference picture extensions [P. Onno, B. Galmiche, G. Laroche (Canon)]

JVET-AL0254 Crosscheck of JVET-AL0182 (AhG-12: On TALF reference picture extensions) [L. Xu (OPPO)] [late] [miss]

JVET-AL0220 AHG12: Reuse of ALF control information [N. Hu, H. Wang, M. Karczewicz, V. Seregin (Qualcomm)]

JVET-AL0230 AhG11/AhG12: Performance of the NNVC ILF in ECM [D. Rusanovskyy, K. Panusopone, S. Hong, L. Wang (Nokia)]

JVET-AL0232 Non-EE2: on TALF input extension [C. Ma, X. Xiu, X. Wang (Kwai)]

JVET-AL0275 Non-EE2: Non-linear clipping for CCALF [N. Song, L. Xu, Y. Yu, H. Yu, D. Wang (OPPO)] [late]

#### Entropy coding, transforms, quantization, and transform coefficient coding (5)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

JVET-AL0112 Non-EE2: Third transform set selection for intraNN [Z. Xie, F. Wang, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AL0150 Non-EE2: CABAC context switch of sig_coeff_flag and abs_level_gtx_flag for LFNST/NSPT [T. Kusakabe, K. Abe, T. Sugio, T. Nishi (Panasonic)]

JVET-AL0198 Non-EE2: Optimization of probability estimation in CABAC [D. Karwowski, D. Mieloch, M. Lorkiewicz, J. Stankowski (PUT)]

JVET-AL0087 AHG12: Predictive transform coefficient coding [T. N. Canh, F. Pu, P. Yin, S. McCarthy (Dolby)]

JVET-AL0306 Cross-check of JVET-AL0087 AhG12: Predictive transform coefficient coding [F. Le Léannec (InterDigital)] [late]

JVET-AL0113 AHG12: Directional sign prediction [L. Xu, Y. Yu, Z. Sun, L. Zhang, H. Yu, D. Wang (OPPO)]

JVET-AL0305 Crosscheck of JVET-AL0113 (on directional sign prediction) [P. Onno (Canon)] [late] [miss]

#### Other (0)

This section is kept as template for future use.

### CTC for EE2/ECM and general ECM improvements (0)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

See also discussion under section 4.4 and 4.14.

# High-level syntax (HLS) and related proposals (78)

## AHG9: Additions for HEVC and AVC (6)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

JVET-AL0057 AHG9: Inclusion of the encoder optimization information SEI message in HEVC [M. M. Hannuksela, J. Boyce (Nokia)]

JVET-AL0059 AHG9: Inclusion of the packed regions information SEI message in HEVC [J. Boyce, M. M. Hannuksela (Nokia)]

JVET-AL0061 AHG 9: Encoder optimization information for AVC and HEVC [C. Kim, H. Tan, J. Nam, J. Lee, J. Lim, S. Kim (LGE)]

JVET-AL0062 AHG 9: AI usage restrictions SEI message for AVC and HEVC [C. Kim, H. Tan, J. Nam, J. Lee, J. Lim, S. Kim (LGE)]

JVET-AL0148 AHG9: Generative face video and generative face video enhancement SEI messages for HEVC and AVC [J. Chen, B. Chen, Y. Ye (Alibaba), S. Gehlot, G.-M. Su, P. Yin, S. McCarthy (Dolby), A. Trioux, F. Yang (Xidian Univ.), Y.-K. Wang (Bytedance), H.-B. Teo, J.-Y. Thong, K. Abe (Panasonic), Y. Xu, K. Yang, Y. Li (SJTU)]

JVET-AL0210 AHG9: On SEI processing order SEI message for HEVC and AVC [Y. Sanchez, T. M. Borges, R. Skupin, C. Hellge, T. Schierl (Fraunhofer HHI)]

## AHG9: Aspects on SEI messages in VSEIv4 and related (47)

### Film grain characteristics SEI message (3+1)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

JVET-AL0086 AHG9/AHG13: On Spatial Resolution for the Film Grain Characteristics (FGC) SEI message [J. Samuelsson-Allendes, S. Deshpande (Sharp)]

JVET-AL0204 AHG9: On picture width and height for the film grain characteristics SEI message [M. M. Hannuksela, J. Boyce (Nokia)]

JVET-AL0211 AHG9: Resolution nesting for FGC SEI message [R. Skupin, Y. Sanchez, A. Wieckowski, T. M. Borges, C. Hellge, T. Schierl (Fraunhofer HHI)]

Aspect on FGC SEI – JVET- AL0211 also relates to 6.4 on potential needs for additional SEI message

JVET-AL0288 AHG2, AHG9, AHG13: on FGC SEI reference picture size [P. de Lagrange (InterDigital)] [late] [miss]

See section 4.2

### SEI processing order and processing order nesting SEI messages (5)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

JVET-AL0063 AHG9: On SPO SEI message [Y. He, M. Karczewicz (Qualcomm)]

JVET-AL0064 AHG9: On SPO sub-chain signaling [Y. He, M. Karczewicz (Qualcomm)]

JVET-AL0065 AHG9: On PON SEI message [Y. He, M. Karczewicz (Qualcomm)]

JVET-AL0178 AHG9: Use cases for energy savings information in SPO SEI messages [C-H. Demarty, E. François, F. Aumont, O. Le Meur (InterDigital)]

JVET-AL0208 AHG9: On SPO root-process signaling constraint [T. M. Borges, Y. Sanchez, R. Skupin, C. Hellge, T. Schierl (Fraunhofer HHI)]

### NNPF SEI extensions (5)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

JVET-AL0075 AHG9: On nnpfa_num_input_pic_shift [M. M. Hannuksela (Nokia)]

JVET-AL0076 AHG9: On multi-purpose NNPFs [M. M. Hannuksela, F. Cricri (Nokia)]

JVET-AL0096 AHG9: On signalling of extension syntax elements in NNPFA SEI message [C. Kim, H. Tan, J. Nam, J. Lee, J. Lim, S. Kim (LGE)]

JVET-AL0159 AHG9: On backbone and tail of NNPFs [F. Cricri, M. M. Hannuksela (Nokia)]

JVET-AL0292 AHG9: On Signalling NNPF Characteristics in NNPFA upon Updates of Purposes [C.-H. Demarty, E. François, F. Aumont, O. Le Meur (InterDigital)] [late]

### Encoder optimization information SEI message (3)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

JVET-AL0056 AHG9: On the encoder optimization information SEI message [M. M. Hannuksela (Nokia)]

JVET-AL0123 AHG9: On the encoder optimization information (EOI) SEI message [J. Xu, Y.-K. Wang (Bytedance)]

JVET-AL0221 AHG9: EOI SEI message with luma range adaptation for machine analysis [T. Partanen, M. M. Hannuksela, H. Zhang, A. Aminlou (Nokia)]

### Object mask information SEI message (3)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

JVET-AL0066 AHG9: Lossy compression with Object mask info SEI [J. Boyce, M. M. Hannuksela (Nokia)]

JVET-AL0067 AHG9: On the OMI SEI [J. Boyce, M. M. Hannuksela (Nokia)]

JVET-AL0071 AHG9: On OMI SEI message [Y. He, M. Karczewicz (Qualcomm)]

### Generative face video SEI messages (3)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

JVET-AL0102 AHG9/AHG16: SEI message and further experiments on QP-adaptive GFVC [W. Kang, L. Liu, C. Jung (Xidian Univ.)]

JVET-AL0155 AHG9: Further fixes and cleanup on GFV and GFVE SEI messages [J. Chen, B. Chen, Y. Ye (Alibaba)]

JVET-AL0156 AHG9/AHG16: Colour calibration for generative face video coding [J. Chen, B. Chen, Y. Ye (Alibaba), S. Yin, S. Wang (CityUHK)]

### Digitally signed content SEI messages (11)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

JVET-AL0078 AHG9: On Digital Signing [S. Deshpande (Sharp)]

JVET-AL0103 AHG9: Editorial changes for the three DSC SEI messages [Y.-K. Wang, J. Xu (Bytedance)]

JVET-AL0115 AHG9: Substream dependency signalling in the DSCI SEI message [Y.-K. Wang (Bytedance), Y. Li, K. Yang (SJTU), J. Xu (Bytedance), Y. Xu (SJTU)]

JVET-AL0116 AHG9: On DSCI content UUID [Y. Li (SJTU), J. Xu, Y.-K. Wang (Bytedance), K. Yang, Y. Xu (SJTU)]

JVET-AL0117 AHG9: On association of NAL units to DSC verification substreams [Y.-K. Wang, J. Xu (Bytedance), Y. Li, K. Yang, Y. Xu (SJTU)]

JVET-AL0118 AHG9: Support of low-delay DSC verification process [Y.-K. Wang, J. Xu (Bytedance), Y. Li, K. Yang, Y. Xu (SJTU)]

JVET-AL0119 AHG9: Miscellaneous changes for the three DSC SEI messages [Y.-K. Wang, J. Xu (Bytedance), Y. Li, K. Yang, Y. Xu (SJTU)]

JVET-AL0131 AHG9: Proposed changes for miscellaneous aspects of SEIs in VSEI v4 [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

Aspects on DSC – JVET- AL0131 also relates to the AI usage restrictions and packed regions SEI messages

JVET-AL0132 AHG9: Editorial updates for VSEI v4 [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

Aspects on DSC – JVET- AL0132 also relates to the packed regions and image format metadata SEI messages

JVET-AL0186 AHG9: Subpicture support for digitally signed content SEI messages [M. Pettersson, R. Sjöberg, M. Damghanian (Ericsson)]

JVET-AL0222 AHG9: On Digitally Signed Content SEI messages [K. Sühring, T. Hinz, Y. Sanchez, J. Pfaff, H. Schwarz, D. Marpe, T. Wiegand (Fraunhofer HHI)]

### Packed regions information SEI message (7+2)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

JVET-AL0070 AHG9: On packed regions information SEI message [T. Chujoh, Z. Fan, R. Ishimoto, T. Ikai (Sharp)]

JVET-AL0072 AHG9: On PRI SEI message [Y. He, M. Karczewicz (Qualcomm)]

JVET-AL0098 AHG9: On target picture in packed regions information SEI message [J. Nam, H. Tan, J. Lee, C. Kim, J. Lim, S. Kim (LGE)]

JVET-AL0120 AHG9: On the packed regions information SEI message - part 1 [Y.-K. Wang, J. Xu (Bytedance)]

JVET-AL0122 AHG9: On the packed regions information (PRI) SEI message - part 2 [J. Xu, Y.-K. Wang (Bytedance)]

JVET-AL0129 AHG9: On signaling of regions in PRI SEI message [J. Lee, H. Tan, J. Nam, C. Kim, J. Lim, S. Kim (LGE)]

JVET-AL0131 AHG9: Proposed changes for miscellaneous aspects of SEIs in VSEI v4 [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

Aspects on PRI – JVET- AL0131 also relates to the digitally signed content and AI usage restrictions SEI messages

JVET-AL0132 AHG9: Editorial updates for VSEI v4 [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

Aspects on PRI – JVET- AL0132 also relates to the digitally signed content and image format metadata SEI messages

JVET-AL0303 AHG9: Editorial fixes to the packed regions information SEI message [F. Urban, E. François, D. Doyen, C.-H. Demarty (InterDigital)] [late]

### Image format metadata SEI message (3+1)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

JVET-AL0068 AHG9: On image format metadata (IFM) SEI [J. Boyce, M. M. Hannuksela (Nokia), A. T. Hinds (Tencent)]

JVET-AL0094 AHG9: On payload length of image format metadata (IFM) SEI [A. T. Hinds, Gilles Teniou, S. Wenger (Tencent), Chris Bai, P. Green (on behalf of ICC)]

JVET-AL0128 AHG9: On image format metadata SEI message [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AL0132 AHG9: Editorial updates for VSEI v4 [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

Aspects on IFM – JVET- AL0132 also relates to the digitally signed content and packed regions SEI messages

### Other (4+1)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

JVET-AL0058 AHG9: On the AI usage restrictions SEI message [M. M. Hannuksela, L. Kondrad, K. Kammachi-Sreedhar, E. B. Aksu, J. Boyce (Nokia)]

JVET-AL0131 AHG9: Proposed changes for miscellaneous aspects of SEIs in VSEI v4 [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

Aspects on AI usage restrictions - JVET- AL0131 also relates to the digitally signed content and packed regions SEI messages

JVET-AL0077 AHG9: On the text description information SEI message [M. M. Hannuksela, J. Boyce (Nokia)]

JVET-AL0130 AHG9: Source picture timing for interlaced video with SPTI SEI message [J. Lee, H. Tan, J. Nam, C. Kim, J. Lim, S. Kim (LGE)]

JVET-AL0249 AHG9: Proposed modifications to VSEI to address national body comments [J. Boyce (editor)] [late]

## AHG9: Aspects on SEI messages in TuC for VSEI (15)

### NNPF SEI messages (3)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

JVET-AL0175 AHG9: On Target Mastering Colour Volume Information in the NNPFC SEI message for Tone Mapping [C-H. Demarty, E. François, F. Aumont, O. Le Meur (InterDigital)]

JVET-AL0177 AHG9: On Signalling Tone Mapping Related Information in NNPFA [C-H. Demarty, E. François, F. Aumont, O. Le Meur (InterDigital)]

JVET-AL0209 AHG9: On providing robustness against layer dropping in multi-layer NNPF [T. M. Borges, Y. Sanchez, R. Skupin, C. Hellge, T. Schierl (Fraunhofer HHI)]

### Constituent rectangles SEI (2)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

JVET-AL0095 AHG 9: On constituent rectangles SEI message [C. Kim, H. Tan, J. Nam, J. Lee, J. Lim, S. Kim (LGE)]

JVET-AL0139 AHG9: On multilayer support for CR SEI [T. Biatek, J. Boyce, M. M. Hannuksela (Nokia)]

### Display rectangles SEI (4)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

JVET-AL0097 AHG 9: Gaussian blur filling method in display rectangles SEI message [C. Kim, H. Tan, J. Nam, J. Lee, J. Lim, S. Kim (LGE)]

JVET-AL0099 AHG9: On design of display rectangles SEI message [J. Nam, H. Tan, J. Lee, C. Kim, J. Lim, S. Kim (LGE)]

JVET-AL0138 AHG9: VVC interface for Display Rectangles SEI [T. Biatek, J. Boyce, M. M. Hannuksela (Nokia)]

JVET-AL0141 AHG9: Showcase on the implementation of display rectangles SEI [T. Biatek, J. Boyce, M. M. Hannuksela (Nokia)] [late] [miss]

### Other (6)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

JVET-AL0091 AHG9: On the multiplane image information (MPII) SEI message [S. McCarthy, S. Oh, W. Husak (Dolby)]

JVET-AL0093 AHG 9: On value range for syntax elements coded as u(v) [H. Tan, C. Kim, J. Nam, J. Lee, J. Lim, S. Kim (LGE)]

JVET-AL0140 AHG9: Region-specific quality metrics for QM SEI [T. Biatek, J. Boyce, M. M. Hannuksela (Nokia)]

JVET-AL0216 [AHG9] Lens Optical Correction SEI – floating point parameters representation [G. Teniou, S. Wenger, A. Hinds (Tencent)]

JVET-AL0226 AHG9: Proposed segmentation plane test sequences (AUX_SEGMENT) [E. Thomas, E. Potetsianakis, E. Alexiou, M.-L. Champel (Xiaomi)]

JVET-AL0286 AHG9: On dual-lens optical correction SEI message [A. Karabutov, E. Alshina (Huawei)] [late]

## Additional SEI message aspects (10+1)

Contributions in this area were discussed during XXXX–XXXX and XXXX–XXXX on XXday XX March 2025 (chaired by XXX).

JVET-AL0069 AHG9/AHG18: Error recovery SEI message [J. Boyce, M. M. Hannuksela (Nokia)]

JVET-AL0073 AHG9/AHG18: Loss recovery information SEI message [C. Kim, H. Tan, J. Nam, J. Lee, J. Lim, S. Kim (LGE)]

JVET-AL0092 AHG9: Confidence information SEI message [J. Boyce, T. Biatek, M. M. Hannuksela (Nokia)]

JVET-AL0127 AHG9: Danmu information SEI message [J. Xu, Y.-K. Wang, L. Zhang (Bytedance)]

JVET-AL0133 AHG9: Signaling of thumbnail information [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AL0211 AHG9: Resolution nesting for FGC SEI message [R. Skupin, Y. Sanchez, A. Wieckowski, T. M. Borges, C. Hellge, T. Schierl (Fraunhofer HHI)]

Aspect on potential need for new SEI – JVET- AL0211 also relates to clause 2.2 study of VSEI v4

JVET-AL0212 AHG9/AHG15: Depth-adaptive picture scaling information SEI [V. Zakharchenko, T. Biatek, J. Boyce (Nokia)]

JVET-AL0219 AHG9: Colour mapping information SEI [J. Boyce, T. Biatek, M. M. Hannuksela (Nokia)]

JVET-AL0224 AHG9: Auxiliary sampling alignment information SEI [V. Zakharchenko, J. Boyce, D. Rusanovskyy, M. M. Hannuksela (Nokia)]

JVET-AL0301 AHG9: VSEI specification changes to reference the 3rd edition of video CICP [J. Xu, Y.-K. Wang (Bytedance)] [late]

JVET-AL0302 Updates of the enhanced colour format information SEI message [D. Podborski, A.M. Tourapis (Apple)] [late]

## Non-SEI HLS aspects (0)

Kept as template for future use.

# Plenary meetings, joint meetings, BoG reports, and liaison communications

## General

The following topics were discussed in JVET plenary XXday XX April XXXX–1345:

- Scheduling for the remaining week (further detail on scheduling is recorded in section 2.12).

- Report from EE/HLS tracks and review/follow-up discussion on documents from sections XX.

Specific discussion on some general issues related to HLS was as follows:

…

## MPEG information sharing meetings

Information sharing sessions with other WGs and AGs of the MPEG community were held on Monday 31 March 0500–0800, Wednesday 2 April 0500–0600, and Friday 4 April 2100–2300.

The status and plans for the work in the MPEG WGs and AGs was reviewed at these information sharing sessions.

## Joint meetings

### Joint session XXXX-XXXX XXday X April on next generation video standardization: MPEG WG 2 / Requirements, MPEG WG 5 / JVET and VCEG (ITU-T Q6/21)

(These notes were recorded by XXX)

The session was chaired by Jörn Ostermann (AG 2 Convenor), Igor Curcio (WG 2 Convenor), Gary Sullivan (Q6/21 Rapporteur), and Jens-Rainer Ohm (JVET chair).

…

## BoGs (0)

Not break-out groups were established at this meeting to conduct discussion and develop recommendations on particular subjects (this section retained for future use).

## Liaison communications (0)

The following liaison statements were received at this meeting (section retained as a template for future use).

…

The liaison response WG 5 N XXX was reviewed in JVET on XXday XX at XXXX-XXXX. The draft reply was also presented in the MPEG AG 3 Communication meeting XXday XX at 1500-1800.

# Project planning

## Software timeline (update)

ECM 16.0 software (including all adoptions) was planned to be available 3 weeks after the meeting (12 February).

The NNVC 12.0 codebase software was planned to be available 3 weeks after the meeting (12 February).

Extensions on top of VTM23.6 software will be released as appropriate (integration and updates of SEI messages included in JVET-AK2006 by the current meeting).

Updates on top of HM18.0 and JM19.1 software will be released as appropriate (e.g., integration and updates of SEI messages included in JVET-AK1006 and JVET-AK1017 by the current meeting).

As a general rule in software development, a person who is executing a merge shall not be from the same company as the person who submitted that merge request.

## Core experiment and exploration experiment planning

An EE on neural network-based video coding was established, as recorded in output document JVET-AL2023.

An EE on enhanced compression technology beyond VVC capability using techniques other than neural-network technology was also established, as recorded in output document JVET-AL2024.

Initial versions of these documents were presented and approved.

## Drafting of specification text, encoder algorithm descriptions, and software

The following agreement has been established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM/VTM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in EEs).

Suggestions for future meetings included the following generally-supported principles:

- Normative contributions (relating to changes in bitstream/decoder) shall include draft specification text

- Proposals shall contain all details relevant for understanding and be self-contained. In cases where the document is a follow-up of a previous contribution, the overall concept and the novelties should be highlighted at minimum

- Coding tool and encoder optimization proposals shall contain Excel sheets that allow assessment on a per-sequence basis

- Algorithm description text is strongly encouraged for non-normative contributions that are intended to be included in model description documents (VTM, ECM, etc.), and that is required for inclusion in TR drafts.

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions (not exercised currently)

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name.

## General issues for experiments

It was emphasized that those rules which had been set up or refined during the 12th JVET meeting should be observed. In particular, for some CEs of some previous meetings, results were available late, and some changes in the experimental setup had not been sufficiently discussed on the JVET reflector.

Group coordinated experiments have been planned as follows:

- “Core experiments” (CEs) are the coordinated experiments on coding tools which are deemed to be interesting but require more investigation and could potentially become part of a draft standard by the next meeting or in the near future.

- “Exploration experiments” (EEs) are also coordinated experiments. These are conducted on technology which is not foreseen to become part of a draft standard in the near future. The investigating methodology for assessment of such technology can also be an important part of an EE. (Further general rules for EEs, as far as deviating from the CE rules below, should be discussed in a future meeting. For the current meeting, procedures as described in the EE description document are deemed to be sufficient.)

- A CE is a test of a specific fully described technology in a specific agreed way. It is not a forum for thinking of new ideas (like an AHG). The CE coordinators are responsible for making sure that the CE description is complete and correct and has adequate detail. Reflector discussions about CE description clarity and other aspects of CE plans are encouraged.

- A description of each experiment is to be approved at the meeting at which the experiment plan is established. This should include the issues that were raised by other experts when the tool was presented, e.g., interference with other tools, contribution of different elements that are part of a package, etc. The experiment description document should provide the names of individual people, not just company names.

- Software for tools investigated in a CE will be provided in one or more separate branches of the software repository. Each CE will have a “fork” of the software, and within the CE there may be multiple branches established by the CE coordinator. The software coordinator will help coordinate the creation of these forks and branches and their naming. All JVET members will have read access to the CE software branches (using shared read-only credentials as described below).

- During the experiment, revisions of the experiment plans can be made, but not substantial changes to the proposed technology. Withdrawing parts of experiments that were intended to show the individual benefits of a tool or parts of a tool is strongly discouraged. Combination tests may not be considered in such cases. Any changes made to individual tools in a combination shall be documented.

- The CE description must match the CE testing that is done. The CE description needs to be revised if there has been some change of plans.

- The CE summary report must describe any changes that were made in the process of finalizing the CE.

- By the next meeting it is expected that at least one independent cross-checker will report a detailed analysis of each proposed feature that has been tested and confirm that the implementation is correct. Commentary on the potential benefits and disadvantages of the proposed technology in cross-checking reports is highly encouraged. Having multiple cross-checking reports is also highly encouraged (especially if the cross-checking involves more than confirmation of correct test results). The reports of cross-checking activities may (and generally should) be integrated into the CE report rather than submitted as separate documents.

- It is mandatory to report encoder optimizations made for the benefit of a tool, and if an equivalent optimization could be applied on the anchor, a comparison against the improved anchor shall be provided.

- A new proposal can be included in a CE based on group decision, regardless if an independent party has already performed a cross-check in the meeting when it was first proposed.

It is possible to define sub-experiments within particular CEs, for example designated as CEX.a, CEX.b, etc., where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the group test model software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments for SDR video are described in the prior output document JVET-T2010.

Experiment descriptions should be written in a way such that it is understood as a JVET output document (written from an objective “third party perspective”, not a proponent perspective – e.g., not referring to methods as “improved”, “optimized”, “enhanced”, etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions contain a basic description of the technology under test, but should not contain excessively verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in documents that are referenced in the CE description that are also available in the JVET document archive.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

The CE development workflow is described at (the following links don’t seem to exist anymore):

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/wikis/Core-experiment-development-workflow

CE read access is available using shared accounts: One account exists for MPEG members, which uses the usual MPEG account data. A second account exists for VCEG members with account information available in the TIES informal ftp area (IFA) system at:

https://www.itu.int/ifa/t/2017/sg16/exchange/wp3/q06/vceg_account.txt

Some agreements relating to CE activities were established as follows:

- Only qualified JVET members can participate in a CE.

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting. Participation was requested by contacting the CE coordinator.

- All software, results, and documents produced in the CE should be announced and made available to JVET in a timely manner.

- A JVET CE reflector will be established and announced on the main JVET reflector. Discussion of logistics arrangements, exchange of data, minor refinement of the test plans, and preparation of documents shall be conducted on the JVET CE reflector, with subject lines prefixed by “[CEx: ]”, where “x” is the number of the CE. All substantial communications about a CE other than such details shall take place on main JVET reflector. In the case that large amounts of data are to be distributed, it is recommended to send a link to the data rather than the data itself, or upload the data as an input contribution to the next meeting.

General timeline for CEs

T1= 3 weeks after the JVET meeting: To revise the CE description and refine questions to be answered. Questions should be discussed and agreed on JVET reflector. Any changes of planned tests after this time need to be announced and discussed on the JVET reflector. Initially assigned description numbers shall not be changed later. If a test is skipped, it is to be marked as “withdrawn”.

T2 = Test model software release + 2 weeks: Integration of all tools into a separate CE branch of the VTM is completed and announced to JVET reflector.

- Initial study by cross-checkers can begin.

- Proponents may continue to modify the software in this branch until T3.

- 3rd parties are encouraged to study and make contributions to the next meeting with proposed changes

T3: 3 weeks before the next JVET meeting or T2 + 1 week, whichever is later: Any changes to the CE test branches of the software must be frozen, so the cross-checkers can know exactly what they are cross-checking. A software version tag should be created at this time. The name of the cross-checkers and list of specific tests for each tool under study in the CE plan description shall be documented in an updated CE description by this time.

T4: Regular document deadline minus 1 week: CE contribution documents including specification text and complete test results shall be uploaded to the JVET document repository (particularly for proposals targeting to be promoted to the draft standard at the next meeting).

The CE summary reports shall be available by the regular contribution deadline. This shall include documentation about crosscheck of software, matching of CE description and confirmation of the appropriateness of the text change, as well as sufficient crosscheck results to create evidence about correctness (crosscheckers must send this information to the CE coordinator at least 3 days ahead of the document deadline). Furthermore, any deviations from the timelines above shall be documented. The numbers used in the summary report shall not be changed relative to the description document.

CE reports may contain additional information about tests of straightforward combinations of the identified technologies. Such supplemental testing needs to be clearly identified in the report if it was not part of the CE plan.

New branches may be created which combine two or more tools included in the CE document or the VTM (as applicable).

It is not necessary to formally name cross-checkers in the initial version of the CE description document. To adopt a proposed feature at the next meeting, JVET would like to see comprehensive cross-checking done, with analysis of whether the description matches the software, and a recommendation of the value of the tool and given tradeoffs.

The establishment of a CE does not indicate that a proposed technology is mature for adoption or that the testing conducted in the CE is fully adequate for assessing the merits of the technology, and a favourable outcome of CE does not indicate a need for adoption of the technology into a standard or test model.

Availability of specification text is important to have a detailed understanding of the technology and also to judge what its impact on the complexity of the specification will be. There must also be sufficient time to study this in detail. CE contributions without sufficiently mature draft specification text in the CE input document should not be considered for adoption.

Lists of participants in CE documents should be pruned to include only the active participants. Read access to software will be available to all members.

# Establishment of ad hoc groups (update)

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups was agreed to be the main JVET reflector (jvet@lists.rwth-aachen.de).

Chairs of AHGs were asked to send draft mandates to JRO before XXXX on X Apr., preferably copy from the table below and sending in a word file with changemarks.

Review of AHG plans was conducted during the plenary on XXday X Apr. 2025 at XXXX–XXXX.

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| Project Management (AHG1) (jvet@lists.rwth-aachen.de) - Coordinate overall JVET interim efforts. - Supervise AHG and experiment studies. - Report on project status to JVET reflector. - Provide a report to the next meeting on project coordination status. - Supervise processing and delivery of output documents | J.-R. Ohm (chair), G. J. Sullivan (vice-chair) | N |
| Draft text and test model algorithm description editing (AHG2) (jvet@lists.rwth-aachen.de) - Produce and finalize draft text outputs of the meeting (JVET-AK1006, JVET-AK1017, JVET-AK2005 and JVET-AK2006). - Collect reports of errata for the VVC, VSEI, HEVC, AVC, CICP, and the published related technical reports and produce the JVET-AK1004 errata output collection. - Coordinate with the test model software development AhG to address issues relating to mismatches between software and text. - Collect and consider errata reports on the texts. | B. Bross, C. Rosewarne (co-chairs), F. Bossen, A. Browne, S. Kim, S. Liu, J.-R. Ohm, G. J. Sullivan, A. Tourapis, Y.-K. Wang, Y. Ye (vice-chairs) | N |
| Test model software development (AHG3) (jvet@lists.rwth-aachen.de) - Coordinate development of test models (VTM, HM, SCM, SHM, HTM, MFC, MFCD, JM, JSVM, JMVM, 3DV-ATM, 360Lib, and HDRTools) software and associated configuration files. - Produce documentation of software usage for distribution with the software. - Enable software support for recently standardized additional SEI messages (for both VTM and HM), and SEI messages in TuC (the latter in a separate branch of VTM). - Discuss and make recommendations on the software development process. - Perform comparative tests of test model behaviour using common test conditions, including HDR, high bit depth and high bit rate. - Suggest configuration files for additional testing of tools. - Investigate how to minimize the number of separate codebases maintained for group reference software. - Coordinate with AHG on Draft text and test model algorithm description editing (AHG2) to identify any mismatches between software and text, and make further updates and cleanups to the software as appropriate. - Prepare drafts of merged and updated CTC documents for HM and VTM, as applicable. | F. Bossen, X. Li, K. Sühring (co-chairs), E. François, Y. He, K. Sharman, V. Seregin, A. Tourapis (vice-chairs) | N |
| Test material and visual assessment (AHG4) (jvet@lists.rwth-aachen.de) - Consider plans for additional verification testing of VVC capability, particularly target conducting tests for VVC multi-layer features, and update the test plan accordingly. - Maintain the video sequence test material database for testing the VVC and HEVC standards and potential future extensions, as well as exploration activities. - Study coding performance and characteristics of available and proposed video test material. - Identify and recommend appropriate test material for testing the VVC standard and potential future extensions, as well as exploration activities. - Identify and characterize missing types of video material, solicit contributions, collect, and make available a variety of video sequence test material, in coordination with other AHGs, as appropriate. - Maintain and update the directory structure for the test sequence repository, as necessary. - Collect information about test sequences that have been made available by other organizations. - Prepare and conduct expert viewing for purposes of subjective quality evaluation. - Coordinate with AG 5 in studying and developing further methods of subjective quality evaluation, e.g. based on crowd sourcing. - Coordinate with AHG15 on investigating sequences with gaming content, and make such sequences available for study. - Coordinate with AHG17 on investigating sequences outside of CTC, and making arrangements for viewing at the AHG meeting. - Prepare availability of viewing equipment and facilities arrangements for future meetings. | V. Baroncini, T. Suzuki, M. Wien (co-chairs), W. Husak, S. Iwamura, P. de Lagrange, S. Liu, X. Meng, S. Puri, A. Segall, S. Wenger (vice-chairs) | N |
| Conformance testing (AHG5) (jvet@lists.rwth-aachen.de) - Prepare for a new edition of VVC conformance with new multi-layer streams (JVET-AI2028) and corrections of the existing streams. - Study the draft conformance bitstreams for new HEVC multiview profiles in JVET-AI1008, and further develop related conformance bitstreams. - Coordinate with AHG3 on implementation of the new HEVC multiview profiles. - Study the requirements of VVC, HEVC, and AVC conformance testing to ensure interoperability. - Maintain and update the conformance bitstream database, and contribute to report problems, and suggest actions to resolve these. - Study additional testing methodologies to fulfil the needs for VVC conformance testing. | I. Moccagatta (chair), F. Bossen, T. Ikai, S. Iwamura, H.-J. Jhu, K. Kawamura, P. de Lagrange, S. Paluri, K. Sühring, Y. Yu (vice-chairs) | N |
| ECM software development (AHG6) (jvet@lists.rwth-aachen.de) - Coordinate development of the ECM software and associated configuration files. - Produce documentation of software usage for distribution with the software. - Prepare and deliver ECM-16.0 software version (and potential updates), corresponding VTM anchor, and the reference configuration encodings according to the ECM common test conditions. - Investigate encoder speedup and other software optimization such as reduction of memory consumption. - Coordinate with ECM algorithm description editors to identify any mismatches between software and text, make further updates and cleanups to the software as appropriate. | V. Seregin (chair), J. Chen, R. Chernyak, F. Le Léannec, K. Zhang (vice-chairs) | N |
| ECM tool assessment (AHG7) (jvet@lists.rwth-aachen.de) - Investigate methodology of tool assessment, such as the aspects of memory access and bandwidth, number of maximum processing cycles, block decoding dependencies, number of context coded bins, pipeline and parallelization. - Coordinate with AHG6 on resolving tool-off test related software issues (missing tool controls and software bugs). - Prepare configuration files and generate bitstreams and results of tool-on/tool-off testing. - Prepare reporting of tool assessment results. - Coordinate with AHG17 to collect simulation results on non-CTC sequences (e.g., those used in previous verification tests), and identify a set of non-CTC sequences that would be appropriate for additional testing. - Develop methodology of more reliable runtime measurement | X. Li (chair), L.-F. Chen, Z. Deng, J. Gan, E. François, R. Ishimoto, H.-J. Jhu, J. Lainema, X. Li, J. Pardo, A. Stein, H. Wang (vice-chairs) | Y (tel., 2 weeks notice) |
| Optimization of encoders and receiving systems for machine analysis of coded video content (AHG8) (jvet@lists.rwth-aachen.de) - Solicit and study non-normative encoder and receiving systems technologies that enhance performance of machine analysis tasks on coded video content. - Identify and collect test materials that are suitable to be used by JVET for machine analysis tasks. - Discuss improvements on the evaluation framework, including evaluation procedures and methodologies. - Coordinate software development and experiments on optimization of encoders and receiving systems for machine analysis of coded video content, including combinations of proposed technologies. - Maintain the software implementation examples and develop tool combination examples in the repository, including sufficient documentation in terms of operation and performance. - Evaluate proposed technologies and their suitability for machine analysis applications. - Propose improvements to JVET-AK2030 on optimization of encoders and receiving systems for machine analysis of coded video content. - Study the potential of using SEI messages for the purpose of machine analysis in coordination with AHG9. - Investigate the impact of using different machine task models in the evaluation of the compression performance of tools optimized for machine analysis tasks. - Coordinate with WG 4 VCM AHG on aspects such as unified common test conditions, evaluation metrics, test and training materials, usage of SEI messages, and on studying characteristics and requirements of machine analysis tasks, etc. Provide WG 4 VCM with AHG software and scripts to experiment non-normative tools and tool combination examples, and provide bitstreams generated by running a dense set of QP points for each sequence by uploading to the mpeg.expert repository. | S. Liu, J. Ström, S. Wang, M. Zhou (AHG chairs) | Y (tel., 2 weeks notice, joint with WG 4 AHG) |
| SEI message studies (AHG9) (jvet@lists.rwth-aachen.de) - Study the SEI messages in VSEI, VVC, HEVC and AVC. - Study JVET-AK1006, JVET-AK1017, JVET-AK2005, and JVET-AK2006, identify any issues and propose solutions as appropriate. - Study JVET-AK2032, and propose improvements. - Collect software and showcase information for SEI messages, including encoder and decoder implementations and bitstreams for demonstration and testing. - Identify potential needs for additional SEI messages, including the study of SEI messages defined in HEVC and AVC for potential use in the VVC context. - Study the alignments of the same SEI messages in different standards. - Coordinate with AHG8 and WG 4 to study mechanisms for signalling metadata in the context of machine analysis of coded video content. - Coordinate with AHG3 for software support of SEI messages for JM, HM, and VTM. | S. McCarthy, Y.-K. Wang (co-chairs), J. Boyce, T. Chujoh, S. Deshpande, C. Fogg, M. M. Hannuksela, P. de Lagrange, G. J. Sullivan, H. Tan, A. Tourapis, S. Wenger, P. Wu (vice-chairs) | N |
| Encoding algorithm optimization (AHG10) (jvet@lists.rwth-aachen.de) - Study the impact of using techniques such as tool adaptation and configuration, and perceptually optimized adaptive quantization for encoder optimization. - Study the impact of non-normative techniques of preprocessing for the benefit of encoder optimization. - Study encoding techniques of optimization for objective quality metrics and their relationship to subjective quality. - Study optimized encoding for reference picture resampling and scalability modes in VTM, and coordinate with AHG4 on improving encoders and test settings for multi-layer verification testing. - Study optimized encoding and suitable test settings for noisy materials, such as sequences containing film grain. - Study optimized encoding and tool combinations for low latency and for low complexity. - Consider neural network-based encoding optimization technologies for video coding standards. - Investigate other methods of improving objective and/or subjective quality, including adaptive coding structures and multi-pass encoding. - Study methods of rate control and rate-distortion optimization and their impact on performance, subjective and objective quality. - Study the potential of defining default or alternate software configuration settings and test conditions optimized for either subjective quality, higher objective quality, or encoding with improved complexity/performance tradeoff, and coordinate such efforts with AHG3 and AHG6. - Study the effect of varying configuration parameters depending on temporal layer, such as those related to deblocking, partitioning, chroma QP. | K. Andersson, P. de Lagrange, A. Duenas (co-chairs), T. Ikai, T. Solovyev, A. Tourapis (vice chairs) | Y (tel., 2 weeks notice) |
| Neural network-based video coding (AHG11) (jvet@lists.rwth-aachen.de) - Evaluate and quantify the performance improvement potential of NN-based video coding technologies compared to existing video coding standards such as VVC, including both individual coding tools, architectures and content adaptation with NN parameters overfitting. - Establish logistics of training data sets, including list, location and md5sums. Generate and distribute anchor encoding, and develop supporting software as needed. - Study potential improvements of the NNVC CTC document JVET-AJ2016. - Study the impact of training (including the impact of loss functions) on the performance of candidate technologies and identify suitable material for testing and training. - Analyse complexity characteristics for technologies under study, including transformers, perform complexity analysis, and develop complexity reductions of candidate technology. - Discuss and propose improved metrics to perform complexity analysis of NN architectures, in particular also considering the impact of sparse models. - Finalize and discuss the EE on neural network-based video coding. - Promote the call for training materials, distribute it, and actively communicate with content owners. - Coordinate with other groups, including SC 29/AG 5 on the evaluation and assessment of visual quality, and AHG12 on the interaction with ECM coding tools. If possible, prepare encodings with combinations of tools included in the NNVC software for visual quality assessment at the next meeting. - Coordinate with AHG14 on items related to NNVC software development and study the impact of training set extension. | E. Alshina, F. Galpin, S. Liu, A. Segall (co-chairs), J. Li, Y. Li, R.-L. Liao, M. Santamaria, T. Shao, M. Wien, P. Wu (vice chairs) | Y (tel., 2 weeks notice), first on Feb. 5, second on March 5 |
| Enhanced compression beyond VVC capability (AHG12) (jvet@lists.rwth-aachen.de) - Solicit and study non-neural-network video coding tools with enhanced compression capabilities beyond VVC. - Discuss and propose refinements to the ECM16 algorithm description JVET-AK2025. - Coordinate with AHG7 to study the performance and complexity tradeoff of these video coding tools. - Coordinate with AHG6 on ECM software development. - Support AHG6 in generating anchors according to the test conditions in JVET-AI2017. - Analyse the results of exploration experiments described in JVET-AK2024 in coordination with the EE coordinators. - Coordinate with AHG11 to study the interaction with neural network-based coding tools. | M. Karczewicz, Y. Ye, L. Zhang (co-chairs), B. Bross, R. Chernyak, X. Li, K. Naser, Y. Yu (vice-chairs) | N |
| Film grain technologies (AHG13) (jvet@lists.rwth-aachen.de) - Study the benefits and characteristics of film grain technologies, including autoregressive and frequency-filtering technologies. - Discuss and propose refinements to the draft of the TR 2nd ed. JVET-AK2020. - Study alternative film grain models and their associated documentation. - Discuss and enumerate updates, improvements, and additions for the second edition of the technical report. - In consultation with AHG4, study and define content characteristics and test conditions that are desirable for the study and testing of film grain technologies, and perform an assessment of newly available test materials in that regard. - Investigate metrics for measuring film grain fidelity in itself, or as present in a video. - Discuss the potential need for film grain conformance guidelines. - Given the study of desirable content characteristics, solicit or create new test material for further determining the operational characteristics of, testing, and developing any related technologies. - Study preprocessing and encoder technologies for determining values for FGC (Film Grain Characteristics) SEI message syntax elements. - Identify potential need for additional film grain technology and signalling, if needed. - Coordinate development of film grain technology software and configuration files. - Coordinate with AG 5 on improving the plan for subjective quality testing of the FGC SEI message JVET-AJ2022, and conduct preparations for such testing. - Coordinate with AHG3 for software support of the FGC SEI message. | W. Husak, P. de Lagrange (co-chairs), A. Duenas, X. Meng, A. Segall, G. Teniou, A. Tourapis (vice-chairs) | Y (tel., 2 weeks notice) |
| NNVC software development (AHG14) (jvet@lists.rwth-aachen.de) - Coordinate development of the NNVC software and associated configuration files. - Prepare and deliver NNVC-12.0 software version (and potential updates), and provide reference configuration encodings according to the NNVC common test conditions as described in JVET-AJ2016. Study the impact of the addition of new dataset on the already integrated models. - Investigate and bridge coding performance of NNVC VTM anchor compared to the latest VTM version and/or VTM anchor in ECM software. - Investigate combinations of tools included in the NNVC software, prepare and release anchor data for all configurations of the software, including anchors for High and Low Operation Point (HOP/LOP) and Very Low Operation Point (VLOP) configurations. - Study and maintain the SADL (Small Adhoc Deep-Learning Library). Identify gaps in functionality and develop improvements as needed. - Coordinate with NNVC algorithm and software description (JVET-AK2019) editors to identify any mismatches between software and description document, suggest further updates to the description document as appropriate. - Coordinate with AHG11 on items related to NNVC activities. | F. Galpin (chair), R. Chang, Yue Li, Yun Li, M. Santamaria, J. N. Shingala, Z. Xie (vice chairs) | Y (tel., 2 weeks notice), first on Feb. 5, second on March 5 |
| Gaming content compression (AHG15) (jvet@lists.rwth-aachen.de) - Identify gaming content application scenarios and their requirements for codec operation. - Identify and characterize required types of content; solicit contributions, collect, and make a variety of gaming content available, in coordination with AHG4 and AG 5. - Produce VTM and ECM anchor encodings according to CTC JVET-AJ2027, and provide test results at the next meeting. - Develop and maintain interfaces for supporting use cases of camera parameters and depth maps in gaming applications, including mechanisms for efficient transporting these elements in the coded video bitstream. - Evaluate JVET test models (such as ECM, VTM, NNVC, etc.) under the proposed test conditions. - Investigate possibilities to enhance compression capability for gaming content. - Study conversion of depth maps using integer representation, and identifying efficient bit-depth resolution of depth maps to support identified use-cases that will be an input to compression. - Solicit contributions from industry on typical bitrate/quality/resolution used for gaming content compression. | S. Puri, J. Sauer (co-chairs), R. Chernyak, A. Duenas, L. Wang, V. Zakharchenko (vice chairs) | Y (tel., 2 weeks notice) |
| Generative face video compression (AHG16) (jvet@lists.rwth-aachen.de) - Maintain GFVC software tools, associated configuration files, and software usage documentation. - Study the extension of GFVC software capability to handle video content with higher resolutions. - Identify and study additional test content, including content with higher resolutions, suitable for use in GFVC performance evaluation. - Study GFVC performance under test conditions defined in JVET-AJ2035, as well as performance on additional test content, and wider bitrate ranges. - Coordinate with AHG9 on further development of the GFV and GFVE SEI messages in JVET-AK2006. | Y. Ye (chair), H.-B. Teo, Z. Lyu, S. McCarthy, S. Wang (vice chairs) | N |
| Testing of video coding technology beyond CTC (AHG17) (jvet@lists.rwth-aachen.de) - Identify test sequences extending the represented range of applications (i.e. beyond those defined in common test conditions documents JVET-AK2010, JVET-AC2011 and JVET-AI2017) and investigate their suitability for use in visual quality assessments. - Prepare encoded bitstreams from the identified test sequences using same VTM and ECM versions as previously. - Make preparations for subjective viewing at the hybrid AHG meeting (for on-site participants). - Develop the output document JVET-AK2026. - Arrange and conduct a hybrid AHG meeting (with remote participation for group discussions), and prepare logistics according to JVET-AK2039. | M. Wien (chair), E. Alshina, V. Baroncini, P. de Lagrange, Y. Ye (vice chairs) | Y - tel., 2 weeks notice, first on Feb. 6 - hybrid, March 10-12 in Aachen, DE |
| Ultra-low latency and packet loss resilience (AHG18) (jvet@lists.rwth-aachen.de) - Investigate and identify test conditions, evaluation criteria and evaluation methodology, supporting a set of end-to-end latency targets in a range of 25-100 ms. - Investigate creation of practical simulation software based on VTM, including network transmission aspects, and conduct performance evaluation. - Identify potential requirements and feasibility of standard based technologies to support ultra-low delay requirements, including packet loss resilient decoding. - Investigate packet loss resilient technologies beyond VVC supporting ultra-low delay coding for interactive and live broadcasting scenarios. | S. Ikonin, S. Wenger, V. Zakharchenko (co-chairs), S. Deshpande, S. Fößel, C. Kim, X. Ma, S. Puri, J. Ström (vice-chairs) | Y (tel., 2 weeks notice) |

It was confirmed that the rules which can be found in document ISO/IEC JTC 1/‌SC 29/‌AG 2 N 046 “Ad hoc group rules for MPEG AGs and WGs” (available at https://www.mpegstandards.org/adhoc/), are consistent with the operation mode of JVET AHGs. It is pointed out that JVET does not maintain separate AHG reflectors, such that any JVET member is implicitly a member of any AHG. This shall be mentioned in the related WG Recommendations. The list above was also issued as a separate WG 5 document (ISO/IEC JTC 1/‌SC 29/‌WG 5 N XXX) in order to make it easy to reference.

# Output documents (update)

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate the editors responsible for the document production. Where applicable, dates of planned finalization and corresponding parent-body document numbers are also noted.

It was reminded that in cases where the JVET document is also made available as a WG 5 output document, a separate version under the WG 5 document header should be generated. This version should be sent to GJS and JRO for upload.

The list of JVET ad hoc groups was also issued as a WG 5 output document WG 5 N XXX, as noted in section 9.

JVET-AK1000 Meeting Report of the 37th JVET Meeting [J.-R. Ohm] [WG 5 N 333] (2025-02-19)

Initial versions of the meeting notes (d0 … d8) were made available on a daily basis during the meeting.

Remains valid – not updated: JVET-AC1001 Guidelines for HM-based software development [K. Sühring, F. Bossen, X. Li (software coordinators)]

Remains valid – not updated: JVET-Y1002 High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Encoder Description Update 16 [C. Rosewarne (primary editor), K. Sharman, R. Sjöberg, G. J. Sullivan (co-editors)] [WG 5 N 103]

Remains valid – not updated: JVET-AH1003 Coding-independent code points for video signal type identification (Draft 3) [G. J. Sullivan, A. Tourapis]

Primary editor: G. J. Sullivan.

JVET-AK1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP [Y.-K. Wang, B. Bross, I. Moccagatta, C. Rosewarne, G. J. Sullivan] (2025-03-14, near next meeting)

Primary editor: Y.-K. Wang.

This includes changes from new bug tickets, some items removed that are resolved and put into corresponding output documents.

Remains valid – not updated: JVET-AH1005 Technology under consideration for future editions of CICP [E. Thomas, A. Tourapis] [WG 5 N 289)]

Can be removed in a future meeting, no activity since April 2024.

JVET-AK1006 HEVC additional profiles and SEI messages (draft 2) [Y.-K. Wang, B. Bross, S. Deshpande, G. J. Sullivan, A. Tourapis] [WG 5 CDAM N 336)] (2025-01-29)

Primary editor: Y.-K. Wang.

Changes agreed at this meeting:

…

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

Remains valid – not updated: JCTVC-V1007 SHVC Test Model 11 (SHM 11) Introduction and Encoder Description [G. Barroux, J. Boyce, J. Chen, M. M. Hannuksela, Y. Ye] [WG 11 N 15778]

Remains valid – not updated: JVET-AI1008 Conformance testing for HEVC multiview extended and monochrome profiles [I. Moccagatta, S. Paluri, A. Tourapis, Y.-K. Wang]

Remains valid – not updated: JVET-AC1009 Common test conditions for SHVC [K. Sühring]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JCTVC-O1010 Guidelines for Conformance Testing Bitstream Preparation [T. Suzuki, W. Wan]

Remains valid – not updated: JVET-AJ1011 White paper on HEVC [B. Bross, J.-R. Ohm, G. J. Sullivan, Y.-K. Wang] [AG 3 N 174]

Remains valid – not updated: JVET-AJ1012 Overview of IT systems used in JVET [J.-R. Ohm, I. Moccagatta, K. Sühring, M. Wien]

Revisit: Potential update of bug tracking?

Remains valid – not updated: JCT3V-G1003 3D-AVC Test Model 9 [D. Rusanovskyy, F. C. Chen, L. Zhang, T. Suzuki] [WG 11 N 14239]

Remains valid – not updated: JCT3V-K1003 Test Model 11 of 3D-HEVC and MV-HEVC [Y. Chen, G. Tech, K. Wegner, S. Yea] [WG 11 N 15141]

Remains valid – not updated: JVET-AE1013 Common test conditions of 3DV experiments [K. Sühring, M. Wien]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated JCTVC-V1014 Screen Content Coding Test Model 7 Encoder Description (SCM 7) [R. Joshi, J. Xu, R. Cohen, S. Liu, Y. Ye] [WG 11 N 16049]

Remains valid – not updated: JVET-AC1015 Common test conditions for SCM-based screen content coding [K. Sühring]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-AH1016 AVC with extensions and corrections (draft 3) [B. Bross, T. Ikai, G. J. Sullivan, A. Tourapis, Y.-K. Wang]

Primary editor: B. Bross.

JVET-AK1017 Support for additional SEI messages in AVC (draft 1) [B. Bross, J. Boyce, G. J. Sullivan, Y.-K. Wang] (2025-02-14)

Primary editor: B. Bross

Changes agreed at this meeting:

…

Resolved bug fixes carried over from JVET-AJ1004 were also to be included.

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

No output: JVET-Axx1018 through JVET-Axx1099

Remains valid – not updated JVET-AA1100 Common Test Conditions for HM Video Coding Experiments [K. Sühring, K. Sharman]

This specifies only the CTC for non-4:2:0 colour formats. The corresponding document for VVC is JVET-T2013, with no unification yet.

Links to test sequences need to be updated due to the change of the content server.

No output: JVET-Axx2001

Remains valid – not updated: JVET-AH2002 Algorithm description for Versatile Video Coding and Test Model 22 (VTM 22) [Y. Ye, A. Browne, S. Kim] [WG 5 N 284]

Primary editor: Y. Ye.

New elements from notes elsewhere in this report:

- …

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

It was suggested that editorial improvements submitted as input to the next meeting would be welcome.

Remains valid – not updated: JVET-AJ2003 Guidelines for VTM-based software development [F. Bossen, X. Li, K. Sühring]

Remains valid – not updated: JVET-T2004 Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 12) [Y. Ye, J. Boyce]

JVET-AK2005 Additions and corrections for VVC version 4 (Draft 11) [G. J. Sullivan, B. Bross, M. M. Hannuksela, Y.-K. Wang] [WG 5 preliminary DAM update N 337)] (2025-03-14)

A DoC WG 5 N XXX on the CDAM was approved … .

Changes agreed at this meeting:

…

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

Primary editor: G. J. Sullivan.

JVET-AK2006 Additional SEI messages for VSEI version 4 (Draft 5) [J. Boyce, J. Chen, S. Deshpande, M. M. Hannuksela, S. McCarthy, G. J. Sullivan, H. Tan, Y.-K. Wang] [WG 5 preliminary DAM update N 335)] (2025-03-14)

A DoC WG 5 N XXX on the CDAM was approved …

Changes agreed at this meeting:

…

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

Primary editor: J. Boyce.

Remains valid – not updated: JVET-AJ2007 Guidelines for NNVC software development [F. Galpin, S. Eadie, L. Wang, Z. Xie, Y. Li]

Remains valid – not updated: JVET-X2008 Conformance testing for versatile video coding (Draft 7) [J. Boyce, F. Bossen, K. Kawamura, I. Moccagatta, W. Wan]

The document number was planned to be re-used for a 3rd edition in ITU , once that is submitted to ITU-T (could be in October 2025).

Remains valid – not updated: JVET-AJ2009 Reference software for versatile video coding 2nd edition (Draft 2) [F. Bossen, K. Sühring, X. Li] [WG 5 DIS N 322)]

Software relating to H.266.2 and ISO/IEC 23090-16 can be found at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tree/2nd-edition.

Primary editor: F. Bossen.

JVET-AK2010 VTM and HM common test conditions and software reference configurations for SDR 4:2:0 10 bit video [F. Bossen, X. Li, V. Seregin, K. Sharman, K. Sühring] (2025-02-14)

Add alternative configurations (see discussion under JVET-AK0181). Also links to test sequences are updated due to the change of the content server.

Remains valid – not updated: JVET-AC2011 VTM and HM common test conditions and evaluation procedures for HDR/WCG video [A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-U2012 JVET common test conditions and evaluation procedures for 360° video [Y. He, J. Boyce, K. Choi, J.-L. Lin]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-T2013 VTM common test conditions and software reference configurations for non-4:2:0 colour formats [Y.-H. Chao, Y.-C. Sun, J. Xu, X. Xu]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-Q2014 JVET common test conditions and software reference configurations for lossless, near lossless, and mixed lossy/lossless coding [T.-C. Ma, A. Nalci, T. Nguyen]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-Q2015 JVET functionality confirmation test conditions for reference picture resampling [J. Luo, V. Seregin]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-AJ2016 Common test conditions and evaluation procedures for neural network-based video coding technology [E. Alshina, F. Galpin, R.-L. Liao, S. Liu, A. Segall]

Remains valid – not updated: JVET-AI2017 Common test conditions and evaluation procedures for enhanced compression tool testing [M. Karczewicz, Y. Ye]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-AA2018 Common test conditions for high bit depth and high bit rate video coding [A. Browne, T. Ikai, D. Rusanovskyy, X. Xiu, Y. Yu]

Links to test sequences need to be updated due to the change of the content server.

JVET-AK2019 Description of algorithms version 10 and software version 12 in neural network-based video coding (NNVC) [F. Galpin, Y. Li, D. Rusanovskyy, J. Ström, L. Wang] (2025-03-14)

New elements in text and software from notes elsewhere in this report:

- …

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

JVET-AK2020 Film grain synthesis technology for video applications ed. 2 (Draft 2) [W. Husak, P. de Lagrange, A. Norkin, A. Tourapis] (2025-03-14)

Draft 1 (the output of the last meeting) was submitted for ITU approval, only containing necessary bug fixes and clarifications on top of the TR already approved in ISO/IEC. This means that currently the ITU and ISO versions are slightly diverging. JVET-AK2020 contains some paragraphs only relevant for v2 of ISO (as those are already included in v1 of ITU), and other paragraphs adding additional aspects that would be relevant for v2 of both ISO and ITU. It is expected that by the time of finalization of v2, alignment of the twin text can be reached.

New elements: First thoughts on other content to be added (from JVET-AK0013), new: JVET-AK0169.

Remains valid – not updated: JVET-AJ2021 Verification test plan for VVC multilayer coding (update 5) [O. Chubach, P. de Lagrange, M. Wien]

Remains valid – not updated: JVET-AJ2022 Plan for subjective quality testing of the FGC SEI message (update 4) [P. de Lagrange, W. Husak, M. Radosavljević, M. Wien]

JVET-AK2023 Exploration experiment on neural network-based video coding (EE1) [E. Alshina, R. Chang, F. Galpin, Yue Li, Yun Li, M. Santamaria, J. Ström, Z. Xie (EE coordinators)] (2025-02-07)

An initial draft of this document was reviewed and approved at XXXX-XXXX on XXday X Apr.

This round of EE1 tests includes:

…

JVET-AK2024 Exploration experiment on enhanced compression beyond VVC capability (EE2) [V. Seregin, J. Chen, R. Chernyak, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang (EE coordinators)] (2025-02-19)

An initial draft of this document was reviewed and approved at XXXX-XXXX on XXday X Apr .

This round of EE2 tests will include:

…

JVET-AK2025 Algorithm description of Enhanced Compression Model 16 (ECM 16) [M. Coban, R.-L. Liao, K. Naser, J. Ström, L. Zhang] (2025-03-14)

New elements from notes elsewhere in this report:

…

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

JVET-AK2026 Testing of video coding technology beyond conditions of exploration experiments [M. Wien, Y. Ye, V. Baroncini, E. Alshina] (2025-02-07)

Initial changes from discussion under JVET-AK0335, to be finalized in AHG telco (Feb. 6).

Remains valid – not updated: JVET-AJ2027 Common test conditions for gaming applications [J. Sauer, R. Chernyak, S. Puri, S. Thiebaud]

JVET-AK2028 Additional conformance bitstreams for VVC multilayer configurations [S. Iwamura, P. de Lagrange, I. Moccagatta]

For CD, WG 5 N XXX to be issued integrating these new bitstreams in ISO style, and deliver some corrected bitstreams.

AK2028 is only a delta text.

Remains valid – not updated: JVET-AH2029 Visual quality comparison of ECM/VTM encoding [V. Baroncini, J.-R. Ohm, M. Wien] [AG 5 N 118]

JVET-AK2030 Optimization of encoders and receiving systems for machine analysis of coded video content (Draft 8, for future updates) [S. Liu, J. Chen, J. Ström] (2025-03-14)

Primary editor: S. Liu.

Note: J. Ström was added as editor of ISO/IEC 23888-3 per recommendation of the 17th WG 5 meeting, but C. Hollmann should be recommended to be removed.

New elements from notes elsewhere in this report:

- …

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

Remains valid – not updated: JVET-AI2031 Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content [S. Liu, C. Hollmann]

JVET-AK2032 Technologies under consideration for future extensions of VSEI (version 7) [S. McCarthy, J. Boyce, J. Chen, S. Deshpande, M. M. Hannuksela, H. Tan, Y.-K. Wang] (2025-03-14)

New elements from notes elsewhere in this report:

…

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

It was agreed that MPI is not further changed, and will be removed upon completion of a corresponding MIV profile.

Remains valid – not updated: JVET-AF2033 Report of verification test on VVC multi-layer coding: Content layering [S. Iwamura, P. de Lagrange, M. Wien] [AG 5 N 105)]

Remains valid – not updated: JVET-AI2034 Call for new HDR materials for future video coding development [E. François, W. Husak, S. Iwamura, D. Rusanovskyy, A. Segall, M. Wien] [WG 5 N 312)]

Remains valid – not updated: JVET-AJ2035 Test conditions and evaluation procedures for generative face video coding [S. McCarthy, B. Chen]

Remains valid – not updated: JVET-AG2036 Call for training materials for neural network-based video coding tool development [E. Alshina, F. Galpin, S. Liu, M. Wien] [WG 5 N 266)]

Remains valid – not updated: JVET-AJ2037 Report on subjective quality testing of the FGC SEI message (AG 5 N 140) [P. de Lagrange, W. Husak, M. Wien] [AG 5 N 140)] (2024-12-31)

JVET-AK2038 Draft white paper on VSEI [J. Boyce, S. McCarthy, S. Deshpande, G. J. Sullivan, Y. Sanchez, Y.-K. Wang] (2025-03-21)

JVET-AK2039 Announcement of JVET AHG17 Meeting in Aachen, DE, 10-12 March 2025 
[J.-R. Ohm, M. Wien] (2025-02-07)

(number can be re-used)

# Future meeting plans, expressions of thanks, a.o.b., and closing of the meeting

The draft of the WG 5 recommendations (see Annex C) was reviewed and approved in JVET at XXXX-XXXX on XXday X Apr .

Future meeting plans were established with the following general guidelines (assuming face-to-face meetings):

- Meeting under ITU-T SG21 auspices when it meets (ordinarily starting meetings on the Tuesday or Wednesday of the first week and closing it on the Wednesday of the second week of the SG21 meeting – a total of 8-9 meeting days), and

- Otherwise meeting under ISO/IEC JTC 1/‌SC 29 auspices when its MPEG WGs meet (ordinarily starting meetings on the Thursday or Friday prior to the main week of such meetings and closing it on the same day as other MPEG WGs – a total of 8–9 meeting days).

In cases where an exceptionally high workload is expected for a meeting, an earlier starting date may be defined. In cases of online meetings, no sessions should be held on weekend days, such that meetings would typically start two days earlier.

Some specific future meeting plans (to be confirmed) were established as follows:

- During Thu. 26 June – Fri. 4 July 2025, 39th meeting under ISO/IEC JTC 1/‌SC 29 auspices in Daejeon, – KR, to be conducted as hybrid meeting,

- During 3 – 12 October 2025, 40th meeting under ITU-T SG21 auspices in Geneva, CH, to be conducted as hybrid meeting (starting Friday 3 Oct. afternoon for opening and AHG reports only),

- During 14 – 23 January 2026, 41st meeting under ISO/IEC JTC 1/‌SC 29 auspices, to be conducted as teleconference meeting,

- During 24 April – 1 May 2026, 42nd meeting under ISO/IEC JTC 1/‌SC 29 auspices in Santa Eulària, ES, to be conducted as hybrid meeting,

- During 7 – 15 July 2026, 43rd meeting under ITU-T SG21 auspices in Geneva, CH, to be conducted as hybrid meeting,

- During 14 – 23 October 2026, 44th meeting under ISO/IEC JTC 1/‌SC 29 auspices in Hangzhou, CN, to be conducted as hybrid meeting,

- During January 2027, 45th meeting under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.

- During April 2027, 46th meeting under ITU-T SG21 auspices in XXXX, to be conducted as hybrid meeting.

The agreed document deadline for the 39th JVET meeting was planned to be Thursday 19 June 2025.

Netflix and University of Bristol were thanked for offering new video material that can be used for training neural network-based algorithms.

ABP, CMG, NERC-DTV and TCL were thanked for offering new HDR test material, Huawei and YouTube were thanked for offering test material showing user generated content.

TCL was thanked for providing 4K displays, and Fraunhofer HHI was thanked for providing play-out equipment used in the experts viewing in Geneva. Christian Lehmann was thanked for helping with the test setup. The experts who volunteered to participate in the viewing were also thanked.

Marius Preda was thanked for the service of managing and maintaining the document site jvet-experts.org. Institut Mines-Télécom was thanked for hosting the sites.

ITU was thanked for the excellent hosting of the 37th JVET meeting in Geneva. Simão Campos and Stefano Polidori were thanked for the help in preparation and organisation, and Kelya-Axelle Beke was thanked for managing the registrations. The following members of ITU staff were thanked for their dedication and daily help in the technical setup of meeting facilities: Gent Bajrami, Marc Antoine Zanou, Ilia Londo, and Paul Marko.

The 38th JVET meeting was closed at approximately XXXX hours UTC on Friday 4 April 2025.

# Annex A to JVET report:
List of documents

Dates and times in the table below are in Paris/Geneva time (1 hr. ahead of UTC before 30 March, 2 hrs. afterwards). It is noted that, if title or authorship of a document deviates from the title or author list in the body of the report, the list of documents in this annex contains the correct title and authors.

# Annex B to JVET report:
List of meeting participants

The remote participants of the thirty-eighth meeting of the JVET, according to the participation records from the Zoom teleconferencing tool used for the meeting sessions (approximately XXX people in total, not including those who attended only the joint sessions with other groups), were as follows:

- …

# Annex C to JVET report:
Recommendations of the 19th meeting of
ISO/IEC JTC 1/SC 29/WG 5 MPEG Joint Video Experts Team with ITU-T SG21

ISO/IEC JTC 1/SC 29/WG 5 N XXX