| Joint Video Experts Team (JVET) of ITU-T SG21 WP3/21 and ISO/IEC JTC 1/SC 29 42nd Meeting, Santa Eulària, ES, 24 April – 1 May 2026 | Document: JVET-AP_notes_d0 |
| --- | --- |

| Title: | Meeting Report of the 42nd Meeting of the Joint Video Experts Team (JVET), by teleconference, 24 April – 1 May 2026 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from the chair of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contacts): | Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen Melatener Straße 23 D-52074 Aachen | Tel: Email: | +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chair of JVET |  |  |

_____________________________

# Summary

The Joint Video Experts Team (JVET) of ITU-T WP3/21 and ISO/IEC JTC 1/‌SC 29 held its forty-second meeting during 24 April – 1 May 2026 at Palacio de Congresos de Ibiza, Santa Eulària, Spain. The meeting was held as a hybrid meeting, where remote participation was provided on best-effort basis for experts who were unable to travel.

For ISO/IEC purposes, JVET is alternatively designated ISO/IEC JTC 1/‌SC 29/‌WG 5, and this was the twenty-third meeting as WG 5. The JVET meeting was held under the chairmanship of Dr Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 2.16 of this document. It is further noted that work items which had originally been conducted by the Joint Collaborative Team on Video Coding (JCT-VC) were continued in JVET as a single joint team, and explorations towards possible future need of standardization in the area of video coding are also conducted by JVET, as negotiated by the parent bodies.

The JVET meeting began at approximately 0900 CEST (0700 UTC) on Friday 24 April 2026. Meeting sessions were held on all days including the weekend days of Saturday and Sunday 25 and 26 April 2026, until the meeting was closed at approximately XXXX hours CEST on Friday 1 May 2026. Approximately XXX people attended the JVET meeting, and approximately XXX input documents (not counting crosschecks, reports, and summary documents), 18 JVET AHG reports, 1 report of the Joint AHG on Gaussian splat coding, 2 EE summary reports, X BoG report(s), and X incoming liaison document(s) were discussed. The meeting took place in coordination with a meeting of various SC29 Working Groups and Advisory Groups – where WG 5 is representing the Joint Video Coding Team(s) and their activities from the perspective of the SC 29 parent body, under whose auspices this JVET meeting was held (also mention co-located meeting of WP3/21). The subject matter of the JVET meeting activities consisted of work on further development and maintenance of the twin-text video coding technology standards Advanced Video Coding (AVC), High Efficiency Video Coding (HEVC), Versatile Video Coding (VVC), Coding-independent Code Points (Video) (CICP), and Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (VSEI), as well as related technical reports, reference software and conformance testing packages. Further important goals were reviewing the results of the Exploration Experiment (EE) on Neural Network-based Video Coding, of the EE on Enhanced Compression beyond VVC capability, of the joint EE on Gaussian splat coding, of other technical input on novel aspects of video coding technology, and to plan next steps for investigation of candidate technology towards further standard development. In the latter context, the development of the Final Draft Joint Call for Proposals on video compression with capability beyond VVC was a major activity at his meeting.

As a primary goal, the JVET meeting reviewed the work that had been performed in the interim period since the forty-first JVET meeting in producing the following documents:

a) JVET documents

JVET-AO1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP

JVET-AO1016 AVC with extensions and corrections (Draft 5)

JVET-AO1017 Support for additional VSEI messages in AVC (Draft 5)

JVET-AO2002 Algorithm description for Versatile Video Coding and Test Model 23 (VTM 23)

JVET-AO2011 VTM and HM common test conditions and evaluation procedures for HDR/WCG video

JVET-AO2019 Description of algorithms version 14 and software version 16 in neural network-based video coding (NNVC)

JVET-AO2021 Verification test plan for VVC multilayer coding (update 8)

JVET-AO2023 Exploration experiment on neural network-based video coding (EE1)

JVET-AO2024 Exploration experiment on enhanced compression beyond VVC capability (EE2)

JVET-AO2026 Draft Joint Call for Proposals on video compression with capability beyond VVC, also issued as WG 5 N 388

JVET-AO2027 Common Test Conditions (CTC) for gaming applications

JVET-AO2030 Optimization of encoders and receiving systems for machine analysis of coded video content (Draft 10), also issued as WG 5 2nd DTR N 387

JVET-AO2032 Technologies under consideration for future extensions of VSEI (version 11)

JVET-AO2040 Complexity reporting template for coding algorithms and tools

JVET-AO2041 Announcement of JVET AHG17 Meeting in Aachen, DE, 25-27 February 2026

b) documents produced as WG 5 documents only:

WG 5 N 384 Preliminary disposition of comments received on ISO/IEC 14496-10:2025/DAM 1

WG 5 N 385 Disposition of comments received on DIS ISO/IEC 23090-15:202X (3rd edition)

WG 5 N 386 Disposition of comments received on ISO/IEC DTR 23888-3

WG 5 N 390 Liaison statement to ISO/IEC JTC 1/SC 29/WG 1 (JPEG) on JPEG AI and explorations on video coding

WG 5 N 391 Liaison statement to SMPTE on requesting SEI mechanism for new SMPTE ST 2094-60 standard

As main results, the JVET produced XX output documents from the current meeting (update):

JVET-AO1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP

JVET-AO1016 AVC with extensions and corrections (Draft 5)

JVET-AO1017 Support for additional VSEI messages in AVC (Draft 5)

JVET-AO2002 Algorithm description for Versatile Video Coding and Test Model 23 (VTM 23)

JVET-AO2011 VTM and HM common test conditions and evaluation procedures for HDR/WCG video

JVET-AO2019 Description of algorithms version 14 and software version 16 in neural network-based video coding (NNVC)

JVET-AO2021 Verification test plan for VVC multilayer coding (update 8)

JVET-AO2023 Exploration experiment on neural network-based video coding (EE1)

JVET-AO2024 Exploration experiment on enhanced compression beyond VVC capability (EE2)

JVET-AO2026 Draft Joint Call for Proposals on video compression with capability beyond VVC, also issued as WG 5 N 388

JVET-AO2027 Common Test Conditions (CTC) for gaming applications

JVET-AO2030 Optimization of encoders and receiving systems for machine analysis of coded video content (Draft 10), also issued as WG 5 2nd DTR N 387

JVET-AO2032 Technologies under consideration for future extensions of VSEI (version 11)

JVET-AO2040 Complexity reporting template for coding algorithms and tools

JVET-AO2041 Announcement of JVET AHG17 Meeting in Aachen, DE, 25-27 February 2026

- The following 5 documents were produced as WG 5 documents only, without a corresponding JVET output document or direct repetition of their content in this meeting report:

WG 5 N 384 Preliminary disposition of comments received on ISO/IEC 14496-10:2025/DAM 1

WG 5 N 385 Disposition of comments received on DIS ISO/IEC 23090-15:202X (3rd edition)

WG 5 N 386 Disposition of comments received on ISO/IEC DTR 23888-3

WG 5 N 390 Liaison statement to ISO/IEC JTC 1/SC 29/WG 1 (JPEG) on JPEG AI and explorations on video coding

WG 5 N 391 Liaison statement to SMPTE on requesting SEI mechanism for new SMPTE ST 2094-60 standard

For the organization and planning of its future work, the JVET established XX “ad hoc groups” (AHGs) to progress the work on particular subject areas. Another Joint AHG on Gaussian splat coding was established together with WG 4 and WG 7. At this meeting, X Exploration Experiments (EE) on video coding aspects, and a Joint EE on Gaussian splat coding (together with WG 4 and WG 7) were defined. The next ten JVET meetings were planned for 7 – 15 July 2026 under ITU-T SG21 auspices in Geneva, CH; during 17 – 23 October 2026 under ISO/IEC JTC 1/‌SC 29 auspices in Hangzhou, CN; during 13 – 22 January 2027 under ISO/IEC JTC 1/‌SC 29 auspices in Brisbane, AU; during 17 – 28 April 2027 under ITU-T SG21 auspices in Geneva, CH; during 7 – 16 July 2027 under ISO/IEC JTC 1/‌SC 29 auspices in Tampere, FI; during 20 – 29 October 2027 under ISO/IEC JTC 1/‌SC 29 auspices in Shenzhen, CN; during January 2028 under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.; during 25 March – 5 April 2028 under ITU-T SG21 auspices in Geneva, CH; during July 2028 under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.; and during October 2028 under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.

The document distribution site https://jvet-experts.org/ was used for distribution of all documents. The most recent versions of JCT-VC and JCT-3V documents can now be accessed directly via the JVET site, whereas all uploaded versions are also available from http://wftp3.itu.int/av-arch/jctvc-site/ and http://wftp3.itu.int/av-arch/jct3v-site/, respectively. JVET documents are also provided via http://wftp3.itu.int/av-arch/jvet-site/, but as this is currently not maintained as a mirror with the jvet-experts.org site, documents are typically only made available with a delay of two or three meeting cycles, to avoid that late uploads of output documents would be missing.

The reflector to be used for discussions by the JVET and all its AHGs is the JVET reflector:
jvet@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see https://lists.rwth-aachen.de/postorius/lists/jvet.lists.rwth-aachen.de/.

# Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Video Experts Team (JVET) is a group of video coding experts from the ITU-T Study Group 21 Visual Coding Experts Group (VCEG) and ISO/IEC JTC 1/‌SC 29/‌WG 5. The parent bodies of the JVET are ITU-T WP3/21 and ISO/IEC JTC 1/‌SC 29.

The Joint Video Experts Team (JVET) of ITU-T WP3/21 and ISO/IEC JTC 1/‌SC 29 held its forty-second meeting during 24 April – 1 May 2026 at Palacio de Congresos de Ibiza, Santa Eulària, Spain. The meeting was held as a hybrid meeting, where remote participation was provided on best-effort basis for experts who were unable to travel.

It is further noted that the unabbreviated name of JVET was formerly known as “Joint Video Exploration Team”, but the parent bodies modified it when entering the phase of formal development of the Versatile Video Coding (VVC) and Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (VSEI) standards, as well as associated conformance test sets, reference software, verification testing, and non-normative guidance information. Furthermore, starting from the twentieth meeting, work items which had originally been conducted by the Joint Collaborative Team on Video Coding (JCT-VC) were continued to be conducted in JVET as a single joint team, as negotiated by the parent bodies. This particularly consists of work on:

- High Efficiency Video Coding (HEVC) and its extensions, the development of associated conformance test sets, reference software, verification testing, and non-normative guidance information,

- Specification of Coding-independent Code Points (Video) (CICP), and associated technical report(s),

- Maintenance and enhancement work on the Advanced Video Coding (AVC) standard, associated conformance test sets and reference software.

Furthermore, explorations towards possible future need of standardization in the area of video coding are also conducted by JVET. Currently, the following topics are under investigation:

- Exploration on Neural Network-based Video Coding

- Exploration on Enhanced Compression beyond VVC capability

- Joint exploration (with WG 4 and WG 7 of SC 29) on Gaussian splat coding

JVET had been tasked by the parent bodies to develop a call for proposals on video compression with capability beyond existing standards, with plan to issue a first draft call from the current meeting.

This report contains three important annexes, as follows:

- Annex A contains a list of the documents of the JVET meeting

- Annex B contains a list of the meeting participants, consisting of two parts, (B1) in-person attendees as recorded by a sign-in sheet circulated in meeting rooms, (B2) remote attendees as recorded by the teleconferencing tool used for the meeting

- Annex C contains the meeting recommendations of ISO/IEC JTC 1/‌SC 29/‌WG 5 for purposes of results reporting to ISO/IEC.

## Meeting logistics

Information regarding logistics arrangements for the meeting had been provided via the email reflector jvet@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jvet-site/2026_04_AP_SantaEularia/.

## Primary goals

As a primary goal, the JVET meeting reviewed the work that was performed in the interim period since the forty-first JVET meeting in producing the following output documents:

a) JVET documents

JVET-AO1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP

JVET-AO1016 AVC with extensions and corrections (Draft 5)

JVET-AO1017 Support for additional VSEI messages in AVC (Draft 5)

JVET-AO2002 Algorithm description for Versatile Video Coding and Test Model 23 (VTM 23)

JVET-AO2011 VTM and HM common test conditions and evaluation procedures for HDR/WCG video

JVET-AO2019 Description of algorithms version 14 and software version 16 in neural network-based video coding (NNVC)

JVET-AO2021 Verification test plan for VVC multilayer coding (update 8)

JVET-AO2023 Exploration experiment on neural network-based video coding (EE1)

JVET-AO2024 Exploration experiment on enhanced compression beyond VVC capability (EE2)

JVET-AO2026 Draft Joint Call for Proposals on video compression with capability beyond VVC, also issued as WG 5 N 388

JVET-AO2027 Common Test Conditions (CTC) for gaming applications

JVET-AO2030 Optimization of encoders and receiving systems for machine analysis of coded video content (Draft 10), also issued as WG 5 2nd DTR N 387

JVET-AO2032 Technologies under consideration for future extensions of VSEI (version 11)

JVET-AO2040 Complexity reporting template for coding algorithms and tools

JVET-AO2041 Announcement of JVET AHG17 Meeting in Aachen, DE, 25-27 February 2026

b) documents produced as WG 5 documents only:

WG 5 N 384 Preliminary disposition of comments received on ISO/IEC 14496-10:2025/DAM 1

WG 5 N 385 Disposition of comments received on DIS ISO/IEC 23090-15:202X (3rd edition)

WG 5 N 386 Disposition of comments received on ISO/IEC DTR 23888-3

WG 5 N 390 Liaison statement to ISO/IEC JTC 1/SC 29/WG 1 (JPEG) on JPEG AI and explorations on video coding

WG 5 N 391 Liaison statement to SMPTE on requesting SEI mechanism for new SMPTE ST 2094-60 standard

Further important goals were reviewing the results of the EE on Neural Network-based Video Coding, of the EE on Enhanced Compression beyond VVC capability, of other technical input on novel aspects of video coding technology, and planning next steps for investigation of candidate technology towards further standard development.

## Documents and document handling considerations

### General

The document distribution site https://jvet-experts.org/ was used for distribution of all documents.

Document registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting follow the UTC time, except as otherwise noted.

Highlighting of recorded decisions in this report is practised as follows:

- Decisions made by the group that might affect the normative content of a future standard are identified in this report by prefixing the description of the decision with the string “Decision:”.

- Decisions that affect one of the various software packages but have no normative effect on text are marked by the string “Decision (SW):”.

- Decisions that fix a “bug” in one of the test model descriptions such as VTM, HM, etc. (an error, oversight, or messiness) or in the associated software package are marked by the string “Decision (BF):”.

- Decisions that are merely editorial without effect on the technical content of a draft standard are marked by the string "Decision (Ed.):". Such editorial decisions are merely suggestions to the editor, who has the discretion to determine the final action taken if their judgment differs.

- Other parenthetical comments may be used for describing the impact or motivation of a decision. Some decisions are recorded with the word “agreed” rather than “Decision:”, especially for work items under study, non-normative, editorial and planning matters.

This meeting report is based primarily on notes taken by the JVET chair, or other experts that were appointed to chair parallel sessions. It is indicated who took the notes in review of a given category (or individual documents în a category), and by which day and time the review was conducted. The preliminary notes were also circulated publicly by ftp and http during the meeting on a daily basis. It should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much information about the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report. Expressions such as “X.XX%” indicate that the desired results were not available at the time the information was recorded.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Friday, 17 April 2026. Any documents uploaded after 1159 hours Paris/Geneva time on Saturday 18 April 2026 were considered “officially late”, with a grace period of 12 hours (to accommodate those living in different time zones of the world). The deadline does not apply to AHG reports and other such reports which can only be produced after the availability of other input documents.

All contribution documents with registration numbers higher than JVET-AP0236 were registered after the “officially late” deadline (and therefore were also uploaded late). However, some documents in the “late” range might include break-out activity reports that were generated during the meeting, or documents which were requested to be produced for the purpose of improving specification text, and are therefore better considered as report documents rather than as late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions has been to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This has been agreed to be a helpful approach to be followed at the meeting.

The following technical design proposal contributions were registered and/or uploaded late:

- JVET-AP0XXX (a proposal on …), uploaded 04-XX,

- …

It may be observed that some of the above-listed contributions were submissions made in response to issues that arose in discussions during the meeting or from the study of other contributions, and thus could not have been submitted by the ordinary deadline.

The following other documents not proposing normative technical content, but with some need for consideration, were registered and/or uploaded late:

- JVET-AO0054 (a contribution on …), uploaded 04-XX,

- …

Almost all cross-verification reports at this meeting (except for JVET-AP0XXX, …) were registered late, and/or uploaded late. In the interest of brevity, these are not specifically identified here. Initial upload times for each document are recorded in Annex A of this report.

The following contribution registrations were noted that were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JVET-AP0141, JVET-AP0198, JVET-AP0203, JVET-AP0206, JVET-AP0208, JVET-AP0209, JVET-AP210, JVET-AP0258, JVET-AP0259, ….

The following cross-verification report(s) were still missing by the end of the meeting, but were uploaded later: JVET-AP0XXX, …. The following report(s) had not become available yet three weeks after the end of the meeting: JVET-AP0XXX, … . These were marked as withdrawn by the JVET chair, assuming the registration had become obsolete (paragraph kept for future use).

“Placeholder” contribution documents that were basically empty of content, or lacking any results showing benefit for the proposed technology, and obviously uploaded with an intent to provide a more complete submission as a revision, had been agreed to be considered unacceptable and to be rejected in the document management system until a more complete version was available (which would then be counted as a late contribution if the update was after the document deadline). At the current meeting, this situation did not apply to any contributions. (did apply to document(s) JVET-AP0XXX, being initially empty of results and flagged as late in the list above, based on the time of the first reasonable document upload | sentence kept for future use).

It was remarked that documents that are substantially revised after the initial upload can also be a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This can especially be a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the “comments” field on the web site can be used to indicate what is different in a revision, although participants tend to seldom notice what is recorded there.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when there was a consensus to consider them and there was sufficient time available for their review. Again, an exception is applied for AHG reports, CE and HLS topic summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there may have been some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

A few contributions may have had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). Any such issues were corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the responsible coordinators).

Some other errors may have also noticed in other initial document uploads (wrong document numbers or meeting dates or meeting locations in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Outputs of the preceding meeting

All output documents of the previous meeting, particularly the meeting report JVET-AO1000, the Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP JVET-AO1004, the AVC with extensions and corrections (draft 5) JVET-AO1016, the Support for additional SEI messages in AVC (draft 5) JVET-AO1017, the Algorithm description for Versatile Video Coding and Test Model 23 (VTM 23) JVET-AO2002, the VTM and HM common test conditions and evaluation procedures for HDR/WCG video JVET-AO2011, the Description of algorithms version 14 and software version 16 in neural network-based video coding (NNVC) JVET-AO2019, the Verification test plan for VVC multilayer coding (update 8) JVET-AO2021, the Description of the EE on Neural Network-based Video Coding JVET-AO2023, the Description of the EE on Enhanced Compression beyond VVC capability JVET-AO2024, the Draft Joint Call for Proposals on video compression with capability beyond VVC JVET-AO2026, the Common Test Conditions (CTC) for gaming applications JVET-AO2027, the Optimization of encoders and receiving systems for machine analysis of coded video content (Draft 10) JVET-AO2030, the Technologies under consideration for future extensions of VSEI (version 11) JVET-AO2032, the Complexity reporting template for coding algorithms and tools JVET-AO2040, and the Announcement of JVET AHG17 Meeting in Aachen, DE, 25-27 February 2026 JVET-AO2041, had been completed and those which were available were approved. In a few cases, the corresponding WG 5 N-numbered documents had not yet been uploaded, and this was requested to be done as soon as possible (kept for future use). The software implementations of VTM version 23.X, ECM version 19.X, and NNVC version 16.0 were also approved.

It was further noted that some editorial modifications were necessary on the WG 5 versions of standard deliverables, to be compliant with the ISO/IEC rules.

Only minor editorial issues were found in the meeting report JVET-AO1000; no need to produce an update was identified (see section 2.14 for details).

The available output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

## Attendance

The list of participants in the JVET meeting can be found in Annexes B1 and B2 of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/21 or ISO/IEC JTC 1/‌SC 29/‌WG 5 (including experts who had been personally invited as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the responsible coordinators.

It was further announced that it is necessary to register for the meeting through the ISO Meetings website for ISO/IEC experts, and through the Q6/21 rapporteur for ITU-T experts. The password for meeting access had been sent to registered participants via these channels. Links to the Zoom sessions (without the necessary password) were available in the posted meeting logistics information and the calendar of meeting sessions in the JVET web site.

The following rules were established for those participating remotely via Zoom teleconference meeting:

- Use the “hand-raising” function to enter yourself in the queue to speak (unless otherwise instructed by the session chair). If you are dialed in by phone, request your queue position verbally. The online queue will be interleaved with the room queue, though it may not always be guaranteed that the sequence perfectly follows the sequence by which hand raising occurred.

- Stay muted unless you have something to say. People are muted by default when they join and need to unmute themselves to speak. The chair may mute anyone who is disrupting the proceedings (e.g. by forgetting they have a live microphone while chatting with their family or by causing bad noise or echo).

- Identify who you are and your affiliation when you begin speaking. The same applies for speakers in the room to let online participants know who is speaking.

- Use your full name and company/organization and country affiliation in your joining information, since the participation list of Zoom would also be used to compile the online part of attendance records.

- Turn on the chat window and watch for chair communication and side commentary there as well as by audio.

- It was pointed out the according to ISO rules, any recording or transcription of zoom sessions is not permitted. Participants who are performing recording will be removed from the session unless they stop recording upon request.

- Generally, do not use video for the teleconferencing calls in order to avoid overloading internet connections; enable only voice and screen sharing.

- Extensive use of screen sharing is encouraged, to enable participants to view the presented material and the meeting notes. At times, multiple sources of screen sharing may be enabled, so it may be necessary for participants to understand that this is happening and to understand how to select which shared screen they want to see.

## Agenda

The agenda for the meeting, for the further development and maintenance of the twin-text video coding technology standards Advanced Video Coding (AVC), High Efficiency Video Coding (HEVC), Versatile Video Coding (VVC), Coding-independent Code Points (Video) (CICP), and Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (VSEI), as well as related technical reports, software and conformance packages, was as follows:

Opening remarks and review of meeting logistics and communication practices

Roll call of participants

Approval of the agenda

Code of conduct policy reminder

IPR policy reminder and declarations

Contribution document allocation

Review of results of the previous meeting

Review of target dates

Reports of ad hoc group (AHG) activities, and Joint AHG on Gaussian splat coding

Report of exploration experiment on neural-network-based video coding (EE1)

Report of exploration experiment on enhanced compression beyond VVC capability (EE2)

Report of joint EE on Gaussian splat coding (JEE9)

Consideration of contributions on high-level syntax

Consideration of contributions and communications on project guidance

Consideration of video coding technology contributions

Consideration of contributions on conformance and reference software development

Consideration of contributions on software development in exploration activities

Consideration of contributions on coding-independent code points for video signal type identification

Consideration of contributions on film grain synthesis technology

Consideration of contributions on gaming content compression

Consideration of contributions on generative face video compression

Consideration of contributions on optimization of encoders and receiving systems for machine analysis of coded video content

Consideration of contributions on assessment of implementation complexity of video coding technology

Consideration of contributions on aspects of ultra-low latency and packet loss resilience in the context of video compression

Consideration of contributions and planning of next steps towards a Call for Proposals on future video coding standardization, development of a Final Draft Call for Proposals, and definition of criteria for technology assessment

Consideration of contributions on errata relating to standards in the domain of JVET

Consideration of contributions on technical reports relating to standards and exploration study activities in the domain of JVET

Consideration of contributions providing non-normative guidance relating to standards and exploration study activities in the domain of JVET

Consideration of information contributions

Consideration of future work items

Coordination of visual quality testing

Liaisons, coordination activities with other organizations

Review of project editor and liaison assignments

Approval of output documents and associated editing periods

Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments (if any), establishment of AHGs, future meeting planning, other planning issues

Other business as appropriate for consideration

Closing

- The agenda was approved as suggested.

- The times of meeting sessions followed the needs of the face-to-face meeting, with highest priority given to the aim of achieving the goals of the meeting (see under section 2.15). Typical meeting hours were expected to be 0900-2000 CEST with coffee breaks and lunch breaks as appropriate, however some early morning or late-night sessions were anticipated to be necessary. Sessions were announced in the online JVET calendar in advance as far as possible, but it was agreed that some activities (such as breakout sessions) could be held at short notice.

## ISO, IEC and ITU-T Code of Conduct reminders

Participants were reminded of the ISO and IEC Codes of Conduct, found at

https://www.iso.org/publication/PUB100011.html.

https://www.iec.ch/basecamp/iec-code-conduct-technical-work

These include points relating to:

- Behave ethically

- Promote and enable all voices to be heard

- Engage constructively in ISO and IEC activities

- Respect others (in meetings, when writing, on social media)

- Declare actual and potential conflicts of interest and manage them appropriately

- Protect confidential information

- Protect ISO and IEC assets

- Avoid and prevent any form of bribery or corruption

- Uphold the consensus process

- When disputes arise, escalate, resolve and uphold agreed resolution

Participants were also reminded of the UN Codes of Conduct, applying to work under ITU-T, found at https://www.un.org/management/sites/www.un.org.management/files/un-system-model-code-conduct.pdf

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JVET and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JVET necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jvet-site (JVET contribution templates)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

The responsible coordinators invited participants to make any necessary verbal reports of previously-unreported IPR in technology that might be considered as prospective candidate for inclusion in future standards, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that the VTM and ECM software implementation packages use the same software copyright license header as the HEVC reference software, where the latter had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time. This license header language is based on the BSD license with a preceding sentence declaring that other contributor or third party rights, including patent rights, are not granted by the license, as recorded in N 10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11 of June/July 2009. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software header is also used in the process of designing the VTM and ECM software, and for evaluating proposals for technology to be potentially included in these designs. This software or parts thereof might be published by ITU-T and ISO/IEC as an example implementation of a future video coding standard and for use as the basis of products to promote adoption of such technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

These considerations apply to the 360Lib video conversion software and HDRTools as well. The SADL and NNVC packages for neural network-based video coding use the same licensing terms.

Software packages that had been developed in prior work of the JVT, the JCT-VC and the JCT-3V have similar considerations and are maintained according to the past practice in that work.

## Communication practices

The documents for the meeting can be found at https://jvet-experts.org/. The site contains links for creating a user account for document uploading and for contacting an administrator – e.g., to update the email address associated with an account, and experts were reminded to keep this information up to date. It was reminded to send a notice to the chairs in cases of changes to document titles, authors, etc.

It was noted that also the most recent versions of JCT-VC and JCT-3V documents can be accessed directly via the JVET site. A mirror for JCT-VC, JCT-3V and JVET document access (including all versions with original upload dates) has also been provided via the ITU ftp site, providing a “documents” subfolder in the directory of each respective meeting.

JVET email lists are managed through the site https://lists.rwth-aachen.de/postorius/lists/jvet.lists.rwth-aachen.de/, and to send email to the reflector, the email address is jvet@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JVET participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work. The current number of subscribers on the JVET email list was 1389 (as of 23 April 2026). All discussions (including those on AVC, HEVC, VVC, CICP, etc.) shall be conducted on the JVET reflector rather than any of the old reflectors (including JVT, JCT-VC, and JCT-3V) which are retained for archiving purposes.

It is further emphasized that the document JVET-AJ1012 gives valuable hints about communication practices as well as other IT resources used in JVET, such as software, conformance, and test materials.

## Terminology

- ACT: Adaptive colour transform

- AFF: Adaptive frame-field

- AI: All-intra

- AIF: Adaptive interpolation filtering

- ALF: Adaptive loop filter

- AMP: Asymmetric motion partitioning – a motion prediction partitioning for which the sub-regions of a region are not equal in size (in HEVC, being N/2x2N and 3N/2x2N or 2NxN/2 and 2Nx3N/2 with 2N equal to 16 or 32 for the luma component)

- AMVP: Adaptive motion vector prediction

- AMT or MTS: Adaptive multi-core transform, or multiple transform selection

- AMVR: (Locally) adaptive motion vector resolution

- APS: Adaptation parameter set

- ARC: Adaptive resolution conversion (synonymous with DRC, and a form of RPR)

- ARMC: Adaptive re-ordering of merge candidates

- ARSS: Adaptive reference sample smoothing

- ATM: AVC-based multiview and 3D test model

- ATMVP or “subblock-based temporal merging candidates”: Alternative temporal motion vector prediction

- AU: Access unit

- AUD: Access unit delimiter

- AVC: Advanced video coding – video coding standard developed by JVT, formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10

- BA: Block adaptive

- BC: See CPR or IBC

- BCW: Biprediction with CU based weighting

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001)

- BDOF: Bi-directional optical flow (formerly known as BIO)

- BDPCM: Block-wise DPCM

- BIF: Bilateral filter

- BL: Base layer

- BMS: Benchmark set (no longer used), a former preliminary compilation of coding tools on top of VTM, which provide somewhat better compression performance, but are not deemed mature for standardization

- BoG: Break-out group

- BR: Bit rate

- BT: Binary tree

- BV: Block vector (used for intra BC prediction)

- CABAC: Context-adaptive binary arithmetic coding

- CBF: Coded block flag(s)

- CC: May refer to context-coded, common (test) conditions, or cross-component

- CCALF: Cross-component ALF

- CCLM: Cross-component linear model

- CCCM: Cross-component convolutional model

- CCP: Cross-component prediction

- CCSAO: Cross-component SAO

- CE: Core Experiment – a coordinated experiment conducted for study of coding technology

- CfE: Call for Evidence

- CfP: Call for Proposals

- CG: Coefficient group

- CGS: Colour gamut scalability (historically, coarse-grained scalability)

- CICP: Coding-independent code points (Rec. ITU-T H.273 | ISO/IEC 23091-2)

- CIIP: Combined inter/intra prediction

- CL-RAS: Cross-layer random-access skip

- CNN: Convolutional neural network

- CPB: Coded picture buffer

- CPMV: Control-point motion vector

- CPMVP: Control-point motion vector prediction (used in affine motion model)

- CPR: Current-picture referencing, also known as IBC – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction

- CST: Chroma separate tree

- CTC: Common test conditions

- CVS: Coded video sequence

- DCI: Decoder capability information

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics)

- DCTIF: DCT-derived interpolation filter

- DF: Deblocking filter

- DIMD: Decoder intra mode derivation

- DMVR: Decoder motion vector refinement

- DoCR: Disposition of comments report

- DPB: Decoded picture buffer

- DPCM: Differential pulse-code modulation

- DPS: Decoding parameter sets

- DRC: Dynamic resolution conversion (synonymous with ARC, and a form of RPR)

- DRF: Deep reference frame (for NNVC inter prediction)

- DT: Decoding time

- DQ: Dependent quantization

- ECS: Entropy coding synchronization (typically synonymous with WPP)

- EE: Exploration experiment – a coordinated experiment conducted for study of coding technology prior during exploration work prior to establishment of a formal project (EEn for some number n is exploration experiment number n)

- EMT: Explicit multiple-core transform

- EOTF: Electro-optical transfer function – a function that converts a representation value to a quantity of output light (e.g., light emitted by a display

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element)

- ECM: Enhanced compression model – a software codebase and test model algorithm for future video coding exploration

- ECV: Extended Colour Volume (up to WCG)

- EIP: Extrapolation based intra prediction

- EL: Enhancement layer

- EOS: End of (coded video) sequence

- ET: Encoding time

- FGRC: Film grain regions characteristics

- FRUC: Frame rate up conversion (pattern matched motion vector derivation)

- GCI: General constraints information

- GDR: Gradual decoding refresh

- GFV: Generative face video

- GLM: Gradient linear model

- GOP: Group of pictures (somewhat ambiguous)

- GPM: Geometry partitioning mode

- GRA: Gradual random access

- GS: Gaussian splatting

- HBD: High bit depth

- HDR: High dynamic range

- HEVC: High Efficiency Video Coding – the video coding standard developed and extended by the JCT-VC, formalized by ITU-T as Rec. ITU-T H.265 and by ISO/IEC as ISO/IEC 23008-2

- HLS: High-level syntax

- HM: HEVC Test Model – a software codebase and video coding design containing selected coding tools that conforms to the HEVC standard design (possibly with under-development extensions) – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM)

- HMVP: History based motion vector prediction

- HOP: High-complexity operating point for NNLF.

- HRD: Hypothetical reference decoder

- HTM: HEVC-based multiview and 3D test model (developed by JCT-3V)

- HyGT: Hyper-cube Givens transform (a type of NSST)

- IBC (also Intra BC): Intra block copy, also known as CPR – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction

- IBDI: Internal bit-depth increase – a technique by which lower bit-depth (8 bits per sample) source video is encoded using higher bit-depth signal processing, ordinarily including higher bit-depth reference picture storage (ordinarily 12 bits per sample)

- IBF: Intra boundary filtering

- ILP: Inter-layer prediction (in scalable coding)

- ILRP: Inter-layer reference picture

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC and HEVC)

- IRAP: Intra random access picture

- ISP: Intra subblock partitioning

- JCCR: Joint coding of chroma residuals

- JCT-3V: Joint collaborative team on 3D video (for AVC and HEVC 3D video extensions)

- JCT-VC: Joint collaborative team on video coding (for HEVC)

- JEM: Joint exploration model – a software codebase previously used for video coding exploration

- JM: Joint model – the primary software codebase that has been developed for the AVC standard

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions

- JVET: Joint video experts team (originally joint video exploration team, initially for VVC, later expanded in scope)

- JVT: Joint video team (for AVC)

- KLT: Karhunen-Loève transform

- kMAC: thousands of multiply-accumulate operations – used as a complexity metric

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures

- LB-CCP: Local-boosting CCP

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP

- LFNST: Low-frequency non-separable transform

- LIC: Local illumination compensation

- LM: Linear model

- LMCS: Luma mapping with chroma scaling (formerly sometimes called “in-loop reshaping”)

- LOP: Low-complexity operating point for NNLF.

- LP or LDP: Low delay P – the variant of the LD conditions that uses P frames

- LUT: Look-up table

- LTRP: Long-term reference picture

- MANE: Media-aware network element

- MC: Motion compensation

- MCP: Motion compensated prediction

- MCTF: Motion compensated temporal pre-filtering

- MDNSST: Mode dependent non-separable secondary transform

- MIP: Matrix-based intra prediction

- MM-CCCM: Multi-model CCCM

- MMLM: Multi-model (cross component) linear mode

- MMVD: Merge with MVD

- MOS: Mean opinion score (a metric for subjective quality evaluation)

- MPEG: Moving picture experts group (an alliance of working groups and advisory groups in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JVET)

- MPM: Most probable mode (in intra prediction)

- MRL: Multiple reference line intra prediction

- MSE: Mean-squared error, a distortion metric

- MV: Motion vector

- MVD: Motion vector difference

- NAL: Network abstraction layer

- NNLF: Neural network loop filter

- NNPF: Neural network post filter

- NNVC: Neural network-based video coding (experimental software package)

- NSQT: Non-square quadtree

- NSPT: Non-separable primary transform

- NSST: Non-separable secondary transform

- NUH: NAL unit header

- NUT: NAL unit type (as in AVC and HEVC)

- OBMC: Overlapped block motion compensation (e.g., as in H.263 Annex F)

- OETF: Opto-electronic transfer function – a function that converts to input light (e.g., light input to a camera) to a representation value

- OLS: Output layer set

- OOTF: Optical-to-optical transfer function – a function that converts input light (e.g. l,ight input to a camera) to output light (e.g., light emitted by a display)

- ONNX: Open Neural Network Exchange – a format used to convert code from common neural network software packages into SADL code

- operation point: A temporal subset of an OLS

- PDPC: Position-dependent (intra) prediction combination

- PERP: Padded equirectangular projection (a 360° projection format)

- PH: Picture header

- PHEC: Padded hybrid equiangular cubemap (a 360° projection format)

- PMMVD: Pattern-matched motion vector derivation

- POC: Picture order count

- PoR: Plan of record

- PROF: Prediction refinement with optical flow

- PPS: Picture parameter set

- PSNR: Peak signal-to-noise ratio, a fidelity metric derived from MSE

- PTL: Profile/tier/level combination

- QM: Quantization matrix

- QP: Quantization parameter (sometimes conflated with quantization step size)

- QT: Quadtree

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD)

- RADL: Random-access decodable leading (type of picture)

- RASL: Random-access skipped leading (type of picture)

- R-D: Rate-distortion

- RDO: Rate-distortion optimization

- RDOQ: Rate-distortion optimized quantization

- RDPCM: Residual DPCM

- ROI: Region of interest

- ROT: Rotation operation for low-frequency transform coefficients

- RPL: Reference picture list

- RPLM: Reference picture list modification

- RPR: Reference picture resampling (e.g., as in H.263 Annex P), a special case of which is also known as ARC or DRC

- RPS: Reference picture set

- RQT: Residual quadtree

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q)

- RVM: Rate variation measure

- SAD: Sum of abolute differences, a measurement of distortion

- SADL: Small adhoc deep learning librar

- SAO: Sample-adaptive offset

- SBT: Subblock transform

- SbTMVP: Subblock based temporal motion vector prediction

- SCIPU: Smallest chroma intra prediction unit

- SCM: Screen content model – a software codebase and test model algorithm for the HEVC screen content coding extensions

- SD: Slice data; alternatively, standard-definition

- SDH: Sign data hiding

- SDT: Signal-dependent transform

- SE: Syntax element

- SEI: Supplemental enhancement information

- SGPM: Spatial geometric partition mode

- SH: Slice header

- SHM: Scalable HM – a software codebase and test model algorithm for SHVC

- SHVC: Scalable high efficiency video coding

- SIF: Switchable (motion) interpolation filter

- SIMD: Single instruction, multiple data

- SMVD: Symmetric MVD

- SPO: SEI processing order

- SPS: Sequence parameter set

- SR: Super-resolution

- STMVP: Spatial-temporal motion vector prediction

- STRP: Short-term reference picture

- STSA: Step-wise temporal sublayer access

- TALF: Temporal adaptive loop filter

- TBA/TBD/TBP: To be announced/determined/presented/public

- TGM: Text and graphics with motion – a category of content that primarily contains rendered text and graphics with motion, mixed with a relatively small amount of camera-captured content

- TIMD: Template-based intra mode derivation

- TM: Template matching

- TMRL: Template-based multiple reference line prediction

- TMVP: Temporal motion vector prediction

- TS: Transform skip

- TSRC: Transform skip residual coding

- TT: Ternary tree

- TuC: Technology under consideration

- UGC: User-generated content

- UWP: Unequal weight prediction

- VCEG: Visual coding experts group (ITU-T Q.6/21, the relevant rapporteur group in ITU-T WP3/21, which is one of the two parent bodies of the JVET)

- VCM: Video coding for machines

- VLOP: Very low-complexity operating point for NNLF

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy

- VQA: Visual quality assessment

- VT: Verification testing

- VTM: VVC Test Model – a software codebase and test model algorithm for the VVC standard

- VUI: Video usability information (part of an SPS, similar to an SEI message that applies to a video sequence)

- VVC: Versatile Video Coding, video coding standard developed by JVET, formalized by ITU-T as Rec. ITU-T H.266 and by ISO/IEC as ISO/IEC 29090-3

- WCG: Wide colour gamut

- WG: Working group, a group of technical experts (usually used to refer to the WGs of ISO/IEC JTC 1/SC 29)

- WPP: Wavefront parallel processing (usually synonymous with ECS)

- Block and unit names in MPEG-2 and AVC:

  - MB: Macroblock – a 16x16 array for the luma component and corresponding 8x8 arrays for the chroma components (basically the same as a CTU in later standards, but with a fixed size)

  - MB pair: A pair of vertically contiguous macroblocks in a frame that is coupled for use in macroblock-adaptive frame/field decoding

  - Sub-MB: One quarter of the samples of a macroblock, i.e., an 8x8 luma block and two corresponding chroma blocks of which one corner is located at a corner of the macroblock for a picture that has three sample arrays or an 8x8 luma block of which one corner is located at a corner of the macroblock for a monochrome picture or a picture that is coded using three separate colour planes.

  - Sub-MB partition: A block of luma samples and two corresponding blocks of chroma samples resulting from a partitioning of a sub-macroblock for inter prediction for a picture that has three sample arrays or a block of luma samples resulting from a partitioning of a sub-macroblock for inter prediction for a monochrome picture or a picture that is coded using three separate colour planes

- Block and unit names in HEVC:

  - CTB: Coding tree block (luma or chroma) – unless the format is monochrome, there are three CTBs per CTU

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU), with a size of 16x16, 32x32, or 64x64 for the luma component

  - CB: Coding block (luma or chroma), a luma or chroma block in a CU

  - CU: Coding unit (containing both luma and chroma), the level at which the prediction mode, such as intra versus inter, is determined in HEVC, with a size of 2Nx2N for 2N equal to 8, 16, 32, or 64 for luma

  - PB: Prediction block (luma or chroma), a luma or chroma block of a PU, the level at which the prediction information is conveyed or the level at which the prediction process is performed in HEVC

  - PU: Prediction unit (containing both luma and chroma), the level of the prediction control syntax within a CU, with eight shape possibilities in HEVC:

    - 2Nx2N: Having the full width and height of the CU

    - 2NxN (or Nx2N): Having two areas that each have the full width and half the height of the CU (or having two areas that each have half the width and the full height of the CU)

    - NxN: Having four areas that each have half the width and half the height of the CU, with N equal to 4, 8, 16, or 32 for intra-predicted luma and N equal to 8, 16, or 32 for inter-predicted luma – a case only used when 2N×2N is the minimum CU size

    - N/2x2N paired with 3N/2x2N or 2NxN/2 paired with 2Nx3N/2: Having two areas that are different in size – cases referred to as AMP, with 2N equal to 16 or 32 for the luma component

  - TB: Transform block (luma or chroma), a luma or chroma block of a TU, with a size of 4x4, 8x8, 16x16, or 32x32

  - TU: Transform unit (containing both luma and chroma), the level of the residual transform (or transform skip or palette coding) segmentation within a CU (which, when using inter prediction in HEVC, may sometimes span across multiple PU regions)

- Block and unit names in VVC:

  - CTB: Coding tree block (luma or chroma) – there are three CTBs per CTU in a P or B slice or in an I slice that uses a single tree, and one CTB per luma CTU and two CTBs per chroma CTU in an I slice that uses separate trees

  - CTU: Coding tree unit (synonymous with LCU, containing both luma and chroma in a P or B slice or in an I slice that uses a single tree, containing only luma or only chroma in an I slice that uses separate trees), with a size of 16x16, 32x32, 64x64, or 128x128 for the luma component

  - CB: Coding block, a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma in P/B slice, containing only luma or chroma in I slice), a leaf node of a QTBT. It’s the level at which the prediction process and residual transform are performed in JEM. A CU can be square or rectangle shape

  - PB: Prediction block, a luma or chroma block of a PU

  - PU: Prediction unit, has the same size as a CU in the VVC context

  - TB: Transform block, a luma or chroma block of a TU

  - TU: Transform unit, has the same size as a CU in the VVC context

## Standards, TRs, supplements and technical papers approval and publication status (update)

- MPEG-2 | H.262 (the video coding specification is common text and not publicly available without cost per ISO special request)

  - ITU-T H.262 V3 was approved in 2012-02; its Amd 1 was approved in 2013-03 and was not published separately; it was instead incorporated directly into the V3 text and published 2013-09

  - ISO/IEC 13818-2:2013 (Ed. 3) FDIS ballot closed 2012-05-08; FDAM 1 ballot closed 2013-04-12 and was not published separately; it was instead incorporated directly into the V3 text and published 2013-10

  - Conformance testing (not joint with ITU-T)

    - ISO/IEC 13818-4:2004 (Ed. 2) FDIS closed 2004-08-22, published 2004-12-12; it specifies conformance testing for Part 1 (Systems), Part 2 (Video), Part 3 (Audio), and Part 7 (AAC)

    - ISO/IEC 13818-4:2004/Amd 3:2009 Level for 1080@50p/60p conformance testing

    - Cor 1:2007, Cor 2:2009, Cor 3:2012 (publication availability unclear, as these do not appear to be directly available from the ISO website) may also have video relevance

  - Reference software (not joint with ITU-T)

    - ISO/IEC TR 13818-5:2005 (Ed. 2) final ballot closed 2005-07-24, published 2005-10; it specifies reference software for Part 1 (Systems), Part 2 (Video), Part 3 (Audio), Part 7 (AAC) and Part 11 (IPMP)

- AVC (twin text)

  - ITU-T H.264 V14 was Consented at 22nd meeting on 2021-04-30 (with annotated regions, shutter interval, and miscellaneous corrections), approved 2021-08-22, published 2021-10-13 (ITU work programme link)

  - ISO/IEC 14496-10:2020 (Ed. 9) FDIS ballot closed 2020-11-27, published 2020-12-15

  - ISO/IEC 14496-10:2022 (Ed. 10), had been forwarded from DIS directly for publication 2022-01-21 (with annotated regions, shutter interval, and miscellaneous corrections) with an editing period, submitted to ITTF in 2022-05 after consultation with ISO staff on format of graphics files, upgraded to “DIS approved for registration” in ISO Project system 2022-07-04, published 2022-11-07

  - Preliminary draft text for YCgCo-Re and YCgCo-Ro issued at 26th meeting, second draft including SMPTE ST 2128 IPT-PQ-C2 issued at 28th meeting 2022-10, third draft issued at 29th meeting 2023-01, fourth draft issued at 30th meeting 2023-04, formal project requested and CD of 11th edition issued at 31st meeting 2023-07, DIS issued at 32nd meeting 2023-10, DAM ballot closed 2024-04-15

    - ITU-T H.264 V15 Consented 2024-04-26, last call began 2024-07-16, approved 2024-08-13, published 2024-11-13

    - ISO/IEC 14496-10:2025 (Ed. 11) FDIS issued at 34th meeting 2024-04, DIS approved for registration as FDIS 2024-09-17, FDIS ballot issued 2025-01-14, FDIS ballot closed 2025-03-23; published 2025-07-10

  - ISO/IEC 14496-10:2025/DAM 1 to support some SEI messages of VSEI v4 requested at 37th meeting 2025-01, registered to work programme 2025-01-25, CDAM issued at 38th meeting 2025-04, consultation initiated 2025-04-23, closed 2025-06-18, DAM issued at 39th meeting 2025-07, ballot opened 2025-10-08, closed 2025-12-31, summary of voting in m75350, ready to issue FDAM or FDIS at current meeting if desired, but it was agreed during the meeting to defer such action to a future meeting (ISO Projects link)

  - ITU-T H.264 (V16) no action at current meeting, targeting consent in 2026-07 (ITU work programme link)

  - Conformance testing (twin text)

    - ITU-T H.264.1 V6 Approved 2016-02-13, published 2016-06-17

    - Various amendments of ISO/IEC 14496-4:2004 (Ed. 2), including:

      - ISO/IEC 14496-4:2004/AMD 6:2005 Advanced Video Coding conformance

      - ISO/IEC 14496-4:2004/AMD 9:2006 AVC fidelity range extensions conformance

      - ISO/IEC 14496-4:2004/AMD 30:2009 Conformance testing for new profiles for professional applications

      - ISO/IEC 14496-4:2004/AMD 31:2009 Conformance testing for SVC profiles

      - ISO/IEC 14496-4:2004/AMD 38:2010 Conformance testing for Multiview Video Coding

      - ISO/IEC 14496-4:2004/AMD 41:2014 Conformance testing of MVC plus depth extension of AVC

      - ISO/IEC 14496-4:2004/AMD 42:2014 Conformance testing of Multi-Resolution Frame Compatible Stereo Coding extension of AVC

      - ISO/IEC 14496-4:2004/AMD 43:2015 3D-AVC conformance testing

      - ISO/IEC 14496-4:2004/AMD 45:2016 Conformance Testing for the Multi-resolution Frame Compatible Stereo Coding with Depth Maps Extension of AVC

  - Reference software (twin text)

    - ITU-T H.264.2 V6 Approved 2016-02-13, published 2016-05-30

    - Various amendments of ISO/IEC 14496-5:2001 (Ed. 2) have been published, including:

      - ISO/IEC 14496-5:2001/AMD 6:2005 Advanced Video Coding (AVC) and High Efficiency Advanced Audio Coding (HE AAC) reference software

      - ISO/IEC 14496-5:2001/AMD 8:2006 AVC fidelity range extensions reference software

      - ISO/IEC 14496-5:2001/AMD 15:2010 Reference software for Multiview Video Coding

      - ISO/IEC 14496-5:2001/AMD 18:2008 Reference software for new profiles for professional applications

      - ISO/IEC 14496-5:2001/AMD 19:2009 Reference software for Scalable Video Coding

      - ISO/IEC 14496-5:2001/AMD 33:2015 Reference software for MVC plus depth extension of AVC

      - ISO/IEC 14496-5:2001/AMD 34:2014 Reference software of the multi-resolution frame compatible stereo coding of AVC

      - ISO/IEC 14496-5:2001/AMD 35:2015 3D-AVC Reference software

      - ISO/IEC 14496-5:2001/AMD 39:2016 Reference software for the Multi-resolution Frame Compatible Stereo Coding with Depth Maps of AVC

      - ISO/IEC 14496-5:2001/AMD 42:2017 Reference software for the alternative depth information SEI message extension of AVC

- HEVC (twin text)

  - ITU-T H.265 V7 approved 2019-11-29, published 2020-01-10

  - ISO/IEC 23008-2:2020 (Ed. 4) FDIS ballot closed 2020-07-16, published 2020-08-27

  - ITU-T H.265 V8 Consented at the 22nd meeting (with shutter interval information SEI message and miscellaneous corrections), approved 2021-08-22, published 2021-10-13

  - ISO/IEC 23008-2:2020/AMD 1:2021 (shutter interval information SEI message) published 2021-07-12

  - ISO/IEC 23008-2:2023 (Ed. 5) began as CDAM 2 High-range levels output of 25th meeting of January 2022, CDAM ballot closed 2022-04-15, conversion to 5th edition with miscellaneous corrections planned at 26th meeting of 2022-04, text submitted for DIS ballot 2022-07-10, DIS ballot closed 2023-01-10, FDIS issued 29th meeting of 2023-01, FDIS ballot opened 2023-08-06, closed 2023-10-02, published 2023-10-30

  - ITU-T H.265 V9 Consented at 31st meeting 2023-07, approved 2023-09-13, and pre-published 2023-09, published 2023-11-24

  - Preliminary draft HEVC text for YCgCo-Re and YCgCo-Ro issued at 26th meeting 2022-04, second draft including SMPTE ST 2128 issued at 28th meeting 2022-10, third draft at 29th meeting 2023-01, fourth draft at 30th meeting 2023-04, formal work item requested and CDAM1 issued 31st meeting 2023-07, DAM issued with new (multiview) profiles and SEI messages at 32nd meeting 2023-10, DAM ballot closed 2024-04-08

    - ITU-T H.265 V10 Consented 2024-04-26, last call opened 2024-07-01, closed 2024-07-28, approved 2024-07-29, pre-published 2024-08-06, published 2024-10-07

    - ISO/IEC 23008-2:2025 (Ed. 6) FDIS issued from 35th meeting 2024-04, DIS approved for registration as FDIS 2024-10-03, FDIS registered for formal approval 2024-10-13, FDIS ballot issued 2024-12-30, FDIS ballot closed 2025-02-24, published 2025-03-20

  - ISO/IEC 23008-2:2025/DAM 1 to support additional (multiview) profiles and some SEI messages of VSEI v4, project requested at 36th meeting 2024-11, project registered in ISO/IEC work programme 2025-01-25, CDAM issued at 37th meeting, consultation began 2025-02-01, closed 2025-03-29, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-07-03, closed 2025-09-15, FDIS issued from 40th meeting 2025-10, DIS approved for registration as FDIS 2026-01-15, pending FDIS ballot, public availability requested 2025-10 (ISO Projects link)

- Post-meeting note: FDIS registered for formal approval 2026-03-09

  - ITU-T H.265 (V11) consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pre-published 2026-01-19 during the current meeting (ITU work programme link)

  - Conformance testing (twin text)

    - ITU-T H.265.1 V3 approved 2018-10-14, published 2019-01-15

    - ISO/IEC 23008-8:2018 (Ed. 2) Conformance specification for HEVC, published 2018-08-06

    - ISO/IEC 23008-8:2018/AMD 1:2019 Conformance testing for HEVC screen content coding (SCC) extensions and non-intra high throughput profiles, published 2019-10-15

  - Reference software (twin text)

    - ITU-T H.265.2 V3 approved 2016-12-22, published 2017-04-10

    - ISO/IEC 23008-5:2017 (Ed. 2) Reference software for high efficiency video coding, FDIS issued from 2016-02 meeting, published 2017-03-01

    - ISO/IEC 23008-5:2017/AMD 1:2017 Reference software for screen content coding extensions, FDAM issued from 2017-04 meeting, FDAM ballot opened 2017-08-10, closed 2017-10-07, published 2017-11-09

- VVC (twin text)

  - ITU-T H.266 V1 approved 2020-08-29, published 2020-11-10

  - ISO/IEC 23090-3:2021 (Ed. 1) published 2021-02-16

  - ITU-T H.266 V2 with operation range extensions, Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-06-06, published 2022-07-12

  - ISO/IEC 23090-3:2022 (Ed. 2) with operation range extensions, approval at WG level to proceed to FDIS 2022-01-21, FDIS ballot opened 2022-06-29, closed 2022-08-24, published 2022-09-25

  - ISO/IEC 23090-3:2024 (Ed. 3), initiated as (Ed. 2) / Amd.1 New level and systems-related supplemental enhancement information, CDAM 1 issued from 26th meeting, ballot closed 2022-07-14, DAM 1 issued from 27th meeting, ballot closed 2023-01-03, FDIS issued at WG level 2023-07, FDIS ballot opened 2024-05-11, closed 2024-06-26, published 2024-07-17

  - ITU-T H.266 V3 Consented 2023-07, approved 2023-09-29 and pre-published 2023-09, published 2023-11-29

    - H.266 (2023) Erratum 1 published 2025-05 to correct errors in a few figures

  - ISO/IEC 23090-3:2024/DAM 1 Request & CDAM issued 2024-04, consultation deferred due to meeting timing, updated text issued 2024-07, consultation initiated 2024-09-05, closed 2024-10-31, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-06-29, closed 2025-09-21, FDIS issued at 40th meeting 2025-10, DIS approved for registration as FDIS 2026-01-15, pending FDIS ballot, public availability requested 2025-10 (ISO Projects link)

- Post-meeting note: FDIS registered for formal approval 2026-03-09

  - ITU-T H.266 (V4) consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pre-published 2026-01-22 during the current meeting (ITU work programme link)

  - Conformance testing (twin text)

    - ITU-T H.266.1 V1 Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-05-17, published 2022-07-12

    - ISO/IEC 23090-15:2022 (Ed. 1) approval at WG level to proceed to FDIS 2022-10-15, upgraded to “DIS approved for registration” in ISO Projects system 2021-10-24, upgraded to “FDIS registered for formal approval” 2022-07-11, FDIS ballot closed 2022-11-04, published 2022-11-24

    - ISO/IEC 23090-15:2024 (Ed. 2) began as Amd.1 Operation range extensions – CDAM 1 issued from 24th meeting 2021-10, DAM 1 issued from 25th meeting 2022-01-21, upgraded to “CD approved for registration as DIS” status in ISO Projects system 2022-05-31, upgraded to “DIS registered” 2022-06-22, DAM ballot closed 2022-11-15, consolidated into FDIS 2nd edition issued as an output of the 29th meeting in January 2023, ballot opened 2024-04-08, closed 2024-06-03, published 2024-07-04

    - ITU-T H.266.1 V2 Consented 2023-07, approved 2023-09-13 and pre-published 2023-09, published 2023-10-19

    - ISO/IEC 23090-15 (Ed. 3) project requested at 37th meeting, project registered in work programme 2025-01-25, CD issued at 38th meeting 2025-04, consultation initiated 2025-05-05, closed 2025-06-30, DIS issued at 39th meeting 2025-07, closed 2025-12-22, summary of voting in m75347, ready for FDIS or to proceed directly to publication at current meeting; it was agreed during the meeting to proceed directly to publication – see Annex C (ISO Projects link)

    - ITU-T H.266.1 (V3) consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pre-published 2026-01-19 during the current meeting (ITU work prgramme link)

  - Reference software (twin text)

    - ITU-T H.266.2 V1 Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-05-17, published 2022-07-12

    - ISO/IEC 23090-16:2022 V1 approval at WG level to proceed to FDIS 2022-01-21, upgraded to “DIS approved for registration” status in ISO Projects system 2022-04-21, upgraded to “FDIS registered for formal approval” 2022-04-22, FDIS ballot initiated 2022-07-24, FDIS ballot closed 2022-09-19, published 2022-10-23

    - ITU-T H.266.2 V2 Consented 2024-04-26, last call opened 2024-05-16, closed 2024-06-12, approved 2024-06-13, pre-published 2024-08-06, published 2024-10-09

    - ISO/IEC 23090-16:2025 (Ed. 2) Request & CD issued 2024-04, consultation deferred due to meeting timing, consultation initiated 2024-09-06, closed 2024-11-01, DIS issued at 36th meeting 2024-11, approved for registration as DIS 2024-12-18, DIS registered 2025-01-7, DIS ballot opened 2025-03-09, closed 2025-06-01, approved to proceed to publication without an FDIS at 29th meeting 2025-07, public availability requested 2025-07, published 2025-11-19, not available without cost. (Roughly corresponding to H.266.2 V2 already approved and published in ITU-T)

- VSEI (twin text)

  - ITU-T H.274 V1 approved 2020-08-29, published 2020-11-10

  - ISO/IEC 23002-7:2021 (Ed. 1) published 2021-01-28

  - ITU-T H.274 V2 Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-05-22 (after 1 Last Call comment and Additional Review), pre-published 2022-06-17, published 2022-07-25

  - ISO/IEC 23002-7:2022 (Ed. 2) approval at WG level to proceed to FDIS 2022-01-21, upgraded to “DIS approved for registration” status in ISO Projects system 2022-05-05 and “FDIS registered for formal approval” 2022-05-08, FDIS ballot closed 2022-09-27, published 2022-10-30

  - ISO/IEC 23002-7:2024 (Ed. 3) began as (2nd Ed.) Amd.1 Request for new edition and CD for additional SEI messages issued at 27th meeting, ballot closed 2022-10-10, DAM registered 2022-11-13, DAM ballot closed 2022-04-06, FDIS 3rd edition issued 2023-07, FDIS ballot began 2024-07-25, closed 2024-09-20, published 2024-10-30

  - ITU-T H.274 V3 Consent 2023-07, approved 2023-09-29, pre-published 2023-10-11, published 2024-03-12

  - ISO/IEC 23002-7:202x (Ed. 4) Request & CDAM 1 issued 2024-04, consultation deferred due to meeting timing, updated text issued 2024-07, CD consultation initiated 2024-09-05, closed 2024-10-31, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-06-30, closed 2025-09-22, FDIS issued at 40th meeting 2025-10, DIS approved for registration as FDIS 2026-01-15, pending FDIS ballot, public availability requested 2025-10 (ISO Projects link)

  - ITU-T H.274 (V4) consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pending pre-publication (ITU work programme link)

- Post-meeting note: Pre-publication occurred on 2026-02-04.

- CICP (twin text)

  - ITU-T H.273 V2 (with 4:2:0 sampling alignment and corrections for range of values for sample aspect ratio, ICTCP equations for HLG, and transfer characteristics function for sYCC of IEC 61966-2-1) Consented on 2021-04-30, Last Call closed during the 23rd meeting with approval on 2021-07-14, published 2021-09-24

  - ISO/IEC 23091-2:2021 (Ed. 2) had been forwarded from DIS directly for publication in 2021-04 and published 2021-10-18

  - ITU-T H.273 V3 Consent 2023-07, approved 2023-09, not published due to waiting for publication of SMPTE ST 2128

  - ITU-T H.273 V4 Consented 2024-04-26, last call opened 2024-06-16, closed 2024-07-13, approved 2024-07-14, pre-published 2024-08-06, published 2024-10-07

  - ISO/IEC 23091-2:2025 (Ed. 3) Request for new edition and CD for new edition (including YCgCo-Re and YCoCg-Ro) issued at 27th meeting, ballot closed 2022-10-10, DIS registered 2022-11-13, DIS ballot closed 2023-04-06, preliminary draft text for including SMPTE ST 2128 issued at 28th meeting, incorporated into preliminary FDIS at 30th meeting 2023-04, FDIS issued 2024-04 (after waiting for publication of SMPTE ST 2128, then proceeding), FDIS registered for formal approval 2024-10-24, FDIS ballot issued 2024-12-03, FDIS ballot closed 2025-01-28, published 2025-02-25

- Conversion and coding practices for HDR/WCG Y′CbCr 4:2:0 video with PQ transfer characteristics (twin text)

  - ITU-T H-Series Supplement H.Sup15 V1, approved 2017-01-27, published 2017-04-12

  - ISO/IEC TR 23008-14:2018 (Ed. 1) published 2018-08-06

- Signalling, backward compatibility and display adaptation for HDR/WCG video coding (twin text)

  - ITU-T H-Series Supplement H.Sup18 V1, approved 2017-10-27, published 2018-01-18

  - ISO/IEC TR 23008-15:2018 (Ed. 1) published 2018-08

- Usage of video signal type code points (twin text)

  - ITU-T H-Series Supplement H.Sup19 V3 approved 2021-04-30, published 2021-06-04

  - ISO/IEC TR 23091-4:2021 (Ed. 3) published 2021-05-23

- Working practices using objective metrics for evaluation of video coding efficiency experiments (twin text)

  - ITU-T H-Series Supplement HSTP-VID-WPOM V1 approved 2020-07-03, published 2020-11

  - ISO/IEC TR 23002-8:2021 (Ed. 1) published 2021-05-20

- Film grain synthesis technologies for video applications (twin text)

  - ISO/IEC TR 23002-9:2024 (Ed. 1) Request for subdivision and WD 1 issued at 25th meeting 2022-01-21, WD 2 issued at 27th meeting, WD 3 issued at 28th meeting, CDTR issued at 29th meeting 2023-01, consultation period ended 2023-07-09, DTR text was issued from the 31st meeting in July 2023, put on hold by ISO staff editors, DTR ballot opened 2024-03-05, closed 2024-04-30, NB comments handled by ISO staff editor in consultation with project editors, published 2024-07-24

  - ITU-T H-Series Supplement H.Sup21 V1 (ex H.Sup-FGST), Agreement 2025-01-24 by ITU-T SG21, pre-publication 2025-03-26, published 2025-05-21

  - ISO/IEC TR 23002-9 (Ed. 2) Edition 2 planned but not yet in formal work programme of ISO/IEC

- Optimization of encoders and receiving systems for machine analysis of coded video content (twin text)

  - ISO/IEC TR 23888-3 (Ed. 1) Request for subdivision issued from 33rd JVET meeting 2024-01, CDTR issued from 34th meeting 2024-04, consultation deferred due to meeting timing, updated text issued from 35th meeting 2024-07, consultation further deferred due to meeting timing, further updated text issued from 36th meeting 2024-11, consultation initiated 2025-01-14, consultation period ended 2025-03-11, DTR issued at 38th meeting 2025-04, DTR ballot opened 2025-09-08, closed 2025-11-03, summary of voting in m75361, ready for action at current meeting if desired, but it was agreed during the meeting to issue a second DTR ballot – See Annex C (ISO Projects link)

  - ITU-T provisional name H.Sup.MACVC (ITU work programme link)

- Freely available ISO/IEC standards (without cost) in the ISO Webstore and IEC Webstore include

  - ISO/IEC 13818-4:2004 Conformance testing for MPEG-2 (confirmed available 2025-10-31)

  - ISO/IEC 13818-4:2004/Amd 3:2009 Level for 1080@50p/60p conformance testing (confirmed available 2025-10-31)

  - ISO/IEC TR 13818-5:2005 Software simulation for MPEG-2 (confirmed available 2025-10-31)

  - Various amendments of ISO/IEC 14496-4:2004 specify conformance for AVC (base text confirmed available 2025-10-31)

  - Various amendments of ISO/IEC 14496-5:2001 (Ed. 2) specify reference software for AVC (base text confirmed available 2025-10-31)

- Intended to be freely available (without cost) but unavailable (without cost) standards and technical reports include the following. (These should be checked for previously issued requests for free availability.)

  - ISO/IEC 14496-10:2025 (Ed. 11) AVC, published 2025-07-10 (public availability request to be confirmed)

  - ISO/IEC 23002-7:2024 (Ed. 3) VSEI, published 2024-10-30 (public availability requested 2023-07)

  - ISO/IEC 23008-2:2025 (Ed. 6) HEVC, published 2025-03 (public availability requested 2024-04)

  - ISO/IEC 23008-5:2017 (Ed. 2) Reference software for HEVC, published 2017-03-01 (public availability requested 2016-02, reiterated 2025-10)

  - ISO/IEC 23008-5:2017/AMD 1:2017 Reference software for HEVC screen content coding extensions, published 2017-11-09 (public availability not requested 2017-01, 2017-04, 2017-07, 2017-10, 2018-01, public availability requested 2025-10)

  - ISO/IEC 23008-8:2018 (Ed. 2) Conformance specification for HEVC, published 2018-08-06 (public availability requested 2018-10, reiterated 2025-10)

  - ISO/IEC 23008-8:2018/AMD 1:2019 Conformance testing for HEVC screen content coding (SCC) extensions and non-intra high throughput profiles, published 2019-10-15 (public availability requested 2025-10)

  - ISO/IEC 23090-3:2024 (Ed. 3) VVC, published 2024-07-17

  - ISO/IEC 23090-15:2024 (Ed. 2) Conformance specification for VVC, published 2024-07-04

  - ISO/IEC 23090-16:202x (Ed. 2) Reference software for VVC, not yet published, public availability requested 2025-07

  - ISO/IEC 23091-2:2025 (Ed. 3) CICP, published 2025-02-25 (public availability requested 2024-04, reiterated 2025-10)

  - ISO/IEC TR 23008-14:2018 (Ed. 1) Conversion and coding practices for HDR/WCG Y′CbCr 4:2:0 video with PQ transfer characteristics, published 2018-08-06, public availability requested 2025-07

  - ISO/IEC TR 23008-15:2018 (Ed. 1) Signalling, backward compatibility and display adaptation for HDR/WCG video coding, published 2018-08, public availability requested 2025-07

  - ISO/IEC 23090-16:2025 (Ed. 2) Reference software for VVC, public availability requested 2025-07

  - ISO/IEC TR 23091-4:2021 (Ed. 3) Usage of video signal type code points, published 2021-05-23, public availability requested 2025-07

  - ISO/IEC TR 23002-8:2021 (Ed. 1) Working practices using objective metrics for evaluation of video coding efficiency experiments, published 2021-05-20, public availability requested 2025-07

  - ISO/IEC TR 23002-9:2024 (Ed. 1) Film grain synthesis technologies for video applications, published 2024-07-24 (not requested 2024-04), public availability requested 2025-07

- It appears necessary to check if all older software and conformance packages are publicly available – it might be that public availability for some was never requested, e.g. for those that were produced by JCT-3V.

## Draft standards progression status for active work items (update)

- AVC ISO/IEC 14496-10:2025/DAM 1 to support some SEI messages of VSEI v4 requested at 37th meeting 2025-01, registered to work programme 2025-01-25, CDAM issued at 38th meeting 2025-04, consultation initiated 2025-04-23, closed 2025-06-18, DAM issued at 39th meeting 2025-07, ballot opened 2025-10-08, closed 2025-12-31, summary of voting in m75350, ready to issue FDAM or FDIS at current meeting if desired, but it was agreed during the meeting to defer such action to a future meeting (ISO Projects link)

  - ITU-T H.264 (V16) no action at current meeting, targeting consent in 2026-07 (ITU work programme link)

- HEVC ISO/IEC 23008-2:2025/DAM 1 to support additional (multiview) profiles and some SEI messages of VSEI v4, project requested at 36th meeting 2024-11, project registered in ISO/IEC work programme 2025-01-25, CDAM issued at 37th meeting, consulation began 2025-02-01, closed 2025-03-29, DAM issued at 38th meeting 2025-04, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-07-03, closed 2025-09-15, FDIS issued from 40th meeting 2025-10, DIS approved for registration as FDIS 2026-01-15, pending FDIS ballot, public availability requested 2025-10 (ISO Projects link)

- Post-meeting note: FDIS registered for formal approval 2026-03-09

  - ITU-T H.265 (V11) consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pre-published 2026-01-19 (ITU work programme link)

- VVC ISO/IEC 23090-3:2024/DAM 1, Request & CDAM issued 2024-04, consultation deferred due to meeting timing, updated text issued 2024-07, consultation initiated 2024-09-05, closed 2024-10-31, DAM issued at 38th meeting 2025-04, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-06-29, closed 2025-09-21, FDIS issued at 40th meeting 2025-10, DIS approved for registration as FDIS 2026-01-15, pending FDIS ballot, public availability requested 2025-10 (ISO Projects link)

- Post-meeting note: FDIS registered for formal approval 2026-03-09

  - ITU-T H.266 (V4) consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pre-published 2026-01-22 (ITU work programme link)

- VVC conformance ISO/IEC DIS 23090-15 (Ed. 3) project requested at 37th meeting, project registered in work programme 2025-01-25, CD issued at 38th meeting 2025-04, consultation initiated 2025-05-05, pending closure 2025-06-30 during the current meeting, DIS issued at 39th meeting 2025-07, closed 2025-12-22, summary of voting in m75347, ready for FDIS or to proceed directly to publication at current meeting; it was agreed during the meeting to proceed directly to publication – see Annex C (ISO Projects link)

  - ITU-T H.266.1 (V3) consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pre-published 2026-01-19 (ITU work programme link)

- VSEI ISO/IEC 23002-7:2024 (Ed. 4) Request & CDAM issued 2024-04, consultation deferred due to meeting timing, updated text issued 2024-07, CD consultation initiated 2024-09-05, closed 2024-10-31, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-06-30, closed 2025-09-22, FDIS issued at 40th meeting 2025-10, DIS approved for registration as FDIS 2026-01-15, pending FDIS ballot, public availability requested 2025-10 (ISO Projects link)

  - ITU-T H.274 (V4) consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pending pre-publication (ITU work programme link)

- Post-meeting note: Pre-publication occurred on 2026-02-04.

- ISO/IEC TR 23002-9 (Ed. 2) Film grain synthesis technology for video applications – Edition 2 planned but not yet in formal work programme of ISO/IEC

- H.Sup.MACVC | ISO/IEC TR 23888-3 (Ed. 1) Optimization of encoders and receiving systems for machine analysis of coded video content – ISO/IEC TR 23888-3 Request for subdivision issued from 33rd JVET meeting 2024-01, CDTR issued from 34th meeting 2024-04, consultation deferred due to meeting timing, updated text issued from 35th meeting 2024-07, consultation further deferred due to meeting timing, further updated text issued from 36th meeting 2024-11, consultation initiated 2025-01-14, consultation period ended 2025-03-11, DTR issued at 38th meeting 2025-04, DTR issued at 38th meeting 2025-04, DTR ballot opened 2025-09-08, closes 2025-11-03, summary of voting in m75361, ready for action at current meeting if desired, but it was agreed during the meeting to issue a second DTR ballot – See Annex C (ISO Projects link)

- A request for free availability in ISO/IEC has to be made for each edition, amendment and corrigendum, and the request needs to be approved in the WG 5 Recommendations. A request form also needs to be filled out (but the form does not need to be issued as a WG 5 document). A freely available URL for the ITU publication should be provided for the ongoing work items when they become finalized.

## Opening remarks

Remarks during the opening session of the meeting Friday 24 April at XXXX-XXXX CEST were as follows.

- Timing and organization of the meeting and online access and calendar posting of session plans were reviewed

  - The initial number of documents was slightly higher than in last meeting (approximately 200 vs. 185 by the time of opening), where an increase is observed in SEI related contributions, whereas other categories have decreased in number (in particular, EE2 has significantly less)

  - Parallel sessions were considered to be necessary (SEI and core coding aspects), starting Wednesday evening

  - Aspects of common interest (final draft CfP, non-SEI HLS, GS SEI, plans for VSEIv5) to be reviewed and discussed at plenary level

- Future non-virtual meetings will be held as hybrid meetings with best-effort remote access (when under ISO/IEC SC 29 auspices) or as physical meetings with remote participation (when under ITU-T SG21 auspices). In terms of practical arrangements, this is identical.

- Plans for subsequent non-virtual meetings were reviewed: July 2026 (Geneva), October 2026 (Hangzhou), January 2027 (Brisbane), April 2027 (Geneva), July 2027 (Tampere), October 2027 (Shenzhen), April 2028 (likely Geneva); meeting location for January 2028 is still to be determined.

- An AHG17 interim meeting was conducted in Aachen, DE, 25-27 February 2026. Good progress was made in preparation of CfP test conditions, with some follow-up investigation at current meeting – viewing planned to be conducted on Saturday, discussion this afternoon about material to be viewed.

- Depending on the status of preparing future standardization activities, it may be necessary to extend the duration of meetings, or plan for AHG meeting days prior to the regular meeting (e.g., for analysis of CfP submissions)

- The meeting logistics, agenda, working practices, policies, and document allocation considerations were reviewed.

  - Access to the meeting was provided using Zoom. The meeting notes by the session chair were to be continually shared via zoom screen sharing

  - Having text and software available is crucial (and not just arriving at the end of the meeting).

- The results of the previous meeting and the meeting report JVET-AO1000 were reviewed (which was finalized some days before the meeting, but sufficiently mature drafts had been available before in the ITU ftp site). Only minor issues in the meeting report were noted and were not considered sufficient to warrant issuing a revision.

- At the current meeting, on-site attendance was recorded via the traditional sign-in sheet. Participants were asked to correct their affiliation and email in case these changed. In the previous meeting, a few cases were found (mostly for those mentioned in the previous bullet point) where the affiliation was outdated. Remote attendance (of those not present in person) will be recorded via the zoom records. It is therefore important to follow the conventions of naming as mentioned before. Participants who cannot be correctly identified will not appear in the attendance sheet.

- There were no objections voiced in the opening plenary to the consideration of late contributions.

- There were again a significant number of documents registered where authors’ given names were not abbreviated according to the JVET custom (which helps produce shorter headings in the JVET meeting report), and/or company affiliation was missing in the authors’ list. Participants were reminded to stick to JVET’s conventions. As for now, the JVET chair took action of correcting those cases in the document registry. Several documents that had been registered without title were marked as withdrawn by the chair when he intended to start categorization in preparing the meeting notes.

- Some general principles:

  - Experts were asked not to pick a specific JVET number for regular documents – this function is reserved for AHG reports, summary reports, and output docs. Reserving numbers without filling a precise title shall also be avoided – the chair may flag such documents as withdrawn, as they cannot be allocated to a certain category in the meeting notes.

  - Experts were asked to always register JVET documents via the “jvet-experts.org” site, not via the MPEG DMS site, as WG 5 docs (as that feature of the DMS site has not been working properly).

  - Experts were asked to inform the chair when the title of a document is changed, or if authors are added. Otherwise, that might not be correct in the meeting notes. Provisional titles such as “EEx contribution” shall be avoided.

- The following ballot result had become available through the SC 29 secretariat:

  - Summary of Voting on ISO/IEC 2nd DTR 23888-3 Optimization of encoders and receiving systems for machine analysis of coded video content mXXXXX

- The FDIS ballots for next editions of HEVC, VVC and VSEI will be started soon (all synchronized in timing).

- The FDIS of a new edition for 14496-10 could be produced at the current meeting (instead of issuing FDAM), and also submission for ITU-T consent on the next versions of H.264 could be conducted, to be submitted to WP3/21 who meet on May 1. A preliminary DoCR WG 5 N 384 had already been produced from the last meeting. It might be good to start the FDIS ballot soon, to have still an overlap in timing with the ballots on HEVC, VVC and VSEI

- If the ballot on the 2nd DTR 23888-3 does not contain comments for technical changes again, it will be forwarded for publication, with editorial updates to be made by ISO editors. No DoCR would be necessary. To be clarified if the corresponding ITU supplement could be submitted from the current meeting, or if it would be better to wait until July to allow better alignment with 23888-3..

- The 3rd edition of VVC conformance 23090-15 was submitted for IS publication.

- Plans for generating the next versions of reference software for AVC, HEVC and VVC (which would need to support new SEI messages from VSEIv4) need to be developed. Earliest target date for ITU-T consent would be July 2026 – should requests be made for ISO? For AVC, it might further be useful to generate a new part of ISO/IEC 14496 containing the entire newest JM software at least, and cancel the corresponding outdated amendments of 14496-5.

- The primary goals of the meeting were:

  - Prepare AVC next version/edition

  - Final Draft CfP preparation, test conditions to be frozen, some alignment on VVC anchors/encodings, complexity analysis and other issues to be improved

  - TuC on VSEI (and CICP?)

  - Non-SEI HLS aspects

  - Gaussian splats – work of Joint AhG (meets Sunday), and Joint EE (was established only at an interim JAhG meeting)

  - Exploration Experiments

    - Neural network-based video coding

    - Enhanced compression beyond VVC

  - Liaison communication – any?

  - VVC white paper update (remove VSEI, some update of the 2021 version)

- Joint meetings were expected with MPEG AG 5 (on matters involving visual quality assessment), and with MPEG WG 2 Requirements and ITU-T VCEG on future video standardization. Another joint meeting was expected Gaussian splatting with VCEG, WGs 2, 4 and 7.

- As a follow-up to communication conducted previously, parent bodies are making further considerations about future JVET management structures. As a step forward, SC 29 decided during its 48th meeting in February to appoint Mathias Wien as WG 5 convenor for the period 2027-2029. This means that (provided SG 21 would agree on this appointment), starting from January 2027 JVET will have two chairs (J.-R. Ohm from SG 21 side and M. Wien from SC 29 side).

- Principles of standards development were discussed.

- Scheduling of sessions was discussed – see under sections 2.6 and 2.15.

## Scheduling of discussions

The times of the meeting sessions followed the needs of the onsite meeting arrangements, with highest priority given to the aim of achieving the goals of the meeting. Typical meeting hours were in the range of 0900-2000 CEST with coffee breaks and lunch breaks as appropriate, however some early morning or late-night sessions were noted to potentially be necessary (sessions could start 0830 at earliest and end by 2100 when the rooms need to be closed). Sessions were announced in the JVET calendar as far as possible in advance, although it was acknowledged that some activities (such as breakout sessions) might be held at short notice.

Coffee breaks were regularly scheduled at 1030 and 1600.

Particular scheduling notes are shown below, although not necessarily 100% accurate or complete:

- Fri. 24 April, 1st day

  - Morning sessions:

    - 0900–XXXX Opening remarks, review of practices, agenda, IPR policy reminder, scheduling

    - XXXX–XXXX Reports of AHGs X-Y

  - Afternoon sessions:

    - TBD – planning viewing of CfP test sequences

    - …

- Sat. 25 April, 2nd day

  - Morning sessions:

    - TBD

    - …

  - Afternoon sessions:

    - TBD

    - …

- Sun. 26 April, 3rd day

  - Morning sessions:

    - TBD

    - …

  - Afternoon sessions:

    - TBD

    - …

- Mon. 27 April, 4th day

  - 0900–1200 MPEG information sharing session

  - Morning sessions:

    - TBD

    - …

  - Afternoon sessions:

    - TBD

    - …

- Tue. 28 April, 5th day

  - Morning sessions:

    - TBD

    - …

  - Afternoon sessions:

    - TBD

    - …

- Wed. 29 April, 6th day

  - 0900–1000 MPEG information sharing session

  - Morning sessions:

    - TBD

    - …

  - Afternoon sessions:

    - TBD

    - …

- Thu. 30 April, 7th day

  - Morning sessions:

    - Plenary level: Liaison review (if any), EE planning, AHG planning, output planning

    - TBD

    - …

  - Afternoon sessions:

    - TBD

    - …

- Fri. 1 May, 8th day

  - 0900–XXXX JVET plenary:

    - Remaining revisits

    - EE document review and approval

    - List of HLS adoptions

    - Establishment of AHGs

    - Approval of output docs (including review as needed)

    - Review of WG 5 meeting recommendations

    - Software timeline

    - Future planning, a.o.b.

  - 1400–XXXX MPEG information sharing session

  - XXXX–XXXX WG 5 approval of meeting recommendations, closing of meeting

## Contribution topic overview (update)

The approximate subject categories and quantity of contributions per category for the meeting were summarized as follows (note that the noted document counts do not include crosschecks and summary reports, and may not be completely accurate; documents which are allocated to multiple sections are only counted in one of them):

- AHG reports (19) (section 3)

- Project development (section 4)

  - AHG1: Deployment and advertisement of standards (2)

  - AHG2: Text development and errata reporting (0)

  - AHG3: Software development (3)

  - AHG3: Test conditions and metrics (1)

  - AHG4: Subjective quality testing and verification testing (2)

  - AHG4: Test and training material (0)

  - AHG5: Conformance test development (1)

  - AHG7: ECM tool assessment (2)

  - AHG8: Optimization of encoders and receiving systems for machine analysis of coded video content (0)

  - AHG10: Encoding algorithm optimization (1)

  - AHG13: Film grain technology (1)

  - Implementation studies (2)

  - Profile/tier/level specification (2)

  - AHG15: Gaming content compression (2)

  - AHG16: Hardware implementation complexity (4)

  - AHG17: CfP on video coding technology beyond VVC (11)

  - AHG18: Ultra-low latency and error resilience (2)

  - CICP (2)

- Low-level tool technology proposals (section 5) with subtopics (number counts excluding BoG and summary reports)

  - AHG11/AHG14 and EE1: Neural network-based video coding (24) (section 5.1)

  - AHG6/AHG12 and EE2: Enhanced compression beyond VVC capability (17) (section 5.2)

- AHG9: High-level syntax (HLS) proposals (section 6) with subtopics

  - Aspects of SEI messages in VSEI, VVC, HEVC and AVC (4) (section 6.1)

  - Study SEI messages in TuC for VSEI (91) (section 6.2)

  - Potential needs for SEI messages on other topics (6) (section 6.3)

  - Usage of SEI messages for Gaussian splatting (14) (section 6.4)

  - SEI software and showcases (2) (section 6.5)

  - Non-SEI HLS aspects (1) (section 6.6)

- Joint meetings, plenary discussions, BoG reports (X) liaison (X), summary of actions (section 7)

- Project planning (section 8)

- Establishment of AHGs (section 9)

- Output documents (section 10)

- Future meeting plans and concluding remarks (section 11)

The document counts above do not include cross-checks and summary reports.

# AHG reports (19)

These reports were discussed during XXXX–XXXX on Friday 24 April 2026 (chaired by JRO).

JVET-AP0001 JVET AHG report: Project Management (AHG1) [J.-R. Ohm (chair), G. J. Sullivan (vice chair)]

JVET-AP0002 JVET AHG report: Draft text and test model algorithm description editing (AHG2) [B. Bross, C. Rosewarne (co-chairs), F. Bossen, A. Browne, S. Kim, S. Liu, J.-R. Ohm, G. J. Sullivan, A. Tourapis, Y.-K. Wang, Y. Ye (vice chairs)]

JVET-AP0003 JVET AHG report: Test model software development (AHG3) [F. Bossen, X. Li, K. Sühring (co-chairs), E. François, Y. He, K. Sharman, V. Seregin, A. Tourapis (vice chairs)]

JVET-AP0004 JVET AHG report: Test material and visual assessment (AHG4) [V. Baroncini, T. Suzuki, M. Wien (co-chairs), W. Husak, S. Iwamura, P. de Lagrange, S. Liu, X. Meng, S. Puri, A. Segall, S. Wenger (vice-chairs)]

JVET-AP0005 JVET AHG report: Conformance testing (AHG5) [I. Moccagatta (chair), F. Bossen, T. Ikai, S. Iwamura, H.-J. Jhu, K. Kawamura, P. de Lagrange, S. Paluri, K. Sühring, Y. Yu (vice chairs)]

JVET-AP0006 JVET AHG report: ECM software development (AHG6) [V. Seregin (chair), J. Chen, R. Chernyak, F. Le Léannec, K. Zhang (vice-chairs)]

JVET-AP0007 JVET AHG report: Tool assessment (AHG7) [X. Li (chair), L.-F. Chen, Z. Deng, J. Gan, E. François, R. Ishimoto, H.-J. Jhu, J. Lainema, X. Li, J. Pardo, A. Stein, H. Wang (vice chairs)]

JVET-AP0008 JVET AHG report: Optimization of encoders and receiving systems for machine analysis of coded video content (AHG8) [S. Liu, J. Ström, S. Wang, M. Zhou (AHG chairs)]

JVET-AP0009 JVET AHG report: SEI message studies (AHG9) [J. Boyce, Y.-K. Wang (co-chairs), T. Chujoh, S. Deshpande, M. M. Hannuksela, Y. He, P. de Lagrange, G. J. Sullivan, H. Tan, A. Tourapis, S. Wenger, P. Wu (vice-chairs)]

JVET-AP0010 JVET AHG report: Encoding algorithm optimization (AHG10) [K. Andersson, P. de Lagrange, A. Duenas (co-chairs), T. Ikai, T. Solovyev, A. Tourapis (vice chairs)]

JVET-AP0011 JVET AHG report: Neural network-based video coding (AHG11) [E. Alshina, F. Galpin, S. Liu (co-chairs), J. Li, Y. Li, R.-L. Liao, M. Santamaria, T. Shao, M. Wien, P. Wu (vice chairs)]

JVET-AP0012 JVET AHG report: Enhanced compression beyond VVC capability (AHG12) [M. Karczewicz, Y. Ye, L. Zhang (co-chairs), B. Bross, R. Chernyak, X. Li, K. Naser, Y. Yu (vice-chairs)]

JVET-AP0013 JVET AHG report: Film grain technologies (AHG13) [W. Husak, P. de Lagrange (co-chairs), S. Deshpande, A. Duenas, X. Meng, M. Radosavljević, A. Segall, G. Teniou, A. Tourapis (vice-chairs)]

JVET-AP0014 JVET AHG report: NNVC software development (AHG14) [F. Galpin (chair), R. Chang, A. Karabutov, Yue Li, Yun Li, M. Santamaria, J. N. Shingala, Z. Xie (vice chairs)]

JVET-AP0015 JVET AHG report: Gaming content compression (AHG15) [S. Puri, J. Sauer (co-chairs), R. Chernyak, A. Duenas, L. Wang, V. Zakharchenko (vice chairs)]

JVET-AP0016 JVET AHG report: Hardware implementation complexity (AHG16) [Y. Zhao, J. Park, I. Moccagatta (co-chairs), H. Huang, T. Ikai, X. Li, K. Naser, N. Song, G. Verba (vice chairs)]

JVET-AP0017 JVET AHG report: Preparation of Call for Proposals (AHG17) [J.-R. Ohm, M. Wien, F. Bossen (co-chairs), E. Alshina, V. Baroncini, J. Chen, R. Chernyak, Z. Deng, P. de Lagrange, C. Lehmann, L. Li, P. Nikitin, D. Rusanovskyy, G. Verba (vice chairs)]

JVET-AP0018 JVET AHG report: Ultra-low latency and packet loss resilience (AHG18) [S. Deshpande, S. Ikonin, V. Zakharchenko (co-chairs), S. Fößel, C. Kim, X. Ma, S. Puri, J. Ström, S. Wenger (vice-chairs)]

(Report of Joint AhG on Gaussian splats JVET-AP0019 to be added and presented after the AhG meets on Sunday)

# Project development (38)

## AHG1: Deployment and advertisement of standards (2)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0020 Deployment status of the HEVC standard [G. J. Sullivan]

JVET-AP0021 Deployment status of the VVC standard [G. J. Sullivan]

## AHG2: Text development and errata reporting (0)

This section is kept as a template for future use.

JVET-AP0058 in 6.1 is related.

## AHG3: Software development (3)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0130 AHG3/AHG9: Public source code release of real-time NNPF implementation in VVdeC [J. Funnell, M. Santamaria, R. Yang, F. Cricri, M. M. Hannuksela, S. Schwarz (Nokia)]

JVET-AP0243 Bug Fixes and Profile Support for MV-HEVC Reference Software (HM) [A. M. Tourapis, D. Podborski, J. Kim, S. Paluri, S. Choi, W. Zia (Apple)] [late]

JVET-AP0245 HDRTools software update for the Enhanced Colour Format Information and Picture Segmentation Information SEI messages [A. M. Tourapis, D. Podborski, J. Kim (Apple)] [late]

## AHG3: Test conditions and metrics (1)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0186 AHG17/AHG4: A Generic Full-Reference Objective Quality Assessment Method for Compressed Video [J. Wang, X. Zhuang, J. Zhang, L. Yu (Zhejiang Univ.)]

## AHG4: Subjective quality testing and verification testing (2)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0054 [AHG4/AHG17] VQQ: An Interactive Display Assessment Tool for Subjective Tests [J. Liu, Z. Chen (Wuhan Univ.)]

JVET-AP0268 Viewing test results of verification test for VVC multilayer coding [M. Wien, V. Baroncini] [miss] [late]

## AHG4: Test and training material (0)

This section is kept as a template for future use.

Related contributions in section 4.16.2 on test materials related to CfP, and JVET-AP0049/JVET-AP0228 in section 5.1.3 on training for neural-network based coding.

## AHG5: Conformance test development (1)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0244 MV-HEVC Conformance Bitstreams for Multiview Extended Profiles [A. M. Tourapis, D. Podborski, J. Kim, S. Paluri, S. Choi, W. Zia (Apple)] [late]

## AHG7: Tool assessment (2)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0170 AHG7: Performance counters for reliable decoding time measurement [S. Hong, Y. Tokumo, T. Ikai (Sharp)]

JVET-AP0207 AHG7: On Per-class BD-Rate Calculation [X. Li (Google)]

## AHG8: Optimization of encoders and receiving systems for machine analysis of coded video content (0)

This section is kept as a template for future use, or be removed after finalization of the TR/Sup.

## AHG10: Encoding algorithm optimization (1)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0070 AHG10: Additional random-access configuration with reduced delay [K. Andersson (Ericsson)]

## AHG13: Film grain technology (1)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0187 AHG13: Proposal to update the technical report on film grain synthesis technology for video applications [M. Radosavljević, Z. Ameur (InterDigital)]

## Implementation studies (2)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0124 Further evaluation of Ali266 for real-time VVC encoding on smartphones [L. Wang, J. Li, J. Liang, K. Xie, S. Fang, X. Zhao, J. Chen, Y. Ye (Alibaba)]

JVET-AP0214 i266 decoder: Towards global VVC deployment on mobile devices [J. R. Arumugam, J. N. Shingala, M. B. Muthukrishnan (Ittiam)]

## Profile/tier/level specification (2)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0242 On the Creation of Multiview 4:4:4 Profiles for HEVC [A. M. Tourapis, D. Podborski, J. Kim, S. Paluri, S. Choi, W. Zia (Apple)] [late] [miss]

JVET-AP0246 HEVC Profile Signalling Reference [A. M. Tourapis, D. Podborski, J. Kim, W. Zia (Apple)] [late]

## AHG15: Gaming content compression (2)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0118 AhG15: Derivation of camera parameters for auxiliary picture reconstruction using Vision Transformers [V. Zakharchenko (Nokia)]

JVET-AP0120 AhG15: Depth map information derivation from reconstructed visual bitstreams consistency evaluation [V. Zakharchenko (Nokia)]

## AHG16: Hardware implementation complexity (4)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0169 AHG16: Further report on weighted sum of all RDOs [Y. Tokumo, S. Hong, T. Ikai (Sharp)]

JVET-AP0191 [AHG16] VTM software extension for counting and constraining per-CU RDOs [K. Naser, F. Le Léannec, F. Galpin, P. de Lagrange (InterDigital), Y. Zhao, E. Alshina (Huawei), M. Wu, J. Yu (HiSilicon), Y. Tokumo, T. Ikai (Sharp)]

JVET-AP0192 [AHG16] Evaluating VTM under HW constraints [K. Naser, F. Le Léannec, F. Galpin, P. de Lagrange, Y. Zhao, E. Alshina (Huawei), Y. Tokumo, T. Ikai (Sharp)]

JVET-AP0253 AHG16: Comparative Study on RDO count and EncT under tool off test [Y. Tokumo, T. Ikai, S. Hong, K.-W. Liang (Sharp)] [late]

## AHG17: CfP on video coding technology beyond VVC (11)

JVET-AP0041 AHG17: AhG meeting notes [M. Wien, J.-R. Ohm, F. Bossen]

### Proposed draft CfP text (4)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0042 AHG17: Suggested editorial changes to the Draft Joint Call for Proposals on Video Compression with Capability beyond VVC [R. Chernyak, S. Liu (Tencent), T. Solovyev, E. Alshina (Huawei)]

JVET-AP0043 AHG17: Suggestion to enable BIM for the GOP-based RPR comparison point [K. Andersson, P. Wennersten (Ericsson)]

JVET-AP0047 AHG17: Updates of Draft Joint CfP text suggested by the AHG [J.-R. Ohm, M. Wien, F. Bossen]

JVET-AP0202 AHG17: On per-frame data reporting in CfP [F. Bossen, F. Galpin, F. Le Léannec, K. Naser, E. François (InterDigital)]

### Test sequences and conditions (7)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0054 and JVET-AP0186 in 4.5 are also related to AHG17.

JVET-AP0044 AhG17/AhG15: New gaming sequences with auxiliary information captured from Carla driving Simulator [Z. Li, P. Jia, J. Sauer, T. Solovyev, E. Alshina (Huawei)]

JVET-AP0045 AHG17: Chroma sample location for CfP sequences [T. Chujoh, H. Kato, Y. Kidani, K. Kawamura (KDDI)]

JVET-AP0046 AHG17: Report on dry-run expert viewing for test sequences under consideration for the CfP [M. Wien]

JVET-AP0050 AHG17: Proposal of target bitrates for the HDR and UHD Gaming sequences for the upcoming CfP [C. Lehmann, A. Wieckowski, B. Bross, J. Pfaff (Fraunhofer HHI), Z. Li, J. Sauer, T. Solovyev, E. Alshina (Huawei)]

JVET-AP0064 AHG17: Liverpool test sequences with updated chroma sample location and target bitrates [H. Dou, A. Foster, D. Collado, J. Calic (BBC)]

JVET-AP0149 AHG17: Additional sequence evaluation results for the upcoming CfP [D. Li, V. Rufitskiy, A. Filippov, H. Qin (TCL)] [late]

JVET-AP0237 AHG17: Improved VTM configuration for 0.2x runtime target [F. Bossen] [late]

## AHG18 Ultra-low latency and packet loss resilience (2)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0226 in 6.2.4 is also related.

JVET-AP0200 AHG18: Random Access GOP4 Configuration for Live-streaming Applications evaluated in ULL test conditions [S. Ikonin, I. Gribushin, M. Sychev, K. Malyshev, V. Khamidullin, B. Shevchenko, X. Ma, E. Alshina (Huawei), Z. Deng (Bytedance)]

JVET-AP0222 AHG18: Decoder side NN-based picture generation for lost picture substitution [S. Ikonin, M. Sychev, K. Malyshev, E. Alshina (Huawei)]

## CICP (2)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0221 Proposed definitions for alpha planes in Video CICP [E. Thomas, E. Potetsianakis, E. Alexiou, M.-L. Champel (Xiaomi)]

JVET-AP0224 Proposed definitions for depth map in Video CICP [E. Thomas, E. Potetsianakis, E. Alexiou, M.-L. Champel (Xiaomi)]

# Low-level tool technology proposals (41)

## AHG11/AHG14: Neural network-based video coding (24)

### Summary and BoG reports

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0023 EE1: Summary report of exploration experiment on neural network-based video coding [E. Alshina, R. Chang, F. Galpin, Yue Li, Yun Li, M. Santamaria, T. Shao, J. Ström, Z. Xie (EE coordinators)]

JVET-AP0048 [AHG11] [AHG14] Teleconference on NNVC [E. Alshina, F. Galpin]

### EE1 contributions: Neural network-based video coding (10)

There was no presentation or discussion about specific proposals in this category – contributions were discussed in the context of the EE summary report JVET-AP0023. For actions decided to be taken, see section 5.2.1, unless otherwise noted.

JVET-AP0051 EE1-3.1: Very Small Deep Reference Frame Generation Network for Inter Prediction Enhancement [T. Shu, X. Chen, W. Zhang, N. Fu, Z. Chen (Wuhan Univ.)]

JVET-AP0272 Crosscheck of JVET-AP0051 (EE1-3.1: Very Small Deep Reference Frame Generation Network for Inter Prediction Enhancement) N. Bhaskar (Huawei) [miss] [late]

JVET-AP0066 EE1-1.1: Dynamic convolution for LOP7 neural in-loop filtering [W. Gwun, K. Choi (KHU), B.-S. Kim, I. Cho, S. Hahm (KBS)]

JVET-AP0189 Crosscheck of JVET-AP0066 (EE1-1.1: Dynamic convolution for LOP7 neural in-loop filtering) [N. J. Gadgil, S. Mishra, R. N. Gadde, W. I. Choi, K. P. Choi (Samsung)] [late]

JVET-AP0255 Crosscheck of JVET-AP0066 (EE1-1.1 and EE1-1.2: Dynamic convolution for LOP7 neural in-loop filtering) [Y. Li (Qualcomm)] [late]

JVET-AP0080 EE1-4: DCVC-RT as a Learned End-to-End Intra Frame Codec [M. Aderdor, T. Solovyev, E. Alshina (Huawei), F. Urban, F. Galpin, E. François (InterDigital)]

JVET-AP0182 EE1-4.1: Integrating End-to-End Learned Intra-Frame Codec with Conventional Codec [N. Zou, A. B. Koyuncu, A. Hallapuro, F. Cricri, H. Zhang, J. Ahonen, M. M. Hannuksela (Nokia)]

JVET-AP0183 EE1-4.2: Integrating End-to-End Learned Intra-Frame Codec with Conventional Codec [N. Zou, A. B. Koyuncu, A. Hallapuro, F. Cricri, H. Zhang, J. Ahonen, M. M. Hannuksela (Nokia)]

JVET-AP0184 EE1-4.3: Integrating End-to-End Learned Intra-Frame Codec with Conventional Codec [A. Hallapuro, N. Zou, A. B. Koyuncu, F. Cricri, H. Zhang, J. Ahonen, M. M. Hannuksela (Nokia)]

JVET-AP0185 EE1-4.4: Integrating End-to-End Learned Intra-Frame Codec with Conventional Codec [A. Hallapuro, N. Zou, A. B. Koyuncu, F. Cricri, H. Zhang, J. Ahonen, M. M. Hannuksela (Nokia)]

JVET-AP0201 EE1-2.1: VLOP4 with new backbone block based on Spatial-Channel Mixing (SCM) [H. Cho, S. Bahk, J. Kim, H. Y. Kim (KHU), D. Kim, S.-C. Lim (ETRI)]

JVET-AP0218 EE1-5: operational bit-exact reproducibility [H. Zhang, N. Le, F. Cricri, N Zou, A. B. Koyuncu, L. Murn (Nokia)]

JVET-AP0232 EE1-4.1.3 and EE1-4.2.3: JPEG-AI as a Learned End-to-End Intra Frame Codec [A. Karabutov, P. Jia, E. Alshina (Huawei)]

### EE1 related and beyond-EE contributions: Neural network-based video coding (10)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0049 [AHG11] On NNVC training sets [E. Alshina, N. Bhaskar, V. Khamidullin (Huawei), F. Galpin, T. Dumas (InterDigital), J. N. Shingala (Ittiam)]

JVET-AP0052 AHG11: Improved H-DRF with Weighted Fusion and Optimized YUV Processing [X. Chen, J. Zhang, Z. Chen (Wuhan Univ.)]

JVET-AP0273 Crosscheck of JVET-AP0052 (AHG11: Improved H-DRF with Weighted Fusion and Optimized YUV Processing) N. Bhaskar (Huawei) [miss] [late]

JVET-AP0063 AHG11: Deep Reference Frame Generation for Inter Prediction Enhancement with motion compensation [P. Bordes, F. Galpin, F. LoBianco, M. Paquiry (InterDigital)]

JVET-AP0140 EE1-related: QP to QI mapping for intra-frame coding with DCVC [V. Rizzello, J. Ström, P. Wennersten, M. Damghanian, D. Liu (Ericsson)]

JVET-AP0144 EE1-related: Statistics on multilayer frame level RDO using DCVC-RT [M. Aderdor, T. Solovyev, E. Alshina (Huawei), F. Urban, F. Galpin, E. François (InterDigital)]

JVET-AP0178 AHG11: Neural Network Super Resolution as Postfilter with Content Adaptation [K. Pham-Dinh, F. Cricri, M. Santamaria, R. Yang, M. M. Hannuksela (Nokia)]

JVET-AP0190 [AHG11] Energy Efficiency of Neural Accelerators [S. Cizel, R. Mullakhmetov, F. Galpin (InterDigital)]

JVET-AP0219 [AHG11] Additional Information on Stable Float Method for Neural Network Inference on GPU devices [H. Zhang, F. Cricri, N. Le, N. Zou, A. B. Koyunku, L. Murn (Nokia)]

JVET-AP0274 Cross-check of JVET-AP0219 ([AHG11] Additional Information on Stable Float Method for Neural Network Inference on GPU devices) A. Karabutov, E. Alshina (Huawei) [miss] [late]

JVET-AP0228 [AHG11] Response to Call for training materials [E. Alshina, J. Ascenso, T. Ebrahimi]

JVET-AP0234 AHG11: Dynamic Quantization and Hardware Optimization for NNIP [A. Akhtar, S. Esenlik, Y. Matsuba, M. Karczewic (Qualcomm)]

### SADL and NNVC implementation, CTC (4)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0053 AhG14: SIMD Improvements of Operators in SADL Library [X. Chen, J. Zhang, Z. Chen (Wuhan Univ.)]

JVET-AP0072 [AHG14] NNLF compact dataset [A. Kherchouche, F. Galpin (InterDigital)]

JVET-AP0211 AHG14: New memory management policy in SADL [F. Galpin (InterDigital)]

JVET-AP0212 AHG14: SADL Update [F. Galpin (InterDigital)]

## AHG6/AHG12: Enhanced compression beyond VVC capability (17)

### Summary and BoG reports

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0024 EE2: Summary report of exploration experiment on enhanced compression beyond VVC capability [V. Seregin, D. Buğdayci Sansli, J. Chen, R. Chernyak, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang (EE coordinators)]

### EE2 contributions: Enhanced compression beyond VVC capability (9)

There was no presentation or discussion about specific proposals in this category – contributions were discussed in the context of the EE summary report JVET-AP0024. For actions decided to be taken, see section 5.2.1, unless otherwise noted.

JVET-AP0059 EE2-3.2: On the Dead-Zone in Quantization [M. Balcilar, M. Blestel, P. Andrivon (Ofinno)]

JVET-AP0241 Crosscheck of JVET-AP0059 (EE2-3.2e: On the Dead-Zone in Quantization) [K. Y. Kim, J.-Y. Kim, J.-H. Son, J.-S. Kwak (WILUS)] [late] [miss]

JVET-AP0248 Crosscheck of JVET-AP0059 (EE2-3.2a: On the Dead-Zone in Quantization) [Y. Yu (OPPO)] [late]

JVET-AP0250 Crosscheck of JVET-AP0059 (EE2-3.2g/h: On the Dead-Zone in Quantization) [Z. Zhang (Alibaba)] [late]

JVET-AP0257 Crosscheck of JVET-AP0059 (EE2-3.2e: On the Dead-Zone in Quantization) [T. N. Canh (Dolby)] [miss] [late]

JVET-AP0269 Crosscheck report of JVET-AP0059: EE2-3.2e [M. G. Sarwer (Google)] [miss] [late]

JVET-AP0271 Crosscheck of JVET-AP0059 (EE2-3.2b/f: On the Dead-Zone in Quantization) C. Zhou (vivo) [miss] [late]

JVET-AP0065 EE2-3.1a/b/c: Dead-Zone Adjustments and Unbiasing of Scalar Quantizers [M. Le Pendu, F. Le Léannec (InterDigital)]

JVET-AP0247 Crosscheck of JVET-AP0065 (EE2-3.1b/c: Dead-Zone Adjustments and Unbiasing of Scalar Quantizers) [Y. Yu (OPPO)] [late]

JVET-AP0270 Crosscheck of JVET-AP0065 (EE2-3.1a: Dead-Zone Adjustments and Unbiasing of Scalar Quantizers) [Y. Sun (Huawei)] [late]

JVET-AP0082 EE2-5.1: Counter-based Temporal Probability Initialization (CTPI) [Z. Xiang, R. Chernyak, H. Huang, S. Liu (Tencent)]

JVET-AP0125 Crosscheck of JVET-AP0082 (EE2-5.1: Counter-based Temporal Probability Initialization (CTPI)) [P. Nikitin (Xiaomi)]

JVET-AP0086 EE2-2.1a/b/c: Modified GPM partition mode [H. Zhang, F. Wang, Y. Yu, H. Yu, D. Wang (OPPO), Y. Kidani, H. Kato, T. Chujoh, K. Kawamura (KDDI)]

JVET-AP0238 Crosscheck of JVET-AP0086 (EE2-2.1a/b/c: Modified GPM partition mode) [X. Li (Alibaba)] [late]

JVET-AP0251 Crosscheck of JVET-AP0086 (EE2-2.1a/b/c: Modified GPM partition mode) [H.-J. Jhu (Kwai)] [late] [miss]

JVET-AP0267 Crosscheck of JVET-AP0086 Test 2.1a (EE2-2.1a/b/c: Modified GPM partition mode) [Z. Deng (Bytedance)] [late]

JVET-AP0087 EE2-4.1: Simplification on TALF [L. Xu, N. Song, Y. Yu, H. Yu, J. Gan, D. Wang (OPPO)]

JVET-AP0254 Crosscheck of JVET-AP0087 (EE2-4.1: Simplification on TALF) [P. Onno (Canon)] [late] [miss]

JVET-AP0107 EE2-3.3 Intra LfnstIdx Prediction [Y. Wang, R. Chernyak, H. Huang, S. Liu (Tencent)]

JVET-AP0264 Crosscheck report of JVET-AP0107 (EE2-3.3 Intra LfnstIdx Prediction) [C. Hollmann (TCL)] [late]

JVET-AP0266 crosscheck of JVET-AP0107: EE2-3.3 Intra LfnstIdx Prediction [K. Naser (InterDigital)] [miss] [late]

JVET-AP0133 EE2-1.1: Improvement on chroma MPM [Z. Li, W. Niu, X. Zeng, M. Jia, Y. Bai (ZTE)]

JVET-AP0265 Crosscheck of JVET-AP0133 (EE2-1.1c: Improvement on chroma MPM) [Y. Liu (Transssion)] [miss] [late]

JVET-AP0145 EE2-1.2: Modification of EIP filter shapes [Y. Liu, Z. Zhang, Y. Huo, Z. Zhang, J. Cai (Transsion)]

JVET-AP0239 Crosscheck of JVET-AP0145 (EE2-1.2: Modification of EIP filter shapes) [H. Qin (TCL)] [late]

JVET-AP0249 Crosscheck of JVET-AP0145 (EE2-1.2: Modification of EIP filter shapes) [W. Niu (ZTE)] [late] [miss]

JVET-AP0168 EE2-1.3: Clipping operation refinements in CCCM modes [P. Onno, G. Laroche, B. Galmiche (Canon), L. Xu, C. Liao, Y. Lu, F. Liang (SYSU)]

JVET-AP0240 Crosscheck of JVET-AP0168 (EE2-1.3: Clipping operation refinements in CCCM modes) [Y. Ahn (Qualcomm)] [late] [miss]

### EE2 related contributions (1)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0143 EE2-related: Combination of JVET-AO0092 and EE2-1.2b [P.-K. Liu, C.-Y. Liu, S.-P. Wang, C.-C. Lin, C.-L. Lin (ITRI), Y.-T. Liu, Z.-H. Zhang, Y.-K. Huo, Z.-H. Zhang, J.-Y. Cai (Transsion)]

JVET-AP0260 Crosscheck of JVET-AP0143 (EE2-related: Combination of JVET-AO0092 and EE2-1.2b) [H. Qin (TCL)] [late]

### ECM modifications and software improvements beyond EE2 (7)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0105 Non-EE2: A bugfix of the subsampling filter for certain CCLM/CCCM tools [T. Chujoh, H. Kato, Y. Kidani, K. Kawamura (KDDI)]

JVET-AP0123 Matrix based MPM prediction [Y. Gao, H. Tian (Shandong Univ.), S. Li, J. Lei, B. Li, F. Xing, P. Han (Hisense)] [late]

JVET-AP0199 Crosscheck of JVET-AP0105 (Non-EE2: A bugfix of the subsampling filter for certain CCLM/CCCM tools) [P. Bordes, F. Galpin (InterDigital)] [miss] [late]

JVET-AP0156 Non-EE2: Adaptive GPM blending weights with linear and Hermite profiles [K. Kishimoto, H. Kato, Y. Kidani (KDDI)]

JVET-AP0180 Non-EE2: Fix for MDIP [W. Niu, Z. Li, M. Jia, X. Zeng, S. Xie, Y. Bai (ZTE)]

JVET-AP0261 Crosscheck of JVET-AP0180 (Non-EE2: Fix for MDIP) [L. Xu (OPPO)] [miss] [late]

JVET-AP0181 Non-EE2: Enhancement of Chroma MPM List Construction [W. Niu, Z. Li, M. Jia, X. Zeng, S. Xie, Y. Bai (ZTE)]

JVET-AP0262 Crosscheck of JVET-AP0181 (Non-EE2: Enhancement of Chroma MPM List Construction) [L. Xu (OPPO)] [miss] [late]

JVET-AP0233 Non-EE2: Improvements on IBC-AMVP candidate list construction [H. Jeong, B. Jeon (SKKU)]

JVET-AP0235 Non-EE2: Temporal CABAC and CTPI at Different CTU Locations [Z. Xiang, R. Chernyak, H. Huang, S. Liu (Tencent)]

### CTC for EE2 and software aspects (including non-ECM SW) (0)

This section is kept as a template for future use.

# High-level syntax (HLS) and related proposals (118)

## AHG9: Study of SEI messages in VSEI, VVC, HEVC and AVC (4)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0058 AHG2/AHG9: Some errata items on VVC, VSEI, HEVC, and/or AVC [Y.-K. Wang, J. Xu (Bytedance)]

JVET-AP0090 AHG9: On the cancel_flag related semantics in the FP, PRI and CTI SEI messages in VSEI v4 [J. Xu, Y.-K. Wang, L. Zhang (Bytedance)]

JVET-AP0091 AHG9: On semantics related to persistence_flag and cancel_flag in various SEI messages in VSEI v4 and VSEI TuC [J. Xu, Y.-K. Wang (Bytedance)]

JVET-AP0217 AHG9: On FGC SEI message payloadType code points in AVC [R. Skupin, Y. Sanchez, C. Hellge, T. Schierl (HHI)]

## AHG9: Study JVET-AO2032 TuC for VSEI (91)

### General (12)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0069 AHG9: On VSEI RBSP [M. M. Hannuksela, J. Boyce, D. Fortin (Nokia)]

JVET-AP0076 AHG9: SW for floating point syntax fl(n) and fl(w,t) [L. Kerofsky, Y. He, S. Zhao, M. Karczewicz (Qualcomm)]

JVET-AP0137 AHG9: On payload sizes for versatile SEI messages [M. Pettersson, M. Damghanian, S. Harribey, R. Sjöberg (Ericsson)]

JVET-AP0138 AHG9: On SEI messages presence [M. Damghanian, S. Harribey, M. Pettersson, R. Sjöberg (Ericsson)]

JVET-AP0160 AHG9: On signaling of identifiers and numbers of layers for SEI messages in TuC [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AP0161 AHG9: On presence of SEI messages in multiple layers [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AP0162 AHG9: On conformance cropping windows for SEI messages in TuC [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AP0163 AHG9: On inference values of syntax elements in SEI messages in TuC [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AP0196 AHG9: Working draft for VSEI v5 [J. Boyce, M. M. Hannuksela, T. Biatek (Nokia)]

JVET-AP0216 AHG9: On SEI extensions for legacy devices [R. Skupin, Y. Sanchez, T. M. Borges, C. Hellge, T. Schierl (HHI)]

JVET-AP0229 AHG9: Signaling options for alpha plane metadata in non-auxiliary picture context in AVC, HEVC and VVC [E. Thomas, E. Potetsianakis, E. Alexiou, M.-L. Champel (Xiaomi)]

Better in 6.6?

JVET-AP0230 AHG9: Signaling options for depth map metadata in non-auxiliary picture context in AVC, HEVC and VVC [E. Thomas, E. Potetsianakis, E. Alexiou, M.-L. Champel (Xiaomi)]

### New extensions or constraints of SEI messages specified in VSEI v4 (7)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0073 AHG9: Proposed Shutter Interval SEI message extension to rolling shutter for version 5 of VSEI [L. Kerofsky, M. Karczewicz (Qualcomm), J. Xu, Y.-K. Wang (Bytedance)]

JVET-AP0083 AHG9: On OMI [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

JVET-AP0110 AHG9: On signaling of AI usage type for AURR SEI message [C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AP0113 AHG9: On signaling content moderation information in TDI SEI message [C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AP0147 AHG9: On the Alpha Channel Information SEI message [C. H. Demarty, E. François, P. de Lagrange, F. Urban, N. Caramelli (InterDigital)]

JVET-AP0175 AHG9: On shutter interval information SEI message [X. Xu, M. Tang, S. Liu (Tencent)]

JVET-AP0252 AHG9: On multiple extensions of SEI messages [H. Tan, C. Kim, J. Nam, J. Lee, J. Lim, S. Kim (LGE)] [late]

### Scalability dimension information (SDI) SEI message (3)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0094 AHG9: On the scalability dimension information SEI message in VSEI TuC [J. Xu, Y.-K. Wang (Bytedance)]

JVET-AP0098 AHG9: On the SDI confidence map [Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm)]

JVET-AP0167 AHG9: Fixes to the Scalability Dimension Information SEI message [J. Chen, Y. Ye (Alibaba)]

### Neural-network post-filter / related URIs (NNPFC / NNPFA) SEI messages (7)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0130 in 4.3 is also related.

JVET-AP0060 AHG9: On nnpfc_uri and nnpfc_tag_uri  [S. Deshpande (Sharp)]

JVET-AP0068 AHG9: Additional tag URIs for the NNPFC SEI message [M. M. Hannuksela, J. Boyce, F. Cricri (Nokia)]

JVET-AP0081 AHG9: On Multiple Inferences for Neural-network Post-filter [S. Deshpande (Sharp)]

JVET-AP0092 AHG9: On redundant bits related to string signalling in the CR and NNPFA SEI messages in VSEI TuC [J. Xu, Y.-K. Wang (Bytedance)]

JVET-AP0171 AHG9: On indication of intermediate data application in EOI, SPO and NNPFC SEI messages [X. Xu, S. Wenger, S. Liu (Tencent)]

JVET-AP0176 AHG9: Bug fixes of NNPF SEI messages in TuC document [X. Xu, S. Liu (Tencent)]

JVET-AP0226 AHG9/AHG18: Missed Picture Generation purpose for NNPF SEI message [S. Ikonin, M. Sychev, K. Malyshev, E. Alshina (Huawei)]

### SEI processing order and processing order nesting (SPO/PON) SEI messages (1)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0215 AHG9: On detection of SEI message extensions in SPO [T. M. Borges, Y. Sanchez, R. Skupin, C. Hellge, T. Schierl (HHI)]

### Encoder optimization information (EOI) SEI message (8)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0067 AHG9: On the encoder optimization information (EOI) SEI message [M. M. Hannuksela, J. Boyce (Nokia)]

JVET-AP0111 AHG9: On spatial quality optimization in EOI SEI message [C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AP0150 AHG9: Suggested Modifications for EOI SEI Message [Y. Gao, P. Wu, S. Xie, Y. Bai (ZTE)]

JVET-AP0152 AHG9: Extension of Temporal Extrapolation Support for EOI SEI Message [Y. Gao, P. Wu, S. Xie, Y. Bai (ZTE)]

JVET-AP0165 AHG9: On temporal resampling information in EOI SEI message [J. Chen, Y. Ye (Alibaba)]

JVET-AP0166 AHG9: On depth information extension in EOI SEI message [J. Chen, Y. Ye (Alibaba)]

JVET-AP0172 AHG9: On temporal sublayer limitation in EOI SEI message [X. Xu, S. Liu (Tencent)]

JVET-AP0173 AHG9: On temporal extrapolation semantics in EOI SEI message [X. Xu, S. Liu (Tencent)]

### Modality information (MI) SEI message (2)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0095 AHG9: On the modality information SEI message extension [J. Xu, Y.-K. Wang (Bytedance)]

JVET-AP0121 AHG9: On MI SEI Extension to Indicate Sample Value Usage[ S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

### Digitally signed content (DSC) SEI messages (5)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0136 AHG9: Fixes to subpicture signing for digitally signed content SEI messages [M. Pettersson, R. Sjöberg, M. Damghanian, S. Harribey (Ericsson)]

JVET-AP0142 AHG9: On subpicture-based DSC SEI messages and subpicture extraction [H. Tan, J. Lee, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AP0164 AHG9: Miscellaneous aspects of digitally signed content SEI messages in TuC [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AP0188 AHG9: TuC DSC subpicture improvements [I. Sodagar (Dolby)]

JVET-AP0231 AHG9: On DSC SEI support for subpictures [J. Boyce, M. M. Hannuksela (Nokia)]

### Packed regions information (PRI) SEI message (1)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0102 AHG9: On the PRI SEI message [Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm)]

### Film grain regions characteristics (FGRC) SEI message (5)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0055 AHG9: Editorial changes for the FGRC SEI message [Y.-K. Wang, J. Xu (Bytedance)]

JVET-AP0056 AHG9: Bug fixes for the FGRC SEI message [Y.-K. Wang, J. Xu, K. Zhang (Bytedance)]

JVET-AP0057 AHG9: Some potential improvements for the FGRC SEI message [Y.-K. Wang, J. Xu (Bytedance)]

JVET-AP0061 AHG9/AHG13: On Film Grain Regions Characteristics SEI message [S. Deshpande, J. Samuelsson-Allendes (Sharp)]

JVET-AP0062 AHG9: On the FGRC SEI message [Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm)]

### Constituent rectangles (CR) SEI message (2+1)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0092 AHG9: On redundant bits related to string signalling in the CR and NNPFA SEI messages in VSEI TuC [J. Xu, Y.-K. Wang (Bytedance)]

Also in 6.2.4

JVET-AP0099 AHG9: On the CR SEI message [Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm)]

JVET-AP0159 AHG9: Miscellaneous aspects of constituent rectangles and enhanced colour format information SEI messages [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

### Quality metrics (QM) SEI message (5)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0088 AHG9: On the QM SEI message [Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm)]

JVET-AP0135 AHG9: On circular reference in the QM SEI message [C. H. Demarty, A. Ak, R. James (InterDigital)]

JVET-AP0139 AHG9: On the QM SEI message [C. H. Demarty, A. Ak, R. James (InterDigital)]

JVET-AP0154 AHG9: On the quality metrics SEI message [Y. Gao, P. Wu, S. Xie, S. Wang, Y. Bai (ZTE)]

JVET-AP0174 AHG9: On quality metrics SEI message [X. Xu, S. Wenger, S. Liu (Tencent)]

### Lens optical correction (LOC) SEI message (1)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0075 AHG9: Proposed Lens Optical Correction SEI message for version 5 of VSEI [L. Kerofsky, Y. He, S. Zhao, M. Karczewicz (Qualcomm), G. Teniou, S. Wenger (Tencent)]

### Display overlays information (DOI) SEI message (6)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0103 AHG9: On the DOI SEI message [Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm)]

JVET-AP0104 AHG9: On the DOI alpha component [Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm)]

JVET-AP0114 AHG9: On miscellaneous aspects of DOI SEI message [C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AP0115 AHG9: On viewer advisory overlay in the DOI SEI message [C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AP0131 AHG9: On display overlays information SEI message [K. Abe, T. Nishi (Panasonic)]

JVET-AP0158 AHG9: On signaling of target picture in display overlays information SEI message [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

### Enhanced colour format information (ECFI) SEI message (2+1)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0245 in 4.3 is also related.

JVET-AP0106 AHG9: On signalling packed channels in ECFI SEI message [H. Tan, C. Kim, J. Lee, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AP0116 AHG9: On single-layer and chroma format support in the ECFI SEI message [C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AP0159 AHG9: Miscellaneous aspects of constituent rectangles and enhanced colour format information SEI messages [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

Also in 6.2.11

### Photosensitive content information (PCI) SEI message (1)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0129 AHG9: On PCI SEI [T. Biatek, S. He, J. Boyce, M. M. Hannuksela (Nokia)]

### Display rectangles (DR) SEI message (3)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0119 AHG9: On display rectangles SEI message [J. Nam, H. Tan, J. Lee, C. Kim, J. Lim, S. Kim (LGE)]

JVET-AP0126 AHG9: On DR SEI [T. Biatek, J. Boyce, M. M. Hannuksela (Nokia)]

JVET-AP0155 AHG9: On the display rectangles SEI message [S. Xie, P. Wu, Y. Gao, W. Niu, Y. Bai (ZTE)]

### Picture segmentation information (PSI) SEI message (1)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0245 in 4.3 is also related.

JVET-AP0089 AHG9: On the PSI SEI message [Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm)]

### Danmu information (DI) SEI message (6)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0097 in 6.5 is also related.

JVET-AP0117 AHG9: On danmu information SEI messages[ C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AP0127 AHG9: On Danmu SEI [T. Biatek, S. He, J. Boyce, M. M. Hannuksela (Nokia)]

JVET-AP0132 AHG9: On Danmu Information SEI [S. He, T. Biatek, J. Boyce, M. M. Hannuksela (Nokia)]

JVET-AP0151 AHG9: On the default parameters in the DI SEI message [S. Xie, P. Wu, Y. Gao, W. Niu, Y. Bai (ZTE)]

JVET-AP0153 AHG9: Enable the use of segmentation planes with the DI SEI message [S. Xie, P. Wu, Y. Gao, W. Niu, Y. Bai (ZTE)]

### Colour mapping information (CMI) SEI message (4)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0071 AHG9: On the colour mapping information SEI message [J. Samuelsson-Allendes, S. Deshpande (Sharp)]

JVET-AP0096 AHG9: On the colour mapping information SEI message [J. Xu, Y.-K. Wang, K. Zhang (Bytedance)]

JVET-AP0122 AHG9: On CMI [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

JVET-AP0128 AHG9: On CMI SEI [T. Biatek, J. Boyce, M. M. Hannuksela (Nokia)]

### Auxiliary sampling information SEI message (1)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0148 AHG9: On Auxiliary sampling alignment information SEI [S. Wang, Y. Gao, P. Wu, S. Xie, Y. Bai (ZTE)]

### Localization and mapping (LAM) SEI message (5)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0074 AHG9: Proposed Localization and Mapping SEI message for version 5 of VSEI [L. Kerofsky, Y. He, S. Zhao, M. Karczewicz (Qualcomm), G. Teniou, S. Wenger (Tencent)]

JVET-AP0093 AHG9: On group cancellation for the localization and mapping SEI message [J. Xu, Y.-K. Wang (Bytedance)]

JVET-AP0112 AHG9: On miscellaneous aspects of LAM SEI message [C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AP0157 AHG9: On localization and mapping SEI message with multiple cameras [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AP0204 AHG9: Rotation precision in LAM SEI message [Y. Sanchez, R. Skupin, P. Helle, T. Hinz, J. Pfaff, H. Schwarz, D. Marpe, T. Wiegand (Fraunhofer HHI)]

### Sample interleaving SEI message (3)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0077 AHG9: On Sample Interleaving [L. Kerofsky, Y. He, S. Zhao, M. Karczewicz (Qualcomm)]

JVET-AP0134 AHG9: On the Sample Interleaving SEI message [C. H. Demarty, A. Ak, R. James (InterDigital)]

JVET-AP0193 AHG9: Syntax for the sample interleaving SEI message [J. Boyce, S. He, T. Biatek, M. M. Hannuksela (Nokia)]

## Identify potential needs for additional SEI messages, including study of AVC and HEVC SEI messages for use in VVC (6)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0078 AHG9: Support of Infrared Auxiliary Picture (AUX_IR) and IR Information SEI for Sample Interpretation [S. Zhao, L. Kerofsky, Y. He, M. Karczewicz (Qualcomm)]

JVET-AP0084 AHG9: Support of Gain Map Auxiliary Picture Type and Gain Map SEI Message [S. Zhao, L. Kerofsky, Y. He, M. Karczewicz (Qualcomm)]

JVET-AP0085 AHG9: Subject Recognition Information (SRI) SEI message [Q. Zhang (BOE)]

JVET-AP0177 AHG9: Transcoding history information SEI message [X. Xu, S. Wenger, S. Liu (Tencent), Johan Pardo, Alexander Karabutov, Hongjie You, Atanas Boev, Johannes Sauer, Timofey Solovyev, Elena Alshina (Huawei)]

JVET-AP0179 AHG9: Historical Information SEI message [J. Pardo, A. Karabutov, H. You, A. Boev, J. Sauer, T. Solovyev, E. Alshina (Huawei), X. Xu, S. Liu (Tencent)]

JVET-AP0197 AHG9: Shot summarization information SEI message [J. Boyce, M. M. Hannuksela (Nokia)]

## Usage of SEI messages for Gaussian splatting technology (14)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0079 AHG9: Triplane video-based implicit Gaussian splatting with a scaffold-based anchor point framework [B. Kathariya, D. Y. Lee, T. Huang, F. Pu, G. Su, P. Yin, G. J. Sullivan, S. Oh, W. Husak (Dolby)]

JVET-AP0100 AHG9: Gaussian splatting information SEI message [Y. He, J. Jung, L. Kerofsky, G. van der Auwera, M. Karczewicz (Qualcomm), J. Ricard, G. Teniou, S. Wenger (Tencent), J. Xu, Y.-K. Wang, L. Zhang (Bytedance)]

JVET-AP0101 AHG9: GSI SEI message workflow and packing examples [Y. He, J. Jung, L. Kerofsky, G. van der Auwera, M. Karczewicz (Qualcomm), J. Ricard, G. Teniou, S. Wenger (Tencent), J. Xu, Y.-K. Wang, L. Zhang (Bytedance)]

JVET-AP0108 AHG9: On Gaussian Splatting Information SEI message [J. Do, H. Lee, G. Bang (ETRI)]

JVET-AP0109 AHG9: Enhancements to the Gaussian Splatting Information SEI message [H. Lee, J. Kang, J. Do, G. Bang (ETRI)]

JVET-AP0194 AHG9: Common picture format for Gaussian splats coding [J. Boyce, M. M. Hannuksela (Nokia)]

JVET-AP0195 AHG9: Splatting information SEI with patches [J. Boyce, M. M. Hannuksela (Nokia)]

JVET-AP0205 AHG9: On the GSI SEI message for spatial random access [Y. He, J. Jung, L. Kerofsky, G. van der Auwera, M. Karczewicz (Qualcomm), J. Ricard, G. Teniou, S. Wenger (Tencent)]

JVET-AP0213 AHG9: On implicit representations with the Gaussian splatting information SEI message [S. Lee, S. Sasse, Y. Berendsohn, Y. Sanchez, R. Skupin, T. M. Borges, C. Hellge, T. Schierl (HHI)]

JVET-AP0256 Crosscheck of JVET-AP0213 (AHG9: On implicit representations with the Gaussian splatting information SEI message) [Y.-G. Kim, R. Koo, E.-S. Ry (SKKU)] [late]

JVET-AP0220 AHG9: Showcase for Real-time Decoding using Video-based Implicit Representations for 4DGS [S. Lee, S. Sasse, Y. Berendsohn, Y. Sanchez, R. Skupin, C. Hellge, T. Schierl (HHI)]

JVET-AP0223 AHG9: Software implementation and results of the GSI SEI message with HM, HM-SCM, and VTM [J. Ricard, G. Teniou, S. Wenger (Tencent)]

JVET-AP0225 AHG9: Extended Gaussian splatting information SEI message with SH non-uniform quantization and 2D transforms [J. Ricard, G. Teniou, S. Wenger (Tencent)]

JVET-AP0227 AHG9: Content-adaptive packing order optimization for 3D Gaussian splatting video coding [J. Ricard, G. Teniou, S. Wenger (Tencent)]

JVET-AP0236 SPARK-GS: SPlatting with kd-tRee partitioning and toKenization (informational) [A. Badshah, Z. Li (UMKC), J. Jung, A. Akhtar, G. van der Auwera (Qualcomm)] [miss] [late]

## SEI software and showcases (2)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0130 in 4.3 and JVET-AP0220 in 6.4 are also related.

JVET-AP0097 AHG9: Implementation and showcase for the danmu information SEI message [J. Xu (Bytedance)] [miss] [late]

JVET-AP0146 AHG9: Software implementation of the Green SEI message and demonstration of the use of attenuation map information metadata for display adaptation [C. H. Demarty, N. Caramelli, F. Aumont (InterDigital)]

## Non-SEI HLS aspects (1)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0263 Subpicture support in HEVC [A. M. Tourapis, D. Podborski, J. Kim, S. Paluri (Apple)] [miss] [late]

# Plenary meetings, joint meetings, BoG reports, and liaison communications

## General

### Plenary on XXday 2X April

The following topics were discussed in this JVET plenary:

- Status of documents

- Liaison communication (section 7.5)

- Scheduling for the remaining week was discussed (further detail on scheduling is recorded in section 2.12)

- Joint meetings (different zoom link, see calendar)

  - Joint meeting with XXX on XXX (XXday XXXX-XXXX)

  - …

- Review status from main track

  - …

- Review status from HLS track and discussion on potential open issues

  - …

- Output document planning (section 10)

- Review of documents at plenary level: sections XX

- Adoptions Summary from the HLS track (approved in plenary)

  - …

In the context of discussion mandates of AHG7, it was suggested that it might be desirable to conduct investigation with non-CTC sequences, potentially under CfE/CfP conditions (rate matching not necessary).

## MPEG information sharing meetings

Information sharing sessions with other WGs and AGs of the MPEG community were held on Monday 27 April 0900–1200, Wednesday 29 April 0900–1000, and Friday 1 May 1400–1600.

The status and plans for the work in the MPEG WGs and AGs was reviewed at these information sharing sessions.

## Joint meetings

### Joint session XXXX-XXXX XXday 2X April on XXX: MPEG WG 4 / Video, MPEG WG 5 / JVET, and VCEG (ITU-T Q6/21)

The session description is based on notes by XXX.

This joint session was chaired by Lu Yu (MPEG WG 4 Convenor), Jens-Rainer Ohm (JVET Chair), Gary J. Sullivan (SC 29 Chair & Q6/21 Rapporteur), Jörn Ostermann (MPEG AG 2 Convenor), and Yan Ye (Q6/21 Associate Rapporteur).

…

### Joint session XXXX-XXXX XXday 2X April on Gaussian splat coding: MPEG WG 4 / Video, MPEG WG 5 / JVET, MPEG WG 7 3D Graphics, and VCEG (ITU-T Q6/21)

The session description is based on notes by XXX.

This session was chaired by Lu Yu (MPEG WG 4 Convenor), Gary Sullivan (SC 29 Chair and Q6/21 Rapporteur), Jörn Ostermann (MPEG AG 2 Convenor), Yan Ye (Q6/21 Associate Rapporteur), and Marius Preda (MPEG WG 7 Convenor).

…

### Joint session XXXX-XXXX XXday 2X April on next generation video standardization Call for Proposals: MPEG WG 2 / Requirements, MPEG WG 5 / JVET, MPEG AG 5 Visual Quality Assessment, and VCEG (ITU-T Q6/21)

This joint session was chaired by Jens-Rainer Ohm (JVET Chair and WG 5 Convenor), Gary Sullivan (VCEG Rapporteur and SC 29 Chair), Yan Ye (VCEG Associate Rapporteur), Mathias Wien (AG 5 Convenor), and Jörn Ostermann (AG 2 Convenor) and Mary-Luc Champel, on behalf of Igor Curcio (WG 2 Convenor).

This section is based on notes taken by XXX

…

## BoGs (2)

…

# Project planning (update)

## Software timeline

ECM 19.1 software was planned to be available 1 week after the meeting (31 Jan.).

The NNVC 16.0 codebase software was planned to be available 4 weeks after the meeting (20 Feb.), including all elements needed for CTC and EE1. Additional integration of various SADL aspects and harmonization with VTM may be deferred for version 16.1.

VTM23.14 software was planned to be released within one week after the meeting (31 Jan.). VTM 24.0 was planned to be released within two week after the meeting (6 Feb.), including full implementation of all SEI messages in JVET-AN1019 and JVET-AN2001. Additional versions may be released as appropriate.

Updates on top of HM18.0 and JM19.1 software were planned to be released within approximately 3 weeks after the meeting (integration and updates of SEI messages included in JVET-AN1018 and JVET-AN1017). Additional versions may be released as appropriate.

An update on top of SHM12.4 was planned to be released within approximately 2 weeks after the meeting (harmonization with HM, updates of build system, and other improvements). Additional versions may be released as appropriate.

As a general rule in software development, a person who is executing a merge shall not be from the same company as the person who submitted that merge request.

## Core experiment and exploration experiment planning (update)

An EE on neural network-based video coding was established, as recorded in output document JVET-AO2023.

An EE on enhanced compression technology beyond VVC capability using techniques other than neural-network technology was also established, as recorded in output document JVET-AO2024.

Initial versions of these documents were presented and approved (see section 10).

JEE Gaussian splat?

## Drafting of specification text, encoder algorithm descriptions, and software

The following agreement has been established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM/VTM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in EEs).

Suggestions for future meetings included the following generally-supported principles:

- Normative contributions (relating to changes in bitstream/decoder) shall include draft specification text

- Proposals shall contain all details relevant for understanding and be self-contained. In cases where the document is a follow-up of a previous contribution, the overall concept and the novelties should be highlighted at minimum

- Coding tool and encoder optimization proposals shall contain Excel sheets that allow assessment on a per-sequence basis

- Algorithm description text is strongly encouraged for non-normative contributions that are intended to be included in model description documents (VTM, ECM, etc.), and that is required for inclusion in TR drafts.

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions (not exercised currently)

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name.

## General issues for experiments

It was emphasized that those rules which had been set up or refined during the 12th JVET meeting should be observed. In particular, for some CEs of some previous meetings, results were available late, and some changes in the experimental setup had not been sufficiently discussed on the JVET reflector.

Group coordinated experiments have been planned as follows:

- “Core experiments” (CEs) are the coordinated experiments on coding tools which are deemed to be interesting but require more investigation and could potentially become part of a draft standard by the next meeting or in the near future.

- “Exploration experiments” (EEs) are also coordinated experiments. These are conducted on technology which is not foreseen to become part of a draft standard in the near future. The investigating methodology for assessment of such technology can also be an important part of an EE. (Further general rules for EEs, as far as deviating from the CE rules below, should be discussed in a future meeting. For the current meeting, procedures as described in the EE description document are deemed to be sufficient.)

- A CE is a test of a specific fully described technology in a specific agreed way. It is not a forum for thinking of new ideas (like an AHG). The CE coordinators are responsible for making sure that the CE description is complete and correct and has adequate detail. Reflector discussions about CE description clarity and other aspects of CE plans are encouraged.

- A description of each experiment is to be approved at the meeting at which the experiment plan is established. This should include the issues that were raised by other experts when the tool was presented, e.g., interference with other tools, contribution of different elements that are part of a package, etc. The experiment description document should provide the names of individual people, not just company names.

- Software for tools investigated in a CE will be provided in one or more separate branches of the software repository. Each CE will have a “fork” of the software, and within the CE there may be multiple branches established by the CE coordinator. The software coordinator will help coordinate the creation of these forks and branches and their naming. All JVET members will have read access to the CE software branches (using shared read-only credentials as described below).

- During the experiment, revisions of the experiment plans can be made, but not substantial changes to the proposed technology. Withdrawing parts of experiments that were intended to show the individual benefits of a tool or parts of a tool is strongly discouraged. Combination tests may not be considered in such cases. Any changes made to individual tools in a combination shall be documented.

- The CE description must match the CE testing that is done. The CE description needs to be revised if there has been some change of plans.

- The CE summary report must describe any changes that were made in the process of finalizing the CE.

- By the next meeting it is expected that at least one independent cross-checker will report a detailed analysis of each proposed feature that has been tested and confirm that the implementation is correct. Commentary on the potential benefits and disadvantages of the proposed technology in cross-checking reports is highly encouraged. Having multiple cross-checking reports is also highly encouraged (especially if the cross-checking involves more than confirmation of correct test results). In cases where combinations of different elements are planned to be tested, mutual cross-checking of the individual elements by other parties of the combination is discouraged. The combination must be cross-checked by an independent party. The reports of cross-checking activities may (and generally should) be integrated into the CE report rather than submitted as separate documents.

- It is mandatory to report encoder optimizations made for the benefit of a tool, and if an equivalent optimization could be applied on the anchor, a comparison against the improved anchor shall be provided.

- A new proposal can be included in a CE based on group decision, regardless if an independent party has already performed a cross-check in the meeting when it was first proposed.

It is possible to define sub-experiments within particular CEs, for example designated as CEX.a, CEX.b, etc., where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the group test model software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments for SDR video are described in the prior output document JVET-AL2010.

Experiment descriptions should be written in a way such that it is understood as a JVET output document (written from an objective “third party perspective”, not a proponent perspective – e.g., not referring to methods as “improved”, “optimized”, “enhanced”, etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions contain a basic description of the technology under test, but should not contain excessively verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in documents that are referenced in the CE description that are also available in the JVET document archive.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

The CE development workflow was previously described at:

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/wikis/Core-experiment-development-workflow

However, it was noted that the link doesn’t seem to exist anymore.

CE read access is available using shared accounts: One account exists for MPEG members, which uses the usual MPEG account data. A second account exists for VCEG members with account information available in the TIES informal ftp area (IFA) system at:

https://www.itu.int/ifa/t/2017/sg16/exchange/wp3/q06/vceg_account.txt

Some agreements relating to CE activities were established as follows:

- Only qualified JVET members can participate in a CE.

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting. Participation was requested by contacting the CE coordinator.

- All software, results, and documents produced in the CE should be announced and made available to JVET in a timely manner.

- A JVET CE reflector will be established and announced on the main JVET reflector. Discussion of logistics arrangements, exchange of data, minor refinement of the test plans, and preparation of documents shall be conducted on the JVET CE reflector, with subject lines prefixed by “[CEx: ]”, where “x” is the number of the CE. All substantial communications about a CE other than such details shall take place on main JVET reflector. In the case that large amounts of data are to be distributed, it is recommended to send a link to the data rather than the data itself, or upload the data as an input contribution to the next meeting.

General timeline for CEs

T1= 3 weeks after the JVET meeting: To revise the CE description and refine questions to be answered. Questions should be discussed and agreed on JVET reflector. Any changes of planned tests after this time need to be announced and discussed on the JVET reflector. Initially assigned description numbers shall not be changed later. If a test is skipped, it is to be marked as “withdrawn”.

T2 = Test model software release + 2 weeks: Integration of all tools into a separate CE branch of the VTM is completed and announced to JVET reflector.

- Initial study by cross-checkers can begin.

- Proponents may continue to modify the software in this branch until T3.

- 3rd parties are encouraged to study and make contributions to the next meeting with proposed changes

T3: 3 weeks before the next JVET meeting or T2 + 1 week, whichever is later: Any changes to the CE test branches of the software must be frozen, so the cross-checkers can know exactly what they are cross-checking. A software version tag should be created at this time. The name of the cross-checkers and list of specific tests for each tool under study in the CE plan description shall be documented in an updated CE description by this time.

T4: Regular document deadline minus 1 week: CE contribution documents including specification text and complete test results shall be uploaded to the JVET document repository (particularly for proposals targeting to be promoted to the draft standard at the next meeting).

The CE summary reports shall be available by the regular contribution deadline. This shall include documentation about crosscheck of software, matching of CE description and confirmation of the appropriateness of the text change, as well as sufficient crosscheck results to create evidence about correctness (crosscheckers must send this information to the CE coordinator at least 3 days ahead of the document deadline). Furthermore, any deviations from the timelines above shall be documented. The numbers used in the summary report shall not be changed relative to the description document.

CE reports may contain additional information about tests of straightforward combinations of the identified technologies. Such supplemental testing needs to be clearly identified in the report if it was not part of the CE plan.

New branches may be created which combine two or more tools included in the CE document or the VTM (as applicable).

It is not necessary to formally name cross-checkers in the initial version of the CE description document. To adopt a proposed feature at the next meeting, JVET would like to see comprehensive cross-checking done, with analysis of whether the description matches the software, and a recommendation of the value of the tool and given tradeoffs.

The establishment of a CE does not indicate that a proposed technology is mature for adoption or that the testing conducted in the CE is fully adequate for assessing the merits of the technology, and a favourable outcome of CE does not indicate a need for adoption of the technology into a standard or test model.

Availability of specification text is important to have a detailed understanding of the technology and also to judge what its impact on the complexity of the specification will be. There must also be sufficient time to study this in detail. CE contributions without sufficiently mature draft specification text in the CE input document should not be considered for adoption.

Lists of participants in CE documents should be pruned to include only the active participants. Read access to software will be available to all members.

# Establishment of ad hoc groups (update)

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups was agreed to be the main JVET reflector (jvet@lists.rwth-aachen.de).

Chairs of AHGs had been asked to send draft mandates to JRO before XXXX on XX April, preferably copy from the table below and sending with changemarks or yellow highlight of changes.

Review of AHG plans was conducted during the plenary on XXday XX April 2026 at XXXX–XXXX.

| Title and Email Reflector | Chairs | Interim mtg. |
| --- | --- | --- |
| Project Management (AHG1) (jvet@lists.rwth-aachen.de) - Coordinate overall JVET interim efforts. - Supervise AHG and experiment studies. - Report on project status to JVET reflector. - Provide a report to the next meeting on project coordination status. - Supervise processing and delivery of output documents. - Conduct consultation with parent bodies on future JVET management structures. | J.-R. Ohm (chair), G. J. Sullivan (vice-chair) | N |
| Draft text and test model algorithm description editing (AHG2) (jvet@lists.rwth-aachen.de) - Produce and finalize draft text outputs of the meeting (JVET-AO1016, JVET-AO1017). - Collect reports of errata for VVC, VSEI, HEVC, AVC, CICP, and the published related technical reports. - Coordinate with AHG3 to address issues relating to mismatches between software and text. | B. Bross, C. Rosewarne (co-chairs), F. Bossen, A. Browne, S. Kim, S. Liu, J.-R. Ohm, G. J. Sullivan, A. Tourapis, Y.-K. Wang, Y. Ye (vice-chairs) | N |
| Test model software development (AHG3) (jvet@lists.rwth-aachen.de) - Coordinate development of test models (VTM, HM, SCM, SHM, HTM, MFC, MFCD, JM, JSVM, JMVM, 3DV-ATM, 360Lib, and HDRTools) software and associated configuration files. - Produce documentation of software usage for distribution with the software. - Enable software support for recently standardized additional SEI messages (for both VTM and HM), and SEI messages in TuC (the latter in a separate branch of VTM). - Discuss and make recommendations on the software development process. - Perform comparative tests of test model behaviour using common test conditions, including HDR, high bit depth and high bit rate. - Suggest configuration files for additional testing of tools. - Investigate how to minimize the number of separate codebases maintained for group reference software. - Coordinate with AHG2 to identify any mismatches between software and text, and make further updates and cleanups to the software as appropriate. - Prepare drafts of merged and updated CTC documents for HM and VTM, as applicable. | F. Bossen, X. Li, K. Sühring (co-chairs), E. François, Y. He, K. Sharman, V. Seregin, A. Tourapis (vice-chairs) | N |
| Test material and visual assessment (AHG4) (jvet@lists.rwth-aachen.de) - Maintain the video sequence test material database for testing the VVC and HEVC standards and potential future extensions, as well as exploration activities. - Study coding performance and characteristics of available and proposed video test material. - Identify and recommend appropriate test material for testing the VVC standard and potential future extensions, as well as exploration activities. - Identify and characterize missing types of video material, solicit contributions, collect, and make available a variety of video sequence test material, in coordination with other AHGs, as appropriate. - Maintain and update the directory structure for the test sequence repository, as necessary. - Collect information about test sequences that have been made available by other organizations. - Prepare and conduct expert viewing for purposes of subjective quality evaluation. - Consider plans for additional verification testing of VVC capability, particularly target conducting tests for VVC multi-layer features, and update the test plan accordingly. - Coordinate with AG 5 in studying and developing further methods of subjective quality evaluation, e.g. based on crowd sourcing, as well as studying objective metrics in that context. - Coordinate with AHG17 on investigating sequences and making arrangements for viewing at the interim AHG17 meeting and at the next JVET meeting. - Coordinate with AHG18 on investigating visual impact of data losses. - Prepare availability of viewing equipment and facilities arrangements for future meetings. | V. Baroncini, T. Suzuki, M. Wien (co-chairs), W. Husak, S. Iwamura, P. de Lagrange, S. Liu, X. Meng, S. Puri, A. Segall, S. Wenger (vice-chairs) | Y (tel., 2 weeks notice) |
| Conformance testing (AHG5) (jvet@lists.rwth-aachen.de) - Study the draft conformance bitstreams for new HEVC multiview profiles in JVET-AM1008, and further develop related conformance bitstreams. - Coordinate with AHG3 on implementation of the new HEVC multiview profiles. - Study the requirements of VVC, HEVC, and AVC conformance testing to ensure interoperability. - Maintain and update the conformance bitstream database, and contribute to report problems, and suggest actions to resolve these. - Study additional testing methodologies to fulfil the needs for VVC conformance testing. | I. Moccagatta (chair), F. Bossen, T. Ikai, S. Iwamura, H.-J. Jhu, K. Kawamura, P. de Lagrange, S. Paluri, K. Sühring, Y. Yu (vice-chairs) | N |
| ECM software development (AHG6) (jvet@lists.rwth-aachen.de) - Coordinate development of the ECM software and associated configuration files. - Produce documentation of software usage for distribution with the software. - Prepare and deliver ECM-19.1 software version (and potential updates), corresponding VTM anchor, and the reference configuration encodings according to the ECM common test conditions. - Investigate encoder speedup and other software optimization such as reduction of memory consumption. - Coordinate with ECM algorithm description editors to identify any mismatches between software and text, make further updates and cleanups to the software as appropriate. | V. Seregin (chair), J. Chen, R. Chernyak, F. Le Léannec, K. Zhang (vice-chairs) | N |
| Tool assessment (AHG7) (jvet@lists.rwth-aachen.de) - Investigate methodology of tool assessment, such as the aspects of memory access and bandwidth, number of maximum processing cycles, block decoding dependencies, number of context coded bins, pipeline and parallelization. - Study JVET-AO2040 and suggest improvements. - Coordinate with AHG6 on resolving tool-off test related software issues (missing tool controls and software bugs). - Prepare configuration files and generate bitstreams and results of tool-on/tool-off testing as applicable. - Prepare reporting of tool assessment results as applicable. - Develop methodology of more reliable runtime measurement. - Collaborate with AHG16 on encoding complexity assessment. | X. Li (chair), L.-F. Chen, Z. Deng, J. Gan, E. François, R. Ishimoto, H.-J. Jhu, J. Lainema, X. Li, J. Pardo, A. Stein, H. Wang (vice-chairs) | Y (tel., 2 weeks notice) |
| Optimization of encoders and receiving systems for machine analysis of coded video content (AHG8) (jvet@lists.rwth-aachen.de) - Solicit and study non-normative encoder and receiving systems technologies that enhance performance of machine analysis tasks on coded video content. - Identify and collect test materials that are suitable to be used by JVET for machine analysis tasks. - Discuss improvements on the evaluation framework, including evaluation procedures and methodologies. - Coordinate software development and experiments on optimization of encoders and receiving systems for machine analysis of coded video content, including combinations of proposed technologies. - Maintain the software implementation examples and develop tool combination examples in the repository, including sufficient documentation in terms of operation and performance. - Evaluate proposed technologies and their suitability for machine analysis applications. - Propose potential improvements to JVET-AO2030 on optimization of encoders and receiving systems for machine analysis of coded video content. - Study the potential of using SEI messages for the purpose of machine analysis in coordination with AHG9. - Investigate the impact of using different machine task models in the evaluation of the compression performance of tools optimized for machine analysis tasks. | S. Liu, J. Ström, S. Wang, M. Zhou (AHG chairs) | N |
| SEI message studies (AHG9) (jvet@lists.rwth-aachen.de) - Study the SEI messages in VSEI, VVC, HEVC and AVC. - Study JVET-AO1017, identify any issues and propose solutions as appropriate. - Study JVET-AO2032, including study of SEI messages with different options, when those are present, and propose improvements. - Collect software and showcase information for SEI messages, including encoder and decoder implementations and bitstreams for demonstration and testing. - Identify potential needs for additional SEI messages. - Study SEI messages specified in HEVC and AVC for potential use in the VVC context and SEI messages in VSEI for potential use in the HEVC or AVC context. - Study the alignment of the same SEI messages in different standards. - Coordinate with AHG3 for software support of SEI messages for JM, HM, and VTM. - Coordinate with the joint AHG on Gaussian splat coding about aspects related to the usage of SEI messages. | J. Boyce, Y.-K. Wang (co-chairs), T. Chujoh, S. Deshpande, M. M. Hannuksela, Y. He, P. de Lagrange, G. J. Sullivan, H. Tan, A. Tourapis, S. Wenger, P. Wu (vice-chairs) | N |
| Encoding algorithm optimization (AHG10) (jvet@lists.rwth-aachen.de) - Study the impact of using techniques such as tool adaptation and configuration, and perceptually optimized adaptive quantization for encoder optimization. - Study the impact of non-normative techniques of preprocessing for the benefit of encoder optimization. - Study encoding techniques of optimization for objective quality metrics and their relationship to subjective quality. - Study optimized encoding for reference picture resampling and scalability modes in VTM, and coordinate with AHG4 on improving encoders and test settings for multi-layer verification testing. - Study optimized encoding and suitable test settings for noisy materials, such as sequences containing film grain. - Study optimized encoding and tool combinations for low latency and for low complexity. - Consider neural network-based encoding optimization technologies for video coding standards. - Investigate other methods of improving objective and/or subjective quality, including adaptive coding structures and multi-pass encoding. - Study methods of rate control and rate-distortion optimization and their impact on performance, subjective and objective quality. - Study the potential of defining default or alternate software configuration settings and test conditions optimized for either subjective quality, higher objective quality, or encoding with improved complexity/performance tradeoff, and coordinate such efforts with AHG3, AHG6, and AHG17. - Study the effect of varying configuration parameters depending on temporal layer, such as those related to deblocking, partitioning, chroma QP. | K. Andersson, P. de Lagrange, A. Duenas (co-chairs), T. Ikai, T. Solovyev, A. Tourapis (vice chairs) | N |
| Neural network-based video coding (AHG11) (jvet@lists.rwth-aachen.de) - Evaluate and quantify the performance improvement potential of NN-based video coding technologies compared to existing video coding standards such as VVC, including both individual coding tools, architectures and content adaptation with NN parameters overfitting. - Study potential improvements of the NNVC CTC document JVET-AJ2016, update a list of training materials. - Study the impact of training (including the impact of loss functions) on the performance of candidate technologies and identify suitable material for testing and training. Promote the call for training materials, distribute it, and actively communicate with content owners. - Analyse complexity characteristics for technologies under study, including transformers, perform complexity analysis, and develop complexity reductions of candidate technology. - Discuss and propose improved metrics to perform complexity analysis of NN architectures, in particular also investigate bit-exact reproducibility of NN-based methods on various platforms in coordination with AhG14. - Finalize and discuss the EE on neural network-based video coding. - Coordinate with other groups, including SC 29/AG 5 on the evaluation and assessment of visual quality, and AHG12 on the interaction with ECM coding tools. - Coordinate with AHG14 on items related to NNVC software development, in particular, evaluate the interface in the NNVC for end-to-end optimized AI coded reference pictures. | E. Alshina, F. Galpin, S. Liu (co-chairs), J. Li, Y. Li, R.-L. Liao, M. Santamaria, T. Shao, M. Wien, P. Wu (vice chairs) | Y (tel., 2 weeks notice), first on Feb. 20, second on March 20 |
| Enhanced compression beyond VVC capability (AHG12) (jvet@lists.rwth-aachen.de) - Solicit and study non-neural-network video coding tools with enhanced compression capabilities beyond VVC. - Discuss and propose refinements to the ECM19 algorithm description JVET-AN2025. - Coordinate with AHG7 to study the performance and complexity tradeoff of these video coding tools. - Coordinate with AHG6 on ECM software development. - Establish a repository to make other codebases that implement enhanced compression available to interested JVET members. - Support AHG6 in generating anchors according to the test conditions in JVET-AI2017. - Analyse the results of exploration experiments described in JVET-AO2024 in coordination with the EE coordinators. - Coordinate with AHG11 to study the interaction with neural network-based coding tools. | M. Karczewicz, Y. Ye, L. Zhang (co-chairs), B. Bross, R. Chernyak, X. Li, K. Naser, Y. Yu (vice-chairs) | N |
| Film grain technologies (AHG13) (jvet@lists.rwth-aachen.de) - Study the benefits and characteristics of film grain technologies, including autoregressive and frequency-filtering technologies. - Discuss and propose refinements to the draft of the TR 2nd ed. JVET-AN2020, and investigate for which elements described in the TR software might be attached. - Study alternative film grain models and their associated documentation. - Discuss and enumerate updates, improvements, and additions for the second edition of the technical report. - In consultation with AHG4, study and define content characteristics and test conditions that are desirable for the study and testing of film grain technologies, and perform an assessment of newly available test materials in that regard. - Investigate metrics for measuring film grain fidelity in itself, or as present in a video. - Discuss the potential need for film grain conformance guidelines. - Given the study of desirable content characteristics, solicit or create new test material for further determining the operational characteristics of, testing, and developing any related technologies. - Study preprocessing and encoder technologies for determining values for FGC (Film Grain Characteristics) SEI message syntax elements. - Study Film grain region characteristics information SEI in TuC and propose improvements as necessary - Identify potential need for additional film grain technology and signalling, if needed. - Coordinate development of film grain technology software and configuration files. - Coordinate with AG 5 on improving the plan for subjective quality testing of the FGC SEI message JVET-AJ2022, and conduct preparations for such testing. - Coordinate with AHG3 for software support of the FGC SEI message. | W. Husak, P. de Lagrange (co-chairs), S. Deshpande, A. Duenas, X. Meng, M. Radosavljević, A. Segall, G. Teniou, A. Tourapis (vice-chairs) | Y (tel., 2 weeks notice) |
| NNVC software development (AHG14) (jvet@lists.rwth-aachen.de) - Coordinate development of the NNVC software and associated configuration files. - Prepare and deliver NNVC-16.0 software version (and potential updates), based on updated VTM with adopted contributions and hybrid framework, and provide reference configuration encodings according to the NNVC common test conditions as described in JVET-AJ2016. Study the impact of the addition of new dataset on the already integrated models. - Continue to bridge the gap between NNVC and most recent VTM as necessary. - Continue to develop missing functionalities for hybrid end-to-end framework exploration. - Investigate combinations of tools included in the NNVC software, prepare and release anchor data for all configurations of the software, including anchors for High and Low Operation Point (HOP/LOP) and Very Low Operation Point (VLOP) configurations. - Study and maintain the SADL (Small Adhoc Deep-Learning Library). Identify gaps in functionality and develop improvements as needed. - Coordinate with NNVC algorithm and software description (JVET-AO2019) editors to identify any mismatches between software and description document, suggest further updates to the description document as appropriate. | F. Galpin (chair), R. Chang, A. Karabutov, Yue Li, Yun Li, M. Santamaria, J. N. Shingala, Z. Xie (vice chairs) | Y (tel., 2 weeks notice), first on Feb. 20, second on March 20 |
| Gaming content compression (AHG15) (jvet@lists.rwth-aachen.de) - Identify gaming content application scenarios and their requirements for codec operation. - Identify and characterize required types of content; solicit contributions, collect, and make a variety of gaming content available, in coordination with AHG4 and AG 5. - Produce VTM and ECM anchor encodings according to CTC JVET-AO2027, and provide test results at the next meeting. - Develop and maintain interfaces for supporting use cases of camera parameters and depth maps in gaming applications, including mechanisms for efficient transporting these elements in the coded video bitstream. - Develop and maintain software for estimation of depth maps for gaming sequences, and evaluate the effect of the estimated maps against the original depth maps on coding efficiency where they are available using established accuracy metrics - Evaluate JVET test models (such as ECM, VTM, NNVC, etc.) under the proposed test conditions. - Investigate possibilities to enhance compression capability for gaming content. - Investigate the possibility to estimate depth maps and camera parameters for those gaming sequences where they are not available. - Study conversion of depth maps using integer representation, and identifying efficient bit-depth resolution of depth maps to support identified use-cases that will be an input to compression. - Solicit contributions from industry on typical bitrate/quality/resolution used for gaming content compression. | S. Puri, J. Sauer (co-chairs), R. Chernyak, A. Duenas, L. Wang, V. Zakharchenko (vice chairs) | N |
| Hardware implementation complexity (AHG16) (jvet@lists.rwth-aachen.de) - Investigate hardware encoding complexity constraints in typical applications (e.g., mobile devices and hardware transcoding), and identify challenges and evaluate coding tools from hardware encoding implementation perspectives. - Solicit and develop hardware encoding complexity measurements (e.g., maximum number of RDO decisions per CU / CTU), and use them to analyze the performance of test models (e.g., VTM) and coding tools. - Consider the possibility of designing, developing and maintaining a hardware-encoding-mimicking simulation framework on top of test models, e.g., constraining RDO number per CU, constraining coding tree decision complexity (e.g., maximum tree depth, per-node split modes), developing rate estimation function interface, low-complexity CU/TU-level bit estimation function, etc. - Collaborate with AHG7 on aspects of hardware encoding and decoding complexity perspectives, and in improving JVET-AO2040. | Y. Zhao, J. Park, I. Moccagatta (co-chairs), H. Huang, T. Ikai, X. Li, K. Naser, N. Song, G. Verba (vice chairs) | Y (tel., 2 weeks notice) |
| Preparation of Call for Proposals (AHG17) (jvet@lists.rwth-aachen.de) - Review the set of categories, test sequences and rate points based on the results of the CfE, and propose updates for the test set to be used in the CfP. - Identify test sequences extending the represented range of applications and investigate the suitability for use in visual quality assessment. - Finalize the draft CfP text JVET-AO2026 based on the latest version contained in JVET-AO0290, and suggest updates towards the final draft CfP. - Coordinate the generation of VTM anchors and other bitstreams for the test cases defined in the updated test set, and make them available according to the timeline. - Prepare logistics and plans for conducting an AHG meeting during 25-27 February, and issue the announcement JVET-AO2041. - Coordinate with AG 5 on preparing logistics and organization of the visual assessment in the context of the CfP. | J.-R. Ohm, M. Wien, F. Bossen (co-chairs), E. Alshina, V. Baroncini, J. Chen, R. Chernyak, Z. Deng, P. de Lagrange, C. Lehmann, L. Li, P. Nikitin, D. Rusanovskyy, G. Verba (vice chairs) | Y (tel., 2 weeks notice), first on 3 Feb. at 1500UTC Hybrid meeting in Aachen, DE, 25-27 February |
| Ultra-low latency and packet loss resilience (AHG18) (jvet@lists.rwth-aachen.de) - Study JVET-AN2039 common test conditions and propose improvements as appropriate. - Investigate creation of practical simulation software, including network transmission aspects, including consideration of network functionality that can aid timely video-coding based error correction, and conduct performance evaluation. - Identify potential requirements and feasibility of standard based technologies to support ultra-low delay requirements, including packet loss resilient decoding. - Investigate packet loss resilient technologies beyond VVC supporting ultra-low delay coding for interactive and live broadcasting scenarios. - Address comments and questions received during CfP proposals discussion. - Coordinate with AHG4 and AHG17 on investigating visual impact of data losses and appropriate evaluation procedure development. | S. Deshpande, S. Ikonin, V. Zakharchenko (co-chairs), S. Fößel, C. Kim, X. Ma, S. Puri, J. Ström, S. Wenger (vice-chairs) | Y (tel., 2 weeks notice) |

It was suggested to discontinue AHG8 by the time of finalization of the TR (April or July 2026). For AHG8 and the previous AHG16 (generative face), it is necessary to find a persistent home for the software.

Related to this, it had been discussed during the 41st JVET meeting that it could be useful to generate a permanent repository (instead of AHG branches that may be forgotten) plus documentation for software parts that do extraction of SEI message parameters directly from video, and perform corresponding synthesis. Examples would be NNPF, FGC, GFV, … This was left for further study.

It was confirmed that the rules which can be found in document ISO/IEC JTC 1/‌SC 29/‌AG 2 N 046 “Ad hoc group rules for MPEG AGs and WGs” (available at https://www.mpegstandards.org/adhoc/), are consistent with the operation mode of JVET AHGs. It is pointed out that JVET does not maintain separate AHG reflectors, such that any JVET member is implicitly a member of any AHG. This shall be mentioned in the related WG Recommendations. The list above was also issued as a separate WG 5 document (ISO/IEC JTC 1/‌SC 29/‌WG 5 N XXX) in order to make it easy to reference.

A Joint AHG group was installed with ISO/IEC JTC 1/‌SC 29/‌WG 4 and ISO/IEC JTC 1/‌SC 29/‌WG 7 as follows:

| Title and Email Reflector | Chairs | Interim mtg. |
| --- | --- | --- |
| Gaussian Splat Coding (mpeg-gsc@lists.aau.at) - Prepare use cases and requirements for Gaussian splat coding. - Collect and prepare data set for Gaussian splat coding. - Define common test conditions for Gaussian splat coding. - Prepare anchors for Gaussian splat coding. - Investigate new Gaussian splat representation and coding technologies. - Consolidate the software tools for Gaussian splat coding. - Coordinate the activity on Lightweight GS (1F). - Prepare subjective evaluation. - Coordinate development of GS technology and its standardization across JVET, WG4, and WG7. - Study encapsulation/high-level syntax technologies including those based on V-PCC, V3C, and VSEI. - Study GS technology that allows cross-encapsulation compatibility and definition of related conformance. | Y. Liao, G. Bang (co-chairs on behalf of WG 4), J. Jung, W. Husak (co-chairs on behalf of JVET), A. Zaghetto, J. Ricard (co-chairs on behalf of WG 7) | Y (tel., UTC) Feb 10 1400–1600 Feb 26 1400–1700 Mar 19 1400–1600 Apr 09 1500–1800 (optional) Apr 14 1200–1500 (optional) Apr 16 2200–0100 Apr 21 1400–1700 |

It is planned that formally only one instance of the joint AHG is installed over all MPEG WGs, and the responsibility for doing this is rotating among the involved WGs. At the current meeting, WG 7 was responsible, at the next meeting, WG 4 will take over (including management of documents submitted to the joint activity).

JVET members are invited to subscribe to the reflector of this AHG via https://lists.aau.at/mailman/listinfo/mpeg-gsc.

GS content is stored at https://content.mpeg.expert/data/Explorations/GSC/CTC/

The CTC is updated during each meeting period, and the version valid during the upcoming meeting cycle can be found as:

| MDS26077 | Common test conditions for Gaussian splat coding | B. Kroon (Philips), P. Rondao Alface (Nokia), J. Jung (Qualcomm), G. Sandri (InterDigital) |
| --- | --- | --- |

For accessing software, test materials and anchors, JVET members should contact the AHG chairs. Documents relevant for usage of SEI messages in Gaussian splat representation should be submitted both as WG 4 documents and JVET documents, such that JVET members who are participating via ITU also can get access.

It was anticipated to agree on a joint exploration experiment on topics related to the last three AHG mandates during the first interim AHG meeting on Feb 10. The result can be found in MDS26086, also described in section 8.2 of this report.

# Output documents (update)

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate the editors responsible for the document production. Where applicable, dates of planned finalization and corresponding parent-body document numbers are also noted.

It was reminded that in cases where the JVET document is also made available as a WG 5 output document, a separate version under the WG 5 document header should be generated. This version should be sent to GJS and JRO for upload.

The list of JVET ad hoc groups was also issued as a WG 5 output document WG 5 N XXX, as noted in section 9.

A list of JVET-only output documents from the current meeting (including links to jvet-experts.org) was issued as a WG 5 document WG 5 N XXX.

JVET-AO1000 Meeting Report of the 41st JVET Meeting [J.-R. Ohm] [WG 5 N 383] (2026-02-20)

Initial versions of the meeting notes (d0 … d9) were made available on a daily basis during the meeting.

Remains valid – not updated: JVET-AC1001 Guidelines for HM-based software development [K. Sühring, F. Bossen, X. Li (software coordinators)]

Remains valid – not updated: JVET-Y1002 High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Encoder Description Update 16 [C. Rosewarne, K. Sharman, R. Sjöberg, G. J. Sullivan] [WG 5 N 103]

Primary editor: C. Rosewarne

Remains valid – not updated: JVET-AH1003 Coding-independent code points for video signal type identification (Draft 3) [G. J. Sullivan, A. Tourapis]

Primary editor: G. J. Sullivan.

JVET-AO1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP [Y.-K. Wang, J. Boyce, B. Bross, I. Moccagatta, C. Rosewarne, K. Sühring, G. J. Sullivan] (2026-02-27)

Primary editor: Y.-K. Wang.

Some issues found in latest editions of VSEI/VVC/HEVC (see list under 7.1.3).

Remains valid – not updated: JVET-AN1005 Future CICP extensions (Draft 2) [J. Boyce, E. Thomas, A. Tourapis] (2025-10-26)

Primary editor: J. Boyce.

Including update from …

Remains valid – not updated JVET-AM1006 HEVC additional profiles and SEI messages (Draft 4) [Y.-K. Wang, B. Bross, S. Deshpande, G. J. Sullivan, A. Tourapis] (2025-08-01)

Primary editor: Y.-K. Wang.

Remains valid – not updated: JCTVC-P1006 Common test conditions and software reference configurations for HEVC range extensions [D. Flynn, C. Rosewarne, K. Sharman (editors)] [WG 11 M 32658]

May require merging with JVET-T2013. JCTVC-P1006 only defines 10-bit sequences for the cases on 4:4:4 and 4:2:2 chroma sampling, and 16-bit sequences for higher bit depth, no 12 bit cases. All sequences specified in this document should be already available in the JVET content server. It is also noted that JVET-AA1100 could be removed once such a merge is done.

Remains valid – not updated: JCTVC-V1007 SHVC Test Model 11 (SHM 11) Introduction and Encoder Description [G. Barroux, J. Boyce, J. Chen, M. M. Hannuksela, Y. Ye] [WG 11 N 15778]

Remains valid – not updated: JVET-AM1008 Conformance testing for HEVC multiview extended and monochrome profiles [I. Moccagatta, T. Fu, S. Paluri, A. Tourapis]

Remains valid – not updated: JVET-AC1009 Common test conditions for SHVC [K. Sühring]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JCTVC-O1010 Guidelines for Conformance Testing Bitstream Preparation [T. Suzuki, W. Wan]

Remains valid – not updated: JVET-AJ1011 White paper on HEVC [B. Bross, J.-R. Ohm, G. J. Sullivan, Y.-K. Wang] [AG 3 N 174]

Remains valid – not updated: JVET-AJ1012 Overview of IT systems used in JVET [J.-R. Ohm, I. Moccagatta, K. Sühring, M. Wien]

Update of bug tracking system description expected in the next meeting, as well as update on zoom meeting rules (no recording etc.).

Remains valid – not updated: JCT3V-G1003 3D-AVC Test Model 9 [D. Rusanovskyy, F. C. Chen, L. Zhang, T. Suzuki] [WG 11 N 14239]

Remains valid – not updated: JCT3V-K1003 Test Model 11 of 3D-HEVC and MV-HEVC [Y. Chen, G. Tech, K. Wegner, S. Yea] [WG 11 N 15141]

Remains valid – not updated: JVET-AE1013 Common test conditions of 3DV experiments [K. Sühring, M. Wien]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated JCTVC-V1014 Screen Content Coding Test Model 7 Encoder Description (SCM 7) [R. Joshi, J. Xu, R. Cohen, S. Liu, Y. Ye] [WG 11 N 16049]

Remains valid – not updated: JVET-AC1015 Common test conditions for SCM-based screen content coding [K. Sühring]

Links to test sequences need to be updated due to the change of the content server.

JVET-AO1016 AVC with extensions and corrections (Draft 5) [K. Sühring, J. Boyce, G. J. Sullivan, Y.-K. Wang] (2026-03-13)

Primary editor: K. Sühring.

Integrated text to prepare the next version of H.264, including changes from JVET-AO1017.

JVET-AO1017 Support for additional SEI messages in AVC (Draft 5) [K. Sühring, J. Boyce, G. J. Sullivan, Y.-K. Wang] (2026-02-27)

Primary editor: K. Sühring

A preliminary DoC WG 5 N 384 on the DAM was reviewed and approved on Thursday 22 Jan. at 2200-2210. Progression to FDAM/FDIS integrated version was planned to be done in July 2026 at latest.

Changes are editorial improvements, interface changes, constraints, and bug fixes agreed at this meeting, including elements from:

- JVET-AO0212 Extension mechanism for new AVC SEI messages

It is noted that the list above may not be complete, and some aspects relating to VSEI interface may first need a correction in VSEIv4 (see JVET-AO1004); if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

Remains valid – not updated: JVET-AN1018 HEVC with extensions and corrections (Draft 3) [Y.-K. Wang, B. Bross, S. Deshpande, G. J. Sullivan, A. Tourapis] (2025-10-17)

Primary editor: Y.-K. Wang.

This was submitted to ITU consent on H.265 11th ed. It is noted that this may not be identical with the final version published by ITU-T.

Remains valid – not updated: JVET-AN1019 VSEI with extensions and corrections (Draft 1) [J. Boyce, J. Chen, S. Deshpande, M. M. Hannuksela, S. McCarthy, G. J. Sullivan, H. Tan, Y.-K. Wang] (2025-10-17)

Primary editor: J. Boyce.

This was submitted to ITU consent on H.274 4th ed. It is noted that this may not be identical with the final version published by ITU-T.

No output: JVET-Axx1020 through JVET-Axx1099

Remains valid – not updated JVET-AA1100 Common Test Conditions for HM Video Coding Experiments [K. Sühring, K. Sharman]

This specifies only the CTC for non-4:2:0 colour formats. The corresponding document for VVC is JVET-T2013, with no unification yet. See note under JCTVC-P1006 above.

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-AN2001 VVC with extensions and corrections (Draft 1) [Y.-K. Wang, B. Bross, M. M. Hannuksela, G. J. Sullivan] (2025-10-17)

Primary editor: Y.-K. Wang.

This was submitted to ITU consent on H.266 4th ed. It is noted that this may not be identical with the final version published by ITU-T

JVET-AO2002 Algorithm description for Versatile Video Coding and Test Model 23 (VTM 23) [Y. Ye, A. Browne, S. Kim] (2026-03-13)

Primary editor: Y. Ye.

New elements from notes elsewhere in this report:

- RPR clarification (JVET-AO0136)

- MCTF (JVET-AO0136 and JVET-AO0288)

- BIM (JVET-AO0288)

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

It was suggested that editorial improvements would also be welcome.

Remains valid – not updated: JVET-AJ2003 Guidelines for VTM-based software development [F. Bossen, X. Li, K. Sühring]

Remains valid – not updated: JVET-T2004 Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 12) [Y. Ye, J. Boyce]

Remains valid – not updated: JVET-AM2005 Additions and corrections for VVC version 4 (Draft 13) [G. J. Sullivan, B. Bross, M. M. Hannuksela, Y.-K. Wang] (2025-08-01)

Primary editor: G. J. Sullivan.

Remains valid – not updated: JVET-AM2006 Additional SEI messages for VSEI version 4 (Draft 7) [J. Boyce, J. Chen, S. Deshpande, M. M. Hannuksela, S. McCarthy, G. J. Sullivan, H. Tan, Y.-K. Wang] (2025-08-01)

Primary editor: J. Boyce.

Remains valid – not updated: JVET-AJ2007 Guidelines for NNVC software development [F. Galpin, S. Eadie, L. Wang, Z. Xie, Y. Li]

Might benefit from an update in the future, regarding some details of SADL implementation.

Remains valid – not updated: JVET-AN2008 Conformance testing for versatile video coding (Draft 8) [I. Moccagatta, F. Bossen, S. Iwamura, P. de Lagrange]

Primary editor: I. Moccagatta

This was submitted to ITU consent on H.266.1 3rd ed., and has been (pre-)published. An equivalent version was under ballot as ISO/IEC DIS 23090-15 3rd ed. A DoC WG 5 N 385 on the DIS was reviewed and approved on Thursday 22 Jan. at 2010-2020. The DIS text will be sent for publication as final standard.

Remains valid – not updated: JVET-AJ2009 Reference software for versatile video coding 2nd edition (Draft 2) [F. Bossen, K. Sühring, X. Li] [WG 5 DIS N 322)]

Software relating to H.266.2 and ISO/IEC 23090-16 can be found at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tree/2nd-edition.

Primary editor: F. Bossen.

[kept for future meetings] A request for next edition(s) of reference software(s) could be issued in the next meeting, and also CD(s) could be produced by the same meeting, as SC 29 is meeting beginning of February and could approve such projects without issuing a ballot. Issuing CD(s) by the January meeting (if mature enough) could avoid that an ISO version would lag behind an ITU version which might be submitted for consent in July 2026. Another consideration could be to start the process for ISO early enough in 2026, such that FDIS and ITU consent could be synchronized in April 2027.

Remains valid – not updated: JVET-AL2010 VTM and HM common test conditions and software reference configurations for SDR 4:2:0 10 bit video [F. Bossen, X. Li, V. Seregin, K. Sharman, K. Sühring]

JVET-AO2011 VTM and HM common test conditions and evaluation procedures for HDR/WCG video [E. François, W. Husak, S. Iwamura, D. Rusanovskyy] (2026-01-31)

Links to test sequences need to be updated due to the change of the content server.

New version including removal of metrics, and clarification for metrics to be used in HLG and PQ, as suggested in JVET-AO0184

Remains valid – not updated: JVET-U2012 JVET common test conditions and evaluation procedures for 360° video [Y. He, J. Boyce, K. Choi, J.-L. Lin]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-T2013 VTM common test conditions and software reference configurations for non-4:2:0 colour formats [Y.-H. Chao, Y.-C. Sun, J. Xu, X. Xu]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-Q2014 JVET common test conditions and software reference configurations for lossless, near lossless, and mixed lossy/lossless coding [T.-C. Ma, A. Nalci, T. Nguyen]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-Q2015 JVET functionality confirmation test conditions for reference picture resampling [J. Luo, V. Seregin]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-AJ2016 Common test conditions and evaluation procedures for neural network-based video coding technology [E. Alshina, F. Galpin, R.-L. Liao, S. Liu, A. Segall]

Remains valid – not updated: JVET-AI2017 Common test conditions and evaluation procedures for enhanced compression tool testing [M. Karczewicz, Y. Ye]

It was commented that it might be good to add conditions relating to HDR, even though currently no HDR tests are conducted in CTC.

Remains valid – not updated: JVET-AA2018 Common test conditions for high bit depth and high bit rate video coding [A. Browne, T. Ikai, D. Rusanovskyy, X. Xiu, Y. Yu]

Links to test sequences need to be updated due to the change of the content server.

JVET-AO2019 Description of algorithms version 14 and software version 16 in neural network-based video coding (NNVC) [F. Galpin, Yue Li, Yun Li, D. Rusanovskyy, T. Shao, J. Ström, L. Wang] (2026-03-27)

New elements in text and software from notes elsewhere in this report:

- JVET-AO0066 test EE1-1.1.2 (both training modification and architecture change).

- JVET-AO0108: Replace current DRF by test EE1-4.1.1, and add EE1-4.1.3 as a “low operation point” of DRF.

- JVET-AO0128 Test EE1-5.1

- include the method proposed in JVET-AO0127 as an optional element of the NNVC software, which may be used in addition to the current training scripts in cases where the training crosscheck is not successful. It may also be helpful as tool to identify implementation problems in the context of training verification.

- JVET-AO0056 global average pool layer for SADL

- JVET-AO0100 SIMD improvements for SADL

- JVET-AO0058 SADL bug fix of conv2d grid misalignment

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

Remains valid – not updated: JVET-AN2020 Film grain synthesis technology for video applications ed. 2 (Draft 4) [W. Husak, P. de Lagrange, A. Norkin, M. Radosavljević, A. Tourapis] (2026-01-09)

JVET-AO2021 Verification test plan for VVC multilayer coding (update 8) [O. Chubach, P. de Lagrange, M. Wien] (2026-02-06)

New version based on JVET-AO0215

No output: JVET-Ax2022

JVET-AO2023 Exploration experiment on neural network-based video coding (EE1) [E. Alshina, R. Chang, F. Galpin, Yue Li, Yun Li, M. Santamaria, T. Shao, J. Ström, Z. Xie (EE coordinators)] (2026-02-06)

An initial draft of this document was reviewed and approved at 1310-1335 on Friday 23 January.

This round of EE1 tests includes:

- EE1-1: LOP in-loop filter

  - EE1-1.1 – Dynamic convolution for LOP7 neural in-loop filtering JVET-AO0055 (KHU, KBS).

  - EE1-1.2 – Combination test of EE1-1.1 and Backbone Block Enhancement of LOP In-Loop Filter with Over-Parameterized Training and Variable Channels JVET-AO0074 (Xidian Univ., UESTC, Transsion)

  - EE1-1.3 – AHG11: Enhancing LOP7 with Re-Exploited Boundary Strength Guidance JVET-AO0144 (HUST)

- EE1-2: VLOP in-loop filter

  - EE1-2.1 – VLOP with new backbone block based on Spatial-Channel Mixing JVET-AO0129

- EE1-3 NN-Inter

  - EE1-3.1 – Very Small Deep Reference Frame Generation Network for Inter Prediction Enhancement JVET-AO0267

- EE1-4: Framework for externally coded pictures

  - EE1-4.1 - Multi-layer framework (as it is in NNVC-16, with residual coding)

  - EE1-4.2 - Multi-layer framework with frame level control (full replacement of VVC I-frame with E2E AI coded picture depending on picture level flag)

  - EE1-4.3 - Single-layer framework as proposed in JVET-AO0173 with different E2E AI codec

  - EE1-4.4 - Single-layer framework as proposed in JVET-AO0173 with frame level control (full replacement of VVC I-frame with E2E AI coded picture depending on picture level flag)

- EE1-5: operational bit-exact reproducibility

  - EE1-5.1 – [AHG11] Stable float convolution for neural network inference JVET-AO0178 (Nokia)

JVET-AO2024 Exploration experiment on enhanced compression beyond VVC capability (EE2) [V. Seregin, D. Buğdayci Sansli, J. Chen, R. Chernyak, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang (EE coordinators)] (2026-01-31)

An initial draft of this document was reviewed and approved at 1335-1355 on Friday 23 January. This round of EE2 tests includes:

|  | Tests | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1 Intra prediction |  |  |  |
| 1.1a | Add more chroma DIMD modes into the MPM list | Z. Li (ZTE) |  |
| 1.1b | Modify the reordering strategy | Z. Li (ZTE) |  |
| 1.1c | Test 1.1a + Test 1.1b | Z. Li (ZTE) |  |
| 1.2a | EIP with modified H-filter and V-filter | Y. Liu (Transsion) | H. Qin (TCL) |
| 1.2b | Test 1.2a + modified S-filter | Y. Liu (Transsion) | H. Qin (TCL) |
| 1.3a | CCCM clipping operations with clipping range adjustment | P. Onno (Canon) L.-C. Xu (SYSU) | Y. Ahn (Qualcomm) |
| 2 Inter prediction |  |  |  |
| 2.1a | Modified GPM partition mode with refined offset direction derivation | Y. Kidani (KDDI) H. Zhang (OPPO) |  |
| 2.1b | Modified GPM partition mode with enabling block-shaped adaptive angle selection in LDB | Y. Kidani (KDDI) |  |
| 2.1c | Test 2.1a + Test 2.1b | Y. Kidani (KDDI) H. Zhang (OPPO) |  |
| 3 Transform and coefficients coding |  |  |  |
| 3.1a | Quantization borders optimization for URQ (non-CTC) | M. Le Pendu (InterDigital) |  |
| 3.1b | Unbiasing URQ (non-CTC) | M. Le Pendu (InterDigital) |  |
| 3.1c | Unbiasing RDOQ (non-CTC) | M. Le Pendu (InterDigital) |  |
| 3.2a | QP adaptive dead-zone size for DQ | M. Balcilar (Ofinno) |  |
| 3.2b | Dead-zone size optimization for DQ | M. Balcilar (Ofinno) |  |
| 3.2c | QP adaptive dead-zone size for RDOQ (non-CTC) | M. Balcilar (Ofinno) |  |
| 3.2d | Dead-zone size optimization for RDOQ (non-CTC) | M. Balcilar (Ofinno) |  |
| 3.3a | Intra LFNST index prediction | Y. Wang (Tencent) | C. Hollmann (TCL) |
| 3.3b | Test 3.3a with sign prediction disabled (non-CTC) | Y. Wang (Tencent) | C. Hollmann (TCL) |
| 4 In-loop filtering |  |  |  |
| 4.1 | Simplification for TALF | L. Xu (OPPO) |  |
| 5 Entropy coding |  |  |  |
| 5.1a | CABAC contexts retraining | Z. Xiang (Tencent) |  |
| 5.1b | Counter-based temporal probability initialization | Z. Xiang (Tencent) |  |
| 5.1c | Test 5.1b + CABAC contexts retraining | Z. Xiang (Tencent) |  |
| 5.1d | Using quotient C1/CN as initialization probability | Z. Xiang (Tencent) |  |

As test 4.2 which was planned in v1 of JVET-AO2024 (on JVET-AO0283) was targeted to fix a bug that is non-CTC and cannot be reproduced by a simple change of configuration file (requires encoder change to enforce it), it was considered being irrelevant at this stage exploration. Therefore, investigation was removed from EE. Further study to clearly state under which circumstances the problem occurs. Could also potentially be resolved by an encoder change.

Remains valid – not updated: JVET-AN2025 Algorithm description of Enhanced Compression Model 19 (ECM 19) [M. Coban, R.-L. Liao, K. Naser, J. Ström, L. Zhang] (2026-01-09)

New elements from notes elsewhere in this report (no new elements at the current meeting):

- …

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

JVET-AO2026 Draft Joint Call for Proposals on video compression with capability beyond VVC [J.-R. Ohm, M. Wien, F. Bossen] [WG 5 N 388] (2026-02-06)

JVET-AO2027 Common test conditions for gaming applications [J. Sauer, R. Chernyak, S. Puri, S. Thiebaud] (2026-01-31)

Add Wukong2 and Wukong3 to class G2 (see discussion under JVET-AO0207). Furthermore, Level1 and Darktree (HDR versions) were removed from class G3 (still retained as SDR versions), as the difference in terms of performance was minor, and rendering of these is somewhat outdated.

Information about availability of depth maps is also updated.

It was further agreed to remove deltaE100 and PSNRE100 (same as for JVET-AO2011).

No output: JVET-Ax2028

Remains valid – not updated: JVET-AN2029 Results of the Joint Call for Evidence on video compression with capability beyond VVC [F. Bossen, J.-R. Ohm, M. Wien] [AG 5 N 180] (2025-12-19)

JVET-AO2030 Optimization of encoders and receiving systems for machine analysis of coded video content (Draft 10) [S. Liu, J. Chen, C. Hollmann, J. Ström] [WG 5 2nd DTR N 387] (2026-02-06)

Primary editor: S. Liu.

A DoC WG 5 N 386 on the previous DTR was reviewed and approved on Thursday 22 Jan. at 2120-2200.

Text of JVET-AO2030 was developed from JVET-AO0292 (was reviewed and approved in this context).

Remains valid – not updated: JVET-AI2031 Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content [S. Liu, C. Hollmann]

JVET-AO2032 Technologies under consideration for future extensions of VSEI (version 11) [S. Deshpande, J. Boyce, J. Chen, M. M. Hannuksela, K. Sühring, H. Tan, J. Xu] (2026-03-13)

Primary editor: S. Deshpande

New elements from notes elsewhere in this report (list reviewed approved in JVET plenary on Friday 23 Jan.):

  - Editorial:

    - JVET-AO0050 AHG9: Miscellaneous editorial changes for VSEI TuC

    - JVET-AO0085 AHG9: Editorial improvements for VSEI v4 and TuC

    - JVET-AO0185 AHG9: Editorial changes on VSEI TuC

  - Versatile SEI

    - JVET-AO0150 AHG9: On versatile SEI RBSP and versatile SEI message

    - JVET-AO0101 AHG9: Text description information SEI message in processing chain

  - Multiple SEI messages

    - JVET-AO0096 AHG9: On interface text for SEI messages in TuC

    - JVET-AO0187 AHG9: Miscellaneous aspects of SEI messages in VSEI TuC

  - Syntax descriptors

    - JVET-AO0169 AHG9: On flf

  - Modality Information

    - JVET-AO0160 AHG9: on MI SEI extension for multispectral picture

  - Photosensitive content information (PCI) SEI

    - JVET-AO0080 AHG9: On photosensitive content information SEI

  - Encoder optimization info (EOI)

    - JVET-AO0097 AHG9: Temporal extrapolation support for the EOI SEI message

    - JVET-AO0172 [AHG9] EOI SEI message: Essential corrections to TuC

    - JVET-AO0281 AHG9: On depth-based optimization in the EOI SEI message

  - Scalability dimension info (SDI) SEI

    - JVET-AO0193 AHG9: On the SDI SEI message

    - JVET-AO0198 AHG9: SDI SEI clean-up related to interaction between JVET-AN0265-v2 and JVET-AM0090

  - NNPF

    - JVET-AO0063 AHG9: On Neural-network Post-filter Signalling

    - JVET-AO0118 AHG9: On a demosaicking purpose for the NNPFC SEI

  - Constituent rectangles (CR)

    - JVET-AO0156 AHG9: On the constituent rectangles SEI message

    - JVET-AO0189 AHG9: On the layer ID of CR and ECFI SEI messages

  - Display rectangles (DR)

    - JVET-AO0124 AHG9: On display rectangles SEI message

    - JVET-AO0165 AHG9: On display rectangles gradient filling

  - Digitally signed content (DSC)

    - JVET-AO0106 AHG9: On bitstreams for subpicture-based signing in digitally signed content SEI messages

    - JVET-AO0107 AHG9: Miscellaneous aspects of digitally signed content SEI messages

    - JVET-AO0214 AHG9: editorial updates for VSEI TuC Digitally Signed Content SEI

  - Film grain regions characteristics (FGRC)

    - JVET-AO0064 AHG9/AHG13: On Film Grain Regions Characteristics SEI Message

    - JVET-AO0099 AHG9: On the use of alpha planes with the FGRC SEI message

    - JVET-AO0102 AHG9: On handling film grain characteristics SEI message and film grain regions characteristics SEI message

    - JVET-AO0103 AHG9: On alpha channel adaptation in film grain regions characteristics SEI message

    - JVET-AO0104 AHG9: On region-based adaptation in film grain regions characteristics SEI message

    - JVET-AO0105 AHG9: Miscellaneous aspects of film grain regions characteristics SEI message

  - Display overlays info (DOI)

    - JVET-AO0079 AHG9: On DOI, BRI, and ECFI SEI messages for multi layer operation

    - JVET-AO0125 AHG9: On display overlays information SEI message

  - Quality metric QM

    - JVET-AO0115 AHG9: On minimum value for the quality metrics SEI message

    - JVET-AO0192 AHG9: On the QM SEI message

  - Colour mapping info (CMI)

    - JVET-AO0166 AHG9: Showcase on Colour Mapping Information SEI

    - JVET-AO0168 AHG9: On CMI

    - JVET-AO0202 AHG9: On the colour map information SEI message

    - JVET-AO0211 On colour mapping SEI message

  - Localization and mapping (LAM) SEI:

    - JVET-AO0216 AHG9: Miscellaneous items for the LAM, PCI and RN SEI messages

  - Danmu information

    - JVET-AO0077 AHG9: On danmu information SEI message

    - JVET-AO0155 AHG9: On the danmu information SEI message

    - JVET-AO0164 AHG9: on the Danmu SEI message

  - Graphics rendering info (GRI)

    - JVET-AO0162 AHG9: on the SDI SEI message dependency for GRI

    - JVET-AO0203 AHG9: On the graphics rendering information SEI message

  - Auxiliary sampling alignment information SEI message syntax (ASAI)

    - JVET-AO0161 AHG9: on the SDI SEI message dependency for ASAI

  - Sample interleaving

    - JVET-AO0149 AHG9: Text fixes for sample interleaving SEI message

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

Remains valid – not updated: JVET-AF2033 Report of verification test on VVC multi-layer coding: Content layering [S. Iwamura, P. de Lagrange, M. Wien] [AG 5 N 105)]

Remains valid – not updated: JVET-AI2034 Call for new HDR materials for future video coding development [E. François, W. Husak, S. Iwamura, D. Rusanovskyy, A. Segall, M. Wien] [WG 5 N 312)]

Remains valid – not updated: JVET-AJ2035 Test conditions and evaluation procedures for generative face video coding [S. McCarthy, B. Chen]

Remains valid – not updated: JVET-AG2036 Call for training materials for neural network-based video coding tool development [E. Alshina, F. Galpin, S. Liu, M. Wien] [WG 5 N 266)]

Remains valid – not updated: JVET-AJ2037 Report on subjective quality testing of the FGC SEI message (AG 5 N 140) [P. de Lagrange, W. Husak, M. Wien] [AG 5 N 140)]

Remains valid – not updated: JVET-AM2038 White paper on VSEI [J. Boyce, S. McCarthy, S. Deshpande, G. J. Sullivan, Y. Sanchez, Y.-K. Wang] [AG 3 N209]

An update of the white paper on VVC (from 2021) should be done in a future meeting, removing the section on VSEI and referencing to JVET-AM2038.

Remains valid – not updated: JVET-AN2039 Common test conditions for technology targeting ultra low latency and packet loss resilience [S. Ikonin, S. Deshpande, V. Zakharchenko] (2025-10-31)

JVET-AO2040 Complexity reporting template for coding algorithms and tools [X. Li, E. Alshina, I. Moccagatta] (2026-02-13)

The changes suggested by the BoG JVET-AO0287 were reviewed and approved by JVET on Friday 23 Jan. 1615UTC. It was agreed to remove the ECM analysis from the examples (only retain VTM), as ECM may be an untypical example about complexity expected by proposals.

JVET-AO2041 Announcement of JVET AHG17 Meeting in Aachen, DE, 25-27 February 2026 [J.-R. Ohm, M. Wien] (2026-02-06)

Meeting will be hybrid.

Experts with need to request for Schengen visa might want to contact J.-R. Ohm and M. Wien as soon as possible.

# Future meeting plans, expressions of thanks, a.o.b., and closing of the meeting (update)

The draft of the WG 5 recommendations (see Annex C) was reviewed and approved in JVET at XXXX–XXXX on Friday 1 May.

Future meeting plans were established with the following general guidelines (assuming face-to-face meetings):

- Meeting under ITU-T SG21 auspices when it meets (ordinarily starting meetings on the Monday, Tuesday or Wednesday of the first week and closing it on the Wednesday of the second week of the SG21 meeting – a total of 8-10 meeting days), and

- Otherwise meeting under ISO/IEC JTC 1/‌SC 29 auspices when its MPEG WGs meet (ordinarily starting meetings on the Wednesday, Thursday or Friday prior to the main week of such meetings and closing it on the same day as other MPEG WGs – a total of 8–10 meeting days).

In cases where an exceptionally high workload is expected for a meeting, an earlier starting date may be defined, or AHG meetings might be scheduled prior to the meeting. In cases of online meetings, no sessions should be held on weekend days, such that meetings would typically start two days earlier.

Some specific future meeting plans were established as follows:

- During 7 – 15 July 2026, 43rd meeting under ITU-T SG21 auspices in Geneva, CH, to be conducted as physical meeting with remote participation,

- During 17 – 23 October 2026, 44th meeting under ISO/IEC JTC 1/‌SC 29 auspices in Hangzhou, CN, to be conducted as hybrid meeting,

- During 13 – 22 January 2027, 45th meeting under ISO/IEC JTC 1/‌SC 29 auspices in Brisbane, AU, to be conducted as hybrid meeting,

- During 17 – 28 April 2027, 46th meeting under ITU-T SG21 auspices in Geneva, CH, to be conducted as physical meeting with remote participation,

- During 7 – 16 July 2027, 47th meeting under ISO/IEC JTC 1/‌SC 29 auspices in Tampere, FI, to be conducted as hybrid meeting,

- During 20 – 29 October 2027, 48th meeting under ISO/IEC JTC 1/‌SC 29 auspices in Shenzhen, CN, to be conducted as hybrid meeting,

- During January 2028, 49th meeting under ISO/IEC JTC 1/‌SC 29, date and location t.b.d,,

- During 25 March – 5 April 2028, 50th meeting under ITU-T SG21 auspices in Geneva, CH, to be conducted as physical meeting with remote participation,

- During July 2028, 51st meeting under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.,

- During October 2028, 52nd meeting under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.

The agreed document deadline for the 43rd JVET meeting was planned to be Tuesday 30 June 2026.

XXX were thanked for offering new video test sequences.

Marius Preda was thanked for the service of managing and maintaining the document site jvet-experts.org. Institut Mines-Télécom was thanked for hosting the sites.

Thank meeting host and organizer …

The 42nd JVET meeting was closed at approximately XXXX hours CEST on Friday 1 May 2026.

# Annex A to JVET report:
List of documents

Dates and times in the table below are in Paris/Geneva time (2 hr. ahead of UTC by the current meeting). It is noted that, if title or authorship of a document deviates from the title or author list in the body of the report, the list of documents in this annex contains the correct title and authors.

# Annex B1 to JVET report:
List of meeting participants attending in person

The participants who were personally present at the meeting site of the thirty-ninth meeting of the JVET, according to a sign-in sheet circulated in the JVET meeting rooms (approximately XXX people in total), were as follows:

# Annex B2 to JVET report:
List of meeting participants attending remotely

The remote participants of the forty-second meeting of the JVET, according to the participation records from the Zoom teleconferencing tool used for the meeting sessions (approximately XXX people in total, not including those who had attended the meeting in person at least part-time (see annex B1), and not including those who attended only the joint sessions with other groups), were as follows:

# Annex C to JVET report:
Recommendations of the 23rd meeting of
ISO/IEC JTC 1/SC 29/WG 5 MPEG Joint Video Experts Team with ITU-T SG21

ISO/IEC JTC 1/SC 29/WG 5 N XXX