| ITU – Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) 54th Meeting: 15–26 October 2016, Chengdu, CN | Document: VCEG-BB06 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Gary J. Sullivan (Microsoft/USA) 1 Microsoft Way Redmond, WA 98052 USA | Tel: Email: | +1 425 703 5308 garysull@microsoft.com |
| Title: | Potential Last Call comments by Microsoft for ITU-T H.273 (V1) |  |  |
| Purpose: | Last Call comment |  |  |

_____________________________

Comment: We support this text on the condition that it be modified as suggested herein.

Attachment: None. Comments are given in the Observation field, no attachment needed.

Observation: We respectfully submit the following comments on this text:

- At the time of Consent for ITU-T H.273 in June 2016, it was known that ITU-R was working on a Draft New Recommendation ITU-R BT.[HDR-TV], but that specification had not yet been approved. The approval of ITU-R BT.[HDR-TV] has since occurred, and it is now Rec. ITU-R BT.2100. Because of this:

  - References to “ITU-R BT.[HDR-TV]” in the text should be replaced with references to (the latest version of) BT.2100.

  - References to “Rep. ITU-R BT.2390” should be studied to determine whether they should also be replaced with references to (the latest version of) BT.2100.

- A small detail regarding the precise scaling factor associated with video_full_range_flag for transfer_characteristics equal to 16 and 18 has remained under study in ITU-R in the development of Rec. ITU-R BT.2100. The final text of ITU-T H.273 should take into account the latest information available on how this aspect should be defined.

- Other potential editorial problems may include:

  - Rep. ITU-R BT.2390 is mentioned in the text but not in the Bibliography

  - For consistency, references to BT.2020 and BT.2390 should refer to the latest version of each (e.g., BT.2020-2 and BT.2390-0) for consistency.

  - For TransferCharacteristics equal to 17, there is an obvious copy/paste error with an extra string “for all values of Lc” that should be deleted (note also that there is no Lc used in the relevant formula).

  - Regarding the described alignment with 23001-8, it would be more precise to say that the specification is “technically aligned with the video code points in ISO/IEC 23001-8:2016 and its in-progress Draft Amendment 1”.

- We also suggest that if any other such editorial problems are noticed in the text, they could be considered for inclusion.

________________________