| ITU – Telecommunications Standardization Sector STUDY GROUP 21 Question 6/21 Video Coding Experts Group (VCEG) 78th Meeting: 13–23 January 2026, by teleconference | Document VCEG-BZ10-v1 |
| --- | --- |

| Question: | 6/21 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Paul Haase (Fraunhofer HHI), Panji Setiawan (Dolby Laboratories) | Email: | paul.haase@hhi.fraunhofer.de panji.setiawan@dolby.com |
| Title: | Report on reference software development and anchor results for H.BWC |  |  |
| Purpose: | Report |  |  |

________________________

# Abstract

This document summarizes reference software development for biomedical waveform coding (H.BWC) between the 77th and 78th VCEG meetings and presents anchor results for version 4.0 of the reference software.

- Introduction

The reference software is a common framework which enables testing of new coding technologies in a unified environment and provides an exemplary implementation conforming to the draft specification of biomedical waveform coding (H.BWC).

This document reports on the progress of reference software development and provides anchor results for BWC-4.0.

- Software Development

Development of the reference software was continued on the GitLab server located at:
https://vcgit.hhi.fraunhofer.de/vceg-sw/bwc

The registration and development workflow are documented at:
https://vcgit.hhi.fraunhofer.de/vceg-sw/bwc/-/wikis/BWC-Software-Development-Workflow

The latest software model version before the start of the meeting was BWC-4.0:

https://vcgit.hhi.fraunhofer.de/vceg-sw/bwc/-/tags/BWC-4.0

The repositories for the core experiments can be found at:
https://vcgit.hhi.fraunhofer.de/vceg-bwc-ce/2510_gen-ce-1/bwc

https://vcgit.hhi.fraunhofer.de/vceg-bwc-ce/2510_gen-ce-2/bwc

https://vcgit.hhi.fraunhofer.de/vceg-bwc-ce/2510_gen-ce-3/bwc

https://vcgit.hhi.fraunhofer.de/vceg-bwc-ce/2510_gen-ce-4/bwc

https://vcgit.hhi.fraunhofer.de/vceg-bwc-ce/2510_gen-ce-5/bwc

https://vcgit.hhi.fraunhofer.de/vceg-bwc-ce/2510_gen-ce-6/bwc

https://vcgit.hhi.fraunhofer.de/vceg-bwc-ce/2510_gen-ce-7/bwc

The CE repositories are only accessible by the CE participants and the CE coordinators. Access can be granted by the CE coordinators.

# Software Development Progress

The progress in software development before the start of this meeting can be summarized as follows:

- Aligned the software with the draft specification [1]

- Bugfixes, code improvements and cleanups

- Integrated adopted proposals from the Geneva Meeting (10/2025)

- Added support for 24 bit signals

- Provided suitable software configurations for lossless coding

- Updated excel reporting templates for common test conditions [2]

- Issued version 4.0 of the BWC reference software (BWC-4.0)

- Created software repositories for the core experiments defined in [3] based on BWC-4.0

- Generated anchor coding results for further testing

# Anchor Results

Anchor results have been generated according to the common test conditions (CTC) described in [2] based on BWC-4.0. For each test set, there are two configurations. The first configuration realizes a joint coding of the channels while the second configuration realizes an independent coding of the channels.

Summaries of the results are shown in Table 1 to Table 4, detailed results are attached to this document as (Excel) xlsm and pdf (plots) files. 

There are two xlsm files reporting results with respect to BWC-3.0 for the joint channel coding configuration and the independent channel coding configuration, respectively.  The xlsm files are based on the excel reporting template, which can be found in the software repository (see sec. 2).

Each xlsm file reports the number of bits per samples (BPS), PSNR1,PSNR2 and Bjøntegaard Delta (BD) rates based on PSNR1 and PSNR2 for each test sequence and dataset as defined in the CTC document [2]. Furthermore, there are plot pdf-files per configuration, input sequence and PSNR measure (PSNR1 and PSNR2) showing coding results with respect to BWC-3.0.

Remark: Please note, that the results for the 24 bit TILT ILLUSION dataset are not reported with respect to BWC-3.0 since it does not support 24 bit signals. Here, the results in the excel sheet are the same for test and reference. The plot files for TILT ILLUSION show curves for independent channel coding with respect to joint channel coding.

Table 1 - Lossy compression results for joint channel coding

|  | Lossy Compression |  |  |  |
| --- | --- | --- | --- | --- |
|  | Over BWC-3.0 |  |  |  |
|  | BD-PSNR1 | BD-PSNR2 | EncT | DecT |
| MIT (ECG) | -0,75% | -0,75% | 79% | 89% |
| INCART (ECG) | -0,64% | -0,64% | 78% | 89% |
| CHBMIT (EEG) | -0,05% | -0,05% | 81% | 94% |
| NMR55 (EEG) | -0,10% | -0,10% | 79% | 94% |
| NMR57 (EEG) | -0,04% | -0,04% | 87% | 94% |
| TILT ILLUSION (EEG) | 0,00% | 0,00% | 100% | 100% |
| Ozdemir (EMG) | -0,04% | -0,03% | 79% | 96% |
| PTT (PPG) | 0,33% | 0,40% | 80% | 93% |
| WristPPG (PPG) | 0,33% | 0,31% | 80% | 96% |
| Overall | -0,11% | -0,10% | 83% | 94% |

Table 2 - Lossless compression results for joint channel coding

|  | Lossless Compression |  |  |
| --- | --- | --- | --- |
|  | Over BWC-3.0 |  |  |
|  | BR-R | EncT | DecT |
| MIT (ECG) | 0,82% | 44% | 92% |
| INCART (ECG) | -20,21% | 73% | 68% |
| CHBMIT (EEG) | -1,25% | 32% | 120% |
| NMR55 (EEG) | -3,70% | 28% | 99% |
| NMR57 (EEG) | -7,16% | 32% | 121% |
| TILT ILLUSION (EEG) | 0,00% | 100% | 100% |
| Ozdemir (EMG) | 0,14% | 97% | 166% |
| PTT (PPG) | -4,15% | 64% | 87% |
| WristPPG (PPG) | -4,05% | 65% | 79% |
| Overall | -4,40% | 59% | 104% |

Table 3 - Lossy compression results for independent channel coding

|  | Lossy Compression |  |  |  |
| --- | --- | --- | --- | --- |
|  | Over BWC-3.0 |  |  |  |
|  | BD-PSNR1 | BD-PSNR2 | EncT | DecT |
| MIT (ECG) | -0,85% | -0,85% | 82% | 94% |
| INCART (ECG) | -0,38% | -0,41% | 82% | 94% |
| CHBMIT (EEG) | -0,27% | -0,27% | 83% | 97% |
| NMR55 (EEG) | -0,17% | -0,16% | 83% | 98% |
| NMR57 (EEG) | -0,21% | -0,21% | 83% | 98% |
| TILT ILLUSION (EEG) | 0,00% | 0,00% | 100% | 100% |
| Ozdemir (EMG) | -0,06% | -0,05% | 82% | 98% |
| PTT (PPG) | -0,23% | -0,21% | 82% | 96% |
| WristPPG (PPG) | 0,24% | 0,19% | 83% | 98% |
| Overall | -0,21% | -0,22% | 85% | 97% |

Table 4 - Lossless compression results for independent channel coding

|  | Lossless Compression |  |  |
| --- | --- | --- | --- |
|  | Over BWC-3.0 |  |  |
|  | BR-R | EncT | DecT |
| MIT (ECG) | 0,10% | 57% | 93% |
| INCART (ECG) | -0,26% | 60% | 96% |
| CHBMIT (EEG) | -0,98% | 8% | 146% |
| NMR55 (EEG) | -5,75% | 8% | 122% |
| NMR57 (EEG) | -4,72% | 8% | 138% |
| TILT ILLUSION (EEG) | 0,00% | 100% | 100% |
| Ozdemir (EMG) | 0,60% | 79% | 169% |
| PTT (PPG) | -7,52% | 61% | 108% |
| WristPPG (PPG) | -3,25% | 73% | 182% |
| Overall | -2,42% | 50% | 128% |

# References

- J. Pfaff, C.Fersch , “H.BWC Draft 4 Specification Text,” VCEG-BZ02, Jan 2026.
https://www.itu.int/wftp3/av-arch/video-site/2601_Tel/

- J. Pfaff, C.Fersch, S. Jelfs, P. Haase , “Common test conditions and evaluation procedures for H.BWC technical experiments,” VCEG-BZ03, Jan 2026.
https://www.itu.int/wftp3/av-arch/video-site/2601_Tel/

- J. Pfaff, C.Fersch , “CE description for H.BWC,” VCEG-BZ04, Jan 2026.
https://www.itu.int/wftp3/av-arch/video-site/2601_Tel/