| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G272-r1 |
| --- | --- |

| Title: | Non-CE10: Core Transform Design for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Dong Yan Ye 9710 Scranton Road, Suite 250, San Diego, CA 92121, USA | Tel: Email: | +1 858 210 4809 jie.dong@interdigital.com yan.ye@interdigital.com |
| Source: | InterDigital Communication LLC |  |  |

_____________________________

# Abstract

This contribution presents the core transform design for HEVC, including 4×4, 8×8, 16×16 and 32×32 forward/inverse transforms. The design has these features: (1) 16 bit data representation before and after each transform stage, (2) transforms can be implemented by full factorization, partial butterfly, or matrix multiplication, (3) 4×4 and 8×8 transforms are orthogonal, and 16×16 and 32×32 transforms are nearly orthogonal, (4) N×N transform matrix is reused as the even part of the 2N×2N transform matrix, (5) no need for correction of different norms of basis vectors during quantization/dequantization. This design was integrated into HM4.0, using both full factorization and partial butterfly. It is reported full factorization for 32×32 transform achieves 6X reduction in the number of multiplications, compared with partial butterfly. The R-D performance was tested under the common test conditions specified in JCTVC-F900. The average BD-rate increase is 0.1% to 0.2% for all intra coding, 0.1% for random access coding, and 0.0% to 0.1% for low delay coding.

# Definitions of Transform Matrices

The transform matrices of the 4×4, 8×8, and 16×16 transforms are given as in (1), (2), and (3), respectively. The transform matrix of the 32×32 transform is appended at the end of this document.

T4=[

128   128   128   128

167    70   -70  -167

128  -128  -128   128

70  -167   167   -70

]

(1)

T8=[

256   256   256   256   256   256   256   256

360   297   198    72   -72  -198  -297  -360

334   140  -140  -334  -334  -140   140   334

297   -72  -360  -198   198   360    72  -297

256  -256  -256   256   256  -256  -256   256

198  -360    72   297  -297   -72   360  -198

140  -334   334  -140  -140   334  -334   140

72  -198   297  -360   360  -297   198   -72

]

(2)

T16=[

1024  1024  1024  1024  1024  1024  1024  1024  1024  1024  1024  1024  1024  1024  1024  1024

1445  1377  1275  1105   935   663   425   119  -119  -425  -663  -935 -1105 -1275 -1377 -1445

1440  1188   792   288  -288  -792 -1188 -1440 -1440 -1188  -792  -288   288   792  1188  1440

1402   886   174  -710 -1270 -1434 -1146  -398   398  1146  1434  1270   710  -174  -886 -1402

1336   560  -560 -1336 -1336  -560   560  1336  1336   560  -560 -1336 -1336  -560   560  1336

1318    86 -1074 -1370  -490   966  1446   622  -622 -1446  -966   490  1370  1074   -86 -1318

1188  -288 -1440  -792   792  1440   288 -1188 -1188   288  1440   792  -792 -1440  -288  1188

1104  -672 -1368   120  1440   432 -1272  -936   936  1272  -432 -1440  -120  1368   672 -1104

1024 -1024 -1024  1024  1024 -1024 -1024  1024  1024 -1024 -1024  1024  1024 -1024 -1024  1024

936 -1272  -432  1440  -120 -1368   672  1104 -1104  -672  1368   120 -1440   432  1272  -936

792 -1440   288  1188 -1188  -288  1440  -792  -792  1440  -288 -1188  1188   288 -1440   792

622 -1446   966   490 -1370  1074    86 -1318  1318   -86 -1074  1370  -490  -966  1446  -622

560 -1336  1336  -560  -560  1336 -1336   560   560 -1336  1336  -560  -560  1336 -1336   560

398 -1146  1434 -1270   710   174  -886  1402 -1402   886  -174  -710  1270 -1434  1146  -398

288  -792  1188 -1440  1440 -1188   792  -288  -288   792 -1188  1440 -1440  1188  -792   288

119  -425   663  -935  1105 -1275  1377 -1445  1445 -1377  1275 -1105   935  -663   425  -119

]

(3)

# Forward Transform, Quantization, Dequantization, and Inverse Transform

Denote the bit-depth of the input to transform as BD, and denote (BD-9) as ΔBD. The block input to transform has the size M×N. When NSQT is enabled, M might be different from N.

## Forward Transform

The input to forward transform is a prediction residual block, denoted as XM×N. To perform a 2-D forward transform on XM×N, the M rows and N columns in XM×N are transformed in each dimension sequentially.

- M is equal to N or M is smaller than N.

The horizontal transform is performed first, as in (4), followed by right shifting with a proper number of bits to store intermediate result in 16-bit.

UM×N = ( XM×N × TNT + f ) >> ( log2(N) + log2(TN(0,0)) + ΔBD - 7 )                           (4)

After the horizontal transform, vertical forward transform is performed on the intermediate block UM×N, as shown in (5).

YM×N = (TM × UM×N + f ) >> (log2(M)+ log2( TM(0,0) ) )                                   (5)

- M is greater than N.

The vertical transform is performed first, as in (6).

UM×N = ( TM × XM×N + f ) >> ( log2(M)  + log2(TM(0,0)) + ΔBD – 7 )                         (6)

After the vertical transform, horizontal transform is performed on the intermediate block UM×N, as shown in (7).

YM×N = (UM×N × TNT + f  ) >> ( log2(N) + log2(TN(0,0) ) )                                 (7)

## Inverse Transform

The input to inverse transform is a dequantized block YM×N. To perform a 2-D inverse transform on YM×N, the M rows and N columns in YM×N are transformed in a sequential manner.

- M is equal to N or M is greater than N.

The horizontal transform is performed first, as in (8).

VM×N = ( YM×N × TN + f ) >>  ( log2( TN(0,0) ) + 1 )                                     (8)

After the horizontal transform, vertical inverse transform is performed on the intermediate block VM×N, as shown in (9).

XM×N = ( TMT × VM×N + f ) >> (log2( TM(0,0) ) + 6 - ΔBD)                            (9)

- M is smaller than N.

The vertical transform is performed first, as in (10).

VM×N = ( TMT × YM×N + f  ) >> ( log2( TM(0,0) ) + 1)                                         (10)

After the vertical transform, horizontal transform is performed on the intermediate block VM×N, as shown in (11).

XM×N = ( VM×N × TN + f ) >>  (log2( TN(0,0) ) + 6 – ΔBD )                                  (11)

## Quantization and Dequantization

Quantization and dequantization in this proposal are the same as in HM4.0 [1].

# Simulation Results

This design was integrated into HM4.0 [1] and its R-D performance was tested under the common test conditions specified in [2]. The average BD-rate increases are 0.1% to 0.2% for all intra coding, 0.1% for random access coding, and 0.0% to 0.1% for low delay coding.

TABLE 1 R-D performance based on all intra settings (measured by BD-rate)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.4% | 0.3% | 0.1% | 0.7% | 0.2% | -0.1% |
| Class B | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% |
| Class C | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class E | 0.1% | 0.2% | 0.2% | 0.2% | 0.0% | 0.1% |
| Overall | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% | 0.0% |
|  | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% | 0.0% |

TABLE 2 R-D performance based on random access settings (measured by BD-rate)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.2% | -0.1% | 0.0% | 0.3% | 0.4% | -0.1% |
| Class B | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% | 0.1% |
| Class C | 0.1% | 0.0% | 0.2% | 0.1% | 0.0% | 0.1% |
| Class D | 0.1% | 0.0% | -0.2% | 0.0% | 0.0% | 0.3% |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |
|  | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |

TABLE 3 R-D performance based on low delay B settings (measured by BD-rate)

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.0% | -0.4% | 0.1% | 0.1% | 0.3% |
| Class C | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class D | 0.1% | -0.1% | -0.2% | 0.0% | 0.1% | 0.1% |
| Class E | 0.1% | -0.9% | 1.1% | 0.0% | 0.4% | -0.4% |
| Overall | 0.1% | -0.2% | 0.0% | 0.0% | 0.1% | 0.0% |
|  | 0.1% | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% |

TABLE 4 R-D performance based on low delay P settings (measured by BD-rate)

|  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.3% | -0.1% | 0.1% | 0.1% | 0.1% |
| Class C | 0.1% | 0.2% | 0.2% | 0.0% | 0.0% | 0.2% |
| Class D | 0.2% | 0.2% | 0.0% | 0.0% | 0.1% | -0.1% |
| Class E | 0.1% | -0.1% | 0.0% | 0.1% | -0.4% | -0.3% |
| Overall | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% |
|  | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% |

# Implementation

The proposed core transform design is realized by both full factorization and partial butterfly. In the enclosed software “JCTVC-G272-software.zip”, the modifications on HM4.0 are indicated by two macros: G272_TRANSFORM and G272_TRANSFORM_FULL_FACTORIZATION. Enabling only G272_TRANSFORM, one makes the proposed forward and inverse transforms performed by partial butterfly; enabling both macros, one makes the proposed transforms performed by full factorization. TABLE 5 reports the numbers of multiplications and additions for N-point 1-D transform ( N = 4, 8, 16, or 32 ), according to the full factorization implementation of the enclosed software, and compares them to matrix multiplication, partial butterfly, and the full factorization implementations from the other two contributions [3] [4] of this meeting.

TABLE 5 Numbers of additions and multiplications for N-point 1-D transform ( N = 4, 8, 16, or 32 )

|  |  | Matrix Multiplication | Partial Butterfly | Factorization in JCTVC-G579 [3] | Factorization in JCTVC-G737 [4] | Proposed Factorization |
| --- | --- | --- | --- | --- | --- | --- |
| 4-point | Addition | 12 | 8 | 9 | 9 | 9 |
|  | Multiplication | 16 | 6 | 3 | 3 | 3 |
| 8-point | Addition | 56 | 28 | 26 | 29 | 31 |
|  | Multiplication | 64 | 22 | 12 | 11 | 11 |
| 16-point | Addition | 240 | 100 | 72 | 81 | 93 |
|  | Multiplication | 256 | 86 | 36 | 31 | 21 |
| 32-point | Addition | 992 | 372 | 186 | 229 | 279 |
|  | Multiplication | 1024 | 342 | 92 | 87 | 56 |

# Conclusion

This contribution presents the core transform design for HEVC, including 4×4, 8×8, 16×16 and 32×32 forward/inverse transforms. The design has these features: (1) 16 bit data representation before and after each transform stage, (2) transforms can be implemented by full factorization, partial butterfly, or matrix multiplication, (3) 4×4 and 8×8 transforms are orthogonal, and 16×16 and 32×32 transforms are nearly orthogonal, (4) N×N transform matrix is reused as the even part of the 2N×2N transform matrix, (5) no need for correction of different norms of basis vectors during quantization/dequantization. This design was integrated into HM4.0, using both full factorization and partial butterfly. It is reported full factorization for 32×32 transform achieves 6X reduction in the number of multiplications, compared with partial butterfly. The R-D performance was tested under the common test conditions specified in JCTVC-F900. The average BD-rate increases are 0.1% to 0.2% for all intra coding, 0.1% for random access coding, and 0.0% to 0.1% for low delay coding. We suggest putting this design into the core experiment for further investigation.

# Reference

- https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-4.0/

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Jul. 2011.

- R. Joshi et al., “CE10: Scaled integer transforms supporting recursive factorization structure”, JCTVC-G579, Nov. 2011.

- E. Alshina et al., “CE10: Full Factorization Core Transforms for HEVC”, JCTVC-G737, Nov. 2011.

# Patent rights declaration(s)

InterDigital Communications LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

T32 = [

2048  2048  2048  2048  2048  2048  2048  2048  2048  2048  2048  2048  2048  2048  2048  2048  2048  2048  2048  2048  2048  2048  2048  2048  2048  2048  2048  2048  2048  2048  2048  2048

2912  2821  2779  2723  2685  2460  2340  2145  2015  1690  1510  1205  1036   665   455    91   -91  -455  -665 -1036 -1205 -1510 -1690 -2015 -2145 -2340 -2460 -2685 -2723 -2779 -2821 -2912

2890  2754  2550  2210  1870  1326   850   238  -238  -850 -1326 -1870 -2210 -2550 -2754 -2890 -2890 -2754 -2550 -2210 -1870 -1326  -850  -238   238   850  1326  1870  2210  2550  2754  2890

2821  2639  2107  1505   660   -90 -1040 -1690 -2340 -2730 -2920 -2810 -2429 -1981 -1183  -455   455  1183  1981  2429  2810  2920  2730  2340  1690  1040    90  -660 -1505 -2107 -2639 -2821

2880  2376  1584   576  -576 -1584 -2376 -2880 -2880 -2376 -1584  -576   576  1584  2376  2880  2880  2376  1584   576  -576 -1584 -2376 -2880 -2880 -2376 -1584  -576   576  1584  2376  2880

2821  2093   959  -413 -1765 -2590 -2860 -2535 -1495  -130  1220  2355  2884  2667  1911   728  -728 -1911 -2667 -2884 -2355 -1220   130  1495  2535  2860  2590  1765   413  -959 -2093 -2821

2804  1772   348 -1420 -2540 -2868 -2292  -796   796  2292  2868  2540  1420  -348 -1772 -2804 -2804 -1772  -348  1420  2540  2868  2292   796  -796 -2292 -2868 -2540 -1420   348  1772  2804

2730  1456  -399 -2135 -2935 -2305  -715  1235  2665  2795  1735  -115 -1974 -2828 -2457 -1001  1001  2457  2828  1974   115 -1735 -2795 -2665 -1235   715  2305  2935  2135   399 -1456 -2730

2672  1120 -1120 -2672 -2672 -1120  1120  2672  2672  1120 -1120 -2672 -2672 -1120  1120  2672  2672  1120 -1120 -2672 -2672 -1120  1120  2672  2672  1120 -1120 -2672 -2672 -1120  1120  2672

2548   728 -1757 -2807 -1925   405  2535  2665   975 -1495 -2915 -2085   112  2324  2821  1183 -1183 -2821 -2324  -112  2085  2915  1495  -975 -2665 -2535  -405  1925  2807  1757  -728 -2548

2636   172 -2148 -2740  -980  1932  2892  1244 -1244 -2892 -1932   980  2740  2148  -172 -2636 -2636  -172  2148  2740   980 -1932 -2892 -1244  1244  2892  1932  -980 -2740 -2148   172  2636

2457  -182 -2618 -2289   415  2790  2210  -715 -2795 -1950   950  2855  1708 -1288 -2912 -1456  1456  2912  1288 -1708 -2855  -950  1950  2795   715 -2210 -2790  -415  2289  2618   182 -2457

2376  -576 -2880 -1584  1584  2880   576 -2376 -2376   576  2880  1584 -1584 -2880  -576  2376  2376  -576 -2880 -1584  1584  2880   576 -2376 -2376   576  2880  1584 -1584 -2880  -576  2376

2275  -910 -2870  -721  2490  2200 -1300 -2860  -390  2600  1900 -1460 -2765  -210  2730  1729 -1729 -2730   210  2765  1460 -1900 -2600   390  2860  1300 -2200 -2490   721  2870   910 -2275

2208 -1344 -2736   240  2880   864 -2544 -1872  1872  2544  -864 -2880  -240  2736  1344 -2208 -2208  1344  2736  -240 -2880  -864  2544  1872 -1872 -2544   864  2880   240 -2736 -1344  2208

2093 -1729 -2506  1197  2690  -705 -2925   130  2860   455 -2875  -980  2569  1519 -2366 -1911  1911  2366 -1519 -2569   980  2875  -455 -2860  -130  2925   705 -2690 -1197  2506  1729 -2093

2048 -2048 -2048  2048  2048 -2048 -2048  2048  2048 -2048 -2048  2048  2048 -2048 -2048  2048  2048 -2048 -2048  2048  2048 -2048 -2048  2048  2048 -2048 -2048  2048  2048 -2048 -2048  2048

1911 -2366 -1519  2569   980 -2875  -455  2860  -130 -2925   705  2690 -1197 -2506  1729  2093 -2093 -1729  2506  1197 -2690  -705  2925   130 -2860   455  2875  -980 -2569  1519  2366 -1911

1872 -2544  -864  2880  -240 -2736  1344  2208 -2208 -1344  2736   240 -2880   864  2544 -1872 -1872  2544   864 -2880   240  2736 -1344 -2208  2208  1344 -2736  -240  2880  -864 -2544  1872

1729 -2730  -210  2765 -1460 -1900  2600   390 -2860  1300  2200 -2490  -721  2870  -910 -2275  2275   910 -2870   721  2490 -2200 -1300  2860  -390 -2600  1900  1460 -2765   210  2730 -1729

1584 -2880   576  2376 -2376  -576  2880 -1584 -1584  2880  -576 -2376  2376   576 -2880  1584  1584 -2880   576  2376 -2376  -576  2880 -1584 -1584  2880  -576 -2376  2376   576 -2880  1584

1456 -2912  1288  1708 -2855   950  1950 -2795   715  2210 -2790   415  2289 -2618   182  2457 -2457  -182  2618 -2289  -415  2790 -2210  -715  2795 -1950  -950  2855 -1708 -1288  2912 -1456

1244 -2892  1932   980 -2740  2148   172 -2636  2636  -172 -2148  2740  -980 -1932  2892 -1244 -1244  2892 -1932  -980  2740 -2148  -172  2636 -2636   172  2148 -2740   980  1932 -2892  1244

1183 -2821  2324  -112 -2085  2915 -1495  -975  2665 -2535   405  1925 -2807  1757   728 -2548  2548  -728 -1757  2807 -1925  -405  2535 -2665   975  1495 -2915  2085   112 -2324  2821 -1183

1120 -2672  2672 -1120 -1120  2672 -2672  1120  1120 -2672  2672 -1120 -1120  2672 -2672  1120  1120 -2672  2672 -1120 -1120  2672 -2672  1120  1120 -2672  2672 -1120 -1120  2672 -2672  1120

1001 -2457  2828 -1974   115  1735 -2795  2665 -1235  -715  2305 -2935  2135  -399 -1456  2730 -2730  1456   399 -2135  2935 -2305   715  1235 -2665  2795 -1735  -115  1974 -2828  2457 -1001

796 -2292  2868 -2540  1420   348 -1772  2804 -2804  1772  -348 -1420  2540 -2868  2292  -796  -796  2292 -2868  2540 -1420  -348  1772 -2804  2804 -1772   348  1420 -2540  2868 -2292   796

728 -1911  2667 -2884  2355 -1220  -130  1495 -2535  2860 -2590  1765  -413  -959  2093 -2821  2821 -2093   959   413 -1765  2590 -2860  2535 -1495   130  1220 -2355  2884 -2667  1911  -728

576 -1584  2376 -2880  2880 -2376  1584  -576  -576  1584 -2376  2880 -2880  2376 -1584   576   576 -1584  2376 -2880  2880 -2376  1584  -576  -576  1584 -2376  2880 -2880  2376 -1584   576

455 -1183  1981 -2429  2810 -2920  2730 -2340  1690 -1040    90   660 -1505  2107 -2639  2821 -2821  2639 -2107  1505  -660   -90  1040 -1690  2340 -2730  2920 -2810  2429 -1981  1183  -455

238  -850  1326 -1870  2210 -2550  2754 -2890  2890 -2754  2550 -2210  1870 -1326   850  -238  -238   850 -1326  1870 -2210  2550 -2754  2890 -2890  2754 -2550  2210 -1870  1326  -850   238

91  -455   665 -1036  1205 -1510  1690 -2015  2145 -2340  2460 -2685  2723 -2779  2821 -2912  2912 -2821  2779 -2723  2685 -2460  2340 -2145  2015 -1690  1510 -1205  1036  -665   455   -91

]