#### 7.3.3.4  Adaptive loop filter parameter syntax

| alf_param() { | Descriptor |
| --- | --- |
| alf_cb_enable_flag | u(1) |
| alf_cr_enable_flag | u(1) |
| alf_one_luma_unit_per_slice_flag |  |
| if( alf_cb_enable_flag ) |  |
| alf_one_cb_unit_per_slice_flag | u(1) |
| if( alf_cr_enable_flag ) |  |
| alf_one_cr_unit_per_slice_flag |  |
| if( !alf_coef_in_slice_flag && !(alf_one_cr_unit_per_slice_flag && alf_one_cb_unit_per_slice_&& flag alf_one_luma_unit_per_slice_flag)) { |  |
| alf_num_units_in_width_minus1 | ue(v) |
| alf_num_units_in_height_minus1 | ue(v) |
| } else |  |
| alf_num_lcu_in_slice_minus1 | u(v) |
| endCtbrY = (numCtb − 1 + firstCtbAddr) / numCtbInWidth |  |
| endCtbrX = (numCtb − 1 + firstCtbAddr) % numCtbInWidth |  |
| for( i = 0; i < numCtb; i++ ) { |  |
| rx = ( i + firstCtbAddr) % numCtbInWidth |  |
| ry = ( i + firstCtbAddr) / numCtbInWidth |  |
| endrX = ( ry = = endCtbrY ) ? ( endCtbrX ) : ( numCtbInWidth − 1 ) |  |
| if( ( rx = = 0 ) && ( i − numCtbInWidth >= 0 ) && ( alf_one_luma_unit_per_slice_flag = =0 ) ) |  |
| alf_repeat_row _flag[0] | u(1) |
| alf_unit( rx, ry, 0, i, endrX, alf_one_luma_unit_per_slice_flag ) |  |
| if( alf_cb_enable_flag ) { |  |
| if( ( rx = = 0 ) && ( i − numCtbInWidth >= 0 ) && ( alf_one_cb_unit_per_slice_flag = =0 ) ) |  |
| alf_repeat_row _flag[1] | u(1) |
| alf_unit(rx, ry, 1, i, endrX, alf_one_cb_unit_per_slice_flag ) |  |
| } |  |
| if( alf_cr_enable_flag ) { |  |
| if( ( rx = = 0) && ( i − numCtbInWidth>= 0) && ( alf_one_cr_unit_per_slice_flag = =0 ) ) |  |
| alf_repeat_row _flag[2] | u(1) |
| alf_unit(rx, ry, 2, i, endrX, alf_one_cr_unit_per_slice_flag ) |  |
| } |  |
| } |  |
| } |  |

#### Adaptive loop filter parameter semantics

The variable NumALFFiltersInStoredBuffer[ cIdx ] is set equal to 0 for every colour component cIdx equal to 0, 1 and 2.

alf_cb_enable_flag equal to 1 specifies that adaptive loop filter is applied on Cb component; equal to 0 specifies that adaptive loop filter is disabled on Cb component.

alf_cr_enable_flag equal to 1 specifies that adaptive loop filter is applied on Cr component; equal to 0 specifies that adaptive loop filter is disabled on Cr component.

alf_one_luma_unit_per_slice_flag equal to 1 specifies that all of coding treeblocks in luma component of current slice are filtered by the same ALF parameter set; equal to 0 specifies that ALF parameter sets in luma component are adaptive on a coding treeblock basis.

alf_one_cb_unit_per_slice_flag equal to 1 specifies that all of coding treeblocks in Cb component of current slice are filtered by the same ALF parameter set; equal to 0 specifies that ALF parameter sets in Cb component are  adaptive on a coding treeblock basis.

alf_one_cr_unit_per_slice_flag equal to 1 specifies that all of coding treeblocks in Cr component of current slice are be filtered by the same ALF parameter set; equal to 0 specifies that ALF parameter sets in Cr component are adaptive on a coding treeblock basis.

alf_num_units_in_width_minus1 plus 1 specifies the number of ALF parameter units in picture width.

- If alf_num_units_in_width_minus1 is present, the variable numCtbInWidth is set to (alf_num_units_in_width_minus1 + 1).

- Otherwise (alf_num_units_in_width_minus1 is not present), numCtbInWidth is set to PicWidthInCtbs.

alf_num_lcu_in_height_minus1 plus 1 specifies the number of ALF parameter units in picture height. The variable numCtbInHeight is set to (alf_num_units_in_height_minus1 + 1).

- If alf_num_units_in_height_minus1 is present, the variable numCtbInHeight is set to (alf_num_units_in_height_minus1 + 1).

- Otherwise (alf_num_units_in_height_minus1 is not present), numCtbInHeight is set to PicHeightInCtbs.

alf_num_lcu_in_slice_minus1 plus 1 specifies the number of coding treeblocks in the current slice. The length of the alf_num_lcu_in_slice_minus1 syntax element is Ceil( Log2(PicWidthInCtbs* PicHeightInCtbs) ) bits.

- If alf_num_lcu_in_slice_minus1 is present, numCtb is set equal to (alf_num_lcu_in_slice_minus1 + 1), alfAcrossSlice is set to slice_loop_filter_across_slices_enabled_flag, and firstCtbAddr is set to SliceCtbAddrRS.

- Otherwise (alf_num_lcu_in_slice_minus1 is not present), numCtb is set equal to (numCtbInWidth*numCtbInHeight), alfAcrossSlice is set to 1, and firstCtbAddr is set to 0.

alf_repeat_row_flag[ cIdx ] equal to 1 specifies that the ALF parameter sets of coding treeblocks in the current coding treeblock row shall be the same as those of the above coding treeblocks for the colour component cIdx; equal to 0 specifies that the current coding treeblock row has new ALF parameter sets for the colour component cIdx. When alf_repeat_row_flag[ cIdx ] is not present, it is inferred to be 0.

#### Adaptive loop filter unit semantics

The number of times the ALF parameters corresponding to an ALF parameter unit are repeated in the same parameter unit row is represented by alfRun[ cIdx ][ rx ][ ry ]. The array index cIdx specifies the colour component; cIdx is equal to 0 for luma, equal to 1 for Cb, and equal to 2 for Cr. The array indices rx and ry specify the location ( rx, ry ) of the ALF parameter unit that is considered in the current coding treeblock. Given the coordinate indices Ctbx and Ctby of the current coding treeblock (relative to the top-left coding treeblock of the picture), the rx and ry are derived as follows:

- rx = floor (((Ctbx + 1) * (alf_num_units_in_width_minus1 + 1)) / num_LCU_in_width) – 0.0001)

- ry = floor (((Ctby + 1) * (alf_num_units_in_height_minus1 + 1)) / num_LCU_in_height) – 0.0001)

alf_lcu_enable_flag[ cIdx ][ ry ][ rx ] equal to 1 specifies that the adaptive loop filter shall be applied on current coding treeblock at position Ctbx and Ctby in the colour component cIdx; equal to 0 specifies that adaptive loop filter shall be disabled for the current coding treeblock at position Ctbx and Ctby in the colour component cIdx.

alf_run_diff specifies the alfRun of current coding treeblock if the current row is the first row, otherwise specifies the difference between the run of current coding treeblock and the run of the above coding treeblock. When alfRun is greater than or equal to 0, the syntax elements in alf_info() are derived from the corresponding syntax elements of the left coding treeblock. The length of the alf_run_diff syntax element is Ceil( Log2(alf_num_lcu_in_width_minus1 − rx + 2) ) bits.

alf_merge_up_flag equal to 1 specifies that the syntax elements in alf_info() are derived from the corresponding syntax elements of the above coding treeblock; equal to 0 specifies that the syntax elements in alf_info() are not derived from the corresponding syntax elements of the above coding treeblock. When alf_merge_up_flag is not present, it is inferred to be equal to 0.

#### 7.4.3.6  Adaptive loop filter info semantics

alf_new_filter_set_flag equal to 1 specifies that the current coding treeblock uses a new filter set; equal to 0 specifies that the current coding treeblock uses the stored filter set with the buffer index equal to alf_stored_filter_set_idx[ cIdx ] of the colour component cIdx.

When alf_new_filter_set_flag is not present, it is inferred to be 1.

When alf_new_filter_set_flag is equal to 1, NumALFFiltersInStoredBuffer[cIdx] is increased by 1.

alf_stored_filter_set_idx[ cIdx ] specifies the buffer index of the stored filters for the colour component cIdx. The length of the alf_stored_filter_set_idx[ cIdx ]  syntax element is Floor( Log2(Min( 1 , NumALFFiltersInStoredBuffer[cIdx] − 1) ) ) + 1 bits.

alf_no_filters_minus1 is used to derive the number of filter sets for the current coding treeblock. It is a requirement of bitstream conformance that the values of alf_no_filters_minus1 are in the range of 0 to 2, inclusive.

alf_start_second_filter specifies the block adaptive (BA) mode index of luma samples for which the second filter is applied.

alf_filter_pattern_flag[ cIdx ][ ry ][ rx ][ i ] equal to 1 specifies that the filter index for the i-th BA mode index is increased by 1: equal to 0 specifies that the filter index for the i-th BA mode index is the same as i − 1 BA mode index.

The number of filter sets AlfNumFilters is derived as follows.

- If alf_no_filters_minus1 is less than 2,

AlfNumFilters = alf_no_filters_minus1 + 1		(7-62)

- Otherwise (alf_no_filters_minus1 is equal to 2)

AlfNumFilters = i alf_filter_pattern_flag[ i ] with i = 0..15	(7-63)

The mapping between the i-th block adaptive (BA) mode index for luma samples and the filter index MapFiltFidx[ ry ][ rx ][ i ] specifying is derived as follows:

- If AlfNumFilters is equal to 1,

MapFiltFidx[ ry ][ rx ][ i ] = 0 with i = 0 .. 15		(7-64)

- Otherwise if AlfNumFilters is equal to 2,

MapFiltFidx[ ry ][ rx ][ i ] = 0 with i = 0 .. alf_start_second_filter−1	(7-65)
MapFiltFidx[ ry ][ rx ] [ i ] = 1 with i = alf_start_second_filter .. 15

- Otherwise (AlfNumFilters is greater than 2),

MapFiltFidx[ ry ][ rx ][ 0 ] = 0
MapFiltFidx[ ry ][ rx ][ i+1 ] =  MapFiltFidx[ ry ][ rx ][ i ] + alf_filter_pattern[ ry ][ rx ][ i − 1 ]	(7-66)
	with i = 1..15

alf_pred_flag[ cIdx ][ ry ][ rx ] equal to 1 specifies that the filter coefficients for the current coding treeblock with at location ( Ctbx, Ctby) for the colour component cIdx are coded in a predictive way; equal to 0 specifies that the filter coefficients are coded independently, where rx and ry are derived as follows:

- rx = floor (((Ctbx + 1) * (alf_num_units_in_width_minus1 + 1)) / num_LCU_in_width) – 0.0001)

- ry = floor (((Ctby + 1) * (alf_num_units_in_height_minus1 + 1)) / num_LCU_in_height) – 0.0001)

alf_min_kstart_minus1 plus 1 specifies the minimum order k of k-th order exponential golomb code for the luma filter coefficients for the adaptive loop filter.

alf_golomb_index_flag[ i ] specifies the difference in the orders of k-th order exponential golomb codes between i-th group and (i+1)-th group of the luma filter coefficients. There are several groups of the luma filter coefficients where each group may have a different order k. The group value of the j-th luma filter coefficient alfCoeffGroup[ j ] is derived as follows.

alfCoeffGroup[ j ] = { 2, 3, 3, 4, 3, 1, 2, 3, 4, 1 } with j = 0..9	(7-67)

The table of different orders k of the j-th coefficient alfkVal[ alfCoeffGroup[ j ] ] with j=0..9 is derived as follows.

alfkVal[ i ] = (alf_min_kstart_minus1 +1)+ Σr alf_golomb_index_flag[r] with i= 1..3 and r= 1.. i	(7-68)

The k-th order of j-th alf_filter_coeff alfkMap[j] is derived as follows.

alfkMap[j] = alfkVal[ alfCoeffGroup[ j ] ] with j = 0..9

When alf_golomb_index_flag [i ] is not present, the alfkMap[ j ] = alfCoeffGroup[ j ]

alf_nb_pred_luma_flag[ cIdx ][ ry ][ rx ][ i ] equal to 1 specifies whether the filter coefficients of i-th filter for the current coding treeblock at location ( Ctbx, Ctby) for the colour component cIdx are coded in a predictive way using spatially neighbouring filter coefficients; euqal to 0 specifies that their are not coded using spatially neighbouring filter coefficients.

alf_filt_coeff[ cIdx ][ ry ][ rx ][ i ][ j ] specifies the j-th filter coefficient of i-th filter used in the adaptive loop filtering process for the current coding treeblock at location ( Ctbx, Ctby) for the colour component cIdx.

### Adaptive loop filter process

An adaptive loop filtering process shall be conditionally performed on a treeblock basis after the completion of the sample adaptive offset process for the entire decoded picture, with all treeblocks in a picture processed in order of increasing treeblock addresses.

Each treeblock is processed on a coding unit basis with the same order as decoding process.

This process is invoked when both adaptive_loop_filter_enabled_flag and slice_adaptive_loop_filter_flag are equal to 1.

This process is performed on a coding unit basis after the completion of the slice construction process prior to adaptive loop filter process for the entire decoded slice, with all coding units in a slice processed in order of coding unit scan order.

The current coding unit with the luma location ( xC, yC ) specifying the top-left luma sample is belonged to the current coding treeblock at location ( Ctbx, Ctby), and the ALF parameter set of this current coding treeblock, including alf_lcu_enable_flag[cIdx][ry][rx], alf_filter_pattern_flag[cIdx][ry][rx], alf_filt_coeff[cIdx][ry][rx] shall be used in the following process for three colour components cIdx, respectively, where rx and ry are derived as follows:

- rx = floor (((Ctbx + 1) * (alf_num_units_in_width_minus1 + 1)) / num_LCU_in_width) – 0.0001)

- ry = floor (((Ctby + 1) * (alf_num_units_in_height_minus1 + 1)) / num_LCU_in_height) – 0.0001)

When alf_lcu_enable_flag[cIdx][ry][rx] is equal to 1, ALF filtering process shall be applied, including filter coefficient derivation specified in subclause 8.7.3.2, filter index derivation for luma specified in subclause 8.7.3.3, filtering process for luma specified in subclause 8.7.3.4, and filtering process for chroma specified in subclause 8.7.3.5.

When interpreting the luma samples of the coding unit as to be filtered, depending on pcm_flag, pcm_loop_filter_disable_flag, alf_lcu_enable_flag[ 0 ][ ry ][ rx ] and alf_cu_control_flag, the following applies.

- If pcm_loop_filter_disable_flag is equal to 1 and pcm_flag is equal to 1, the luma samples are not filtered.

- Otherwise, if qpprime_y_zero_transquant_bypass_flag is equal to 1 and QP’Y is equal to 0, luma samples are not filtered,

- Otherwise, if alf_lcu_enable_flag[ 0 ][ ry ][ rx ] is equal to 0, the luma samples are not filtered,

- Otherwise, if alf_cu_control_flag is equal to 0, the luma samples are filtered,

- Otherwise (alf_cu_control_flag is equal to 1), if AlfCuFlag[ xC ][ yC ] is equal to 1 where xC and yC are the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture, the luma samples are filtered.

- Otherwise, the luma samples are not filtered.

When interpreting the Cb chroma samples of the coding unit as to be filtered, depending on pcm_flag, pcm_loop_filter_disable_flag alf_lcu_enable_flag[1][ry][rx], and alf_cb_enable_flag, the following applies.

- If pcm_loop_filter_disable_flag is equal to 1 and pcm_flag is equal to 1, the samples of the colour component 1 are not filtered.

- Otherwise, if qpprime_y_zero_transquant_bypass_flag is equal to 1 and QP’Y is equal to 0, the samples of the colour component 1 are not filtered.

- Otherwise, if alf_cb_enable_flag is equal to 0, the samples in the colour component 1 are not filtered.

- Otherwise, if alf_lcu_enable_flag[1][ry][rx] is equal to 1, the samples of the colour component 1 are filtered.

- Otherwise (alf_lcu_enable_flag[1][ry][rx] is equal to 0), the samples of the colour component 1 are not filtered.

When interpreting the Cr samples of the coding unit as to be filtered, depending on pcm_flag, pcm_loop_filter_disable_flag, alf_lcu_enable_flag[2][ry][rx], and alf_cr_enable_flag, the following applies.

- If pcm_loop_filter_disable_flag is equal to 1 and pcm_flag is equal to 1, the samples of the colour component 2 are not filtered.

- Otherwise, if qpprime_y_zero_transquant_bypass_flag is equal to 1 and QP’Y is equal to 0, the samples of the colour component 2 are not filtered.

- Otherwise, if alf_cr_enable_flag is equal to 0, the samples in the colour component 2 are not filtered.

- Otherwise, if alf_lcu_enable_flag[2][ry][rx] is equal to 1, the samples of the colour component 2 are filtered.

- Otherwise (alf_lcu_enable_flag[2][ry][rx] is equal to 0), the samples of the colour component 2 are not filtered.

For the luma samples of the coding unit interpreted as to be filtered, the filter coefficient derivation specified in subclause 8.7.3.2 is invoked with the colour component index cIdx equal to 0, and the filtering process for luma samples specified in subclause 8.7.3.4 is invoked with the luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture, the coding unit size log2CbSize, and the filter index array fIdx as inputs and the output is the modified filtered picture, recFiltPictureL.

For the chroma samples of the coding unit interpreted as to be filtered, the filter coefficient derivation specified in subclause 8.7.3.2 is invoked with the colour component index cIdx equal to 1, and the filtering process for chroma samples specified in subclause 8.7.3.5 is invoked with the chroma location ( xC/2, yC/2 ) specifying the top-left chroma sample of the current coding unit relative to the top left chroma sample of the current picture, the coding unit size log2CbSize−1 and the chroma component index cIdx equal to 1 as inputs and the output is the modified filtered picture, recFiltPictureCb.

For the chroma samples of the coding unit interpreted as to be filtered, the filter coefficient derivation specified in subclause 8.7.3.2 is invoked with the colour component index cIdx equal to 2, and the filtering process for chroma samples specified in subclause 8.7.3.5 is invoked with the chroma location ( xC/2, yC/2 ) specifying the top-left chroma sample of the current coding unit relative to the top left chroma sample of the current picture, the coding unit size log2CbSize−1 and the chroma component index cIdx equal to 2 as inputs and the output is the modified filtered picture, recFiltPictureCr.