| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 13th Meeting: Incheon, South Korea, 18–26 April 2013 | Document: JCTVC-M0211 |
| --- | --- |

| Title: | AHG9: On Scaling List Signalling for SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shan Liu, Ximin Zhang, Shawmin Lei MediaTek USA 2860 Junction Ave, San Jose, CA 95134, USA Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Tel: Email: | +1 4085261899 {shan.liu, shawmin.lei} @mediatek.com +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | MediaTek Inc. and Sony Corp. |  |  |

_____________________________

# Abstract

This contribution proposes the syntax and methods for signaling scaling list data in the enhancement layer for SHVC. Due to the similarity between an enhancement layer and its temporal collocated base layer, it is intuitive to derive the enhancement layer scaling list data from the base (or reference) layer scaling list data. Hence, this contribution proposes a few solutions for achieving this purpose, including (1) modifying the semantics of existing syntax elements “sps_scaling_list_data_present_flag” and “pps_scaling_list_data_present_flag”; (2) introducing new syntax elements “sps_base_pred_scaling_list_flag” and ““pps_base_pred_scaling_list_flag” to existing SPS and PPS syntax; (3) modifying the semantics of existing syntax element “scaling_list_pred_matrix_id_delta”; and (4) introducing new syntax element “base_pred_scaling_list_flag” to scaling list data syntax. Besides, inter-layer scaling list prediction for the size 16x16 and 32x32 is not allowed if the reference layer is the base layer and the value of avc_base_layer_flag in vps_extension() is 1. Furthermore, it is proposed to apply Inter scaling list data on blocks that are inter-layer texture predicted.

Introduction

In the prior art SVC (scalable video coding) standard as well as the current SHVC/SHM, the scaling list (quantization matrix) data derivation and signalling are independent in each spatial or quality layer. However due to the high correlations among different spatial and quality layers with the same time stamp, it is intuitive to apply the same scaling list data on those temporal collocated spatial and quality layers. Therefore, this contribution proposes several solutions for deriving and signalling scaling list data of an enhancement (higher) layer from the base (reference) layer such that some redundancies from signalling scaling list information as in the prior art standard can be removed.

# Proposed syntax and semantics

Four solutions are proposed to achieve the goal of deriving the enhancement layer scaling list data from temporal collocated base (reference) layer scaling list data.

In the proposed method 1, the semantic descriptions of enhancement layer syntax elements are modified as follows.

- sps_scaling_list_data_present_flag equal to 1 specifies that scaling list data are present in the sequence parameter set. sps_scaling_list_data_present_flag equal to 0 specifies that scaling list data are not present in the sequence parameter set. When not present, the value of sps_scaling_list_data_present_flag is inferred to be equal to 0. When scaling_list_enabled_flag is equal to 1 and sps_scaling_list_data_present_flag is equal to 0,

when nuh_layer_id equal to 0, or when nuh_layer_id is greater than 0 and sizeID is 2 or 3, the reference layer is base layer and the value avc_base_layer_flag in vps_extension() is 1, the default scaling list data is used to derive the array ScalingFactor as described in the scaling list data semantics specified in subclause 7.4.5.

Otherwise, scaling list data of the reference layer is used for coding the current layer. The reference layer can be the immediate lower layer or the base layer, or indicated by a reference_layer_id.

- pps_scaling_list_data_present_flag equal to 1 specifies that parameters are present in the picture parameter set to modify the scaling lists specified in the active sequence parameter set. pps_scaling_list_data_present_flag equal to 0 specifies that the scaling lists used for the pictures referring to the picture parameter set is inferred to be equal to those specified by the active sequence parameter set. When scaling_list_enabled_flag is equal to 0, the value of pps_scaling_list_data_present_flag shall be equal to 0. When scaling_list_enabled_flag is equal to 1, sps_scaling_list_data_present_flag is equal to 0, and pps_scaling_list_data_present_flag is equal to 0,

when nuh_layer_id equal to 0, or when nuh_layer_id is greater than 0 and sizeID is 2 or 3, the reference layer is base layer and the value avc_base_layer_flag in vps_extension() is 1, the default scaling list data is used to derive the array ScalingFactor as described in the scaling list data semantics specified in subclause 7.4.5.

Otherwise, scaling list data of the reference layer is used for coding the current layer. The reference layer can be the immediate lower layer or the base layer, or indicated by a reference_layer_id.

In the proposed method 2, a new syntax element “sps_base_pred_scaling_list_flag” is introduced to SPS and a syntax element “pps_base_pred_scaling_list_flag” is introduced to PPS of the higher (or enhancement) layers with nuh_layer_id greater than 0. The added syntax and semantics are as follows.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... ... |  |
| scaling_list_enabled_flag | u(1) |
| if( scaling_list_enaled_flag ) { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) |  |
| sps_base_pred_scaling_list_flag | u(1) |
| if( !sps_base_pred_scaling_list_flag ) |  |
| scaling_list_data( ) |  |
| } |  |
| ... ... |  |
| rbsp_trailing_bits( ) |  |
| } |  |

- sps_base_pred_scaling_list_flag is signalled if sps_scaling_list_data_present_flag is equal to 1. sps_base_pred_scaling_list_flag equal to 1 specifies that the scaling list data are inferred to be equal to those specified in the reference layer sequence parameter set. sps_base_pred_scaling_list_flag equal to 0 specifies that scaling list data present in the sequence parameter set. The value shall be 0 if the reference layer is the base layer and the value avc_base_layer_flag in vps_extension() is 1. When not present, the value is inferred to be 0.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... ... |  |
| pps_scaling_list_data_present_flag | u(1) |
| if( pps_scaling_list_data_present_flag ) |  |
| pps_base_pred_scaling_list_flag | u(1) |
| if( !pps_base_pred_scaling_list_flag ) |  |
| scaling_list_data( ) |  |
| } |  |
| ... ... |  |
| rbsp_trailing_bits( ) |  |
| } |  |

- pps_base_pred_scaling_list_flag is signalled if pps_scaling_list_data_present_flag is equal to 1. pps_base_pred_scaling_list_flag equal to 1 specifies that the scaling list data are inferred to be equal to those specified in the reference layer picture parameter set. pps_base_pred_scaling_list_flag equal to 0 specifies that scaling list data present in the sequence parameter set. The value shall be 0 if the reference layer is the base layer and the value of avc_base_layer_flag in vps_extension() is 1. When not present, the value is inferred to be 0.

In the proposed method 3, the semantics of the syntax element “scaling_list_pred_matrix_id_delta” in scaling list data syntax is modified, as follows. (highlighted in yellow)

- scaling_list_pred_matrix_id_delta[ sizeId ][ matrixId ] specifies the reference scaling list used to derive ScalingList[ sizeId ][ matrixId ] as follows:

–	If scaling_list_pred_matrix_id_delta is equal to 0, the scaling list is inferred from the default scaling list ScalingList[ sizeId ][ matrixId ][ i ] as specified in Table 7-5 and Table 7-6 for i = 0..Min( 64, ( 1  <<  ( 4 + ( sizeId  <<  1 ) ) ) ) when nuh_layer_id is equal to 0 (i.e. base layer.) When nul_layer_id is greater than 0 (i.e. enhancement layer,) the scaling list is inferred from the reference layer scaling list ScalingList[ sizeId ][ matrixId ][ i ] for i = 0..Min( 64, ( 1  <<  ( 4 + ( sizeId  <<  1 ) ) ) )

–	Otherwise, the scaling list is inferred from the reference scaling list as follows: ...

In the proposed method 4, a new syntax element “base_pred_scaling_list_flag” is introduced to scaling list data syntax. The syntax and semantics are described as follows.

| scaling_list_data( ) { | Descriptor |
| --- | --- |
| for( sizeId = 0; sizeId < 4; sizeId++ ) |  |
| for( matrixId = 0; matrixId < ( ( sizeId = = 3 ) ? 2 : 6 ); matrixId++ ) { |  |
| scaling_list_pred_mode_flag[ sizeId ][ matrixId ] | u(1) |
| if( !scaling_list_pred_mode_flag[ sizeId ][ matrixId ] ) |  |
| base_pred_scaling_list_flag | u(1) |
| scaling_list_pred_matrix_id_delta[ sizeId ][ matrixId ] | ue(v) |
| else { |  |
| ... ... |  |
| } |  |
| } |  |
| } |  |

- base_pred_scaling_list_flag is signalled if scaling_list_pred_mode_flag is equal to 0. base_pred_scaling_list_flag equal to 1 specifies the reference scaling list used to derive ScalingList[ sizeId ][ matrixId ] is from a reference layer. base_pred_scaling_list_flag equal to 1 specifies the reference scaling list used to derive ScalingList[ sizeId ][ matrixId ] is from the same layer. The value shall be 0 if the value of sizeId shall be 2 or 3, the reference layer is the base layer, and the value of avc_base_layer_flag in vps_extension() is 1.

In single layer video coding there are two types of predictions, i.e. Intra prediction and Inter prediction. When a block is Intra predicted, Intra scaling list with the corresponding size and colour space is applied. When a block is Inter predicted, Inter scaling list with the corresponding size and color space is applied. In scalable video coding, there is another prediction mode, i.e. inter-layer texture prediction mode (or “Intra_BL” mode in prior art standard.) In this contribution we propose to treat this mode the same as Inter prediction mode. That is, if a block is inter-layer texture predicted, Inter scaling list with the corresponding size and color space is applied.

# Patent rights declaration(s)

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Corp. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).