INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC1/SC29/WG11 MPEG2016/m37624

February 2016, San José, CA, US

| Source | Samsung |
| --- | --- |
| Status | Input Document to JCT-VC |
| Title | Adaptive Gamut Expansion for Chroma Components |
| Author | A Dsouza Aishwarya K Pachauri |

# Abstract

This proposal describes a method for content adaptive gamut expansion of chroma components before quantization in order to minimize the noise. The document also proposes the addition of an additional SEI message to be used for post processing. Objective gains of 14% at scaling factor of 1.4 in deltaE BD-Rate is reported for the HDR anchor sequences 3.2.

- Introduction

Figure 1 describes the end-to-end coding chain for HDR video content as specified by the MPEG HDR/WCG CfE. The input video is captured in linear light domain, in 4:4:4 RGB color space with floating point accuracy. This data is then converted from a half-precision floating point 4:4:4 RGB linear-light format to single precision floating point and mapped using a PQ transfer function to R’G’B’(float). The mapped R’G’B’ values are then converted to Y’CbCr(float). It is then quantized to 10-bit fixed point format (DY’DCbDCr), followed by chroma down-sampling from 4:4:4 to 4:2:0 and is encoded using HEVC Main10 profile encoder.

Figure 1: End-end-end HDR coding chain

The scope of this document lies in the single precision floating point to fixed point conversion by suggesting content dependent adaptive gamut expansion for chroma components.

The current CfE uses the following process with BitDepthY and BitDepthC set to 10.

DY'=Clip1YRound1≪ BitDepthY-8* 219*Y'+16

DCb=Clip1CRound1≪ BitDepthC-8* 224*Cb+128

DCr=Clip1CRound1≪ BitDepthC-8* 224*Cr+128

with

Round(x) = Sign( x ) * Floor( Abs( x ) + 0.5 )

Sign (x) = -1 if x < 0, 0 if x=0, 1 if x > 0

Floor(x) the largest integer less than or equal to x

Abs(x) = x if x>=0, -x if x<0

Clip1Y(x) = Clip3( 0, ( 1  <<  BitDepthY ) − 1, x )

Clip1C(x) = Clip3( 0, ( 1  <<  BitDepthC ) − 1, x )

Clip3(x,y,z) = x if z<x, y if z>y, z otherwise.

It was observed that the actual gamut of chroma components was significantly lower than the total permissible range. This can be seen in the histogram as shown in Figure 2.

Figure 2 Market Frame 1 Histogram

- Proposed Method

The range of chroma information in the anchor streams was observed to be significantly lower than the total permissible range. It was also observed that considerable quantization noise is introduced during the conversion from floating point to a signal of bit-depth BitDepthC. The quantization noise in the color components can be mitigated by expanding their gamut before quantization. It was found that expanding the range for chroma values per frame results in significant improvement in the coding efficiency over Anchors.

The quantization process for chroma was modified as follows.

DCb=Clip1c(Round((1≪BitDepthC-8)*(224*Cb'+128)))

DCr=Clip1c(Round((1≪BitDepthC-8)*(224*Cr'+128)))

Where

Cb'=SCb*Cb-Cbmin+Cbmid-SCb*(Cbmid-Cbmin)

Cr'=SCr*Cr-Crmin+Crmid-SCr*(Crmid-Crmin)

Cbmin=Minimum Floating Point Value for Cb

Cbmax=Maximum Floating Point Value for Cb

Crmin=Minimum Floating Point Value for Cr

Crmax=Maximum Floating Point Value for Cr

Cbmid=Cbmax+Cbmin2

Crmid=Crmax+Crmin2

The values of Cbmin, Cbmax, Crmin and Crmax are signalled in an SEI message.

At the decoder end, floating point to fixed point conversion is performed using the following process:

Cb'=(D'Cb1 ≪BitDepthC-8-128)/224

Cr'=(D'Cr1 ≪BitDepthC-8-128)/224

The original range of the float signal is then recovered to obtain the original gamut of the chroma components using the below step

Cb=Cb'SCb+  Cbmax+Cbmin 2*SCb-1SCb

Cr=Cr'SCr+  Crmax+Crmin 2*SCr-1SCr

Cbmax, Cbmin, Crmax and Crmin are retrieved from the SEI message sent by the encoder.

- Results

The objective results of our proposal are presented in the table below. As can be seen, the proposal shows considerable objective gain over anchor performance. Detailed results for the same are provided in the excel sheet attached.

Table 1. Objective Results At Scaling Factor 1.4

- Subjective Results

# It was observed that the above method improved the subjective quality of the test sequences as compared to Anchor 3.2. For example, the rope in BalloonFestival has improved quality and visibility with AGE as compared to SuperAnchor 3.2. Figure 3 to 5 shows comparison of AGE with respect to Anchor 3.2 with the most significant improvement circled.

SuperAnchor 3.2

AGE

Figure 3 Balloon Festival R4 Frame 27

SuperAnchor 3.2

AGE

Figure 4 MagicHour Cut1 R4 Frame 48

SuperAnchor 3.2

AGE

Figure 5 Market R4 Frame 7

- Signaling Mechanism

The above results show that increasing the gamut of chroma components for various sequences can provide significant objective gains in deltaE metric. The encoder can send the minimum and maximum values for the color components per frame along with the scaling factor used which can be used by the decoder to restore the original signal.

Hence the following syntax and text modifications relative to the current HEVC standards are proposed.

In 7.3.2.3.1 (General picture parameter set RBSP syntax), replace the syntax table with the following:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| dependent_slice_segments_enabled_flag | u(1) |
| output_flag_present_flag | u(1) |
| num_extra_slice_header_bits | u(3) |
| sign_data_hiding_enabled_flag | u(1) |
| cabac_init_present_flag | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| init_qp_minus26 | se(v) |
| constrained_intra_pred_flag | u(1) |
| transform_skip_enabled_flag | u(1) |
| cu_qp_delta_enabled_flag | u(1) |
| if( cu_qp_delta_enabled_flag ) |  |
| diff_cu_qp_delta_depth | ue(v) |
| pps_cb_qp_offset | se(v) |
| pps_cr_qp_offset | se(v) |
| pps_slice_chroma_qp_offsets_present_flag | u(1) |
| weighted_pred_flag | u(1) |
| weighted_bipred_flag | u(1) |
| transquant_bypass_enabled_flag | u(1) |
| tiles_enabled_flag | u(1) |
| entropy_coding_sync_enabled_flag | u(1) |
| if( tiles_enabled_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height_minus1[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |
| pps_loop_filter_across_slices_enabled_flag | u(1) |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_deblocking_filter_disabled_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| pps_scaling_list_data_present_flag | u(1) |
| if( pps_scaling_list_data_present_flag ) |  |
| scaling_list_data( ) |  |
| lists_modification_present_flag | u(1) |
| log2_parallel_merge_level_minus2 | ue(v) |
| slice_segment_header_extension_present_flag | u(1) |
| pps_extension_present_flag | u(1) |
| if( pps_extension_present_flag ) { |  |
| pps_range_extension_flag | u(1) |
| pps_multilayer_extension_flag | u(1) |
| pps_3d_extension_flag | u(1) |
| pps_scc_extension_flag | u(1) |
| pps_hdr_age_extension_flag | u(1) |
| pps_extension_3bits | u(3) |
| } |  |
| if( pps_range_extension_flag ) |  |
| pps_range_extension( ) |  |
| if( pps_multilayer_extension_flag ) |  |
| pps_multilayer_extension( ) /* specified in Annex F */ |  |
| if( pps_3d_extension_flag ) |  |
| pps_3d_extension( ) /* specified in Annex I */ |  |
| if( pps_scc_extension_flag ) |  |
| pps_scc_extension( ) |  |
| if( pps_hdr_age_extension_flag ) |  |
| pps_age_extension( ) |  |
| if( pps_extension_3bits ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

Add a new 7.3.2.3.4 syntax table (pps_hdrwcg_extension):

| pps_hdr_age_extension( ) { | Descriptor |
| --- | --- |
| for( c = 0; c < 2; c++) |  |
| { |  |
| age_scale_factor_minus1[ c ] | ue(v) |
| } |  |
| } |  |

Semantics changes

In 7.4.3.3.1 (General picture parameter set RBSP semantics), add the following semantics:

pps_hdr_age_extension_flag equal to 1 specifies that the pps_hdr_age_extension( ) syntax structure is present in the PPS RBSP syntax structure. pps_hdr_age_extension_flag equal to 0 specifies that this syntax structure is not present. When not present, the value of pps_hdr_age_extension_flag is inferred to be equal to 0.

pps_extension_3bits equal to 0 specifies that no pps_extension_data_flag syntax elements are present in the PPS RBSP syntax structure. When present, pps_extension_3bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Values of pps_extension_3bits not equal to 0 are reserved for future use by ITU-T | ISO/IEC. Decoders shall allow the value of pps_extension_3bits to be not equal to 0 and shall ignore all sps_extension_data_flag syntax elements in an PPS NAL unit. When not present, the value of pps_extension_3bits is inferred to be equal to 0.

Add a new semantics section 7.4.3.3.5 (Picture parameter set HDR gamut expansion table semantics)

age_scale_factor_minus1[ c ] specifies the quantized scale factor used for gamut expansion in the pre-processor for color component c. When not present the age_scale_factor_minus1[ c ] is inferred to be equal to 1. The value of age_scale_factor_minus1[ c ] shall be less than or equal to 256.

For the delivery of the required information to the decoder, a new Adaptive Gamut Expansion SEI message has been proposed whose syntax and semantics are as shown below.

Syntax:

| adaptive_gamut_expansion ( payloadSize ) { | Descriptor |
| --- | --- |
| age_cancel_flag | u(1) |
| age_range_id | ue(v) |
| if( !age_cancel_flag ) { |  |
| age_persistence_flag | u(1) |
| age_num_comp_minus1 | ue(v) |
| for( c=0; c <= age_num_comp_minus1+1; c++) { |  |
| age_min_val[c] | u(v) |
| age_max_val[c] | u(v) |
| } |  |
| } |  |
| } |  |

Semantics:

The adaptive gamut expansion SEI message provides information to be used in the post-processing of the decoded picture. The process of gamut expansion and restoration has been described in the above sections and can be performed using the transmitted SEI message.

age_cancel_flag equal to 1 indicates that the adaptive gamut expansion SEI message cancels the persistence of any previous AGE SEI message in output order that applies to the current layer. age_cancel_flag equal to 0 indicates that adaptive gamut expansion information follows.

age_range_id contains an identifying number that may be used to identify the purpose of the one or more adaptive gamut expansion SEI messages. The value of age_range_id shall be in the range of 0 to 232 − 2, inclusive.

age_persistence_flag specifies the persistence of the adaptive gamut expansion SEI message for the current layer. age_persistence_flag equal to 0 specifies that the adaptive gamut expansion applies to the current picture only.

Let picA be the current picture. age_persistence_flag equal to 1 specifies that the adaptive gamut expansion persists for the current layer in output order until either of the following conditions is true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB in the current layer in an access unit containing a adaptive gamut expansion SEI message with the same value of age_range_id and applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

age_num_comp_minus1 specifies the number of components for which the adaptive gamut expansion information will be signalled. The value of age_num_comp_minus1 shall be in the range of 0 to 2, inclusive.

age_min_val[ c ] specifies the quantized minimum value of the component in the frame.

age_max_val[ c ] specifies the quantized maximum value of the component in the frame.

Where,

c = 0 denotes the Cb component;

c = 1 denotes the Cr component.

- Conclusion

This contribution shows that the existing HDR/WCG video coding technology used in generating the MPEG CfE anchors can be significantly improved. The quantization noises introduced in the float-to-fixed conversion can be mitigated substantially by using the methods shown in the document. The objective results show considerable gains versus the CfE Anchors.

- References

- A. Luthra, E. Francois and W. Husak, “Call for Evidence (CfE) for HDR and WCG Video Coding”, Doc. N15083, February 2015, Geneva, Switzerland.

- E. François, “AhG HDR/WCG: Report of EE3 on Objective Test Methods”, MPEG document m34638, October 2014, Strasbourg, France.